Oviphone Technology Limited Company: B2315L device LORAWAN protocol

Catalogue

1. Overview	1

2. Equipment instructions	1

2.3 Device downlink description	1

2.1 Device functions and instructions for use	3

2.2 Default reporting logic for the device	4

3. Protocol Data Packet Structure	4

3.1Message ID	5

3.2 Payload	5

3.3 Checksum	6

4. messages	7

4.1 Equipment information related	7

4.1.1 Battery Power, Predometer, Signal Level Upload) (0xF9)	7

4.1.2 Device Firmware Version Upload(0xBB)	7

4.2 Positioning related reporting	8

4.2.1GPS/BDS Position Reporting: Location Data Reporting (0x03)	8

4.2.2 Bluetooth positioning information(LBE Location)（0xD6）	9

4.3：Alarm related reporting	10

4.3.1Alarm message（MsgId=0x02）	10

4.3.2Alarm data upload (MsgId=0x21) (supplement to 02)	10

4.4 Equipment information and status reporting	11

4.5 Downstream feedback report	11

4.5.1Download Message Check(0xC0)	11

4.6Health related reports	12

4.6.1 Health data(MSGID=0x32)	12

4.6.2Heart rate, Blood Pressure(0xC2)--Previously used equipment	13

4.6.3 Temperature（MsgId=0xBA）---Previously used equipment	14

5. Setting	15

5.1Downlink	15

5.1.1 Send Message to Device (MSGID=0x28)	15

5.1.2 Set periodic positioning (MSGID=0x17)	15

5.1.3 Setting（0XCE）---See detailed instructions-Important Downside	16

5.1.4 Set time zone (MSGID=0xD2)(* Specific version support)	18

5.1.5Issue Extended Inactivity Alarm trigger time（MSGID=0XCC）---Special firmware support	18

4.6 Drop alarm sensitivity setting (0xCE15) --Special firmware support（0xCE15）---Special firmware support	19

6. Server time synchronization	20

6.1 Request time calibration data command	20

6.2 Time calibration request data reply format	20

- Overview

| V1.0 |  | 2025-05-08 | Organize the directory, update the agreement content, and add the use instructions of new equipment |
| --- | --- | --- | --- |
| V2.1 |  | 2025-06-25 | Add blood oxygen device B2315PL, and report relevant description of health. The default general version B2315L has no blood oxygen function |

This agreement is applicable to the Oviphone B2315L LoRaWAN watch. If downlink confirmation (full duplex mode) or other protocols are required, please consult with Oviphone Communication (Oviphone Communication has other customized protocol applications).

This protocol use for Oviphone B2315L LoRaWAN wristband) .

If you need downlink confirmation (full-duplex mode) or other protocols,  please check with Oviphone.

With the continuous improvement and enrichment of device functions, this agreement will be constantly updated. Please download the latest version from the server. This document will continue updape, please download the newest version.

The module over there:

Wristband register network method for watches:

wristband register network：Activation by Personalization (ABP)

Default  APPSKEY： 2B7E151628AED2A6ABF7158809CF4F3C

Default  NWKSKEY：735F2F22103042BE724197AC1727EA94

If you need special KEY, please contact with Oviphone.

- Equipment instructions

2.3 Device downlink description

- General version:

- Equipment positioning report frequency is issued (0x17):

- The reported frequency is set to the lowest 1 minute. After receiving the downlink command, the device receives the command according to the time period and frequency of the issued command Report the data according to the default reporting frequency outside the time period, such as: 00:00-18:00 10-minute location report Report outside the time period at the default reporting frequency of 10 minutes

- Text message sent (0x28)：

- GB2312 Code, up to 20 bytes (no more than one screen display, the screen can display a maximum of 8 Chinese characters), one Chinese character occupies 2 bytes, one English letter occupies 1 byte

- After receiving a text message, the green light flashes, the bracelet vibrates, and the message is displayed on the interface. If you click the button before the screen goes off, the text message disappears; if you do not click the button before the screen goes off, the text message will remain in the menu

- Equipment positioning priority distribution (0xCE01):

The default positioning priority is Bluetooth beacon> gps If the positioning priority of the following is: gps> Bluetooth beacon, then the GPS positioning cannot be switched

Bluetooth beacon, when the positioning is successful, will not switch to the next positioning priority to produce positioning

- Equipment health sampling frequency issued (0xCE02):

- The default health sampling frequency is 2 minutes, and the minimum reporting time is 2 minutes

(5)Drop alarm switch (0xCE07):

After the downward close, the equipment does not report a Drop alarm

(6)Extended Inactivity Alarm switch(0xCE08):

The device does not report Extended Inactivity Alarm after the shutdown of the downward movement

(7)Extended Inactivity Alarm triggering time (0xCC):

Value range: 2 minutes to 60 minutes. For example, if the lower value is 10 minutes trigger time, then the device will trigger Extended Inactivity Alarm if it does not move for 10 minutes

(8)Drop sensitivity and height (0xCE15):

Sensitivity: The degree of compliance with the drop algorithm, providing 5 setting levels (0-4): low-medium-low-medium-medium-high-high.

Height: The height that triggers a drop alarm, providing 5 setting levels (0-4):0.5m-1.0m-1.5m-2.0m-2.5m.

## 2.1 Device functions and instructions for use

General version:

- Power on：

- Please charge fully before the first use. Charging light effect: Red light flashes during charging, green light stays on during full charge,

- Note: Do not check the device signal while charging

- Automatic start: when fully charged, unplug the charging cable and start automatically

- Manual start: Long press the upper key for 10s and then release, "Welcome" will appear on the interface

- Note: The default wearing state is on, and no heart rate report was detected to trigger the fall alarm

- power off：

- Low power shutdown: the screen goes off after the interface displays Byebye

- Manual shutdown: Shutdown: Stay on the "About" page of the interface, and you will automatically enter the version page. Then continue to automatically enter the ICCID interface, and finally enter the IMEI interface. Wait for the specific IMEI number to pop up, and then hold down the touch button for 3s. When the screen shows 'Bye', release the button

- Charging shutdown: After connecting the charging cable, the red light flashes to enter the charging state

- SOS function:

- Trigger mode: The device does not go into sleep after triggering. Long press the button for 3s, and the red light will be on for a long time after SOS appears on the interface

Cancel mode: In SOS mode, light up the screen and hold down the button for 3s. After the interface displays SOS cancel, the red light goes out

- signal condition ：

- No signal: The interface signal step column graph shows 'X' in the signal grid

- There is a signal: the interface signal has a stepped columnar graphic

- Note: The signal of lora device depends on whether the data of the device is reported successfully. If the device fails to report, the signal will become the state of no signal

- The device is dormant：

- Trigger conditions: The device does not move for 40 minutes, enters the sleep mode, and does not report the location health data

2.2 Default reporting logic for the device

- General version:

- Power signal status related reporting

- F9: A location and health report will follow

- Location-related reporting

- gps/ Bluetooth beacon: the default reporting frequency is 10 minutes, and the default positioning priority is Bluetooth beacon> gps. Bluetooth positioning is preferred, and GPS is switched when positioning is not available

- Report of alarm related events

- SOS alarm (0x02): triggered by the user. See the previous section for the triggering method

SOS cancellation (0x02): The user actively triggers the cancellation. See the previous section for the triggering method

Power-off alarm (0x21): The device shuts down actively or at low power. See the previous section for triggering mode

Wearing and falling off alarm (0x02): The device determines the wearing according to the healthy sampling reporting frequency, reports the wearing alarm when the heart rate is measured, and reports the falling off alarm when the heart rate is not measured

Low power alarm (0x21): triggered when the current power level of the device is less than or equal to 0

Dorp alarm (0x02): Requires device firmware support to trigger the fall algorithm when the device falls from a certain height

Extended Inactivity Alarm (0x02): Requires device firmware support, triggered when the device remains motionless for a long time while wearing

- Health-related reporting

- Step count, heart rate, body temperature & wrist temperature, blood pressure（B2315PL support） (0x32): the default reporting frequency is the same as the location reporting frequency, 10 minutes

- Equipment information and status report

- Software version and model (0xBB): The computer will report one

- Downstream feedback

- Downstream feedback (0xC0): The server downstream instruction device reports it after receiving it

- Protocol Data Packet Structure

A basic protocol data packet structure is shown in Figure 1:

3.1Message ID

MessageId represents the content as described in Chapter 3.

3.2 Payload

The payload below refers to the effective content of the protocol, excluding the head token and checksum. The length of the content is indicated afterwards.

The data formats used in the payload are shown in the following table:

[U-unsigned; I-signed; X-bitfield; number-bytes occupied]

In the protocol below, little-endian is used for all data types except for ch, u8, i8, and x8.

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | explain |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | char |
| u8 | Unsigned Char | 1 | 0..255 | 1 | unsigned short |
| i8 | Signed Char | 1 | -128..127 | 1 | short |
| x8 | Bitfield | 1 | - | - | bit |
| u16 | Unsigned Short | 2 | 0..65，535 | 1 | unsigned int |
| i16 | Unsigned Short | 2 | -32,768..32,767 | 1 | int |
| x16 | Bitfield | 2 | - | - | （bit）2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | unsigned long |
| i32 | Signed Long | 4 | -2,147,483,648..2,147,483,647 | 1 | long |
| u64 | Uint64_t | 8 | 0..18,446,744,073,709,551,616 | 1 | uint64_t |
| float | float | 4 | -3.44*10e38..3.4*10e38 | - | float |

3.3 Checksum

The content to be added to the checksum includes the payload, as shown in Figure 1. The algorithm is as follows, where Buffer[N] represents the data to be accumulated.

Ck_sum = 0

For(i=0; i<N; i++)

{

ck_sum = ck_sum + Buffer[i]

ck_sum = ck_sum % 0x100

}

ck_sum = 0xFF – ck_sum

Return ck_sum

Where ck_sum cannot exceed 0xFF, so after each loop, it must be modulo 0x100 and then taken the remainder.

- messages

4.1 Equipment information related

4.1.1 Battery Power, Predometer, Signal Level Upload) (0xF9)

payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Power type: 2 percent |
| 2 | u16 | Bat_volt |  | -/- | Electricity value |
| 1 | U8 | Signal_type |  |  | Signal type: 0 percent |
| 2 | I16 | Signal_strength |  |  | signal intensity |
| 1 | U8 | step_type |  |  | Step counting type: 0 full step counting |
| 4 | U32 | step |  |  | Step |
| 4 | U32 | timestamp | -/- | -/- | utc time stamp |

Example： BD F9 02 1E00 00 5000 00 94040000 8CAF5667 CD

F9 :		 	MSGID

02：			Percentage of power

1E00 :		littele Endian，Power 30%

00：			Signal strength percentage is indicated

5000： 		littele Endian，Signal strength 80%

00：			Count all steps

0x00000494：Prodometer 1172 (step)；

8CAF5667：	Timestamp: Beijing time2024-12-09 16:51:24

4.1.2 Device Firmware Version Upload(0xBB)

evice power on, upload the firmware version）

payload contents

| Byte offset | Format | Name | Unit | Decription |
| --- | --- | --- | --- | --- |
| 1 | UINT8 | Version_len | / | Software version number and length |
| N | ASCII[n] |  | / | Software version |

Example：

BD BB 10 43323331304C2E3437302E41412E30 31

Report the content C2310L.470.AA.01

4.2 Positioning related reporting

4.2.1GPS/BDS Position Reporting: Location Data Reporting (0x03)

payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | -/- | longitude |
| 8 | Double | lat |  |  | latitude |
| 1 | U8 | north_south |  |  | /*N or S*/ |
| 1 | U8 | east_west |  |  | /*E or W*/ |
| 1 | U8 | status |  |  | /*A or V*/ |
| 4 | U32 | Timestamp |  |  | 时间戳(Timestamp) |

Example：bd 03 22fb20cb827a5c40 21ea3e00a9953640 4e 45 41 cf084e5f 13

03:					MSGID

22fb20cb827a5c40 :	小端（littele Endian），0x405c7a82cb20fb22，数据为double类型，需要转为浮点数，longitude值为：113.9142330000000 （dd.dddd格式）；(Double type, need change the data to Floating point

21ea3e00a9953640 :	小端（littele Endian），0x403695a9003eea21，数据为double类型，需要转为浮点数，longitude值为：23.5846100000000（TYPE:dd.dddd）；(Double type, need change the data to Floating point)

4E :					ASCII 编码表述，南、北纬度，范围为/*N or S*/，表示为：N（北纬）；

(ASCII code, south and north latitude, the range is /*N or S*/,: N (north latitude))

45 :					ASCII 编码表述，东、西经度，范围为/*E or W*/，表示为：E（东经）；

(ASCII code, east and west longitude, range is /*E or W*/: E (east longitude))

41 :					ASCII 编码表述，定位状态，范围为/*A or V*/，表示为：A（有效）；

(ASCII code representation, positioning status, range is /*A or V*/, expressed as: A (valid))

cf084e5f :			小端（littele Endian），0x5f4e08cf，Unix时间戳转换后，值为：2020/9/1 16:39:43 ；

Example of GPS parsing (JAVA)：

DBDBDBDB037d9f84ac81815c40e766926b1d8936404e4541749d695f0b

//DBDBDBDB03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public static void main(String[] args){

//eg:7d9f84ac81815c40 -->405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d",16)));  //114.02353966666665

//message e766926b1d893640  actual value  4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16))) ;//22.535605166666667

HexToStr(data.Substring(“4e”));   //N

HexToStr(data.Substring(“45”));//E

HexToStr(data.Substring(“41”));//AA represents data "OK", V represents a warning

//Message 749d695f actual value 5f699d74

Date date=new Date();

date.setTime(Long.parseLong(“5f699d74",16)*1000);

SimpleDateFormatsdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

4.2.2 Bluetooth positioning information(LBE Location)（0xD6）

Payload:

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | 目前固定为0 (Fix value 0) |
| U8 | Total_groups | 1 | 总组数, Lora 版本默认1组 |
| Int32 | Utc | 4 | Utc时间戳 (the UTC timestamp of the first group) |
| U8 | Total_PackCount | 1 | 当前时间的包总数 (the ibeacon’s count of the first group) |
| U16 | Major0 | 2 | Major |
| U16 | Minor0 | 2 | Minor |
| S8 | Rssi0 | 1 | Rssi |
| U16 | MajorN | 2 | Major |
| U16 | MinorN | 2 | Minor |
| S8 | RssiN] | 1 | Rssi |

Example：

BD D6 00 01 8CAF5667 04 234E D888 CC 4527 535C C2 4327 D72A BD 234E F3D6 BC D7

D6:	 		MSGID

00:			fixed

01：			A total of 1 group

8CAF5667 	时间戳：北京时间2024-12-09 16:51:24 (Timestamp: Beijing time 2024-12-09 16:51:24)

04			There are currently 4 Bluetooth location messages

234E 		小端（littele Endian）， major = 20031

D888 		小端（littele Endian）， minor = 35032

CC 			-52 (rssi)

4527 		小端（littele Endian）， major = 10053

535C 		小端（littele Endian）， minor = 23635

C2 			-62 (rssi)

4327 		小端（littele Endian）， major = 10051

D72A 		小端（littele Endian）， minor = 10967

BD 			-67 (rssi)

234E 		小端（littele Endian）， major = 20003

F3D6		小端（littele Endian）， minor = 55027

BC			-68 (rssi)

4.3：Alarm related reporting

4.3.1Alarm message（MsgId=0x02）

| Size(Bytes) | Format | Name | Drscription |
| --- | --- | --- | --- |
| 2 | U16 | Upl_warn | Bitfield see below |
| 4 | U32 | Timestamp | timestamp, little endian |

Bitfield WRN:

| 15 |  |  |  |  |  |  | 8 |  |  |  | 4 |  |  | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| bit | Name | Description | 调整后的16进制 Hexadecimal | 十进制 Decimal |
| --- | --- | --- | --- | --- |
| 8 | Wear status | Wear status | 0100 | 1*256=256 |
| 7 | SOS cancel) | SOS cancel | 0080 | 1*128=128 |
| 4 | Takeoff status | Takeoff status | 0010 | 1*16=16 |
| 2 | 关机 Power off | 关机Power off | 0004 | 4 |
| 1 | SOS |  | 0002 | 2 |
| 0 | Low Power | Low Power | 0001 | 1 |
| 14 | Drop the alarm-Special firmware support | Drop the alarm | 1 | 4000 |
| 5 | Sedentary alarm-Special firmware support | Sedentary alarm | 10 | 0020 |

Example：

Low battery	         BD 020100 8CAF5667 B9

Wear status		    BD 020001 8CAF5667 B9

takeoff status	         BD 021000 8CAF5667 B9

SOS					BD 020200 C9BE5667 FA

SOS cancel			BD 028000 19C05667 2A

4.3.2Alarm data upload (MsgId=0x21) (supplement to 02)

Payload contents

| Byte Offset | Format |  | Name |  | Scale | Unit | Drscription |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | U16 |  | type |  |  |  | Alert type |  |
| 4 | U32 |  | Upl_warn |  | - | - | Bitfield see below(Small end first) |  |
| 4 | U32 | Timestamp |  |  |  |  |  | Time stamp (time stamp will be added later when you send it) |

Alarm type =1

| 2 | The device is charging and has been turned off (the firmware needs to support the function) |  | 29 | 0004 | 4 |
| --- | --- | --- | --- | --- | --- |
| 1 | The device is low on power and has been shut down (the firmware needs to support the function) |  | 30 | 0002 | 2 |
| 0 | Device shuts down actively (firmware needs to support the function) |  | 31 | 0001 | 1 |

Example：BD21 0100 01000000 49CD5667 4C

4.4 Equipment information and status reporting

--To be updated

4.5 Downstream feedback report

4.5.1Download Message Check(0xC0)

payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | token长度 |
| N | n*U8 | type | -/- |  | N个token |

Example：BD C0 01 28 59

4.6Health related reports

4.6.1 Health data(MSGID=0x32)

payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | 00 |
| 4 | U32 | Timestamp |  |  | time stamp |
| 2 | U16 | contentLength |  |  | The total length of the following content |
| 1 | U8 | ID |  |  | Id (type (5) + reported value length (3))) |
| N | U8 | Val1 |  |  | The reported value of the ID |
| 1 | U8 | ID |  |  | Id (type (5) + reported value length (3))) |
| N | U8 | Val1 |  |  | The reported value of Idn |

The ID codes are as follows (the protocol defines the high 5 bits of the ID):

01 Step count

02 Heart rate

03 Body temperature

04 Wrist temperature

05 Blood sugar--nonsupport

06 Diastolic pressure

07 Systolic pressure

08 Blood oxygen--B2315PL support

10 Number of sit-ups--nonsupport

11 Running pace and distance--nonsupport

12 Skipping speed and count--nonsupport

Eg1:

BD3200B3C4F26311000A1E00114B314A39711A4A0122bc00416212

B3C4F263:	63f2c4b3(Hexadecimal timestamp)

0F00 :Total length excluding the last byte (checksum)

0A1E00：0x0A=00001 010. The first five bits are 00001, representing the data ID for step counting (0x01). The last three bits are 010, indicating that the length of the step count data is 2 bytes. 0x001E represents step count data, totaling 30 steps.

114B：0x11=00010001. The first five bits are 00010, representing the data ID for heart rate (0x02). The last three bits are 001, indicating that the length of the heart rate data is 1 byte. 0x4B represents the heart rate data, with a heart rate of 75.

314A：0x31=00110001. The first five bits are 00110, representing the data ID for diastolic blood pressure (0x06). The last three bits are 001, indicating that the length of the diastolic blood pressure data is 1 byte. 0x4A represents the diastolic blood pressure data, which is 74.

3971：0x39=00111 001. The first five bits are 00111, representing the data ID for systolic blood pressure (0x07). The last three bits are 001, indicating that the length of the systolic blood pressure data is 1 byte. 0x71 represents the systolic blood pressure data, which is 113.

1A4A01：0x1A=00011 010. The first five bits are 00011, representing the data ID for temperature (0x03). The last three bits are 010, indicating that the length of the temperature data is 2 bytes. 0x014A represents the temperature data, which is 330.

22BC00 ：0x22=00100 010. The first five bits are 00100, representing the data ID for wrist temperature (0x04). The last three bits are 010, indicating that the length of the wrist temperature data is 2 bytes. 0x00BC represents the wrist temperature data, which is 188.

4162:0x41=01000001 The top five are 01000, code data id blood oxygen (0x08), and the last three bits are 001, which means that the length of blood oxygen data is 1 byte. 0x62 is blood oxygen data, which is 98

4.6.2Heart rate, Blood Pressure(0xC2)--Previously used equipment

payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | bp_high | - | - | Systolic Blood:2byte |
| 2 | U16 | bp_low | - | - | Diastolic Blood：2byte |
| 2 | U16 | Bp_heart | - | - | Heart rate:2byte |
| 4 | U32 | Timestamp |  |  | Timestamp |

例：C2 7500 4D00 4800 28F2CD5F

C2 :	 	MSGID；

7500 :	littele Endian， 0x0075  117 (Systolic Blood Pressure Value 117)

4D00 :	littele Endian， 0x004D，77 (Diastolic Blood Pressure Value 77)

4800 :	littele Endian，0x0048，72 (Heart Rate Value 72)

28F2CD5F :	Timestamp: Beijing time2020-12-07 17:13:12

4.6.3 Temperature（MsgId=0xBA）---Previously used equipment

| Byte size | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Time stamp logo | Must |  | 00 -with timestamp； 01 -without timestamp |
| 4 | Int32 | timestamp | Optional |  | timestamp ID is 01, this field is not required |
| 1 | U8 | Temp. type | Must |  | 1: upload wrist and body temp 2:upload wrist, body and environment temp. |
| 2 | S16 | wrist Temp. | Optional |  | One digit after the decimal point is reserved for body surface temperature (×10). The reported value is an integer. It is determined whether there is this field according to the temperature type |
| 2 | S16 | Body Temp. | Optional |  | One digit after the decimal point is reserved for body surface temperature (×10). The reported value is an integer. It is determined whether there is this field according to the temperature type |
| 2 | S16 | environment temperature | Optional | / | One digit after the decimal point is reserved for body surface temperature (×10). The reported value is an integer. It is determined whether there is this field according to the temperature type |

- Setting

5.1Downlink

Note: classA mode receives downlink when reporting, and classC mode receives downlink in real time

说明：classA模式上报时接收下行，classC模式实时接收下行

5.1.1 Send Message to Device (MSGID=0x28)

payload contents

| Byte size | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | U8 | type | Message type, if it is downlink information, the fixed value is 03 |
| 4 | Uint32 | seqID | Information id, uniqueness |
| 1 | U8 | CONTENT LEN | Contect Length |
| N | N | CONTENT | (content, Chinese is GB2312 code, English is ascii code)the maximum support 20 bytes of data |

Example：

bd 28 03 03 00 00 00 0b 68 65 6c 6c 6f 2c 77 6f 72 6c 64dd

Type: 		03

seqId:		03 00 00 00

CONTENT LEN: 0b (10)

CONTENT :	68 65 6c 6c 6f 2c 77 6f 72 6c 64 ( hello,world)

5.1.2 Set periodic positioning (MSGID=0x17)

The platform sets 1~2 time periods to be issued to the terminal, and the terminal uploads data within the specified time period after receiving it

payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Enabled? | Time Slot 1 |
| 1 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -h |  |
| 1 | u8 | time_start_m |  |  | -m |  |
| 1 | u8 | time_end_h |  |  | -h |  |
| 1 | u8 | time_end_m |  |  | -m |  |
| 1 | u8 | enable | -/- | -/- | Enabled? | Time Slot 2 |
| 1 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -h |  |
| 1 | u8 | time_start_m |  |  | -m |  |
| 1 | u8 | time_end_h |  |  | -h |  |
| 1 | u8 | time_end_m |  |  | -m |  |

Example：

Bd 17 01 0300 00001300 00000000000000 dd

0 o'clock to 19 o'clock, positioning once every 3 minutes

5.1.3 Setting（0XCE）---See detailed instructions-Important Downside

| Message | MSG_HRD_DATA |  |  |  |
| --- | --- | --- | --- | --- |
| Decription | setting，Server => Terminal downlink |  |  |  |
| Firmware | -/- |  |  |  |
| Payload Length | 4+n bytes |  |  |  |
| Message structure | Header | Message ID | Payload | Checksum |
|  | token | 0xCE | See below | CK_sum |

payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 0 | u8 | Type | -/- | -/- | Type see below |
| 1 | u8 | Valid | -/- | -/- | Temporary valid, always valid |
| 2 | U16 | Len | -/- | -/- | Length of the following instruction |
| 4 | n |  | -/- | -/- | Main text |

Type 01 Positioning  02 health   03 Alarm (not reserved yet)   04 Local storage (not reserved yet)

05Bluetooth radio switch  06Position and health data reporting switch  07 Fall alarm switch  08 Stay alarm switch, expandable at the back

Valid 00 Always valid 01 Effective this time 02 Off

Length Length of the following instruction

Main text:

Type

01 Positioning function The main text can only be the basic class 01 - gps 02 - wifi 03 - Bluetooth beacon 04 - LBS base station 05 - 125k and later expanded basic class The main text can be a combination of 01 or 010204;

Such as 010203 means using gps wifi Bluetooth beacon;The lora device does not support wifi

Example: wifi positioning priority ( Bluetooth> gps): BDCE0100030003010033

gps positioning priority (gps> Bluetooth): BDCE0100030001030033

Type = 02  Frequency of healthy sampling and reporting

Main text first byte 00-all (heart rate, blood pressure, body temperature)

The second byte is the time interval, and the third byte is the interval unit 00 minutes 01 hours

The third byte indicates that the state is to be maintained indefinitely

Example：BD CE 02 00 0300 00 0200 33

02		Health

00 		good through

0300	Length 0x03

00		fixed

02		Interval unit 2

00		minute

33      checksum

The interval time for collecting and reporting health data (heart rate, blood pressure, wrist temperature and body temperature) is 2 minutes

Type=07 Drop alarm switch

00---open eg:BDBDBDBDCE0700000093

02---close eg:BDBDBDBDCE0702000093

Type=08 Extended Inactivity Alarmswitch

00---open  eg:BDBDBDBDCE0800000093

02---close  eg:BDBDBDBDCE0802000093

5.1.4 Set time zone (MSGID=0xD2)(* Specific version support)

Previously used equipment

| Size(Bytes) | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | S8 |  | time zone |

Example：

BD D2 F8 DD

Set the time zone to -8

Latest shipment equipment usage

| Description | Server=>Terminal |  |
| --- | --- | --- |
| Message structure | Hearer | Message ID |
|  | Token | 0xD2 |

| Format | Name | Unit | Decription |
| --- | --- | --- | --- |
| I8 | Zone | 1 | Time Zone - Hour（-12 ~ +12） |
| U8 | Zone_Minute | 1 | Time Zone - minute |

- Example:

- 1. Set the device time zone to UTC+05:30 (Indian Standard Time):

- bdbdbdbd d2 05 1e ff

- 2. Set the device time zone to UTC-08:00 (Pacific Time):

- bdbdbdbd d2 f8 00 ff

### 5.1.5Issue Extended Inactivity Alarm trigger time（MSGID=0XCC）---Special firmware support

| Message | MSG_HRD_DATA |  |  |  |
| --- | --- | --- | --- | --- |
| Decription | Server => Terminal |  |  |  |
| Firmware | -/- |  |  |  |
| Payload Length | 4+n bytes |  |  |  |
| Message structure | Hearer | Message ID | Payload | Checksum |
|  | token | 0xCC | See below | CK_sum |

payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | minute | -/- | -/- | Issue the extended inactivity alarm duration (unit: minutes; value range 2--60)，No activity during this time will trigger a Issue the extended inactivity alarm |

Example：

bd cc 05 dd

05--0x05 No activity for 5 consecutive minutes. The watch reports a Issue the extended inactivity alarm  (when the watch is worn)

### 4.6 Drop alarm sensitivity setting (0xCE15) --Special firmware support（0xCE15）---Special firmware support

Sensitivity default: medium high, 0.5m

| Message | MSG_CFG_DATA |  |
| --- | --- | --- |
| Description | Server=>Terminal |  |
| Message structure | Hearer | Message ID |
|  | Token | 0xCE |

| Format | Name | Unit | Decription |
| --- | --- | --- | --- |
| U8 | Type | 1 | Type=0x15 |
| U8 | Valid | 1 | Leave it 0 by default, no impact |
| U16 | Len | 2 | contention length |
| U8 | Param Type | 1 | Parameter type: 0x00: sensitivity, 0x01: height |
| U8 | Param Level | 1 | 5 levels (0-4) |

（1）Sensitivity: The degree to which the drop algorithm is satisfied. Five Settings (0-4) are provided: Low-Medium Low (low) -Medium-Medium High (high).

（2）Height: refers to the height that meets the trigger fall alarm, providing 5 setting levels (0-4):0.5m-1.0m-1.5m-2.0m-2.5m.

If the height is set to "1.5m", the equipment needs to be at least 1.5m to trigger the fall prediction alarm.

- Example: Note that sensitivity and height cannot be combined to send a message.

- Set the sensitivity to "medium-low": bdbdbdbdce150002000001ff.

- Set the height to "1.5m": bdbdbdbdce150002000102ff

- Server time synchronization

This synchronization time instruction is supported only by specific versions, and the general version will automatically synchronize after entering the network

6.1 Request time calibration data command

| Byte offset | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | U8 | HEADER | 0xFF |
| 1 | U8 | SeqId | 0x00 |
| 1 | U8 | End | 0xFF |

Example：FF00FF    --Equipment reported

6.2 Time calibration request data reply format

| Byte offset | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | U8 | HEADER | 0xFF |
| 1 | U8 | SeqId | 0x10 |
| 2 | U16 | years | years |
| 1 | U8 | month | month |
| 1 | U8 | Day | Day |
| 1 | U8 | time | time |
| 1 | U8 | Minute | Minute |
| 1 | U8 | Seconds | Seconds |
| 1 | U8 | End | 0xFF |

Example：FF1007E409020B1B28FF

FF :	Header

10 :	Seqid

07E4:	Year（2020）

09 :	Month，（09）

02 :	Day，（02）

0B :	Hour，（11）

1B :	Minitus，（27）

28 :	Second，（40）

FF :	End

Note: After the device is turned on, it will automatically send the request time synchronization instruction. After receiving the synchronization instruction, it needs to reply the synchronization time data frame to synchronize the device time