Oviphone Device Protocol

（V1.1）

2020-10-22

content

1. overview	1

2. data package structure	1

2.1 Up data package format	1

2.2 Down data package format	4

- Overview

This document defines the Oviphone device protocol between server and device. The protocol base on UDP standard, and use JSON format.

- Data package structure

2.1 Up data package

format：

{

"imei":"869768047575669",  //device ID，Required fields

"data":

[

{

"tm":1607557507,

"wi":"603A7CCE57D2,-80;34967281274C,-75;1852825735DD,-95;320D9E863F59,-100;0495E650FCB1,-25"

"st":2345,

"te":235,

"ba":3,

"bp":"120,80"

...    //fields descript see the following table

},

{

"tm":1607557808,

"wi":"603A7CCE57D2,-80;34967281274C,-75;1852825735DD,-95;320D9E863F59,-100;"0495E650FCB1,-25"

"ste":2350,

"te":235,

"ba":3,

"hr":80

...  ///fields descript see the following table	  },

]

}

Field name are all lowcase

The description of each field in "data" is as follows

| Code | Type | Requred input | Description | Remarks |
| --- | --- | --- | --- | --- |
| tm | int | Y | Timestamp |  |
| gp | String | Optional | Gps， Value Format:“longitude，latitude” | East longitude/ North latitude is positive, west longitude/ south latitude is negative。 eg ： “gp”:”116.13070954,39.809007” |
| wi | String | Optional | Wifi’s Location information，format： “bssid1,rssi1;bssid2,rssi2;bssidN,rssiN;” | eg： "wi":"603A7CCE57D2,-80;34967281274C,-75;1852825735DD,-95;320D9E863F59,-100;0495E650FCB1,-25；" |
| b1 | String | Optional | Blueteeth data type 1 Value format： “major,minor,rssi;major,minor,rssi;major,minor,rssi;” |  |
| b2 | String | Optional | Blueteeth data type 2 valueformat： “mac,rssi;mac,rssi;mac,rssi;” |  |
| wn | int | Optional | Alarm | 1：Low battery， 2：sos, 3:shutdown, 4:Take off - 7:take on - 13:Cancel sos by long press button - 14:Cancel sos by cmd from the server - Eg: - "wn":2 // sos - "wn":13 // cancel sos |
| st | int | Optional | steps | Total steps every day |
| ba | int | Optional | battery | ba=0 10% percent ba=1 30% percent ba=2 60% percent ba=3 100%percent |
| sn | int | Optional | RSSI | percent |
| te | int | Optional | Tempreture,format;“skin-temp，body,env” | Upload many, show many, "," as the separator |
| hr | num | Optional | heartrate |  |
| bp | string | Optional | Bloood Presure，value format： “bsp,bdp” | eg： "bp":"120,80" |
| bs | int | Optional | Blood sugar |  |
| bo | int | Optional | Blood oxygen |  |
| sl | string | Optional | sleep，format： “start_time,delay,status” | start_time: the timestamp of this status beginning delay :delay time unit is second status: 1—deep sleep(ds) 2—light sleep(ls) 3—awake(aw) |
| hw | string | Optional | Health’s Alarm infomation，format： “type，data” | type： 1：heartRate Alarm eg： “hw”：“1,90”// heartRate，is 90 |
| ic | string | Optional | iccid |  |
| ve | string | Optional | version |  |

2.2 Down data package

{

“lp”:”1,2”,

“li”:”5@10001200,3@14001600”,

“hw”:”1,1,300,120,50”

}

| Code | type | Required | Description | Remarks |
| --- | --- | --- | --- | --- |
| eo | int | optional | event | 0:cancel sos Eg: "eo":0 //cancel sos |
| lp | String | optional | Lacation priority setting， format： “Lacation 1，Lacation 2，Lacation 3” | Location type： 1：gps 2.wifi 3.blueteeth eg：if define gps>wfii, “lp”:”1,2” |
| li | String | optional | Positioning interval setting，format： “interval @HHMMHHMM, interval @HHMMHHMM, interval @HHMMHHMM, interval @HHMMHHMM，interval @HHMMHHMM” | interval unit is second HHMMHHMM start hour minute end hour minute. not more than 4 times segment. eg： - “li”:”5@10001200,3@14001600” every day 10 o’clock to 12 o’clock 5 minute location 1 times，14 o’clock to 16 o’clock 3 minute location 1 times - "li":"0" Close interval setting |
| hw | String | optional | Health alarm，format - “type，on/off，interval，upper limit ，low limit ” | type： - Heart rate threshold setting - Tempreture threshold setting - BloodOxygen threshold setting On/Off： 0=off，1=on Interval unit： second eg：heart rate threshole setting “hw”:”1,1,300,120,50” 1-- type 1--on 300—5*60(s) 5 minute 120—upper limit 50 --low limit |