G626-4GCat.1-tcp communication protocol

Directory

1 Overview	3

2 Instructions for use of equipment	3

2.1 Device function and instructions for use	3

2.2 Device default reporting logic	4

2.3 Device downlink description	5

3 Protocol Packet Structure	6

3.1 Data Header	6

3.2 Message ID	6

3.3 Token Generation Mechanism	6

3.4 Payload	7

3.5 Checksum	8

4 Report messages	9

4.1 Connection related reports	9

4.1.1 LNK-LIN (MSGID=0xF0) Request connection (TCP only)	9

4.1.2 Connection Reply (MSGID=0xF1) (TCP Only) - Important	10

4.1.3 New Heartbeat Packet Protocol (MSGID=0xF9)-Important	10

4.2 Alarm-related reporting	13

4.2.1 Alarm data upload-1(MSGID=0x02)	13

4.2.2 Alarm data upload-2(MSGID=0x21)(Supplement to 02)	14

4.3 Device information and status report	16

4.3.1 SIM card ICCID upload (MSGID=0xF3)	16

4.3.2 Status Parameter Reporting (MSGID=0xA9)	17

4.4 Health-related reporting	19

4.4.1 UV data upload (0x2E)	19

4.5 Downlink Feedback Related Reporting	20

4.5.1 Downlink Feedback (MSGID=0xC0)	20

5 Settings	22

5.1 Downlink	22

5.1.1 IP & Domain Name Settings (0xC3) (TCP Only)	22

1 Overview

This protocol is suitable for 4G CAT1 devices and currently supports G626 products. It uses a 32-bit data header for synchronization and terminal identification; uses a low-overhead checksum algorithm for checksum protection; and uses message identifiers to identify different messages. It needs to be parsed according to the actual report. The document is currently being continuously maintained. If you find any errors or omissions, please give feedback in time. Thank you very much. The protocol content is 4g report content. If you need Bluetooth broadcast data protocol, please consult relevant docking personnel.

2 Instructions for use of equipment

2.1 Device function and instructions for use

General version:

(1) Power on:

Please fully charge it before using it for the first time. Charging light effect: Charging: red light flashes, fully charged: green light is always on,

Automatic power-on: After charging, unplug the charging cable to automatically power on. Manual power-on: In the shutdown state, press and hold for 5 seconds, and the red and green lights flash alternately.

(2) Shutdown:

Charging shutdown: The red light flashes after connecting the charging cable to enter the charging state. Manual shutdown: In the power-on state, long press for 5 seconds, the red light starts to flash, wait for the red light to stop flashing, and the device shuts down. Low battery shutdown: The red light flashes and then stops flashing, and the device shuts down.

(3) Device status:

Signal status: When the device is turned on, the red and green lights flash alternately to indicate that it has been connected to the network.

Device power status: The default is 4.2 V battery, and the voltage is divided into 5 levels, each level 20 %.

Short press the button, the green light flashes to indicate the power level, flashes 5 times greater than 80%, flashes 4 times the power 60 %-80%,

Flashes 3 times the power 40 %-60%, flashes 2 times the power 20 %-40%, the red light flashes 3 times the power is less than 20 %

2.2 Device default reporting logic

General version:

(1) Connection related reporting

F0 request: The device is a short link. Under normal server connection and network conditions,

It will be reported once when it is turned on, and then each report will request a connection -- a reply is required to connect to the server

F9: Power and signal reporting, a report will be sent along with location reporting -- requires server response

(2) Health-related reporting

UV data (0x2E):uv/pho/irData collection: Sampling time is:6:00～20:00,

Sampling interval is1minute, default is1hour per packet, and the collected data is sent every two hours

(3) Alarm-related reporting

Shutdown alarm (0x21): The device actively shuts down, charging shutdown or low battery shutdown, see the previous section for triggering methods

Low battery alarm (0x02): Triggered when the device's current battery level is less than or equal to 0

(4) Device information and status reporting

Status parameters (0xA9): One report will be made when the device is turned on

SIM card ICCID (0xF3): One report will be made when the device is turned on

(5) Downlink feedback

Downlink feedback (0xC0): The server downlink instruction is reported after the device receives it.

Note: The device reports include the case of combined reporting, that is, a data packet contains multiple complete messages. Pay attention not to miss any. The message is a complete message, and there will be no phenomenon of being interrupted in the middle and in the next data packet.

Eg:bdbdbdbdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9bdbdbdbdf9010000006400002800000019a9cf61ca

This data packet contains messages of (0xD6) Bluetooth positioning and (0xF9) power signal.

(0xD6) Bluetooth positioning: bdbdbdbdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9

(0xF9) Power signal: bdbdbdbdf9010000006400002800000019a9cf61ca

2.3 Device downlink description

General version: The downlink instruction needs to be continuously downlinked twice within 20 milliseconds to ensure success.
Note: For short-link devices, the downlink instruction is received by the device when the device reports. It will take effect in the next reporting cycle after receiving it.

(1) Modify IP and port instruction issuance (0xC3):

The default general version points to the Smart Cloud Platform: 118.178.184.219:8825. If you need to change it, please consult relevant personnel or visit the official website.

3 Protocol Packet Structure

A basic protocol packet structure is shown in Figure 1:

3.1 Data Header

Each packet starts with a 4-byte Header or token (timestamp is used instead in some response messages):

Currently, the token uploaded by OuFu devices is fixed to BDBDBDBD

Header: 0xBD0xBD0xBD0xBD;

Timestamp: 32bits, generated by the server

3.2 Message ID

See Chapter 4 for the content represented by MessgeId.

When TCP is connected each time, the device side will first report the 0xF0 message, which contains the device's unique identifier IMEI.

The server needs to record the IMEI as an identifier and reply with the 0xf1 message. Only then will the device consider the connection successful, otherwise the connection will be disconnected.

The device communicates normally, and the long connection reports a 0xF9 heartbeat packet every 4 minutes by default. The 0xF9 heartbeat packet is reported once after the health location is reported.

The short connection reports a 0xF9 heartbeat packet once at startup, and a 0xF9 heartbeat packet is reported once after the health location is reported. The reporting of the two is not affected by each other.

Note that after the server receives the F9 message, the server needs to issue a command to reply, otherwise the connection will be disconnected.

3.3 Token Generation Mechanism

Currently fixed as BDBDBDBD

3.4 Payload

The payload below refers to the valid text content in the protocol except for the head token and checksum. The following note is the length of the text.

The data format used in the payload is shown in the following table:

[U-unsigned; I-signed; X-bitfield; Number-bytes occupied]

The following protocols use little-endian priority except for ch, u8, i8, and x8

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Explanation |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short Integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short Integer |
| x8 | Bitfield | 1 | - | - | Bit (bit) |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Integer |
| x16 | Bitfield | 2 | - | - | Bit 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long Integer |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long Integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit Long Integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating point type |

3.5 Checksum

The checksum includes the payload, as shown in Figure 1. The algorithm is as follows. By default, the general-purpose version of the device does not require checksum verification, and this part can be ignored. Any byte can be used for the downlink instruction.

//Suppose content="BDBDBDBDE9000600010A00000A00", then sum.ToString("X2") result is 07//The following is the C# code calling method

private string CheckSum(string content)

{

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2");

}

4 Report messages

4.1 Connection related reports

4.1.1 LNK-LIN (MSGID=0xF0) Request connection (TCP only)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xF0 | See below for definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | u64 | IMEI | 1 | - | IMEI number (little endian) |
| 2 | x16 | version |  | - | Bitfield see below |

The device's reported F0 request message must have a server downlink 0XF1 message in response (see the next section for the specific format), otherwise the login will fail.

Example:

Original16hexadecimal message: BDBDBDBDF09B51731BC61603000000C2

BDBDBDBD:4byte message headerHeader

F0:Message ID(Message ID)9B51731BC6160300：IMEI number(Little Endian), converted to Big Endian:000316C61B73519B，

The corresponding decimal is IMEI:869465050010011，0000：version

C2：checksum校验和 (Checksum)

Connection related Q&A:

Each time TCP creates a new connection, it will first report an F0 request. The F0 contains the IMEI, and then the server records this IMEI.

After that, all the data in this connection belongs to this IMEI.

Note: If F0 is not received or F0 communication is abnormal, you can use a third-party network testing tool to verify whether the server communication is normal.

4.1.2 Connection Reply (MSGID=0xF1) (TCP Only) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Timestamp(unix)时间戳 (Timestamp) | 0xF1 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | u32 | Token | 1 | - | Token: Fixed as BDBDBDBD |

Note that the message sequence is different from the normal reported message. This message is issued by the server, not the content reported by the device. The server reply needs to be replied in the current channel>Note that the message sequence is different from the normal reported message. This message is issued by the server, not the content reported by the device. The server reply needs to be replied in the current channel

Example:

Original16Hex message:07FD8860F1BDBDBDBD2E

07FD8860(little endian),Timestamp(unix) timestamp (unit: seconds), converted to big endian6088FD07 converted to decimal:1619590407

Convert to standard time formatUTCTime:2021-04-28 06:13:27

F1:Message ID(message id)

BDBDBDBD:Token

2E:checksumchecksum

The timestamp 10-base to standard time format is the result of adding 1619590407 seconds to 1970-01-01 00:00:00. The device uses the timestamp (unit: seconds) in this message to synchronize the time.

Note: The total number of bytes replied by the server downstream is 10 bytes, such as: 0xBD--1 byte; if the device receives 20 bytes, the connection will not be established

4.1.3 New Heartbeat Packet Protocol (MSGID=0xF9)-Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xF9 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery type 0: 4-level system, 1: 5-level system, 2: percentage, 3: voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery level If Bat_type is 0: The battery level range is 0-3, (0 is 25%, 3 is 100%) If Bat_type is 1: The battery level range is 0-4 (0 is 20%, 4 is 100%) If Bat_type is 2: The battery level range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: percentage, 1: 5-level system 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal strength |
| 1 | U8 | Other_type |  |  | Extended type 0: Full amount of steps 1: Incremental steps 2: Vibration |
| 4 | U32 | Num |  |  | Extended Value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Note: The F9 heartbeat packet must have a server downlink reply. The device considers the server connection to be unbroken only when it receives the server downlink reply; otherwise, it will re-report the F0 request connection.

Example:

Original hexadecimal message: BDBDBDBDF90104000050000095000000E377BD67AA

BDBDBDBD: Header

F9: Message ID

01: Bat_type battery type 01 --> 1 --> 5-level battery (0-4)

0400: Bat_volt battery value converted to big-endian 0004 --> 4 --> Level 4 corresponds to 100% battery

00: Signal_type signal type 00 --> 0 --> percentage

5000: Signal_strength signal strength converted to big-endian 0050 --> converted to decimal 80 --> 80% signal strength

00:Other_type extension type 00-->0--> Full step counting

95000000:Num extension value - step counting converted to big-endian00000095--> Convert to decimal149--> Number of steps is149

E377BD67:Timestamp converted to big-endian67BD77E3--> Convert to10base1740470243--> Timestamp is1740470243seconds --> Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

AA:checksum

Note: The server receives the F9 heartbeat packet and can reply with a fixed message: Fixed reply example BDBDBDBDF301 Number of bytes:6

4.2 Alarm-related reporting

4.2.1 Alarm data upload-1(MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x02 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below |
| 4 | U32 | Timestamp | - | - | Timestamp (a timestamp will be added at the end when re-uploading) |

Upl_warn contents

| bit (corresponds to the position of 1 in binary) | Name | Description | Original message little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 0 | Low battery | Low battery | 0100 | 0001 | 1 |

Example:

Low battery alarm: BDBDBDBD020200E377BD678A

BDBDBDBD:Header

02:Message ID

0100: Upl_warn alarm content, converted to big-endian 0001 --> Low battery alarm

E377BD67: Timestamp, converted to big-endian 67 BD77E3 --> converted to base 10 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

8 A: checksum

4.2.2 Alarm data upload-2(MSGID=0x21)(Supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x21 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | Alarm type |
| N |  | Upl_warn | - | - | Alarm content See below for different Alarm type type definitions |
| 4 | U32 | Timestamp |  |  | Timestamp (a timestamp will be added at the end when re-uploading) |

Alarm type=1

Upl_warn contents

| bit (corresponds to the position of 1 in binary) | Name | Description | Original message little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging shutdown alarm | Device automatically shuts down when charging (firmware support required) | 04000000 | 0004 | 4 |
| 1 | Low battery shutdown alarm | Device shuts down due to low battery (firmware support required) | 02000000 | 0002 | 2 |
| 0 | Active shutdown alarm | Device is manually shut down by someone (firmware support required) | 01000000 | 0001 | 1 |

Example:

Charging shutdown alarm: BDBDBDBD21010004000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

04000000:Upl_warn alarm content, converted to big-endian 0004-->charging shutdown alarm

E377BD67:Timestamp converts the timestamp to big-endian 67BD77E3-->converts to decimal 1740470243-->the timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Low battery shutdown alarm: BDBDBDBD21010002000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

02000000:Upl_warn alarm content, converted to big-endian 0002-->Low battery shutdown alarm

E377BD67:Timestamp converts the timestamp to big-endian 67BD77E3-->converts to decimal 1740470243-->the timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Active shutdown alarm: BDBDBDBD21010001000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type alarm type converted to big-endian 0001-->Alarm type=1

01000000:Upl_warn alarm content, converted to big-endian 0001 --> active shutdown alarm

E377BD67:Timestamp, converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

4.3 Device information and status report

4.3.1 SIM card ICCID upload (MSGID=0xF3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xF3 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 10 | 10*U8 | ICCID | 1 | - | ICCID number - Device SIM card number ICCID |

Note: This message is reported once every time the device is powered on.

Example:

Original hexadecimal message:BDBDBDBDF389861118236001639994CC

BDBDBDBD:Header

F3:Message ID

89861118236001639994:ICCID Device SIM card number ICCID is: 89861118236001639994

CC:checksum

4.3.2 Status Parameter Reporting (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Number of parameter types |
| 1 |  |  |  |  | (Reserved 00) |
| 1 | u8 | Type | -/- | -/- | Parameter type 1, type definition: 00—MCU (firmware), 01—module, 02 sensor (generic version does not have this type) |
| 1 | U8 | NameLen | -/- | -/- | Length of parameter name 1 |
| n | N*u8 | Name | -/- | -/- | Parameter name |
| 1 | u8 | Type | -/- | -/- | Parameter Type 2, Type Definition: 00—MCU (Firmware), 01—Module, 02 Sensor (This type is not currently available in the general version) |
| 1 | U8 | NameLen | -/- | -/- | Parameter Type 2 Length |
| n | N*u8 | Name | -/- | -/- | Parameter name |
| ... | ... | ... | ... | ... | ... |

Note: This message is reported once by default when the device is powered on, uploading the device firmware version number and module version number. It can be left unparsed. The Message ID of the previously shipped device was 0xBB, which can be sent by contacting relevant personnel.

Example:

BDBDBDBDA902000009423732352E4F56303601174E5432364B434E4230304E4E412D4C3032303330393530B5

BDBDBDBD:Header

A9:Message ID

02:TypeCnt indicates that there are 2 types of parameters

00:Reserved field

00:Type=00  00 represents MCU (firmware)

09:The length of the MCU name after is 9 bytes

423732352E4F563036--->Hexadecimal to text (device firmware version number)--->B725.OV06

01:Type=01 01 represents the module

17:The length of the communication module name is 23 bytes

4E5432364B434E4230304E4E412D4C3032303330393530:Hexadecimal to text (4G module version number)

--->NT26KCNB00NNA-L02030950

B5---checksum

4.4 Health-related reporting

4.4.1 UV data upload (0x2E)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x2E | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  | -/- | 0x01 |
| 4 | U8*4 | Timestamp |  | -/- | UTC timestamp |
| 2 | U16 | interval |  | -/- | Data collection time interval (unit: seconds) |
| 1 | U8 | UV_count |  | -/- | Number of UV data in each packet |
| 6*N | U163N | UV_data |  | -/- | One group of UV data has 6 bytes. The first two bytes are the UV value, the middle two bytes are the PHO value, and the last two bytes are the IR value. |

Example:

UV value represents: ultraviolet value, PHO value represents: light intensity value, IR value represents: infrared value

BDBDBDBD2E01E377BD673C003C90019400500070018C00500060018A00500060018700500060018800500060018300400060018800500030017600400050018000400050018100400050018000400050018300400050018100400050018100400050018200400050017F00400060018500400050017F004000700189005000800192005000A0019B005000B001A1006000C001A6006000D001A9006000F001B20060000002BB0070000002B9007000F001B4006000F001B4006000F001B60070001002BF0070002002C30070003002C50070002002C40070002002C30070002002C10070003002C80070004002CB0080004002CA0080004002CB0080004002C9007000F001AC007000E001B3007000D001A9006000B0019A00600090019100500080018D00500070018800500050017

D00400050017C00400050017C00400040017600400020016200400000015E00400000015D00400010016600400010016800400030017000400050018000400080018E005000E7

BDBDBDBD:Header

2 E:Message ID

01 : type

E377BD67 : Timestamp 转为大端 67 BD77E3--> 转 10 进制 1740470243 --> 时间戳为 1740470243 秒-->

转标准时间格式UTC时间： 2025 - 02 - 25 07: 57: 23 --> 转北京时间： 2025 - 02 - 25 15: 57: 23

3 C00:interval 数据采集的时间间隔 转大端 003 C---> 转十进制 60 ---> 数据采集的时间间隔 60 S

3 C:UV_count 后面UV数据的数量有 60 组

9001 : UV_data1 第 1 组UV数据uv的值---> 转大端 0190 ---> 转十进制 400 ---> UV值为 400

9400:UV_data1 The value of UV data pho in group1---> Convert to big endian0094---> Convert to decimal148--->pho value is148

5000:UV_data1 The value of the 1st group of UV data ir ---> Convert to big-endian 0050---> Convert to decimal 80---> ir value is 80

7001:UV_data2 The value of the 2nd group of UV data uv ---> Convert to big-endian 0170---> Convert to decimal 368---> UV value is 368

8C00:UV_data2 The value of the 2nd group of UV data pho ---> Convert to big-endian 008C---> Convert to decimal 140---> pho value is 140

5000:UV_data2 The value of the 2nd group of UV data ir ---> Convert to big-endian 0050---> Convert to decimal 80---> ir value is 80

………And so on until the last group, the number is determined by the 11th byte of the message, in the example, 3C represents 60 groups of data

E7:checksum

4.5 Downlink Feedback Related Reporting

4.5.1 Downlink Feedback (MSGID=0xC0)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC0 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID Length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This command is used for feedback of downlink commands, returning the Message ID of the downlink command received earlier (can be a collective return of multiple Message IDs)

Original hexadecimal message:BDBDBDBDC0011720

BDBDBDBD:Header

C0:Message ID

01:length Message ID length Convert decimal 01--> Message ID length is 1 byte

17Type indicates that the device received the downlink command Message ID is 17

20:checksum

5 Settings

5.1 Downlink

5.1.1 IP & Domain Name Settings (0xC3) (TCP Only)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | Issue Type: Type =1 IPv4, Type =2 IPv6 (Reserved - Not Supported Yet), Type =3 Domain Name (Currently Only Supports ASCII Encoded Domain Names) |
| 2 | U16 | Port |  |  | Port Number (2 Bytes) |
| 1 | U8 | Len |  |  | Length of Subsequent Parameter Content |
| N | U8 | Domainname |  |  | Specific IP or Domain Name Content |

Note: After issuance, the device will no longer communicate on the original server, but will point to the issued and modified server.

Example:

Type=1 IPv4:Modify the downlink IP to:118.178.184.219, port: 8825 BDBDBDBDC30179220476B2B8DB33

BDBDBDBD:Header

C3:Message ID

01:Type 1--IPv4

7922:Port Convert to big-endian 2279-->Convert to decimal 8825-->Port: 8825

04:Len The length of the following parameter content is converted to decimal 04-->The length is 4 bytes

76B2B8DB:Domainname IP 76-->Convert to decimal 118, B2-->Convert to decimal 178, B8-->Convert to decimal 184, DB-->Convert to decimal 219 IP is: 118.178.184.219

33:checksum

Type=3 Domain nameModify the downlink domain name to:aiday.com.cn, port: 8825

BDBDBDBD:Header

C3:Message ID

03:type=3, domain name

7922:Port Port converts to big-endian--2279 --> converts to decimal Port: 8825

12:Len The length of the content after the parameter is converted to decimal 18 --> The message length is 18 bytes

61696461792e636f6d2e636e:Domainname domain name converted to string -->aiday.com.cn-->domain name is: aiday.com.cn

1D:checksum