Hong Kong-W200PG-4GCat.1-tcp communication protocol

Directory

1 Overview	4

2 Device Usage Instructions	4

2.1 Device Function and Usage Instructions	4

2.2 Device default reporting logic	6

2.3 Device downlink instructions	8

3 Protocol Data Packet Structure	13

3.1 Data Header	13

3.2 Message ID	13

3.3 Token generation mechanism	13

3.4 Payload	14

3.5 Checksum	15

4 Report messages message	16

4.1 Connection related reports	16

4.1.1 LNK-LIN (MSGID=0xF0) Request Connection (TCP Only)	16

4.1.2 Connection Reply (MSGID=0xF1) (TCP Only) - Important	17

4.1.3 New Heartbeat Packet Protocol (MSGID=0xF9)-Important	18

4.2 Alarm-related reporting	20

4.2.1 Alarm data upload-1(MSGID=0x02)	20

4.2.2 Alarm data upload-2(MSGID=0x21)(Supplement to 02)	24

4.2.3 Abnormal Data Alarm (MSGID=0x16)	27

4.3 Location-related reporting	29

4.3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)	29

4.3.2 Wi-Fi and base station information upload (MSGID=0xA4)	30

4.3.3 Bluetooth Location Information (LBE Location) (MsgId=0xD6)	36

4.3.4 Base station latitude and longitude reporting (MSGID=0x15) — wifi positioning supplement	39

4.4 Device Information and Status Reporting	40

4.4.1 ICCID Upload of SIM Card (MSGID=0xF3)	40

4.4.2 Device charging status upload (MSGID=0xC3)	41

4.4.3 Status parameter reporting (MSGID=0xA9)	43

4.4.4 Device Status (MSGID=0xE9)	45

4.5 Health-related reporting	46

4.5.1 Health Data (MSGID=0x32)	46

4.5.2 Device Sleep Analysis Data Upload (MSGID=0xC5)	48

4.6 Downlink feedback related reports	49

4.6.1 Downlink feedback (MSGID=0xC0)	49

4.6.2 Message Status Report (MSGID=0x28)	50

5 Settings	52

5.1 Downlink	52

5.1.1 Set the location reporting frequency (MSGID=0x17)	52

5.1.2 Message Send (MSGID=0X28)	54

5.1.3 Settings - Location priority settings (0XCE01&0XCE02)	55

5.1.4 Settings - Health Sample Reporting Frequency Settings (0XCE02)	58

5.1.5 Settings - Device Alarm Settings (0XCE03)	59

5.1.6 Settings - Switch Settings (0XCE04-24)	61

5.1.7 Settings - Drop Sensitivity Setting (0xCE15)	71

5.1.8 Settings - Vibration Settings (0XCE23)	73

5.1.9 Settings - Interface Display and Hide Settings (0xCE27)	74

5.1.10 IP&域名设置(0xC3)（TCP专用）	75

5.1.11 Issue sedentary stay alarm trigger time (MSGID=0XCC)	77

5.1.12 Shutdown restart (0x77)	77

5.1.13 Personal Information Distribution (0xCA)	78

5.1.14 Sleep statistics time period setting (MSGID = 0X1D)	80

5.1.15 Remote OTA upgrade distribution (0xA9)	83

5.1.16 Weather warning (0XCB)	84

1 Overview

This protocol is suitable for 4G CAT1 devices, currently supporting W200P products. It uses a 32-bit data header for synchronization and terminal identification; employs a low-overhead checksum algorithm for checksum protection; and uses message identifiers to identify different messages.
Parsing is required based on actual reporting. The document is currently being continuously maintained. If any errors or omissions are found, please provide timely feedback. Thank you very much.
The protocol content is for 4G reporting. If you need the Bluetooth broadcast data protocol, please consult the relevant personnel.

2 Device Usage Instructions

2.1 Device Function and Usage Instructions

General version:

(1) Power on:

Please fully charge the device before the first use. The device will automatically power on when the charging reaches the power required for startup. The charging status displays a charging icon.

A full charge is indicated by a green icon. Note: Do not check the device signal while charging.    Manual power on: Press and hold the up button for 10 seconds and release. The interface will display the word “Welcome.”    Note: The device defaults to the wearing state upon power-on, and a removal alarm is reported if no heart rate is detected.

(2) Power off:

Low battery shutdown: The screen turns off after the interface displays Byebye. Manual shutdown: Shutdown: Press and hold the button for more than 10 seconds, the interface displays Byebye and then the screen turns off. Downlink command shutdown: Downlink shutdown command in the power-on state, the interface displays Byebye and then the screen turns off. Note: Charging in the power-on state does not shut down

(3) SOS function:

Trigger mode: After triggering, the device does not enter sleep mode. Long press the down key for 3s, the interface displays SOS SEND OK / SOS sending successfully, and then the SOS icon appears. Cancel mode: In SOS mode, long press the down key for 3s, the interface displays SEND CENCEL / SOS canceled, and the SOS icon disappears

(4) Signal status:

No signal: The interface signal is a horizontal line. Signal available: The interface signal has a stepped bar graph

(5) Device sleep:

Trigger condition: The device does not move for 40 minutes and enters sleep mode, does not report location health data, and only reports F9 heartbeat packets to maintain a long connection

(6) Ultra-long standby mode:

Manual change: Switch settings - ultra-long standby mode, click enable, the device will switch to ultra-long standby mode Remote command issued: Issue a switch to ultra-long standby mode, the device will switch to ultra-long standby mode Effect: There is a yellow bar below the battery icon on the time interface, short connection, detects health and location every 10 minutes,

When the health data is normal and the location has not changed, the data is reported every 1 hour; otherwise, the data is reported every 10 minutes

(7) Device parameters: Settings - Parameters interface: There are multiple pages, which can be switched by sliding the screen down

sos (Yes): Yes: Indicates that the device can be triggered by pressing the sos button.

sos (No): Indicates that the sos cannot be triggered by pressing the button.

Health (10 min): Indicates that the device has enabled the health report function.

10min: Indicates that the current health data reporting frequency is 10 minutes.

Health (No): Indicates that the device health reporting function has been turned off.

Location (BWG, 2 min): Indicates that the device has enabled the location reporting function.

BWG: Indicates the location priority Bluetooth\>wifi>gps.

2min: Indicates that the current location reporting frequency is 2 minutes.

Location (No): Indicates that the device location reporting function has been turned off.

Fall (Medium, 1.5m): Indicates that the device has enabled the fall alarm trigger function.

Medium: Indicates that the device's fall alarm trigger sensitivity is medium.

1.5m: Indicates that the device's fall alarm trigger height is 1.5m.

Fall (No): Indicates that the device has disabled the fall alarm trigger function.

Sedentary (2min): Indicates that the device has enabled the sedentary stay alarm trigger function.

2min: Indicates that the time to trigger the sedentary stay alarm is 2 minutes of inactivity.

Sedentary (No): Indicates that the device has disabled the sedentary stay alarm trigger function.

Health Broadcast (Yes):

Yes: Indicates that the device has enabled Bluetooth broadcasting of health data.

No: Indicates that the device has disabled Bluetooth broadcasting of health data.

Sleep statistics reporting time: (21:00-08:00), indicating that the sleep statistics reporting time is from 9 PM to 8 AM the next day.

Global vibration duration (1.500s): The current device vibration duration is1.5seconds

Health alarm threshold: such as heart rate threshold (90,100)BPM, indicating that the normal heart rate range is90-100,

Health abnormality alarms will be reported outside the normal range. Heart rate threshold (No): indicates that the heart rate threshold is turned off.

Allow work hibernation (Yes):

Yes: Indicates that the device has turned on the hibernation function. No: Indicates that the device has turned off the hibernation function.

Note: The hibernation function means that the device does not report data when it is in the detached state, and the device does not report data when it is stationary for 40minutes.

Network long connection (Yes):

Yes: Indicates that the current device tcpconnection mode: long connection (always maintains the connection state, and can receive downlink instructions in real time),

No: Indicates short connection (disconnect after reporting, the device accepts downlink instructions when reporting data) GPSAlways on switch (No):

Yes: The internal interface is always open to collect GPS. In general environments, it can speed upgpspositioning time, and the power consumption will increase.

The default is No, and it does not need to be turned on in a normal environment.

(8) Vibration:

Receive the downlink text message, health threshold alarm (the interface has text reminders and vibration)

The vibration duration can be controlled by downlink (see section 2.3)

2.2 Device default reporting logic

General version

(1) Connection related reporting

F0 request: The device is a long link. Under normal server connection and network conditions, it will report once when it is turned on--A reply is required to connect to the server

F9: Heartbeat package reporting, default4minutes reporting once, (previous firmware) location and health reporting will also follow a report--A reply is required to maintain a long connection

(2) Location related reporting

gps/wifi/Bluetooth beacon: Default reporting frequency10minutes, default positioning priority: wifi>gps, wifi positioning is preferred, and gps positioning is switched if positioning fails

Base station (0x15): The latitude and longitude of the communication base station automatically requested from the module after the wifi positioning fails.

Low accuracy is used for auxiliary reference. This report is generally not available in normal environments, and location priority cannot be assigned and bound to Wi-Fi positioning.

(3) Alarm-related reports

SOSAlarm (0x02): User actively triggers, see the previous section for triggering method

SOSCancel (0x02): User actively triggers cancellation, see the previous section for triggering method

Shutdown alarm (0x21): Device actively shuts down or shuts down due to low battery, see the previous section for triggering method

Wearing and falling off alarm (0x02): The device determines wearing according to the health sampling report frequency. It reports wearing alarm when heart rate is measured, and reports falling off alarm when heart rate is not measured.

Sedentary stay alarm (0x02): Default15minutes of immobility triggers the report

Fall alarm (0x02): The device falls freely from a certain height and meets the fall algorithm trigger

Low battery alarm (0x02): Triggered when the current battery level of the device is less than or equal to0

Abnormal temperature alarm (0x16): Reported when the device detects a temperature exceeding40degrees Celsius

Health threshold alarm (0x2105): No alarm by default. After the downlink health threshold, if the health data is not within the threshold range, an alarm is reported.

Note: When the health threshold alarm is reported, the device vibrates and the interface displays abnormal values.

(4) Health-related reporting

Step count, heart rate, body temperature&wrist temperature, blood pressure, blood oxygen (0x32): Default reporting frequency10minutes

Sleep (0xC5): Statistics time period21:00-08:00, the sleep data will be reported according to the status during this time period

(5) Device information and status reporting

Status parameter (0xA9): A record will be reported when the device is turned on

SIM card ICCID (0xF3): A record will be reported when the device is turned on

Device status (0xE9): A record will be reported when the device is turned on, and a record will be reported when the reporting frequency is changed

Charging status (0xC3): Report when charging starts, charging ends, and when fully charged

(6) Downlink Feedback

Downlink feedback (0xC0): The server's downlink command is reported after the device receives it.

Message status feedback (0x28): After the downlink text message, the device clicks accept or reject.

Note: The device reports in a combined packet, that is, a data packet contains multiple complete messages. Pay attention not to miss any. The message is a complete message, and there will be no interruption in the middle in the next data packet.

Eg:BDBDBDBDD6000119A9CF610445270387BF452708A1BC44279D18B74427E518B7F9BDBDBDBDF9010000006400002800000019A9CF61CA

This data packet contains messages for (0xD6) Bluetooth positioning and (0xF9) power signal.

(0xD6) Bluetooth positioning: BDBDBDBDD6000119A9CF610445270387BF452708A1BC44279D18B74427E518B7F9

(0xF9) Power signal: BDBDBDBDF9010000006400002800000019A9CF61CA

2.3 Device downlink instructions

General version: Downlink commands need to be continuously downlinked twice within 20 milliseconds to ensure success.

(1) Device location reporting frequency distribution (0x17):

The default reporting frequency is 2 minutes, with a maximum of 1 minute and a minimum of 1440 minutes. After receiving the downlink command, the device will report data according to the time period and frequency of the issued command.

Report data. Outside the time period, report according to the default reporting frequency, such as: 00:00-18:00 2-minute positioning report,

Then report according to the default 10-minute reporting frequency outside the time period.

(2) Text message delivery (0x28):

Unicodeencoding, up to40Chinese characters, one Chinese character occupies2bytes, and one English letter occupies2bytes

Message list - the history message can save up to50items (Note: The firmware without message list can save up to20items)

(3) Device positioning priority delivery (0xCE01):

Default positioning priority wifi>gps, positioning priority

If the issued positioning priority is: wifi>gps>Bluetooth beacon, then if wifi cannot be located, switch to gps, and if gps cannot be located, switch to

Bluetooth beacon, when the positioning is successful, it will not switch to the next positioning priority to generate positioning

(4) Device health sampling frequency issuance (0xCE02):

The default health sampling frequency is 10 minutes, with a maximum reporting frequency of 2 minutes.

(5) Issue command to modify IP and port (0xC3):

The default general version points to the Smart Cloud Platform: 118.178.184.219:8825. If you need to change it, please consult relevant personnel or visit the official website.

(6) Issue command to set sedentary stay alarm trigger time (0xCC):

Value range: 2 minutes - 60 minutes. For example, if the downlink is 10 minutes trigger time, the device will trigger and report a sedentary stay alarm if it does not move for 10 minutes.

(7) Issue command to shut down and restart the device (0x77):

Note that this command must be effective when the device is turned on, and cannot be received after shutdown.

Shutdown command: When the device is turned on, issuing this command will shut down the device, and downlink cannot be received after shutdown.

Restart command: When the device is turned on, issuing this command will restart the device.

(8) Issue command to set drop sensitivity and height (0xCE15):

The latest general firmware defaults to a low (medium-low) drop sensitivity. Drop height: 1.5m

Sensitivity: Refers to the degree to which the drop algorithm is satisfied, providing 5 setting levels (0 - 4): Low - Lower (Medium-Low) - Medium - Higher (Medium-High) - High.

Height: Refers to the height that satisfies the trigger drop alarm, providing 5 setting levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m- 2.5m.

(9) Remote OTA distribution (0xA9):

Remote update consumes traffic and is affected by the network. The device's SIM card is 30M per month, so it cannot be updated too many times.

Otherwise, there will be insufficient traffic. This function requires the device hardware to support remote updates. Previous devices do not support it.

(10) Key shutdown switch command distribution (0xCE16):

By default, the device can be shut down via the button. After the downlink is turned off, the device cannot be shut down via the button.

(11) Control device to trigger sleep switch distribution (0xCE18):

The default is on. The device enters sleep mode after 40 minutes of inactivity. After the downlink is turned off, the device does not enter sleep mode.

(12) Fall Alarm Switch (0xCE07):

The fall alarm is enabled by default. After it is turned off, the device will not report the fall alarm.

(13) Key triggers SOS start switch (0xCE19)

It is enabled by default. Long press the charging cable button to trigger SOS. After it is turned off, long press the charging cable button will not trigger the report of SOS.

(14) Device health and location reporting switch (0xCE20)

The default health report is enabled, and the default location report is enabled.

You can set the location / health on and off states separately. After it is turned off, the device will not report health / location.

(15) Long connection and short connection mode switching (0xCE22)

The default is long connection. You can switch between long and short connection commands downstream. The device will restart after receiving it and become long/short connection. Note that the long connection mode consumes slightly more power than the short connection. By default, a heartbeat packet is reported every 4 minutes (0xF9).

(16) Health threshold setting (0xCE0300)

The health threshold is issued. It is a downstream command. After the device receives it, the normal range of the health threshold is issued. Health data outside this range will report an alarm.

(17) Device Vibration Setting (0xCE23)

After the downlink command is received by the device, the device vibration duration changes to the downlink duration, ranging from 0-60 seconds.

(18) Report Location Immediately (0xCE0A)

After the downlink command is received by the device, the device immediately starts positioning and reporting.

(19) Report Health Immediately (0xCE0B)

After the downlink command is received by the device, the device immediately starts health sampling and reporting.

(20) Sleep Statistics Time Period Distribution (0x1D)

Different time periods can be distributed, such as: 13:00-14:00, then sleep statistics data will be collected and reported within this time period.

(21) GPS Always On Switch (0xCE24):

After it is turned on, the internal interface always collects GPS. In general environments, it can speed up GPS positioning time, but the power consumption will increase. The default is off.

(22) Switch to ultra-long standby mode (0xCE17):

After it is turned on, the device switches to ultra-long standby mode, and there is a yellow bar under the battery icon on the time interface, short connection.

Health and location are checked every 10 minutes. If the health data is normal and the location has not changed, the data is reported every 1 hour; otherwise, it is reported every 10 minutes.

(23) Watch health related interface hidden and displayed (0xCE27):

The default watch controllable interfaces are: heart rate, blood pressure, body temperature, blood oxygen, step counting, exercise,HRV(This function and interface are not available in the general version)

By default, all interfaces are displayed. After the downlink hides the interface command, the watch interface does not display the interface.

(24) Personal information issued (0xCA)

There is no personal information by default. Chinese name, English name, blood type, contact number can be issued,Unicodeencoding,

Maximum number of bytes: Name40bytes, English name40bytes, Phone40bytes, Blood type10bytes

(25) Sedentary stay alarm switch (0xCE08)

It is enabled by default, and the trigger time is 15 minutes. After the downlink is turned off, the device does not report a sedentary stay alarm.

(26) Weather Alert Issued (0xCB)

Multiple sets of weather alerts can be issued. Note that each set of weather alerts must be different and cannot have the same type of alert.

3 Protocol Data Packet Structure

A basic protocol data packet structure is shown in Figure 1:

3.1 Data Header

Each data packet starts with a 4-byte Header or token (timestamp replaces it in some response messages):

Currently, the token uploaded by OuFu devices is fixed as BDBDBDBD

Header: 0xBD 0xBD0xBD0xBD；

Timestamp: 32bits, generated by the server

3.2 Message ID

See Chapter 4 for the content represented by MessgeId.

Each time a TCP connection is made, the device will first report a 0xF0 message, which contains the device's unique identifier IMEI.

The server needs to record this IMEI as an identifier and reply with a 0xf1 message. Only then will the device consider the connection successful; otherwise, it will disconnect.

For normal device communication, the long connection reports a 0xF9 heartbeat packet every 4 minutes by default. The 0xF9 heartbeat packet is reported once after the health location is reported.

The short connection reports a 0xF9 heartbeat packet once at startup, and a 0xF9 heartbeat packet is reported once after the health location is reported. The two reports are not affected by each other.

Note that after the server receives the F9 message, the server needs to issue a command to reply; otherwise, the connection will be disconnected.

3.3 Token generation mechanism

Currently fixed as BDBDBDBD

3.4 Payload

The payload below refers to the valid body content in the protocol, excluding the head token and checksum. The length of the body is noted after.

The data format used in the payload is shown in the following table:

【U-unsigned; I-signed; X-bitfield; Number-bytes occupied】

The following protocols use little-endian except for ch, u8, i8, and x8

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Explanation |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short Integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short Integer |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Integer |
| x16 | Bitfield | 2 | - | - | Bit 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long Integer |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long Integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit long integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating point type |

3.5 Checksum

The checksummed content includes the payload, as shown in Figure 1. The algorithm is as follows. By default, checksum is not required for the general version of the device, and this part can be ignored. Any byte can be used for the downlink instruction.

//Assume content="BDBDBDBDE9000600010A00000A00", then the sum.ToString("X2") result is 07//The following is the C# code calling method

private string CheckSum(string content)

{

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2");

}

4 Report messages message

4.1 Connection related reports

4.1.1 LNK-LIN (MSGID=0xF0) Request Connection (TCP Only)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xF0 | See below见下方定义 | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | u64 | IMEI | 1 | - | IMEI number (little endian) |
| 2 | x16 | version |  | - | Bitfield see below |

The device's reported F0 request message must have a server downlink 0XF1 message (see the next section for the specific format) to reply, otherwise the login will fail.

Example:

Original16hexadecimal message: BDBDBDBDF09B51731BC61603000000C2

BDBDBDBD:4byte message headerHeader

F0:Message ID(Message ID)9B51731BC6160300: IMEI number(little-endian), converted to big-endian:000316C61B73519B,

The corresponding decimal is IMEI:869465050010011,0000: version

C2: checksum

Connection related Q&A:

Each time a new connection is created via TCP, an F0 request is reported first. The F0 contains the IMEI, and then the server records this IMEI.

After that, all the data in this connection belongs to this IMEI.

Note: If F0 is not received or F0 communication is abnormal, you can use a third-party network testing tool to verify whether the server communication is normal.

4.1.2 Connection Reply (MSGID=0xF1) (TCP Only) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Timestamp (unix) timestamp | 0xF1 | See below for definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | u32 | Token | 1 | - | Token: Fixed as BDBDBDBD |

Note that the message sequence is different from the normal reporting message. This message is issued by the server, not the content reported by the device. The server reply needs to be replied in the current channel > Note that the message sequence is different from the normal reporting message. This message is issued by the server, not the content reported by the device. The server reply needs to be replied in the current channel

Example:

Original16Hexadecimal message:07FD8860F1BDBDBDBD2E

07FD8860(Little endian),Timestamp(unix) timestamp (unit: seconds), converted to big endian6088FD07 Converted to decimal: 1619590407

Convert to standard time formatUTCTime:2021-04-28 06:13:27

F1:Message ID(Message ID)

BDBDBDBD:Token

2E:checksumChecksum

TimestampDecimal 10conversion to standard time format is the result of adding1619590407 seconds to 1970-01-0100:00:00. The device uses the timestamp (in seconds) in this message to synchronize time.

Note: Injava, the unit is generally milliseconds. Be careful to convert to the unit: seconds.

Note: The total number of bytes replied by the server is10bytes, such as:0xBD--1byte; if the device receives20bytes, the connection will not be established.

4.1.3 New Heartbeat Packet Protocol (MSGID=0xF9)-Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xF9 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery type 0: 4-level system, 1: 5-level system, 2: percentage, 3: voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery value If Bat_type is 0: the battery value range is 0-3 (0 is 25%, 3 is 100%) If Bat_type is 1: the battery value range is 0-4 (0 is 20%, 4 is 100%) If Bat_type is 2: the battery value range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: percentage, 1: 5-level system 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal strength |
| 1 | U8 | Other_type |  |  | Extension Type 0: Full step counting 1: Incremental step counting 2: Vibration |
| 4 | U32 | Num |  |  | Extension Value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Note: The F9 heartbeat packet must have a server downlink reply. The device considers the server connection to be unbroken only after receiving the server downlink reply; otherwise, it will re-report the F0 request to connect.

Example:

Original16hexadecimal message: BDBDBDBDF90104000050000095000000E377BD67AA

BDBDBDBD: Header

F9: Message ID

01:Bat_type Battery Type 01-->1-->5-level battery (0-4)

0400:Bat_volt battery level value converted to big-endian 0004-->4-->Level4corresponds to100% battery level

00:Signal_type signal type 00-->0-->Percentage

5000:Signal_strength signal strength converted to big-endian0050-->converted to decimal80-->80% signal strength

00:Other_type extension type 00-->0-->Full step counting

95000000:Num extension value - step counting converted to big-endian00000095-->converted to decimal149-->number of steps is149

E377BD67:Timestamp timestamp converted to big-endian67BD77E3-->converted to10decimal1740470243-->timestamp is1740470243seconds--> converted to standard time format UTC time:2025-02-25 07:57:23-->converted to Beijing time:2025-02-25 15:57:23

AA:checksum

Note: The server receives the F9 heartbeat packet and can reply with a fixed message: Fixed reply example BDBDBDBDF301 Number of bytes:6

4.2 Alarm-related reporting

4.2.1 Alarm data upload-1(MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x02 | See below for definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below for definition |
| 4 | U32 | Timestamp | - | - | Timestamp (a timestamp will be added at the end during retransmission) |

Upl_warn contents

| bit (corresponding to the position of 1 in binary) | Name | Description | Original message little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 14 | Fall Alarm | Fall Alarm | 0040 | 4000 | 4*4096=16384 |
| 8 | Device wearing | Device wearing | 0001 | 0100 | 1*256=256 |
| 7 | SOS cancellation | SOS cancellation | 8000 | 0080 | 8*16=128 |
| 5 | Sedentary alarm (no movement) | Sedentary alarm (no movement) | 2000 | 0020 | 2*16=32 |
| 4 | Take off (drop) the device | Take off (drop) the device | 1000 | 0010 | 1*16=16 |
| 2 | Shutdown | Shutdown | 0400 | 0004 | 4 |
| 1 | SOS Alarm | SOS Alarm | 0200 | 0002 | 2 |
| 0 | Low battery | Low battery | 0100 | 0001 | 1 |

Example:

Wearing alarm: BDBDBDBD020001E377BD678A

BDBDBDBD:Header

02:Message ID

0001:Upl_warn alarm content, converted to big-endian:0100-->Wearing alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

Fall off alarm: BDBDBDBD021000E377BD678A

BDBDBDBD:Header

02:Message ID

1000:Upl_warn alarm content, converted to big-endian0010--> detachment alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

SOS Alarm: BDBDBDBD020200E377BD678A

BDBDBDBD:Header

02:Message ID

0200:Upl_warn alarm content, converted to big-endian0002-->SOS alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

SOS Cancellation: BDBDBDBD028000E377BD678A

BDBDBDBD:Header

02:Message ID

8000:Upl_warn alarm content, converted to big-endian0080-->SOS cancellation

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

Fall Alarm: BDBDBDBD020040E377BD678A

BDBDBDBD:Header

02:Message ID

0040:Upl_warn alarm content, converted to big-endian4000-->Fall alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

E7: checksum

Sedentary Alarm: BDBDBDBD022000E377BD678A

BDBDBDBD:Header

02:Message ID

2000:Upl_warn alarm content, converted to big-endian0020-->Sedentary alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8A: checksum

Low battery alarm: BDBDBDBD020200E377BD678A

BDBDBDBD:Header

02:Message ID

0100:Upl_warn alarm content, converted to big-endian0001-->Low battery alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8A: checksum

Shutdown alarm: BDBDBDBD020400E377BD678A

BDBDBDBD:Header

02:Message ID

0400:Upl_warn alarm content, converted to big-endian0004-->Shutdown alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8A: checksum

Note: This alarm is used for devices that do not distinguish between shutdown types. See the next section for devices that distinguish between shutdown types.

4.2.2 Alarm data upload-2(MSGID=0x21)(Supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x21 | See below for definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | Alarm type |
| N |  | Upl_warn | - | - | Alarm content See below for Alarm type definitions |
| 4 | U32 | Timestamp |  |  | Timestamp (a timestamp will be added at the end during retransmission) |

Alarm type=1

Upl_warn contents

| bit (corresponding to the position of 1 in binary) | Name | Description | Original message little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging shutdown alarm | Device automatically shuts down when charging (firmware requires function support) | 04000000 | 0004 | 4 |
| 1 | Low battery shutdown alarm | Device shuts down due to low battery (firmware requires function support) | 02000000 | 0002 | 2 |
| 0 | Active shutdown alarm | Device is manually shut down by someone (firmware requires function support) | 01000000 | 0001 | 1 |

Example:

Charging shutdown alarm: BDBDBDBD21010004000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

04000000:Upl_warn alarm content, converted to big-endian 0004-->charging shutdown alarm

E377BD67:Timestamp converts the timestamp to big-endian 67BD77E3-->converts to decimal 1740470243-->the timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Low battery shutdown alarm: BDBDBDBD21010002000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

02000000:Upl_warn alarm content, converted to big-endian 0002-->Low battery shutdown alarm

E377BD67:Timestamp converts the timestamp to big-endian 67BD77E3-->converts to decimal 1740470243-->the timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Active shutdown alarm: BDBDBDBD21010001000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type is converted to big-endian 0001-->Alarm type=1

01000000:Upl_warn alarm content, converted to big-endian 0001-->Active shutdown alarm

E377BD67:Timestamp is converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Alarm type=5

Upl_warn contents

| Format | Name | Unit | Decription |
| --- | --- | --- | --- |
| U16 | Type | 2 | Fixed as 0x05 |
| U32 | Timestamp | 4 | Timestamp |
| U16 | Content Len | 2 | Alarm content length |
| U8 | Health Warn Type1 | 1 | Health threshold alarm type: 0x01 heart rate, 0x02 systolic blood pressure (SBP), 0x03 diastolic blood pressure (DBP), 0x04 blood oxygen, 0x05 temperature |
| U8 | Type1 | 1 | 01 less than the set health threshold, 02 greater than the set health threshold |
| I16 | Health Value1 | 2 | Health result value |
| U8 | Type2 | 1 | 01 less than the set health threshold, 02 greater than the set health threshold |
| I16 | Health Value2 | 2 | Health result value |
| 。。。。 | 。。。。 | 。。。。 | 。。。。 |

Example:

The device detected that the heart rate (65) is outside the range of the health threshold issued: BDBDBDBD210500E377BD6704000102410073

BDBDBDBD：Header

21:Message ID

0500:Alarm type Convert to big-endian 0005-->Alarm type=5

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

0400:Content Len Alarm content length Convert to big-endian0004-->The alarm content length is 4 bytes. Note: The length will increase if there are multiple health threshold alarms.

01: Health Warn Type Health Threshold Alarm Type 01--> Heart Rate Alarm

02:Type Greater than the set health threshold & Less than the set health threshold 01-->Less than the set health threshold

4100:Health Value Actually reported health value, convert to big-endian0041-->Convert to decimal 65-->Reported heart rate value is 65

73:checksum

4.2.3 Abnormal Data Alarm (MSGID=0x16)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x16 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type | -/- | -/- | Alarm type (0: Abnormal heart rate value alarm, 1: Abnormal temperature value alarm) |
| 2 | U16 | Heart | -/- | -/- | Heart rate value |
| 2 | U16 | temperature | -/- | -/- | Temperature |
| 2 | U16 | Pa | -/- | -/- | Air pressure |
| 4 | 4*U8 or U32 | expand | -/- | -/- | 4 bytes expansion for other sensors |

Note: The reported outlier has no practical reference significance, it only indicates that the current detected data is abnormal (such as the value exceeding the range or the sensor detection is abnormal, etc.). The outlier will be removed in other normal messages, and it is reflected in this message, which can be ignored.

Example:

Temperature anomaly value alarm: BDBDBDBD16010000FF0F000000000000E6

BDBDBDBD:Header

16:Message ID

01:Type alarm type 01-->Temperature anomaly value alarm

0000:Heart rate value. If this value is not available, the default is 0000.

FF0F:temperature temperature Convert big-endian 0FFF-->Temperature anomaly value FFF

0000:Pa air pressure No such value defaults to 0000

00000000 expand expansion, no such value defaults to 0000

E6:checksum

4.3 Location-related reporting

4.3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x16 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude longitude |
| 8 | Double | lat |  | - | latitude latitude |
| 1 | U8 | north_south |  | - | /N or S/ North or South latitude |
| 1 | U8 | east_west |  | - | /E or W/ East or West longitude |
| 1 | U8 | status |  | - | /A or V/ Valid data or invalid data |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original16hexadecimal message: BDBDBDBD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BDBDBDBD:Header

03:Message ID

963E870C3E675E40:lon longitude Convert to big-endian405E673E0C873E96-->Longitude:121.6131622 Note: See the parsing example below for conversion parsing

FF2110C2B6343F40:lat latitude Convert to big-endian`403F34B6C21021FF-->Latitude:31.2059137 Note: See the parsing example below for conversion parsing

4E:north_south 4E-->Convert to ASCII code N-->North latitude

45:east_west 45-->Convert to ASCII code E-->East longitude

41:status data validity 41-->Convert to ASCII code A-->Valid data

E377BD67:Timestamp timestamp Convert to big-endian67BD77E3-->Convert to base101740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

78:checksum

Note: The parsed GPS latitude and longitude are in the WGS-84coordinate system. If the map uses Baidu AMAP, etc., the coordinate system needs to be converted.

Parsing satellite positioning message Java code example:

//报文：BDBDBDBD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BDBDBDBD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public static void main(String[] args){

//Message 7d9f84ac81815c40 Actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d",16)));  //114.02353966666665

//Message e766926b1d893640 Actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16))) ;//22.535605166666667

HexToStr(data.Substring(“4e”));   //N

HexToStr(data.Substring(“45”));//E

HexToStr(data.Substring(“41”));//A   A indicates valid data, V indicates invalid data, which can be discarded//Message 749d695f Actual value 5f699d74

Date date=new Date();

date.setTime(Long.parseLong(“5f699d74",16)*1000);

SimpleDateFormatsdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.3.2 Wi-Fi and base station information upload (MSGID=0xA4)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xA4 | See below for definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 1 | u8 | Cell_cnt | 1 | - | Number of base stations (0-7) Number of cell info payload.Valid value:0~7 |
| 2 | u16 | Cell[0].MCC | - | - | MMC of base station (0) mobile country code of cell[0] |
| 2 | u16 | Cell[0].MNC | - | - | MNC of base station (0) mobile network code of cell[0] |
| 2 | u16 | Cell[0].LAC | - | - | LAC of base station (0) Location area code of cell[0] |
| 4 | U32 | Cell[0].CELL_ID | - | - | ID of base station (0) Cell id of cell[0] |
| 2 | i16 | Cell[0].RSSI | - | dbm | Signal strength RSSI of base station (0) RSSI in dbm of cell[0] |
| ... | ... | ... | ... | ... | ... |
| 2 | u16 | Cell[cell_cnt-1.MCC | - | - | MMC of base station (N) mobile country code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.MNC | - | - | MNC of base station (N) mobile network code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.LAC | - | - | LAC of base station (N) Location area code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1].LAC | - | - | LAC of base station (N) Location area code of cell[[cell_cnt-1] |
| 4 | U32 | Cell[cell_cnt-1].CELL_ID | - | - | ID of base station (N) Cell id of cell[[cell_cnt-1] |
| 2 | I16 | Cell[cell_cnt-1].RSSI | - | dbm | Signal strength RSSI of base station (N) RSSI in dbm of cell[[cell_cnt-1] |
| 1 | U8 | Wifi_cnt |  |  | Number of wifi Number 0f wifi |
| 6 | 6*U8 | Wifi[0].bssid[0]-Wifi[0].bssid[5] |  |  | MAC address of wifi(0) |
| 4 | I32 | Wifi[0].RSSI |  |  | Signal strength RSSI of wifi(0) |
| ... | ... | ... | ... | ... | ... |
| 6 | 6*U8 | Wifi[Wifi_cnt-1].bssid[0]-Wifi[Wifi_cnt-1].bssid[5] |  |  | MAC address of wifi(N) |
| 4 | I32 | Wifi[0].RSSI |  |  | Signal strength RSSI of wifi(0) |

Note: The device may not report base stations every time. The absence of base stations does not affect positioning. If the number of base stations in the report is 0, then the following is directly the number of Wi-Fi networks.

Example:

Original16Hexadecimal message: BDBDBDBDA4E377BD6701CC010000C218D1AD160BF9FF078CBEBE1A8162C6FFFFFFC061180AF42AC1FFFFFF200BC726E000B6FFFFFFA8154DF6517EB2FFFFFFE005C5B1F824CCFFFFFFE8FCAFA02663AFFFFFFF6409805B2B9CAEFFFFFF94

BDBDBDBD:Header

A4: Message ID message id

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

01: Cell_cnt Number of base stations 01 --> The number of base stations is 1

CC01: Cell[0].MCC Base station MCC converted to big-endian 01CC --> Convert to decimal 460 --> Base station MCC is 460

0000: Cell[0].MNC Base station MNC converted to big-endian 0000 --> Convert to decimal 0 --> Base station MNC is 0

C218: Cell[0].LAC Base station LAC converted to big-endian 18C2 --> Convert to decimal 6338 --> Base station LAC is 6338

D1AD160B: Cell[0].CELL_ID Base station ID converted to big-endian 0B16ADD1 --> Convert to decimal 186035665 --> Base station ID is 186035665

F9FF: Cell[0].RSSI Base station signal strength converted to big-endian FFF9 --> Convert to decimal complement -7 --> Base station signal strength is -7

07: Wifi_cnt Number of wifi 07 --> 7 wifi

8CBEBE1A8162: wifi mac address 8C:BE:BE:1A:81:62 --> wifi[0]'s mac address is 8C:BE:BE:1A:81:62

C6FFFFFF: wifi signal strength RSSI converted to big-endian FFFFFFC6 --> Convert to decimal complement -58 ---> wifi[0]'s RSSI signal strength is -58

C061180AF42A: wifi mac address C0:61:18:0A:F4:2A --> wifi[1]'s mac address is C0:61:18:0A:F4:2A

C1FFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFC1 --> converted to decimal complement -63 ---> WiFi[0] RSSI signal strength is -63

200 BC726E000: WiFi MAC address 20: 0:C7:26:E0:00 --> WiFi[2] MAC address is 20: 0:C7:26:E0:00

B6FFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFB6 --> converted to decimal complement -74 ---> WiFi[0] RSSI signal strength is -74

A8154DF6517E: WiFi MAC address A8:15:4D:F6:51:7E --> WiFi[3] MAC address is A8:15:4D:F6:51:7E

B2FFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFB2 --> converted to decimal complement -78 ---> WiFi[0] RSSI signal strength is -78

E005C5B1F824: WiFi MAC address E0:05:C5:B1:F8:24 --> WiFi[4] MAC address is E0:05:C5:B1:F8:24

CCFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFCC --> converted to decimal complement -52 ---> WiFi[0] RSSI signal strength is -52

E8FCAFA02663: WiFi MAC address E8:FC:AF:A0:26:63 --> WiFi[5] MAC address is E8:FC:AF:A0:26:63

AFFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFA --> converted to decimal complement -81 ----> WiFi[0] RSSI signal strength is -81

6409805 B2B9C: WiFi MAC address 64:09:80:5B:2B:9C --> WiFi[6] MAC address is 64:09:80:5B:2B:9C

AEFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFFFAE --> converted to decimal complement code - 82 ---> WiFi[0] RSSI signal strength is - 82

94:checksum

The parameters parsed from the message need to be connected to the map wifi database, and the latitude and longitude data is finally obtained through the map party. Taking AMAP connected to the wiif database as an example:

According to the access network method, AMAP wifi positioning protocol document is divided into three positioning scenarios. The following gives corresponding examples for the three positioning scenarios.

1. The network connection method is wifi access, that is, the accesstype parameter is 1

http://apilocate.amap.com/position?accesstype=1&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&mmac=50:a7:2b:1e:56:58,-60,alibaba-inc&macs=4c:48:da:25:0b:11,-59,alibaba-

inc|4c:48:da:25:1a:11,-77,alibaba-inc&serverip=10.2.166.4&output=xml&key=<用户 Key>

2. The network connection method is mobile access and the mobile phone card is a non-cdma card, that is, when the accesstype parameter is 0 and cdma=0:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=0&bts=460,01,40977,2205409,-65&nearbts=460,01,40977,2205409,-65|460,0

1,40977,2205409,-65|460,01,40977,2205409,-65&serverip=10.2.166.4&output=xml&key=<User Key>

3. When the network type is mobile access and the mobile card is a CDMA card, that is, when the accesstype parameter is 0 and cdma=1:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=1&bts=13824,1,1838,1674723,575739,-52&serverip=10.2.166.4&output=xml&key=<User Key>

W200P watch: AMAP wifi positioning protocol calls AMAP interface description: When selecting parameters, generally select the first type

You can also select the second type, but if the device does not report the base station, the bts and neabts parameters need to be removed before you can

The first type, an example of incoming parameters: mmac is the first WIFI mac address in the messagehttp://apilocate.amap.com/position?accesstype=1&imei=869907057780989&mmac=04:8C:16:A9:FB:A8,-44,TP-Link&macs=04:8C:16:A9:FB:A8,-44,TP-Link|C8:3A:35:F4:CE:18,-59,TP-Link|1C:78:4E:D5:07:FE,-79,TP-Link|C8:3A:35:9D:0B:E0,-87,TP-Link|4A:D8:90:E8:0A:82,-88,TP-Link|AC:AD:4B:C9:C3:87,-89,TP-Link|A4:16:E7:55:84:AC,-91,TP-Link&output=json&key=xxxx

For the specific AMAP wifi positioning protocol PDF, you can click to view and download:(not a screenshot, you can click to view)
This document is provided by AMAP, and you can directly ask AMAP for specific docking
Note that the name of the downloaded document will change, you can rename it yourself

4.3.3 Bluetooth Location Information (LBE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xD6 | See below for definition | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 0 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple sets of information, and each group may contain multiple ibeacons |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (One of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (One of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| ... | ... | ... | ... |

Note: In the general version, the total number of Bluetooth beacon groups is fixed to 1, and the maximum number of Bluetooth beacons is 4. The sorting is based on the signal strength from largest to smallest. By default, the watch only recognizes our own beacons. If you need to connect to other beacons, please consult relevant personnel via WeChat.
Example:

Original hexadecimal message: BDBDBDBDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

bdbdbdbd: Header

D6: Message ID

00: type

01: Only one set of ibeacon data

E377BD67:Timestamp converts to big-endian 67BD77E3-->converts to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

04: The first group has 4 beacon information

4327: Convert to big-endian-->major : 0x2743-->convert to decimal 10051

1794: Convert to big-endian-->minor: 0x9417-->convert to decimal 37911

AC:  RSSI 0xAC-->convert to decimal complement -84

4327: Convert to big-endian-->major: 0x2743-->convert to decimal 10051

3094: Convert to big-endian-->minor:0x9430-->convert to decimal 37936

AA:  RSSI 0xAA-->convert to decimal complement -86

4327:Convert to big endian -->major: 0x2743-->Convert to decimal 10051

b956:Convert to big endian -->minor:0x56b9-->Convert to decimal 22201

A5:   RSSI 0xA5-->Convert to decimal complement -91

4327: Convert to big-endian-->major: 0x2743-->convert to decimal 10051

FE94: Convert to big endian -->minor:0x94FE-->Convert to decimal 38142

A5: rssi:0xA5-->Convert to decimal complement -91

6a --Checksum

Bluetooth beacon major, minor description: The default shipping Bluetooth beacon has a beacon code attached.

For example: 00081005345866, then the major of the Bluetooth beacon is 10053, the minor is 45866, and the manufacturer ID is 0008 (this is useless and has no practical significance).

4.3.4 Base station latitude and longitude reporting (MSGID=0x15) — wifi positioning supplement

Note: This message is generally when wifi positioning fails, and the module automatically requests the latitude and longitude of the communication base station for positioning (the positioning priority cannot be changed).
The location is the location of the communication base station, which can only be used as a reference for auxiliary positioning. The accuracy of the communication base station is not high.

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x15 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | flag |  | -/- | Extended segment identifier |
| 4 | U32 | lon |  | -/- | Longitude (*10 to the power of 6 or 7) longitude |
| 4 | U32 | lat |  | -/- | latitude (*10 to the power of 6 or 7) latitude |
| 1 | U8 | north_south |  | -/- | /N or S/ South or North latitude |
| 1 | U8 | east_west |  | -/- | /E or W/ East or West longitude |
| 1 | U8 | status |  | -/- | /A or B or V/ A, B are valid data, B corresponds to 7-bit precision, V is invalid data |
| 4 | U32 | Timestamp |  | -/- | Timestamp |
| N |  | Num |  | -/- | The universal version has no extensions for the time being. The table only retains the definition. Definition: Extended segment value 1 can be the nth value defined by the extended segment. When multiple extended segments are used at the same time, extended segment 1 corresponds to the lowest bit content, and the content is extended to the nth digit in sequence. |
| ... | ... | ... | ... | ... | ... |

Example:

Original16Hexadecimal message: BDBDBDBD1502C8C9E53F855AD30F4E4542E377BD67F4

BDBDBDBD:Header

15:Message ID

02:type  type=02Indicates LBS base station positioning

C8C9E53F: Longitude--->Convert to big-endian4857963E--->Convert to longitude:26.5509509, Note: Refer to4.3.1GPS/ BDS Location Reporting: Location Data Reporting (0x03) section for parsing example

855AD30F: Latitude--->Convert to big-endian129B82E0--->Convert to latitude:107.2024008, Note: Refer to4.3.1GPS/ BDS Location Reporting: Location Data Reporting (0x03) section for parsing example

4E:north_south 4E-->Convert to ASCII code N-->North latitude

45:east_west 45-->Convert to ASCII code E-->East longitude

42:status 42-->Convert to ASCII code B, valid data, the precision is 7 decimal places

E377BD67:Timestamp Convert to big-endian67BD77E3-->Convert to decimal101740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

08:checksum

4.4 Device Information and Status Reporting

4.4.1 ICCID Upload of SIM Card (MSGID=0xF3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xF3 | See below for definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 10 | 10*U8 | ICCID | 1 | - | ICCID number - Device SIM card number ICCID |

Note: This message will be reported once every time the device is turned on.

Example:

Original hexadecimal message:BDBDBDBDF389861118236001639994CC

BDBDBDBD:Header

F3:Message ID

89861118236001639994:ICCID The device SIM card number ICCID is: 89861118236001639994

CC:checksum

4.4.2 Device charging status upload (MSGID=0xC3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Status | / | / | 0 starts, 1 ends, 2 fully charged |
| 1 | U32 | Timestamp | / | / | Timestamp (a timestamp will be added at the end during retransmission) |

Note: This message requires device firmware support, and the latest general version is supported by default.
Example:

Original hexadecimal message:BDBDBDBDC301E377BD678A

BDBDBDBD:Header

C3:Message ID

01:Device finished charging

E377BD67:Timestamp converts to big-endian 67BD77E3-->converts to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

Original hexadecimal message:BDBDBDBDC300DB4D2F668A

BDBDBDBD:Header

C3:Message ID

00:Device started charging

E377BD67:Timestamp converts to big-endian 67BD77E3-->converts to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

Original hexadecimal message:BDBDBDBDC302DB4D2F668A

BDBDBDBD:Header

C3:Message ID

02:The device is fully charged

E377BD67:Timestamp converts to big-endian 67BD77E3-->converts to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

4.4.3 Status parameter reporting (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Number of parameter types |
| 1 |  |  |  |  | (Reserved 00) |
| 1 | u8 | Type | -/- | -/- | Parameter type 1, type definition: 00—MCU (firmware), 01—module, 02 sensor (general version does not have this type) |
| 1 | U8 | NameLen | -/- | -/- | Parameter name 1 length |
| n | N*u8 | Name | -/- | -/- | Parameter Name |
| 1 | u8 | Type | -/- | -/- | Parameter Type 2, Type Definition: 00—MCU (Firmware), 01—Module, 02 Sensor (General version does not have this type) |
| 1 | U8 | NameLen | -/- | -/- | Parameter Type 2 Length |
| n | N*u8 | Name | -/- | -/- | Parameter Name |
| ... | ... | ... | ... | ... | ... |

Note: This message is reported once by default at startup, uploading the device firmware version number, module version number, which can be left unparsed. Previously shipped devices had a Message ID of 0xBB, which can be sent by contacting relevant personnel.

Example:

BDBDBDBDA902000009423732352E4F56303601174E5432364B434E4230304E4E412D4C3032303330393530B5

BDBDBDBD:Header

A9:Message ID

02:TypeCnt indicates that there are 2 types of parameters

00:Reserved field

00:Type=00  00 represents MCU (Firmware)

09:MCU name length is 9 bytes

423732352E4F563036--->Hexadecimal to text (device firmware version number)--->B725.OV06

01:Type=01 01 represents the module

17:The length of the communication module name is 23 bytes

4E5432364B434E4230304E4E412D4C3032303330393530:Hexadecimal to text (4G module version number)

--->NT26KCNB00NNA-L02030950

B5---checksum

4.4.4 Device Status (MSGID=0xE9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xE9 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | type |  | / | Default—00 |
| 2 | u16 | length |  | / | Length of the following message |
| 1 | u8 | location |  | / | Location reporting frequency: 00 - Default, not modified, 01 - Modified by downlink (if there are multiple time periods in the downlink, only report the reporting frequency of the current time period) |
| 2 | u16 | frequency |  | / | Frequency, unit - minutes |
| 1 | u8 | health |  | / | Health reporting frequency: 00 - Default, not modified, 01 - Modified by downlink |
| 2 | u16 | frequency |  | / | Frequency: unit - minutes |

Note: This message is reported once at startup, and once after the downlink modifies the frequency

Example:

Original hexadecimal message:BDBDBDBDE9000600010A00010A0033

BDBDBDBD:Header

E9:Message ID

00:type=00

0600:The message length after length is converted to big-endian --> 0006 --> converted to decimal 6, and the message length is 6 bytes.

01:location 01 indicates that the downlink has modified the location reporting frequency

0A00:frequency The health reporting frequency is converted to big-endian --> 000A --> converted to decimal 10, and the reporting frequency is 10 minutes.

01:health 01 indicates that the downlink has modified the health reporting frequency

0A00:frequency The health reporting frequency is converted to big-endian --> 000A --> converted to decimal 10, and the reporting frequency is 10 minutes.

33:checksum

If the device has no health reporting function or no location reporting function, then the health reporting frequency and

the location reporting frequency will remain the same, indicating the device reporting frequency

4.5 Health-related reporting

4.5.1 Health Data (MSGID=0x32)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x32 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | 00 |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 2 | U16 | contentLength |  |  | Total length of the following content |
| 1 | U8 | ID |  |  | Id (Type + Reported Value Length), see below for ID type definition |
| N | N*U8 | Val1 |  |  | Reported value of Id (byte length varies depending on the reported value length) |
| ... | ... | ... | ... | ... | ... |
| 1 | U8 | ID |  |  | Id (Type + Reported Value Length), see below for ID type definition |
| N | N*U8 | Val1 |  |  | Reported value of Id (byte length varies depending on the reported value length) |

ID type definition (hexadecimal):

01 Step Count02 Heart Rate 03 Body Temperature 04 Wrist Temperature 06 Diastolic Pressure 07 Systolic Pressure 08 Blood Oxygen 09 HRV(Special version support)

(05 Blood glucose 10 Number of sit-ups11 Running pace and distance 12 Speed and number of jump ropes )---Reserved, none for now

Default general version support:01 Step counting02 Heart rate 03 Body temperature 04 Wrist temperature 06 Diastolic pressure 07 Systolic pressure 08 Blood oxygen

Example:

Original16Hexadecimal message: BDBDBDBD3200E377BD6711000A1E00114B314A39711A4A0122bc00416212

BDBDBDBD:Header

32:Message ID

00:type  type=00

E377BD67:Timestamp Convert to big-endian67BD77E3-->Convert to decimal101740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

1100: The length of the message behind is converted to big-endian -->0011--> converted to decimal17, the length of the message behind is17bytes

0A:ID 0x0A--> convert to binary00001010 The first five digits are 00001--> convert to16hexadecimal01, which represents the data ID step counting (0x01),

The last three digits are 010--> convert to decimal02, which represents the length of the step counting data is2bytes

1E00: Step counting reported value converted to big-endian -->001E--> converted to decimal30, a total of30steps counted

11:ID 0x11--> convert to binary00010001 The first five digits are 00010--> convert to16hexadecimal02, which represents the data ID heart rate (0x02),

The last three digits are 001 --> converted to decimal 1, which represents the length of the heart rate data as 1 byte

4 B: Heart rate reporting value converted to decimal --> 75, heart rate data is 75

31: ID 0x31 --> converted to binary 00110001 The first five digits are 00110 --> converted to hexadecimal 06, which represents the data ID diastolic blood pressure (0x06),

The last three digits are 001 --> converted to decimal 1, which represents the length of the diastolic blood pressure data as 1 byte

4 A: Blood pressure diastolic reporting value, converted to decimal --> 74, diastolic blood pressure data is 74

39: ID 0x3 --> converted to binary 00111001 The first five digits are 00111 --> converted to hexadecimal 07, which represents the data ID systolic blood pressure (0x07),

The last three digits are 001 --> converted to decimal 1, which represents the length of the systolic blood pressure data as 1 byte

71: Blood pressure systolic reporting value, converted to decimal --> 113, systolic blood pressure data is 113

1 A: ID 0x1A --> converted to binary 00011010 The first five digits are 00011 --> converted to hexadecimal 03, which represents the data ID body temperature (0x03),

The last three digits are 010 --> converted to decimal 2, which represents the length of the body temperature data as 2 bytes

4A01: Body temperature reporting value, converted to big-endian -->014A--> Convert to decimal330-->330*0.1--> Body temperature data is33degrees Celsius

22:ID 0x22 --> Convert to binary 00100010 The first five digits are 00100 --> Convert to 16 Hexadecimal 04, which represents the data ID wrist temperature (0x04),

The last three digits are 010 --> Convert to decimal 2, which means the length of the wrist temperature data is 2 bytes

BC00: Wrist temperature reported value, converted to big-endian --> 00 BC --> Convert to decimal 188 --> 188 * 0.1 --> Wrist temperature data is 18.8 degrees Celsius

41:ID 0x41 --> Convert to binary 01000001 The first five digits are 01000 --> Convert to 16 Hexadecimal 08, code data ID blood oxygen (0x08),

The last three digits are 001 --> Convert to decimal 1, which means the length of the blood oxygen data is 1 byte

62: Blood pressure reported value, converted to decimal --> 98, blood oxygen data is 98

12: checksum

4.5.2 Device Sleep Analysis Data Upload (MSGID=0xC5)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xC5 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 4 | Int32 | DateTime |  | -/- | Upload UTC start time |
| 4 | Int32 | DateTime |  |  | Upload UTC end time |
| 2 | Int16 | Sleepminute |  |  | Upload sleep time in minutes |
| 4 | Int32 | Type |  |  | Sleep state: Upload type, 1 is deep sleep, 2 is light sleep, 3 is wake-up time |

Example:

Original hexadecimal message: BDBDBDBDC5AC338860693B8860210001000000D1

BDBDBDBD:Header

C5: Message ID

AC338860: Start time, converted to big-endian 608833 AC--> converted to decimal --> timestamp is 1619538860 seconds -->

Convert to standard time format UTC time: 2021 - 04 - 27 15:54:20 --> Convert to Beijing time: 2021 - 04 - 27 23:54:20

693B8860: End time, converted to big-endian60883BAC-->Convert to decimal-->Timestamp is1619540908seconds-->

Convert to standard time format UTC time: 2021 - 04 - 27 16 : 28 : 28 --> Convert to Beijing time: 2021 - 04 - 28 00 : 28 : 28

2100: Sleepminute sleep time, converted to big-endian 0021 --> converted to decimal 33 --> The total sleep time is 33 minutes

01000000: Type sleep state, converted to big-endian 00000001 --> converted to decimal 1 --> 1 is deep sleep, deep sleep time is 33 minutes

D1: checksum

4.6 Downlink feedback related reports

4.6.1 Downlink feedback (MSGID=0xC0)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xC0 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This command is used for feedback of downlink commands, returning the Message ID of the previously received downlink command (can be a collective return of multiple Message IDs).

Original hexadecimal message: BDBDBDBDC0011720

BDBDBDBD:Header

C0:Message ID

01:length Message ID length Convert decimal 01--> Message ID length is 1 byte

17  Type indicates that the device received the downlink command Message ID is 17

20:checksum

4.6.2 Message Status Report (MSGID=0x28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x28 | See below for definition | CK_sum |

Payload contents

| Bytes | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | timestamp | -/- | -/- | Timestamp |
| 1 | U8 | Type | -/- | -/- | Character encoding type (corresponding to downlink 0X28), GB2312 encoding: 0x03; Unicode encoding: 0x05 |
| 1 | U8 | status | -/- | -/- | Message Status - 0x01: Confirmed, 0x02: Rejected |
| 4 | U32 | seqID | -/- | -/- | (Corresponds to the downlink 0X28 message ID seqID) |

Example:

Note:Currently, the general version does not distinguish the character encoding type and is fixed to 03. Subsequent versions will distinguish it.

Original hexadecimal message:BDBDBDBD28FD3BA068030268A03BE477

BDBDBDBD:Header

28:Message ID

FD3BA068:timestamp Timestamp converted to big-endian 68A03BFD-->converted to decimal-->timestamp is 1755331581 seconds-->

Convert to standard time format UTC time: 2025-08-16 08:06:21-->Convert to Beijing time: 2025-08-16 16:06:21

03:Type 03--fixed value

02:status Message status 02--The interface clicks the reject button--Rejected

68A03BE4:seqID The ID of the downlink message is 68A03BE4

77:checksum

5 Settings

5.1 Downlink

5.1.1 Set the location reporting frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x17 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time Interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time Interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time Interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time Interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |

Note: A day is from 00:00-23:59
Example:

00:00-23:59 Report location every 10minutes:BDBDBDBD17010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

17:Message ID

01:enable is enabled Convert to decimal01-->1--Enabled

0A00:Interval time interval converted to big-endian000A-->converted to decimal10-->time interval is10minutes

00:time_start_h Start time - hour, convert to decimal00-->Start time - hour is00

00:time_start_m Start time - minute, convert to decimal00-->Start time - minute is00

17:time_end_h End time - hour, convert to decimal23-->End time - hour is 23

3B:time_end_m End time - minute, convert to decimal59-->End time - minute is 59

00:enable is enabled Convert to decimal00-->0--Not enabled

000000000000: All filled in disabled state00

00:enable is enabled Convert to decimal00-->0--Not enabled

000000000000: All filled in disabled state00

00:enable is enabled Convert to decimal00-->0--Not enabled

000000000000: All filled in disabled state00

97:checksum

5.1.2 Message Send (MSGID=0X28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x28 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | U8 | type | Message type, if it is downlink text information, Unicode encoding is 05 |
| 4 | Uint32 | seqID | The id of the information, unique |
| 1 | U8 | CONTENT LEN | Content length |
| N | N | CONTENT | Content, Chinese is Unicode encoded (little-endian first, pay attention to the conversion, one Chinese character occupies 2 bytes, one letter occupies 2 bytes) |

Note: The default Chinese character encoding in the Hong Kong general version is Unicode
The information id cannot be repeated for each transaction, and devices cannot receive repeated ids
Example:

Text message: hello,world  BDBDBDBD28050300000016680065006C006C006F002C0077006F0072006C006400DD

BDBDBDBD:Header

28: Message ID

05: type message type Convert to decimal 05-->type=05

03000000: seqID information id Information id is 03000000

16: CONTENT LEN content length Convert to decimal 22, the content length is 22 bytes

680065006C006C006F002C0077006F0072006C006400:

Convert every 2 bytes to big-endian --> convert to Unicode character --> combine the final - text message content:hello,world

DD:checksum

5.1.3 Settings - Location priority settings (0XCE01&0XCE02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

Type=01 Location Priority Setting

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 valid this time |
| 2 | U16 | Len | -/- | -/- | The length of the following instruction |
| n | n |  | -/- | -/- | Text |

Positioning priority setting definition--See2.3 Device downlink description--Device positioning priority distribution (0xCE01)

Positioning priority setting text: The current general version supports:01--GPS,02--wifi(wifi+base station) 03--Bluetooth beacon (additional deployment of Bluetooth beacon is required); Special equipment support:04--LBS base station 05--125k

Example:

wifi positioning priority (wifi>Bluetooth beacon>GPS):BDBDBDBDCE0100030002030133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

020301:Text: 02--wifi 03--Bluetooth beacon 01--GPS Positioning priority is wifi>Bluetooth beacon>GPS

33:checksum

GPS positioning priority (GPS>wifi>Bluetooth beacon):BDBDBDBDCE0100030001020333

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

010203:Text: 01--GPS 02--wifi 03--Bluetooth beacon Positioning priority is GPS>wifi>Bluetooth beacon

33:checksum

Bluetooth positioning priority (Bluetooth beacon>wifi>GPS):BDBDBDBDCE0100030003020133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

030201:Text: 03--Bluetooth beacon 02--wifi 01--GPS Positioning priority is Bluetooth beacon>wifi>GPS

33:checksum

Single GPS positioning:BDBDBDBDCE0100030001000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

030201:Text: 01--GPS 00--No positioning 00--No positioning Positioning priority is single GPS positioning

33:checksum

Single WiFi positioning:BDBDBDBDCE0100030002000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

020000:Text: 02--wifi 00--No positioning 00--No positioning Positioning priority is single WiFi positioning

33:checksum

Single Bluetooth beacon positioning:BDBDBDBDCE0100030003000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

020000:Text: 03--Bluetooth beacon 00--No positioning 00--No positioning Positioning priority is single Bluetooth beacon positioning

33:checksum

5.1.4 Settings - Health Sample Reporting Frequency Settings (0XCE02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

Type=02 Health Sample Reporting Frequency Settings

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=02 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 valid this time |
| 2 | U16 | Len | -/- | -/- | The length of the following instruction |
| 1 | U8 | Healthtype | -/- | -/- | 00—Indicates all |
| 1 | U8 |  | -/- | -/- | Text |
| 1 | u8 | Time Unit | -/- | -/- | Time unit 00—minutes, 01—hours |

Health sampling reporting frequency setting definition--see 2.3 Device downlink description--Device health sampling frequency distribution (0xCE02)

Health sampling reporting frequency setting text: The current general version supports:00-All, reserved item--Not yet implemented: 01 Step counting 02 Heart rate 03 Temperature 04 Sleep 05 Blood pressure 06 Blood sugar 07 Blood oxygen

Example:

Health sampling reported once every 5 minutes:BDBDBDBDCE0200030000050033

BDBDBDBD:Header

CE:Message ID

02:Type 02 Health sampling reporting frequency setting

00:Valid Validity 00 Always valid

0300:Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

00:Healthtype 00---All

05:Body: Convert decimal 05 --> Health sampling report frequency, report once every 5 minutes

00:Time Unit Time unit 00--minutes

33:checksum

5.1.5 Settings - Device Alarm Settings (0XCE03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

Type=03 Device alarm settings

| Byte offset | Format | Name | Unit | Decription |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Fixed to 0x03 - Device alarm settings |
| 1 | U8 | Valid | 1 | Fixed to 0x00 - Health threshold, always valid |
| 2 | U16 | Cfg Len | 2 | Instruction length |
| 1*N | U8*N | [Bit7] Enable [Bit6 - 0] Threshold Type | 1*N | [Bit7] Whether to enable threshold detection of this type [Bit6 - 0] Set threshold type: 0x01 heart rate, 0x02 systolic blood pressure (SBP) 0x03 diastolic blood pressure (DBP), 0x04 blood oxygen, 0x05 temperature (threshold unit is *10, for example, 37.3 is 373) |
| 2*N | I16*N | Threshold Min | 2*N | Lower threshold (this content is available only when threshold detection is enabled) |
| 2*N | I16*N | Threshold Max | 2*N | Upper threshold (this content is available only when threshold detection is enabled) |

Device alarm setting definition-Health threshold--See2.3 Device downlink description--Health threshold setting (0xCE0300)

Note: The default is the off state, and the threshold is the normal range. The device will report an alarm outside this range.

Heart rate threshold distribution range (20-200), blood pressure systolic threshold distribution range (20-200), blood pressure diastolic threshold distribution range (20-200), blood oxygen threshold distribution range (80-100), temperature threshold distribution range (160-600)

Example:

Set multiple health thresholds,1. Heart rate [60,120],2. Turn off blood oxygen threshold detection,3. Temperature [36.0,37.3]:BDBDBDBDCE03000B00813C007800048568017501FF

BDBDBDBD:Header

CE:Message ID

03:Type 03--Device alarm setting

00:Valid 00--Health threshold, always valid

0B00: Convert to big-endian-->000B--> Convert to decimal11--> The byte length of the subsequent instruction message (excluding checksum) 11 bytes

81: Convert to binary-->10000001 bit7:1---Open Open status bit6-0:000001-->0x01 Heart rate

3C00: Convert to big-endian-->003C--> Convert to decimal60, the lower limit of the heart rate threshold is60

7800: Convert to big-endian-->0078--> Convert to decimal120, the upper limit of the heart rate threshold is120

04: Convert to binary-->00000100 bit7:0---Close, no threshold after closing bit6-0:0000100-->0x04 Blood oxygen

85: Convert to binary-->10000101 bit7:1---Open Open status bit6-0:0000101-->0x05 Temperature

6801: Convert to big-endian-->0168--> Convert to decimal360(actual temperature *10)--> The lower limit of the temperature threshold is36.0

7501: Convert to big-endian --> 0175 --> Convert to decimal 373 (actual temperature * 10) --> Temperature threshold upper limit is 37.3

FF: checksum

5.1.6 Settings - Switch Settings (0XCE04-24)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Please refer to the instructions for the specific type |
| 1 | u8 | switch | -/- | -/- | Switch 00 on, 02 off, 01 single effective |
| 2 | U16 |  | -/- | -/- | Fixed 0000 |

Device alarm setting - switch type: specific function description -- see 2.3 Device downlink description

05 Bluetooth broadcast switch 06 Positioning and health data reporting switch ---- Previously used firmware, the latest firmware is no longer used

07 Fall alarm switch 08 Stay alarm switch

0A Immediately report location information    0B Immediately report health data

0D Immediately report version number data  16 Whether the downlink can use the key to shut down

17 Downlink supports long standby mode  18 Sleep function switch

19 Key trigger sos alarm switch 20 Report data switch--Separate settings (Note: This switch structure is special, and there are differences between the message and the table definition. Please see the specific example)

22 Long and short connection mode switching    24 GPS always-on switch

Type=05 Bluetooth broadcast switch
Example:

00---Open:BDBDBDBDCE0500000093

BDBDBDBD:Header

CE:Message ID

05:Type 05 Bluetooth broadcast switch

00:switch switch 00 On

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE0502000093

BDBDBDBD:Header

CE:Message ID

05:Type 05 Bluetooth broadcast switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=06 Location and health data reporting switch - used by previous firmware, not used by the latest firmware, reserved here
Example:

00---Enable (will be reported immediately after being issued when closed):BDBDBDBDCE0600000093

BDBDBDBD:Header

CE:Message ID

06:Type 06 Location and health data reporting switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE0602000093

BDBDBDBD:Header

CE:Message ID

06:Type 06 Location and health data reporting switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=07 Fall alarm switch
Example:

00---Enable:BDBDBDBDCE0700000093

BDBDBDBD:Header

CE:Message ID

07:Type 07 fall alarm switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE0702000093

BDBDBDBD:Header

CE:Message ID

07:Type 07 Fall Alarm Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=08 Stay Alarm Switch
Example:

00---Open:BDBDBDBDCE0800000093

BDBDBDBD:Header

CE:Message ID

08:Type 08 Stay Alarm Switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE0802000093

BDBDBDBD:Header

CE:Message ID

08:Type 08 Stay Alarm Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=0A Report Location Information Immediately
Example:

01---Single Report:BDBDBDBDCE0A01000093

BDBDBDBD:Header

CE:Message ID

0A:Type 0A Report Location Information Immediately

02: switch switch 01--single issue single report

0000:Fixed value

93:checksum

Type=0B Immediately report health data Example:

01---Single report: BDBDBDBDCE0B01000093

BDBDBDBD:Header

CE:Message ID

0B: Type 0B Immediately report health data

02: switch switch 01--single issue single report

0000:Fixed value

93:checksum

Type=0D Immediately report version number data Example:

01---Single report: BDBDBDBDCE0D01000093

BDBDBDBD:Header

CE:Message ID

0D: Type 0D Immediately report version number data

02: switch switch 01--single issue single report

0000:Fixed value

93:checksum

Type=16 Whether the downlink can use the key to shut down Example:

00---Enable: BDBDBDBDCE1600000093

BDBDBDBD:Header

CE:Message ID

16: Type 16 Whether the downlink can use the key to shut down

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE1602000093

BDBDBDBD:Header

CE:Message ID

16: Type 16 Can the button be used to shut down downlink?

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=17 Downlink supports long standby mode Example:

00---On: BDBDBDBDCE1700000093

BDBDBDBD:Header

CE:Message ID

17: Type 17 Downlink supports long standby mode

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Off: BDBDBDBDCE1702000093

BDBDBDBD:Header

CE:Message ID

17: Type 17 Downlink supports long standby mode

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=18 Sleep function switch Example:

00---On, enter sleep mode after being still for a period of time, and do not report data: BDBDBDBDCE1800000093

BDBDBDBD:Header

CE:Message ID

18: Type 18 Sleep function switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Off, normal data reporting in static or charging state: BDBDBDBDCE1802000093

BDBDBDBD:Header

CE:Message ID

18:Type 18 Sleep Function Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=19 Key Triggered SOS Alarm Switch
Example:

00---On:BDBDBDBDCE1900000093

BDBDBDBD:Header

CE:Message ID

19:Type 19 Key Triggered SOS Alarm Switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Off:BDBDBDBDCE1902000093

BDBDBDBD:Header

CE:Message ID

19:Type 19 Key Triggered SOS Alarm Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=20 Report Data Switch—Separate Settings
Example:

Added field: Body length: 2 bytesBody: 0x00 (location data reporting) + switch (0x00: on, 0x02: off)

0x01 (health data reporting) + switch (0x00: on, 0x02: off)

Location data reporting on:BDBDBDBDCE20000200000093

BDBDBDBD:Header

CE:Message ID

20:Type 20 Report Data Switch

00:Fixed value

0200:Text length converted to big-endian 0002-->converted to decimal 02-->Text length is 2 bytes

00:Text: 00--Location data reporting switch

00:Text: 00--Enable reporting

93:checksum

Location data reporting disabled:BDBDBDBDCE2000020000293

BDBDBDBD:Header

CE:Message ID

20:Type 20 Report Data Switch

00:Fixed value

0200:Text length converted to big-endian 0002-->converted to decimal 02-->Text length is 2 bytes

00:Text: 00--Location data reporting switch

02:Text: 00--Disable reporting

93:checksum

Health data reporting enabled:BDBDBDBDCE20000200010093

BDBDBDBD:Header

CE:Message ID

20:Type 20 Report Data Switch

00:Fixed value

0200:Text length converted to big-endian 0002-->converted to decimal 02-->Text length is 2 bytes

01:Text: 00--Health data reporting switch

00:Text: 00--Enable reporting

93:checksum

Health data reporting disabled:BDBDBDBDCE20000200010293

BDBDBDBD:Header

CE:Message ID

20:Type 20 Report Data Switch

00:Fixed value

0200:Text length converted to big-endian 0002-->converted to decimal 02-->Text length is 2 bytes

01:Text: 00--Health data reporting switch

02:Text: 02--Disable reporting

93:checksum

Type=22 Long and short connection mode switching
Example:

00---Long connection mode, default is long connection:BDBDBDBDCE2000000093

BDBDBDBD:Header

CE:Message ID

22:Type 22 Long and short connection mode switching

02:switch switch 00 Enable long connection mode

0000:Fixed value

93:checksum

02---Short connection mode:BDBDBDBDCE2002000093

BDBDBDBD:Header

CE:Message ID

22:Type 22 Long and short connection mode switching

02:switch switch 02 Turn off long connection mode--short connection mode

0000:Fixed value

93:checksum

Type=24 GPS always-on switch
Example:

00---Enable:BDBDBDBDCE2400000093

BDBDBDBD:Header

CE:Message ID

24:Type 24 GPS always-on switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE2402000093

BDBDBDBD:Header

CE:Message ID

24:Type24 GPS always-on switch

02:switch switch 02 off

0000:Fixed value

93:checksum

5.1.7 Settings - Drop Sensitivity Setting (0xCE15)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Decription |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Type is 0x15 |
| 1 | U8 | Valid | 1 | The default is 0, which has no impact for now. |
| 2 | U16 | Len | 2 | Length of the following parameter content |
| 1 | U8 | Param Type | 1 | Parameter type: 0x00: Sensitivity, 0x01: Height |
| 1 | U8 | Param Level | 1 | Parameter level: 5 levels (0 - 4) |

Drop sensitivity setting-Parameter description:

Sensitivity: Refers to the degree to which the drop algorithm is triggered, providing5setting levels (0 - 4): Low - Lower (Medium Low) - Medium - Higher (Medium High) - High.

Height: Refers to the height that satisfies the trigger of the drop alarm, providing5setting levels (0 - 4):0.5m - 1.0m - 1.5m - 2.0m - 2.5m.

If the height is set to "1.5m", the device needs to be ≥1.5mto trigger the fall alarm.

Note that sensitivity and height cannot be combined in one message.

Example:

Set sensitivity to "Medium Low": BDBDBDBDCE150002000001FF

BDBDBDBD:Header

CE:Message ID

15:Type Fall Sensitivity Setting

00:Fixed value

0200:Len

00:Param Type Sensitivity Setting

01:Param Level Sensitivity Level 1-->Lower (Medium Low)

FF:checksum

Set height to "1.5m": BDBDBDBDCE150002000102FF

BDBDBDBD:Header

CE:Message ID

15:Type Fall Sensitivity Setting

00:Fixed value

0200:Len

01:Param Type Height Setting

02:Param Level Height Level 2-->1.5m

FF:checksum

5.1.8 Settings - Vibration Settings (0XCE23)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Decription |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Fixed at 0x23 |
| 1 | U8 | Valid | 1 | 0x00 |
| 2 | U16 | Cfg Len | 2 | Length of the following parameter content |
| 1 | U8 | Motor Type | 1 | Setting type: 1. Vibration duration - 0x00 |
| 4 | U32 | Value | 4 | Setting content: 1. Vibration duration (unit: milliseconds, range: 0 - 60 seconds) |

Settings - Vibration Settings Definition: This message can be used to modify the global vibration duration, and any vibration duration will be changed to the duration set by this command.

Example:

The default global vibration duration of the device is 1.5 seconds (1500 milliseconds):BDBDBDBDCE2300050000dc050000FF

BDBDBDBD:Header

CE:Message ID

23: Type 23 Settings - Vibration Settings

00: Fixed value 00 - Always valid

0500:The length of the following parameter content is converted to big-endian 0005--> converted to decimal 05--> the length of the following parameter content is 5 bytes

00:Motor Type Vibration Duration Setting

dc050000:Value Vibration duration Converted to big-endian 000005dc--> converted to decimal 1500--> vibration duration is 1500 milliseconds (1.5 seconds)

FF:checksum

5.1.9 Settings - Interface Display and Hide Settings (0xCE27)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Decription |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Type is 0x27 |
| 1 | U8 | Valid | 1 | Fill with 0x00, no impact for now |
| 2 | U16 | Len | 2 | Parameter content length |
| 1*N | U8*N | Param Type | 1*N | [Bit 7] Whether to enable: 0x00: Hide, 0x01: Show [Bit 6:0] Interface type: 0x01: Heart rate, 0x02: Blood pressure, 0x03: Blood oxygen, 0x04: HRV (General version does not have this interface, special version has), 0x05: Body temperature, 0x06: Step counting, 0x07: Exercise |

Example:

Hide heart rate, blood pressure, step counting interface, display blood oxygen interface: BDBDBDBDCE2700040001020683FF

BDBDBDBD:Header

CE:Message ID

27: Type 27 Interface display and hide settings

00: Fixed value

0400: Parameter content length converted to big-endian 0004 --> converted to decimal 04 --> parameter content length is 4 bytes

01: Convert to binary ---> 00000001 bit7: 0 --> 00 Hide bit6-0: 0000001 --> 01 Heart rate

02: Convert to binary ---> 00000010 bit7: 0 --> 00 Hide bit6-0: 0000010 --> 02 Blood pressure

06: Convert to binary ---> 00000110 bit7: 0 --> 00 Hide bit6-0: 0000110 --> 06 Step counting

83: Convert to binary ---> 10000011 bit7: 1 --> 01 Show bit6-0: 0000011 --> 03 Blood oxygen

FF: checksum

5.1.10 IP & Domain Name Settings (0xC3) (TCP Dedicated)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC3 | See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | Issue type: Type =1 IPv4, Type =2 IPv6 (reserved - not supported yet), Type =3 Domain name (only supports ASCII encoded domain names for now) |
| 2 | U16 | Port |  |  | Port number (2 bytes) |
| 1 | U8 | Len |  |  | Length of content after parameter |
| N | U8 | Domainname |  |  | Specific IP or domain content |

Note: After distribution, the device will no longer communicate on the original server, but will point to the modified server.

Example:

Type=1 IPv4:Downlink modifies IP to:118.178.184.219, port: 8825 BDBDBDBDC30179220476B2B8DB33

BDBDBDBD:Header

C3:Message ID

01:Type 1--IPv4

7922:Port Convert to big-endian 2279-->Convert to decimal 8825-->Port: 8825

04:Len The length of the content of the following parameters Convert to decimal 04-->The length is 4 bytes

76B2B8DB:Domainname IP 76-->Convert to decimal 118, B2-->Convert to decimal 178, B8-->Convert to decimal 184, DB-->Convert to decimal 219 IP is: 118.178.184.219

33:checksum

Type=3 Domain nameDownlink modifies domain name to:aiday.com.cn, port: 8825

BDBDBDBD:Header

C3:Message ID

03:type=3, domain name

7922:Port Convert to big-endian--2279 -->Convert to decimal Port: 8825

12:Len The length of the content of the following parameters Convert to decimal 18-->The message length is 18 bytes

61696461792e636f6d2e636e:Domainname domain name converted to string --> aiday.com.cn --> domain name is: aiday.com.cn

1D:checksum

5.1.11 Issue sedentary stay alarm trigger time (MSGID=0XCC)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xCC | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | minute | -/- | -/- | Stay alarm trigger duration (unit: minutes; value range 2 — 60), inactivity within this time length will trigger a stay alarm |

Example:

No activity for 5 consecutive minutes The watch reports a stay alarm (when the watch is worn): BDBDBDBDCC05DDBDBDBDBD:HeaderCC:Message ID05: minute Stay alarm trigger duration Convert to decimal 05-->5 minutesDD:checksum

5.1.12 Shutdown restart (0x77)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x77 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- |  | 01—Shutdown, 00—Restart |

Note: Shutdown and restart are both received in the power-on state. Shutdown: When the command is received in the power-on state, the watch shuts down and cannot receive any further commands after shutdown; Restart: When the command is received in the power-on state, the watch restarts.

Example:

Shutdown:BDBDBDBD770193

BDBDBDBD:Header

77:Message ID

01:Type 01--Shutdown

93:checksum

Restart:BDBDBDBD770093

BDBDBDBD:Header

77:Message ID

01:Type 00--Restart

93:checksum

5.1.13 Personal Information Distribution (0xCA)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xCA | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | count | -/- |  | Issue several sets of information |
| 1 | U8 | type | -/- |  | Type: 1 – Name, 2 – English Name, 3 – Phone, 4 – Blood Type |
| 1 | U8 | len | -/- |  | Length |
| N | U8 | content | -/- |  | Content (Unicode encoding) |
| ... | ... | ... | ... | ... | ... |

Example:

Name: Cheng Da Wen, English Name: Chan Tai Man, Phone:99886633, Blood Type: A

BDBDBDBDCA0401060B7A2759876502184300680061006E00200054006100690020004D0061006E0003103900390038003800360036003300330004024100DD

BDBDBDBD:Header

CA: Message ID

04:count Convert to decimal04-->4 groups of information

01:type 01--Name

06:len Convert to decimal06-->Length 6 bytes

0B7A27598765:content

0B7A converted to big-endian-->7A0B-->Convert Unicode encoded character-->Cheng,

2759converted to big-endian-->5927-->Convert Unicode encoded character-->Da,

8765converted to big-endian-->6587-->Convert Unicode encoded character-->Wen Chinese name is Cheng Da Wen

02:type 02--English name

18 len Convert to decimal24-->Length 24 bytes

4300680061006E00200054006100690020004D0061006E00:content

Similarly, every2bytes converted to big-endian-->Convert Unicode character-->Final: Chan Tai Man

03:type 03--Phone number

10 : len Convert to decimal 16 --> Length 16 bytes

39003900380038003600360033003300 :content

Similarly, every 2 bytes are converted to big-endian --> Convert to Unicode string --> Final: 99886633

04 : type 04 -- Blood type

02 : len Convert to decimal 02 --> Length 2 bytes

41: Convert to Unicode string --> A --> Blood type: A

DD: checksum

5.1.14 Sleep statistics time period setting (MSGID = 0X1D)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x1D | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time period 1 |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 2 |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 3 |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 4 |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |

Example:

The sleep data statistics and reporting time period is 13:00 - 15:00 and 22:30 - 08:00 BDBDBDBD1D010D000F0001161E080000000000000000000000FF

BDBDBDBD:Header

1D:Message ID

01:enable Whether to enable Convert to decimal 01-->1--Enable

0D:time_start_h Start time - hour Convert to decimal 13

00:time_start_m Start time - minute Convert to decimal 00 Start statistics and report time is 13:00

0F:time_end_h End time - hour Convert to decimal 15

00:time_end_m End time - minute Convert to decimal 00 End statistics and report time is 15:00

01:enable Whether to enable Convert to decimal 01-->1--Enable

16:time_start_h Start time - hour Convert to decimal 22

1E:time_start_m Start time - minute Convert to decimal 30 Start statistics and report time is 22:30

08:time_end_h End time - hour Convert to decimal 08

00:time_end_m End time - minute Convert to decimal 00 End statistics and report time is 08:00

00:Whether enable is enabled Convert decimal 00-->0--Not enabled

00000000:Unenabled status is all filled with 00

00:Whether enable is enabled Convert decimal 00-->0--Not enabled

00000000:Unenabled status is all filled with 00

FF:checksum

5.1.15 Remote OTA upgrade distribution (0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Number of types (reserved 00) |
| 1 | u8 |  | -/- | -/- | Fixed value (reserved byte 00) |
| 1 | u8 | Type | -/- | -/- | Type 1 00—mcu firmware |
| 1 | U8 | PathLen | -/- | -/- | Path 1 length |
| n | N*u8 | Path | -/- | -/- | Path |
| 2 | U16 | PackageSize |  |  | File package size is fixed at 0000 |
| ... | ... | ... | ... | ... | ... |
| 1 | u8 | Type | -/- | -/- | Type n 00—mcu firmware |
| 1 | U8 | PathLen | -/- | -/- | Path n length |
| n | N*u8 | Path | -/- | -/- | Path |
| 2 | U16 | PackageSize |  |  | File package size is fixed at 0000 |
| ... | ... | ... | ... | ... | ... |

Note: Remote updates consume traffic and are affected by the network. The device's SIM card has 30M per month, so it cannot be updated too many times, otherwise it will cause insufficient traffic.

Example:

The firmware download address is (requires server support for http download):http://tools.aiday.com.cn/File/MCU/W200PG/W200PG_E42.BWGHOL25.bin

After the update is successful, the device will restart and reconnect to the server, reporting a version number change

BDBDBDBDA902000041687474703A2F2F746F6F6C732E61696461792E636F6D2E636E2F46696C652F4D43552F5732303050472F5732303050475F4534322E425747484F4C32352E62696E00000A

BDBDBDBD:Header

A9:Message ID

01:TypeCnt Number of types 01--Indicates1type

00:Reserved bytes, fixed value

00:Type 00--Indicates MCU firmware upgrade

41:Path Download path length Convert to decimal 65-->Download path length is 65

687474703A2F2F746F6F6C732E61696461792E636F6D2E636E2F46696C652F4D43552F5732303050472F5732303050475F4534322E425747484F4C32352E62696E

File download path Convert to string--> http: //tools.aiday.com.cn/File/MCU/W200PG/W200PG_E42.BWGHOL25.bin

0000:PackageSize Currently fixed at 0000

0 A: checksum

5.1.16 Weather warning (0XCB)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xCB | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Count | -/- | -/- | How many groups of weather warnings in total |
| 1 | U8 | Type | -/- | -/- | Weather alert type, see below for specific types |
| 1 | U8 | Status | -/- | -/- | Status, see below for specific statuses |
| … | … | …… |  |  | … |

Weather Type - Status Table:

| Type | Decription | Status | notes - Click image to enlarge |
| --- | --- | --- | --- |
| 1 | Storm Warning | 0 | Cancel Alert |
|  |  | 1 - 8 |  |
| 2 | Heavy Rain Warning | 0 | Cancel Alert |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |
| 3 | Thunderstorm Warning | 0 | Cancel Alert |
|  |  | 1 |  |
| 4 | Flood Warning | 0 | Cancel Alert |
|  |  | 1 |  |
| 5 | Landslide Warning | 0 | Cancel Alert |
|  |  | 1 |  |
| 6 | Strong Monsoon Warning | 0 | Cancel Alert |
|  |  | 1 |  |
| 7 | Frost Warning | 0 | Cancel Alert |
|  |  | 1 |  |
| 8 | Fire Danger Warning | 0 | Cancel Alert |
|  |  | 1 |  |
|  |  | 2 |  |
| 9 | Temperature Warning | 0 | Cancel Alert |
|  |  | 1 |  |
|  |  | 2 |  |
| 10 | Tsunami Warning | 0 | Cancel Alert |
|  |  | 1 |  |
| 11 | Heat Index | 0 | Cancel Alert |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |

Note: After the issue, the device vibrates to display the weather warning picture. Multiple sets of weather warnings can be issued at the same time. Each set of weather warnings must be different. The same type of warning cannot appear in one instruction, otherwise it will be invalid
Example:

10Groups of weather warnings issued: BDBDBDBD CB 0A 0101 0201 0301 0401 0501 0601 0701 0801 0901 0A01 F5

BDBDBDBD:Header

CB:Message ID

0A:Count Number of weather groups converted to decimal10-->Issued10Weather groups

0101:Type&Status Weather type-status 01--Storm warning 01--1Alert signal

0201:Type&Status Weather type-status 02--Rainstorm warning 01--Yellow rainstorm warning signal

0301:Type&Status Weather type-status 03--Thunderstorm warning 01--Thunderstorm warning

0401:Type&Status Weather type-status 04--Flooding warning 01--Special report of flooding in the northern New Territories

0501:Type&Status Weather type-status 05--Landslip warning 01--Landslip warning

0601:Type&Status Weather type-status 06--Strong monsoon warning 01--Strong monsoon signal

0701:Type&Status Weather type-status 07--Frost warning 01--Frost signal

0801:Type&Status Weather type-status 08--Fire danger warning 01--Yellow fire danger warning

0901:Type&Status Weather Type-Status 09--Temperature Warning 01--Cold Weather Warning

0A01: Type&Status Weather Type-Status 10 --Tsunami Warning 01 --Tsunami Advisory

F5: checksum