G618L-Lorawan-Communication Protocol

Directory

1 Overview	3

2 Device Instructions	3

2.1 Device Function and Usage Instructions	3

2.2 Device Default Reporting Logic	4

2.3 Device Downlink Description	5

3 Protocol Data Packet Structure	6

3.1 Data Header	6

3.2 Message Identifier (Message ID)	6

3.3 Token Generation Mechanism	6

3.4 Payload	6

3.5 Checksum	8

4 Report messages message	9

4.1 Alarm-related reporting	9

4.1.1 Alarm data upload-1(MSGID=0x02)	9

4.1.2 Warning Data (MSGID=0xC7)	11

4.2 Location-related reporting	12

4.2.1 GPS/BDS Location Reporting: Location Data Reporting (MSGID=0x03)	12

4.2.2 Bluetooth Location Information (LBE Location) (MsgId=0xD6)	14

4.  2 Device Information and Status Reporting	17

4.  2.  1 Power Signal (MSGID=0xF9)-Important	17

5 Settings	20

5.1 Downlink	20

5.1.1 Set Location Report Frequency (MSGID=0x17)	20

5.1.2 Settings - Location Priority Settings (0XCE01)	22

5.2 Server time synchronization information	24

5.2.1 Device reports request time calibration data	24

5.2.2 Time Calibration Request Data Reply	25

1 Overview

This protocol is suitable for Lorawan devices, currently supporting G618L products, using a 32-bit data header for synchronization and terminal identification; using a low-overhead checksum algorithm for checksum protection; and using message identifiers to identify different messages.
It needs to be parsed according to the actual reporting. The document is currently being continuously maintained. If any errors or omissions are found, please provide timely feedback. Thank you very much.
The protocol content is 4g reporting content. If you need the Bluetooth broadcast data protocol, please consult relevant personnel.

2 Device Instructions

2.1 Device Function and Usage Instructions

General version: The default high-frequency general purpose is AS923 (923.2-924.6), and the low-frequency general purpose is CN470 (470.3-471.7) OTAA-CLASSA. If other frequency bands are required, please submit a request.

Network access related keys:

OTAA:

Application key : 2B7E151628AED2A6ABF7158809CF4F3C

ABP:

Device address: Last eight digits of the watch DEVEUI

Application session key: EF 6 D 6 E 2503 F 57 AE 2 FA 151 CDA 87455 F 18

Network session key: 2 E 8 C 8650 B 4041672 BBB 9 A 399 F 2 DEB 427

(1) Power on:

Please fully charge the device before the first use. It will automatically power on when charging. The light effect during charging is: red light. The light effect when fully charged is: solid green light.

Manual power on when the device is off: Connect the charging cable (there is a button on the charging cable), long press the charging cable button for 2 seconds, the power-on light effect: red and green lights flashing alternately

(2) Power off:

Charging: Automatic shutdown Manual shutdown: Shutdown: Long press the button on the charging cable for 10 seconds, keep pressing until the red light flashes 3 times, then goes off, and release immediately Low battery shutdown: Flashing red light then goes off

(3) SOS function:

Trigger: Connect the charging cable, long press the button on the charging cable for 3 seconds, release after the red light is on, sos light effect: red light is on Cancel: Under the red light is on, long press the button on the charging cable for 3 seconds, the red light goes off, cancel the SOS alarm

(4) Device hibernation:

Trigger condition: If the device remains motionless for 20 minutes, it enters sleep mode and does not report data.

2.2 Device Default Reporting Logic

General Version:

(2) Location-related reporting

GPS / Bluetooth beacon: Default reporting frequency 10 minutes, default positioning priority: Bluetooth > GPS, Bluetooth positioning is preferred, switch to GPS positioning if no positioning is available

(3) Alarm-related reporting

SOS Alarm (0x02): User actively triggers, see the previous section for triggering methods

SOS Cancel (0x02): User actively triggers cancellation, see the previous section for triggering methods

Shutdown Alarm (0x21): Device actively shuts down or shuts down due to low battery, see the previous section for triggering methods

Low Battery Alarm (0x02): Triggered when the device's current battery level is less than or equal to 0

Fall alarm (0x02): The device falls freely from a certain height, satisfying the fall algorithm trigger

(4) Device information and status report

F9: Power and signal report, a report will follow when the location is reported

(5) Time synchronization request report

Time synchronization request (FF00FF): The device reports once every time it is turned on, requesting the server to send a downlink command to calibrate the time

Note: The device reports in a combined package, that is, a data packet contains multiple complete messages. Pay attention not to miss any. The message is a complete message, and there will be no phenomenon of being interrupted in the middle and in the next data packet.

Eg:bdbdbdbdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9bdbdbdbdf9010000006400002800000019a9cf61ca

This data packet contains (0xD6) Bluetooth positioning and (0xF9) power signal messages

(0xD6) Bluetooth positioning:bdbdbdbdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9

(0xF9) Power signal:bdbdbdbdf9010000006400002800000019a9cf61ca

2.3 Device Downlink Description

General version:

(1) Device location reporting frequency issued (0x17):

The default reporting frequency is 10 minutes, with a maximum of 1 minute. After the device receives the downlink command, the device reports according to the time period and frequency of the issued command.

Report data, and report according to the default reporting frequency outside the time period, such as: 00:00-18:00 2 minutes location reporting,

Then report according to the default 10-minute reporting frequency outside the time period

(2) Device location priority issued (0xCE01)

The default location priority is Bluetooth > gps,

If the issued location priority is: gps>Bluetooth beacon, gps positioning is preferred, and gps positioning cannot be switched

Bluetooth beacon, when the positioning is successful, the next positioning priority will not be switched to generate positioning

(3) Calibration time command downlink (0XFF)

When the device reports a time synchronization request message, this command is immediately sent to calibrate the watch time.

3 Protocol Data Packet Structure

A basic protocol data packet structure is shown in Figure 1:

3.1 Data Header

Each data packet starts with a 4-byte Header or token (timestamp replaces it in some response messages):

Currently, the token uploaded by Oufu devices is fixed as BDBDBDBD

Header: 0xBD0xBD0xBD0xBD;

Timestamp: 32bits, generated by the server

3.2 Message Identifier (Message ID)

See Chapter 4 for the content represented by MessgeId.

3.3 Token Generation Mechanism

Currently fixed as BDBDBDBD except for FF00FF time synchronization requests and response messages.

3.4 Payload

The payload we refer to below is the valid body content in the protocol, excluding the head token and checksum. The length of the body is noted after it.

The data format used in the payload is shown in the following table:

[U-unsigned; I-signed; X-bitfield; Number - number of bytes occupied]

The following protocols use little-endian priority except for ch, u8, i8, and x8.

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Explanation |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short Integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short Integer |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Plastic surgery |
| x16 | Bitfield | 2 | - | - | Bit 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long Integer |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit long integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating point type |

3.5 Checksum

The content added to the checksum includes the payload, as shown in Figure 1. The algorithm is as follows. The default general-purpose version of the device does not require verification and can ignore this part. Any byte can be used for the downlink instruction.

// Assuming content="BDBDBDBDE9000600010A00000A00", then sum.ToString("X2") results in 07// The following is the C# code calling method

private string CheckSum(string content)

{

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2");

}

4 Report messages message

4.1 Alarm-related reporting

4.1.1 Alarm data upload-1(MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x02 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below for definition |
| 4 | U32 | Timestamp | - | - | Timestamp (a timestamp will be added at the end when re-uploading) |

Upl_warn contents

| bit (corresponding to position 1 in binary) | Name | Description | Original message in little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Shutdown | Shutdown | 0400 | 0004 | 4 |
| 0 | Low battery | Low battery | 0100 | 0001 | 1 |
| 14 | Fall alarm | Fall alarm | 0040 | 4000 | 16384 |
| 1 | SOS alarm | SOS alarm | 0200 | 0002 | 2 |
| 7 | SOS canceled | SOS canceled | 8000 | 0080 | 128 |

Example:

Low battery alarm: BDBDBDBD020200E377BD678A

BDBDBDBD:Header

02:Message ID

0100:Upl_warn alarm content, converted to big-endian0001-->Low battery alarm

E377BD67: Timestamp converts the timestamp to big-endian 67 BD77E3 --> converts to 10 hexadecimal 1740470243 --> The timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

8 A: checksum

Shutdown alarm: BDBDBDBD020400E377BD678A

BDBDBDBD:Header

02:Message ID

0400 : Upl_warn alarm content, converted to big-endian 0004 --> Shutdown alarm

E377BD67: Timestamp converts the timestamp to big-endian 67 BD77E3 --> converts to 10 hexadecimal 1740470243 --> The timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

8 A: checksum

Note: This alarm is used for devices that do not distinguish between shutdown types. See the next section for devices that distinguish between shutdown types.

SOS alarm: BDBDBDBD020200E377BD678A

BDBDBDBD: Header

02 : Message ID

0200 : Upl_warn alarm content, converted to big-endian 0002 --> SOS alarm

E377BD67: Timestamp Convert to big-endian 67 BD77E3 --> Convert to decimal 1740470243 --> The timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

8 a: checksum

SOS Cancellation: BDBDBDBD028000E377BD678A

BDBDBDBD: Header

02 : Message ID

8000 :Upl_warn alarm content, converted to big-endian 0080 -->SOS cancellation

E377BD67: Timestamp Convert to big-endian 67 BD77E3 --> Convert to decimal 1740470243 --> The timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

8 a: checksum

Drop Alarm: BDBDBDBD020040E377BD678A

BDBDBDBD: Header

02 : Message ID

0040 :Upl_warn alarm content, converted to big-endian 4000 -->Drop alarm

E377BD67: Timestamp Convert to big-endian 67 BD77E3 --> Convert to decimal 1740470243 --> The timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

E7: checksum

4.1.2 Warning Data (MSGID=0xC7)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xC7 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Type |  |  | The type is determined by whether the binary bit is 1 to determine whether there is this alarm |

Type:

bit7:1--Location failure

Example:

Location failure: BDBDBDBDC72000FF

BDBDBDBD: Header

C7: Message ID

2000: Convert to big-endian 0020 --> Convert to binary 0000000000100000, bit7 is 1, indicating location failure alarm

FF: Checksum

4.2 Location-related reporting

4.2.1 GPS/BDS Location Reporting: Location Data Reporting (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x16 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude longitude |
| 8 | Double | lat |  | - | latitude latitude |
| 1 | U8 | north_south |  | - | /N or S/ North or South Latitude |
| 1 | U8 | east_west |  | - | /E or W/ East or West longitude |
| 1 | U8 | status |  | - | /A or V/ Valid data or invalid data |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original16Hexadecimal message: BDBDBDBD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BDBDBDBD:Header

03:Message ID Message ID

963E870C3E675E40:lon longitude Convert to big-endian405E673E0C873E96-->Longitude:121.6131622 Note: Refer to the parsing example below for conversion and parsing

FF2110C2B6343F40:lat latitude Convert to big-endian `403F34B6C21021FF-->Latitude:31.2059137 Note: Refer to the parsing example below for conversion and parsing

4E:north_south 4E-->Convert to ASCII code N-->North latitude

45:east_west 45-->Convert to ASCII code E-->East longitude

41:status data validity 41-->Convert to ASCII code A-->Valid data

E377BD67:Timestamp Convert to big endian67BD77E3-->Convert to base101740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

78:checksum

Note: The parsed GPS latitude and longitude are in the WGS-84coordinate system. If the map uses Baidu Gaode, etc., the coordinate system needs to be converted.

Parsing satellite positioning message Java code example:

//Message: BDBDBDBD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BDBDBDBD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public static void main (String[] args){

//Message 7d9f84ac81815c40 actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d",16)));  //114.02353966666665//Message e766926b1d893640 Actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16))) ;//22.535605166666667

HexToStr(data.Substring(“4e”));   //N

HexToStr(data.Substring(“45”));//E

HexToStr(data.Substring(“41”));//A A indicates valid data, and V indicates invalid data, which can be discarded//Message 749d695f actual value 5f699d74

Date date=new Date();

date.setTime(Long.parseLong(“5f699d74",16)*1000);

SimpleDateFormatsdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.2.2 Bluetooth Location Information (LBE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xD6 | See below See definition below | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 0 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple sets of information, and each group may contain multiple ibeacons |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| ... | ... | ... | ... |

Note: In the general version, the total number of Bluetooth beacon groups is fixed at 1, and the maximum number of Bluetooth beacons is 4. The sorting is based on signal strength from strongest to weakest. By default, the watch only recognizes our own beacons. If you need to connect to other manufacturers' beacons, please consult relevant personnel via WeChat.
Example:

Original hexadecimal message: BDBDBDBDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

bdbdbdbd: Header

D6: Message ID

00: type

01: Only one set of ibeacon data

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

04: The first group has 4 beacon information

4327: Convert to big-endian-->major : 0x2743-->Convert to decimal 10051

1794: Convert big-endian to little-endian --> minor: 0x9417 --> Convert to decimal 37911

AC:  RSSI 0xAC-->Convert to decimal complement -84

4327: Convert to big endian-->major: 0x2743-->Convert to decimal 10051

3094: Convert to big endian-->minor:0x9430-->Convert to decimal 37936

AA:  RSSI 0xAA-->Convert to decimal complement -86

4327:Convert to big endian-->major: 0x2743-->Convert to decimal 10051

b956:Convert to big endian-->minor:0x56b9-->Convert to decimal 22201

A5:   RSSI 0xA5-->Convert to decimal complement -91

4327: Convert to big endian-->major: 0x2743-->Convert to decimal 10051

FE94: Convert to big endian-->minor:0x94FE-->Convert to decimal 38142

A5: rssi:0xA5-->Convert to decimal complement -91

6a --checksum

Bluetooth beacon major, minor description: The default shipping Bluetooth beacon has a beacon code attached.

For example: 00081005345866, then the major of the Bluetooth beacon is 10053, the minor is 45866, and the manufacturer ID is 0008 (this is useless and has no practical significance).

4.  2 Device Information and Status Reporting

4.  2.  1 Power Signal (MSGID=0xF9)-Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xF9 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Power Type 0: 4-level system, 1: 5-level system, 2: Percentage, 3: Voltage value |
| 2 | u16 | Bat_volt |  | -/- | Power value If Bat_type is 0: the power value range is 0-3, (0 is 25%, 3 is 100%) If Bat_type is 1: the power value range is 0-4 (0 is 20%, 4 is 100%) If Bat_type is 2: the power value range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: percentage, 1: 5-level system, 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal strength |
| 1 | U8 | Other_type |  |  | Extended type 0: Full step counting 1: Incremental step counting 2: Vibration |
| 4 | U32 | Num |  |  | Extended value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Note: The F9 heartbeat packet must have a server downlink reply. The device considers the server connection to be unbroken only after receiving the server downlink reply; otherwise, it will re-report the F0 request to connect.

Example:

Original16hexadecimal message: BDBDBDBDF90104000050000095000000E377BD67AA

BDBDBDBD: Header

F9: Message ID (Message ID)

01:Bat_type battery type 01-->1-->5level battery (0-4)

0400:Bat_volt battery value converted to big-endian 0004-->4-->Level4corresponds to100% battery

00:Signal_type signal type 00-->0-->percentage

5000:Signal_strength signal strength converted to big-endian0050-->converted to decimal80-->80% signal strength

00:Other_type extension type 00-->0-->full step counting

95000000:Num extension value - step counting converted to big-endian00000095-->converted to decimal149-->number of steps is149

E377BD67: Timestamp timestamp converted to big-endian67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

AA: checksum

5 Settings

5.1 Downlink

5.1.1 Set Location Report Frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x17 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |

Note: A day is from 00:00-23:59
Example:

00:00-23:59 Report location every 10minutes:BDBDBDBD17010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

17:Message ID

01:enable Whether to enable Convert to decimal01-->1--Enabled

0A00:Interval time interval converted to big-endian000A-->converted to decimal10-->time interval is10minutes

00:time_start_h Start time-hour, convert to decimal00-->Start time-hour is00

00:time_start_m Start time-minute, convert to decimal00-->Start time-minute is00

17:time_end_h End time-hour, convert to decimal23-->End time-hour is23

3B:time_end_m End time-minute, convert to decimal59-->End time-minute is59

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All fill00 in the disabled state

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All fill00 in the disabled state

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All fill00 in the disabled state

97:checksum

5.1.2 Settings - Location Priority Settings (0XCE01)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See definition below | CK_sum |

Payload contents

Type=01 Location Priority Settings

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 valid for this time |
| 2 | U16 | Len | -/- | -/- | Length of the following instruction |
| n | n |  | -/- | -/- | Text |

Location priority setting definition--see 2.3 Device downlink description--Device location priority distribution (0xCE01)

Body of location priority settings: Currently, the general version supports:01--GPS,03--Bluetooth beacon (requires additional deployment of Bluetooth beacons),

Example:

GPS positioning priority (GPS>Bluetooth beacon):BDBDBDBDCE0100030001020333

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The length of the text is 3 bytes

010300:Text: 01--GPS 03--Bluetooth beacon 00--No positioning Positioning priority is GPS>Bluetooth beacon

33:checksum

Bluetooth positioning priority (Bluetooth beacon>GPS):BDBDBDBDCE0100030003020133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The length of the text is 3 bytes

030100:Text: 03--Bluetooth beacon 01--GPS 00--No positioning Positioning priority is Bluetooth beacon>GPS

33:checksum

Single GPS positioning:BDBDBDBDCE0100030001000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The length of the text is 3 bytes

030201:Text: 01--GPS 00--No positioning 00--No positioning Positioning priority is single GPS positioning

33:checksum

Single Bluetooth beacon positioning:BDBDBDBDCE0100030003000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The length of the text is 3 bytes

020000:Text: 03--Bluetooth beacon 00--No positioning 00--No positioning Positioning priority is single Bluetooth beacon positioning

33:checksum

5.2 Server time synchronization information

5.2.1 Device reports request time calibration data

Description: This message is reported by the device

After the device is powered on, it will automatically send a request time synchronization command. After receiving the synchronization command, it is necessary to reply with the synchronized time data frame for device time synchronization
If the device has been calibrated with downlink time before shutting down, then after the device restarts, the device interface displays the time before shutting down, and the device will report FF00FF to re-request time calibration

| Byte offset | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | U8 | HEADER | Fill: 0xFF |
| 1 | U8 | SeqId | Padding: 0x00 |
| 1 | U8 | End | Fill: 0xFF |

Example:

FF00FF //The server needs to reply immediately after receiving it, without any delay

5.2.2 Time Calibration Request Data Reply

| Byte offset | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | U8 | HEADER | Fill: 0xFF |
| 1 | U8 | SeqId | Padding: 0x10 |
| 2 | U16 | years | Year data |
| 1 | U8 | month | Month data |
| 1 | U8 | Day | Day, data |
| 1 | U8 | time | Clock, data |
| 1 | U8 | Minute | Minutes, data |
| 1 | U8 | Seconds | Seconds, data |
| 1 | U8 | End | Fill: 0xFF |

Example:

After the server receives FF00FF, it replies with the downlink command to calibrate the time. Note: The example is for 2020 and only serves as a structural analysis example. The current year needs to be issued, otherwise it will not be parsed. FF1007E409020B1B28FF

FF : Header

10 : Seqid

07E4: Year (2020)

09: Month, (09)

02: Day, (02)

0B: Hour, (11)

1B: Minutes, (27)

28: Second, (40)

FF: End