W200PLU-Lorawan-Communication Protocol

Catalog

1 Overview	3

2 Device Usage Instructions	3

2.1 Device Functions and Usage Instructions	3

2.2 Default Reporting Logic for Devices	4

2.3 Device downlink description	5

3 Protocol Packet Structure	7

3.1 Data Header	7

3.2 Message Identifier (Message ID)	7

3.3 Payload	8

3.5 Checksum	9

4 Reporting messages	10

4.1 Alarm-related reporting	10

4.1.1 Alarm data upload-1 (MSGID=0x02)	10

4.1.2 Alarm data upload-2 (MSGID=0x21) (Supplement to 02)	13

4.1.3 SOS Upload (MSGID=0XB5)	17

4.2 Location-related Reporting	18

4.2.1 GPS/BDS Position Report: Location Data Reporting (MSGID=0x03)	18

4.2.2 Bluetooth Location Information (LBE Location) (MsgId=0xD6)	19

4.2.3 UWB Location Information (UWB Location) (MsgId=0xD7)	22

4.3 Device information and status reporting	23

4.3.1 Device Charging Status Upload (MSGID=0xC3)	24

4.3.2 Power Signal (MSGID=0xF9)	25

4.4 Health-related reporting	27

4.4.1 Health data (MSGID=0x32)	27

4.5 Downlink feedback related report	29

4.5.1 Downlink feedback (MSGID=0xC0)—-No general version available	29

4.5.2 Message Status Report (MSGID=0x28) — General version not available	30

5 Settings	32

5.1 Downlink	32

5.1.1 Set positioning reporting frequency (MSGID=0x17)	32

5.1.2 Information Dispatch (Message Send) (MSGID=0X28)	34

5.1.3 Settings - Location Priority Setting (0XCE01)	36

5.1.4 Settings - Health Sample Reporting Frequency Settings (0XCE02)	38

5.1.5 Settings - Switch Settings (0XCE04-24)	39

5.1.6 Issuing Sit-and-Stay Alarm Trigger Time (MSGID=0XCC)	41

5.1.7 Shutdown Restart (0x77)	42

5.1.8 UWB Distance Measurement Alarm Configuration (0x7901)	43

5.2 Server Time Synchronization Information	43

5.2.1 Device Report Request Time Calibration Data	43

5.2.2 Time Calibration Request Data Response	44

1 Overview

This protocol is suitable for standard LoRaWAN protocol devices and currently supports W200PLU products. It uses a 32-bit data header for synchronization and terminal identification; employs a low-overhead checksum algorithm for verification protection; and utilizes message identifiers to distinguish different messages.  
    Parsing must be conducted according to the actual report, and the document is currently under continuous maintenance. If any errors or omissions are found, please provide timely feedback. Thank you very much.

2 Device Usage Instructions

2.1 Device Functions and Usage Instructions

General version: The default high-frequency general is AS923 (923.2-924.6), and the low-frequency general is CN470 (470.3-471.7) OTAA-CLASSA. If other frequency bands are needed, please submit your request.

Network access related keys:

OTAA:

Application key: 2B7E151628AED2A6ABF7158809CF4F3C

ABP:

Deviceaddress: last eight digits of the watch DEVEUI

Applicationsession key: EF6D6E2503F57AE2FA151CDA87455F18

Networksession key:2E8C8650B4041672BBB9A399F2DEB427

(1) Power On

Please fully charge before the first use. Once the device reaches the required power for startup, it will automatically power on. The charging status shows a charging icon,a fully charged status shows a green icon. Note: Do not check the device signal while charging.

Manual power on: Long press the upper button for 10 seconds and then release; the interface will display the word "Welcome." Note: The default state when powered on is wearing mode, and if no heart rate is detected, an alert for removal will be reported.

(2) Power Off

Low battery power off: The screen will extinguish after displaying "Byebye."

Manual power off: Long press the button for more than 10 seconds, the interface will display "Byebye" and then the screen will extinguish.

Note: The device does not power off while charging during the power-on state.

(3) SOS Function

Trigger method: After triggering, the device will not enter sleep mode. Long press the lower button for 3 seconds, and the interface will display "SOS SEND OK/SOS sent successfully," followed by the appearance of the SOS icon.

Cancel method: In SOS mode, long press the lower button for 3 seconds, and the interface will display "SEND CANCEL/SOS canceled," after which the SOS icon will disappear.

(4) Signal Status

No Signal: The interface signal is represented by a horizontal line, indicating that the device has failed to report or is not connected (out of the Lora gateway range).

Signal Present: The interface signal shows a stepped bar graph.

(5) Device Sleep

Trigger condition: If the device remains stationary for 40 minutes, it enters sleep mode and does not report location health data.

2.2 Default Reporting Logic for Devices

General version: Reporting must be within the Lora gateway range; otherwise, reporting fails. The default interval for failing to report is 5 minutes before rejoining the network.

(1) Location-Related Reporting

UWB/gps/Bluetooth beacon: Default reporting frequency is 10 minutes. Default location priority: UWB > Bluetooth > GPS.

UWB is prioritized; if UWB fails, it switches to Bluetooth positioning. If Bluetooth fails, it switches to GPS positioning. Successful positioning does not switch.

(3) Alarm-Related Reporting

SOS Alarm (0 xB5): Triggered actively by the user; the triggering method is detailed in the previous section.

SOS Cancel (0 x02): Triggered actively by the user to cancel; the triggering method is detailed in the previous section.

Shutdown Alarm (0 x21): The device actively shuts down or shuts down due to low battery; the trigger method is described in the previous section.

Wear Detachment Alarm (0 x02): The device determines wear based on the frequency of health sampling reports. If heart rate is detected, it reports wear; if no heart rate is detected, it reports detachment.

Sitting Duration Alarm (0 x02): Default triggers a report if there has been no movement for 15 minutes.

Fall Alarm (0 x02): The device falls freely from a certain height, meeting the criteria of the fall algorithm to trigger.

Low Battery Alarm (0 x02): Triggered when the current battery level is less than or equal to 0.

UWB Distance Measuring Alarm (0 x21): Default is enabled, all beacons will trigger the alarm. After setting the UWB distance alarm beacon, it is triggered when the device is less than the specified UWB alarm distance.

Vibration is triggered in real time; the wear status reports to the server only once within 1 minute. If not worn, it does not report to the server and does not produce vibrations.

(4) Health-related Reports

Steps, Heart Rate, Body Temperature & Wrist Temperature, Blood Pressure, Blood Oxygen (0 x32): Default reporting frequency is 10 minutes.

(5) Device Information and Status Reporting

Charging status (0 xC3): Start charging, stop charging, report when fully charged

Power signal (0 xF9): One report at startup, and another report will follow during positioning and health reporting.

(6) Time synchronization request report

Time synchronization request (FF00FF): The device will report once at startup, requesting the server to send commands to calibrate the time.

Note: The device may report in a bundled manner, meaning that one data packet contains multiple complete messages. Be careful not to miss any, as the messages are complete and will not be interrupted and continued in the next data packet.

Eg: bdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9bdf9010000006400002800000019a9cf61ca

This data packet contains the Bluetooth positioning message (0xD6) and the power signal message (0xF9).

(0xD6) Bluetooth positioning: bdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9

(0xF9) Power signal: bdf9010000006400002800000019a9cf61ca

2.3 Device downlink description

General version:

(1) Equipment positioning report frequency issued (0x17)

Default reporting frequency is 10 minutes, maximum is 1 minute, minimum is 1440 minutes. After receiving the downlink command, the device reports data based on the command's timing and frequency.

Outside the specified time range, it reports at the default frequency. For example: from 00:00 to 18:00, it will report positioning every 2 minutes;

thus, outside this time range, it will report at the default frequency of 10 minutes.

(2) Text message issued (0x28)

Unicode encoding, up to 20 Chinese characters, each Chinese character occupies 2 bytes, each English letter occupies 2 bytes.

Previous low-frequency firmware version used: GB2312 encoding, up to 20 Chinese characters, each Chinese character occupies 2 bytes, each English letter occupies 1 byte.

Message list - up to 50 historical messages can be saved (Note: firmware without a message list can save a maximum of 20 messages).

(3) Device positioning priority issued (0xCE01)

The default positioning priority is UWB > Bluetooth > GPS.

The priority for localization is as follows: gps > Bluetooth beacon, gps localization is preferred, switch if gps localization is unavailable.

When the Bluetooth beacon successfully locates, it will not switch to the next positioning priority to generate a position.

(4) Device health sampling frequency issued (0xCE02)

Health sampling frequency defaults to 10 minutes, with a maximum report interval of 2 minutes.

(5) Sedentary alert trigger time issued (0xCC)

Value range: 2 minutes to 60 minutes. Note that this will only be effective if the sedentary alert switch is turned on.

For example: if the downstream is set to a 10-minute trigger time, then if the device remains still for 10 minutes, it will trigger and report a sedentary alert.

(6) Device shutdown and restart issued (0x77)

Note that this command must be issued when the device is powered on; it cannot receive commands after shutdown.

Shutdown issued: Send this command while the device is powered on, and the device will shut down. It cannot receive commands after shutdown.

Restart issued: Send this command while the device is powered on, and the device will restart.

(7) Sedentary Alert Switch (0xCE08)

Default is off, the device does not report sedentary alerts. When enabled, the default trigger time is 15 minutes; inactivity for 15 minutes triggers the sedentary alert.

(8) Drop Alert Switch (0xCE07)

Default is off for drop alerts. When enabled, the device reports a drop alert after detecting a fall.

(10) Calibration Time Command Downlink (0XFF)

When the device reports a time calibration request message, this command is immediately sent downlink for watch time calibration.

3 Protocol Packet Structure

A basic protocol packet structure is shown in Figure 1:

3.1 Data Header

Each packet begins with a 1-byte Header (except for the request calibration time message: ff00ff):

Currently, the uploaded equipment Header is fixed as BD (excluding request calibration time messages: ff00ff)

Header: 0xBD；

3.2 Message Identifier (Message ID)

Contents represented by MessageId can be found in Chapter 4.

3.3 Payload

The payload we refer to below is the valid body content in the protocol, excluding the header and checksum. The following notation indicates the length of the body.

The data formats used in the payload are shown in the table below:

[U-unsigned; I-signed; X-bitfield; Number - number of bytes occupied]

In the following protocol, all data types except ch, u8, i8, and x8 use little-endian format.

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Description |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned short integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short Integer |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Integer |
| x16 | Bitfield | 2 | - | - | Bit (bit) 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long Integer |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long Integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit Long Integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating Point |

3.5 Checksum

The contents included in the checksum are the payload, as shown in Figure 1. The algorithm is as follows; by default, checksum validation is not strictly required for general version devices and can be ignored. For downstream commands, any single byte will suffice.

// Assuming content="BDE9000600010A00000A00", the result of sum.ToString("X2") will be 07// The following is the C# code calling method

private string CheckSum(string content)

{

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2");

}

4 Reporting messages

4.1 Alarm-related reporting

4.1.1 Alarm data upload-1 (MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0x02 | See below for definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below 见下方定义 |
| 4 | U32 | Timestamp | - | - | Timestamp (when resending, a timestamp will be added at the end) |

Upl_warn contents

| bit (corresponding to the position of 1 in the binary) | Name | Description | Raw message in little-endian hexadecimal | Hexadecimal to Big Endian | Decimal |
| --- | --- | --- | --- | --- | --- |
| 14 | Drop Alarm | Drop Alarm | 0040 | 4000 | 4*4096=16384 |
| 8 | Device Wear | Device Wear | 0001 | 0100 | 1*256=256 |
| 7 | SOS Cancel | SOS Cancel | 8000 | 0080 | 8*16=128 |
| 5 | Prolonged Sitting Alarm (Inactive) | Prolonged Sitting Alarm (Inactive) | 2000 | 0020 | 2*16=32 |
| 4 | Remove (fall off) equipment | Remove (fall off) equipment | 1000 | 0010 | 1*16=16 |
| 2 | Shut down | Shut down | 0400 | 0004 | 4 |
| 0 | Low battery | Low battery | 0100 | 0001 | 1 |

Example:

Wear alert: BD020001E377BD678A

BD: Header

02: Message ID

0001: Upl_warn alert content, converted to big-endian:0100--> Wear alert

E377BD67: Timestamp converted to big-endian 67BD77E3-->Convert to decimal1740470243--> Timestamp is1740470243seconds-->

Converted to standard time format UTC:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

Drop Alarm: BD021000E377BD678A

BD: Header

02: Message ID

1000: Upl_warn alarm content, converted to big-endian0010--> Drop Alarm

E377BD67: Timestamp converted to big-endian 67BD77E3-->Convert to decimal1740470243--> Timestamp is1740470243seconds-->

Converted to standard time format UTC:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

SOS Cancel: BD028000E377BD678A

BD: Header

02: Message ID

8000: Upl_warn alarm content, converted to big-endian0080--> SOS Cancel

E377BD67: Timestamp converted to big-endian 67BD77E3-->Convert to decimal1740470243--> Timestamp is1740470243seconds-->

Converted to standard time format UTC:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

Fall Alarm: BD020040E377BD678A

BD: Header

02: Message ID

0040: Upl_warn alarm content, converted to big-endian4000--> Fall Alarm

E377BD67: Timestamp converted to big-endian 67BD77E3-->Convert to decimal1740470243--> Timestamp is1740470243seconds-->

Converted to standard time format UTC:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

E7: checksum

Sitting Alarm: BD022000E377BD678A

BD: Header

02: Message ID

2000: Upl_warn alarm content, converted to big-endian0020--> Sitting Alarm

E377BD67: Timestamp converted to big-endian 67BD77E3-->Convert to decimal1740470243--> Timestamp is1740470243seconds-->

Converted to standard time format UTC:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

8 A: checksum

Low battery alarm: BD020200E377BD678A

BD: Header

02: Message ID

0100:Upl_warn alarm content, converted to big-endian0001--> Low battery alarm

E377BD67: Timestamp converted to big-endian 67BD77E3-->Convert to decimal1740470243--> Timestamp is1740470243seconds-->

Converted to standard time format UTC:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

8 A: checksum

Shutdown alarm: BD020400E377BD678A

BD: Header

02: Message ID

0400:Upl_warn alarm content, converted to big-endian0004--> Shutdown alarm

E377BD67: Timestamp converted to big-endian 67BD77E3-->Convert to decimal1740470243--> Timestamp is1740470243seconds-->

Converted to standard time format UTC:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

8 A: checksum

Note: This alarm is used for devices that do not distinguish shutdown types; devices that distinguish shutdown types are detailed in the next section

4.1.2 Alarm data upload-2 (MSGID=0x21) (Supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0x21 | See below for definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | 报警类型 |
| N |  | Upl_warn | - | - | Alarm content See below for different Alarm type definitions |
| 4 | U32 | Timestamp |  |  | Timestamp (when resending, a timestamp will be added at the end) |

Alarm type=1

Upl_warn contents

| bit (corresponding to the position of 1 in the binary) | Name | Description | Raw message in little-endian hexadecimal | Hexadecimal to Big Endian | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging shutdown alarm | Device charging automatic shutdown (firmware must support this feature) | 04000000 | 0004 | 4 |
| 1 | Low battery shutdown alarm | Device shuts down due to low battery (firmware must support this feature) | 02000000 | 0002 | 2 |
| 0 | Manual shutdown alarm | Device has been manually shut down (firmware must support this feature) | 01000000 | 0001 | 1 |

Example:

Charging shutdown alarm: BD21010004000000E377BD6767

BD:Header

21 : Message ID

0100:Alarm type Alarm type converted to big-endian 0001-->Alarm type=1

04000000:Upl_warn Alarm content, converted to big-endian 0004-->Charging shutdown alarm

E377BD67:Timestamp Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Low battery shutdown alarm: BD21010002000000E377BD6767

BD:Header

21 : Message ID

0100:Alarm type Alarm type converted to big-endian 0001-->Alarm type=1

02000000:Upl_warn Alarm content, converted to big-endian 0002-->Low battery shutdown alarm

E377BD67:Timestamp Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Active shutdown alarm: BD21010001000000E377BD6767

BD:Header

21 : Message ID

0100:Alarm type Alarm type converted to big-endian 0001-->Alarm type=1

01000000:Upl_warn Alarm content, converted to big-endian 0001-->Active shutdown alarm

E377BD67:Timestamp Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Alarm type=4

Upl_warn contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | 02—UWB Distance Alarm |
| U8 | Total_groups | 1 | Total number of groups; there may be multiple sets of information, and each group may contain multiple Ubeacons |
| U8 | Total_PackCount | 1 | Total number of alarm Ubeacons at the current time |
| U32 | UBeacon0 mac | 4 | UBeacon mac |
| U16 | Distance0 | 2 | Distance (unit: centimeters) |
| U32 | UBeacon1 mac | 4 | UBeacon mac |
| U16 | Distance1 | 2 | Distance (unit: centimeters) |
| U32 | UBeacon2 mac | 4 | UBeacon mac |
| U16 | Distance2 | 2 | Distance (unit: centimeters) |
| 。。。。。 | 。。。。。 | 。。。。。 | 。。。。。 |

Example:

Original 16-bit hex message: BD210400020103320600009100E70500006800380500005C00E377BD6755

BD: Header

21: Message ID

0400: Alarm type, converted to big-endian 0004 --> Alarm type=4

02: Type, converted to decimal 02 -- UWB distance measurement alarm

01:Total_groups Total number of groups Converted to decimal 01, 1 group

03:Total_PackCount Total number of Ubeacons at the current time Converted to decimal 03, 3 UWB beacons

32060000:UBeacon0 mac Converted to big-endian 00000632, UWB beacon id is 00000632

9100:Distance0 Distance Converted to big-endian 0091 --> converted to decimal 145 --> the distance to the UWB beacon is 145 centimeters

E7050000:UBeacon1 mac

6800:Distance1 Distance Converted to big-endian 0068 --> converted to decimal 105 --> the distance to the UWB beacon is 105 centimeters

38050000:UBeacon2 mac

5C00:Distance2 Distance Converted to big-endian 005C --> converted to decimal 92 --> the distance to the UWB beacon is 92 centimeters

E377BD67:Timestamp Timestamp Converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> the timestamp is 1740470243 seconds

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

55:checksum

4.1.3 SOS Upload (MSGID=0XB5)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0xB5 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Description |
| --- | --- | --- | --- |
| 1 | U8 | Status | Status: 1: SOS |
| 4 | Int32 | timestamp | Timestamp, little-endian (timestamp little Endian) |

Example:

Hex raw message: BDB501E377BD6755

BD: Header

B5: Message ID

01:Status 01--SOS

E377BD67:Timestamp Timestamp Converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> the timestamp is 1740470243 seconds

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

55:checksum

4.2 Location-related Reporting

4.2.1 GPS/BDS Position Report: Location Data Reporting (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0x16 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude |
| 8 | Double | lat |  | - | latitude |
| 1 | U8 | north_south |  | - | /N or S/ North or South Latitude |
| 1 | U8 | east_west |  | - | /E or W/ East or West Longitude |
| 1 | U8 | status |  | - | /A or V/ Valid Data or Invalid Data |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original 16 Hex Message: BD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BD: Header

03 : Message ID

963E870 C3E675E40: lon Longitude converted to Big Endian 405E673 E0C873E96 --> Longitude: 121.6131622 Note: See the parse example below for conversion reference

FF2110C2B6343F40:lat dimension  Convert to big-endian `    403    F34B6C21021FF-->Latitude:    31.2059137      Note: Conversion parsing refers to the example below

4    E:north_south    4    E-->Convert to ASCII code N-->North latitude

45    :east_west    45    -->Convert to ASCII code E-->East longitude

41    :status data validity    41    -->Convert to ASCII code A-->Valid data

E377BD67:Timestamp timestamp  Convert to big-endian    67    BD77E3-->Convert    10    Decimal    1740470243    -->Timestamp is    1740470243    seconds-->

Converted to standard time format UTC:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

78    :checksum

Note: The parsed GPS coordinates are in WGS-84 coordinate system. If the map uses Baidu or Gaode, the coordinate system needs to be converted.

Example of satellite positioning message parsing in Java:

//Message: BD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public     static     void     main (String[] args){

//Message 7d9f84ac81815c40 Actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d", 16)));  //114.02353966666665//Message e766926b1d893640 actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16)));//22.535605166666667

HexToStr(data.Substring("4e"));   //N

HexToStr(data.Substring("45"));//E

HexToStr(data.Substring("41"));//A   A indicates valid data, V indicates invalid data and can be discarded//Message 749d695f actual value 5f699d74

Date date=new Date();

date.setTime(Long.parseLong("5f699d74", 16)*1000);

SimpleDateFormatsdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.2.2 Bluetooth Location Information (LBE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0xD6 | See below for definition | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 0 |
| U8 | Total_groups | 1 | Total number of groups, which may contain multiple pieces of information, each group may have multiple iBeacons |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth Beacon Signal Strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth Beacon Signal Strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (One of the Bluetooth Beacon Identification Codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth Beacon Signal Strength) |
| U16 | MajorN | 2 | Major (One of the Bluetooth Beacon Identification Codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth Beacon Signal Strength) |
| ... | ... | ... | ... |

Note: In the universal version, the total number of Bluetooth beacons is fixed at 1, and the maximum number of Bluetooth beacons is 4. They are sorted by signal strength from highest to lowest. By default, the watch only recognizes our own beacons. If you need to connect to beacons from other brands, please consult relevant personnel via WeChat.  
Example:

Raw hexadecimal message: BDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

BD: Header

D6: Message ID

00: type

01: Only one set of ibeacon data

E377BD67:Timestamp Timestamp Converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> the timestamp is 1740470243 seconds

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

04: The first group has 4 beacon information

4327: Convert to big-endian --> major : 0x2743 --> Convert to decimal 10051

1794: Convert to big-endian --> minor: 0x9417 --> Convert to decimal 37911

AC:  RSSI 0xAC --> Convert to decimal two's complement -84

4327: Convert to big-endian --> major: 0x2743 --> Convert to decimal 10051

3094: Convert to big-endian --> minor: 0x9430 --> Convert to decimal 37936

AA:  RSSI 0xAA --> Convert to decimal two's complement -86

4327:Convert to major-->0x2743-->Convert to decimal 10051

b956:Big-endian conversion-->minor:0x56b9-->Decimal conversion 22201

A5:   RSSI 0xA5-->Decimal conversion two's complement -91

4327: Convert to big-endian --> major: 0x2743 --> Convert to decimal 10051

FE94: Big-endian conversion-->minor:0x94FE-->Decimal conversion 38142

A5: rssi:0xA5-->Decimal conversion two's complement -91

6a --Checksum

Bluetooth beacon major and minor description: The default Bluetooth beacon is labeled with a beacon code,

for example: 00081005345866, so the Bluetooth beacon's major is 10053, minor is 45866, and the manufacturer identifier is 0008 (which has no practical use or significance).

4.2.3 UWB Location Information (UWB Location) (MsgId=0xD7)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0xD7 | See below for definition | CK_sum |

Payload contents

| Format() | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 01 |
| U8 | Total_groups | 1 | Total number of groups; there may be multiple sets of information, and each group may contain multiple Ubeacons |
| Int32 | Timestamp | 4 | Utc timestamp |
| U8 | Total_PackCount | 1 | Total number of Ubeacons at the current time |
| U32 | UBeacon0 mac | 4 | UBeacon devid (little-endian) |
| U16 | Distance0 | 2 | Distance (cm) |
| U32 | UBeacon1 mac | 4 | UBeacon devid (little-endian) |
| U16 | Distance1 | 2 | Distance (cm) |
| U32 | UBeacon2 mac | 4 | UBeacon devid (little-endian) |
| U16 | Distance2 | 2 | Distance (cm) |
| ... | ... | ... | ... |

Note: The W200PLU universal version can report a maximum of 1 group with up to 4 Ubeacons

Example:

Raw 16 hexadecimal message: BDD70101E377BD6703320600009100E70500006800380500005C00A1

BD: Header

D7: Message ID

01: Type 01

01: Total_groups Total number of groups, converted to decimal01-->The total number of groups is1

E377BD67: Timestamp converted to big-endian 67 BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds-->

Converted to standard time format UTC:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

03: Total_PackCount number of UWB beacons converted to decimal 03 --> number of UWB beacons is 3

32060000: UBeacon0 mac UWB beacon id converted to big-endian 00000632 --> UWB beacon id is 00000632

6800: Distance0 distance measurement converted to big-endian 0091 --> converted to decimal 145 --> distance measurement is 145 cm

E7050000: UBeacon1 mac UWB beacon id converted to big-endian 000005E7 --> UWB beacon id is 000005E7

9100: Distance1 distance measurement converted to big-endian 0068 --> converted to decimal 104 --> distance measurement is 104 cm

38050000: UBeacon2 mac UWB beacon id converted to big-endian 00000538 --> UWB beacon id is 00000538

5 C00: Distance2 distance measurement converted to big-endian 005 C --> converted to decimal 92 --> distance measurement is 92 cm

A1: checksum

4.3 Device information and status reporting

4.3.1 Device Charging Status Upload (MSGID=0xC3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0xC3 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Status | / | / | 0 Start, 1 End, 2 Fully Charged |
| 1 | U32 | Timestamp | / | / | Timestamp (when resending, a timestamp will be added at the end) |

Note: This message requires firmware support from the device, and the latest universal version is supported by default.<br>Example:

Raw Hexadecimal Message: BDC301E377BD678A

BD: Header

C3: Message ID

01: Device has finished charging

E377BD67:Timestamp Timestamp Converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> the timestamp is 1740470243 seconds

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A: Checksum

Raw Hexadecimal Message: BDC300DB4D2F668A

BD: Header

C3: Message ID

00: Device has started charging

E377BD67:Timestamp Timestamp Converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> the timestamp is 1740470243 seconds

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A: Checksum

Original Hexadecimal Message: BDC302DB4D2F668A

BD: Header

C3: Message ID

02:The device is fully charged

E377BD67:Timestamp Timestamp Converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> the timestamp is 1740470243 seconds

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A: Checksum

4.3.2 Power Signal (MSGID=0xF9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0xF9 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery Type 0: 4-level system, 1: 5-level system, 2: Percentage, 3: Voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery Value If Bat_type is 0: the battery value range is 0-3 (0 represents 25%, 3 represents 100%) If Bat_type is 1: the battery value range is 0-4 (0 represents 20%, 4 represents 100%) If Bat_type is 2: the battery value range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal Type 0: Percentage, 1: 5-level system, 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal Strength |
| 1 | U8 | Other_type |  |  | Extended Type 0: Full Step Count 1: Incremental Step Count 2: Vibration |
| 4 | U32 | Num |  |  | Extended Value |
| 4 | U32 | Timestamp | -/- | -/- | UTC Timestamp (in seconds) |

Example:

Raw 16 Hexadecimal Message: BDF90104000050000095000000E377BD67AA

BD: Header

F9:Message ID

01:Battery_type Battery type 01-->1-->5 Levels of power (0-4)

0400:Battery_volt Battery value converted to big-endian 0004-->4--> Level 4 corresponds to 100% battery

00:Signal_type Signal type 00-->0--> Percentage

5000:Signal_strength Signal strength converted to big-endian0050--> Converted to decimal80-->80% signal strength

00:Other_type Extended type 00-->0--> Full step count

95000000:Num Extended value - step count converted to big-endian00000095--> Converted to decimal149--> Step count is149

E377BD67:Timestamp Timestamp converted to big-endian67BD77E3--> Converted to decimal1740470243--> Timestamp is1740470243seconds-->

Converted to standard time format UTC:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

AA:checksum

4.4 Health-related reporting

4.4.1 Health data (MSGID=0x32)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0x32 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | 00 |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 2 | U16 | contentLength |  |  | Total length of the content that follows |
| 1 | U8 | ID |  |  | Id (type + reported value length), ID type definitions are specified below |
| N | N*U8 | Val1 |  |  | Reported value of Id (the byte length varies according to the reported value length) |
| ... | ... | ... | ... | ... | ... |
| 1 | U8 | ID |  |  | Id (type + reported value length), ID type definitions are specified below |
| N | N*U8 | Val1 |  |  | Reported value of Id (the byte length varies according to the reported value length) |

ID type definitions (hexadecimal):

01    Step Count    02    Heart Rate    03    Body Temperature    04    Wrist Temperature    06    Diastolic Pressure    07    Systolic Pressure    08    Blood Oxygen    09     HRV (special version support)

(    05    Blood Sugar    10    Number of Sit-ups    11    Running Pace and Distance    12    Jump Rope Speed and Count )    ---    Reserved, not available

Default universal version support:01 Steps02 Heart Rate03 Body Temperature04 Wrist Temperature06 Diastolic Pressure07 Systolic Pressure08 Blood Oxygen

Example:

Raw 16 hexadecimal message: BD3200E377BD6711000A1E00114B314A39711A4A0122bc00416212

BD: Header

32:Message ID

00:typetype=00

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to decimal1740470243-->timestamp is1740470243seconds-->

Converted to standard time format UTC:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

1100:Length of the following message converted to big-endian-->0011-->converted to decimal17, the length of the following message is17bytes

0A:ID  0x0A-->converted to binary00001010First five bits are00001-->converted to hexadecimal01, represents data ID Steps (0x01),

Last three bits are010-->converted to decimal02, represents the length of step data as2bytes

1E00:Step reporting value converted to big-endian-->001E-->converted to decimal30, a total of30steps of step data

11:ID  0x11-->converted to binary00010001First five bits are00010-->converted to hexadecimal02, represents data ID Heart Rate (0x02),

The last three digits are    001    --> convert to decimal    1, which represents that the heart rate data length is    1    byte

4B: The reported heart rate value converts to decimal-->    75, the heart rate data is    75

31:ID  0x31--> convert to binary    00110001    The first five digits are    00110    --> convert to hexadecimal    06, which represents data ID for diastolic pressure (    0x06    ),

The last three digits are    001    --> convert to decimal    1, which represents that the diastolic pressure data length is    1    byte

4A: Reported diastolic pressure value, converts to decimal-->    74, the diastolic pressure data is    74

39:ID  0x3--> convert to binary    00111001    The first five digits are    00111    --> convert to hexadecimal    07, which represents data ID for systolic pressure (    0x07    ),

The last three digits are    001    --> convert to decimal    1, which represents that the systolic pressure data length is    1    byte

71: Reported systolic pressure value, converts to decimal-->    113, the systolic pressure data is    113

1A:ID  0x1A--> convert to binary    00011010    The first five digits are    00011    --> convert to hexadecimal    03, which represents data ID for body temperature (    0x03    ),

The last three digits are    010    --> convert to decimal    2, which represents that the body temperature data length is    2    bytes

4A01: Body temperature reporting value, converted to big-end-->014A--> convert to decimal330-->330*0.1--> body temperature data is33degrees Celsius

22:ID  0x22-->Convert to binary00100010   The first five bits are00100-->Convert to16hexadecimal04, representing data ID wrist temperature (0x04),

The last three bits are010-->Convert to decimal2, representing the length of the wrist temperature data as2bytes

BC00: Wrist temperature reported value, convert to big-endian-->00BC-->Convert to decimal188-->188*0.1-->Wrist temperature data is18.8degrees Celsius

41:ID  0x41-->Convert to binary01000001    The first five bits are01000-->Convert to16hexadecimal08, code data ID blood oxygen (0x08),

The last three bits are001-->Convert to decimal1, representing the length of the blood oxygen data as1byte

62:Blood oxygen reported value, convert to decimal-->98, blood oxygen data is98

12:checksum

4.5 Downlink feedback related report

4.5.1 Downlink feedback (MSGID=0xC0)—-No general version available

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0xC0 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This instruction is used for feedback of downlink commands, returning the Message ID of the downlink commands received earlier (multiple Message IDs can be returned collectively). Some downlink commands do not have C0 feedback.

Raw hexadecimal message: BDC0011720

BD: Header

C0:Message ID

01:length Message ID length converted to decimal 01 --> Message ID length is 1 byte

17:Type indicates that the device received the downlink command Message ID as 17

20:checksum

4.5.2 Message Status Report (MSGID=0x28) — General version not available

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0x28 | See below for definition | CK_sum |

Payload contents

| Bytes | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | timestamp | -/- | -/- | Timestamp |
| 1 | U8 | Type | -/- | -/- | Character encoding type (corresponding to the following 0x28), GB2312 encoding: 0x03; Unicode encoding: 0x05 |
| 1 | U8 | status | -/- | -/- | Message status - 0x01: Confirmed, 0x02: Rejected |
| 4 | U32 | seqID | -/- | -/- | (corresponding to the message id seqID for the following 0x28) |

Example:

Note: The current universal version does not differentiate character encoding types, fixed at 03; future versions will.

Raw hexadecimal message: BD28FD3BA068030268A03BE477

BD: Header

28:Message ID

FD3BA068:Timestamp converted to big-endian 68A03BFD --> converted to decimal --> timestamp is 1755331581 seconds -->

Convert to standard time format UTC: 2025-08-16 08:06:21 --> Convert to Beijing time: 2025-08-16 16:06:21

03: Type 03 -- Fixed value

02: status message status 02 -- Interface click reject button -- Rejected

68A03BE4: seqID The ID of the downlink message is 68A03BE4

77: checksum

5 Settings

5.1 Downlink

5.1.1 Set positioning reporting frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0x17 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 means enabled, 0 means disabled | Interval 1 |
| 2 | U16 | Interval |  |  | Time Interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 means enabled, 0 means disabled | Interval 2 |
| 2 | U16 | Interval |  |  | Time Interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 means enabled, 0 means disabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time Interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 means enabled, 0 means disabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time Interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |

Note: One day is from 00:00 to 23:59<br>Example:

00 : 00 - 23 : 59 (All day) Location data is reported every 10 minutes: BD 17010 A 000000173 B 00000000000000000000000000000000000000000097

BD : Header

1C : Message ID

01 : enable (是否启用) Convert to decimal 01 --> 1 -- Enabled

0A00 : Interval (时间间隔) Convert to big-endian 000 A --> Convert to decimal 10 --> Interval is 10 minutes

00 : time_start_h (开始时间-小时), Convert to decimal 00 --> Start time - hour is 00

00 : time_start_m (开始时间-分钟), Convert to decimal 00 --> Start time - minute is 00

17:time_end_h End time - hour, converted to decimal23--> End time - hour is23

3B:time_end_m End time - minutes, converted to decimal59--> End time - minutes is59

00:enable Whether to enable, converted to decimal00-->0-- Not enabled

000000000000: Not enabled status fully populated00

00:enable Whether to enable, converted to decimal00-->0-- Not enabled

000000000000: Not enabled status fully populated00

00:enable Whether to enable, converted to decimal00-->0-- Not enabled

000000000000: Not enabled status fully populated00

97:checksum

5.1.2 Information Dispatch (Message Send) (MSGID=0X28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0x28 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Description |
| --- | --- | --- | --- |
| 1 | U8 | type | Message type, if it is a downlink text message, Unicode encoding is 05, GB2312 encoding is 03 |
| 4 | Uint32 | seqID | Message ID, uniqueness |
| 1 | U8 | CONTENT LEN | Content Length |
| N | N | CONTENT | Content, Unicode encoding (little-endian, note conversion, one Chinese character occupies 2 bytes, one letter occupies 2 bytes) GB2312 encoding (little-endian, one Chinese character occupies 2 bytes, one letter occupies 1 byte) |

Note: The default latest common version of Chinese character encoding is Unicode, while earlier low-frequency firmware versions use GB2312 encoding
Each information ID must be unique; devices cannot receive duplicate IDs
Example:

Unicode EncodingText Message: hello, world  BD28050300000016680065006C006C006F002C0077006F0072006C006400DD

BD: Header

28: Message ID

05: Type Message Type Decimal conversion 05-->type=05

03000000: seqID Information ID  Information ID is 03000000

16: CONTENT LEN Content Length Decimal conversion 22, content length is 22 bytes

680065006C006C006F002C0077006F0072006C006400:

Convert every 2 bytes to big-endian --> Convert to Unicode characters --> Combine into final text message content: hello, world

DD:checksum

Previously, low-frequency version firmware used GB2312 encoding:Text message: Hello WorldBD28036895D13F08C4E3BAC3CAC0BDE779

BD: Header

28:Message ID

03:type message type Convert to decimal 03-->type=03

6895D13F: seqID information id  Information id is 6895D13F

08:CONTENT LEN length of content Convert to decimal 8

C4E3BAC3CAC0BDE7:Convert to string Hello World-->Text message content: Hello World

79:checksum

5.1.3 Settings - Location Priority Setting (0XCE01)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

Type=01 Location priority setting

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 effective this time |
| 2 | U16 | Len | -/- | -/- | The length of the subsequent command |
| n | n |  | -/- | -/- | Body |

Positioning priority setting definition -- refer to 2.3 Device Downlink Description -- Device positioning priority distribution (0xCE01)

Positioning priority setting body: Currently supported common versions: 01--GPS, 03--Bluetooth beacon (additional deployment of Bluetooth beacon required);

Example:

GPS positioning priority (GPS > Bluetooth beacon): BDCE0100030001030033

BD: Header

CE: Message ID

01:Type 01 Priority Setting

Valid consistency 00 remains valid

Len converted to big-endian 0003 --> converted to decimal 03 --> payload length is 3 bytes

Payload: 01--GPS 03--Bluetooth beacon positioning priority is GPS > Bluetooth beacon

checksum

Bluetooth positioning priority (Bluetooth beacon > GPS): BDCE0100030003010033

BD: Header

CE: Message ID

01:Type 01 Priority Setting

Valid consistency 00 remains valid

Len converted to big-endian 0003 --> converted to decimal 03 --> payload length is 3 bytes

Payload: 03--Bluetooth beacon 01--GPS positioning priority is Bluetooth beacon > GPS

checksum

Single GPS positioning: BDCE0100030001000033

BD: Header

CE: Message ID

01:Type 01 Priority Setting

Valid consistency 00 remains valid

Len converted to big-endian 0003 --> converted to decimal 03 --> payload length is 3 bytes

Payload: 01--GPS 00--no positioning 00--no positioning positioning priority is single GPS positioning

checksum

Single Bluetooth beacon positioning: BDCE0100030003000033

BD: Header

CE: Message ID

01:Type 01 Priority Setting

Valid consistency 00 remains valid

Len converted to big-endian 0003 --> converted to decimal 03 --> payload length is 3 bytes

Payload: 03--Bluetooth beacon 00--no positioning 00--no positioning positioning priority is single Bluetooth beacon positioning

checksum

5.1.4 Settings - Health Sample Reporting Frequency Settings (0XCE02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

Type=02 Health Sample Reporting Frequency Settings

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=02 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 effective this time |
| 2 | U16 | Len | -/- | -/- | The length of the subsequent command |
| 1 | U8 | Healthtype | -/- | -/- | 00—indicates all |
| 1 | U8 |  | -/- | -/- | Body |
| 1 | u8 | Time Unit | -/- | -/- | Time Unit 00—minutes, 01—hours |

Health Sample Reporting Frequency Settings Definition -- see 2.3 Device Downlink Description -- Device Health Sample Frequency Issuance (0xCE02)

Health Sample Reporting Frequency Settings Details: Currently, the universal version supports: 00 - all, reserved items -- not yet implemented: 01 Steps 02 Heart Rate 03 Temperature 04 Sleep 05 Blood Pressure 06 Blood Sugar 07 Blood Oxygen

Example:

Health sample reporting frequency reports once every 5 minutes: BDCE0200030000050033

BD: Header

CE: Message ID

02:Type 02 Health Sampling Reporting Frequency Setting

Valid consistency 00 remains valid

0300: Convert to big-endian 0003 --> Convert to decimal 03 --> Length of the main body is 3 bytes

00: Health type 00 --- All

05: Main body: Convert to decimal 05 --> Health sampling reporting frequency once every 5 minutes

00: Time Unit: 00 -- Minutes

checksum

5.1.5 Settings - Switch Settings (0XCE04-24)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Please refer to the instructions for specific types |
| 1 | u8 | switch | -/- | -/- | Switch 00: On, 02: Off, 01: One-time valid |
| 2 | U16 |  | -/- | -/- | Fixed 0000 |

Device Alarm Settings - Switch Type: Specific Function Description -- See 2.3 Device Downlink Description

07 Drop Alarm Switch

08 Stay Alarm Switch

Type=07 Drop Alarm Switch
Example:

00---On:BDCE0700000093

BD: Header

CE: Message ID

07:Type 07 Drop Alarm Switch

02:switch 00 On

0000:Fixed Value

93:Checksum

02---Off:BDCE0702000093

BD: Header

CE: Message ID

07:Type 07 Drop Alarm Switch

02: switch switch 02 off

0000:Fixed Value

93:Checksum

Type=08 Stay Alarm Switch
Example:

00---On:BDCE0800000093

BD: Header

CE: Message ID

08: Type 08 Stay Alarm Switch

02:switch 00 On

0000:Fixed Value

93:Checksum

02---Off:BDCE0802000093

BD: Header

CE: Message ID

08: Type 08 Stay Alarm Switch

02: switch switch 02 off

0000:Fixed Value

93:Checksum

5.1.6 Issuing Sit-and-Stay Alarm Trigger Time (MSGID=0XCC)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xCC | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | minute | -/- | -/- | Stay alarm trigger duration (unit: minutes; range: 2 — 60), no activity within this time will trigger the stay alarm |

Example:

No activity for 5 minutes. The watch reports a stay alert (while wearing the watch): BDCC05DD

BD: Header

CC: Message ID

05: minute stay alert triggered duration converted to decimal 05 --> 5 minutes

DD: checksum

5.1.7 Shutdown Restart (0x77)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0x77 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- |  | 01—Shutdown, 00—Restart |

Note: Shutdown and Restart can only be received when the device is powered on. Shutdown: Receives the command while powered on, the watch turns off and cannot receive any commands after that; Restart: Receives the command while powered on, the watch restarts.

Example:

Shutdown: BD770193

BD: Header

Message ID

01:Type 01--Shutdown

93:Checksum

Reboot:BD770093

BD: Header

Message ID

01:Type 00--Reboot

93:Checksum

5.1.8 UWB Distance Measurement Alarm Configuration (0x7901)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0x79 | See below for definition | CK_sum |

Payload contents

| Format | Name | Unit | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Type 0x00—-UWB Beacon Distance Measurement Alarm Configuration |
| U8 | distance | 1 | UWB distance measurement alarm distance (unit: meters; value range 0-20), any value less than or equal to this distance will trigger the alarm |

Example:

Alarm distance 1 meter:BD790001C3

BD: Header

79:Message ID

00: Type 0x00---UWB Beacon Distance Alarm Configuration

01: Distance 01 converted to decimal 1, alarm distance is 1 meter

C3: Checksum

Note: If the distance is 0, no alarm will be triggered: BD790000C3

5.2 Server Time Synchronization Information

5.2.1 Device Report Request Time Calibration Data

Note: This message is sent by the device

After the device is powered on, it will automatically send a request time synchronization command. It needs to receive the synchronization command and reply with the synchronized time data frame for device time synchronization. If the device was calibrated before shutdown, then after rebooting, the device interface will display the time from before shutdown, and the device will report FF00FF to request time calibration again.

| Byte offset | Format | Name | Description |
| --- | --- | --- | --- |
| 1 | U8 | HEADER | Padding: 0xFF |
| 1 | U8 | SeqId | Padding: 0x00 |
| 1 | U8 | End | Padding: 0xFF |

Example:

FF00FF    // The server must respond immediately after receiving, with no delay

5.2.2 Time Calibration Request Data Response

| Byte offset | Format | Name | Description |
| --- | --- | --- | --- |
| 1 | U8 | HEADER | Padding: 0xFF |
| 1 | U8 | SeqId | Padding: 0x10 |
| 2 | U16 | years | Year Data |
| 1 | U8 | month | Month Data |
| 1 | U8 | Day | Day, data |
| 1 | U8 | time | Clock, data |
| 1 | U8 | Minute | Minute, data |
| 1 | U8 | Seconds | Seconds, data |
| 1 | U8 | End | Padding: 0xFF |

Example:

After receiving FF00FF, the server replies with a downlink command to calibrate the time. Note: The example is for the year 2020; it is only a structural analysis example. The current year must be sent; otherwise, the parsing of FF1007E409020B1B28FF will not be performed.

FF : Header

10 : Seqid

07E4: Year（2020）

09 : Month，（09）

02 : Day，（02）

0B : Hour，（11）

1B : Minitus，（27）

28 : Second，（40）

FF : End