W300PL-Lorawan-Communication Protocol

Table of Contents

1 Overview	3

2 Device Usage Instructions	3

2.1 Device Functions and Usage Instructions	3

2.2 Default Reporting Logic of Equipment	5

2.3 Device Downlink Description	6

3 Protocol data packet structure	9

3.1 Data Header	9

3.2 Message Identifier (Message ID)	9

3.3 Payload	9

3.5 Checksum	11

4 Report messages	12

4.1 Alarm-related reporting	12

4.1.1 Alarm Data Upload-1 (MSGID=0x02)	12

4.1.2 Alarm data upload-2 (MSGID=0x21) (Supplement to 02)	16

4.1.3 SOS Upload (MSGID=0XB5)	20

4.2 Location-related Reporting	21

4.2.1 GPS/BDS Location Reporting: Location data reporting (MSGID=0x03)	21

4.2.2 Bluetooth Location Information (LBE Location) (MsgId=0xD6)	23

4.3 Device Information and Status Reporting	26

4.3.1 Device Charging Status Upload (MSGID=0xC3)	26

4.3.2 Device Status (MSGID=0xE9)	28

4.3.3 Battery Signal (MSGID=0xF9)	30

4.3.4 Firmware Version Number (MSGID=0xBB)	32

4.4 Health-related Reporting	33

4.4.1 Health Data (MSGID=0x32)	33

4.5 Downlink feedback related reports	35

4.5.1 Downlink feedback (MSGID=0xC0) — General firmware not available, will be supported in future	36

4.5.2 Message status report (MSGID=0x28) — No general firmware available, will support it in the future	36

5 Settings	39

5.1 Downlink	39

5.1.1 Set Position Reporting Frequency (MSGID=0x17)	39

5.1.2 Message Sending (MSGID=0X28)	41

5.1.3 Set - Setting location priority (0XCE01)	43

5.1.4 Set - Health sampling report frequency setting (0X1C)	45

5.1.5 Settings - Switch Settings (0XCE04-24)	48

5.1.6 Settings - Drop Sensitivity Setting (0xCE15)	50

5.1.7 Issue Sedentary Stay Alarm Trigger Time (MSGID=0XCC)	51

5.1.8 Power off restart (0x77)	52

5.1.9 Weather Warning (0XCB)	53

5.1.10 LORA Parameter Settings (0xCF)	57

5.2 Server Time Synchronization Information	63

5.2.1 Device Reporting Request Time Calibration Data	63

5.2.2 Time Calibration Request Data Reply	64

1 Overview

This protocol is suitable for standard LoRaWAN protocol devices and currently supports the W300PL products. It uses a 32-bit data header for synchronization and terminal identification; employs a low-overhead checksum algorithm for integrity protection; and utilizes message identifiers to distinguish different messages.  
It needs to be parsed according to actual reports; this document is continually maintained. If any errors or omissions are found, please provide timely feedback. Thank you very much.

2 Device Usage Instructions

2.1 Device Functions and Usage Instructions

General version: The default high-frequency general setting is AS923 (923.2-924.6), and the low-frequency general setting is CN470 (470.3-471.7) OTAA-CLASSA. If other frequency bands are required, requests can be made.

Network access related key:

OTAA:

Application key:  2 B 7 E 151628 AED 2 A 6 ABF 7158809 CF 4 F 3 C

ABP:

Deviceaddress: The last eight digits of the watch DEVEUI

Applicationsession key: EF6D6E2503F57AE2FA151CDA87455F18

Networksession key:2E8C8650B4041672BBB9A399F2DEB427

(1) Power On

Before using for the first time, please charge it fully. The device will power on automatically when the charge reaches the required level, showing a charging icon during charging,a full charge is indicated by a green icon.

Note: Do not check the device signal while it is charging.

Manual power on: Press and hold the upper button for 10 seconds and then release; the interface will display the word “Welcome.” Note: The default wearing status at power on will trigger a fall alarm if no heart rate is detected.

(2) Power Off

Low battery shutdown: The screen will turn off after displaying "Byebye."

Manual shutdown: Hold the button for more than 10 seconds, and the screen will display "Byebye" before turning off.

Note: The device will not power off while charging.

(3) SOS Function

Activation method: After activation, the device will not enter sleep mode. Press and hold the lower button for 3 seconds; the interface will display "SOS SEND OK" / SOS sent successfully, and the SOS icon will appear.

Cancellation method: In SOS mode, press and hold the lower button for 3 seconds; the interface will display "SEND CANCEL" / SOS canceled, and the SOS icon will disappear.

(4) Signal Status

No signal: The interface signal is represented by a horizontal line, indicating that the device report has failed or is not connected to the network (not within the LoRa gateway range).

Signal present: The interface signal displays a stepped bar graph.

(5) Device Sleep

Trigger condition: The device remains inactive for 40 minutes, entering sleep mode and not reporting positioning health data.

(6) Vibration:

Received a text message from the downlink, turning on

2.2 Default Reporting Logic of Equipment

The universal version: Reporting must be within the range of the Lora gateway; otherwise, the report will fail. The default interval for failure reporting is 5 minutes before rejoining the network.

(1) Positioning-related reporting

GPS/Bluetooth Beacon: Default reporting frequency is 10 minutes, default positioning priority: Bluetooth Beacon > GPS, with Bluetooth location as the priority. If location cannot be determined, switch to GPS positioning.

Note: GPS is difficult to locate indoors; please test in an open outdoor environment.

(2) Alarm-related Reporting

SOS Alarm (0 xB5): Triggered actively by the user, for triggering methods see the previous section

SOS Cancel (0 x02): Cancel triggered actively by the user, for triggering methods see the previous section

Shutdown Alarm (0 x21): Triggered when the device shuts down actively or due to low battery, for triggering methods see the previous section

Wear Removal Alarm (0 x02): The device judges whether it is being worn based on health sampling reporting frequency; if heart rate is detected, it reports a wear alarm; if no heart rate is detected, it reports a removal alarm

Prolonged Sitting Alarm (0 x02): Defaults to triggering reporting after 15 minutes of inactivity

Fall Alarm (0 x02): Triggered when the device free-falls from a certain height, satisfying the fall algorithm

Low Battery Alarm (0 x02): Triggered when the current battery level is less than or equal to 0

(3) Health-related Reporting

Step count, heart rate, body temperature & wrist temperature, blood pressure, blood oxygen (0 x32): Default reporting frequency is 10 minutes

(4) Equipment Information and Status Reporting

Charging Status (0 xC3): Start charging, end charging, report when fully charged

Battery Level Signal (0 xF9): Report once upon startup, and will also report during positioning and health checks

Software Version Number (0 xBB): Report once upon startup

(5) Downlink Feedback

Downlink Feedback (0xC0): Report to server after the device receives downlink instructions -- General firmware not yet available, will be supported later

Message Status Feedback (0x28): After downlink text message, the device can click accept or decline -- General firmware not yet available, will be supported later

(6) Time Synchronization Request Report

Time Synchronization Request (FF00FF): The device reports once every startup to request downlink instructions from the server to calibrate the time

Note: The device may report packed data, meaning a single data packet contains multiple complete messages; be careful not to miss any, as messages are complete and will not be interrupted in the next data packet

Eg: bdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9bdf9010000006400002800000019a9cf61ca

This data packet contains messages for Bluetooth positioning (0xD6) and battery level signals (0xF9).

(0xD6) Bluetooth positioning: bdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9

(0xF9) Battery level signal: bdf9010000006400002800000019a9cf61ca

2.3 Device Downlink Description

General Version:

(1) Device positioning report frequency issued (0x17)

The default reporting frequency is 10 minutes, maximum 1 minute, minimum 1440 minutes. After receiving the downlink command, the device reports data according to the issued time period and frequency.

Outside the time period, reports will be made according to the default reporting frequency. For example: from 00:00 to 18:00, positioning report every 2 minutes,

then outside the time period, it will report according to the default frequency of 10 minutes.

(2) Text message issued (0x28)

Unicode encoding, up to 20 Chinese characters, with each Chinese character occupying 2 bytes and each English letter occupying 2 bytes

The previous low-frequency firmware version was: GB2312 encoding, with a maximum of 20 Chinese characters, where one Chinese character occupies 2 bytes and one English letter occupies 1 byte.

Message List - Up to 50 historical messages can be saved (Note: firmware without a message list can save a maximum of 20 messages)

(3) Device positioning priority assignment (0xCE01)

Default positioning priority: Bluetooth > GPS,

The positioning priority is as follows: gps > Bluetooth beacon, gps positioning is prioritized, switch when gps positioning is unavailable.

Bluetooth beacons, when positioning is successful, will not switch to the next positioning priority to generate positioning.

(4) Device health sampling frequency issuance (0x1C)

The default health sampling frequency is 10 minutes, with a maximum of 2 minutes. After receiving the downlink command, the device follows the issued instructions.

Data reporting by time period and frequency; no reporting outside the time period. For example: if location data is reported every 10 minutes from 00:00 to 18:00, then there will be no reporting outside this time period.

(5) Sitting timeout alarm trigger time delivered (0xCC)

Value range: 2 minutes - 60 minutes. Note that it only works when the sedentary alert switch is turned on.

For example: If the downward action has a trigger time of 10 minutes, the device will report a sedentary alert if it remains idle for 10 minutes.

(6) Device shutdown and restart command (0x77)

Note, this command must be effective when the device is powered on; it cannot receive commands when powered off.

Shutdown command: Send this command while the device is powered on, the device will shut down, and it cannot receive additional commands when powered off.

Restart command: Send this command while the device is powered on, and the device will restart.

(7) Fall sensitivity and height command (0xCE15)

Latest universal firmware default fall sensitivity: low (medium-low). Fall height: 1.5m.

Sensitivity: Refers to the degree that meets the fall detection algorithm, providing 5 setting levels (0 - 4): low - lower (medium-low) - medium - higher (medium-high) - high.

Height: Refers to the height required to trigger the fall alert, providing 5 setting levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m - 2.5m.

(8) Sedentary Alarm Switch (0xCE08)

By default, the device does not report prolonged inactivity alerts. When enabled, the default trigger time is 15 minutes, meaning that if the user remains inactive for 15 minutes, a prolonged inactivity alert is triggered.

(9) Drop alarm switch (0xCE07)

By default, the drop alarm is off. When enabled, the device will report a drop alarm after triggering a drop.

(10) Weather warning issue (0xCB)

Multiple sets of weather warnings can be issued. Note that each set of weather warnings must be different, and the same type of warning cannot be repeated.

(11) LORA parameter issuing (0XCF)

Downlink instructions can set network access mode, message type, how many times to report failure before reaccessing the network, etc. -- see the downlink instruction section for specifics.

(12) Calibration time command downlink (0XFF)

When the device reports a time calibration request message, this command is immediately downlinked for watch time calibration.

3 Protocol data packet structure

A basic protocol packet structure is shown in Figure 1:

3.1 Data Header

Each packet starts with a 1-byte Header (except for the request calibration time message: ff00ff):

Currently, the device uploads Header fixed to BD (except for the request calibration time message: ff00ff)

Header: 0xBD;

3.2 Message Identifier (Message ID)

The content represented by MessageId is detailed in Chapter 4.

3.3 Payload

The payload referred to here is the valid content of the protocol excluding the header and checksum. The length of the content follows.

The data format used in the payload is shown in the table below:

[U-unsigned; I-signed; X-bitfield; number - occupied byte size]

In the following protocol, all data types are used in little-endian format except for ch, u8, i8, and x8.

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Description |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short Integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short Integer |
| x8 | Bitfield | 1 | - | - | bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Integer |
| x16 | Bitfield | 2 | - | - | Bit (bit) 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long Integer |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long Integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit long integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | float type |

3.5 Checksum

The contents included in the checksum are the payload, as shown in Figure 1. The algorithm is as follows; by default, general version devices do not require checksum, and this part can be ignored. Any one byte will suffice for the downlink command.

// Assuming content="BDE9000600010A00000A00", the result of sum.ToString("X2") is 07// Below is the method for calling in C# code.

private string CheckSum(string content)

{

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2");

}

4 Report messages

4.1 Alarm-related reporting

4.1.1 Alarm Data Upload-1 (MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0x02 | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below 见下方定义 |
| 4 | U32 | Timestamp | - | - | Timestamp (additional transmission will append the timestamp) |

Upl_warn contents

| bit (the position of 1 in binary) | Name | Description | Raw message in little-endian hexadecimal | Converted to big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 14 | Drop alarm | Drop alarm | 0040 | 4000 | 4*4096=16384 |
| 8 | Device wear | Device wear | 0001 | 0100 | 1*256=256 |
| 7 | SOS cancel | SOS cancel | 8000 | 0080 | 8*16=128 |
| 5 | Sit idle alarm (inactive) | Sit idle alarm (inactive) | 2000 | 0020 | 2*16=32 |
| 4 | Remove (fall off) device | Remove (fall off) device | 1000 | 0010 | 1*16=16 |
| 2 | Power off | Power off | 0400 | 0004 | 4 |
| 0 | Low battery | Low battery | 0100 | 0001 | 1 |

Example:

Wearing alarm: BD020001E377BD678A

BD:Header

02 :Message ID

0001: Upl_warn alarm content, converted to big-endian:0100--> Wearable alarm

E377BD67: Timestamp timestamp converted to big-endian67BD77E3 --> converted10decimal1740470243--> Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Converted Beijing time:2025-02-25 15:57:23

8a: checksum

Fall alarm: BD021000E377BD678A

BD:Header

02 :Message ID

1000: Upl_warn alarm content, converted to big-endian0010--> Fall alarm

E377BD67: Timestamp timestamp converted to big-endian67BD77E3 --> converted10decimal1740470243--> Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Converted Beijing time:2025-02-25 15:57:23

8a: checksum

SOS cancel: BD028000E377BD678A

BD:Header

02 :Message ID

8000: Upl_warn alarm content, converted to big-endian0080--> SOS cancel

E377BD67: Timestamp timestamp converted to big-endian67BD77E3 --> converted10decimal1740470243--> Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Converted Beijing time:2025-02-25 15:57:23

8a: checksum

Drop alarm: BD020040E377BD678A

BD:Header

02 :Message ID

0040: Upl_warn alarm content, converted to big-endian4000--> Drop alarm

E377BD67: Timestamp timestamp converted to big-endian67BD77E3 --> converted10decimal1740470243--> Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Converted Beijing time:2025-02-25 15:57:23

E7: checksum

Prolonged sitting alarm: BD022000E377BD678A

BD:Header

02 :Message ID

2000:Upl_warn alarm content, converted to big-endian0020--> Prolonged sitting alarm

E377BD67: Timestamp timestamp converted to big-endian67BD77E3 --> converted10decimal1740470243--> Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Converted Beijing time:2025-02-25 15:57:23

8A: checksum

Low battery alarm: BD020200E377BD678A

BD:Header

02 :Message ID

0100:Upl_warn alarm content, converted to big-endian0001--> Low battery alarm

E377BD67: Timestamp timestamp converted to big-endian67BD77E3 --> converted10decimal1740470243--> Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Converted Beijing time:2025-02-25 15:57:23

8A: checksum

Shutdown alarm: BD020400E377BD678A

BD:Header

02 :Message ID

0400:Upl_warn alarm content, converted to big-endian0004--> Shutdown alarm

E377BD67: Timestamp timestamp converted to big-endian67BD77E3 --> converted10decimal1740470243--> Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Converted Beijing time:2025-02-25 15:57:23

8A: checksum

Note: This alarm is used for devices that do not distinguish between shutdown types; devices that do distinguish between shutdown types are covered in the next section.

4.1.2 Alarm data upload-2 (MSGID=0x21) (Supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0x21 | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | 报警类型 |
| N |  | Upl_warn | - | - | Alarm content See below 见不同Alarm type类型定义 |
| 4 | U32 | Timestamp |  |  | Timestamp (additional transmission will append the timestamp) |

Alarm type=1

Upl_warn contents

| bit (the position of 1 in binary) | Name | Description | Raw message in little-endian hexadecimal | Converted to big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging Shutdown Alarm | Device charges and shuts down automatically (firmware must support this function) | 04000000 | 0004 | 4 |
| 1 | Low Battery Shutdown Alarm | Device shuts down due to low battery (firmware must support this function) | 02000000 | 0002 | 2 |
| 0 | Active Shutdown Alarm | The device was manually shut down (firmware support required) | 01000000 | 0001 | 1 |

Example:

Shutdown alarm while charging: BD21010002000000E377BD6767

BD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001 --> Alarm type = 1

04000000:Upl_warn alarm content, converted to big-endian 0004 --> Shutdown alarm while charging

E377BD67:Timestamp, converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> Timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

67:checksum

Low battery shutdown alarm: BD21010002000000E377BD6767

BD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001 --> Alarm type = 1

Upl_warn alarm content, converted to big-end 0002 --> low power shutdown alarm

E377BD67:Timestamp, converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> Timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

67:checksum

Active Shutdown Alarm: BD21010001000000E377BD6767

BD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001 --> Alarm type = 1

01000000:Upl_warn alarm content, converted to big-end 0001 --> Active shutdown alarm

E377BD67:Timestamp, converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> Timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

67:checksum

Alarm type=5

Upl_warn contents

| Format | Name | Unit | Description |
| --- | --- | --- | --- |
| U16 | Type | 2 | Fixed to 0x05 |
| U32 | Timestamp | 4 | Timestamp |
| U16 | Content Len | 2 | Alarm content length |
| U8 | Health Warn Type1 | 1 | Health threshold alarm types: 0x01 heart rate, 0x02 systolic blood pressure (SBP), 0x03 diastolic blood pressure (DBP), 0x04 blood oxygen, 0x05 temperature |
| U8 | Type1 | 1 | 01 less than the set health threshold, 02 greater than the set health threshold |
| I16 | Health Value1 | 2 | Health result value |
| U8 | Type2 | 1 | 01 less than the set health threshold, 02 greater than the set health threshold |
| I16 | Health Value2 | 2 | Health result value |
| ...... | ...... | ...... | ...... |

Example:

The device has detected that the heart rate (65) is beyond the issued health threshold: BD210500E377BD6704000102410073

BD: Header

21: Message ID

0500: Alarm type  Convert to big-endian0005--> Alarm type=5

E377BD67: Timestamp  Convert to big-endian67BD77E3--> Convert to decimal 1740470243 --> Timestamp is 1740470243 seconds -->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time: 2025-02-25 15:57:23

0400: Content Len  Alarm content length  Convert to big-endian0004--> Alarm content length is 4 bytes, Note: Length increases with multiple health threshold alarms

01: Health Warn Type  Health threshold alarm type 01--> Heart rate alarm

02: Type Greater than the set health threshold & less than the set health threshold  01--> Less than the set health threshold

4100: Health Value  Reported health value  Convert to big-endian0041--> Convert to decimal 65 --> Reported heart rate value is 65

73: checksum

4.1.3 SOS Upload (MSGID=0XB5)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xB5 | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Description |
| --- | --- | --- | --- |
| 1 | U8 | Status | Status: 1: SOS |
| 4 | Int32 | timestamp | Timestamp, little endian (timestamp little Endian) |

Example:

Hexadecimal raw message: BDB501E377BD6755

BD:Header

B5:Message ID

01: Status 01--SOS

E377BD67: Timestamp timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

55: checksum

4.2 Location-related Reporting

4.2.1 GPS/BDS Location Reporting: Location data reporting (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0x16 | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude |
| 8 | Double | lat |  | - | latitude |
| 1 | U8 | north_south |  | - | /N or S/ Northern or Southern Latitude |
| 1 | U8 | east_west |  | - | /E or W/ East or West longitude |
| 1 | U8 | status |  | - | /A or V/ valid data or invalid data |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original 16 Hexadecimal Message: BD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BD:Header

03: Message ID消息id

963E870C3E675E40: lon Longitude  Convert to Big-endian405E673E0C873E96--> Longitude:121.6131622  Note: Conversion analysis refers to the examples below

FF2110C2B6343F40: lat Latitude  Convert to Big-endian`403F34B6C21021FF--> Latitude:31.2059137  Note: Conversion analysis refers to the examples below

4E: north_south  4E--> Convert to ASCII Code N--> North Latitude

45: east_west 45--> Convert to ASCII Code E--> East Longitude

41: status Data Validity  41--> Convert to ASCII Code A--> Valid Data

E377BD67: Timestamp Convert to Big-endian67BD77E3--> Convert to10Base1740470243--> Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Converted Beijing time:2025-02-25 15:57:23

78: checksum

Note: The parsed GPS coordinates are in the WGS-84 coordinate system; if the map uses Baidu or Gaode, the coordinate system needs to be converted

Parsing Satellite Positioning Message Java Code Example:

// Message: BD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

publicstaticvoidmain(String[] args){

//Message 7d9f84ac81815c40 actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d", 16)));   //114.02353966666665 //Message e766926b1d893640 actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16))); //22.535605166666667

HexToStr(data.Substring("4e"));    //N

HexToStr(data.Substring("45"));    //E

HexToStr(data.Substring("41"));    //A   A indicates valid data, while V indicates invalid data and can be discarded //Message 749d695f actual value 5f699d74

Date date=new Date();

date.setTime(Long.parseLong("5f699d74", 16) * 1000);

SimpleDateFormat sdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.2.2 Bluetooth Location Information (LBE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xD6 | See below for definition 见下方定义 | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 0 |
| U8 | Total_groups | 1 | Total number of groups, which may contain multiple sets of information, each of which may have multiple iBeacons |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| ... | ... | ... | ... |

Note: In the universal version, the total number of Bluetooth beacons is fixed at 1, and the maximum number of Bluetooth beacons is 4. They are sorted in descending order by signal strength. By default, the smartwatch only recognizes our own beacons. If you need to connect to other brands’ beacons, please consult the relevant personnel via WeChat.

Original hexadecimal message: BDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

BD: Header

D6: Message ID

00: type

01: There is only one set of iBeacon data

E377BD67: Timestamp timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

04: The first set has 4 beacon information

4327: Convert to big-endian --> major: 0x2743 --> Convert to decimal 10051

1794: Convert to big-endian --> minor: 0x9417 --> Convert to decimal 37911

AC:  RSSI 0xAC --> Convert to decimal two's complement -84

4327: Convert to big-endian --> major: 0x2743 --> Convert to decimal 10051

3094: Convert to big-endian --> minor: 0x9430 --> Convert to decimal 37936

AA:  RSSI 0xAA --> Convert to decimal signed -86

4327:Convert to big-endian --> major: 0x2743 --> Convert to decimal 10051

b956:Convert to big-endian --> minor: 0x56b9 --> Convert to decimal 22201

A5:   RSSI 0xA5 --> Convert to decimal signed -91

4327: Convert to big-endian --> major: 0x2743 --> Convert to decimal 10051

FE94: Convert to big-endian --> minor: 0x94FE --> Convert to decimal 38142

A5: rssi: 0xA5 --> Convert to decimal signed -91

6a -- Checksum

Bluetooth beacon major and minor description: The default Bluetooth beacon shipped comes with a label.

For example: 00081005345866, the major of the Bluetooth beacon is 10053, the minor is 45866, and the manufacturer identifier is 0008 (which is useless and has no actual significance).

4.3 Device Information and Status Reporting

4.3.1 Device Charging Status Upload (MSGID=0xC3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xC3 | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Status | / | / | 0 start, 1 end, 2 fully charged |
| 1 | U32 | Timestamp | / | / | Timestamp (additional transmission will append the timestamp) |

Note: This message requires device firmware support, and the default latest universal version is supported.  
    Example:

Original hex message: BDC301E377BD678A

BD:Header

C3: Message ID

01: Device has finished charging

E377BD67: Timestamp timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

8A: checksum

Original hex message: BDC300DB4D2F668A

BD:Header

C3: Message ID

00:The device has started charging

E377BD67: Timestamp timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

8A: checksum

Original hexadecimal message:BDC302DB4D2F668A

BD:Header

C3: Message ID

02:The device is fully charged

E377BD67: Timestamp timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

8A: checksum

4.3.2 Device Status (MSGID=0xE9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xE9 | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | type |  | / | Default—00 |
| 2 | u16 | length |  | / | Length of subsequent message |
| 1 | u8 | location |  | / | Reporting frequency for positioning: 00—Default unchanged, 01—Modified in the downlink (if there are multiple time periods for downlink, only report the frequency for the current time period) |
| 2 | u16 | frequency |  | / | Frequency, unit - minutes |
| 1 | u8 | health |  | / | Health reporting frequency: 00—- default unchanged, 01—- modified with downlink |
| 2 | u16 | frequency |  | / | Frequency: unit - minutes |

Note: This message reports once at startup, and will report once again after the downlink modification frequency.

Example:

Raw hexadecimal message: BDE9000600010A00010A0033

BD:Header

E9 : Message ID

00 : type=00

0600 : length of the following message converted to big-endian --> 0006 --> converted to decimal 6, message length is 6 bytes

01: location 01 indicates that the downlink has modified the positioning reporting frequency.

0A00: frequency The health reporting frequency converts to big-endian -->000A--> converts to decimal 10, with a reporting frequency of 10 minutes.

01: health 01 indicates that the downlink has modified the health reporting frequency.

0A00: frequency The health reporting frequency converts to big-endian -->000A--> converts to decimal 10, with a reporting frequency of 10 minutes.

33: checksum.

If the device has no health reporting function or no positioning reporting function, the health reporting frequency and positioning reporting frequency will be consistent, indicating the reporting frequency of the device.

4.3.3 Battery Signal (MSGID=0xF9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xF9 | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery type 0: 4-level, 1: 5-level, 2: percentage, 3: voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery value If Bat_type is 0: the battery value range is 0-3 (0 means 25%, 3 means 100%) If Bat_type is 1: the battery value range is 0-4 (0 means 20%, 4 means 100%) If Bat_type is 2: the battery value range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: percentage, 1: 5-level system, 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal strength |
| 1 | U8 | Other_type |  |  | Extended type 0: Full-step counting 1: Incremental counting 2: Vibration |
| 4 | U32 | Num |  |  | Extended value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Example:

Original 16 Hex Message: BDF90104000050000095000000E377BD67AA

BD: Header

F9: Message ID

01: Bat_type Battery Type 01 --> 1 --> Level 5 Battery (0 - 4)

0400: Bat_volt Battery Value converted to Big Endian 0004 --> 4 --> Level 4 corresponds to 100% Battery

00: Signal_type Signal Type 00 --> 0 --> Percentage

5000: Signal_strength Signal Strength converted to Big Endian 0050 --> converted to Decimal 80 --> 80% Signal Strength

00: Other_type Extended Type 00 --> 0 --> Total Steps

95000000: Num Extended Value - Steps converted to Big Endian 00000095 --> converted to Decimal 149 --> Step Count is 149

E377BD67: Timestamp converted to Big Endian 67 BD77E3 --> converted to Decimal 1740470243 --> Timestamp is 1740470243 seconds

Convert to standard time format UTC time:2025-02-25 07:57:23--> Converted Beijing time:2025-02-25 15:57:23

AA:checksum

4.3.4 Firmware Version Number (MSGID=0xBB)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xBB | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit Description |
| --- | --- | --- | --- |
| 1 | UINT8 | Version_len | Software Version Length |
| N | ASCII[n] | Version | Software Version |

Example:

BDBB1043323331304C2E3437302E41412E3031

BD:Header

BB:Message ID

10:Version_len converted to 10 decimal 16--> The software version number length is16 bytes

43323331304C2E3437302E41412E30:Convert to string --> C2310L.470.AA.01

31:checksum

4.4 Health-related Reporting

4.4.1 Health Data (MSGID=0x32)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0x32 | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | 00 |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 2 | U16 | contentLength |  |  | Total length of the following content |
| 1 | U8 | ID |  |  | Id(type + reported value length), ID type definitions are provided below |
| N | N*U8 | Val1 |  |  | Reported value of Id (changes in byte length based on reported value length) |
| ... | ... | ... | ... | ... | ... |
| 1 | U8 | ID |  |  | Id(type + reported value length), ID type definitions are provided below |
| N | N*U8 | Val1 |  |  | Reported value of Id (changes in byte length based on reported value length) |

ID type definitions (hexadecimal):

01 Step count 02 Heart rate 03 Body temperature 04 Wrist temperature 06 Diastolic pressure 07 Systolic pressure 08 Blood oxygen 09 HRV (special version support)

(05 Blood sugar 10 Number of sit-ups 11 Running pace and distance 12 Rope skipping speed and count) --- Reserved, no current support

Default universal version support: 01 Step count 02 Heart rate 03 Body temperature 04 Wrist temperature 06 Diastolic pressure 07 Systolic pressure 08 Blood oxygen

Example:

Raw 16 hexadecimal message: BD3200E377BD6711000A1E00114B314A39711A4A0122bc00416212

BD:Header

32:Message ID

00:type  type=00

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10base1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Converted Beijing time:2025-02-25 15:57:23

1100:Length of the following message converted to big-endian-->0011-->converted to decimal17, the length of the following message is17bytes

0A:ID  0x0A-->converted to binary00001010   The first five bits are00001-->converted to16base01, representing data ID step count (0x01),

The last three bits are010-->converted to decimal02, representing the length of the step count data as2bytes

1E00:Step count report value converted to big-endian-->001E-->converted to decimal30, total step count data is30steps

11:ID  0x11-->converted to binary00010001   The first five bits are00010-->converted to16base02, representing data ID heart rate (0x02),

The last three bits are001-->converted to decimal1, representing the length of the heart rate data as1byte

4B:Heart rate report value converted to decimal-->75, heart rate data is75

31:ID  0x31--> Convert to binary00110001   The first five bits are00110--> Convert to hexadecimal06, representing data ID diastolic pressure (0x06),

The last three digits are 001 --> converted to decimal 1, representing that the diastolic pressure data length is 1 byte

4A: Diastolic blood pressure reported value, converted to decimal --> 74, diastolic pressure data is 74

39:ID  0x3 --> converted to binary 00111001    The first five digits are 00111 --> converted to hexadecimal 07, representing data ID for systolic pressure (0x07),

The last three digits are 001 --> converted to decimal 1, representing that the systolic pressure data length is 1 byte

71: Systolic blood pressure reported value, converted to decimal --> 113, systolic pressure data is 113

1A: ID  0x1A --> converted to binary 00011010   The first five digits are 00011 --> converted to hexadecimal 03, representing data ID for body temperature (0x03),

The last three digits are 010 --> converted to decimal 2, representing that the body temperature data length is 2 bytes

4A01: Body temperature reported value, converted to big-endian --> 014 A --> converted to decimal 330 --> 330 * 0.1 --> body temperature data is 33 degrees Celsius

22:ID  0x22 --> converted to binary 00100010   The first five digits are 00100 --> converted to hexadecimal 04, representing data ID for wrist temperature (0x04),

The last three digits are 010 --> converted to decimal 2, representing that the wrist temperature data length is 2 bytes

BC00: Wrist temperature report value, converted to big endian--> 00 BC--> converted to decimal 188 --> 188 * 0.1 --> wrist temperature data is 18.8 degrees Celsius

41 :ID 0x41 --> converted to binary 01000001    The first five bits are 01000 --> converted to hexadecimal 08, code data ID for blood oxygen (0x08),

the last three bits 001 --> converted to decimal 1, representing the blood oxygen data length of 1 byte

62 : Blood oxygen report value, converted to decimal--> 98, blood oxygen data is 98

12 : checksum

4.5 Downlink feedback related reports

4.5.1 Downlink feedback (MSGID=0xC0) — General firmware not available, will be supported in future

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xC0 | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This instruction is used for feedback on downlink commands, returning the Message ID of the downlink commands received earlier (multiple Message IDs can be returned collectively). Some downlink commands do not have C0 feedback.

Original hexadecimal message: BDC0011720

BD:Header

C0:Message ID

01:length Message ID length converted to decimal 01 --> Message ID length is 1 byte

17:Type indicates the device received the downlink command Message ID as 17

20:checksum

4.5.2 Message status report (MSGID=0x28) — No general firmware available, will support it in the future

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0x28 | See below for definition 见下方定义 | CK_sum |

Payload contents

| Bytes | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | timestamp | -/- | -/- | Timestamp |
| 1 | U8 | Type | -/- | -/- | Character encoding type (corresponding to downlink 0X28), GB2312 encoding: 0x03; Unicode encoding: 0x05 |
| 1 | U8 | status | -/- | -/- | Message Status - 0x01: Confirmed, 0x02: Rejected |
| 4 | U32 | seqID | -/- | -/- | (Corresponding message ID seqID for downstream 0X28) |

Example:

Note: The current general version does not distinguish character encoding types, fixed at 03; future versions will differentiate.

Original Hexadecimal Message: BD28FD3BA068030268A03BE477

BD:Header

28: Message ID

FD3BA068: timestamp 转为大端 68A03BFD --> 转十进制 --> timestamp is 1755331581 seconds -->

Convert to standard time format UTC: 2025-08-16 08:06:21 --> Convert to Beijing time: 2025-08-16 16:06:21

03: Type 03 -- fixed value

02: status message status 02 -- clicked reject button on the interface -- rejected

68A03BE4:the seqID of the downstream message is 68A03BE4

77:checksum

5 Settings

5.1 Downlink

5.1.1 Set Position Reporting Frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0x17 | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |

Note: One day's time is from 00:00 to 23:59
Example:

00:00-23:59(全日) 每隔10分钟上报一次定位数据: BD17010A000000173B00000000000000000000000000000000000000000097

BD:Header

1C: Message ID

01: enable status, convert to decimal01-->1-- enabled

0A00: Interval time, convert to Big Endian000A--> convert to decimal10--> interval time is10minutes

00: time_start_h start time - hours, convert to decimal00--> start time - hours is00

00: time_start_m start time - minutes, convert to decimal00--> start time - minutes is00

17: time_end_h end time - hours, convert to decimal23--> end time - hours is23

3B: time_end_m end time - minutes, convert to decimal59--> end time - minutes is59

00: enable status, convert to decimal00-->0-- not enabled

000000000000: all filled for not enabled status00

00: enable status, convert to decimal00-->0-- not enabled

000000000000: all filled for not enabled status00

00: enable status, convert to decimal00-->0-- not enabled

000000000000: all filled for not enabled status00

97: checksum

5.1.2 Message Sending (MSGID=0X28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0x28 | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Description |
| --- | --- | --- | --- |
| 1 | U8 | type | Message type; if it is a downlink text message, the Unicode encoding is 05, and the GB2312 encoding is 03. |
| 4 | Uint32 | seqID | The ID of the message, which must be unique. |
| 1 | U8 | CONTENT LEN | Content length |
| N | N | CONTENT | Content, Unicode encoding (little-endian, be careful with conversion; one Chinese character occupies 2 bytes, one letter occupies 2 bytes) GB2312 encoding (little-endian; one Chinese character occupies 2 bytes, one letter occupies 1 byte) |

Note: The default latest universal version for Chinese character encoding is Unicode; previous low-frequency firmware versions used GB2312 encoding.  
The message ID must not be repeated, as devices cannot receive duplicate IDs.  
Example:

Unicode encoding  Text message: hello, world  BD28050300000016680065006C006C006F002C0077006F0072006C006400DD

BD:Header

Message ID

05: type message type Convert to decimal 05-->type=05

03000000: seqID information id  Information id is 03000000

16: CONTENT LEN content length Convert to decimal 22, content length is 22 bytes

680065006C006C006F002C0077006F0072006C006400:

Convert every 2 bytes to big-endian-->Convert to Unicode characters-->Combine final text message content:hello,world

DD:checksum

5.1.3 Set - Setting location priority (0XCE01)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xCE | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 effective this time |
| 2 | U16 | Len | -/- | -/- | Length of the following instructions |
| n | n |  | -/- | -/- | Body |

Location priority settings definition -- see 2.3 Device downlink instructions -- Device location priority issuance (0xCE01)

Location priority settings body: Current general version supports: 01--GPS, 03--Bluetooth beacon (additional Bluetooth beacon deployment required);

Example:

GPS location priority (GPS > Bluetooth beacon): BDCE0100030001030033

BD:Header

CE: Message ID

01: Type 01 Location priority settings

Valid 00 has always been valid

0300 : Len converted to big-endian 0003 --> converted to decimal 03 --> body length is 3 bytes

010300 : Body: 01--GPS 03--Bluetooth beacon positioning priority is GPS > Bluetooth beacon

33: checksum.

Bluetooth positioning priority (Bluetooth beacon > GPS) : BDCE0100030003010033

BD:Header

CE: Message ID

01: Type 01 Location priority settings

Valid 00 has always been valid

0300 : Len converted to big-endian 0003 --> converted to decimal 03 --> body length is 3 bytes

030100 : Body: 03--Bluetooth beacon 01--GPS positioning priority is Bluetooth beacon > GPS

33: checksum.

Single GPS positioning : BDCE0100030001000033

BD:Header

CE: Message ID

01: Type 01 Location priority settings

Valid 00 has always been valid

0300 : Len converted to big-endian 0003 --> converted to decimal 03 --> body length is 3 bytes

030201 : Body: 01--GPS 00--No positioning 00--No positioning positioning priority is single GPS positioning

33: checksum.

Single Bluetooth beacon positioning : BDCE0100030003000033

BD:Header

CE: Message ID

01: Type 01 Location priority settings

Valid 00 has always been valid

0300 : Len converted to big-endian 0003 --> converted to decimal 03 --> body length is 3 bytes

020000 : Body: 03--Bluetooth beacon 00--No positioning 00--No positioning positioning priority is single Bluetooth beacon positioning

33: checksum.

5.1.4 Set - Health sampling report frequency setting (0X1C)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0x1C | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |

Health sampling reporting frequency setting definition -- see 2.3 Device Downlink Description -- Device health sampling frequency issued (0x1D)

Note: After setting, report according to the time period; no reporting outside the time period

Example:

00:00 - 23:59 (24 hours) Report health data every 10 minutes: BD 1 C 010 A 000000173 B 00000000000000000000000000000000000000000097

BD:Header

1C: Message ID

01: enable status, convert to decimal01-->1-- enabled

0A00: Interval time, convert to Big Endian000A--> convert to decimal10--> interval time is10minutes

00: time_start_h start time - hours, convert to decimal00--> start time - hours is00

00: time_start_m start time - minutes, convert to decimal00--> start time - minutes is00

17: time_end_h end time - hours, convert to decimal23--> end time - hours is23

3B: time_end_m end time - minutes, convert to decimal59--> end time - minutes is59

00: enable status, convert to decimal00-->0-- not enabled

000000000000: all filled for not enabled status00

00: enable status, convert to decimal00-->0-- not enabled

000000000000: all filled for not enabled status00

00: enable status, convert to decimal00-->0-- not enabled

000000000000: all filled for not enabled status00

97: checksum

5.1.5 Settings - Switch Settings (0XCE04-24)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xCE | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Please refer to the instructions for specific types |
| 1 | u8 | switch | -/- | -/- | Switch 00 on, 02 off, 01 single use |
| 2 | U16 |  | -/- | -/- | Fixed 0000 |

Device alarm setting - switch type: specific function description -- see 2.3 Device Downlink Description

07   Fall Alarm Switch   08   Stay Alarm Switch

Type=07 Drop Alarm Switch
Example:

00---On:  BDCE0700000093

BD:Header

CE: Message ID

07: Type 07 Drop Alarm Switch

02: switch 00 On

0000: Fixed value

93: checksum

02---Off:  BDCE0702000093

BD:Header

CE: Message ID

07: Type 07 Drop Alarm Switch

02: switch 02 Off

0000: Fixed value

93: checksum

Type=08 Stay Alarm Switch
Example:

00---Open:BDCE0800000093

BD:Header

CE: Message ID

08:Type 08 Stay Alarm Switch

02: switch 00 On

0000: Fixed value

93: checksum

02---Close:BDCE0802000093

BD:Header

CE: Message ID

08:Type 08 Stay Alarm Switch

02: switch 02 Off

0000: Fixed value

93: checksum

5.1.6 Settings - Drop Sensitivity Setting (0xCE15)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xCE | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Description |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Type is 0x15 |
| 1 | U8 | Valid | 1 | The default can be 0, no effect at present |
| 2 | U16 | Len | 2 | Content length of subsequent parameters |
| 1 | U8 | Param Type | 1 | Parameter Type: 0x00: Sensitivity, 0x01: Height |
| 1 | U8 | Param Level | 1 | Parameter Levels: 5 Levels (0 - 4) |

Drop Sensitivity Settings - Parameter Description:

Sensitivity: Refers to the extent that triggers the drop algorithm, providing 5 settings levels (0 - 4): Low - Lower (Medium Low) - Medium - Higher (Medium High) - High.

Height: Refers to the height that triggers the drop alarm, providing 5 settings levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m - 2.5m.

If the height is set to “1.5m,” the device needs to be at least ≥ 1.5m to possibly trigger the drop alarm.

Note that sensitivity and height cannot be combined in a single message.

Example:

Set sensitivity to “Medium Low”: BDCE150002000001FF

BD:Header

CE: Message ID

15:Type Drop Sensitivity Setting

00:Fixed Value

0200:Len

00:Param Type Sensitivity Setting

01:Param Level Sensitivity Level 1-->Low (Medium-Low)

FF:checksum

Set height to “1.5m”: BDCE150002000102FF

BD:Header

CE: Message ID

15:Type Drop Sensitivity Setting

00:Fixed Value

0200:Len

01:Param Type Height Setting

02:Param Level Height Level 2-->1.5m

FF:checksum

5.1.7 Issue Sedentary Stay Alarm Trigger Time (MSGID=0XCC)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xCC | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | minute | -/- | -/- | Stay alarm trigger duration (unit: minutes; range 2 — 60), remaining inactive for this duration will trigger the stay alarm |

Example:

No activity for 5 consecutive minutes. The watch reports a stay alarm (when the watch is worn): BDCC05DD

BD:Header

CC:Message ID

05: minute stay alarm trigger duration converted to decimal 05-->5 minutes

DD:checksum

5.1.8 Power off restart (0x77)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0x77 | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- |  | 01—Power off, 00—Restart |

Note: Power off and restart are both received in the powered-on state. Power off: Command received during powered-on state, the watch powers off and cannot receive any commands after that; Restart: Command received during powered-on state, the watch restarts.

Example:

Power off: BD770193

BD:Header

77: Message ID

01:Type 01--Shutdown

93: checksum

Restart:BD770093

BD:Header

77: Message ID

01:Type 00--Restart

93: checksum

5.1.9 Weather Warning (0XCB)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xCB | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Count | -/- | -/- | Number of Weather Warnings |
| 1 | U8 | Type | -/- | -/- | Weather warning type, refer to below for details |
| 1 | U8 | Status | -/- | -/- | Status, refer to below for details |
| … | … | …… |  |  | … |

Weather Type - Status Table:

| Type | Description | Status | Notes - Clickable images for enlarged view |
| --- | --- | --- | --- |
| 1 | Storm Warning | 0 | Cancel Alarm |
|  |  | 1 - 8 |  |
| 2 | Heavy Rain Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |
| 3 | Thunderstorm Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 4 | Flood Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 5 | Debris Flow Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 6 | Strong Monsoon Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 7 | Frost Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 8 | Fire Danger Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
| 9 | Temperature Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
| 10 | Tsunami Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 11 | Heat Index | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |

Note: After issuance, the device will vibrate to display the weather warning images. Multiple sets of weather warnings can be issued simultaneously, and each set must be different. The same type of warning cannot appear in one command, otherwise it will be invalid. 
 Example:

10 Sets of Weather Warnings Issued: BD CB 0 A 0101 0201 0301 0401 0501 0601 0701 0801 0901 0 A 01 F 5

BD:Header

CB: Message ID

0A: Count of Weather Groups in Decimal 10 --> Issuing 10 Sets of Weather Warnings

0101: Type&Status Weather Type - Status 01 -- Storm Warning 01 -- Signal 1 Alert

0201:Type&Status Weather Type-Status 02--Heavy Rain Warning 01--Yellow Heavy Rain Warning Signal

0301:Type&Status Weather Type-Status 03--Thunderstorm Warning 01--Thunderstorm Alert

0401:Type&Status Weather Type-Status 04--Flooding Warning 01--Northern New Territories Flooding Special Report

0501:Type&Status Weather Type-Status 05--Landslide Warning 01--Landslide Alert

0601:Type&Status Weather Type-Status 06--Strong Monsoon Warning 01--Severe Monsoon Signal

0701:Type&Status Weather Type-Status 07--Frost Warning 01--Frost Signal

0801:Type&Status Weather Type-Status 08--Fire Danger Warning 01--Yellow Fire Danger Alert

0901:Type&Status Weather Type-Status 09--Temperature Warning 01--Cold Weather Alert

0A01:Type&Status Weather Type-Status 10--Tsunami Warning 01--Tsunami Alert

F5:checksum

5.1.10 LORA Parameter Settings (0xCF)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xCF | See below for definition 见下方定义 | CK_sum |

Payload contents

| Byte size | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | count | -/- | -/- | How many sets of parameters are issued |
| 1 | U8 | type | -/- | -/- | Parameter type |
| 1 | U8 | content len | -/- |  | Content length |
| N | U8 | content | -/- | -/- | Parameter content |
| 1 | U8 | type | -/- | -/- | Parameter type |
| 1 | U8 | content len | -/- |  | Content length |
| N | U8 | content | -/- | -/- | Parameter content |

type: Parameter type:

type=01 Operating mode (After this parameter is set, the device will automatically restart)

| Byte size | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | Work mode | -/- | -/- | 00 – Class A, 01 – Class C |

type = 02 Join mode (This parameter setting will cause the device to restart automatically)

| Byte size | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | join mode | -/- | -/- | 00 – OTAA, 01 – ABP |

type = 03 Report mode (This parameter setting will cause the device to restart automatically)

| Byte size | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | report mode | -/- | -/- | 00 – confirm (requires ack response), 01 – unconfirm (does not require ack response) |

type = 04 Frequency band switching (only for high-frequency modules) (This parameter setting will cause the device to restart automatically)

| Byte size | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | cband | -/- | -/- | 00 — Default Frequency Band 01 — EU868 02 — US915 03 — AU915 04 — AS923_1 05 — AS923_2 06 — KR920 07 — IN865 |

In the reporting mode of type = 05 confirm, not every message will obtain an ACK; an interval time (in seconds) can be set.

| Byte size | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length is fixed at 4 |
| 1 | U32 | Min_interval | -/- | -/- | Interval time (seconds) 00— Each message must obtain ACK, 1~18000 – Interval time between two messages obtaining ACK during regular reporting |

type = 06 In OTAA network mode, the consecutive reporting failure count

| Byte size | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | Failure count | -/- | -/- | In OTAA network mode, the network process will restart only after a specified number of continuous reporting failures (range 0-20) |

type = 07 Fixed rate After this command is set, the default ADR is disabled (the device will automatically restart after this parameter is set)

| Byte size | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | cdata_rate | -/- | -/- | 00 — SF12, 01 — SF11, 02 — SF10, 03 — SF9, 04 — SF8, 05 — SF7 |

type = 08 ADR Enable (After this parameter is set, the device will automatically restart)

| Byte size | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | adr switch |  |  | 01 — ADR Enable, 00 — ADR Disable |

type = 09 Set Frequency Group (Channel) (After this parameter is set, the device will automatically restart)

| Byte size | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | channel |  |  | 00: Channel 0 ~ 7, 8: Channel 8 ~ 15, 16: Channel 16 ~ 23,… Note: 0xFF: Restore default frequency group |

Type = 10 In OTAA mode, restart module wait interval (in minutes) — rejoining time

| Byte size | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | Minutes |  |  | Interval minutes for restarting the module: 0 ~ 120 |

Example:

A total of 3 sets of parameters are sent: class A mode, ABP joining, unconfirmed reporting, ADR disabled

Note: After modifying the downlink, promptly adjust the lora ns configuration and match the parameters to enable normal communication

BDCF04010100020101030101080100dd

BD:Header

CF:Message ID

04: 04--indicates that there are 4 types of parameters being sent

01: type type=01 operating mode

01: Len parameter length 01 converted to decimal is 01, length is 1

00:  00--class A mode

02: type type=02 network entry mode

01: Len parameter length 01 converted to decimal is 01, length is 1

01  01--ABP network entry mode

03: type type=03 reporting mode

01: Len parameter length 01 converted to decimal is 01, length is 1

01 : 01--unconfirm reporting

08: type type=08 ADR Enable

01: Len parameter length 01 converted to decimal is 01, length is 1

00: 00--ADR Not Enabled

DD:checksum

Send 1 group Every 20 minutes, one transaction requires an ACK response message: BDCF010504B0040000DD

BD:Header

CF:Message ID

01: 01--Indicates that 1 type of parameter is sent

05: type type=05 In confirm reporting mode, not every message gets an ACK; set interval time

04: Len  Parameter length  04 converted to decimal 04, length is 4 bytes

B0040000:Min_interval converted to big-endian 000004B0-->1200 seconds

DD:checksum

5.2 Server Time Synchronization Information

5.2.1 Device Reporting Request Time Calibration Data

Note: This message is reported by the smartwatch

After the device is powered on, the watch will automatically send a request for time synchronization. It needs to receive a synchronization command and respond with a synchronized time data frame for device time synchronization.  
If the device had downlink calibration before power off, then after restarting, the device interface will display the time from before it was powered off, and the device will report FF00FF to request time calibration again.

| Byte offset | Format | Name | Description |
| --- | --- | --- | --- |
| 1 | U8 | HEADER | Fill: 0xFF |
| 1 | U8 | SeqId | Fill: 0x00 |
| 1 | U8 | End | Fill: 0xFF |

Example:

FF00FF    // The server must respond immediately after receiving, without delay

5.2.2 Time Calibration Request Data Reply

| Byte offset | Format | Name | Description |
| --- | --- | --- | --- |
| 1 | U8 | HEADER | Fill: 0xFF |
| 1 | U8 | SeqId | Fill: 0x10 |
| 2 | U16 | years | Year data |
| 1 | U8 | month | Month data |
| 1 | U8 | Day | Day, data |
| 1 | U8 | time | Clock, data |
| 1 | U8 | Minute | Minute, data |
| 1 | U8 | Seconds | Seconds, data |
| 1 | U8 | End | Fill: 0xFF |

Example:

After the server receives FF00FF, it replies with a downlink command to calibrate the time. Note: The example is for the year 2020 and only serves as a structural analysis example. The current year must be sent for it to be parsed. Otherwise, it will not be parsed. FF1007E409020B1B28FF

FF : Header

10 : Seqid

07E4: Year (2020)

09 : Month, (09)

02 : Day, (02)

0B : Hour, (11)

1B : Minitus, (27)

28 : Second, (40)

FF : End