G908GU-T-4GCat.1-tcp communication protocol

Directory

1 Overview	3

2 Device Usage Instructions	3

2.1 Device Function and Usage Instructions	3

2.2 Device default reporting logic	4

2.3 Device downlink description	5

3 Protocol Data Packet Structure	7

3.1 Data Header	7

3.2 Message ID	7

3.3 Token Generation Mechanism	7

3.4 Payload	8

3.5 Checksum	9

4 Report messages message	10

4.1 Connection-related reporting	10

4.1.1 LNK-LIN (MSGID=0xF0) Request connection (TCP only)	10

4.1.2 Connection Reply (MSGID=0xF1) (TCP Only) - Important	11

4.1.3 New Heartbeat Package Protocol (MSGID=0xF9) - Important	12

4.2 Alarm-related reporting	14

4.2.1 Alarm Data Upload-1 (MSGID=0x02)	14

4.2.2 Alarm data upload-2 (MSGID=0x21) (Supplement to 02)	15

4.3 Location-related reporting	17

4.3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)	17

4.3.2 Upload of WiFi and Base Station Information (MSGID=0xA4)	19

4.3.3 Bluetooth Location Information (LBE Location) (MsgId=0xD6)	24

4.4 Device Information and Status Reporting	26

4.4.1 ICCID Upload of SIM Card (MSGID=0xF3)	26

4.4.2 Device Charging Status Upload (MSGID=0xC3)	27

4.4.3 Status parameter reporting (MSGID=0xA9)	28

4.4.4 Device Status (MSGID=0xE9)	30

4.5 Downlink Feedback Related Reporting	31

4.5.1 Downlink Feedback (MSGID=0xC0)	31

5 Settings	33

5.1 Downlink	33

5.1.1 Set the location reporting frequency (MSGID=0x17)	33

5.1.2 Settings - Positioning Priority Settings (0XCE01&0XCE02)	35

5.1.3 Settings - Switch Settings (0XCE04-24)	38

5.1.4 Audible and visual alarm settings (0XCE25)	42

5.1.5 IP & Domain Name Settings (0xC3) (TCP Only)	44

1 Overview

This protocol is suitable for 4G CAT1 devices, currently supporting the G908GU-T product, mainly used with W300GU or W200PGU. It uses a 32-bit data header for synchronization and terminal identification; it uses a low-overhead checksum algorithm for checksum protection; it uses message identifiers to identify different messages.
It needs to be parsed according to the actual report. The document is currently being continuously maintained. If any errors or omissions are found, please provide feedback in time. Thank you very much.
The protocol content is the 4g reporting content. If you need the Bluetooth broadcast data protocol, please consult relevant personnel.

2 Device Usage Instructions

2.1 Device Function and Usage Instructions

General version:

(1) Power on:

Please fully charge the device before using it for the first time. It will automatically power on when charging. Charging light effect: red light flashes, fully charged light effect: green light is always on.    Manual power on: Manual power on: Long press the power button for 5 seconds, power on light effect: red and green lights flash

(2) Power off:

Charging: Automatic shutdown    Manual shutdown: Shutdown: In the power-on state, switch the power button and the left button back and forth three times, right-left-right-left-right-left, short press the left button and the red light does not light up, indicating that the locator has been turned off.    Low battery shutdown: Green and red lights flash and then go out    Device low battery status: Lights flash every 5s

(3) Signal status:

Not networked: Short press the left button once, the light effect is red. Networked: Short press the left button once, the light effect is green.

(4) Device hibernation:

The default trigger hibernation is on. Trigger condition: The device is stationary for 40 minutes to enter hibernation, and it does not hibernate in other cases.

If the device does not move for 12 hours, it will actively report a location and heartbeat package.

2.2 Device default reporting logic

General version:

(1) Connection related reporting

F0 request: The device is a long connection. Under normal server connection and network conditions, it will report once at startup - a reply is required to connect to the server.

F9: Heartbeat package reporting, the default is to report once every 4 minutes (previous firmware). A message will also be sent after location and health reporting - a reply is required to maintain a long connection.

(2) Location-related reporting - Note: Location is not reported by default. A downlink command is required to turn on the data reporting switch.

GPS / WiFi / Bluetooth beacon: Default reporting frequency 10 minutes, default positioning priority: WiFi > GPS, WiFi positioning is preferred, switch to GPS positioning if WiFi positioning is not available

(3) Alarm-related reporting

Shutdown alarm (0x21): The device actively shuts down or shuts down due to low battery. See the previous section for the triggering method.

Low battery alarm (0x02): Triggered when the device's current battery level is less than or equal to 0.

(4) Device information and status reporting

Status parameters (0xA9): A record will be reported when the device is turned on.

SIM card ICCID (0xF3): A record will be reported when the device is turned on.

Device status (0xE9): A record will be reported when the device is turned on, and a record will be reported when the reporting frequency is changed.

Charging status (0xC3): Report when charging starts, charging ends, and when fully charged---report when charging and turning on.

The default is charging shutdown, and the charging status is not reported when charging shutdown.

(5) Downlink feedback

Downlink feedback (0xC0): The server's downlink command is reported after the device receives it.

Note that for short-link devices, the downlink command is only received by the device when the device reports.

Note: There are cases where devices report with packet aggregation, meaning that one data packet contains multiple complete messages. Be careful not to miss any. The messages are complete and will not be interrupted in the middle and appear in the next data packet.

Eg:bdbdbdbdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9bdbdbdbdf9010000006400002800000019a9cf61ca

This data packet contains (0xD6) Bluetooth positioning and (0xF9) battery signal messages.

(0xD6) Bluetooth positioning:bdbdbdbdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9

(0xF9) Battery signal:bdbdbdbdf9010000006400002800000019a9cf61ca

2.3 Device downlink description

General version: The downlink command needs to be sent twice continuously within 20 milliseconds to ensure success.
Note: For short-link devices, the downlink command is only received by the device when the device reports, and it takes effect in the next reporting cycle after receiving it.

(1) Device location reporting frequency distribution (0x17):—-Note: Location is not reported by default, and the data reporting switch needs to be turned on.

The default reporting frequency is 10 minutes, with a maximum of 1 minute and a minimum of 1440 minutes. After the device receives the downlink command, the device will follow the time period and frequency of the issued command.

Report data, report at the default reporting frequency outside the time period, such as: 00:00-18:00 2 minutes positioning report,

Then report at the default 10-minute reporting frequency outside the time period

(2) Device positioning priority delivery (0xCE01):—-Note that the default is not to report positioning, you need to turn on the data reporting switch

Default positioning priority wifi>gps, positioning priority

If the issued positioning priority is: wifi>gps>Bluetooth beacon, then wifi cannot be located to switch gps, gps cannot be located to switch

Bluetooth beacon, when the positioning is successful, it will not switch to the next positioning priority to generate positioning

(3) Modify ip and port instruction delivery (0xC3):

The default general version points to the Smart Cloud Platform: 118.178.184.219:8825. If you need to change it, please consult relevant docking personnel or visit the official website

(4) Bluetooth broadcast switch delivery (0xCE05):

The default is on, and the device will not perform Bluetooth broadcast after it is turned off

(5) Data reporting switch (0xCE09):

The default reporting is off, and it can be turned off by downlink. After it is turned off, only the F9 heartbeat packet is reported, and no other data is reported.

UWB ranging alarm flashing light can be triggered normally, but the alarm is not reported to the server

(6) Key shutdown switch command issued (0xCE16):

By default, the device can be shut down via a key. After the downlink is turned off, the device cannot be shut down via a key.

(7) Control device to trigger sleep switch issued (0xCE18):

The default is on. The device enters sleep mode if it is not moved for 40 minutes. After the downlink is turned off, the device does not enter sleep mode.

(8) Control device charging switch state (0xCE21):

The default charging is off. After the downlink is turned on, the device does not shut down when charging, and the device reports the charging status.

(9) Sound and light alarm settings (0xCE25):

Immediately trigger an audible and visual alarm. After the downlink command is received, both the sound and the light will activate, but no message will be reported to the server.

Close the audible and visual alarm. The downlink command receives a triggered UWB ranging alarm and will not produce audible and visual signals.

Open the audible and visual alarm. The downlink command receives a triggered UWB ranging alarm and will produce audible and visual signals (the default is the on state, and the alarm duration is 5 seconds).

(10) UWB broadcast switch (0xCE29):

The default is on. After it is turned off, it is equivalent to turning off the UWB function. G908GU cannot be scanned and ranged by W300GU/W200PGU.

3 Protocol Data Packet Structure

A basic protocol data packet structure is shown in Figure 1:

3.1 Data Header

Each data packet starts with a 4-byte Header or token (in some response messages, timestamp is used instead):

Currently, the token uploaded by OuFu devices is fixed at BDBDBDBD

Header: 0xBD0xBD0xBD0xBD；

Timestamp: 32bits, generated by the server

3.2 Message ID

See Chapter 4 for the content represented by MessgeId.

Each time TCP is connected, the device will first report the 0xF0 message, which contains the device's unique identifier IMEI.

The server needs to record the IMEI as an identifier and reply with the 0xf1 message. The device will consider the connection successful; otherwise, the connection will be disconnected.

For normal device communication, the long connection reports an 0xF9 heartbeat packet every 4 minutes by default. The 0xF9 heartbeat packet is reported once after the health location is reported.

For short connections, the 0xF9 heartbeat packet is reported once at startup, and the 0xF9 heartbeat packet is reported once after the health location is reported. The two reports are not affected by each other.

Note that after the server receives the F9 message, the server needs to issue a command to reply; otherwise, the connection will be disconnected.

3.3 Token Generation Mechanism

Currently fixed as BDBDBDBD

3.4 Payload

The payload we refer to below refers to the valid body content in the protocol, excluding the head token and checksum. The length of the body is noted in parentheses.

The data format used in the payload is shown in the table below:

【U-unsigned；I-signed；X-bitfield；Number-bytes occupied】

The following protocols use little-endian priority except for ch, u8, i8, and x8

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Explanation |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short Integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short Integer |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Integer |
| x16 | Bitfield | 2 | - | - | Bit 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long Integer |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long Integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit long integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating point type |

3.5 Checksum

The content added to the checksum includes the payload, as shown in Figure 1. The algorithm is as follows. By default, the general-purpose version of the device does not require checksum verification, and this part can be ignored. Any byte can be used for the downlink instruction.

//Assuming content="BDBDBDBDE9000600010A00000A00", then the sum.ToString("X2") result is 07//The following is the C# code calling method

private string CheckSum(string content) {

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2");

}

4 Report messages message

4.1 Connection-related reporting

4.1.1 LNK-LIN (MSGID=0xF0) Request connection (TCP only)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xF0 | See below见下方定义 | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | u64 | IMEI | 1 | - | IMEI number (little endian) |
| 2 | x16 | version |  | - | Bitfield see below |

The device's reported F0 request message must have a server downlink 0XF1 message in response (see the next section for the specific format); otherwise, login will fail.

Example:

Original16hexadecimal message: BDBDBDBDF09B51731BC61603000000C2

BDBDBDBD:4byte message headerHeader

F0:Message ID(message id)9B51731BC6160300: IMEI number(little-endian mode), converted to big-endian:000316C61B73519B,

The corresponding decimal is IMEI:869465050010011,0000: version

C2: checksum

Connection-related Q&A:

Each time a new connection is created via TCP, an F0 request is reported first. The F0 contains the IMEI, and then the server records this IMEI.

After that, all the data in this connection belongs to this IMEI.

Note: If you do not receive F0 or experience F0 communication errors, you can use a third-party network testing tool to verify whether server communication is normal.

4.1.2 Connection Reply (MSGID=0xF1) (TCP Only) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Timestamp (unix) timestamp | 0xF1 | See below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | u32 | Token | 1 | - | Token: Fixed as BDBDBDBD |

Note that the message order is different from the normal reported message. This message is sent by the server, not the content reported by the device. The server reply needs to be replied in the current channel > Note that the message order is different from the normal reported message. This message is sent by the server, not the content reported by the device. The server reply needs to be replied in the current channel

Example:

Original16hexadecimal message:07FD8860F1BDBDBDBD2E

07FD8860(little endian),Timestamp(unix) timestamp (unit: seconds), converted to big endian6088FD07 converted to decimal:1619590407

Convert to standard time formatUTCTime:2021-04-28 06:13:27

F1:Message ID(Message ID)

BDBDBDBD:Token

2E:checksum

Timestamp: Convert the base 10 to standard time format. It is the result of adding 1619590407 seconds to 1970-01-01 00:00:00. The device uses the timestamp (in seconds) in this message to synchronize the time.

Note: The unit in Java is generally milliseconds. Pay attention to convert to the unit: seconds.

Note: The total number of bytes of the server downlink reply is 10 bytes, such as: 0xBD--1 byte; if the device receives 20 bytes, the connection will not be established.

4.1.3 New Heartbeat Package Protocol (MSGID=0xF9) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xF9 | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery type 0: 4-level system, 1: 5-level system, 2: percentage, 3: voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery value If Bat_type is 0: the battery value range is 0-3 (0 is 25%, 3 is 100%) If Bat_type is 1: the battery value range is 0-4 (0 is 20%, 4 is 100%) If Bat_type is 2: the battery value range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: percentage, 1: 5-level system 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal Strength |
| 1 | U8 | Other_type |  |  | Extended type 0: Full step counting 1: Incremental step counting 2: Vibration |
| 4 | U32 | Num |  |  | Extended value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Note: The F9 heartbeat packet must have a server downlink reply. The device considers the server connection to be unbroken only after receiving the server downlink reply; otherwise, it will re-report the F0 request to connect.

Example:

Original16hexadecimal message: BDBDBDBDF90104000050000095000000E377BD67AA

BDBDBDBD: Header

F9:Message ID

01:Bat_type battery type 01-->1-->5-level battery (0-4)

0400:Bat_volt battery value converted to big-endian 0004-->4-->Level4corresponds to100% battery

00:Signal_type signal type 00-->0-->percentage

5000:Signal_strength signal strength converted to big-endian0050-->converted to decimal80-->80% signal strength

00:Other_type extension type 00-->0-->full step counting

95000000:Num extension value - step counting converted to big-endian00000095-->converted to decimal149-->number of steps is149

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10base1740470243-->timestamp is1740470243seconds--> converted to standard time format UTC time:2025-02-25 07:57:23-->converted to Beijing time:2025-02-25 15:57:23

AA:checksum

Note: The server receives the F9 heartbeat packet and can reply with a fixed message: Fixed reply example BDBDBDBDF301 Number of bytes:6

4.2 Alarm-related reporting

4.2.1 Alarm Data Upload-1 (MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x02 | See below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below for definition |
| 4 | U32 | Timestamp | - | - | Timestamp (timestamp will be added at the end during retransmission) |

Upl_warn contents

| bit (corresponding to the position of 1 in binary) | Name | Description | Original message in little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Shutdown | Shutdown | 0400 | 0004 | 4 |
| 0 | Low battery | Low battery | 0100 | 0001 | 1 |

Example:

Low battery alarm: BDBDBDBD020200E377BD678A

BDBDBDBD:Header

02:Message ID

0100:Upl_warn alarm content, converted to big-endian0001-->Low battery alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8A：checksum

Shutdown alarm: BDBDBDBD020400E377BD678A

BDBDBDBD:Header

02:Message ID

0400:Upl_warn alarm content, converted to big-endian0004-->Shutdown alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8A：checksum

Note: This alarm is for devices that do not distinguish between shutdown types. See the next section for devices that distinguish between shutdown types.

4.2.2 Alarm data upload-2 (MSGID=0x21) (Supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x21 | See below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | Alarm type |
| N |  | Upl_warn | - | - | Alarm content See below for different Alarm type type definitions |
| 4 | U32 | Timestamp |  |  | Timestamp (timestamp will be added at the end during retransmission) |

Alarm type=1

Upl_warn contents

| bit (corresponding to the position of 1 in binary) | Name | Description | Original message in little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging shutdown alarm | Automatic shutdown when device is charging (firmware support required) | 04000000 | 0004 | 4 |
| 1 | Low battery shutdown alarm | Device shuts down due to low battery (firmware support required) | 02000000 | 0002 | 2 |
| 0 | Active shutdown alarm | Device is manually shut down by someone (firmware support required) | 01000000 | 0001 | 1 |

Example:

Charging shutdown alarm: BDBDBDBD21010004000000E377BD6767

BDBDBDBD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001-->Alarm type=1

04000000:Upl_warn alarm content, converted to big-endian 0004-->Charging shutdown alarm

E377BD67:Timestamp timestamp is converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Low battery shutdown alarm: BDBDBDBD21010002000000E377BD6767

BDBDBDBD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001-->Alarm type=1

02000000:Upl_warn alarm content, converted to big-endian 0002 --> low battery shutdown alarm

E377BD67:Timestamp timestamp is converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Active shutdown alarm: BDBDBDBD21010001000000E377BD6767

BDBDBDBD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001-->Alarm type=1

01000000:Upl_warn alarm content, converted to big-endian 0001 --> active shutdown alarm

E377BD67:Timestamp timestamp is converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

4.3 Location-related reporting

4.3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x03 | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude |
| 8 | Double | lat |  | - | latitude |
| 1 | U8 | north_south |  | - | /N or S/ |
| 1 | U8 | east_west |  | - | /E or W/ |
| 1 | U8 | status |  | - | /A or V/ Valid data or invalid data |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original hexadecimal message: BDBDBDBD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BDBDBDBD:Header

03:Message ID message id

963E870C3E675E40:lon longitude Convert to big endian405E673E0C873E96-->Longitude:121.6131622 Note: Refer to the parsing example below for conversion and parsing

FF2110C2B6343F40:lat latitude Convert to big endian `403F34B6C21021FF-->Latitude:31.2059137 Note: Refer to the parsing example below for conversion and parsing

4E:north_south 4E-->Convert to ASCII code N-->North latitude

45:east_west 45-->Convert to ASCII code E-->East longitude

41:status data validity 41-->Convert to ASCII code A-->Valid data

E377BD67:Timestamp timestamp Convert to big endian 67BD77E3-->Convert to decimal101740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

78: checksum

Note: The parsed GPS latitude and longitude are in the WGS-84 coordinate system. If the map uses Baidu AMAP, etc., the coordinate system needs to be converted.

Parsing satellite positioning message Java code example:

//Message: BDBDBDBD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BDBDBDBD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public static void main(String[] args){

//Message 7d9f84ac81815c40 Actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d",16)));  //114.02353966666665//Message e766926b1d893640 Actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16))) ;//22.535605166666667

HexToStr(data.Substring(“4e”)); //N

HexToStr(data.Substring(“45”));//E

HexToStr(data.Substring("41"));//A   A indicates valid data, while V indicates invalid data, which can be discarded. //Message 749d695f actual value 5f699d74

Date date = new Date();

date.setTime(Long.parseLong("5f699d74",16)*1000);

SimpleDateFormatsdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.3.2 Upload of WiFi and Base Station Information (MSGID=0xA4)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xA4 | See below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 1 | u8 | Cell_cnt | 1 | - | Number of base stations (0-7) Number of cell info payload.Valid value:0~7 |
| 2 | u16 | Cell[0].MCC | - | - | MMC of base station (0) mobile country code of cell[0] |
| 2 | u16 | Cell[0].MNC | - | - | MNC of base station (0) mobile network code of cell[0] |
| 2 | u16 | Cell[0].LAC | - | - | LAC of base station (0) Location area code of cell[0] |
| 4 | U32 | Cell[0].CELL_ID | - | - | ID of base station (0) Cell id of cell[0] |
| 2 | i16 | Cell[0].RSSI | - | dbm | Signal strength RSSI of base station (0) RSSI in dbm of cell[0] |
| ... | ... | ... | ... | ... | ... |
| 2 | u16 | Cell[cell_cnt-1.MCC | - | - | MMC of base station (N) mobile country code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.MNC | - | - | MNC of base station (N) mobile network code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.LAC | - | - | LAC of base station (N) Location area code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1].LAC | - | - | LAC of base station (N) Location area code of cell[[cell_cnt-1] |
| 4 | U32 | Cell[cell_cnt-1].CELL_ID | - | - | ID of base station (N) Cell id of cell[[cell_cnt-1] |
| 2 | I16 | Cell[cell_cnt-1].RSSI | - | dbm | RSSI signal strength of base station (N) RSSI in dbm of cell[[cell_cnt-1] |
| 1 | U8 | Wifi_cnt |  |  | Number of wifi Number 0f wifi |
| 6 | 6*U8 | Wifi[0].bssid[0]-Wifi[0].bssid[5] |  |  | MAC address of wifi(0) |
| 4 | I32 | Wifi[0].RSSI |  |  | RSSI signal strength of wifi(0) |
| ... | ... | ... | ... | ... | ... |
| 6 | 6*U8 | Wifi[Wifi_cnt-1].bssid[0]-Wifi[Wifi_cnt-1].bssid[5] |  |  | MAC address of WiFi(N) |
| 4 | I32 | Wifi[0].RSSI |  |  | RSSI signal strength of wifi(0) |

Note: The device report does not necessarily have a base station every time. The absence of a base station does not affect positioning. If the number of base stations in the report is 0, then the number of Wi-Fi connections will follow directly.

Example:

Original16hexadecimal message: BDBDBDBDA4E377BD6701CC010000C218D1AD160BF9FF078CBEBE1A8162C6FFFFFFC061180AF42AC1FFFFFFE005C5B1F824CCFFFFFFE8FCAFA02663AFFFFFFF6409805B2B9CAEFFFFFF94200BC726E000B6FFFFFFA8154DF6517EB2FFFFFFE005C5B1F824CCFFFFFFE8FCAFA02663AFFFFFFF6409805B2B9CAEFFFFFF94

BDBDBDBD:Header

A4: Message ID

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

01: Cell_cnt number of base stations 01 --> number of base stations is 1

CC01: Cell[0].MCC Base station MCC converted to big-endian 01CC--> converted to decimal 460 --> Base station MCC is 460

0000: Cell[0].MNC Base station MNC converted to big-endian 0000 --> converted to decimal 0 --> Base station MNC is 0

C218: Cell[0].LAC Base station LAC converted to big-endian 18C2--> converted to decimal 6338 --> Base station LAC is 6338

D1AD160B: Cell[0].CELL_ID Base station ID converted to big-endian 0B16ADD1--> converted to decimal 186035665 --> Base station ID is 186035665

F9FF: Cell[0].RSSI Base station signal strength converted to big-endian FFF9--> converted to decimal complement -7 --> Base station signal strength is -7

07: Wifi_cnt number of wifi connections 07 --> 7 wifi connections

8CBEBE1A8162:wifi mac address 8C:BE:BE:1A:81:62-->The mac address of wifi[0] is8C:BE:BE:1A:81:62

C6FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFC6-->converted to decimal complement-58--->The RSSI signal strength of wifi[0] is -58

C061180AF42A:wifi mac address C0:61:18:0A:F4:2A-->The mac address of wifi[1] is C0:61:18:0A:F4:2A

C1FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFC1-->converted to decimal complement-63--->The RSSI signal strength of wifi[0] is -63

200BC726E000:wifi mac address 20:0B:C7:26:E0:00-->The mac address of wifi[2] is20:0B:C7:26:E0:00

B6FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFB6-->converted to decimal complement-74--->The RSSI signal strength of wifi[0] is -74

A8154DF6517E:wifi mac address A8:15:4D:F6:51:7E-->The mac address of wifi[3] is A8:15:4D:F6:51:7E

B2FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFB2-->converted to decimal complement-78--->The RSSI signal strength of wifi[0] is -78

E005C5B1F824:wifi mac address E0:05:C5:B1:F8:24-->The mac address of wifi[4] is E0:05:C5:B1:F8:24

CCFFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFCC-->converted to decimal complement-52--->The RSSI signal strength of wifi[0] is -52

E8FCAFA02663: WiFi MAC address E8:FC:AF:A0:26:63--> The MAC address of wifi[5] is E8:FC:AF:A0:26:63

AFFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFFFAF --> converted to decimal complement -81 ----> WiFi [0] RSSI signal strength is -81

6409805 B2B9C: WiFi MAC address 64: 09: 80: 5B: 2B: 9C--> WiFi [6] MAC address is 64: 09: 80: 5B: 2B: 9C

AEFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFFFAE--> converted to decimal complement -82 ---> WiFi [0] RSSI signal strength is -82

94: checksum

The parameters parsed from the message need to be connected to the map WiFi database, and the latitude and longitude data is finally obtained through the map side. Take the AMAP connecting to the WiFi database as an example:

According to the access network method, AMAP WiFi positioning protocol document is divided into three positioning scenarios. The following gives corresponding examples for the three positioning scenarios.

1. The network connection method is WiFi access, that is, the accesstype parameter is 1

http://apilocate.amap.com/position?accesstype=1&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&mmac=50:a7:2b:1e:56:58,-60,alibaba-inc&macs=4c:48:da:25:0b:11,-59,alibaba-

inc|4c:48:da:25:1a:11,-77,alibaba-inc&serverip=10.2.166.4&output=xml&key=<用户 Key>

2. When the network connection method is mobile access and the mobile phone card is a non-CDMA card, that is, when the accesstype parameter is 0 and cdma=0:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=0&bts=460,01,40977,2205409,-65&nearbts=460,01,40977,2205409,-65|460,0

1,40977,2205409,-65|460,01,40977,2205409,-65&serverip=10.2.166.4&output=xml&key=<User Key>

3. When the network connection method is mobile access and the mobile phone card is a CDMA card, that is, when the accesstype parameter is 0 and cdma=1:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=1&bts=13824,1,1838,1674723,575739,-52&serverip=10.2.166.4&output=xml&key=<User Key>

W200P watch: AMAP WiFi positioning protocol calls AMAP interface description: When selecting parameters, generally select the first type

You can also select the second type, but if the device does not report the base station, the bts and neabts parameters need to be removed before it can

The first type, example of incoming parameters: mmac is the first WIFI mac address in the messagehttp://apilocate.amap.com/position?accesstype=1&imei=869907057780989&mmac=04:8C:16:A9:FB:A8,-44,TP-Link&macs=04:8C:16:A9:FB:A8,-44,TP-Link|C8:3A:35:F4:CE:18,-59,TP-Link|1C:78:4E:D5:07:FE,-79,TP-Link|C8:3A:35:9D:0B:E0,-87,TP-Link|4A:D8:90:E8:0A:82,-88,TP-Link|AC:AD:4B:C9:C3:87,-89,TP-Link|A4:16:E7:55:84:AC,-91,TP-Link&output=json&key=xxxx

For the specific AMAP WiFi positioning protocol PDF, you can click to view and download:(not a screenshot, you can click to view)
This document is provided by AMAP, and you can directly ask AMAP for specific docking
Note that the name of the downloaded document will change, you can rename it yourself

4.3.3 Bluetooth Location Information (LBE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xD6 | See below | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 0 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple sets of information, and each group may contain multiple ibeacons |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| ... | ... | ... | ... |

Note: In the general version, the total number of Bluetooth beacon groups is fixed at 1, and the maximum number of Bluetooth beacons is 4. The sorting is based on signal strength from strongest to weakest. By default, the watch only recognizes our own beacons. If you need to connect to other manufacturers' beacons, please consult relevant personnel via WeChat.
Example:

Original hexadecimal message: BDBDBDBDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

bdbdbdbd: Header

D6: Message ID

00: type

01: Only one set of ibeacon data

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->The timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

04: The first group has 4 beacon information

4327: Convert to big-endian-->major : 0x2743-->Convert to decimal 10051

1794: Convert to big-endian-->minor: 0x9417-->Convert to decimal 37911

AC:  RSSI 0xAC-->Convert to decimal complement -84

4327: Convert to big-endian-->major: 0x2743-->Convert to decimal 10051

3094: Convert to big-endian-->minor:0x9430-->Convert to decimal 37936

AA:  RSSI 0xAA-->Convert to decimal complement -86

4327:Convert to big-endian-->major: 0x2743-->Convert to decimal 10051

b956:Convert to big-endian-->minor:0x56b9-->Convert to decimal 22201

A5:   RSSI 0xA5-->Convert to decimal complement -91

4327: Convert to big-endian-->major: 0x2743-->Convert to decimal 10051

FE94: Convert to big-endian-->minor:0x94FE-->Convert to decimal 38142

A5: rssi:0xA5-->Convert to decimal complement -91

6a --Checksum

Bluetooth beacon major, minor description: The default Bluetooth beacon comes with a beacon code.

Like: 00081005345866, then the major of the Bluetooth beacon is 10053, the minor is 45866, and the manufacturer ID is 0008 (this is useless and has no practical significance)

4.4 Device Information and Status Reporting

4.4.1 ICCID Upload of SIM Card (MSGID=0xF3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xF3 | See below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 10 | 10*U8 | ICCID | 1 | - | ICCID number - ICCID number of the device SIM card |

Note: This message will be reported once every time the device is powered on.

Example:

Original hexadecimal message:BDBDBDBDF389861118236001639994CC

BDBDBDBD:Header

F3:Message ID

89861118236001639994:ICCID Device SIM card number ICCID is: 89861118236001639994

CC:checksum

4.4.2 Device Charging Status Upload (MSGID=0xC3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Status | / | / | 0 start, 1 end, 2 fully charged |
| 1 | U32 | Timestamp | / | / | Timestamp (timestamp will be added at the end during retransmission) |

Note: This message requires device firmware support. The default latest general version supports it. This message will only be reported when the downlink command is enabled: charging and booting.
Example:

Original hexadecimal message:BDBDBDBDC301E377BD678A

BDBDBDBD:Header

C3:Message ID

01:Device finished charging

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->The timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

Original hexadecimal message:BDBDBDBDC300DB4D2F668A

BDBDBDBD:Header

C3:Message ID

00:Device starts charging

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->The timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

Original hexadecimal message:BDBDBDBDC302DB4D2F668A

BDBDBDBD:Header

C3:Message ID

02:Device is fully charged

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->The timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

4.4.3 Status parameter reporting (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Number of parameter types |
| 1 |  |  |  |  | (Reserved 00) |
| 1 | u8 | Type | -/- | -/- | Parameter type 1, Type definition: 00—MCU (firmware), 01—Module, 02 Sensor (this type is not available in the general version) |
| 1 | U8 | NameLen | -/- | -/- | Parameter name 1 length |
| n | N*u8 | Name | -/- | -/- | Parameter name |
| 1 | u8 | Type | -/- | -/- | Parameter type 2, Type definition: 00—MCU (firmware), 01—Module, 02 Sensor (this type is not available in the general version) |
| 1 | U8 | NameLen | -/- | -/- | Parameter type 2 length |
| n | N*u8 | Name | -/- | -/- | Parameter name |
| ... | ... | ... | ... | ... | ... |

Note: This message is reported once by default at startup, uploading the device firmware version number and module version number, which can be left unparsed. The Message ID of the previously shipped device was 0xBB, and relevant personnel can be contacted to send it.

Example:

BDBDBDBDA902000009423732352E4F56303601174E5432364B434E4230304E4E412D4C3032303330393530B5

BDBDBDBD:Header

A9:Message ID

02:TypeCnt indicates that there are 2 types of parameters

00:Reserved field

00:Type=00 00 represents MCU (firmware)

09:The following MCU name length is 9 bytes

423732352E4F563036--->Hexadecimal to text (device firmware version number)--->B725.OV06

01:Type=01 01 represents the module

17:The following communication module name length is 23 bytes

4E5432364B434E4230304E4E412D4C3032303330393530:Hexadecimal to text (4G module version number)

--->NT26KCNB00NNA-L02030950

B5---checksum

4.4.4 Device Status (MSGID=0xE9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xE9 | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | type |  | / | Default—00 |
| 2 | u16 | length |  | / | Length of the following message |
| 1 | u8 | location |  | / | Location reporting frequency: 00—-default not modified, 01—-modified by downlink (if there are multiple time periods in the downlink, only the reporting frequency of the current time period is reported) |
| 2 | u16 | frequency |  | / | Frequency, unit - minutes |
| 1 | u8 | health |  | / | Health reporting frequency: 00 - default not modified, 01 - modified by downlink |
| 2 | u16 | frequency |  | / | Frequency: unit - minutes |

Note: This message is reported once at startup, and once after the downlink modifies the frequency.

Example:

Original hexadecimal message:BDBDBDBDE9000600010A00010A0033

BDBDBDBD:Header

E9:Message ID

00:type=00

0600:length The length of the message behind it is converted to big-endian -->0006--> converted to decimal 6, and the message length is 6 bytes

01:location 01 indicates that the location reporting frequency has been modified by the downlink

0A00:frequency Health reporting frequency converted to big-endian -->000A--> converted to decimal 10, the reporting frequency is 10 minutes

01:health 01 indicates that the health reporting frequency has been modified by the downlink

0A00:frequency Health reporting frequency converted to big-endian -->000A--> converted to decimal 10, the reporting frequency is 10 minutes

33:checksum

If the device has no health reporting function or no positioning reporting function, the two values of health reporting frequency and positioning reporting frequency will remain the same, indicating the device reporting frequency.

4.5 Downlink Feedback Related Reporting

4.5.1 Downlink Feedback (MSGID=0xC0)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xC0 | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID Length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This command is used for feedback of downlink commands, returning the Message ID of the downlink command received earlier (multiple Message IDs can be returned collectively).

Original hexadecimal message:BDBDBDBDC0011720

BDBDBDBD:Header

C0: Message ID

01:length Message ID length Convert decimal 01--> Message ID length is 1 byte

17Type Indicates that the device receives the downlink command Message ID is 17

20:checksum

5 Settings

5.1 Downlink

5.1.1 Set the location reporting frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x17 | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time Interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time Interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time Interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time Interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |

Note: One day is 00:00-23:59
Example:

00:00-23:59 Report location every10minutes: BDBDBDBD17010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

17:Message ID

01:enable is enabled  Convert to decimal01-->1--Enabled

0A00:Interval time interval Convert to big-endian000A-->Convert to decimal10-->Time interval is10minutes

00:time_start_h Start time-hour, convert to decimal00-->Start time-hour is00

00:time_start_m Start time-minute, convert to decimal00-->Start time-minute is00

17:time_end_h End time-hour, convert to decimal23-->End time-hour is 23

3B:time_end_m End time-minute, convert to decimal59-->End time-minute is 59

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

97:checksum

5.1.2 Settings - Positioning Priority Settings (0XCE01&0XCE02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xCE | See below | CK_sum |

Payload contents

Type=01 Positioning priority setting

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 is always valid, 01 is valid this time |
| 2 | U16 | Len | -/- | -/- | Length of subsequent instruction |
| n | n |  | -/- | -/- | Main body |

Definition of positioning priority setting--See2.3 Device downlink description--Device positioning priority delivery (0xCE01)

Positioning priority setting text: Currently, the general version supports:01--GPS,02--wifi(wifi+base station) 03--Bluetooth beacon (requires additional deployment of Bluetooth beacons);

Special equipment support:04--LBS base station 05--125k

Example:

Wifi positioning priority (wifi>Bluetooth beacon>GPS):BDBDBDBDCE0100030002030133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

020301:Text: 02--wifi 03--Bluetooth beacon 01--GPS Positioning priority is wifi>Bluetooth beacon>GPS

33:checksum

GPS positioning priority (GPS>wifi>Bluetooth beacon):BDBDBDBDCE0100030001020333

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

010203:Text: 01--GPS 02--wifi 03--Bluetooth beacon Positioning priority is GPS>wifi>Bluetooth beacon

33:checksum

Bluetooth positioning priority (Bluetooth beacon>wifi>GPS):BDBDBDBDCE0100030003020133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

030201:Text: 03--Bluetooth beacon 02--wifi 01--GPS Positioning priority is Bluetooth beacon>wifi>GPS

33:checksum

Single GPS positioning:BDBDBDBDCE0100030001000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

030201:Text: 01--GPS 00--No positioning 00--No positioning Positioning priority is single GPS positioning

33:checksum

Single wifi positioning:BDBDBDBDCE0100030002000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

020000:Text: 02--wifi 00--No positioning 00--No positioning Positioning priority is single wifi positioning

33:checksum

Single Bluetooth beacon positioning:BDBDBDBDCE0100030003000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

020000:Text: 03--Bluetooth beacon 00--No positioning 00--No positioning Positioning priority is single Bluetooth beacon positioning

33:checksum

5.1.3 Settings - Switch Settings (0XCE04-24)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xCE | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Please refer to the instructions for specific types |
| 1 | u8 | switch | -/- | -/- | Switch 00 on, 02 off, 01 single effective |
| 2 | U16 |  | -/- | -/- | Fixed 0000 |

Device alarm setting - switch type: specific function description--see 2.3 Device downlink description

05 Bluetooth broadcast switch

16 Whether the downlink can use the key to shut down

18 Sleep function switch

19 Key trigger SOS alarm switch

21 Charging is power on or power off state

29 UWB broadcast switch

Type=05 Bluetooth broadcast switch
Example:

00---On:BDBDBDBDCE0500000093

BDBDBDBD:Header

CE:Message ID

05:Type 05 Bluetooth broadcast switch

00:switch switch 00 on

0000:Fixed value

93:checksum

02---Off:BDBDBDBDCE0502000093

BDBDBDBD:Header

CE:Message ID

05:Type 05 Bluetooth broadcast switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=16 Whether the downlink can use the key to shut down
Example:

00---Open:BDBDBDBDCE1600000093

BDBDBDBD:Header

CE:Message ID

16:Type 16 Whether the downlink can use the key to shut down

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Off:BDBDBDBDCE1602000093

BDBDBDBD:Header

CE:Message ID

16:Type 16 Whether the downlink can use the key to shut down

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=18 Sleep function switch
Example:

00---On, enter sleep after a period of stillness, and do not report data:BDBDBDBDCE1800000093

BDBDBDBD:Header

CE:Message ID

18:Type 18 Sleep function switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Off, Still or charging state, also normal reporting data:BDBDBDBDCE1802000093

BDBDBDBD:Header

CE:Message ID

18:Type 18 Sleep function switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=19 Key trigger sos alarm switch
Example:

00---Open:BDBDBDBDCE1900000093

BDBDBDBD:Header

CE:Message ID

19:Type 19 button triggers sos alarm switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE1902000093

BDBDBDBD:Header

CE:Message ID

19:Type 19 button triggers sos alarm switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=21 Charging is power on or power off
Example:

00---Charging is power on:BDBDBDBDCE2100000093

BDBDBDBD:Header

CE:Message ID

21:Type 21 Charging power on or charging power off

02:switch switch 00 Charging is power on

0000:Fixed value

93:checksum

02---Charging is power off:BDBDBDBDCE2102000093

BDBDBDBD:Header

CE:Message ID

21:Type 21 Charging power on or charging power off

02:switch switch 02 Charging is power off

0000:Fixed value

93:checksum

Type=29 UWB broadcast switch
Example:

00---Open:BDBDBDBDCE2900000093

BDBDBDBD:Header

CE:Message ID

29:Type 29 UWB broadcast switch

02:switch switch 00 on,

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE2902000093

BDBDBDBD:Header

CE:Message ID

29:Type 29 UWB broadcast switch

02:switch switch 02 off,

Note:UWB ranging function is off, W300GU/W200PGU cannot range with this device

0000:Fixed value

93:checksum

5.1.4 Audible and visual alarm settings (0XCE25)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xCE | See below | CK_sum |

Payload contents

| Format | Name | Unit | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Fixed at 0x25 |
| U8 | Alarmtype | 1 | 0x01—Immediate audible and visual alarm, 0x00—-Enable audible and visual alarm (followed by 0000), 0x02—-Disable audible and visual alarm (followed by 0000) |
| U16 | Len | 2 | Instruction length |
| U8 | Motor Type | 1 | Setting type: 1. Immediate audible and visual alarm - 0x00 |
| U32 | Value | 4 | Setting content: 1. Alarm duration (in milliseconds, range 0 - 60 seconds) |

Example:

Enable audible and visual alarm: BDBDBDBDCE2500000018

BDBDBDBD:Header

CE:Message ID

25:Type Audible and visual alarm settings

00:Alarmtype 00 Enable audible and visual alarm

00000:Fixed value

18:checksum

Disable audible and visual alarm: BDBDBDBDCE2502000018

BDBDBDBD:Header

CE:Message ID

25:Type Audible and visual alarm settings

02:Alarmtype 02 Close audible and visual alarm

00000:Fixed value

18:checksum

Immediately trigger an audible and visual alarm for a duration of 5 seconds:BDBDBDBDCE25010500008813000077

BDBDBDBD:Header

CE:Message ID

25:Type Audible and visual alarm settings

01:Alarmtype Immediate audible and visual alarm

0500:Len Convert to big-endian 0005-->Convert to decimal 05-->Instruction length 5 bytes

00:Motor Type 00 Immediate audible and visual alarm

88130000:Value Alarm duration Convert to big-endian 00001388-->Convert to decimal 5000-->Alarm duration: 5000 milliseconds

77:checksum

5.1.5 IP & Domain Name Settings (0xC3) (TCP Only)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | Issue type: Type =1 IPv4, Type =2 IPv6 (Reserved - not currently supported), Type =3 Domain Name (Currently only supports ASCII encoded domain names) |
| 2 | U16 | Port |  |  | Port number (2 bytes) |
| 1 | U8 | Len |  |  | Length of the following parameter content |
| N | U8 | Domainname |  |  | Specific IP or domain name content |

Note: After delivery, the device will not communicate on the original server, but will point to the modified server.

Example:

Type=1 IPv4:Modify the downlink IP to:118.178.184.219, port: 8825 BDBDBDBDC30179220476B2B8DB33

BDBDBDBD:Header

C3:Message ID

01:Type 1--IPv4

7922:Port Convert to big-endian 2279-->Convert to decimal 8825-->Port: 8825

04:Len Length of the following parameter content Convert to decimal 04-->Length is 4 bytes

76B2B8DB:Domainname IP 76-->Convert to decimal 118, B2-->Convert to decimal 178, B8-->Convert to decimal 184, DB-->Convert to decimal 219 IP is: 118.178.184.219

33:checksum

Type=3 Domain nameModify the downlink domain name to:aiday.com.cn, port: 8825

BDBDBDBD:Header

C3:Message ID

03:type=3, Domain Name

7922: Port port to big-endian --2279 --> to decimal port: 8825

12: Len The length of the parameter content behind is converted to decimal 18-->The message length is 18 bytes

61696461792e636f6d2e636e: Domainname domain name converted to string-->aiday.com.cn-->domain name is: aiday.com.cn

1D: checksum