Bluetooth AOA Broadcast Protocol

Overview:

Currently supports ultra-low power Bluetooth BLE5.1, supports docking with Bluetooth gateways, Bluetooth AoA high-precision gateways (Hesin IoT, Lan Ce, and other mainstream gateways); connects to Bluetooth devices; scans beacons and all other Bluetooth protocols and functions

1. AOA Broadcast Sending Format

| Field Name | Description |
| --- | --- |
| Header (0x02) | 1 octet, the message type is non-connectable advertising |
| Length (0x25) | This length refers to the effective payload data length in the PDU, excluding the header and length. |
| Send MAC address | 6 octets |
| Length (0x1E) | 1 octet, data length occupied by data type «Manufacturer Specific Data» |
| Data type (0xFF) | 1 octet, Manufacturer Specific Data |
| Company ID (0x0D00) | 2 octets |
| Packet ID (0x04) | 1 octet |
| User data | 6 octets, note that the last two bytes are the CRC16 checksum, which occupies the last two bytes of the User data, with the high byte sent before the low byte. Please refer to the user-defined data protocol below for the specific protocol. The CRC16 checksum result is the result of the operation of the first 15 bytes; that is, the calculation starts from the MAC address to the end of the user data. |
| DF field | BLE 37 channel sends the following content: 0x2F,0x61,0xAC,0xCC,0x27,0x45,0x67,0xF7,0xDB,0x34,0xC4,0x03,0x8E,0x5C,0x0B,0xAA,0x97,0x30,0x56,0xE6 BLE 38 channel sends the following content: 0x6F,0xD3,0x10,0x0F,0x38,0x72,0x2D,0xA8,0x5E,0xC2,0x58,0x99,0x4F,0x8A,0xCE,0xEE,0xB7,0x69,0x88,0x07 BLE 39 channel sends the following content: 0x50,0xBD,0x84,0xB1,0x32,0x9F,0x14,0x9D,0xDD,0x6F,0xD3,0x10,0x0F,0x38,0x72,0x2D,0xA8,0x5E,0xC2,0x58 |

2. AOA - Beacon end uplink data format

The specific format of User data is: command word (1Byte) + data content (3Bytes) + CRC16 (2Bytes)
Among them, command word 0x00--->configuration information; 0x08--->report acceleration information, command byte range 0x09-0x0f represents other user data, the specific content is shown below

Transmitting channel list bit occupation

Table (1):

| Field | Description |
| --- | --- |
| 000b ——->0 | 2401MHZ |
| 001b ——->1 | 2402MHZ |
| 010b ——->2 | 2426MHZ |
| 011b ——->3 | 2480MHZ |
| 100b ——->4 | 2481MHZ |

Transmission frequency rule list

Table (2):

Beacon Type Nordic Transmit Power Table

Table (3)

| Actual transmission power | Corresponding numerical value |
| --- | --- |
| 0 dBm | 0 |
| 3 dBm | 1 |
| 4 dBm | 2 |
| -40 dBm | 3 |
| -20 dBm | 4 |
| -16 dBm | 5 |
| -12 dBm | 6 |
| -8 dBm | 7 |
| -4 dBm | 8 |
| -30 dBm | 9 |

Beacon Type TI Transmit Power Table

Table (4):

| Actual transmission power | Corresponding numerical value |
| --- | --- |
| 0 dBm | 0 |
| 1 dBm | 1 |
| 2 dBm | 2 |
| 3 dBm | 3 |
| 4 dBm | 4 |
| 5 dBm | 5 |
| -3 dBm | 6 |
| -6 dBm | 7 |
| -9 dBm | 8 |
| -12 dBm | 9 |

Beacon end uplink data configuration

Table (5):

bit[0:3]: Specific implementation command, the specific meaning is as followsbit[4-5]: Tell the base station the current version number of the beacon end at this time;

00: Scheme 3 beacon (the original beacon scheme provides SDK, the default is to open the receiving window every 4 seconds for 5ms and also supports private frequency points, note that it is displayed as method 0 in CCS, the beacon end needs to report this value using this scheme)

01: Scheme 1 beacon (provides the SDK's current uplink and downlink alignment scheme, the default is to request downlink data from the base station every 1 second and also supports private frequency points, note that it is displayed as method 1 in CCS, the beacon end needs to report this value using this scheme)

10: Scheme 2 beacon (using the standard official website provided SDK, only supports 37 38 39 three channels, does not support private frequency points, note that it is displayed as method 2 in CCS, the beacon end needs to report this value using this scheme)

11: Reserved bit6: Tell the base station whether the beacon end is currently opening the receiving window;

0: Do not open the window

1: Open the window bit7: Request base station downlink whitening data or non-whitening data;

0: Non-whitening (Scheme 3 and Scheme 1 providing SDK, or completed through 2.4g private protocol)

1: Albinism (using the SDK provided by the standard official website)

Configuration Information Protocol (Command 0x00)

Note: CCS will parse and display it through this command as shown in the screenshot 1 below

Parameter command bit [0:3] ---- > 0x00, Bit [4:7] as shown in Table (5)

bit [0:2]: Transmitting channel (as shown in list 1 above, the actual work corresponds to five channels).

bit [3]: Whether the highest bit enables the receiving mode. 1 indicates that the receiving mode is enabled when power is on, and 0 indicates that the receiving mode is not enabled when power is on.

: bit [4:7]: Value range 0-9 corresponds to the beacon transmission power as shown in Table (3) and Table (4).

bit [0:2]: Beacon device type 000: TI, 001: Nordic, other numbers are reserved.

bit [3]: SOS alarm status value, 0 no alarm, 1 alarm.

bit [4:7]: Battery level in percentage, range 0-10.

bit[0:6]: Transmitting frequency, the rules are shown in the table (2).

bit[7]: ReservedCRCHigh ByteCRCLow Byte

Screenshot 1:

Acceleration Sensor Data Protocol

Note: The x, y, and z-axis data each occupy 8 bits, with a sign bit (8-bit resolution). The acceleration sensor measurement range needs to be set to ±2g.

| Serial Number | Description |
| --- | --- |
| 1 | bit[0:3] Parameter command 0x08, bit[4:7] reserved |
| 2 | Sensor Data x-axis |
| 3 | Sensor Data y-axis |
| 4 | Sensor Data z-axis |
| 5 | CRC High Byte |
| 6 | CRC low eight bits |

3. User-defined data protocol structure (User data)

| Serial Number | Description |
| --- | --- |
| 1 | Parameter command——>(0x09——>0x0F), bit[4:7] reserved. (1octet) |
| 2 | User-defined data (1octet) |
| 3 | User-defined data (1octet) |
| 4 | User-defined data (1octet) |
| 5 | CRC high eight bits (1octet) |
| 6 | CRC low eight bits (1octet) |

3.1 Command word 0x00 [Configuration information]

| Field | Description |
| --- | --- |
| Command word | 0x00 |
| Data 1 | bit[0:2]: Transmission channel (1: channel 37, 2: channel 38, 3: channel 39); bit[3]: Whether the highest bit enables the receiving mode. 1 indicates that the receiving mode is enabled upon power-on, and 0 indicates that the receiving mode is not enabled upon power-on; bit[4:7]: The value range is 0-9, corresponding to the beacon transmission power as shown in Table (3) and Table (4); |
| Data 2 | bit[0:2]: Beacon device type 000: TI, 001: Nordic, other numbers are reserved; bit[3]: Alarm status value, 0 no alarm, 1 alarm; bit[4:7]: Battery level in percentage, range 0-10 |
| Data 3 | bit[0:6]: Transmission frequency, rules are shown in Table (2). bit[7]: Reserved |

3.2 Command word 0x08 [Acceleration sensor data]

Note: The x, y, and z-axis data each occupy 8 bits, with a sign bit (8-bit resolution). The acceleration sensor measurement range needs to be set to ±2g.

| Field | Description |
| --- | --- |
| Command word | 0x08 is decimal 08 |
| Data 1 | Sensor Data x-axis |
| Data 2 | Sensor Data y-axis |
| Data 3 | Sensor Data z-axis |

3.3 Command word 0x09 [Device status]

| Field | Description |
| --- | --- |
| Command word | 0x09 is decimal 09 (data 0 is the value of this field) |
| Data 1 | Bits 0-1: Reserved, default value is 0 Bit 2: Charger insertion status 1 inserted; 0 not inserted Bit 3: Charging status 1 charging 0 not charging/full Bit 4: SOS status 1 alarm 0 normal Bit 5: Wearing status 1 not wearing 0 wearing Bit 6: Motion status 1 moving 0 stationary Bit 7: Exercise mode 1 on 0 off |
| Data 2 | Software version |
| Data 3 | Voltage value is (reading/2556.6) V Example: If the reading is decimal 159, then the voltage is 159/2556.6=4.1V |

3.4 Command word 0x0A

| Field | Description |
| --- | --- |
| Command word | 0x0A is decimal 10 |
| Data 1 | Heart rate (BPM): 200 (decimal) or less is a normal heart rate; 252 (decimal) measurement abnormal; 250 (decimal) not worn, 251 (decimal) heart rate module abnormal, 255 (decimal): tag does not support heart rate module, 0 (decimal) measurement not completed |
| Data 2 | 255: The tag does not support the heart rate module. 0 Measurement not completed, systolic blood pressure (decimal) |
| Data 3 | 255: The tag does not support the heart rate module. 0 Measurement not completed, diastolic blood pressure (decimal) |

3.5 Command word 0x0C

| Field | Description |
| --- | --- |
| Command word | 0x0C is decimal 12 |
| Data 1 | Body surface temperature (value + 200)/10; for example, 156 represents 35.6 degrees Celsius 0xAA |
| Data 2 | Step count low byte |
| Data 3 | Step count high byte |

Step calculation

For example, if the low byte is 0x56 and the high byte is 0x78, then the total step count is 0x7856, which can then be converted to decimal.

3.6 Command word 0x0D [Sleep state]

| Field | Description |
| --- | --- |
| Command word | 0x0D is decimal 13 |
| Data 1 | Sleep state (0x00: Awake; 0X01: Light sleep; 0X02: Deep sleep; 0XFF: Not detected) |
| Data 2 | Light sleep time (statistics of hours and minutes, uploaded in units of 10 minutes, for example, 5h30min, the reported data is: (5*60+30)/10=33, report 33) |
| Data 3 | Deep sleep time (statistics of hours and minutes, uploaded in units of 10 minutes, for example, 5h30min, the reported data is: (5*60+30)/10=33, report 33) |

3.7 Command word 0x0E [Device identification]

| Field | Description |
| --- | --- |
| Command word | 0x0E is decimal 14 |
| Data 1 | Device identification high byte, such as X3W, is 0x08 |
| Data 2 | Device identification low byte, such as X3W, is 0x26 |
| Data 3 | Reserved item |

4. CRC16bit check calculation reference

/* CRC byte value table - high byte */const unsigned char gClient_auchCRCHi[256] = {0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0,0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41,0x00, 0xC1, 0x81, 0x40, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0,0x80, 0x41, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40,0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1,0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0, 0x80, 0x41,0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1,0x81, 0x40, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41,0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0,0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x00, 0xC1, 0x81, 0x40,0x01, 0xC0,0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0,0x80, 0x41, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40,0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0,0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41,0x00, 0xC1, 0x81, 0x40, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0,0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41,0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0,0x80, 0x41, 0x00, 0xC1, 0x81, 0x40, 0x00, 0xC1, 0x81, 0x40,0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0, 0x80, 0x41, 0x00, 0xC1,0x81, 0x40, 0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41,0x00, 0xC1, 0x81, 0x40, 0x01, 0xC0, 0x80, 0x41, 0x01, 0xC0,0x80, 0x41, 0x00, 0xC1, 0x81, 0x40

} ;

/* CRC byte value table - low byte */const unsigned char gClient_auchCRCLo[256] = {0x00, 0xC0, 0xC1, 0x01, 0xC3, 0x03, 0x02, 0xC2, 0xC6, 0x06,0x07, 0xC7, 0x05, 0xC5, 0xC4, 0x04, 0xCC, 0x0C, 0x0D, 0xCD,0x0F, 0xCF, 0xCE, 0x0E, 0x0A, 0xCA, 0xCB, 0x0B, 0xC9, 0x09,0x08, 0xC8, 0xD8, 0x18, 0x19, 0xD9, 0x1B, 0xDB, 0xDA, 0x1A,0x1E, 0xDE, 0xDF, 0x1F, 0xDD, 0x1D, 0x1C, 0xDC, 0x14, 0xD4,0xD5, 0x15, 0xD7, 0x17, 0x16, 0xD6, 0xD2, 0x12, 0x13, 0xD3,0x11, 0xD1, 0xD0, 0x10, 0xF0, 0x30, 0x31, 0xF1, 0x33, 0xF3,0xF2, 0x32, 0x36, 0xF6, 0xF7, 0x37, 0xF5, 0x35, 0x34, 0xF4,0x3C, 0xFC, 0xFD, 0x3D, 0xFF, 0x3F, 0x3E, 0xFE, 0xFA, 0x3A,0x3B, 0xFB, 0x39, 0xF9, 0xF8, 0x38, 0x28, 0xE8, 0xE9, 0x29,0xEB, 0x2B, 0x2A, 0xEA, 0xEE, 0x2E, 0x2F, 0xEF, 0x2D, 0xED,0xEC, 0x2C, 0xE4, 0x24, 0x25, 0xE5, 0x27, 0xE7, 0xE6, 0x26,0x22, 0xE2, 0xE3, 0x23, 0xE1, 0x21, 0x20, 0xE0, 0xA0, 0x60,0x61, 0xA1, 0x63, 0xA3, 0xA2, 0x62, 0x66, 0xA6, 0xA7, 0x67,0xA5, 0x65,0x64, 0xA4, 0x6C, 0xAC, 0xAD, 0x6D, 0xAF, 0x6F,0x6E, 0xAE, 0xAA, 0x6A, 0x6B, 0xAB, 0x69, 0xA9, 0xA8, 0x68,0x78, 0xB8, 0xB9, 0x79, 0xBB, 0x7B, 0x7A, 0xBA, 0xBE, 0x7E,0x7F, 0xBF, 0x7D, 0xBD, 0xBC, 0x7C, 0xB4, 0x74, 0x75, 0xB5,0x77, 0xB7, 0xB6, 0x76, 0x72, 0xB2, 0xB3, 0x73, 0xB1, 0x71,0x70, 0xB0, 0x50, 0x90, 0x91, 0x51, 0x93, 0x53, 0x52, 0x92,0x96, 0x56, 0x57, 0x97, 0x55, 0x95, 0x94, 0x54, 0x9C, 0x5C,0x5D, 0x9D, 0x5F, 0x9F, 0x9E, 0x5E, 0x5A, 0x9A, 0x9B, 0x5B,0x99, 0x59, 0x58, 0x98, 0x88, 0x48, 0x49, 0x89, 0x4B, 0x8B,0x8A, 0x4A, 0x4E, 0x8E, 0x8F, 0x4F, 0x8D, 0x4D, 0x4C, 0x8C,0x44, 0x84, 0x85, 0x45, 0x87, 0x47, 0x46, 0x86, 0x82, 0x42,0x43, 0x83, 0x41, 0x81, 0x80, 0x40 } ;

//CRC16static uint16_t AoA_SendCrc16(uint16_t Len, uint8_t *pBuf){

uint8_t  uchCRCHi ; /* High CRC byte initialization */

uint8_t uchCRCLo ; /* Low CRC byte initialization */

uint8_t uIndex ; /* Index in CRC loop */

uint16_t CrcValue;

uchCRCHi = 0xFF ;

uchCRCLo = 0xFF ;

while (Len--) /* Transfer message buffer */

{

uIndex = uchCRCHi ^ (*pBuf++ ); /* Calculate CRC */

uchCRCHi = uchCRCLo ^ gClient_auchCRCHi[uIndex];

uchCRCLo = gClient_auchCRCLo[uIndex];

}

CrcValue = uchCRCHi;CrcValue <<= 8;CrcValue |= uchCRCLo;

return CrcValue;

}

5. Sending Example

5.1 Example of Specific Content of Acceleration Sensor Data Sent by BLE Broadcast

Example:

0x02    //Fixed byte

0x25    //Fixed byte

0x01,0x02,0x03,0x04,0x05,0x06 //MAC address Fixed6bytes

0x1E   //Fixed byte

0xFF   //Fixed byte

0x0D   //Fixed byte

0x00   //Fixed byte

0x04   //Fixed byte

0x08   //Acceleration command word

0x01   //Accelerometer x-axis data (note that the raw x-axis data of the accelerometer is read here, resolution8bits)

0x01   //Accelerometer y-axis data (note that the raw y-axis data of the accelerometer is read here, resolution8bits)

0x3E   //Accelerometer z-axis data (note that the raw z-axis data of the accelerometer is read here, resolution8bits)

0xB7   //CRC16bit checksum high byte. The result of the checksum calculation for the preceding15bytes; that is, the calculation starts from the MAC address and ends with the user data;

0xE6   //CRC16bit checksum low byte;0x2F,0x61,0xAC,0xCC,0x27,0x45,0x67,0xF7,0xDB,0x34,0xC4,0x03,0x8E,0x5C,0x0B,0xAA,0x97,0x30,0x56,0xE6

5.2 Example of specific user data content sent by BLE broadcast

Example:

0x02    //Fixed byte

0x25    //Fixed byte

0x01,0x02,0x03,0x04,0x05,0x06 //MAC address Fixed6bytes

0x1E   //Fixed byte

0xFF   //Fixed byte

0x0D   //Fixed byte

0x00   //Fixed byte

0x04   //Fixed byte

0x09 //User command word range0x09-0x0F;

0x02 //User-defined data (note that here0x02only serves as an example, users can customize other data)

0x03 //User-defined data (note that here0x03only serves as an example, users can customize other data)

0x04 //User-defined data (note that here0x04only serves as an example, users can customize other data)

0xC7 //CRC16bit checksum high byte. The result of the first 15bytes of the checksum operation; that is, the calculation starts from the MAC address to the user data cutoff;

0x69 //CRC16bit checksum low byte;

0x2F,0x61,0xAC,0xCC,0x27,0x45,0x67,0xF7,0xDB,0x34,0xC4,0x03,0x8E,0x5C,0x0B,0xAA,0x97,0x30,0x56,0xE6