W200P - Bluetooth Connection Protocol

Bluetooth parameters:

Service UUID:tspp_service_uuid128[16] = {0x9E, 0xCA, 0xDC, 0x24, 0x0E, 0xE5, 0xA9, 0xE0, 0x93, 0xF3, 0xA3, 0xB5, 0x01, 0xFF, 0x40, 0x6E};

Character UUID:

ATT_SERVICE_TSPP 0xFF01

ATT_CHAR_TSPP_REV1 0xFF02

ATT_CHAR_TSPP_UPLOAD 0xFF03

Descriptor UUID 0x2902

Protocol data structure:

The wearable device is a slave, and the Bluetooth gateway, app, etc. are masters.
Wearable device (slave) sampling data: heart rate, blood pressure, blood oxygen, body temperature, step counting, sleep. After establishing a connection with the host, the wearable device (slave) sends data to the host through Notify.

Data structure:

| Byte Size | Format | Name | Unit | Description |
| --- | --- | --- | --- | --- |
| 1 | U8 | Hrs | Bpm | Heart rate |
| 1 | U8 | Sbp | mmHg | Blood pressure - Systolic pressure |
| 1 | U8 | Dbp | mmHg | Blood pressure - Diastolic pressure |
| 2 | U16 | Temp | °C | Body temperature (Temp / 10 = Actual body temperature) |
| 1 | U8 | Spo2 | % | Blood oxygen |
| 4 | U32 | Steps | Step Count | Pedometer |
| 2 | U16 | Sleep Wake State | Minutes | Sleep - Awake Duration |
| 2 | U16 | Sleep Light State | Minutes | Sleep - Light Sleep Duration |
| 2 | U16 | Sleep Deep State | Minutes | Sleep - Deep Sleep Duration |

Example:

426E4B016A6300000E83000D014E009C

42： 0x42-->Hexadecimal to decimal-->66 (Bpm, heart rate)

6E： 0x6E-->Hexadecimal to decimal-->110 (mmHg, blood pressure - systolic)

4B： 0x4B-->Hexadecimal to decimal-->75 (mmHg, blood pressure - diastolic)

016A： 0x016A-->Hexadecimal to decimal-->362 = 36.2 (°C, body temperature)

63： 0x63-->Hexadecimal to decimal-->99 (%, blood oxygen)

00000E83： 0x00000E83-->Hexadecimal to decimal-->3715 (steps, step count)

000D： 0x000D-->Hexadecimal to decimal-->13 (minutes, sleep - awake duration)

014E： 0x014E-->Hexadecimal to decimal-->334 (minutes, sleep - light sleep duration)

009C: 0x009C-->Hexadecimal to Decimal-->156 (minutes, sleep - deep sleep duration)

Calibration time:

Service  UUID: 0xFFF0

Connect downlink command time calibration: #time:yymmddhhmmss

Example: 2025 year 1 month 1 day 1 hour 1 minute 1 second #time:250101010101