B2315L-Lorawan-Communication Protocol

Table of Contents

1 Overview	3

2 Device Instructions	3

2.1 Device Function and Instructions	3

2.2 Device default reporting logic	4

2.3 Device Downlink Description	6

3 Protocol Data Packet Structure	7

3.1 Data Header	8

3.2 Message Identifier (Message ID)	8

3.3 Payload	8

3.5 Checksum	9

4 Report messages	11

4.1 Alarm-related reporting	11

4.1.1 Alarm data upload-1 (MSGID=0x02)	11

4.1.2 Alarm data upload-2 (MSGID=0x21) (Supplement to 02)	15

4.2 Location-related reporting	17

4.2.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)	17

4.2.2 Bluetooth Location Information (LBE Location) (MsgId=0xD6)	19

4.  3 Device Information and Status Reporting	21

4.  3.1 Firmware Version Number (MSGID=0xBB)	21

4.3.2 Power Signal (MSGID=0xF9)	22

4.4 Health-related reporting	24

4.4.1 Health data (MSGID=0x32)	24

4.5 Downlink feedback related reporting	26

4.5.1 Downlink feedback (MSGID=0xC0)	26

5 Settings	28

5.1 Downlink	28

5.1.1 Set Location Reporting Frequency (MSGID=0x17)	28

5.1.2 Message Send (MSGID=0X28)	30

5.1.3 Settings - Location priority settings (0XCE01)	31

5.1.4 Settings - Health Sampling Reporting Frequency Setting (0XCE02)	33

5.1.5 Settings - Switch Settings (0XCE04-24)	35

5.1.6 Setting - Drop Sensitivity Setting (0xCE15)	36

5.1.7 Issue sedentary stay alarm trigger time (MSGID=0XCC)	38

5.1.8 Set Time Zone (MSGID=0xD2)—Special Version Support	39

5.2 Server Time Synchronization Information - Special Version Support	40

5.2.1 Device reports request time calibration data - Special version support	40

5.2.2 Time Calibration Request Data Reply - Special Version Downlink	40

1 Overview

This protocol is suitable for standard Lorawan protocol devices and currently supports the B2315L product. It uses a 32-bit data header for synchronization and terminal identification; it uses a low-overhead checksum algorithm for checksum protection; and it uses a message identifier to identify different messages.
It needs to be parsed according to the actual report. The document is currently being continuously maintained. If you find any errors or omissions, please give feedback in time. Thank you very much.

2 Device Instructions

2.1 Device Function and Instructions

General version: The default high-frequency general purpose is AS923 (923.2-924.6), the low-frequency general purpose is CN470 (470.3-471.7), ABP-CLASSA, if other frequency bands are required, please submit a request

Network access related key:

OTAA:

Application key ： 2B7E151628AED2A6ABF7158809CF4F3C

ABP:

Device address: Last eight digits of the watch DEVEUI

Application session key: 2 B 7 E 151628 AED 2 A 6 ABF 7158809 CF 4 F 3 C

Network session key: 735 F 2 F 22103042 BE 724197 AC 1727 EA 94

(1) Power on

Please fully charge before using for the first time. Charging light effect: Charging: red light flashes, fully charged: green light is always on. Automatic power on: After charging, unplug the charging cable to automatically power on.

Note: The default is the detached state when powered on. Manual power on: Press and hold the button for 10s and release, the interface will display "Welcome".

(2) Shutdown

Low battery shutdown: The screen goes off after the interface displays Byebye. Manual shutdown: Shutdown: Stay on the "About" page of the interface, it will automatically enter the version page, then automatically enter the APPEUI interface, and finally automatically enter the DEVEUI interface.

, wait for the specific DEVEUI number to pop up, then long press the touch button for 3s until ‘Bye’ appears on the screen, then release the button.

Charging shutdown: The red light flashes after connecting the charging cable to enter the charging state.

(3) SOS function

Trigger method: After triggering, the device does not enter sleep mode. Press and hold the button for 3s, the red light will be on after the SOS appears on the interface.

Cancellation method: In SOS mode, light up the screen, press and hold the button for 3s, the red light will go off after the interface displays SOS cancellation.

(4) Signal Status

No signal: The interface signal ladder bar graph shows 'X', indicating that the device reported a failure or is not connected to the network (not within the Lora gateway range).

Signal available: The interface signal has a ladder bar graph.

(5) Device hibernation

Trigger condition: The device does not move for 40 minutes and enters hibernation mode, and does not report location health data.

(6) Vibration

Receive the downlink text message and turn it on

2.2 Device default reporting logic

General version:

(1) Location-related reporting

gps/Bluetooth beacon: Default reporting frequency10minutes, default positioning priority: Bluetooth beacon>gps,

Bluetooth positioning is preferred, and gps positioning is switched if positioning fails.

(3) Alarm-related reporting

SOS Alarm (0xB5): User actively triggers, see the previous section for triggering methods

SOS Cancel (0x02): User actively triggers cancellation, see the previous section for triggering methods

Shutdown Alarm (0x21): Device actively shuts down or low battery shutdown, see the previous section for triggering methods

Wearing Drop Alarm (0x02): The device determines wearing according to the health sampling report frequency. It reports the wearing alarm when the heart rate is measured, and reports the drop alarm when the heart rate is not measured.

Sedentary Stay Alarm (0x02): Default 15 minutes of immobility triggers reporting

Fall Alarm (0x02): Requires device firmware function support. When the device falls from a certain height, it meets the fall algorithm trigger.

Sedentary Alarm (0x02): Requires device firmware function support. When the device is worn and does not move for a long time, it is triggered by default in 15 minutes.

(4) Health-related reporting

Step count, heart rate, body temperature & wrist temperature, blood pressure, blood oxygen (B2315PL supported, default B2315L does not support blood oxygen) (0x32): Default reporting frequency 10 minutes

(5) Device information and status reporting

Software version and model (0xBB): A record will be reported upon startup

Battery signal (0xF9): A record is reported at startup, and a record will also be reported after positioning and health reporting.

(6) Downlink feedback

Downlink feedback (0xC0): The device reports after receiving the downlink command from the server.

(7) Time synchronization request - available in special versions, universal versions automatically calibrate the time and do not report

Time synchronization request (FF00FF): Special version B2315L reports once every time it is turned on, requesting the server to downlink a command to calibrate the time.

Note: The device reports in a packet reporting situation, that is, a data packet contains multiple complete messages. Pay attention not to miss it. The message is a complete message, and there will be no phenomenon of being interrupted in the middle and in the next data packet.

Eg:bdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9bdf9010000006400002800000019a9cf61ca

This data packet contains messages of (0xD6) Bluetooth positioning and (0xF9) battery signal.

(0xD6) Bluetooth positioning: bdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9

(0xF9) Battery signal: bdf9010000006400002800000019a9cf61ca

2.3 Device Downlink Description

General version:

(1) Device location reporting frequency issued (0x17)

The default reporting frequency is 10 minutes, up to 1 minute, and down to 1440 minutes. After the device receives the downlink command, the device reports according to the time period and frequency of the issued command.

Report data, report at the default reporting frequency outside the time period, such as: 00:00-18:00 2 minutes location report,

Then report at the default 10-minute reporting frequency outside the time period

(2) Text message issued (0x28)

GB2312 encoding, up to20bytes (cannot be displayed on one screen, the screen can display up to8Chinese characters), one Chinese character occupies2bytes, and one English letter occupies1byte

After receiving the text message, the green light flashes, the bracelet vibrates, the interface displays the message, click the button before turning off the screen, the text message disappears,

If you do not click the button before turning off the screen, the text message will remain in the menu

(3) Device Location Priority Distribution (0xCE01)

Default positioning priority Bluetooth > GPS,

If the issued positioning priority is: GPS > Bluetooth beacon, GPS positioning is preferred, switch if GPS positioning is not available

Bluetooth beacon, when the positioning is successful, it will not switch to the next positioning priority to generate positioning

(4) Device health sampling frequency issued (0xCE02)

The default health sampling frequency is 10 minutes, with a maximum reporting frequency of 2 minutes.

(5) Sedentary stay alarm trigger time issued (0xCC)

Value range: 2 minutes - 60 minutes. Note that the downlink is only effective when the sedentary alarm switch is turned on.

For example: if the downlink is a 10-minute trigger time, the device will trigger and report a sedentary stay alarm if it does not move for 10 minutes.

(6) Fall sensitivity and height issued (0xCE15)

The default fall sensitivity of the latest general firmware is: Low (Medium-Low). Fall height: 1.5m

Sensitivity: Refers to the degree to which the drop algorithm is satisfied, providing 5 setting levels (0 - 4): Low - Lower (Medium Low) - Medium - Higher (Medium High) - High.

Height: Refers to the height at which the drop alarm is triggered, providing 5 setting levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m- 2.5m.

(7) Sedentary Stay Alarm Switch (0xCE08)

The device does not report sedentary stay alarms by default. After it is turned on, the default trigger time is 15 minutes.

If there is no movement for 15 minutes, the sedentary stay alarm is triggered.

(8) Control Device to Trigger Sleep Switch Issuance (0xCE18)

The default is on. The device enters sleep mode if it does not move for 40 minutes. After the downlink is turned off, the device does not enter sleep mode.

(9) Fall Alarm Switch (0xCE07)

The fall alarm is off by default. After it is turned on, the device reports a fall alarm after a fall is triggered.

(10) Set Time Zone (0x2D) — Special Version Support

Used to set the time zone of the device. Different countries have different time zones.

Different time zones can be sent to the device to make its time consistent with the local time.

(11) Calibration Time Instruction Downlink (0XFF) — Special versions require, general versions automatically calibrate time

When the device reports a time synchronization request message, this command is immediately sent to calibrate the watch time.

3 Protocol Data Packet Structure

A basic protocol data packet structure is shown in Figure 1:

3.1 Data Header

Each data packet starts with a 1-byte Header (except for the special version B2315L requesting calibration time message: ff00ff):

Currently, the Header uploaded by OuFu devices is fixed to BD (except for the special version B2315L requesting calibration time message: ff00ff)

Header: 0xBD ;

3.2 Message Identifier (Message ID)

See Chapter 4 for the content represented by MessgeId.

3.3 Payload

The payload we refer to below refers to the valid body content in the protocol, excluding the head and checksum. The length of the body is noted in parentheses.

The data format used in the payload is shown in the following table:

【U-unsigned；I-signed；X-bitfield；Number-bytes occupied】

The following protocols use little-endian except for ch, u8, i8, and x8

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Explanation |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short Integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short Integer |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Integer |
| x16 | Bitfield | 2 | - | - | Bit 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long Integer |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long Integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit long integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating point type |

3.5 Checksum

The content added to the checksum includes the payload, as shown in Figure 1. The algorithm is as follows. By default, the general-purpose version of the device does not require checksum verification, and this part can be ignored. Any byte can be used for the downlink instruction.

//Assuming content="BDE9000600010A00000A00", then the result of sum.ToString("X2") is 07//The following is the C# code calling method

private string CheckSum(string content)

{

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2");

}

4 Report messages

4.1 Alarm-related reporting

4.1.1 Alarm data upload-1 (MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0x02 | See below 见下方定义 | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below for definition |
| 4 | U32 | Timestamp | - | - | Timestamp (a timestamp will be added at the end when re-uploading) |

Upl_warn contents

| bit (position corresponding to 1 in binary) | Name | Description | Original message in little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 14 | Drop alarm | Drop alarm | 0040 | 4000 | 4*4096=16384 |
| 8 | Device Wearing | Device Wearing | 0001 | 0100 | 1*256=256 |
| 7 | SOS Cancel | SOS Cancel | 8000 | 0080 | 8*16=128 |
| 5 | Sedentary Alarm (No Movement) | Sedentary Alarm (No Movement) | 2000 | 0020 | 2*16=32 |
| 4 | Remove (Drop) Device | Remove (Drop) Device | 1000 | 0010 | 1*16=16 |
| 2 | Shutdown | Shutdown | 0400 | 0004 | 4 |
| 1 | SOS Alarm | SOS Alarm | 0200 | 0002 | 2 |
| 0 | Low Battery | Low Battery | 0100 | 0001 | 1 |

Example:

Wear alarm: BD020001E377BD678A

BD:Header

02:Message ID

0001:Upl_warn alarm content, converted to big-endian:0100-->Wear alarm

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

Fall off alarm: BD021000E377BD678A

BD:Header

02:Message ID

1000:Upl_warn alarm content, converted to big-endian0010-->Fall off alarm

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

SOS Alarm: BD020200E377BD678A

BD:Header

02:Message ID

0200:Upl_warn alarm content, converted to big-endian0002-->SOS alarm

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

SOS cancellation: BD028000E377BD678A

BD:Header

02:Message ID

8000:Upl_warn alarm content, converted to big-endian0080-->SOS cancellation

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

Drop alarm: BD020040E377BD678A

BD:Header

02:Message ID

0040:Upl_warn alarm content, converted to big-endian4000-->Drop alarm

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

E7: checksum

Sedentary alarm: BD022000E377BD678A

BD:Header

02:Message ID

2000:Upl_warn alarm content, converted to big-endian0020-->Sedentary alarm

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8A: checksum

Low battery alarm: BD020200E377BD678A

BD:Header

02:Message ID

0100: Upl_warn alarm content, converted to big-endian 0001 --> Low battery alarm

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8A: checksum

Shutdown alarm: BD020400E377BD678A

BD:Header

02:Message ID

0400: Upl_warn alarm content, converted to big-endian 0004 --> Shutdown alarm

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8A: checksum

Note: This alarm is for devices that do not distinguish between shutdown types. See the next section for devices that distinguish between shutdown types.

4.1.2 Alarm data upload-2 (MSGID=0x21) (Supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0x21 | See below 见下方定义 | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | Alarm type |
| N |  | Upl_warn | - | - | Alarm content See below for different Alarm type type definitions |
| 4 | U32 | Timestamp |  |  | Timestamp (a timestamp will be added at the end when re-uploading) |

Alarm type=1

Upl_warn contents

| bit (position corresponding to 1 in binary) | Name | Description | Original message in little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging shutdown alarm | Device automatically shuts down when charging (firmware support required) | 04000000 | 0004 | 4 |
| 1 | Low battery shutdown alarm | Device shuts down due to low battery (firmware support required) | 02000000 | 0002 | 2 |
| 0 | Active shutdown alarm | Device is manually shut down by someone (firmware support required) | 01000000 | 0001 | 1 |

Example:

Charging shutdown alarm: BD21010004000000E377BD6767

BD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

04000000:Upl_warn alarm content, converted to big-endian 0004 --> Charging shutdown alarm

E377BD67:Timestamp, converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Low battery shutdown alarm: BD21010004000000E377BD6767

BD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

02000000:Upl_warn alarm content, converted to big-endian 0002 --> Low battery shutdown alarm

E377BD67:Timestamp, converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Active shutdown alarm: BD21010001000000E377BD6767

BD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

01000000:Upl_warn alarm content, converted to big-endian 0001 --> Active shutdown alarm

E377BD67:Timestamp, converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

4.2 Location-related reporting

4.2.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0x03 | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude longitude |
| 8 | Double | lat |  | - | latitude latitude |
| 1 | U8 | north_south |  | - | /N or S/ North or South Latitude |
| 1 | U8 | east_west |  | - | /E or W/ East or West Longitude |
| 1 | U8 | status |  | - | /A or V/ Valid data or invalid data |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original16Hexadecimal message: BD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BD:Header

03:Message ID message id

963E870C3E675E40:lon longitude Convert to big endian405E673E0C873E96-->Longitude:121.6131622 Note: See the parsing example below for conversion and parsing

FF2110C2B6343F40: lat dimension Convert to big-endian `403F34B6C21021FF-->Latitude:31.2059137Note: Refer to the parsing example below for conversion and parsing

4E:north_south  4E-->Convert to ASCII code N-->North latitude

45:east_west 45-->Convert to ASCII code E-->East longitude

41:status Data validity  41-->Convert to ASCII code A-->Valid data

E377BD67:Timestamp Timestamp Converted to big-endian67BD77E3-->Convert to base101740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

78:checksum

Note: The parsed GPS latitude and longitude are in the WGS-84coordinate system. If the map uses Baidu Gaode, etc., the coordinate system needs to be converted.

Parsing satellite positioning message Java codeExample:

//Message: BD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public static void main(String[] args){

//Message 7d9f84ac81815c40 actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d",16)));  //114.02353966666665//Message e766926b1d893640 Actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16))) ;//22.535605166666667

HexToStr(data.Substring(“4e”));   //N

HexToStr(data.Substring(“45”));//E

HexToStr(data.Substring(“41”));//A   A indicates valid data, V indicates invalid data, which can be discarded//Message 749d695f actual value 5f699d74

Date date=new Date();

date.setTime(Long.parseLong(“5f699d74",16)*1000);

SimpleDateFormatsdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.2.2 Bluetooth Location Information (LBE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xD6 | See below 见下方定义 | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 0 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple sets of information, and each group may contain multiple ibeacons |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| ... | ... | ... | ... |

Note: In the general version, the total number of Bluetooth beacons is fixed at 1, and the maximum number of Bluetooth beacons is 4. The sorting is based on the signal strength from largest to smallest. By default, the watch only recognizes our own beacons. If you need to connect to other manufacturers' beacons, please consult relevant personnel via WeChat.
Example:

Original hexadecimal message: BDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

BD: Header

D6: Message ID

00: type

01: Only one set of ibeacon data

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

04: The first group has 4 beacon information

4327: Convert to big-endian-->major : 0x2743-->Convert to decimal 10051

1794: Convert to big-endian-->minor: 0x9417-->Convert to decimal 37911

AC:  RSSI 0xAC-->Convert to decimal complement -84

4327: Convert to big endian-->major: 0x2743-->Convert to decimal 10051

3094: Convert to big endian-->minor:0x9430-->Convert to decimal 37936

AA:  RSSI 0xAA-->Convert to decimal complement -86

4327:Convert to big endian-->major: 0x2743-->Convert to decimal 10051

b956:Convert to big endian-->minor:0x56b9-->Convert to decimal 22201

A5:   RSSI 0xA5-->Convert to decimal complement -91

4327: Convert to big endian-->major: 0x2743-->Convert to decimal 10051

FE94: Convert to big endian-->minor:0x94FE-->Convert to decimal 38142

A5: rssi:0xA5-->Convert to decimal complement -91

6a :checksum

Bluetooth beacon major, minor description: The default Bluetooth beacon comes with a beacon code.

For example: 00081005345866, then the major of the Bluetooth beacon is 10053, the minor is 45866, and the manufacturer ID is 0008 (this is useless and has no practical significance)

4.  3 Device Information and Status Reporting

4.  3.1 Firmware Version Number (MSGID=0xBB)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xBB | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit Decription |
| --- | --- | --- | --- |
| 1 | UINT8 | Version_len | Software version number length |
| N | ASCII[n] | Version | Software version number |

Example:

BDBB1043323331304C2E3437302E41412E3031

BD:Header

BB:Message ID

10:Version_len to10hexadecimal16-->The software version number length is16bytes

43323331304C2E3437302E41412E30:Convert to string-->C2310L.470.AA.01

31:checksum

4.3.2 Power Signal (MSGID=0xF9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xF9 | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery Type 0: 4-level system, 1: 5-level system, 2: Percentage, 3: Voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery level If Bat_type is 0: The battery level range is 0-3 (0 is 25%, 3 is 100%) If Bat_type is 1: The battery level range is 0-4 (0 is 20%, 4 is 100%) If Bat_type is 2: The battery level range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: Percentage, 1: 5-level system, 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal strength |
| 1 | U8 | Other_type |  |  | Extended type 0: Full step counting 1: Incremental step counting 2: Vibration |
| 4 | U32 | Num |  |  | Extended value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Example:

Original16Hexadecimal message: BDF90104000050000095000000E377BD67AA

BD: Header

F9: Message ID (message id)01: Bat_type battery type 01-->1-->5Level battery (0-4)0400: Bat_volt battery value converted to big-endian 0004-->4-->Level4corresponds to100% battery00: Signal_type signal type 00-->0-->Percentage5000: Signal_strength signal strength converted to big-endian0050-->converted to decimal80-->80% signal strength00: Other_type extension type 00-->0-->Full step counting95000000: Num extended value - step counting converted to big-endian00000095-->converted to decimal149-->number of steps is149

E377BD67: Timestamp timestamp converted to big-endian67BD77E3-->converted to10decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

AA: checksum

4.4 Health-related reporting

4.4.1 Health data (MSGID=0x32)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0x32 | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | 00 |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 2 | U16 | contentLength |  |  | Total length of the following content |
| 1 | U8 | ID |  |  | Id (Type + Reported Value Length), see below for ID type definition |
| N | N*U8 | Val1 |  |  | Reported value of Id (byte length varies depending on the reported value length) |
| ... | ... | ... | ... | ... | ... |
| 1 | U8 | ID |  |  | Id (Type + Reported Value Length), see below for ID type definition |
| N | N*U8 | Val1 |  |  | Reported value of Id (byte length varies depending on the reported value length) |

ID type definition (hexadecimal):

01 Steps02 Heart Rate 03 Body Temperature 04 Wrist Temperature 06 Diastolic Blood Pressure 07 Systolic Blood Pressure 08 Blood Oxygen (General version B2315L not supported) 09 HRV(Special version supported)

(05 Blood Glucose 10 Sit-up Count11 Running Pace and Distance 12 Jump Rope Speed and Count )---Reserved, currently unavailable

Default universal version support:01 Step counting02 Heart rate 03 Body temperature 04 Wrist temperature 06 Diastolic pressure 07 Systolic pressure 08 Blood oxygen (Universal version B2315L does not support)

Example:

Original16hexadecimal message:BD3200E377BD6711000A1E00114B314A39711A4A0122bc00416212

BD:Header

32:Message ID

00:type  type=00

E377BD67:Timestamp converts to big endian67BD77E3-->convert to10decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

1100:The length of the following message is converted to big endian-->0011-->converted to decimal17, the length of the following message is17bytes

0A:ID  0x0A-->convert to binary00001010   The first five digits are00001-->convert to16hexadecimal01, which represents the data ID step counting (0x01),

The last three digits are010-->convert to decimal02, which means the length of the step counting data is2bytes

1E00:Step counting upload value converted to big endian-->001E-->converted to decimal30, a total of30steps for step counting data

11:ID 0x11-->Convert to binary00010001 The first five digits are00010-->Convert to16hexadecimal02, which represents the data ID heart rate (0x02),

The last three digits are 001 --> converted to decimal 1, which represents the length of the heart rate data as 1 byte

4 B: Heart rate reporting value converted to decimal --> 75, heart rate data is 75

31: ID 0x31 --> converted to binary 00110001 The first five digits are 00110 --> converted to hexadecimal 16 hexadecimal 06, which represents the data ID diastolic blood pressure (0x06),

The last three digits are 001 --> converted to decimal 1, which represents the length of the diastolic blood pressure data as 1 byte

4 A: Blood pressure diastolic reporting value, converted to decimal --> 74, diastolic blood pressure data is 74

39: ID 0x3 --> converted to binary 00111001 The first five digits are 00111 --> converted to hexadecimal 16 hexadecimal 07, which represents the data ID systolic blood pressure (0x07),

The last three digits are 001 --> converted to decimal 1, which represents the length of the systolic blood pressure data as 1 byte

71: Blood pressure systolic reporting value, converted to decimal --> 113, systolic blood pressure data is 113

1 A: ID 0x1A --> converted to binary 00011010 The first five digits are 00011 --> converted to hexadecimal 16 hexadecimal 03, which represents the data ID body temperature (0x03),

The last three digits are 010 --> converted to decimal 2, which represents the length of the body temperature data as 2 bytes

4A01: Body temperature reporting value, converted to big-endian -->014A--> Convert to decimal330-->330*0.1--> Body temperature data is33degrees Celsius

22:ID  0x22--> Convert to binary00100010   The first five digits are 00100--> Convert to16hexadecimal04, representing data ID wrist temperature (0x04),

The last three digits are 010--> Convert to decimal2, representing the length of the wrist temperature data as2bytes

BC00: Wrist temperature reporting value, converted to big-endian -->00BC--> Convert to decimal188-->188*0.1--> Wrist temperature data is18.8degrees Celsius

41:ID  0x41--> Convert to binary01000001    The first five digits are 01000--> Convert to16hexadecimal08, code data ID blood oxygen (0x08),

The last three digits are 001--> Convert to decimal1, representing the blood oxygen data length as1byte

62: Blood oxygen reporting value, converted to decimal -->98, blood oxygen data is98

12:checksum

4.5 Downlink feedback related reporting

4.5.1 Downlink feedback (MSGID=0xC0)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xC0 | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID Length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This command is used for feedback of downlink commands, returning the Message ID of the previously received downlink command (multiple Message IDs can be returned collectively). Some downlink commands do not have C0 feedback.

Original hexadecimal message:BDC0011720

BD:Header

C0:Message ID

01:length Message ID length Convert decimal 01--> Message ID length is 1 byte

17  Type indicates that the device received the downlink command Message ID is 17

20:checksum

5 Settings

5.1 Downlink

5.1.1 Set Location Reporting Frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0x17 | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |

Note: A day is from 00:00-23:59
Example:

00:00-23:59(All day) Report location data every10minutes:BD17010A000000173B00000000000000000000000000000000000000000097

BD:Header

1C:Message ID

01:enable Whether to enable Convert to decimal01-->1--Enabled

0A00:Interval time interval converted to big-endian000A-->converted to decimal10-->time interval is10minutes

00:time_start_h Start time-hour, convert to decimal00-->Start time-hour is00

00:time_start_m Start time-minute, convert to decimal00-->Start time-minute is00

17:time_end_h End time-hour, convert to decimal23-->End time-hour is23

3B:time_end_m End time-minute, convert to decimal59-->End time-minute is59

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All fill00 in disabled state

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All fill00 in disabled state

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All fill00 in disabled state

97:checksum

5.1.2 Message Send (MSGID=0X28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0x28 | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | U8 | type | Message type, if it is downlink text information, GB2312 encoding is 03 |
| 4 | Uint32 | seqID | The id of the information, unique |
| 1 | U8 | CONTENT LEN | Content length |
| N | N | CONTENT | Content, GB2312 encoding (little endian, one Chinese character occupies 2 bytes, one letter occupies 1 byte) |

Note: The information ID cannot be repeated for each transaction. The device cannot receive repeated IDs.
Example:

Text message: Hello World  BD28036895D13F08C4E3BAC3CAC0BDE779

BD:Header

28:Message ID

03:type message type Convert to decimal 03-->type=03

6895D13F: seqID information ID Information ID is 6895D13F

08:CONTENT LEN content length Convert to decimal 8

C4E3BAC3CAC0BDE7:Converted to string Hello World-->Text message content: Hello World

79:checksum

5.1.3 Settings - Location priority settings (0XCE01)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xCE | See below 见下方定义 | CK_sum |

Payload contents

Type=01 Location Priority Setting

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 valid this time |
| 2 | U16 | Len | -/- | -/- | The length of the following instruction |
| n | n |  | -/- | -/- | Text |

Positioning priority setting definition--See2.3 Device downlink description--Device positioning priority distribution (0xCE01)

Positioning priority setting text: The current general version supports:01--GPS, 03--Bluetooth beacon (additional deployment of Bluetooth beacon is required);

Example:

GPS positioning priority (GPS>Bluetooth beacon):BDCE0100030001030033

BD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Convert to decimal 03-->The length of the text is 3 bytes

010300:Text: 01--GPS 03--Bluetooth beacon Positioning priority is GPS>Bluetooth beacon

33:checksum

Bluetooth positioning priority (Bluetooth beacon>GPS):BDCE0100030003010033

BD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Convert to decimal 03-->The length of the text is 3 bytes

030100:Text: 03--Bluetooth beacon 01--GPS Positioning priority is Bluetooth beacon>GPS

33:checksum

Single GPS positioning:BDCE0100030001000033

BD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Convert to decimal 03-->The length of the text is 3 bytes

030201:Text: 01--GPS 00--No positioning 00--No positioning Positioning priority is single GPS positioning

33:checksum

Single Bluetooth beacon positioning:BDCE0100030003000033

BD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Convert to decimal 03-->The length of the text is 3 bytes

020000:Body: 03--Bluetooth Beacon 00--No Positioning 00--No Positioning Positioning priority is single Bluetooth beacon positioning

33:checksum

5.1.4 Settings - Health Sampling Reporting Frequency Setting (0XCE02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xCE | See below 见下方定义 | CK_sum |

Payload contents

Type=02 Health Sampling Reporting Frequency Setting

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=02 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 valid this time |
| 2 | U16 | Len | -/- | -/- | The length of the following instruction |
| 1 | U8 | Healthtype | -/- | -/- | 00 - Indicates all |
| 1 | U8 |  | -/- | -/- | Text |
| 1 | u8 | Time Unit | -/- | -/- | Time unit 00 - minutes, 01 - hours |

Health sampling reporting frequency setting definition--See 2.3 Device Downlink Description--Device Health Sampling Frequency Distribution (0xCE02)

Health sampling reporting frequency setting text: The current general version supports:00-All, reserved item--Not yet implemented: 01 Step counting 02 Heart rate 03 Temperature 04 Sleep 05 Blood pressure 06 Blood sugar 07 Blood oxygen

Example:

Health sampling reporting frequency is reported once every 5 minutes:BDCE0200030000050033

BD:Header

CE:Message ID

02:Type 02 Health Sampling Reporting Frequency Setting

00:Valid Validity 00 Always valid

0300:Convert to big-endian 0003-->Convert to decimal 03-->The text length is 3 bytes

00:Healthtype 00---All

05:Text: Convert to decimal 05-->Health sampling report frequency is 5 minutes

00:Time Unit 00--Minutes

33:checksum

5.1.5 Settings - Switch Settings (0XCE04-24)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xCE | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Please refer to the instructions for specific types |
| 1 | u8 | switch | -/- | -/- | Switch 00 on, 02 off, 01 single effective |
| 2 | U16 |  | -/- | -/- | Fixed 0000 |

Device Alarm Setting - Switch Type: Specific Function Description-- See 2.3 Device Downlink Description

07 Fall Alarm Switch 08 Stay Alarm Switch

Type=07 Fall Alarm Switch
Example:

00---On:BDCE0700000093

BD:Header

CE:Message ID

07:Type 07 Fall Alarm Switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Off:BDCE0702000093

BD:Header

CE:Message ID

07:Type 07 Fall Alarm Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=08 Stop Alarm Switch
Example:

00---Open:BDCE0800000093

BD:Header

CE:Message ID

08:Type 08 Stop Alarm Switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDCE0802000093

BD:Header

CE:Message ID

08:Type 08 Stop Alarm Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

5.1.6 Setting - Drop Sensitivity Setting (0xCE15)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xCE | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Decription |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Type is 0x15 |
| 1 | U8 | Valid | 1 | The default is 0, which has no impact for now |
| 2 | U16 | Len | 2 | Length of subsequent parameter content |
| 1 | U8 | Parameter Type | 1 | Parameter type: 0x00: Sensitivity, 0x01: Height |
| 1 | U8 | Param Level | 1 | Parameter level: 5 levels (0 - 4) |

Drop Sensitivity Setting-Parameter Description:

Sensitivity: Refers to the degree to which the drop algorithm is triggered, providing 5 setting levels (0 - 4): Low - Lower (Medium Low) - Medium - Higher (Medium High) - High.

Height: Refers to the height that satisfies the trigger of the fall alarm, providing 5 setting levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m - 2.5m.

If the set height is "1.5m", the device needs to be ≥ 1.5m to trigger the fall alarm.

Note that sensitivity and height cannot be combined in one message

Example:

Set the sensitivity to "Medium Low": BDCE150002000001FF

BD:Header

CE:Message ID

15:Type Drop Sensitivity Setting

00:Fixed value

0200:Len

00:Param Type Sensitivity setting

01:Param Level Sensitivity level 1-->lower (medium-low)

FF:checksum

Set the height to “1.5m”: BDCE150002000102FF

BD:Header

CE:Message ID

15:Type Drop Sensitivity Setting

00:Fixed value

0200:Len

01:Param Type Height setting

02:Param Level Height level 2-->1.5m

FF:checksum

5.1.7 Issue sedentary stay alarm trigger time (MSGID=0XCC)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xCC | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | minute | -/- | -/- | Stay alarm trigger duration (unit: minutes; value range 2 — 60), inactivity within this time period will trigger a stay alarm |

Example:

Report stay alarm on the watch after 5 consecutive minutes of inactivity (when the watch is worn): BDCC05DD

BD: Header

CC: Message ID

05: minute Stay alarm trigger duration Convert decimal 05-->5 minutes

DD: checksum

5.1.8 Set Time Zone (MSGID=0xD2)—Special Version Support

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xCC | See below 见下方定义 | CK_sum |

Payload contents

| Format | Name | Unit | Decription |
| --- | --- | --- | --- |
| I8 | Zone | 1 | Time zone hours (-12 ~ +12) |
| U8 | Zone_Minute | 1 | Time Zone Minutes |

Example:

Set the device time zone to UTC+05:30 (Indian Standard Time): BDD2051EFF

BD:Header

D2:Message ID

05:Zone converts to decimal complement 5, zone hour is +5 hours

1E:Zone_Minute converts to decimal complement 30, Zone_Minute is 30 minutes

FF:checksum

Set the device time zone to UTC-08:00 (Pacific Time): BDD2F800FF

BD:Header

D2:Message ID

F8:Zone converts to decimal complement -8, zone hour is -8 hours

00:Zone_Minute converts to decimal complement 00, Zone_Minute is 00 minutes

FF:checksum

5.2 Server Time Synchronization Information - Special Version Support

5.2.1 Device reports request time calibration data - Special version support

Note: This message is reported by the special version B2315L. The general version B2315L automatically obtains the gateway lora ns time and automatically synchronizes the time.

Request time synchronization instructions:
After the device is powered on, the wristband will automatically send a request time synchronization command. After receiving the synchronization command, it is necessary to reply to the synchronization time data frame to synchronize the device time.
If the device has been calibrated down before shutting down, then after the device restarts, the device interface displays the time before shutting down, and the device will report FF00FF to re-request time synchronization.

| Byte offset | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | U8 | HEADER | Fill: 0xFF |
| 1 | U8 | SeqId | Fill: 0x00 |
| 1 | U8 | End | Fill: 0xFF |

Example:

FF00FF //The server needs to reply immediately after receiving it, without any delay

5.2.2 Time Calibration Request Data Reply - Special Version Downlink

| Byte offset | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | U8 | HEADER | Fill: 0xFF |
| 1 | U8 | SeqId | Padding: 0x10 |
| 2 | U16 | years | Year data |
| 1 | U8 | month | Month data |
| 1 | U8 | Day | Day, data |
| 1 | U8 | time | Clock, data |
| 1 | U8 | Minute | Minutes, data |
| 1 | U8 | Seconds | Seconds, data |
| 1 | U8 | End | Fill: 0xFF |

Example:

After the server receives FF00FF, it replies with a downlink command to calibrate the time. Note: The example is for the year 2020 and only serves as a structural analysis example. The current year must be sent for it to be parsed. Otherwise, it will not be parsed. FF1007E409020B1B28FF

FF : Header

10 : Seqid

07E4: Year (2020)

09 : Month, (09)

02 : Day, (02)

0B : Hour, (11)

1B : Minitus, (27)

28 : Second, (40)

FF : End