G808G-4GCat.1-TCP Communication Protocol

Directory

1 Overview	3

2 Device Usage Instructions	3

2.1 Device Function and Usage Instructions	3

2.2 Device default reporting logic	4

2.3 Device Downlink Instructions	5

3 Protocol Data Packet Structure	8

3.1 Data Header	8

3.2 Message Identifier (Message ID)	8

3.3 Token Generation Mechanism	9

3.4 Payload	9

3.5 Checksum	10

4 Report messages	12

4.1 Connection related reporting	12

4.1.1 LNK-LIN (MSGID=0xF0) Request connection (TCP only)	12

4.1.2 Connection Reply (MSGID=0xF1) (TCP Dedicated) - Important	13

4.1.3 New heartbeat packet protocol (MSGID=0xF9) - Important	14

4.2 Alarm-related reporting	16

4.2.1 Alarm data upload-1 (MSGID=0x02)	16

4.2.2 Alarm data upload-2(MSGID=0x21)(supplement to 02)	19

4.3 Location-related reporting	22

4.3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)	22

4.3.2 Wi-Fi and base station information upload (MSGID=0xA4)	24

4.3.3 Bluetooth Location Information (BLE Location) (MsgId=0xD6)	29

4.4 Device information and status reporting	32

4.4.1 ICCID Upload of SIM Card (MSGID=0xF3)	32

4.4.2 Device charging status upload (MSGID=0xC3)	33

4.4.3 Status parameter reporting (MSGID=0xA9)	35

4.4.4 Device Status (MSGID=0xE9)	37

4.5 Downlink feedback related reporting	38

4.5.1 Downlink feedback (MSGID=0xC0)	38

5 Settings	40

5.1 Downlink	40

5.1.1 Set Location Report Frequency (MSGID=0x17)	40

5.1.2 Settings - Positioning priority settings (0XCE01&0XCE02)	42

5.1.3 Settings - Switch Settings (0XCE04-24)	45

5.1.4 IP&Domain Name Settings (0xC3) (TCP Only)	51

1 Overview

This protocol is suitable for 4G CAT1 devices and currently supports G808G products. It uses a 32-bit data header for synchronization and terminal identification; uses a low-overhead checksum algorithm for checksum protection; and uses message identifiers to identify different messages. It needs to be parsed according to the actual report. The document is currently being continuously maintained. If you find any errors or omissions, please give feedback in time. Thank you very much. The protocol content is 4G report content. If you need Bluetooth broadcast data protocol, please consult relevant docking personnel.

2 Device Usage Instructions

2.1 Device Function and Usage Instructions

General version:

(1) Power on:

Please fully charge before using it for the first time. Charging light effect: red light flashes, fully charged light effect: green light is always on.

Manual power on: Press and hold the rightmost button for 5 seconds and release the button. The power-on light effect alternates between red and green lights.

(2) Shutdown:

Charging: Automatic shutdown Manual shutdown: Long press the rightmost button for more than 5 seconds, the red and green lights flash and then go out

Low battery shutdown: The green and red lights flash and then go out

(3) SOS function:

Trigger: After triggering, the device does not enter sleep mode. Long press the middle button for 5s, release it after the red light flashes, and the red light stays on.

Cancel: When the red light is on, long press the middle button for 5 seconds, release it after the red light flashes, and cancel the SOS alarm.

(4) Device status:

Not connected to the network: Short press the leftmost button, the light effect is red light.

Connected to the network: Short press the leftmost button, the light effect is green light. Device battery is low: Red light flashes intermittently.

(5) Device sleep:

Trigger condition: The device does not move for 20 minutes and enters sleep mode without reporting data.

2.2 Device default reporting logic

General version:

(1) Connection related reporting

F0 request: The device is a short link, under normal server connection and network conditions,

The startup will report once, and then each subsequent report will request a connection--A reply is required to connect to the server

F9: Power and signal reporting, a message will be reported along with location reporting -- requires server reply

(2) Location-related reporting

Satellite (GPS/BDS)/ WiFi / Bluetooth beacon: Default reporting frequency 10 minutes, default positioning priority: Bluetooth beacon > WiFi > Satellite (GPS/BDS), Bluetooth positioning is preferred, switch to WiFi positioning if no positioning is available, switch to Satellite (GPS/BDS) if no WiFi positioning is available

(3) Alarm-related reporting

SOS Alarm (0x02): Triggered by the user, see the previous section for the triggering method

SOS Cancel (0x02): The user actively triggers the cancellation, see the previous section for the triggering method

Shutdown alarm (0x21): The device actively shuts down or shuts down due to low power, see the previous section for the triggering method

Low battery alarm (0x02): Triggered when the device's current battery level is less than or equal to 0

Drop alarm (0x02): The device falls freely from a certain height, and the device falls from a high altitude (at least 1.8 meters) to trigger

(4) Device information and status reporting

Status parameter (0xA9): A report will be made upon startup

SIM card ICCID (0xF3): A report will be made upon startup

Device status (0xE9): A report will be made upon startup, and another report will be made when the reporting frequency changes

Charging status (0xC3): Report when charging starts, charging ends, and when fully charged---Report when charging and powered on,

The default is charging shutdown, and the charging status is not reported when charging is shutdown

(5) Downlink feedback

Downlink feedback (0xC0): The device reports after receiving the downlink command from the server

Note: The device reports in a combined package, that is, a data packet contains multiple complete messages. Pay attention not to miss any. The message is a complete message, and there will be no interruption in the middle in the next data packet.

Eg:bdbdbdbdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9bdbdbdbdf9010000006400002800000019a9cf61ca

This data packet contains messages for (0xD6) Bluetooth positioning and (0xF9) power signal

(0xD6) Bluetooth positioning: bdbdbdbdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9

(0xF9) Battery signal: bdbdbdbdf9010000006400002800000019a9cf61ca

2.3 Device Downlink Instructions

General version: Downlink instructions need to be continuously sent twice within 20 milliseconds to ensure success.
Note: For short-link devices, downlink instructions are only received by the device when it reports. After receiving, the instructions take effect in the next reporting cycle.

(1) Device location reporting frequency downlink (0x17):

The default reporting frequency is 10 minutes, with a maximum of 1 minute. After the device receives the downlink instruction, it will report according to the time period and frequency of the downlink instruction.

Report data, and report according to the default reporting frequency outside the time period, such as: 00:00-18:00 2 minutes location reporting,

Then report according to the default 10-minute reporting frequency outside the time period.

(2) Device location priority downlink (0xCE01):—-Note: The default is not to report location, and the data reporting switch needs to be turned on.

Default location priority wifi>Satellite (GPS/BDS), location priority

If the downlink location priority is: wifi>Satellite (GPS/BDS)>Bluetooth beacon, if wifi cannot locate, switch to Satellite (GPS/BDS), and if Satellite (GPS/BDS) cannot locate, switch to

Bluetooth beacon, when the positioning is successful, it will not switch to the next positioning priority to generate positioning

(3) Modify IP and port command issuance (0xC3):

The default general version points to the Smart Cloud Platform: 118.178.184.219:8825. If you need to change it, please consult relevant personnel or visit the official website

(4) Bluetooth broadcast switch issuance (0xCE05):

The default is the on state, the device does not perform Bluetooth broadcast after it is turned off

(5) Fall alarm switch (0xCE07):

The default fall alarm is on, the device does not report fall alarms after it is turned off

(6) Key shutdown switch command issuance (0xCE16):

By default, the device can be shut down via the key. After the downlink is turned off, the device cannot be shut down via the key

(7) Control device trigger sleep switch issuance (0xCE18):

The default is the on state. The device enters sleep mode if it is not moved for 40 minutes.

After the downlink is turned off, the device does not enter sleep mode.

(8) Button triggers SOS start switch (0xCE19)

The default is on. Long press the charging cable button to trigger SOS. After it is turned off, long press the charging cable button will not trigger the SOS report.

(9) Control device charging on/off status (0xCE21):

The default charging is off. After the downlink is turned on, the device will not shut down when charging, and the device will report the charging status.

(10) Long connection and short connection mode switching (0xCE22)

The default is short connection. The downlink switches the long connection command. After the device receives it, it restarts and becomes a long connection. Note that the power consumption of the long connection mode will increase. The default state is to report a heartbeat package every 4 minutes (0xF9)

3 Protocol Data Packet Structure

A basic protocol data packet structure is shown in Figure 1:

3.1 Data Header

Each data packet starts with a 4-byte Header or token (timestamp replaces it in some response messages):

Currently, the token uploaded by Oufu devices is fixed as BDBDBDBD.

Header: 0xBD0xBD0xBD0xBD;

Timestamp: 32bits, generated by the server

3.2 Message Identifier (Message ID)

See Chapter 4 for the content represented by MessgeId.

Each time TCP is connected, the device side will first report the 0xF0 message, which contains the device's unique identifier IMEI.

The server needs to record the IMEI as an identifier and reply with the 0xf1 message. Only then will the device consider the connection successful; otherwise, the connection will be disconnected.

For normal device communication, the long connection reports an 0xF9 heartbeat packet every 4 minutes by default. The 0xF9 heartbeat packet is reported once after the health location is reported.

For short connections, the 0xF9 heartbeat packet is reported once at startup, and the 0xF9 heartbeat packet is reported once after the health location is reported. The two reports are not affected by each other.

Note that after the server receives the F9 message, the server needs to issue a command to reply; otherwise, the connection will be disconnected.

3.3 Token Generation Mechanism

Currently fixed as BDBDBDBD

3.4 Payload

The payload below refers to the valid body content in the protocol, excluding the head token and checksum. The annotation after it is the length of the body content.

The data format used in the payload is shown in the following table:

【U-unsigned；I-signed；X-bitfield；Number-Bytes occupied】

The following protocols use little-endian priority except for ch, u8, i8, and x8

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Explanation |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short Integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short Integer |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Plastic surgery |
| x16 | Bitfield | 2 | - | - | Bit 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long Integer |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit long integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating point type |

3.5 Checksum

The content added to the checksum includes the payload, as shown in Figure 1. The algorithm is as follows. The default general-purpose version of the device does not require verification and can ignore this part. Any byte can be used for the downlink instruction.

// Assuming content="BDBDBDBDE9000600010A00000A00", then sum.ToString("X2") result is 07// The following is the C# code calling method

private string CheckSum(string content)

{

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2");

}

4 Report messages

4.1 Connection related reporting

4.1.1 LNK-LIN (MSGID=0xF0) Request connection (TCP only)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xF0 | See below Definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | u64 | IMEI | 1 | - | IMEI number (little endian) |
| 2 | x16 | version |  | - | Bitfield see below |

The device's reported F0 request message must have a server downlink 0XF1 message in response (see the next section for the specific format), otherwise the login will fail.

Example:

Original16hexadecimal message: BDBDBDBDF09B51731BC61603000000C2

BDBDBDBD:4bytes message headerHeader

F0:Message ID(Message ID)9B51731BC6160300: IMEI number(little endian), converted to big endian:000316C61B73519B,

The corresponding decimal is IMEI:869465050010011,0000: version

C2: checksum

Connection related Q&A:

Each time a new TCP connection is created, an F0 request is first reported, which contains the IMEI, and then the server records this IMEI.

After that, all the data in this connection belongs to this IMEI.

Note: If F0 is not received or F0 communication is abnormal, you can use a third-party network testing tool to verify whether the server communication is normal.

4.1.2 Connection Reply (MSGID=0xF1) (TCP Dedicated) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Timestamp(unix) Timestamp | 0xF1 | See below See below definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | u32 | Token | 1 | - | Token: Fixed as BDBDBDBD |

Note that the message sequence is different from the normal reported message. This message is sent by the server and is not the content reported by the device. The server reply needs to be in the current channel.>Note that the message sequence is different from the normal reported message. This message is sent by the server and is not the content reported by the device. The server reply needs to be in the current channel.

Example:

Original hexadecimal message: 07FD8860F1BDBDBDBD2E

07FD8860 (little endian), Timestamp (unix) timestamp (unit: seconds), converted to big endian 6088FD07 converted to decimal: 1619590407

Convert to standard time format UTC time: 2021-04-28 06:13:27

F1: Message ID (message id)

BDBDBDBD: Token

2E: checksum

The timestamp 10-base to standard time format is the result of adding 1619590407 seconds to 1970-01-01 00:00:00. The device uses the timestamp (unit: seconds) in this message to synchronize the time.

Note: In java, the general unit is milliseconds, pay attention to convert to unit: seconds

Note: The total number of bytes in the server's downlink reply is 10 bytes, such as: 0xBD--1 bytes; if the device receives 20 bytes, the connection will not be established

4.1.3 New heartbeat packet protocol (MSGID=0xF9) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xF9 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery Type 0: 4-level system, 1: 5-level system, 2: Percentage, 3: Voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery Value If Bat_type is 0: The battery value range is 0-3 (0 is 25%, 3 is 100%) If Bat_type is 1: The battery value range is 0-4 (0 is 20%, 4 is 100%) If Bat_type is 2: The battery value range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: percentage, 1: 5-level system, 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal strength |
| 1 | U8 | Other_type |  |  | Extended type 0: Full step counting 1: Incremental step counting 2: Vibration |
| 4 | U32 | Num |  |  | Extended value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Note: The F9 heartbeat packet must have a server downlink reply. The device considers the server connection to be unbroken only when it receives the server downlink reply; otherwise, it will re-report the F0 request connection.

Example:

Original16hexadecimal message: BDBDBDBDF90104000050000095000000E377BD67AA

BDBDBDBD: Header

F9: Message ID

01:Bat_type battery type 01-->1-->5-level battery (0-4)

0400:Bat_volt battery value converted to big-endian 0004-->4-->level4corresponds to100% battery

00:Signal_type signal type 00-->0-->percentage

5000:Signal_strength signal strength converted to big-endian0050-->converted to decimal80-->80% signal strength

00:Other_type extension type 00-->0-->full step counting

95000000:Num extension value - step counting converted to big-endian00000095-->converted to decimal149-->number of steps is149

E377BD67: Timestamp converted to big-endian67BD77E3--> converted to10hexadecimal1740470243--> timestamp is1740470243seconds --> converted to standard time format UTC time:2025-02-25 07:57:23--> converted to Beijing time:2025-02-25 15:57:23

AA: checksum

Note: The server can reply with a fixed message upon receiving the F9 heartbeat packet: Fixed reply example BDBDBDBDF301 Byte count:6

4.2 Alarm-related reporting

4.2.1 Alarm data upload-1 (MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x02 | See below See below definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below for definition |
| 4 | U32 | Timestamp | - | - | Timestamp (timestamp will be added at the end during retransmission) |

Upl_warn contents

| bit (position of 1 in corresponding binary) | Name | Description | Original message little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Shutdown | Shutdown | 0400 | 0004 | 4 |
| 0 | Low battery | Low battery | 0100 | 0001 | 1 |
| 1 | SOS alarm | SOS alarm | 0200 | 0002 | 2 |
| 7 | SOS cancel | SOS cancel | 8000 | 0080 | 128 |
| 14 | Fall alarm | Fall alarm | 0040 | 4000 | 16384 |

Example:

Low battery alarm: BDBDBDBD020200E377BD678A

BDBDBDBD: Header

02:Message ID

0100:Upl_warn alarm content, converted to big-endian0001-->Low battery alarm

E377BD67:Timestamp Convert to big-endian67BD77E3-->Convert to decimal1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8A: checksum

Shutdown alarm: BDBDBDBD020400E377BD678A

BDBDBDBD: Header

02:Message ID

0400:Upl_warn alarm content, converted to big-endian0004-->Shutdown alarm

E377BD67:Timestamp Convert to big-endian67BD77E3-->Convert to decimal1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8A: checksum

Note: This alarm is for devices that do not distinguish between shutdown types. See the next section for devices that distinguish between shutdown types.

SOS alarm: BDBDBDBD020200E377BD678A

BDBDBDBD:Header

02:Message ID

0200:Upl_warn alarm content, converted to big-endian0002-->SOS alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

SOS cancellation: BDBDBDBD028000E377BD678A

BDBDBDBD:Header

02:Message ID

8000:Upl_warn alarm content, converted to big-endian0080-->SOS cancellation

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

Fall alarm: BDBDBDBD020040E377BD678A

BDBDBDBD:Header

02:Message ID

0040:Upl_warn alarm content, converted to big-endian4000-->Fall alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

E7: checksum

4.2.2 Alarm data upload-2(MSGID=0x21)(supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x21 | See below See below definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | Alarm type |
| N |  | Upl_warn | - | - | Alarm content See below for different Alarm type definitions |
| 4 | U32 | Timestamp |  |  | Timestamp (timestamp will be added at the end during retransmission) |

Alarm type=1

Upl_warn contents

| bit (position of 1 in corresponding binary) | Name | Description | Original message little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging shutdown alarm | Device automatically shuts down when charging (firmware function support required) | 04000000 | 0004 | 4 |
| 1 | Low battery shutdown alarm | Device shuts down due to low battery (firmware function support required) | 02000000 | 0002 | 2 |
| 0 | Active shutdown alarm | The device was manually shut down (firmware requires function support) | 01000000 | 0001 | 1 |

Example:

Charging shutdown alarm: BDBDBDBD21010004000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

04000000:Upl_warn alarm content, converted to big-endian 0004-->charging shutdown alarm

E377BD67:Timestamp converts the timestamp to big-endian 67BD77E3-->converts to decimal 1740470243-->the timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Low battery shutdown alarm: BDBDBDBD21010002000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

02000000:Upl_warn alarm content, converted to big-endian 0002-->Low battery shutdown alarm

E377BD67:Timestamp converts the timestamp to big-endian 67BD77E3-->converts to decimal 1740470243-->the timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Active shutdown alarm: BDBDBDBD21010001000000E377BD6767

BDBDBDBD:Header

21:Message ID

0100:Alarm type Convert to big-endian 0001-->Alarm type=1

01000000:Upl_warn alarm content, converted to big-endian 0001-->Active shutdown alarm

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

4.3 Location-related reporting

4.3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x03 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude longitude |
| 8 | Double | lat |  | - | latitude latitude |
| 1 | U8 | north_south |  | - | /N or S/ North or South latitude |
| 1 | U8 | east_west |  | - | /E or W/ East or West longitude |
| 1 | U8 | status |  | - | /A or V/ Valid data or invalid data |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original16hexadecimal message: BDBDBDBD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BDBDBDBD:Header

03:Message ID

963E870C3E675E40:lon longitude to big endian405E673E0C873E96-->Longitude:121.6131622Note: Refer to the following parsing example for conversion and parsing

FF2110C2B6343F40: lat dimension Convert to big-endian `403F34B6C21021FF-->Latitude:31.2059137Note: Refer to the parsing example below for conversion and parsing

4E:north_south4E-->Convert to ASCII code N-->North latitude

45:east_west 45-->Converted to ASCII code E-->East longitude

41: Status data validity 41 --> Convert to ASCII code A --> Valid data

E377BD67: Timestamp converts the timestamp to big-endian 67BD77E3-->convert to10base1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

78: Checksum

Note: The parsed Satellite (GPS/BDS) latitude and longitude are in the WGS-84 coordinate system. If the map uses Baidu AMAP, etc., the coordinate system needs to be converted.

Parsing satellite positioning message Java code example:

//Message: BDBDBDBD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BDBDBDBD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public static void main(String[] args){

//Message 7d9f84ac81815c40 Actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d",16)));  //114.02353966666665//Message e766926b1d893640 Actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16))) ;//22.535605166666667

HexToStr(data.Substring(“4e”));   //N

HexToStr(data.Substring(“45”));//E

HexToStr(data.Substring(“41”));//A   A indicates valid data, V indicates invalid data, which can be discarded//Message 749d695f Actual value 5f699d74

Date date=new Date();

date.setTime(Long.parseLong(“5f699d74",16)*1000);

SimpleDateFormatsdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.3.2 Wi-Fi and base station information upload (MSGID=0xA4)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xA4 | See below See below definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 1 | u8 | Cell_cnt | 1 | - | Number of base stations (0-7) Number of cell info payload.Valid value:0~7 |
| 2 | u16 | Cell[0].MCC | - | - | MMC of base station (0) mobile country code of cell[0] |
| 2 | u16 | Cell[0].MNC | - | - | MNC of base station (0) mobile network code of cell[0] |
| 2 | u16 | Cell[0].LAC | - | - | LAC of base station (0) Location area code of cell[0] |
| 4 | U32 | Cell[0].CELL_ID | - | - | ID of base station (0) Cell id of cell[0] |
| 2 | i16 | Cell[0].RSSI | - | dbm | Signal strength RSSI of base station (0) RSSI in dbm of cell[0] |
| ... | ... | ... | ... | ... | ... |
| 2 | u16 | Cell[cell_cnt-1.MCC | - | - | MMC of base station (N) mobile country code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.MNC | - | - | MNC of base station (N) mobile network code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.LAC | - | - | LAC of base station (N) Location area code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1].LAC | - | - | LAC of base station (N) Location area code of cell[[cell_cnt-1] |
| 4 | U32 | Cell[cell_cnt-1].CELL_ID | - | - | ID of base station (N) Cell id of cell[[cell_cnt-1] |
| 2 | I16 | Cell[cell_cnt-1].RSSI | - | dbm | Signal strength RSSI of base station (N) RSSI in dbm of cell[[cell_cnt-1] |
| 1 | U8 | Wifi_cnt |  |  | Number of wifi Number 0f wifi |
| 6 | 6*U8 | Wifi[0].bssid[0]-Wifi[0].bssid[5] |  |  | MAC address of wifi(0) |
| 4 | I32 | Wifi[0].RSSI |  |  | Signal strength RSSI of wifi(0) |
| ... | ... | ... | ... | ... | ... |
| 6 | 6*U8 | Wifi[Wifi_cnt-1].bssid[0]-Wifi[Wifi_cnt-1].bssid[5] |  |  | MAC address of wifi(N) |
| 4 | I32 | Wifi[0].RSSI |  |  | Signal strength RSSI of wifi(0) |

Note: The device may not report base stations every time. The absence of base stations does not affect positioning. If the number of base stations in the message is 0, then the following is directly the number of Wi-Fi networks.

Example:

Original16Hexadecimal message: BDBDBDBDA4E377BD6701CC010000C218D1AD160BF9FF078CBEBE1A8162C6FFFFFFC061180AF42AC1FFFFFF200BC726E000B6FFFFFFA8154DF6517EB2FFFFFFE005C5B1F824CCFFFFFFE8FCAFA02663AFFFFFFF6409805B2B9CAEFFFFFF94

BDBDBDBD: Header

A4:Message ID message id

E377BD67:Timestamp Convert to big-endian67BD77E3-->Convert to decimal1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

01:Cell_cnt Number of base stations 01--> The number of base stations is1

CC01:Cell[0].MCC Base station MCC converted to big-endian01CC--> converted to decimal460--> Base station MCC is460

0000:Cell[0].MNC Base station MNC converted to big-endian 0000--> converted to decimal 0--> Base station MNC is 0

C218:Cell[0].LAC Base station LAC converted to big-endian18C2--> converted to decimal6338--> Base station LAC is6338

D1AD160B:Cell[0].CELL_ID Base station ID converted to big-endian0B16ADD1--> converted to decimal186035665--> Base station ID is186035665

F9FF:Cell[0].RSSI Base station signal strength converted to big-endian FFF9--> converted to decimal complement -7--> Base station signal strength is -7

07: Wifi_cnt number of wifi 07-->7wifis

8CBEBE1A8162:wifi mac address 8C:BE:BE:1A:81:62-->wifi[0]'s mac address is8C:BE:BE:1A:81:62

C6FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFC6--> converted to decimal complement -58--->wifi[0]'s RSSI signal strength is -58

C061180AF42A:wifi mac address C0:61:18:0A:F4:2A-->wifi[1]'s mac address is C0:61:18:0A:F4:2A

C1FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFC1--> converted to decimal complement -63--->wifi[0]'s RSSI signal strength is -63

200BC726E000:wifi mac address 20:0B:C7:26:E0:00-->wifi[2]'s mac address is20:0B:C7:26:E0:00

B6FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFB6-->converted to decimal complement-74--->wifi[0]'s RSSI signal strength is-74

A8154DF6517E:wifi mac address A8:15:4D:F6:51:7E-->wifi[3]'s mac address is A8:15:4D:F6:51:7E

B2FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFB2-->converted to decimal complement-78--->wifi[0]'s RSSI signal strength is-78

E005C5B1F824:wifi mac address E0:05:C5:B1:F8:24-->wifi[4]'s mac address is E0:05:C5:B1:F8:24

CCFFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFCC-->converted to decimal complement-52--->wifi[0]'s RSSI signal strength is-52

E8FCAFA02663:wifi mac address E8:FC:AF:A0:26:63-->wifi[5]'s mac address is E8:FC:AF:A0:26:63

AFFFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFFAF-->converted to decimal complement -81---->wifi[0]'s RSSI signal strength is -81

6409805B2B9C:wifi mac address 64:09:80:5B:2B:9C-->wifi[6]'s mac address is64:09:80:5B:2B:9C

AEFFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFAE-->converted to decimal complement-82--->wifi[0]'s RSSI signal strength is-82

94: checksum

The parameters parsed from the message need to be connected to the map wifi database, and the latitude and longitude data is finally obtained through the map side. Take the AMAP connecting to the wiif database as an example:

According to the access network method, the AMAP wifi positioning protocol document is divided into three positioning scenarios. The following gives corresponding examples for the three positioning scenarios.

1. The network connection method is wifi access, that is, the accesstype parameter is 1

http://apilocate.amap.com/position?accesstype=1&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&mmac=50:a7:2b:1e:56:58,-60,alibaba-inc&macs=4c:48:da:25:0b:11,-59,alibaba-

inc|4c:48:da:25:1a:11,-77,alibaba-inc&serverip=10.2.166.4&output=xml&key=<用户 Key>

2. The network connection method is mobile access and the mobile phone card is a non-cdma card, that is, when the accesstype parameter is 0 and cdma=0:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=0&bts=460,01,40977,2205409,-65&nearbts=460,01,40977,2205409,-65|460,0

1,40977,2205409,-65|460,01,40977,2205409,-65&serverip=10.2.166.4&output=xml&key=<用户 Key>

3. When the network access method is mobile access and the mobile phone card is a CDMA card, that is, when the accesstype parameter is 0 and cdma=1:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=1&bts=13824,1,1838,1674723,575739,-52&serverip=10.2.166.4&output=xml&key=<User Key>

W200P watch: AMAP wifi positioning protocol calls AMAP interface description: When selecting parameters, generally select the first type

You can also select the second type, but if the device does not report the base station, the bts and neabts parameters need to be removed.

The first type, example of incoming parameters: mmac is the first WIFI mac address in the messagehttp://apilocate.amap.com/position?accesstype=1&imei=869907057780989&mmac=04:8C:16:A9:FB:A8,-44,TP-Link&macs=04:8C:16:A9:FB:A8,-44,TP-Link|C8:3A:35:F4:CE:18,-59,TP-Link|1C:78:4E:D5:07:FE,-79,TP-Link|C8:3A:35:9D:0B:E0,-87,TP-Link|4A:D8:90:E8:0A:82,-88,TP-Link|AC:AD:4B:C9:C3:87,-89,TP-Link|A4:16:E7:55:84:AC,-91,TP-Link&output=json&key=xxxx

For the specific AMAP wifi positioning protocol PDF, you can click to view and download:(not a screenshot, you can click to view)
This document is provided by AMAP, you can directly ask AMAP for specific docking
Note that the name of the downloaded document will change, you can rename it yourself

4.3.3 Bluetooth Location Information (BLE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xD6 | See below See below definition | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 0 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple sets of information, and there may be multiple ibeacons in each group |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (One of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (One of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| ... | ... | ... | ... |

Note: In the general version, the total number of Bluetooth beacon groups is fixed to 1, and the maximum number of Bluetooth beacons is 4. The sorting is based on the signal strength from largest to smallest. By default, the watch only recognizes our own beacons. If you need to connect to other beacons, please consult relevant personnel via WeChat.
Example:

Original hexadecimal message: BDBDBDBDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

bdbdbdbd: Header

D6: Message ID

00: type

01: Only one set of ibeacon data

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->The timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

04: The first group has 4 beacon information

4327: Convert to big-endian-->major : 0x2743-->Convert to decimal 10051

1794: Convert to big-endian-->minor: 0x9417-->Convert to decimal 37911

AC:  RSSI 0xAC-->Convert to decimal complement -84

4327: Convert to big-endian-->major: 0x2743-->Convert to decimal 10051

3094: Convert to big-endian-->minor:0x9430-->Convert to decimal 37936

AA:  RSSI 0xAA-->Convert to decimal complement -86

4327:Convert to big endian -->major: 0x2743-->Convert to decimal 10051

b956:Convert to big endian -->minor:0x56b9-->Convert to decimal 22201

A5:   RSSI 0xA5-->Convert to decimal complement -91

4327: Convert to big-endian-->major: 0x2743-->Convert to decimal 10051

FE94: Convert to big endian -->minor:0x94FE-->Convert to decimal 38142

A5: rssi:0xA5-->Convert to decimal complement -91

6a --checksum

Bluetooth beacon major, minor description: The default shipping Bluetooth beacon has a beacon code attached.

For example: 00081005345866, then the major of the Bluetooth beacon is 10053, the minor is 45866, and the manufacturer identification is 0008 (this is useless and has no practical significance)

4.4 Device information and status reporting

4.4.1 ICCID Upload of SIM Card (MSGID=0xF3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xF3 | See below See below definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 10 | 10*U8 | ICCID | 1 | - | ICCID number - Device SIM card number ICCID |

Note: This message is reported once every time the device is powered on.

Example:

Original hexadecimal message:BDBDBDBDF389861118236001639994CC

BDBDBDBD:Header

F3:Message ID

89861118236001639994:ICCID Device SIM card number ICCID is: 89861118236001639994

CC:checksum

4.4.2 Device charging status upload (MSGID=0xC3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Status | / | / | 0 starts, 1 ends, 2 is fully charged |
| 1 | U32 | Timestamp | / | / | Timestamp (timestamp will be added at the end during retransmission) |

Note: This message requires device firmware support. The latest general version is supported by default. This message will only be reported when the downlink command is enabled: charging and booting. Example:

Original hexadecimal message: BDBDBDBDC301E377BD678A

BDBDBDBD:Header

C3: Message ID

01: The device has finished charging

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->The timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A: checksum

Original hexadecimal message: BDBDBDBDC300DB4D2F668A

BDBDBDBD:Header

C3: Message ID

00: The device starts charging

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->The timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A: checksum

Original hexadecimal message:BDBDBDBDC302DB4D2F668A

BDBDBDBD:Header

C3: Message ID

02:The device is fully charged

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->The timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A: checksum

4.4.3 Status parameter reporting (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Number of parameter types |
| 1 |  |  |  |  | (Reserved 00) |
| 1 | u8 | Type | -/- | -/- | Parameter type 1, type definition: 00—MCU (firmware), 01—module, 02 sensor (general version does not have this type) |
| 1 | U8 | NameLen | -/- | -/- | Parameter name 1 length |
| n | N*u8 | Name | -/- | -/- | Parameter Name |
| 1 | u8 | Type | -/- | -/- | Parameter Type 2, Type Definition: 00—MCU (Firmware), 01—Module, 02 Sensor (General version does not have this type) |
| 1 | U8 | NameLen | -/- | -/- | Parameter Type 2 Length |
| n | N*u8 | Name | -/- | -/- | Parameter Name |
| ... | ... | ... | ... | ... | ... |

Note: This message is reported once by default at startup, uploading the device firmware version number, module version number, which can be left unparsed. Previously shipped devices had a Message ID of 0xBB, which can be sent by contacting relevant personnel.

Example:

BDBDBDBDA902000009423732352E4F56303601174E5432364B434E4230304E4E412D4C3032303330393530B5

BDBDBDBD:Header

A9:Message ID

02:TypeCnt indicates that there are 2 types of parameters

00:Reserved field

00:Type=00  00 represents MCU (Firmware)

09:MCU name length is 9 bytes

423732352E4F563036--->Hexadecimal to text (device firmware version number)--->B725.OV06

01:Type=01 01 represents the module

17:The length of the communication module name is 23 bytes

4E5432364B434E4230304E4E412D4C3032303330393530:Hexadecimal to text (4G module version number)

--->NT26KCNB00NNA-L02030950

B5---checksum

4.4.4 Device Status (MSGID=0xE9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xE9 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | type |  | / | Default—00 |
| 2 | u16 | length |  | / | Length of the following message |
| 1 | u8 | location |  | / | Location reporting frequency: 00 — Default, not modified, 01 — Modified by downlink (if there are multiple time periods in the downlink, only report the reporting frequency of the current time period) |
| 2 | u16 | frequency |  | / | Frequency, unit - minutes |
| 1 | u8 | health |  | / | Health reporting frequency: 00 — Default, not modified, 01 — Modified by downlink |
| 2 | u16 | frequency |  | / | Frequency: unit - minutes |

Note: This message is reported once at startup, and once after the downlink modifies the frequency. The latest general version has it, but the previous version does not.

Example:

Original hexadecimal message:BDBDBDBDE9000600010A00010A0033

BDBDBDBD:Header

E9:Message ID

00:type=00

0600:The message length after length is converted to big-endian --> 0006 --> converted to decimal 6, and the message length is 6 bytes

01:location 01 indicates that the downlink has modified the location reporting frequency

0A00:frequency The location reporting frequency is converted to big-endian --> 000A --> converted to decimal 10, and the reporting frequency is 10 minutes

01:health 01 indicates that the downlink has modified the health reporting frequency

0A00:frequency The health reporting frequency is converted to big-endian --> 000A --> converted to decimal 10, and the reporting frequency is 10 minutes

33:checksum

If the device has no health reporting function or no location reporting function, the two values of health reporting frequency and location reporting frequency will remain the same, indicating the device reporting frequency

4.5 Downlink feedback related reporting

4.5.1 Downlink feedback (MSGID=0xC0)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xC0 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID Length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This command is used for feedback of downlink commands, returning the Message ID of the downlink command received earlier (can be a collective return of multiple Message IDs)

Original hexadecimal message:BDBDBDBDC0011720

BDBDBDBD:Header

C0:Message ID

01:length Message ID length Convert decimal 01--> Message ID length is 1 byte

17Type indicates that the device received the downlink command Message ID is 17

20:checksum

5 Settings

5.1 Downlink

5.1.1 Set Location Report Frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x17 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - minute |  |

Note: A day is from 00:00-23:59
Example:

00:00-23:59 Report the location every 10minutes:BDBDBDBD17010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

17:Message ID

01:enable Whether to enable Convert to decimal01-->1--Enabled

0A00:Interval time interval Converted to big-endian000A-->Convert to decimal10-->Time interval is10minutes

00:time_start_h Start time-hour, convert to decimal00-->Start time-hour is00

00:time_start_m Start time-minute, convert to decimal00-->Start time-minute is00

17:time_end_h End time-hour, convert to decimal23-->End time-hour is 23

3B:time_end_m End time-minute, convert to decimal59-->End time-minute is 59

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in disabled state00

97:checksum

5.1.2 Settings - Positioning priority settings (0XCE01&0XCE02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See below definition | CK_sum |

Payload contents

Type=01 Location Priority Setting

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 valid for this time |
| 2 | U16 | Len | -/- | -/- | Length of the following instruction |
| n | n |  | -/- | -/- | Text |

Location priority setting definition--See 2.3 Device Downlink Description--Device Location Priority Distribution (0xCE01)

Location priority setting text:

The current general version supports:01--Satellite (GPS/BDS),02--wifi (wifi + base station)03--Bluetooth beacon (requires additional deployment of Bluetooth beacons),

Special equipment support:04--LBS base station 05--125k

Example:

WiFi positioning priority (WiFi > Bluetooth beacon > Satellite (GPS/BDS)):BDBDBDBDCE0100030002030133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003 --> converted to decimal 03 --> The length of the text is 3 bytes

020301:Text: 02--WiFi 03--Bluetooth beacon 01--Satellite (GPS/BDS) Positioning priority is WiFi > Bluetooth beacon > GPS

33:checksum

Satellite (GPS/BDS) positioning priority (Satellite (GPS/BDS)> WiFi > Bluetooth beacon):BDBDBDBDCE0100030001020333

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003 --> converted to decimal 03 --> The length of the text is 3 bytes

010203:Text: 01--Satellite (GPS/BDS) 02--WiFi 03--Bluetooth beacon Positioning priority is Satellite (GPS/BDS)> WiFi > Bluetooth beacon

33:checksum

Bluetooth positioning priority (Bluetooth beacon > WiFi > Satellite (GPS/BDS)):BDBDBDBDCE0100030003020133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003 --> converted to decimal 03 --> The length of the text is 3 bytes

030201:Text: 03--Bluetooth beacon 02--wifi 01--Satellite (GPS/BDS) Positioning priority is Bluetooth beacon>wifi>Satellite (GPS/BDS)

33:checksum

Single Satellite (GPS/BDS) positioning:BDBDBDBDCE0100030001000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003 --> converted to decimal 03 --> The length of the text is 3 bytes

030201:Text: 01--Satellite (GPS/BDS) 00--No positioning 00--No positioning Positioning priority is single Satellite (GPS/BDS) positioning

33:checksum

Single wifi positioning:BDBDBDBDCE0100030002000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003 --> converted to decimal 03 --> The length of the text is 3 bytes

020000:Text: 02--wifi 00--No positioning 00--No positioning Positioning priority is single wifi positioning

33:checksum

Single Bluetooth beacon positioning:BDBDBDBDCE0100030003000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003 --> converted to decimal 03 --> The length of the text is 3 bytes

020000:Text: 03--Bluetooth beacon 00--No positioning 00--No positioning Positioning priority is single Bluetooth beacon positioning

33:checksum

5.1.3 Settings - Switch Settings (0XCE04-24)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Please refer to the instructions for specific types |
| 1 | u8 | switch | -/- | -/- | Switch 00 is on, 02 is off, 01 is valid for a single use |
| 2 | U16 |  | -/- | -/- | Fixed 0000 |

Device alarm setting - switch type: Specific function description -- see 2.3 Device downlink description

05 Bluetooth broadcast switch

16 Whether the downlink can use the button to shut down

18 Sleep function switch

19 Button trigger SOS alarm switch

21 Charging is power on or power off state

22 Long and short connection mode switching

Type=05 Bluetooth broadcast switch
Example:

00---On:BDBDBDBDCE0500000093

BDBDBDBD:Header

CE:Message ID

05:Type 05 Bluetooth broadcast switch

00:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE0502000093

BDBDBDBD:Header

CE:Message ID

05:Type 05 Bluetooth broadcast switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=07 Fall alarm switch
Example:

00---On:BDBDBDBDCE0700000093

BDBDBDBD:Header

CE:Message ID

07:Type 07 Fall alarm switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE0702000093

BDBDBDBD:Header

CE:Message ID

07:Type 07 Fall Alarm Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=16 Whether the downlink can use the key to shut down Example:

00---Open:BDBDBDBDCE1600000093

BDBDBDBD:Header

CE:Message ID

16:Type 16 Whether the downlink can use the key to shut down

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE1602000093

BDBDBDBD:Header

CE:Message ID

16:Type 16 Whether the downlink can use the key to shut down

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=18 Sleep function switch Example:

00---Open, enter sleep after being still for a period of time, and do not report data:BDBDBDBDCE1800000093

BDBDBDBD:Header

CE:Message ID

18:Type 18 Sleep function switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close, still or charging state, also report data normally:BDBDBDBDCE1802000093

BDBDBDBD:Header

CE:Message ID

18:Type 18 Sleep function switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=19 Key trigger SOS alarm switch
Example:

00---Open:BDBDBDBDCE1900000093

BDBDBDBD:Header

CE:Message ID

19:Type 19 Key trigger SOS alarm switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE1902000093

BDBDBDBD:Header

CE:Message ID

19:Type 19 Key trigger SOS alarm switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=21 Charging is power on or power off
Example:

00---Charging is power on:BDBDBDBDCE2100000093

BDBDBDBD:Header

CE:Message ID

21:Type 21 Charging power on or charging power off

02:switch switch 00 Charging is power on

0000:Fixed value

93:checksum

02---Charging is power off:BDBDBDBDCE2102000093

BDBDBDBD:Header

CE:Message ID

21:Type 21 Charging power on or charging power off

02:switch switch 02 Charging is off

0000:Fixed value

93:checksum

Type=22 Long and short connection mode switching Example:

00---Long connection mode, the default is long connection : BDBDBDBDCE2000000093

BDBDBDBD:Header

CE:Message ID

22:Type 22 Long and short connection mode switching

02:switch Switch 00 turns on the long connection mode

0000:Fixed value

93:checksum

02---Short connection mode : BDBDBDBDCE2002000093

BDBDBDBD:Header

CE:Message ID

22:Type 22 Long and short connection mode switching

02:switch Switch 02 turns off the long connection mode--short connection mode

0000:Fixed value

93:checksum

5.1.4 IP&Domain Name Settings (0xC3) (TCP Only)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | Issue type: Type =1 IPv4, Type =2 IPv6 (reserved item - not currently supported), Type =3 domain name (currently only supports ascii encoded domain names) |
| 2 | U16 | Port |  |  | Port number (2 bytes) |
| 1 | U8 | Len |  |  | Length of subsequent parameter content |
| N | U8 | Domainname |  |  | Specific IP or domain name content |

Note: After delivery, the device will no longer communicate on the original server, but will point to the modified server.

Example:

Type=1 IPv4:Downlink modifies IP to:118.178.184.219, port is: 8825 BDBDBDBDC30179220476B2B8DB33

BDBDBDBD:Header

C3: Message ID

01:Type 1--IPv4

7922:Port converts the port to big-endian 2279-->converts to decimal 8825-->port: 8825

04:Len Length of subsequent parameter content Convert decimal 04-->length is 4 bytes

76B2B8DB:Domainname IP 76-->convert to decimal 118, B2-->convert to decimal 178, B8-->convert to decimal 184, DB-->convert to decimal 219 IP is: 118.178.184.219

33:checksum

Type=3 Domain nameDownlink modifies domain name to:aiday.com.cn, port is: 8825

BDBDBDBD:Header

C3: Message ID

03:type=3, Domain Name

7922: Port port to big-endian --2279 --> to decimal port: 8825

12: Len The length of the parameter content behind is converted to decimal 18-->The message length is 18 bytes

61696461792e636f6d2e636e: Domainname domain name converted to string-->aiday.com.cn-->domain name is: aiday.com.cn

1D: checksum