G908GU-R-4GCat.1-TCP Communication Protocol

Directory

1 Overview	3

2 Device Usage Instructions	3

2.1 Device Function and Usage Instructions	3

2.2 Device default reporting logic	4

2.3 Device Downlink Description	6

3 Protocol Data Packet Structure	8

3.1 Data Header	8

3.2 Message Identifier (Message ID)	8

3.3 Token Generation Mechanism	9

3.4 Payload	9

3.5 Checksum	10

4 Report messages	12

4.1 Connection related reports	12

4.1.1 LNK-LIN (MSGID=0xF0) Request connection (TCP only)	12

4.1.2 Connection Reply (MSGID=0xF1) (TCP Only) - Important	13

4.1.3 New Heartbeat Package Protocol (MSGID=0xF9) - Important	14

4.  2 Alarm-related reporting	16

4.  2.1 Alarm data upload-1 (MSGID=0x02)	16

4.2.2 Alarm data upload-2(MSGID=0x21)(supplement to 02)	19

4.3 Location-related reporting	24

4.3.1 GPS/BDS Location Reporting: Location Data Reporting (MSGID=0x03)	24

4.3.2 WiFi and base station information upload (MSGID=0xA4)	26

4.3.3 Bluetooth Location Information (BLE Location) (MsgId=0xD6)	31

4.3.3 UWB Location Information (MsgId=0xD7)	34

4.4 Device Information and Status Reporting	36

4.4.1 ICCID Upload of SIM Card (MSGID=0xF3)	36

4.4.2 Device Charging Status Upload (MSGID=0xC3)	36

4.4.3 Status parameter report (MSGID=0xA9)	38

4.4.4 Device Status (MSGID=0xE9)	40

4.5 Downlink Feedback Related Reporting	42

4.5.1 Downlink Feedback (MSGID=0xC0)	42

5 Settings	43

5.1 Downlink	43

5.1.1 Set the location reporting frequency (MSGID=0x17)	43

5.1.2 Settings - Positioning Priority Settings (0XCE01 & 0XCE02)	45

5.1.3 Settings - Switch Settings (0XCE04-24)	49

5.1.4 IP&Domain Name Settings (0xC3) (TCP Dedicated)	53

5.1.5 UWB alarm distance issued (0x7900)	55

1 Overview

This protocol is suitable for 4G CAT1 devices, currently supporting the G908GU-R product, mainly used with UB01 (UWB beacon) or G908GU-T. It uses a 32-bit data header for synchronization and terminal identification; uses a low-overhead checksum algorithm for checksum protection; and uses message identifiers to identify different messages.
It needs to be parsed according to the actual report. The document is currently being continuously maintained. If any errors or omissions are found, please provide timely feedback. Thank you very much.
The protocol content is the 4G reporting content. If you need the Bluetooth broadcast data protocol, please consult the relevant contact person.

2 Device Usage Instructions

2.1 Device Function and Usage Instructions

General version:

(1) Power on:

Please fully charge before using for the first time. Charging automatically turns on the device.

Charging light effect: red light flashes, fully charged light effect: green light is always on.

Manual power on: Manual power on: Press and hold the power button for 5 seconds, power on light effect: red and green lights flash

(2) Shutdown:

Charging: Automatic shutdown

Manual shutdown: Shutdown: Switch the power button and the left button back and forth three times in the power-on state, right-left-right-left-right-left, short press the left button and the red light does not light up, indicating that the locator has been turned off.

Low battery shutdown: Green and red lights flash and then go out

Device low battery status: Lights flash every 5s

(3) SOS function:

Trigger: Long press the SOS button for 3 seconds to trigger, after triggering, the indicator light will be solid red.

Cancel: Long press SOS to cancel in SOS state, the red light will be canceled.

(4) Signal status:

Not networked: Short press the left button once, the light effect is red.

Networked: Short press the left button once, the light effect is green.

(5) Device hibernation:

Trigger condition: The default device does not hibernate, and charging can work normally.

2.2 Device default reporting logic

General version:

(1) Connection related reporting

F0 request: The device is a short link. Under normal server connection and network conditions,

It will be reported once at startup, and then each report will request a connection--A reply is required to connect to the server

F9: Power and signal reporting, a report will be sent along with location reporting -- requires server response

(2) Location-related reporting

UWB/gps/wifi/Bluetooth beacon: Default reporting frequency10minutes, default positioning priority: UWB>wifi>gps, UWB positioning is preferred, switch to wifi positioning if UWB positioning is not available, switch to GPS if wifi positioning is not available

(3) Alarm-related reporting

SOSAlarm(0x02): User actively triggers, see the previous section for triggering method

SOSCancel(0x02): User actively triggers cancellation, see the previous section for triggering method

Shutdown alarm(0x21): Device actively shuts down or shuts down due to low battery, see the previous section for triggering method

Low battery alarm(0x02): Device current battery level is less than or equal to0when triggered

UWBranging alarm(0x21): Device default danger distance5m, default1minute to report an alarm

(4) Device information and status reporting

Status parameter(0xA9): A record will be reported when the device is turned on

SIM card ICCID (0xF3): Report once when booting up

Device status (0xE9): Report once when booting up, and report once when the reporting frequency changes

Charging status (0xC3): Report when charging starts, charging ends, and when fully charged---report when booting up while charging

The default is charging shutdown, and the charging status is not reported when charging shutdown

(5) Downlink feedback

Downlink feedback (0xC0): The device reports after receiving the downlink command from the server

Note: The device reports in a combined packet, that is, a data packet contains multiple complete messages. Pay attention not to miss any. The message is a complete message, and there will be no phenomenon of being interrupted in the middle and in the next data packet.

Eg:bdbdbdbdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9bdbdbdbdf9010000006400002800000019a9cf61ca

This data packet contains messages for (0xD6) Bluetooth positioning and (0xF9) power signal

(0xD6) Bluetooth positioning: bdbdbdbdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9

(0xF9) Battery signal: bdbdbdbdf9010000006400002800000019a9cf61ca

2.3 Device Downlink Description

General version: The downlink command needs to be continuously downlinked twice within 20 milliseconds to ensure success. Note: For short-link devices, the downlink command is only received by the device when the device reports. After receiving it, it takes effect in the next reporting cycle.

(1) Device location reporting frequency distribution (0x17):

The default reporting frequency is 10 minutes, the highest is 1 minute, and the lowest is 1440 minutes. After the device receives the downlink command, the device presses the time period and frequency of the distribution.

Report data, report at the default reporting frequency outside the time period, such as: 00:00-18:00 2 minutes location report,

Then report at the default 10-minute reporting frequency outside the time period

(2) Device location priority distribution (0xCE01):—-Note that location is not reported by default, and the data reporting switch needs to be turned on

Default positioning priority UWB>wifi>Satellite (GPS/BDS), positioning priority

If the distribution positioning priority is: wifi>Satellite (GPS/BDS)>Bluetooth beacon, then wifi cannot locate and switch to Satellite (GPS/BDS), Satellite (GPS/BDS)cannot locate and switch

Bluetooth beacon, when the positioning is successful, will not switch to the next positioning priority to generate positioning

(3) Modify IP and port command issuance (0xC3):

The default general version points to the Smart Cloud Platform: 118.178.184.219:8825. If you need to change it, please consult relevant personnel or visit the official website.

(4) Bluetooth broadcast switch issuance (0xCE05):

The default is the on state, the device does not perform Bluetooth broadcast after it is turned off

(5) Key shutdown switch command issuance (0xCE16):

By default, the device can be shut down via the key. After the downlink is turned off, the device cannot be shut down via the key.

(6) Control device to trigger sleep switch issuance (0xCE18):

The default is the on state, the device enters sleep after 40 minutes of inactivity. After the downlink is turned off, the device does not enter sleep.

(7) Key trigger SOS alarm switch (0xCE19)

The default is the on state, triggered by long pressing the key. After it is turned off, long pressing the key will not trigger the report of SOS.

(8) Control device charging on/off status (0xCE21):

Default charging is off. After the downlink is enabled, the device will not shut down when charging, and the device will report the charging status.

(9) Long connection and short connection mode switching (0xCE22)

The default is short connection. The downlink switches the long connection command. After the device receives it, it restarts and becomes a long connection. Note that the power consumption of the long connection mode will increase. The default state is to report a heartbeat package every 4 minutes (0xF9).

(10) UWB alarm distance (0x79):

Control the distance of UWB ranging alarm, where 0 meters means no alarm

3 Protocol Data Packet Structure

A basic protocol data packet structure is shown in Figure 1:

3.1 Data Header

Each data packet starts with a 4-byte Header or token (timestamp is used instead in some response messages):

Currently, the token uploaded by Oufu devices is fixed at BDBDBDBD

Header: 0xBD0xBD0xBD0xBD；

Timestamp: 32bits, generated by the server

3.2 Message Identifier (Message ID)

See Chapter 4 for the content represented by MessgeId.

Each time the TCP is connected, the device side will first report the 0xF0 message, which contains the device unique identifier IMEI.

The server needs to record the IMEI as an identifier and reply with the 0xf1 message. The device will consider the connection successful, otherwise the connection will be disconnected.

For normal device communication, the long connection reports a 0xF9 heartbeat packet every 4 minutes by default. The 0xF9 heartbeat packet is reported once after the health location is reported.

For short connections, the 0xF9 heartbeat packet is reported once at startup, and the 0xF9 heartbeat packet is reported once after the health location is reported. The two reports are not affected by each other.

Note that after the server receives the F9 message, the server needs to issue a command to reply, otherwise the connection will be disconnected.

3.3 Token Generation Mechanism

Currently fixed as BDBDBDBD

3.4 Payload

The payload below refers to the valid body content in the protocol, excluding the head token and checksum. The length of the body is noted in parentheses.

The data format used in the payload is shown in the following table:

[U-unsigned; I-signed; X-bitfield; Number - Number of bytes]

The following protocols use little-endian priority except for ch, u8, i8, and x8

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Explanation |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short Integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short Integer |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Plastic surgery |
| x16 | Bitfield | 2 | - | - | Bit 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long Integer |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit long integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating point type |

3.5 Checksum

The content added to the checksum includes the payload, as shown in Figure 1. The algorithm is as follows. By default, the general version of the device does not require checksum, and this part can be ignored. Any byte can be used for the downlink instruction.

//Assume content="BDBDBDBDE9000600010A00000A00", then the sum.ToString("X2") result is 07//The following is the C# code calling method

private string CheckSum(string content)

{

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2");

}

4 Report messages

4.1 Connection related reports

4.1.1 LNK-LIN (MSGID=0xF0) Request connection (TCP only)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xF0 | See below for definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 8 | u64 | IMEI | 1 | - | IMEI number (little endian) |
| 2 | x16 | version |  | - | Bitfield see below |

The device's reported F0 request message must have a server downlink 0XF1 message in response (see the next section for the specific format); otherwise, login will fail.

Example:

Original16Hexadecimal message: BDBDBDBDF09B51731BC61603000000C2

BDBDBDBD:4Bytes message headerHeader

F0:Message ID(Message id)9B51731BC6160300: IMEI number(Little endian), converted to big endian:000316C61B73519B,

The corresponding decimal is IMEI:869465050010011,0000: version

C2: checksum

Connection related Q&A:

Each time TCP creates a new connection, it will first report an F0 request. The F0 carries the IMEI, and then the server records this IMEI.

After that, all the data in this connection belongs to this IMEI.

Note: If F0 is not received or F0 communication is abnormal, you can use a third-party network testing tool to verify whether the server communication is normal.

4.1.2 Connection Reply (MSGID=0xF1) (TCP Only) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Timestamp (unix) timestamp | 0xF1 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 4 | u32 | Token | 1 | - | Token: Fixed as BDBDBDBD |

Note that the message sequence is different from the normal reported message. This message is issued by the server, not the content reported by the device. The server reply needs to be replied in the current channel>Note that the message sequence is different from the normal reported message. This message is issued by the server, not the content reported by the device. The server reply needs to be replied in the current channel

Example:

Original16hexadecimal message:07FD8860F1BDBDBDBD2E

07FD8860(Little Endian),Timestamp(unix) timestamp (unit: seconds), converted to big endian6088FD07 Converted to decimal:1619590407

Convert to standard time formatUTCTime:2021-04-28 06:13:27

F1:Message ID(Message ID)

BDBDBDBD:Token

2E:checksum

The timestamp10-base to standard time format is the result of adding1619590407 seconds to 1970-01-0100:00:00. The device uses the timestamp (unit: seconds) in this message to synchronize time.

Note:javagenerally uses milliseconds as the unit. Pay attention to converting to the unit: seconds.

Note: The total number of bytes replied by the server is10bytes, such as:0xBD--1byte; if the device receives20bytes, the connection will not be established.

4.1.3 New Heartbeat Package Protocol (MSGID=0xF9) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xF9 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery type 0: 4-level system, 1: 5-level system, 2: percentage, 3: voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery level If Bat_type is 0: the battery level range is 0-3 (0 is 25%, 3 is 100%) If Bat_type is 1: the battery level range is 0-4 (0 is 20%, 4 is 100%) If Bat_type is 2: the battery level range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: percentage, 1: 5-level system, 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal Strength |
| 1 | U8 | Other_type |  |  | Extension Type 0: Full Step Counting 1: Incremental Step Counting 2: Vibration |
| 4 | U32 | Num |  |  | Extended Value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Note: The F9 heartbeat packet must have a server downlink reply. The device side considers the server connection to be unbroken only after receiving the server downlink reply; otherwise, it will re-report the F0 request connection.

Example:

Original16hexadecimal message: BDBDBDBDF90104000050000095000000E377BD67AA

BDBDBDBD: Header

F9: Message ID

01:Bat_type battery type 01-->1-->5level battery (0-4)

0400:Bat_volt battery value converted to big-endian 0004-->4-->Level4corresponds to100% battery

00:Signal_type signal type 00-->0-->percentage

5000:Signal_strength signal strength converted to big-endian0050-->converted to decimal80-->80% signal strength

00:Other_type extension type 00-->0-->full step counting

95000000:Num extension value - step counting converted to big-endian00000095-->converted to decimal149-->number of steps is149

E377BD67: Timestamp converted to big-endian67BD77E3-->converted to10hexadecimal1740470243-->timestamp is1740470243seconds--> converted to standard time format UTC time:2025-02-25 07:57:23-->converted to Beijing time:2025-02-25 15:57:23

AA: checksum

Note: The server can reply with a fixed message upon receiving the F9 heartbeat packet: Fixed reply example BDBDBDBDF301 Byte count:6

4.  2 Alarm-related reporting

4.  2.1 Alarm data upload-1 (MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x02 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below for definition |
| 4 | U32 | Timestamp | - | - | Timestamp (timestamp will be added at the end during retransmission) |

Upl_warn contents

| bit (position of 1 in corresponding binary) | Name | Description | Original message little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Shutdown | Shutdown | 0400 | 0004 | 4 |
| 0 | Low battery | Low battery | 0100 | 0001 | 1 |
| 7 | SOS Cancel | SOS Cancel | 8000 | 0080 | 8*16=128 |
| 1 | SOS alarm | SOS alarm | 0200 | 0002 | 2 |

Example:

Low battery alarm: BDBDBDBD020200E377BD678A

BDBDBDBD:Header

02:Message ID

0100: Upl_warn alarm content, converted to big-endian 0001- -> Low battery alarm

E377BD67: Timestamp, converted to big-endian 67 BD77E3--> converted to decimal 1740470243 --> timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8 A: Checksum

Shutdown alarm: BDBDBDBD020400E377BD678A

BDBDBDBD:Header

02:Message ID

0400: Upl_warn alarm content, converted to big-endian 0004-->Shutdown alarm

E377BD67: Timestamp, converted to big-endian 67 BD77E3--> converted to decimal 1740470243 --> timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8 A: Checksum

Note: This alarm is used for devices that do not distinguish between shutdown types. See the next section for devices that distinguish between shutdown types.

SOS alarm: BDBDBDBD020200E377BD678A

BDBDBDBD: Header

02: Message ID

0200:Upl_warn alarm content, converted to big-endian0002-->SOS alarm

E377BD67:Timestamp timestamp converted to big-endian67BD77E3-->convert to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8a: checksum

SOS cancellation: BDBDBDBD028000E377BD678A

BDBDBDBD: Header

02: Message ID

8000:Upl_warn alarm content, converted to big-endian0080-->SOS cancellation

E377BD67:Timestamp timestamp converted to big-endian67BD77E3-->convert to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8a: checksum

4.2.2 Alarm data upload-2(MSGID=0x21)(supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x21 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | Alarm type |
| N |  | Upl_warn | - | - | Alarm content See below for Alarm type definitions |
| 4 | U32 | Timestamp |  |  | Timestamp (timestamp will be added at the end during retransmission) |

Alarm type=1

Upl_warn contents

| bit (position of 1 in corresponding binary) | Name | Description | Original message little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging shutdown alarm | Device automatically shuts down when charging (firmware requires function support) | 04000000 | 0004 | 4 |
| 1 | Low battery shutdown alarm | Device shuts down due to low battery (firmware requires function support) | 02000000 | 0002 | 2 |
| 0 | Active shutdown alarm | Device is manually shut down by someone (firmware requires function support) | 01000000 | 0001 | 1 |

Example:

Charging shutdown alarm: BDBDBDBD21010004000000E377BD6767

BDBDBDBD:Header

21:Message ID消息id

0100:Alarm type alarm type converted to big endian 0001-->Alarm type=1

04000000:Upl_warn alarm content, converted to big endian 0004-->charging shutdown alarm

E377BD67:Timestamp timestamp converted to big endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Low battery shutdown alarm: BDBDBDBD21010002000000E377BD6767

BDBDBDBD:Header

21:Message ID消息id

0100:Alarm type alarm type converted to big endian 0001-->Alarm type=1

02000000:Upl_warn alarm content, converted to big endian 0002-->low battery shutdown alarm

E377BD67:Timestamp timestamp converted to big endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Active shutdown alarm: BDBDBDBD21010001000000E377BD6767

BDBDBDBD:Header

21:Message ID消息id

0100:Alarm type alarm type converted to big endian 0001-->Alarm type=1

01000000:Upl_warn alarm content, converted to big endian 0001-->active shutdown alarm

E377BD67:Timestamp timestamp converted to big endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Alarm type=4

Upl_warn contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | 02—UWB ranging alarm |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple sets of information, and there may be multiple Ubeacons in each group |
| U8 | Total_PackCount | 1 | Total number of alarm Ubeacons at the current time |
| U32 | UBeacon0 mac | 4 | UBeacon mac |
| U16 | Distance0 | 2 | Distance (in centimeters) |
| U32 | UBeacon1 mac | 4 | UBeacon mac |
| U16 | Distance1 | 2 | Distance (in centimeters) |
| U32 | UBeacon2 mac | 4 | UBeacon mac |
| U16 | Distance2 | 2 | Distance (in centimeters) |
| ...... | ...... | ...... | ...... |

Example:

Original hexadecimal message:BDBDBDBD210400020103320600009100E70500006800380500005C00E377BD6755

BDBDBDBD:Header

21:Message ID message id

0400:Alarm type Convert to big-endian 0004-->Alarm type=4

02:Type Convert to decimal 02--UWB ranging alarm

01:Total_groups Total number of groups converted to decimal 01, 1 group

03:Total_PackCount Total number of alarm Ubeacons at the current time. Convert to decimal 03, 3 UWB beacons

32060000:UBeacon0 mac Convert to big endian 00000632, UWB beacon id is 00000632

9100:Distance0 Distance Convert to big endian 0091-->Convert to decimal 145-->The distance from the UWB beacon is 145 cm

E7050000:UBeacon1 mac

6800:Distance1 Distance Convert to big endian 0068-->Convert to decimal 105-->The distance from the UWB beacon is 105 cm

38050000:UBeacon2 mac

5C00:Distance2 Distance Convert to big endian 005C-->Convert to decimal 92-->The distance from the UWB beacon is 92 cm

E377BD67:Timestamp Timestamp Convert to big endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

55:checksum

4.3 Location-related reporting

4.3.1 GPS/BDS Location Reporting: Location Data Reporting (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x03 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude |
| 8 | Double | lat |  | - | latitude |
| 1 | U8 | north_south |  | - | /N or S/ North or South Latitude |
| 1 | U8 | east_west |  | - | E or W/ East or West longitude |
| 1 | U8 | status |  | - | A or V/ Valid data or invalid data |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original16Hexadecimal message: BDBDBDBD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BDBDBDBD: Header

03: Message ID message id

963E870C3E675E40: lon longitude Convert to big endian405E673E0C873E96-->Longitude:121.6131622 Note: Refer to the parsing example below for conversion and parsing

FF2110C2B6343F40: lat latitude Convert to big endian`403F34B6C21021FF-->Latitude:31.2059137 Note: Refer to the parsing example below for conversion and parsing

4E: north_south 4E-->Convert to ASCII encoding N-->North latitude

45:east_west 45-->Converted to ASCII code E-->East longitude

41:status data validity 41--> convert to ASCII code A --> valid data

E377BD67: Timestamp converts the timestamp to big-endian 67 BD77E3 --> converts to 10 hexadecimal 1740470243 --> The timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

78: checksum

Note: The parsed Satellite (GPS/BDS) latitude and longitude are in the WGS-84 coordinate system. If the map uses Baidu AMAP, etc., the coordinate system needs to be converted.

Parsing satellite positioning message Java code example:

//Message: BDBDBDBD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BDBDBDBD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public static void main(String[] args){

//Message 7d9f84ac81815c40 Actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d",16)));  //114.02353966666665//Message e766926b1d893640 Actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16))) ;//22.535605166666667

HexToStr(data.Substring("4e")); //N

HexToStr(data.Substring("45"));//E

HexToStr(data.Substring("41"));//A A indicates valid data, and V indicates invalid data, which can be discarded//Message 749d695f actual value 5f699d74

Date date=new Date();

date.setTime(Long.parseLong("5f699d74",16)*1000);

SimpleDateFormatsdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.3.2 WiFi and base station information upload (MSGID=0xA4)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xA4 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 1 | u8 | Cell_cnt | 1 | - | Number of cell info payload (0-7). Valid value: 0~7 |
| 2 | u16 | Cell[0].MCC | - | - | MMC of cell (0) mobile country code of cell[0] |
| 2 | u16 | Cell[0].MNC | - | - | MNC of cell (0) mobile network code of cell[0] |
| 2 | u16 | Cell[0].LAC | - | - | LAC of cell (0) Location area code of cell[0] |
| 4 | U32 | Cell[0].CELL_ID | - | - | ID of cell (0) Cell id of cell[0] |
| 2 | i16 | Cell[0].RSSI | - | dbm | RSSI in dbm of cell[0] |
| ... | ... | ... | ... | ... | ... |
| 2 | u16 | Cell[cell_cnt-1.MCC | - | - | mobile country code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.MNC | - | - | mobile network code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.LAC | - | - | Location area code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1].LAC | - | - | Location area code of cell[[cell_cnt-1] |
| 4 | U32 | Cell[cell_cnt-1].CELL_ID | - | - | ID of base station (N) Cell id of cell[[cell_cnt-1] |
| 2 | I16 | Cell[cell_cnt-1].RSSI | - | dbm | Signal strength RSSI of base station (N) RSSI in dbm of cell[[cell_cnt-1] |
| 1 | U8 | Wifi_cnt |  |  | Number of wifi Number 0f wifi |
| 6 | 6*U8 | Wifi[0].bssid[0]-Wifi[0].bssid[5] |  |  | MAC address of wifi(0) |
| 4 | I32 | Wifi[0].RSSI |  |  | RSSI signal strength of wifi(0) |
| ... | ... | ... | ... | ... | ... |
| 6 | 6*U8 | Wifi[Wifi_cnt-1].bssid[0]-Wifi[Wifi_cnt-1].bssid[5] |  |  | MAC address of wifi(N) |
| 4 | I32 | Wifi[0].RSSI |  |  | RSSI signal strength of wifi(0) |

Note: The device may not report base stations every time. The absence of base stations does not affect positioning. If the number of base stations in the message is 0, then the following is directly the number of Wi-Fi networks.

Example:

Original16hexadecimal message: BDBDBDBDA4E377BD6701CC010000C218D1AD160BF9FF078CBEBE1A8162C6FFFFFFC061180AF42AC1FFFFFF200BC726E000B6FFFFFFA8154DF6517EB2FFFFFFE005C5B1F824CCFFFFFFE8FCAFA02663AFFFFFFF6409805B2B9CAEFFFFFF94

BDBDBDBD:Header

A4: Message ID message id

E377BD67: Timestamp, converted to big-endian 67 BD77E3--> converted to decimal 1740470243 --> timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

01: Cell_cnt base station count 01--> base station count is 1

CC01: Cell[0].MCC Base station MCC converted to big-endian01CC--> converted to decimal460--> base station MCC is460

0000: Cell[0].MNC Base station MNC converted to big-endian 0000--> converted to decimal 0--> base station MNC is 0

C218: Cell[0].LAC Base station LAC converted to big-endian18C2--> converted to decimal6338--> base station LAC is6338

D1AD160B:Cell[0].CELL_ID Base station ID converted to big endian0B16ADD1-->converted to decimal186035665-->Base station ID is186035665

F9FF:Cell[0].RSSI Base station signal strength converted to big endian FFF9-->converted to decimal complement-7-->Base station signal strength is-7

07: Wifi_cnt wifi count 07-->7wifi

8CBEBE1A8162:wifi mac address 8C:BE:BE:1A:81:62-->wifi[0]'s mac address is8C:BE:BE:1A:81:62

C6FFFFFF:wifi signal strength RSSI converted to big endian FFFFFFC6-->converted to decimal complement-58--->wifi[0]'s RSSI signal strength is-58

C061180AF42A:wifi mac address C0:61:18:0A:F4:2A-->wifi[1]'s mac address is C0:61:18:0A:F4:2A

C1FFFFFF:wifi signal strength RSSI converted to big endian FFFFFFC1-->converted to decimal complement-63--->wifi[0]'s RSSI signal strength is-63

200BC726E000:wifi mac address 20:0B:C7:26:E0:00-->wifi[2]'s mac address is20:0B:C7:26:E0:00

B6FFFFFF:wifi signal strength RSSI converted to big endian FFFFFFB6-->converted to decimal complement-74--->wifi[0]'s RSSI signal strength is-74

A8154DF6517E:wifi mac address A8:15:4D:F6:51:7E-->wifi[3]'s mac address is A8:15:4D:F6:51:7E

B2FFFFFF: Convert WiFi signal strength RSSI to big-endian FFFFFFB2 --> Convert to decimal complement - 78 ---> RSSI signal strength of WiFi [0] is - 78

E005C5B1F824: WiFi MAC address E0:05:C5:B1:F8:24--> MAC address of WiFi[4] is E0:05:C5:B1:F8:24

CCFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFCC--> converted to decimal complement -52---> RSSI signal strength of WiFi[0] is -52

E8FCAFA02663: WiFi MAC address E8:FC:AF:A0:26:63--> MAC address of WiFi[5] is E8:FC:AF:A0:26:63

AFFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFFFAF--> converted to decimal complement -81----> RSSI signal strength of WiFi[0] is -81

6409805B2B9C: WiFi MAC address 64:09:80:5B:2B:9C--> MAC address of WiFi[6] is64:09:80:5B:2B:9C

AEFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFAE--> converted to decimal complement-82---> RSSI signal strength of WiFi[0] is-82

94: checksum

The parameters parsed from the message need to be connected to the map WiFi database, and the latitude and longitude data is finally obtained through the map party. Take AMAP connecting to the WiFi database as an example:

According to the access network mode, the AMAP WiFi positioning protocol document is divided into three positioning scenarios. The following gives corresponding examples for the three positioning scenarios.

1. The network connection mode is WiFi access, that is, the accesstype parameter is 1

http://apilocate.amap.com/position?accesstype=1&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&mmac=50:a7:2b:1e:56:58,-60,alibaba-inc&macs=4c:48:da:25:0b:11,-59,alibaba-

inc|4c:48:da:25:1a:11,-77,alibaba-inc&serverip=10.2.166.4&output=xml&key=<User Key>

2. When the network connection mode is mobile access and the mobile phone card is a non-CDMA card, that is, when the accesstype parameter is 0 and cdma=0:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=0&bts=460,01,40977,2205409,-65&nearbts=460,01,40977,2205409,-65|460,0

1,40977,2205409,-65|460,01,40977,2205409,-65&serverip=10.2.166.4&output=xml&key=<User Key>

3. When the network connection mode is mobile access and the mobile phone card is a CDMA card, that is, when the accesstype parameter is 0 and cdma=1:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=1&bts=13824,1,1838,1674723,575739,-52&serverip=10.2.166.4&output=xml&key=<User Key>

W200P Watch: AMAP wifi positioning protocol calls AMAP interface description: When selecting parameters, generally select the first type

You can also choose the second option, but if the device does not report the base station, then the two parameters bts and neabts need to be removed before it can be done.

The first type, incoming parameter example: mmac is the first WIFI mac address in the messagehttp://apilocate.amap.com/position?accesstype=1&imei=869907057780989&mmac=04:8C:16:A9:FB:A8,-44,TP-Link&macs=04:8C:16:A9:FB:A8,-44,TP-Link|C8:3A:35:F4:CE:18,-59,TP-Link|1C:78:4E:D5:07:FE,-79,TP-Link|C8:3A:35:9D:0B:E0,-87,TP-Link|4A:D8:90:E8:0A:82,-88,TP-Link|AC:AD:4B:C9:C3:87,-89,TP-Link|A4:16:E7:55:84:AC,-91,TP-Link&output=json&key=xxxx

Click to view and download the specific AMAPwifi positioning protocol PDF:(not a screenshot, click to view)

This document is provided by AMAP. For specific docking, you can directly ask AMAP
Note that the name of the downloaded document will change, you can rename it yourself

4.3.3 Bluetooth Location Information (BLE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xD6 | See below See definition below | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed to 0 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple sets of information, each group may have multiple ibeacons |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (One of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (One of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (One of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (One of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | RSSI (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (One of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | RSSI (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (One of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| ... | ... | ... | ... |

Note: In the universal version, the total number of Bluetooth beacon groups is fixed at 1, and the maximum number of Bluetooth beacons is 4. The sorting is based on signal strength from strongest to weakest. By default, the watch only recognizes our own beacons. If you need to connect to other manufacturers' beacons, please consult relevant personnel via WeChat. Example:

Original hexadecimal message: BDBDBDBDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

bdbdbdbd: Header

D6: Message ID

00: type

01: Only one set of ibeacon data

E377BD67:Timestamp Timestamp Convert to big endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

04: The first group has 4 beacon information

4327: Convert to big-endian --> major : 0x2743-->Convert to decimal 10051

1794: Convert to big-endian -->minor: 0x9417-->Convert to decimal 37911

AC:  RSSI 0xAC-->Convert to decimal complement -84

4327: Convert to big-endian-->major: 0x2743-->Convert to decimal 10051

3094: Convert to big-endian-->minor:0x9430-->Convert to decimal 37936

AA:  RSSI 0xAA-->Convert to decimal complement -86

4327:Convert to big-endian-->major: 0x2743-->Convert to decimal 10051

b956:Convert to big-endian-->minor:0x56b9-->Convert to decimal 22201

A5:   RSSI 0xA5-->Convert to decimal complement -91

4327: Convert to big-endian-->major: 0x2743-->Convert to decimal 10051

FE94: Convert to big-endian-->minor:0x94FE-->Convert to decimal 38142

A5: rssi:0xA5-->Convert to decimal complement -91

6a --Checksum

Bluetooth beacon major, minor description: The default Bluetooth beacon comes with a beacon code.

For example: 00081005345866, then the major of the Bluetooth beacon is 10053, the minor is 45866, and the manufacturer ID is 0008 (this is useless and has no practical significance)

4.3.3 UWB Location Information (MsgId=0xD7)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xD7 | See below See definition below | CK_sum |

Payload contents

| Format() | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 01 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple sets of information, and there may be multiple Ubeacons in each group |
| Int32 | Timestamp | 4 | UTC timestamp |
| U8 | Total_PackCount | 1 | Total number of UBeacons at the current time |
| U32 | UBeacon0 mac | 4 | UBeacon devid (little endian first) |
| U16 | Distance0 | 2 | Distance (centimeters) |
| U32 | UBeacon1 mac | 4 | UBeacon devid (little endian first) |
| U16 | Distance1 | 2 | Distance (centimeters) |
| U32 | UBeacon2 mac | 4 | UBeacon devid (little endian first) |
| U16 | Distance2 | 2 | Distance (centimeters) |
| ... | ... | ... | ... |

Note: The general version reports a maximum of 1 group, with a maximum of 4 UBeacons

Example:

Original16hexadecimal message: BDBDBDBDD70101E377BD6703320600009100E70500006800380500005C00A1

BDBDBDBD: Header

D7: Message ID

01: Type 01

01: Total_groups Total number of groups, converted to decimal01--> The total number of groups is1

E377BD67: Timestamp Convert to big-endian 67BD77E3--> Convert to decimal1740470243--> Timestamp is1740470243seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

03: Total_PackCount Number of UWB beacons Convert to10hexadecimal03--> Number of UWB beacons is3

32060000: UBeacon0 mac UWB beacon id Convert to big-endian00000632--> UWB beacon id is00000632

6800: Distance0 Ranging distance Convert to big-endian0091--> Convert to decimal145--> Ranging distance is145cm

E7050000: UBeacon1 mac UWB beacon id Convert to big-endian 000005E7--> UWB beacon id is000005E7

9100: Distance1 Ranging distance Convert to big-endian0068--> Convert to decimal104--> Ranging distance is104cm

38050000: UBeacon2 mac UWB beacon's id is converted to big-endian00000538-->UWB beacon id is00000538

5 C00: Distance2 Distance measurement distance converted to big-endian 005 C-->Convert to decimal 92 -->Distance measurement distance is 92 cm

A1: checksum

4.4 Device Information and Status Reporting

4.4.1 ICCID Upload of SIM Card (MSGID=0xF3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xF3 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 10 | 10*U8 | ICCID | 1 | - | ICCID number - Device SIM card number ICCID |

Note: This message will be reported once every time the device is powered on

Example:

Original hexadecimal message: BDBDBDBDF389861118236001639994CC

BDBDBDBD:Header

F3:Message ID

89861118236001639994:ICCID Device SIM card number ICCID is: 89861118236001639994

CC:checksum

4.4.2 Device Charging Status Upload (MSGID=0xC3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Status | / | / | 0 start, 1 end, 2 fully charged |
| 1 | U32 | Timestamp | / | / | Timestamp (timestamp will be added at the end during retransmission) |

Note: This message requires device firmware support. The default latest general version supports it. This message will only be reported when the downlink command is enabled: charging and booting.
Example:

Original hexadecimal message:BDBDBDBDC301E377BD678A

BDBDBDBD:Header

C3:Message ID

01:Device finished charging

E377BD67:Timestamp Timestamp Convert to big endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

Original hexadecimal message:BDBDBDBDC300DB4D2F668A

BDBDBDBD:Header

C3:Message ID

00:Device starts charging

E377BD67:Timestamp Timestamp Convert to big endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

Original hexadecimal message:BDBDBDBDC302DB4D2F668A

BDBDBDBD:Header

C3:Message ID

02:Device is fully charged

E377BD67:Timestamp Timestamp Convert to big endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

4.4.3 Status parameter report (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Number of parameter types |
| 1 |  |  |  |  | (Reserved 00) |
| 1 | u8 | Type | -/- | -/- | Parameter type 1, Type definition: 00—MCU (firmware), 01—module, 02 sensor (general version does not have this type) |
| 1 | U8 | NameLen | -/- | -/- | Parameter name 1 length |
| n | N*u8 | Name | -/- | -/- | Parameter name |
| 1 | u8 | Type | -/- | -/- | Parameter type 2, Type definition: 00—MCU (firmware), 01—module, 02 sensor (general version does not have this type) |
| 1 | U8 | NameLen | -/- | -/- | Parameter type 2 length |
| n | N*u8 | Name | -/- | -/- | Parameter name |
| ... | ... | ... | ... | ... | ... |

Note: This message is reported once by default at startup, uploading the device firmware version number, module version number, which can be left unparsed. The Message ID of previously shipped devices was 0xBB, which can be sent by contacting relevant personnel.

Example:

BDBDBDBDA902000009423732352E4F56303601174E5432364B434E4230304E4E412D4C3032303330393530B5

BDBDBDBD:Header

A9:Message ID

02:TypeCnt indicates that there are 2 types of parameters

00:Reserved field

00:Type=00 00 represents MCU (firmware)

09:MCU name length 9 bytes after

423732352E4F563036--->Hex to text (device firmware version number)--->B725.OV06

01:Type=01 01 represents module

17:Communication module name length 23 bytes after

4E5432364B434E4230304E4E412D4C3032303330393530:Hex to text (4G module version number)

--->NT26KCNB00NNA-L02030950

B5---checksum

4.4.4 Device Status (MSGID=0xE9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xE9 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | type |  | / | Default—00 |
| 2 | u16 | length |  | / | Length of the following message |
| 1 | u8 | location |  | / | Location reporting frequency: 00—-default not modified, 01—-modified by downlink (if there are multiple time periods in the downlink, only the reporting frequency of the current time period is reported) |
| 2 | u16 | frequency |  | / | Frequency, unit - minutes |
| 1 | u8 | health |  | / | Health reporting frequency: 00 - default not modified, 01 - modified by downlink |
| 2 | u16 | frequency |  | / | Frequency: Unit - minutes |

Note: This message is reported once at startup, and once after the downlink modifies the frequency. The latest general version has it, but previous versions do not.

Example:

Original hexadecimal message:BDBDBDBDE9000600010A00010A0033

BDBDBDBD:Header

E9:Message ID

00:type=00

0600:length The length of the message behind it is converted to big-endian -->0006--> converted to decimal 6, and the message length is 6 bytes

01:location 01 indicates that the downlink has modified the location reporting frequency

0A00:frequency The location reporting frequency is converted to big-endian -->000A--> converted to decimal 10, and the reporting frequency is 10 minutes

01:health 01 indicates that the downlink has modified the health reporting frequency

0A00:frequency The health reporting frequency is converted to big-endian -->000A--> converted to decimal 10, and the reporting frequency is 10 minutes

33:checksum

If the device does not have a health reporting function or a location reporting function, the two values of health reporting frequency and location reporting frequency will remain the same, indicating the device reporting frequency.

4.5 Downlink Feedback Related Reporting

4.5.1 Downlink Feedback (MSGID=0xC0)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xC0 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID Length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This command is used for feedback of downlink commands, returning the Message ID of the downlink command received earlier (multiple Message IDs can be returned collectively).

Original Hexadecimal Message:BDBDBDBDC0011720

BDBDBDBD:Header

C0:Message ID

01:length Message ID length Convert decimal 01--> Message ID length is 1 byte

17  Type indicates that the device received a downlink instruction with Message ID 17

20:checksum

5 Settings

5.1 Downlink

5.1.1 Set the location reporting frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x17 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - hour |  |
| 1 | u8 | time_start_m |  |  | - minute |  |
| 1 | u8 | time_end_h |  |  | - hour |  |
| 1 | u8 | time_end_m |  |  | - minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - hour |  |
| 1 | u8 | time_start_m |  |  | - minute |  |
| 1 | u8 | time_end_h |  |  | - hour |  |
| 1 | u8 | time_end_m |  |  | - minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - hour |  |
| 1 | u8 | time_start_m |  |  | - minute |  |
| 1 | u8 | time_end_h |  |  | - hour |  |
| 1 | u8 | time_end_m |  |  | - minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - hour |  |
| 1 | u8 | time_start_m |  |  | - minute |  |
| 1 | u8 | time_end_h |  |  | - hour |  |
| 1 | u8 | time_end_m |  |  | - minute |  |

Note: One day is from 00:00-23:59
Example:

00:00-23:59 Report location every 10minutes:BDBDBDBD17010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

17:Message ID

01:enable is enabled Convert to decimal01-->1--Enabled

0A00:Interval time interval converted to big-endian000A-->converted to decimal10-->time interval is10minutes

00:time_start_h Start time-hour, convert to decimal00-->Start time-hour is00

00:time_start_m Start time-minute, convert to decimal00-->Start time-minute is00

17:time_end_h End time-hour, convert to decimal23-->End time-hour is 23

3B:time_end_m End time-minute, convert to decimal59-->End time-minute is 59

00:enable is enabled Convert to decimal00-->0--Not enabled

000000000000: All filled in disabled state 00

00:enable is enabled Convert to decimal00-->0--Not enabled

000000000000: All filled in disabled state 00

00:enable is enabled Convert to decimal00-->0--Not enabled

000000000000: All filled in disabled state 00

97: checksum

5.1.2 Settings - Positioning Priority Settings (0XCE01 & 0XCE02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See definition below | CK_sum |

Payload contents

Type=01 Positioning priority setting

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 is always valid, 01 is effective this time |
| 2 | U16 | Len | -/- | -/- | Length of the following instruction |
| n | n |  | -/- | -/- | Body |

Location priority setting definition -- See 2.3 Device Downlink Description -- Device Location Priority Distribution (0xCE01)

Location priority setting text:

Currently, the general version supports: 01 -- Satellite (GPS/BDS), 02 -- wifi (wifi + base station) 03 -- Bluetooth beacon (requires additional deployment of Bluetooth beacons), 06 --- UWB beacon (requires additional deployment of UWB beacons);

Special equipment support: 04 -- LBS base station 05--125k

Example:

wifi positioning priority (wifi>Bluetooth beacon>Satellite (GPS/BDS)) : BDBDBDBDCE0100030002030133

BDBDBDBD:Header

CE : Message ID

01 : Type 01 Location priority setting

00 : Valid Validity 00 Always valid

0300 : Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

020301 : Text: 02--wifi 03--Bluetooth beacon 01--Satellite (GPS/BDS)  The location priority is wifi>Bluetooth beacon>Satellite (GPS/BDS)

33:checksum

Satellite (GPS/BDS) positioning priority (Satellite (GPS/BDS)>wifi>Bluetooth beacon):BDBDBDBDCE0100030001020333

BDBDBDBD:Header

CE : Message ID

01 : Type 01 Location priority setting

00 : Valid Validity 00 Always valid

0300 : Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

010203:Text: 01--Satellite (GPS/BDS) 02--wifi 03--Bluetooth beacon Positioning priority is Satellite (GPS/BDS)>wifi>Bluetooth beacon

33:checksum

Bluetooth positioning priority (Bluetooth beacon>wifi>Satellite (GPS/BDS)):BDBDBDBDCE0100030003020133

BDBDBDBD:Header

CE : Message ID

01 : Type 01 Location priority setting

00 : Valid Validity 00 Always valid

0300 : Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

030201:Text: 03--Bluetooth beacon 02--wifi 01--Satellite (GPS/BDS) Positioning priority is Bluetooth beacon>wifi>Satellite (GPS/BDS)

33:checksum

UWB positioning priority (UWB beacon>wifi>Satellite (GPS/BDS)):BDBDBDBDCE0100030006020133

BDBDBDBD:Header

CE : Message ID

01 : Type 01 Location priority setting

00 : Valid Validity 00 Always valid

0300 : Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

060201:Text: 06--UWB beacon 02--wifi 01--Satellite (GPS/BDS) Positioning priority is Bluetooth beacon>wifi>Satellite (GPS/BDS)

33:checksum

Single Satellite (GPS/BDS) positioning:BDBDBDBDCE0100030001000033

BDBDBDBD:Header

CE : Message ID

01 : Type 01 Location priority setting

00 : Valid Validity 00 Always valid

0300 : Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

030201:Text: 01--Satellite (GPS/BDS) 00--No positioning 00--No positioning Positioning priority is single Satellite (GPS/BDS) positioning

33:checksum

Single wifi positioning:BDBDBDBDCE0100030002000033

BDBDBDBD:Header

CE : Message ID

01 : Type 01 Location priority setting

00 : Valid Validity 00 Always valid

0300 : Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

020000:Text: 02--wifi 00--No positioning 00--No positioning Positioning priority is single wifi positioning

33:checksum

Single Bluetooth beacon positioning:BDBDBDBDCE0100030003000033

BDBDBDBD:Header

CE : Message ID

01 : Type 01 Location priority setting

00 : Valid Validity 00 Always valid

0300 : Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

020000:Body: 03--Bluetooth beacon 00--No positioning 00--No positioning Positioning priority is single Bluetooth beacon positioning

33:checksum

Single UWB beacon positioning:BDBDBDBDCE0100030006000033

BDBDBDBD:Header

CE : Message ID

01 : Type 01 Location priority setting

00 : Valid Validity 00 Always valid

0300 : Len Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

060000:Body: 06--UWB beacon 00--No positioning 00--No positioning Positioning priority is single Bluetooth beacon positioning

33:checksum

5.1.3 Settings - Switch Settings (0XCE04-24)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Please refer to the instructions for the specific types |
| 1 | u8 | switch | -/- | -/- | Switch 00 On, 02 Off, 01 Single Effective |
| 2 | U16 |  | -/- | -/- | Fixed 0000 |

Device alarm setting - switch type: specific function description--see 2.3 Device downlink description

05 Bluetooth broadcast switch

16Is it possible to use the button to shut down downstream?

18Hibernate function switch

19Button triggers SOS alarm switch

21Charging is power on or power off state

22Long and short connection mode switching

Type=05 Bluetooth broadcast switch
Example:

00---Open:BDBDBDBDCE0500000093

BDBDBDBD:Header

CE : Message ID

05:Type 05 Bluetooth broadcast switch

00:switch switch 00 on

0000:Fixed value

93: checksum

02---Close:BDBDBDBDCE0502000093

BDBDBDBD:Header

CE : Message ID

05:Type 05 Bluetooth broadcast switch

02:switch switch 02 off

0000:Fixed value

93: checksum

Type=16 Whether the downlink can use the button to shut down Example:

00---Open:BDBDBDBDCE1600000093

BDBDBDBD:Header

CE : Message ID

16:Type 16 Whether the downlink can use the button to shut down

02:switch switch 00 on

0000:Fixed value

93: checksum

02---Close:BDBDBDBDCE1602000093

BDBDBDBD:Header

CE : Message ID

16:Type 16 Whether the downlink can use the button to shut down

02:switch switch 02 off

0000:Fixed value

93: checksum

Type=18 Sleep function switch Example:

00---Turn on, enter sleep after a period of stillness, and do not report data:BDBDBDBDCE1800000093

BDBDBDBD:Header

CE : Message ID

18:Type 18 sleep function switch

02:switch switch 00 on

0000:Fixed value

93: checksum

02---Close, still or charging state, also normally report data:BDBDBDBDCE1802000093

BDBDBDBD:Header

CE : Message ID

18:Type 18 sleep function switch

02:switch switch 02 off

0000:Fixed value

93: checksum

Type=19 Key trigger sos alarm switch
Example:

00---On:BDBDBDBDCE1900000093

BDBDBDBD:Header

CE : Message ID

19:Type 19 key trigger sos alarm switch

02:switch switch 00 on

0000:Fixed value

93: checksum

02---Off:BDBDBDBDCE1902000093

BDBDBDBD:Header

CE : Message ID

19:Type 19 key trigger sos alarm switch

02:switch switch 02 off

0000:Fixed value

93: checksum

Type=21 Charging is power on or power off state
Example:

00---Charging is power on state:BDBDBDBDCE2100000093

BDBDBDBD:Header

CE : Message ID

21:Type 21 Charge to power on or charge to power off

02:switch The switch 00 is charging to power on

0000:Fixed value

93: checksum

02---Charging is off:BDBDBDBDCE2102000093

BDBDBDBD:Header

CE : Message ID

21:Type 21 Charge to power on or charge to power off

02:switch switch 02 Charging is off

0000:Fixed value

93: checksum

Type=22 Long and short connection mode switching
Example:

00---Long connection mode, default is long connection:BDBDBDBDCE2000000093

BDBDBDBD:Header

CE : Message ID

22:Type 22 Long and short connection mode switching

02:switch switch 00 Enable long connection mode

0000:Fixed value

93: checksum

02---Short connection mode:BDBDBDBDCE2002000093

BDBDBDBD:Header

CE : Message ID

22:Type 22 Long and short connection mode switching

02:switch switch 02 Turn off long connection mode--Short connection mode

0000:Fixed value

93: checksum

5.1.4 IP&Domain Name Settings (0xC3) (TCP Dedicated)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | Issue type: Type =1 IPv4, Type =2 IPv6 (reserved item - not supported yet), Type =3 domain name (currently only supports ascii encoded domain names) |
| 2 | U16 | Port |  |  | Port number (2 bytes) |
| 1 | U8 | Len |  |  | Length of subsequent parameter content |
| N | U8 | Domainname |  |  | Specific IP or domain name content |

Note: After being issued, the device will no longer communicate on the original server, but will point to the modified server.

Example:

Type=1 IPv4:Downlink modifies the IP to:118.178.184.219, and the port is: 8825 BDBDBDBDC30179220476B2B8DB33

BDBDBDBD:Header

C3:Message ID

01:Type 1--IPv4

7922:Port Port converted to big-endian 2279-->converted to decimal 8825-->Port: 8825

04:Len Length of subsequent parameter content converted to decimal 04-->Length is 4 bytes

76B2B8DB:Domainname IP 76-->Convert to decimal 118, B2-->Convert to decimal 178, B8-->Convert to decimal 184, DB-->Convert to decimal 219 IP is: 118.178.184.219

33:checksum

Type=3 Domain nameDownstream modification domain name is:aiday.com.cn, port is: 8825

BDBDBDBD:Header

C3:Message ID

03:type=3, domain name

7922:Port port converts to big-endian -- 2279 --> converts to decimal port: 8825

12:Len The length of the content after the parameter is converted to decimal 18 --> The message length is 18 bytes

61696461792e636f6d2e636e:Domainname domain name converted to string --> aiday.com.cn --> domain name is: aiday.com.cn

1D:checksum

5.1.5 UWB alarm distance issued (0x7900)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x79 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type 0 - UWB ranging alarm distance |
| 1 | U8 | distance | -/- | -/- | The distance for UWB ranging alarm (unit: meters; value range 0-20), an alarm will be triggered if it is less than or equal to this distance. |

Example:

The alarm distance is 1 meter. If the ranging distance between the watch and the UWB beacon is less than 1 meter, the watch will vibrate and alarm BDBDBDBD790001C3.

BDBDBDBD:Header

79:Message ID

00:Type 00--UWB ranging alarm distance

01:distance The distance of UWB ranging alarm is converted to decimal 01-->The distance of UWB ranging alarm is 1 meter

C3:checksum