W200PG-4GCat.1-MQTT Communication Protocol

Directory

1 Overview	3

1.1 Transmission Rules	3

1.2 Topic Definition	3

2 Device Usage Instructions	3

2.1 Device Function and Usage Instructions	3

2.2 Device default reporting logic	5

2.3 Device downlink description	6

3 Protocol Data Packet Structure	8

3.1 Data Header	8

3.2 Message Identifier (Message ID)	8

3.3 Token Generation Mechanism	9

3.4 Payload	9

3.5 Checksum	10

4 Report messages message	12

4.1 Battery SignalRelated Reporting	12

4.1.3 Battery Signal Upload(MSGID=0xF9)	12

4.2 Alarm-related reporting	14

4.2.1 Alarm data upload - 1 (MSGID=0x02)	14

4.2.2 Alarm Data Upload-2 (MSGID=0x21) (Supplement to 02)	18

4.2.3 Abnormal data alarm (MSGID=0x16)	23

4.3 Location-related reporting	24

4.3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)	24

4.3.2 WiFi and base station information upload (MSGID=0xA4)	27

4.3.3 Bluetooth Positioning Information (LBE Location) (MsgId=0xD6)	31

4.3.4 Base station latitude and longitude reporting (MSGID=0x15) — wifi positioning supplement	35

4.4 Device Information and Status Reporting	37

4.4.1 ICCID Upload of SIM Card (MSGID=0xF3)	37

4.4.2 Device charging status upload (MSGID=0xC3)	38

4.4.3 Status parameter reporting (MSGID=0xA9)	39

4.  5 Health-related reporting	41

4.  5.1 Health Data Data (MSGID=0x32)	41

4.5.2 Device Sleep Analysis Data Upload (MSGID=0xC5)	44

4.6 Downlink feedback related reporting	45

4.6.1 Downlink feedback (MSGID=0xC0)	45

4.6.2 Message Status Report (MSGID=0x28)	46

5 Settings	48

5.1 Downlink	48

5.1.1 Set the location reporting frequency (MSGID=0x17)	48

5.1.2 Message Send (MSGID=0X28)	50

5.1.3 Settings - Location Priority Settings (0XCE01)	52

5.1.4 Settings - Health Sample Reporting Frequency Settings (0XCE02)	55

1 Overview

This protocol is suitable for 4G CAT1 devices and currently supports W200PG products. It uses a 32-bit data header for synchronization and terminal identification; it uses a low-overhead verification algorithm for verification protection; and it uses message identifiers to identify different messages.
It needs to be parsed according to the actual report. The document is currently being continuously maintained. If you find any errors or omissions, please give feedback in time. Thank you very much.
The protocol content is 4g report content. If you need Bluetooth broadcast data protocol, please consult relevant docking personnel.

1.1 Transmission Rules

Data is transmitted in hexadecimal, and integer data is transmitted in little-endian byte order.

1.2 Topic Definition

Device reporting data Topic: oc_device_report/devices/{Device ID}/properties

Device issuing data Topic: oc_device_report/devices/{Device ID}/down

Downlink format: Downlink raw message16hexadecimal

Where the device ID is the device IMEI number

2 Device Usage Instructions

2.1 Device Function and Usage Instructions

General version:

(1) Power on:

Please fully charge the device before using it for the first time. The device will automatically power on when the charging reaches the power required for booting. The charging status displays the charging icon.

A full charge is indicated by a green icon. Note: Do not check the device signal while charging. Manual power on: Press and hold the up button for 10 seconds and release. The interface will display the word “Welcome”.

Note: The default wearing state is enabled upon startup, and a detachment alarm will be reported if no heart rate is detected.

(2) Power off:

Low battery shutdown: The screen will turn off after the interface displays Byebye.

Manual shutdown: Shutdown: Press and hold the button for more than 10 seconds, the interface displays Byebye and then the screen goes off.

(3) SOS function:

Trigger mode: After triggering, the device does not enter sleep mode. Long press the down key for 3s, the interface displays SOS SEND OK/SOS sending successfully, and then the SOS icon appears.

Cancellation method: In SOS mode, long press the down key for 3s, the interface displays SEND CENCEL/SOS canceled, and the SOS icon disappears.

(4) Signal status:

No signal: The interface signal is a horizontal line. Signal available: The interface signal has a stepped bar graph.

(5) Device sleep:

Trigger condition: The device does not move for 40 minutes and enters sleep mode, does not report location health data, and only reports F9 heartbeat packets to maintain a long connection.

(7) Vibration:

Received downlink text message, abnormal data alarm

2.2 Device default reporting logic

General version:

(1) Location-related reporting

Satellite (GPS/BDS)/ WiFi / Bluetooth beacon: Default reporting frequency 10 minutes, default positioning priority: WiFi > Satellite (GPS/BDS), WiFi positioning is preferred, switch to Satellite (GPS/BDS) positioning if WiFi positioning is not available.

Base station (0x15): The latitude and longitude of the communication base station obtained by automatically requesting from the module after WiFi positioning fails. The accuracy is not high and is used for auxiliary reference. Generally, there is no such report in the environment. The positioning priority cannot be issued and is bound to WiFi positioning.

(2) Alarm-related reports

SOS alarm (0x02): User actively triggers, see the previous section for triggering method

SOS cancellation (0x02): User actively triggers cancellation, see the previous section for triggering method

Power-off alarm (0x21): Device actively powers off or low battery powers off, see the previous section for triggering method

Wearing and falling off alarm (0x02): The device determines wearing according to the health sampling reporting frequency. If the heart rate is measured, the wearing alarm is reported. If the heart rate is not measured, the falling off alarm is reported.

Sedentary stay alarm (0x02): Default 15 minutes of immobility triggers reporting

Fall alarm (0x02): The device falls freely from a certain height and meets the fall algorithm to trigger

Low battery alarm (0x02): Triggered when the current battery level of the device is less than or equal to 0

ExceptionDataAlarm(0x16): Report when the device detects a temperature exceeding40degrees Celsius

(3) Health-related reporting

Step count, heart rate, body temperature&wrist temperature, blood pressure, blood oxygen (0x32): Default reporting frequency10minutes

Sleep (0xC5): Statistical time period21:00-08:00, sleep data will be reported according to the status during this time period

(4) Device information and status reporting

Status parameters (0xA9): One report will be made when the device is turned on

SIM card ICCID (0xF3): One report will be made when the device is turned on

Charging status (0xC3): Report when charging starts, charging ends, and when fully charged

Battery signal (0xF9), reported once every 4 minutes by default, (previous firmware) a report will also be made after positioning and health reporting

(5) Downlink feedback

Downlink feedback (0xC0): Reported after the device receives the downlink command from the server

Message Status Feedback (0x28): After the downlink text message, the device clicks accept or reject

Note: The device reports the situation of packet reporting, that is, a data packet contains multiple complete messages. Pay attention not to miss it. The message is a complete message, and there will be no phenomenon of being interrupted in the middle in the next data packet.

Eg:bdbdbdbdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9bdbdbdbdf9010000006400002800000019a9cf61ca

This data packet contains (0xD6) Bluetooth positioning and (0xF9) power signal messages

(0xD6) Bluetooth positioning:bdbdbdbdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9

(0xF9) Power signal:bdbdbdbdf9010000006400002800000019a9cf61ca

2.3 Device downlink description

General version: The downlink command needs to be continuously downlinked twice within 20 milliseconds to ensure success.

(1) Device location reporting frequency distribution (0x17):

The default reporting frequency is 10 minutes, the highest is 1 minute, and the lowest is 1440 minutes. After the device receives the downlink command, the device presses the time period and frequency of the issued command.

Report data, report according to the default reporting frequency outside the time period, such as: 00:00-18:00 2 minutes of positioning report,

Then report at the default 10-minute reporting frequency outside the time period.

(2) Text message delivery (0x28):

GB2312 encoding, up to 40 Chinese characters, one Chinese character occupies 2 bytes, and one English letter occupies 1 byte.

Message list - up to 50 historical messages can be saved (Note: Firmware without a message list can save up to 20 messages)

(3) Device positioning priority delivery (0xCE01):

Default positioning priority wifi>Satellite (GPS/BDS), positioning priority

If the positioning priority is delivered as: wifi>Satellite (GPS/BDS)>Bluetooth beacon, then if wifi cannot be located, switch to Satellite (GPS/BDS), and if Satellite (GPS/BDS) cannot be located, switch to

Bluetooth beacon, when the positioning is successful, it will not switch to the next positioning priority to generate positioning.

(4) Device health sampling frequency delivery (0xCE02):

The default health sampling frequency is 10 minutes, and the maximum is 2 minutes.

3 Protocol Data Packet Structure

A basic protocol data packet structure is shown in Figure 1:

3.1 Data Header

Each data packet starts with a 4-byte Header or token (in some response messages, timestamp is used instead):

Currently, the token uploaded by OuFu devices is fixed at BDBDBDBD

Header: 0xBD 0xBD0xBD0xBD；

Timestamp: 32bits, generated by the server

3.2 Message Identifier (Message ID)

See Chapter 4 for the content represented by MessgeId.

Each time a TCP connection is made, the device will first report a 0xF0 message, which contains the device's unique identifier IMEI.

The server side needs to record this IMEI as an identifier and reply with a 0xf1 message. Only then will the device side consider this connection successful; otherwise, the connection will be disconnected.

The device communicates normally. The long connection reports a 0xF9 heartbeat packet every 4 minutes by default. The 0xF9 heartbeat packet is reported once after the health location is reported.

The short connection reports a 0xF9 heartbeat packet once at startup, and a 0xF9 heartbeat packet is reported once after the health location is reported. The reporting of the two is not affected by each other.

Note that after the server receives the F9 message, the server needs to issue a command to reply, otherwise the connection will be disconnected.

3.3 Token Generation Mechanism

Currently fixed as BDBDBDBD

3.4 Payload

The payload below refers to the valid body content in the protocol except for the head token and checksum. The following note is the length of the body.

The data format used in the payload is shown in the following table:

【U-unsigned；I-signed；X-bitfield；Number-bytes occupied】

The following protocols use little-endian priority except for ch, u8, i8, and x8

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Description |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short Integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short Integer |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Integer |
| x16 | Bitfield | 2 | - | - | Bit (bit) 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long Integer |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long Integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit Long Integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating point type |

3.5 Checksum

The content added to the checksum includes the payload, as shown in Figure 1. The algorithm is as follows. By default, the general-purpose version of the device does not require checksum, and this part can be ignored. Any byte can be used for the downlink instruction.

//Assuming content="BDBDBDBDE9000600010A00000A00", then sum.ToString("X2") result is 07//The following is the C# code calling method

private string CheckSum(string content)

{

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2");

}

4 Report messages message

4.1 Power signal related report

4.1.3 Power signal upload (MSGID=0xF9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xF9 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery type 0: 4-level system, 1: 5-level system, 2: Percentage, 3: Voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery value If Bat_type is 0: The battery value range is 0-3 (0 is 25%, 3 is 100%) If Bat_type is 1: The battery value range is 0-4 (0 is 20%, 4 is 100%) If Bat_type is 2: The battery value range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: percentage, 1: 5-level system, 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal strength |
| 1 | U8 | Other_type |  |  | Extended type 0: Full step counting 1: Incremental step counting 2: Vibration |
| 4 | U32 | Num |  |  | Extended value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Example:

Original hexadecimal message: BDBDBDBDF90104000050000095000000E377BD67AA

BDBDBDBD: Header

F9: Message ID

01:Bat_type battery type 01-->1-->5-level battery (0-4)

0400:Bat_volt battery value converted to big-endian 0004-->4-->level4corresponds to100% battery

00:Signal_type signal type 00-->0-->percentage

5000:Signal_strength signal strength converted to big-endian0050-->converted to decimal80-->80% signal strength

00:Other_type extension type 00-->0-->full step counting

95000000:Num extension value - step counting converted to big-endian00000095-->converted to decimal149-->number of steps is149

E377BD67: Timestamp converts to big-endian 67BD77E3 --> converts to decimal 1740470243 --> timestamp is 1740470243 seconds --> converts to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> converts to Beijing time: 2025 - 02 - 25 15 : 57 : 23

AA: checksum

4.2 Alarm-related reporting

4.2.1 Alarm data upload - 1 (MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x02 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below for definition |
| 4 | U32 | Timestamp | - | - | Timestamp (a timestamp will be added at the end during retransmission) |

Upl_warn contents

| bit (corresponding to the position of 1 in binary) | Name | Description | Original message in little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 14 | Fall alarm | Fall alarm | 0040 | 4000 | 4*4096=16384 |
| 8 | Device wearing | Device wearing | 0001 | 0100 | 1*256=256 |
| 7 | SOS Cancel | SOS Cancel | 8000 | 0080 | 8*16=128 |
| 5 | Sedentary alarm (no movement) | Sedentary alarm (no movement) | 2000 | 0020 | 2*16=32 |
| 4 | Remove (detach) device | Remove (detach) device | 1000 | 0010 | 1*16=16 |
| 2 | Shutdown | Shutdown | 0400 | 0004 | 4 |
| 1 | SOS alarm | SOS alarm | 0200 | 0002 | 2 |
| 0 | Low battery | Low battery | 0100 | 0001 | 1 |

Example:

Wearing alarm: BDBDBDBD020001E377BD678A

BDBDBDBD:Header

02:Message ID

0001:Upl_warn alarm content, converted to big-endian:0100-->Wearing alarm

E377BD67:Timestamp timestamp converted to big-endian67BD77E3-->convert to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

Fall off alarm: BDBDBDBD021000E377BD678A

BDBDBDBD:Header

02:Message ID

1000:Upl_warn alarm content, converted to big-endian0010-->Fall off alarm

E377BD67:Timestamp timestamp converted to big-endian67BD77E3-->convert to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

SOS alarm: BDBDBDBD020200E377BD678A

BDBDBDBD:Header

02:Message ID

0200:Upl_warn alarm content, converted to big-endian0002-->SOS alarm

E377BD67:Timestamp timestamp converted to big-endian67BD77E3-->convert to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

SOS cancellation: BDBDBDBD028000E377BD678A

BDBDBDBD:Header

02:Message ID

8000:Upl_warn alarm content, converted to big-endian0080-->SOS cancellation

E377BD67:Timestamp timestamp converted to big-endian67BD77E3-->convert to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8a: checksum

Fall alarm: BDBDBDBD020040E377BD678A

BDBDBDBD:Header

02:Message ID

0040:Upl_warn alarm content, converted to big-endian4000-->Fall alarm

E377BD67:Timestamp timestamp converted to big-endian67BD77E3-->convert to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

E7: checksum checksum

Sedentary alarm: BDBDBDBD022000E377BD678A

BDBDBDBD:Header

02:Message ID

2000:Upl_warn alarm content, converted to big-endian0020-->Sedentary alarm

E377BD67:Timestamp timestamp converted to big-endian67BD77E3-->convert to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8A: checksum checksum

Low battery alarm: BDBDBDBD020200E377BD678A

BDBDBDBD:Header

02:Message ID

0100:Upl_warn alarm content, converted to big-endian0001-->Low battery alarm

E377BD67:Timestamp timestamp converted to big-endian67BD77E3-->convert to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8A: checksum checksum

Shutdown alarm: BDBDBDBD020400E377BD678A

BDBDBDBD:Header

02:Message ID

0400:Upl_warn alarm content, converted to big-endian0004-->Shutdown alarm

E377BD67:Timestamp timestamp converted to big-endian67BD77E3-->convert to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8A: checksum checksum

Note: This alarm is for devices that do not differentiate between shutdown types. See the next section for devices that do differentiate between shutdown types.

4.2.2 Alarm Data Upload-2 (MSGID=0x21) (Supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x21 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | Alarm type |
| N |  | Upl_warn | - | - | Alarm content See below for different Alarm type type definitions |
| 4 | U32 | Timestamp |  |  | Timestamp (a timestamp will be added at the end during retransmission) |

Alarm type=1

Upl_warn contents

| bit (corresponding to the position of 1 in binary) | Name | Description | Original message in little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging shutdown alarm | Device charging automatic shutdown (firmware requires function support) | 04000000 | 0004 | 4 |
| 1 | Low battery shutdown alarm | Device power is too low to shut down (firmware requires function support) | 02000000 | 0002 | 2 |
| 0 | Active shutdown alarm | Device is manually shut down by someone (firmware requires function support) | 01000000 | 0001 | 1 |

Example:

Charging shutdown alarm: BDBDBDBD21040001000000E377BD6767

BDBDBDBD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001-->Alarm type=1

04000000:Upl_warn alarm content, converted to big-endian 0004-->Charging shutdown alarm

E377BD67:Timestamp, converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum checksum

Low battery shutdown alarm: BDBDBDBD21020001000000E377BD6767

BDBDBDBD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001-->Alarm type=1

02000000:Upl_warn alarm content, converted to big-endian 0002-->Low battery shutdown alarm

E377BD67:Timestamp, converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum checksum

Active shutdown alarm: BDBDBDBD21010001000000E377BD6767

BDBDBDBD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001-->Alarm type=1

01000000:Upl_warn alarm content, converted to big-endian 0001-->Active shutdown alarm

E377BD67:Timestamp, converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum checksum

Alarm type=5

Upl_warn contents

| Format | Name | Unit | Decription |
| --- | --- | --- | --- |
| U16 | Type | 2 | Fixed to 0x05 |
| U32 | Timestamp | 4 | Timestamp |
| U16 | Content Len | 2 | Alarm content length |
| U8 | Health Warn Type1 | 1 | Health threshold alarm type: 0x01 heart rate, 0x02 systolic blood pressure (SBP), 0x03 diastolic blood pressure (DBP), 0x04 blood oxygen , 0x05 temperature |
| U8 | Type1 | 1 | 01 is less than the set health threshold, 02 is greater than the set health threshold |
| I16 | Health Value1 | 2 | Health result value |
| U8 | Type2 | 1 | 01 is less than the set health threshold, 02 is greater than the set health threshold |
| I16 | Health Value2 | 2 | Health result value |
| .... | .... | .... | .... |

Example:

The device detects that the heart rate (65) is outside the range of the issued health threshold: BDBDBDBD210500E377BD6704000102410073

BDBDBDBD: Header

21:Message ID message id

0500:Alarm typeAlarm type Convert to big-endian0005-->Alarm type=5

E377BD67:Timestamp Convert to big-endian67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

0400:Content Len Alarm content length Convert to big-endian0004-->Alarm content length is 4 bytes, Note: The length of multiple health threshold alarms will increase

01:Health Warn Type Health threshold alarm type 01-->Heart rate alarm

02:Type Greater than the set health threshold & less than the set health threshold  01-->Less than the set health threshold

4100:Health Value Actually reported health value Convert to big-endian0041-->Convert to decimal 65-->Reported heart rate value is 65

73:checksum checksum

4.2.3 Abnormal data alarm (MSGID=0x16)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x16 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type | -/- | -/- | Alarm type (0: abnormal heart rate value alarm, 1: abnormal temperature value alarm) |
| 2 | U16 | Heart | -/- | -/- | Heart rate value |
| 2 | U16 | temperature | -/- | -/- | Temperature |
| 2 | U16 | Pa | -/- | -/- | Air pressure |
| 4 | 4*U8 or U32 | expand | -/- | -/- | 4 bytes extension for other sensors |

Note: The reported outlier has no practical reference value, but only indicates that the currently detected data is abnormal (such as the value exceeding the range or the sensor detecting abnormally, etc.). The outlier will be removed in other normal messages and reflected in this message, so it can be ignored.

Example:

Temperature anomaly value alarm: BDBDBDBD16010000FF0F000000000000E6

BDBDBDBD:Header

16:Message ID

01:Type alarm type 01-->Temperature anomaly value alarm

0000:Heart rate value. If there is no value, fill in 0000 by default.

FF0F:temperature temperature Convert to big-endian 0FFF-->Temperature anomaly value FFF

0000:Pa air pressure. If there is no value, fill in 0000 by default.

00000000 expand expansion, fill in 0000 by default if there is no value

E6:checksum checksum

4.3 Location-related reporting

4.3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x03 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude longitude |
| 8 | Double | lat |  | - | latitude latitude |
| 1 | U8 | north_south |  | - | /N or S/ North or South latitude |
| 1 | U8 | east_west |  | - | E or W/East or West longitude |
| 1 | U8 | status |  | - | A or V/ Valid data or invalid data |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original16Hexadecimal message: BDBDBDBD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BDBDBDBD:Header

03:Message ID message id

963E870C3E675E40:lon longitude Convert to big endian405E673E0C873E96-->Longitude:121.6131622 Note: Refer to the parsing example below for conversion and parsing

FF2110C2B6343F40:lat latitude Convert to big endian`403F34B6C21021FF-->Latitude:31.2059137 Note: Refer to the parsing example below for conversion and parsing

4E:north_south 4E-->Convert to ASCII encoding N-->North latitude

45:east_west 45-->Convert to ASCII encoding E-->East longitude

41:status data validity 41--> convert to ASCII code A --> valid data

E377BD67:Timestamp timestamp converted to big-endian 67BD77E3--> convert to10system1740470243--> timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

78:checksum checksum

Note: The parsed Satellite (GPS/BDS) latitude and longitude are in the WGS-84coordinate system. If the map uses Baidu AMAP, etc., the coordinate system needs to be converted.

Parsing satellite positioning message Java code example:

//Message: BDBDBDBD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BDBDBDBD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public static void main(String[] args){

//Message 7d9f84ac81815c40 actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d",16)));  //114.02353966666665//Message e766926b1d893640 actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16))) ;//22.535605166666667

HexToStr(data.Substring(“4e”));   //N

HexToStr(data.Substring("45"));//E

HexToStr(data.Substring("41"));//A A indicates valid data, V indicates invalid data, which can be discarded//Message 749d695f actual value 5f699d74

Date date=new Date();

date.setTime(Long.parseLong("5f699d74",16)*1000);

SimpleDateFormatsdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.3.2 WiFi and base station information upload (MSGID=0xA4)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xA4 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 1 | u8 | Cell_cnt | 1 | - | Number of base stations (0-7) Number of cell info payload.Valid value:0~7 |
| 2 | u16 | Cell[0].MCC | - | - | MMC of base station (0) mobile country code of cell[0] |
| 2 | u16 | Cell[0].MNC | - | - | MNC of base station (0) mobile network code of cell[0] |
| 2 | u16 | Cell[0].LAC | - | - | LAC of base station (0) Location area code of cell[0] |
| 4 | U32 | Cell[0].CELL_ID | - | - | ID of base station (0) Cell id of cell[0] |
| 2 | i16 | Cell[0].RSSI | - | dbm | Signal Strength RSSI of Base Station (0) RSSI in dbm of cell[0] |
| ... | ... | ... | ... | ... | ... |
| 2 | u16 | Cell[cell_cnt-1].MCC | - | - | MMC of base station (N) mobile country code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1].MNC | - | - | MNC of base station (N) mobile network code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1].LAC | - | - | LAC of base station (N) Location area code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1].LAC | - | - | LAC of base station (N) Location area code of cell[[cell_cnt-1] |
| 4 | U32 | Cell[cell_cnt-1].CELL_ID | - | - | ID of base station (N) Cell id of cell[[cell_cnt-1] |
| 2 | I16 | Cell[cell_cnt-1].RSSI | - | dbm | Signal strength RSSI of base station (N) RSSI in dbm of cell[[cell_cnt-1] |
| 1 | U8 | Wifi_cnt |  |  | Number of wifi Number 0f wifi |
| 6 | 6*U8 | Wifi[0].bssid[0]-Wifi[0].bssid[5] |  |  | MAC address of wifi(0) |
| 4 | I32 | Wifi[0].RSSI |  |  | Signal strength RSSI of wifi(0) |
| ... | ... | ... | ... | ... | ... |
| 6 | 6*U8 | Wifi[Wifi_cnt-1].bssid[0]-Wifi[Wifi_cnt-1].bssid[5] |  |  | MAC address of wifi(N) |
| 4 | I32 | Wifi[0].RSSI |  |  | Signal strength RSSI of wifi(0) |

Note: The device does not necessarily report base stations every time. The absence of base stations does not affect positioning. If the number of base stations in the message is 0, then the number of wifi is directly followed.

Example:

Original16hexadecimal message: BDBDBDBDA4E377BD6701CC010000C218D1AD160BF9FF078CBEBE1A8162C6FFFFFFC061180AF42AC1FFFFFF200BC726E000B6FFFFFFA8154DF6517EB2FFFFFFE005C5B1F824CCFFFFFFE8FCAFA02663AFFFFFFF6409805B2B9CAEFFFFFF94

BDBDBDBD:Header

A4:Message ID message id

E377BD67:Timestamp timestamp converted to big-endian67BD77E3-->convert to10hexadecimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

01:Cell_cnt number of base stations 01-->The number of base stations is1

CC01:Cell[0].MCC Base station MCC converted to big-endian01CC-->converted to decimal460-->Base station MCC is460

0000:Cell[0].MNC Base station MNC converted to big-endian0000-->converted to decimal0-->Base station MNC is0

C218:Cell[0].LAC Base station LAC converted to big-endian18C2-->converted to decimal6338-->Base station LAC is6338

D1AD160B:Cell[0].CELL_ID Base station ID converted to big-endian0B16ADD1-->converted to decimal186035665-->Base station id is186035665

F9FF:Cell[ 0 ].RSSI base station signal strength converted to big-endian FFF9--> converted to decimal complement-7--> base station signal strength is-7

07: Wifi_cnt wifi number 07-->7wifi

8CBEBE1A8162:wifi mac address 8C:BE:BE:1A:81:62-->wifi[0]'s mac address is8C:BE:BE:1A:81:62

C6FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFC6--> converted to decimal complement-58--->wifi[0]'s RSSI signal strength is-58

C061180AF42A:wifi mac address C0:61:18:0A:F4:2A-->wifi[1]'s mac address is C0:61:18:0A:F4:2A

C1FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFC1--> converted to decimal complement-63--->wifi[0]'s RSSI signal strength is-63

200BC726E000:wifi mac address 20:0B:C7:26:E0:00-->wifi[2]'s mac address is20:0B:C7:26:E0:00

B6FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFB6--> converted to decimal complement-74--->wifi[0]'s RSSI signal strength is-74

A8154DF6517E:wifi mac address A8:15:4D:F6:51:7E-->wifi[3]'s mac address is A8:15:4D:F6:51:7E

B2FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFB2--> converted to decimal complement-78--->wifi[0]'s RSSI signal strength is-78

E005C5B1F824: WiFi MAC address E0:05:C5:B1:F8:24--> MAC address of WiFi[4] is E0:05:C5:B1:F8:24

CCFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFCC --> converted to decimal complement - 52 ---> WiFi[ 0 ] RSSI signal strength is - 52

E8FCAFA02663: WiFi MAC address E8:FC:AF:A0: 26 : 63 --> WiFi[ 5 ] MAC address is E8:FC:AF:A0: 26 : 63

AFFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFAF--> converted to decimal complement - 81 ----> WiFi[ 0 ] RSSI signal strength is - 81

6409805B2B9C: WiFi MAC address 64 : 09 : 80 : 5 B: 2 B: 9 C--> WiFi[ 6 ] MAC address is 64 : 09 : 80 : 5 B: 2 B: 9 C

AEFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFAE--> converted to decimal complement - 82 ---> WiFi[ 0 ] RSSI signal strength is - 82

94: checksum checksum

The parameters parsed from the message need to be connected to the map WiFi database, and the latitude and longitude data is finally obtained through the map party. Take AMAP connecting to the WiFi database as an example:

According to the access network method, the AMAP WiFi positioning protocol document is divided into three positioning scenarios. The following gives corresponding examples for the three positioning scenarios.

1. The networking method is WiFi access, that is, the accesstype parameter is 1

http://apilocate.amap.com/position?accesstype=1&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&mmac=50:a7:2b:1e:56:58,-60,alibaba-inc&macs=4c:48:da:25:0b:11,-59,alibaba-

inc|4c:48:da:25:1a:11,-77,alibaba-inc&serverip=10.2.166.4&output=xml&key=<User Key>

2. When the network access method is mobile access and the mobile phone card is a non-CDMA card, that is, when the accesstype parameter is 0 and cdma=0:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=0&bts=460,01,40977,2205409,-65&nearbts=460,01,40977,2205409,-65|460,0

1,40977,2205409,-65|460,01,40977,2205409,-65&serverip=10.2.166.4&output=xml&key=<User Key>

3. When the network access method is mobile access and the mobile phone card is a CDMA card, that is, when the accesstype parameter is 0 and cdma=1:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=1&bts=13824,1,1838,1674723,575739,-52&serverip=10.2.166.4&output=xml&key=<User Key>

W200P watch: AMAP interface description called by AMAP WiFi positioning protocol: When selecting parameters, generally select the first type.

You can also select the second type, but if the device does not report the base station, the bts and neabts parameters need to be removed.

The first type, example of incoming parameters: mmac is the first WIFI mac address in the message:http://apilocate.amap.com/position?accesstype=1&imei=869907057780989&mmac=04:8C:16:A9:FB:A8,-44,TP-Link&macs=04:8C:16:A9:FB:A8,-44,TP-Link|C8:3A:35:F4:CE:18,-59,TP-Link|1C:78:4E:D5:07:FE,-79,TP-Link|C8:3A:35:9D:0B:E0,-87,TP-Link|4A:D8:90:E8:0A:82,-88,TP-Link|AC:AD:4B:C9:C3:87,-89,TP-Link|A4:16:E7:55:84:AC,-91,TP-Link&output=json&key=xxxx

For the specific AMAP  WiFi positioning protocol PDF, click to view and download:(Not a screenshot, can click to view)
This document is provided by AMAP . For specific integration, you can directly inquire with AMAP .
Note that the downloaded document name may change, you can rename it yourself

4.3.3 Bluetooth Positioning Information (LBE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xD6 | See below See definition below | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 0 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple groups of information, each group may have multiple ibeacons |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (One of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (One of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (One of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (One of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | RSSI (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (One of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | RSSI (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (One of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| ... | ... | ... | ... |

Note: In the universal version, the total number of Bluetooth beacon groups is fixed at 1, and the maximum number of Bluetooth beacons is 4. The sorting is based on signal strength from strongest to weakest. By default, the watch only recognizes our own beacons. If you need to connect to other manufacturers' beacons, please consult relevant personnel via WeChat. Example:

Original hexadecimal message: BDBDBDBDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

bdbdbdbd: Header

D6: Message ID

00: type

01: Only one set of ibeacon data

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

04: The first group has 4 beacon information

4327: Convert to big-endian-->major : 0x2743-->Convert to decimal 10051

1794: Big endian --> minor: 0x9417 --> Decimal conversion 37911

AC:  RSSI 0xAC --> Convert to decimal complement -84

4327: Big endian --> major: 0x2743 --> Decimal conversion 10051

3094: Big endian --> minor:0x9430 --> Decimal conversion 37936

AA:  RSSI 0xAA --> Convert to decimal complement -86

4327:Big endian --> major: 0x2743 --> Decimal conversion 10051

b956:Big endian --> minor:0x56b9 --> Decimal conversion 22201

A5:   RSSI 0xA5 --> Convert to decimal complement -91

4327: Big endian --> major: 0x2743 --> Decimal conversion 10051

FE94: Big endian --> minor:0x94FE --> Decimal conversion 38142

A5: rssi:0xA5 --> Convert to decimal complement -91

6a --Checksum

Bluetooth beacon major, minor description: The default shipping Bluetooth beacon has a beacon code attached.

For example: 00081005345866, then the major of the Bluetooth beacon is 10053, the minor is 45866, and the manufacturer ID is 0008 (this is useless and has no practical significance).

4.3.4 Base station latitude and longitude reporting (MSGID=0x15) — wifi positioning supplement

Note: This message is generally when wifi positioning fails, and the module automatically requests the latitude and longitude of the communication base station for positioning (the positioning priority cannot be changed). The location is the location of the communication base station and can only be used as a reference for auxiliary positioning. The accuracy of the communication base station is not high.

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x15 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | flag |  | -/- | Extended segment identifier |
| 4 | U32 | lon |  | -/- | Longitude (*10 to the power of 6 or 7) longitude |
| 4 | U32 | lat |  | -/- | latitude (*10 to the power of 6 or 7) latitude |
| 1 | U8 | north_south |  | -/- | /N or S/ South or North Latitude |
| 1 | U8 | east_west |  | -/- | E or W/East or West longitude |
| 1 | U8 | status |  | -/- | /A or B or V/ A, B are valid data, B corresponds to 7-bit precision, V is invalid data |
| 4 | U32 | Timestamp |  | -/- | Timestamp |
| N |  | Num |  | -/- | The general version has no extensions, and the table only retains the definition. Definition: Extended segment value 1 can be the nth value of the extended segment definition. When multiple extended segments are used simultaneously, extended segment 1 corresponds to the lowest bit content, and the content is extended to bit n in sequence. |
| ... | ... | ... | ... | ... | ... |

Example:

Original16Hexadecimal message: BDBDBDBD1502C8C9E53F855AD30F4E4542E377BD67F4

BDBDBDBD: Header

15: Message ID

02:type  type=02indicates LBS base station positioning

C8C9E53F: Longitude ---> converted to big endian 4857963E---> converted to longitude:26.5509509, Note: Refer to4.3.1GPS/ BDS location reporting for conversion and parsing: Positioning data reporting (0x03) section parsing example

855AD30F: Latitude ---> converted to big endian129B82E0---> converted to latitude:107.2024008, Note: Refer to4.3.1GPS/ BDS location reporting for conversion and parsing: Positioning data reporting (0x03) section parsing example

4E:north_south  4E--> converted to ASCII code N--> North Latitude

45:east_west 45--> Convert to ASCII code E--> East longitude

42:status 42-->Convert to ASCII code B, valid data, precision is 7 digits after the decimal point

E377BD67:Timestamp Convert to big endian 67 BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

08:checksum checksum

4.4 Device Information and Status Reporting

4.4.1 ICCID Upload of SIM Card (MSGID=0xF3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xF3 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 10 | 10*U8 | ICCID | 1 | - | ICCID number - Device SIM card number ICCID |

Note: This message will be reported once every time the device is powered on

Example:

Original hexadecimal message:BDBDBDBDF389861118236001639994CC

BDBDBDBD:Header

F3:Message ID

89861118236001639994:ICCID Device SIM card number ICCID is: 89861118236001639994

CC:checksum

4.4.2 Device charging status upload (MSGID=0xC3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Status | / | / | 0 start, 1 end, 2 fully charged |
| 1 | U32 | Timestamp | / | / | Timestamp (a timestamp will be added at the end during retransmission) |

Note: This message requires device firmware support, the default latest general version supports Example:

Original hexadecimal message:BDBDBDBDC301E377BD678A

BDBDBDBD:Header

C3: Message ID

01:Device finished charging

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum checksum

Original hexadecimal message:BDBDBDBDC300DB4D2F668A

BDBDBDBD:Header

C3: Message ID

00:Device started charging

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum checksum

Original hexadecimal message:BDBDBDBDC302DB4D2F668A

BDBDBDBD:Header

C3: Message ID

02:Device is fully charged

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum checksum

4.4.3 Status parameter reporting (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Number of parameter types |
| 1 |  |  |  |  | (Reserved 00) |
| 1 | u8 | Type | -/- | -/- | Parameter type 1, Type definition: 00—MCU (firmware), 01—module, 02 sensor (this type is not available in the general version) |
| 1 | U8 | NameLen | -/- | -/- | Parameter name 1 length |
| n | N*u8 | Name | -/- | -/- | Parameter name |
| 1 | u8 | Type | -/- | -/- | Parameter type 2, Type definition: 00—MCU (firmware), 01—module, 02 sensor (this type is not available in the general version) |
| 1 | U8 | NameLen | -/- | -/- | Parameter type 2 length |
| n | N*u8 | Name | -/- | -/- | Parameter name |
| ... | ... | ... | ... | ... | ... |

Note: This message is reported once by default at startup, uploading the device firmware version number and module version number, which can be left unparsed. The Message ID of previously shipped devices was 0xBB, and relevant personnel can be contacted to send it.

Example:

BDBDBDBDA902000009423732352E4F56303601174E5432364B434E4230304E4E412D4C3032303330393530B5

BDBDBDBD:Header

A9: Message ID

02: TypeCnt indicates that there are 2 types of parameters

00: Reserved field

00: Type=00 00 represents MCU (firmware)

09: The following MCU name length is 9 bytes

423732352E4F563036--->Hexadecimal to text (device firmware version number)--->B725.OV06

01: Type=01 01 represents the module

17: The following communication module name length is 23 bytes

4E5432364B434E4230304E4E412D4C3032303330393530: Hexadecimal to text (4G module version number)

--->NT26KCNB00NNA-L02030950

B5---checksum checksum

4.  5 Health-related reporting

4.  5.1 Health Data Data (MSGID=0x32)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x32 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | 00 |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 2 | U16 | contentLength |  |  | Total length of the following content |
| 1 | U8 | ID |  |  | Id (Type + Reported Value Length), see below for ID type definition |
| N | N*U8 | Val1 |  |  | The reported value of Id (the byte length varies depending on the length of the reported value) |
| ... | ... | ... | ... | ... | ... |
| 1 | U8 | ID |  |  | Id (Type + Reported Value Length), see below for ID type definition |
| N | N*U8 | Val1 |  |  | The reported value of Id (the byte length varies depending on the length of the reported value) |

ID type definition (hexadecimal):

01 Step counting02 Heart rate 03 Body temperature 04 Wrist temperature 06 Diastolic pressure 07 Systolic pressure 08 Blood oxygen 09 HRV(Special version support)

(05 Blood glucose 10 Sit-up count11 Running pace and distance 12 Rope skipping speed and count )---Reserved, not available yet

Default universal version support:01 Step counting02 Heart rate 03 Body temperature 04 Wrist temperature 06 Diastolic pressure 07 Systolic pressure 08 Blood oxygen

Example:

Original16hexadecimal message:BDBDBDBD3200E377BD6711000A1E00114B314A39711A4A0122bc00416212

BDBDBDBD: Header

32:Message ID

00:type  type=00

E377BD67:Timestamp Convert to big endian 67 BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

1100:The length of the following message is converted to big-endian-->0011-->converted to decimal17, and the length of the following message is17bytes

0A:ID  0x0A-->convert to binary00001010   The first five digits are00001-->convert to16hexadecimal01, which represents the data ID step counting (0x01),

The last three digits are010-->convert to decimal02, which means the length of the step counting data is2bytes

1E00: Step count reported value converted to big-endian -->001E--> converted to decimal30, total step count data30steps

11:ID  0x11--> converted to binary00010001   The first five digits are 00010--> converted to16hexadecimal02, representing data ID heart rate (0x02),

The last three digits are 001--> converted to decimal1, representing the length of the heart rate data as1byte

4B: Heart rate reported value converted to decimal -->75, heart rate data is75

31:ID  0x31--> converted to binary00110001   The first five digits are 00110--> converted to16hexadecimal06, representing data ID diastolic blood pressure (0x06),

The last three digits are 001--> converted to decimal1, representing the length of the diastolic blood pressure data as1byte

4A: Blood pressure diastolic reported value, converted to decimal -->74, diastolic blood pressure data is74

39:ID  0x3--> converted to binary00111001    The first five digits are 00111--> converted to16hexadecimal07, representing data ID systolic blood pressure (0x07),

The last three digits are 001--> converted to decimal1, representing the length of the systolic blood pressure data as1 byte

71: Blood pressure systolic reported value, converted to decimal -->113, systolic blood pressure data is113

1A:ID 0x1A-->Convert to binary00011010 The first five digits are00011-->Convert to16Hexadecimal03, which represents the data ID body temperature (0x03),

The last three digits are 010--> convert to decimal2, representing the length of the body temperature data as2bytes

4A01: Body temperature reporting value, converted to big-endian -->014A--> convert to decimal330-->330*0.1--> body temperature data is33degrees Celsius

22:ID 0x22--> convert to binary00100010 The first five digits are 00100--> convert to16hexadecimal04, representing the data ID wrist temperature (0x04),

The last three digits are 010--> convert to decimal2, representing the length of the wrist temperature data as2bytes

BC00: Wrist temperature reporting value, converted to big-endian -->00BC--> convert to decimal188-->188*0.1--> wrist temperature data is18.8degrees Celsius

41:ID 0x41--> convert to binary01000001 The first five digits are 01000--> convert to16hexadecimal08, code data ID blood oxygen (0x08),

The last three digits are 001--> convert to decimal1, representing the length of the blood oxygen data as1byte

62: Blood pressure reporting value, converted to decimal -->98, blood oxygen data is98

12: checksum checksum

4.5.2 Device Sleep Analysis Data Upload (MSGID=0xC5)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xC5 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 4 | Int32 | DateTime |  | -/- | Upload UTC start time |
| 4 | Int32 | DateTime |  |  | Upload UTC end time |
| 2 | Int16 | Sleepminute |  |  | Number of minutes of sleep time uploaded |
| 4 | Int32 | Type |  |  | Sleep state: Upload type, 1 is deep sleep, 2 is light sleep, 3 is wake-up duration |

Example:

Original16Hexadecimal message: BDBDBDBDC5AC338860693B8860210001000000D1

BDBDBDBD: Header

C5:Message ID

AC338860: Start time, converted to big-endian608833AC-->Convert to decimal-->Timestamp is1619538860seconds-->

Convert to standard time format UTC time: 2021 - 04 - 27 15 : 54 : 20 --> Convert to Beijing time: 2021 - 04 - 27 23 : 54 : 20

693B8860: End time, converted to big-endian 60883BAC--> converted to decimal --> timestamp is 1619540908 seconds -->

Convert to standard time format UTC time: 2021 - 04 - 27 16 : 28 : 28 --> Convert to Beijing time: 2021 - 04 - 28 00 : 28 : 28

2100: Sleepminute sleep time, converted to big-endian 0021--> converted to decimal 33--> The total sleep time is 33 minutes

01000000: Type sleep state, converted to big-endian 00000001--> converted to decimal 1--> 1 is deep sleep, deep sleep time is 33 minutes

D1: checksum checksum

4.6 Downlink feedback related reporting

4.6.1 Downlink feedback (MSGID=0xC0)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xC0 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID Length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This command is used for feedback of downlink commands, returning the Message ID of the downlink command received earlier (multiple Message IDs can be returned collectively).

Original hexadecimal message::BDBDBDBDC0011720

BDBDBDBD:Header

C0:Message ID

01:length Message ID length Convert decimal 01--> Message ID length is 1 byte

17Type indicates that the device received the downlink command Message ID is 17

20:checksum checksum

4.6.2 Message Status Report (MSGID=0x28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x28 | See below See definition below | CK_sum |

Payload contents

| Bytes | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | timestamp | -/- | -/- | Timestamp |
| 1 | U8 | Type | -/- | -/- | Character encoding type (corresponding to downlink 0X28), GB2312 encoding: 0x03; Unicode encoding: 0x05 |
| 1 | U8 | status | -/- | -/- | Message status - 0x01: Confirmed, 0x02: Rejected |
| 4 | U32 | seqID | -/- | -/- | (Corresponding message ID seqID to downlink 0X28) |

Example:

Note:Currently, the general version does not distinguish character encoding types and is fixed at 03. Subsequent versions will distinguish.

Original hexadecimal message:BDBDBDBD28FD3BA068030268A03BE477

BDBDBDBD:Header

28:Message ID

FD3BA068:timestamp Timestamp converted to big-endian 68A03BFD-->converted to decimal-->timestamp is 1755331581 seconds-->

Convert to standard time format UTC time: 2025-08-16 08:06:21-->Convert to Beijing time: 2025-08-16 16:06:21

03:Type 03--Fixed value

02:status Message status 02--Interface click reject button--Rejected

68A03BE4:seqID The id of the downlink message is 68A03BE4

77:checksum Checksum

5 Settings

5.1 Downlink

5.1.1 Set the location reporting frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x17 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - hour |  |
| 1 | u8 | time_start_m |  |  | - minute |  |
| 1 | u8 | time_end_h |  |  | - hour |  |
| 1 | u8 | time_end_m |  |  | - minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - hour |  |
| 1 | u8 | time_start_m |  |  | - minute |  |
| 1 | u8 | time_end_h |  |  | - hour |  |
| 1 | u8 | time_end_m |  |  | - minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - hour |  |
| 1 | u8 | time_start_m |  |  | - minute |  |
| 1 | u8 | time_end_h |  |  | - hour |  |
| 1 | u8 | time_end_m |  |  | - minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - hour |  |
| 1 | u8 | time_start_m |  |  | - minute |  |
| 1 | u8 | time_end_h |  |  | - hour |  |
| 1 | u8 | time_end_m |  |  | - minute |  |

Note: One day is from 00:00-23:59
Example:

00:00-23:59 Report location every10minutes:BDBDBDBD17010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

17:Message ID

01:enable is enabled Convert to decimal01-->1--Enabled

0A00:Interval time interval Convert to big endian000A-->Convert to decimal10-->Time interval is10minutes

00:time_start_h Start time-hour, convert to decimal00-->Start time-hour is00

00:time_start_m Start time-minute, convert to decimal00-->Start time-minute is00

17:time_end_h End time-hour, convert to decimal23-->End time-hour is 23

3B:time_end_m End Time - Minutes, convert to decimal59--> End Time - Minutes is 59

00:enable Whether to enable Convert to decimal00-->0-- Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0-- Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0-- Not enabled

000000000000:All filled in the disabled state00

97:checksum Checksum

5.1.2 Message Send (MSGID=0X28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x28 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | U8 | type | Message type, if it is downlink text information, GB2312 encoding is 03 |
| 4 | Uint32 | seqID | The ID of the information, unique |
| 1 | U8 | CONTENT LEN | Content Length |
| N | N | CONTENT | Content, Chinese is GB2312 encoded (one Chinese character occupies 2 bytes), English is ASCII encoded (one English letter occupies 1 byte) |

Note: The default Chinese character encoding for the domestic general version is GB2312
The information ID cannot be repeated for each transaction, and the device cannot receive repeated IDs
Example:

Text message: hello,world  BDBDBDBD2803030000000B68656C6C6F2C776F726C64DD

BDBDBDBD:Header

28: Message ID

03: type message type Convert to decimal 03-->type=03

03000000: seqID information id Information id is 03000000

0B: CONTENT LEN content length Convert to decimal 11, content length is 11 bytes

68656C6C6F2C776F726C64: Convert to string hello,world-->Text message content:hello,world

DD:checksum checksum

Text message: Hello World  BDBDBDBD28036895D13F08C4E3BAC3CAC0BDE779

BDBDBDBD:Header

28:Message ID

03:type message type Convert to decimal 03-->type=03

6895D13F: seqID information id Information id is 6895D13F

08:CONTENT LEN content length Convert to decimal 8

C4E3BAC3CAC0BDE7:Converted to string Hello World-->Text message content: Hello World

79:checksum

5.1.3 Settings - Location Priority Settings (0XCE01)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See definition below | CK_sum |

Payload contents

Type=01 Positioning Priority Setting

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 valid for this time |
| 2 | U16 | Len | -/- | -/- | Length of the following instruction |
| n | n |  | -/- | -/- | Body |

Positioning priority setting definition--See2.3 Device downlink description--Device positioning priority distribution(0xCE01)

Positioning priority setting body: Currently, the general version supports:01--Satellite (GPS/BDS),02--wifi(wifi+base station) 03--Bluetooth beacon (requires additional deployment of Bluetooth beacons);

Special equipment support:04--LBSbase station 05--125k

Example:

WiFi positioning priority (WiFi > Bluetooth beacon > Satellite (GPS/BDS)):BDBDBDBDCE0100030002030133

BDBDBDBD:Header

CE: Message ID

01: Type 01 Location Priority Setting

00: Valid Validity 00 Always Valid

0300: Len Converted to Big Endian 0003-->Converted to Decimal 03-->The text length is 3 bytes

020301: Text: 02--wifi 03--Bluetooth beacon 01--Satellite (GPS/BDS) The positioning priority is wifi>Bluetooth beacon>Satellite (GPS/BDS)

33: checksum Checksum

GPS positioning priority (Satellite (GPS/BDS)>wifi>Bluetooth beacon): BDBDBDBDCE0100030001020333

BDBDBDBD:Header

CE: Message ID

01: Type 01 Location Priority Setting

00: Valid Validity 00 Always Valid

0300: Len Converted to Big Endian 0003-->Converted to Decimal 03-->The text length is 3 bytes

010203: Text: 01--Satellite (GPS/BDS) 02--wifi 03--Bluetooth beacon The positioning priority is Satellite (GPS/BDS)>wifi>Bluetooth beacon

33: checksum Checksum

Bluetooth positioning priority (Bluetooth beacon>wifi>Satellite (GPS/BDS)): BDBDBDBDCE0100030003020133

BDBDBDBD:Header

CE: Message ID

01: Type 01 Location Priority Setting

00: Valid Validity 00 Always Valid

0300: Len Converted to Big Endian 0003-->Converted to Decimal 03-->The text length is 3 bytes

030201: Text: 03--Bluetooth beacon 02--wifi 01--Satellite (GPS/BDS) The positioning priority is Bluetooth beacon>wifi>Satellite (GPS/BDS)

33: checksum Checksum

Single Satellite (GPS/BDS) positioning:BDBDBDBDCE0100030001000033

BDBDBDBD:Header

CE: Message ID

01: Type 01 Location Priority Setting

00: Valid Validity 00 Always Valid

0300: Len Converted to Big Endian 0003-->Converted to Decimal 03-->The text length is 3 bytes

030201:Text: 01--Satellite (GPS/BDS) 00--No positioning 00--No positioning Positioning priority is single Satellite (GPS/BDS) positioning

33:checksum checksum

Single wifi positioning:BDBDBDBDCE0100030002000033

BDBDBDBD:Header

CE: Message ID

01: Type 01 Location Priority Setting

00: Valid Validity 00 Always Valid

0300: Len Converted to Big Endian 0003-->Converted to Decimal 03-->The text length is 3 bytes

020000:Text: 02--wifi 00--No positioning 00--No positioning Positioning priority is single wifi positioning

33:checksum checksum

Single Bluetooth beacon positioning:BDBDBDBDCE0100030003000033

BDBDBDBD:Header

CE: Message ID

01: Type 01 Location Priority Setting

00: Valid Validity 00 Always Valid

0300: Len Converted to Big Endian 0003-->Converted to Decimal 03-->The text length is 3 bytes

020000:Text: 03--Bluetooth beacon 00--No positioning 00--No positioning Positioning priority is single Bluetooth beacon positioning

33: checksum Checksum

5.1.4 Settings - Health Sample Reporting Frequency Settings (0XCE02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See definition below | CK_sum |

Payload contents

Type=02 Health sample reporting frequency setting

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=02 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 valid for this time |
| 2 | U16 | Len | -/- | -/- | Length of the following instruction |
| 1 | U8 | Healthtype | -/- | -/- | 00—Indicates all |
| 1 | U8 |  | -/- | -/- | Body |
| 1 | u8 | Time Unit | -/- | -/- | Time unit 00—minutes, 01—hours |

Health sampling reporting frequency setting definition--See 2.3 Device downlink description--Device health sampling frequency distribution (0xCE02)

Health sampling reporting frequency setting text: The current general version supports:00-All, reserved item--Not yet implemented: 01 Step counting 02 Heart rate 03 Temperature 04 Sleep 05 Blood pressure 06 Blood sugar 07 Blood oxygen

Example:

Health sampling reported once every 5 minutes:BDBDBDBDCE0200030000050033

BDBDBDBD:Header

CE:Message ID

02:Type 02 Health sampling reporting frequency setting

00:Valid Validity 00 Always valid

0300:Convert to big-endian 0003-->Convert to decimal 03-->The length of the text is 3 bytes

00:Healthtype 00---All

05:Text: Convert to decimal 05-->Health sampling report frequency is once every 5 minutes

00:Time Unit 00--Minutes

33:checksum checksum