W200PL-LORA Private Communication Protocol

Directory

1 Overview	3

2 Equipment Usage Instructions	3

2.1 Equipment Functions and Usage Instructions	3

2.2 Device default reporting logic	4

2.3 Device downlink specification	5

3 Protocol Data Packet Structure	7

3.1 Data Header	7

3.2 Message Identifier (Message ID)	7

3.3 Payload	7

3.5 Checksum	8

4 Reporting messages	10

Alarm-related reporting	10

4.1.1 Alarm Data Upload - 1 (MSGID=0x02)	10

4.1.2 Alarm Data Upload-2 (MSGID=0x21) (Supplement to 02) — No general version available	13

4.1.3 SOS upload (MSGID=0XB5)	17

4.2 Location-related reporting	18

4.2.1 GPS/BDS Position Reporting: Position Data Reporting (MSGID=0x03)	18

4.2.2 Bluetooth Location Information (LBE Location) (MsgId=0xD6)	20

4.3 Device information and status reporting	22

4.3.1 Device Charging Status Upload (MSGID=0xC3)	22

4.3.2 Battery Signal (MSGID=0xF9)	24

4.4 Health-Related Reporting	26

4.4.1 Health Data (MSGID=0x32)	26

5 Settings	28

5.1 Downlink	28

5.1.1 Set Location Reporting Frequency (MSGID=0x17) — Broadcast	28

5.1.2 Message Send (MSGID=0X28) — Broadcast	30

5.1.3 Settings - Location Priority Settings (0XCE01) - Broadcast	31

5.1.4 Settings - Health Sampling Reporting Frequency Settings (0XCE02) — Broadcast	33

5.1.5 Settings - Switch Settings (0XCE04-24) — Broadcast	34

5.1.6 Settings - Drop Sensitivity Settings (0xCE15) — Broadcast	37

5.1.7 Issue Sitting Duration Alarm Trigger Time (MSGID=0XCC)—Broadcast	39

5.1.8Settings - Device Alarm Settings (0XCE03)--Broadcast--Special Version Support	40

5.2 Server time synchronization information	42

5.2.1Time Calibration - Broadcast	42

1 Overview

This agreement is suitable for Lora private protocol devices, currently supporting the W200PL-Lora private protocol product. It uses a 32-bit data header for synchronization and terminal identification; employs a low-overhead checksum algorithm for verification protection; and uses message identifiers to distinguish different messages.
Parsing needs to be based on the actual report, and the document is continuously being maintained. If any errors or omissions are found, please give timely feedback, thank you very much.

2 Equipment Usage Instructions

2.1 Equipment Functions and Usage Instructions

The general version: The default general is CN470 (475.5-476.9), which needs to be used in conjunction with a private protocol gateway. If other frequency bands are required, please make a request.

The gateway configuration parameters - the default gateway must be changed to the following parameters to report downstream correctly:

(1) Power On

Before using for the first time, please fully charge the device. Once charged to the required power for startup, the device will automatically turn on, and the charging status will display a charging icon.

The fully charged icon is green. Note: Do not check the device signal while charging.    Manual power on: Long press the upper button for 10 seconds and then release, the interface will display the word "Welcome".    Note: The default wearing status when powering on, if no heart rate is detected, it will report a fall alarm.

(2) Shutdown

Low battery shutdown: The screen turns off after the interface displays Byebye. Manual shutdown: Shutdown: Press and hold the button for more than 10 seconds, the interface displays Byebye and then the screen turns off. Note: Charging in the power-on state does not shut down.

(3) SOS function

Trigger mode: After triggering, the device does not enter sleep mode. Long press the down key for 3s, the interface displays SOS SEND OK/SOS sending successfully, and then the SOS icon appears. Cancel mode: In SOS mode, long press the down key for 3s, the interface displays SEND CENCEL/SOS canceled, and the SOS icon disappears.

(4) Signal status

No signal: The default interface signal is a horizontal line. Signal: The interface signal has a stepped bar graph. The signal is obtained after receiving the downlink command. The device will display the signal after it is obtained.

(5) Device sleep

Trigger condition: The device does not move for 40 minutes and enters sleep mode, and does not report location health data.

2.2 Device default reporting logic

General version: Reporting needs to be within the lora gateway range

(1) Location-related reporting

satellite（GPS/BDS）/ Bluetooth beacon: Default reporting frequency 10 minutes, default positioning priority: Bluetooth beacon > satellite（GPS/BDS）, Bluetooth positioning priority, switch to satellite（GPS/BDS）positioning if positioning fails

(2) Alarm-related reporting

SOS alarm (0xB5): User actively triggers, see the previous section for the triggering method

SOS cancellation (0x02): User actively triggers cancellation, see the previous section for the triggering method

Shutdown alarm (0x02): Device actively shuts down or shuts down due to low battery, see the previous section for the triggering method

Wearing and falling off alarm (0x02): The device determines wearing according to the health sampling reporting frequency. If the heart rate is measured, the wearing alarm is reported. If the heart rate is not measured, the falling off alarm is reported.

Sedentary stay alarm (0x02): The default is 15 minutes of immobility to trigger reporting

Fall alarm (0x02): The device falls freely from a certain height and meets the fall algorithm trigger

Low battery alarm (0x02): Triggered when the current battery level of the device is less than or equal to 0

(3) Health-related reporting

Step count, heart rate, body temperature & wrist temperature, blood pressure, blood oxygen (0 x32): default reporting frequency 10 minutes

(4) Device information and status reporting

Charging status (0 xC3): report when charging starts, ends, and when fully charged

Battery signal (0 xF9): report once when powered on, and also report during positioning and health checks

(5) Downlink feedback

Downlink feedback (0xC0): report after the server's downlink command is received by the device

2.3 Device downlink specification

General version: broadcast downlink command

(1) Device positioning reporting frequency assignment (0x17)

Default reporting frequency is 10 minutes, maximum is 1 minute, minimum is 1440 minutes. After receiving the downlink command, the device reports according to the time period and frequency of the assigned command

Report data, outside the time period use the default reporting frequency, for example: 00:00-18:00  2-minute positioning report,

Then report with the default frequency of 10 minutes outside the time period.

(2) Text message delivery (0x28)

GB2312 encoding, up to 20 Chinese characters, with each Chinese character occupying 2 bytes and each English letter occupying 1 byte.

Message list - a maximum of 50 historical messages are saved (Note: Firmware without a message list saves a maximum of 20 messages).

(3) Device location priority delivery (0xCE01)

Default location priority: Bluetooth > satellite（GPS/BDS）.

The location priority is delivered as follows: satellite（GPS/BDS）> Bluetooth beacon, satellite（GPS/BDS） location is prioritized, switch to Bluetooth beacon when satellite（GPS/BDS） location is unavailable; when the location is successful, it will not switch to the next priority to generate location.

(4) Device health sampling frequency delivery (0xCE02)

Device health sampling frequency delivery (0xCE02)    The default health sampling frequency is 10 minutes, with a maximum report time of 2 minutes.

(5) Sedentary stay alarm trigger time delivery (0xCC)

Value range: 2 minutes - 60 minutes. Note that descending can only be effective when the sedentary alert switch is turned on.

For example: if the descending trigger time is 10 minutes, the device will report a sedentary alert if it remains inactive for 10 minutes.

(6) Drop sensitivity and height settings (0xCE15)

The default drop sensitivity of the latest general firmware: low (mid-low). Drop height: 1.5m.

Sensitivity: refers to the extent to which the drop algorithm is satisfied, providing 5 setting levels (0 - 4): Low - Low (mid-low) - Medium - High (mid-high) - High.

Height: refers to the height required to trigger a drop alert, providing 5 setting levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m - 2.5m.

(7) Sedentary alert switch (0xCE08)

Default is off, and the device does not report sedentary alerts. When turned on, the default trigger time is 15 minutes, triggering a sedentary alert after 15 minutes of inactivity.

(8) Control device trigger sleep switch settings (0xCE18)

Default is on; the device enters sleep mode if inactive for 40 minutes. If the downlink is turned off, the device will not enter sleep mode.

(9) Fall Alarm Switch (0xCE07)

The fall alarm is closed by default. After it is enabled, the device reports a fall alarm when a fall is triggered.

(10) GPS Always On Switch (0xCE24)

When enabled, the internal interface will always collect GPS. In general environments, this can speed up GPS positioning time, but the power consumption will increase. The default is off.

(11) Calibration Time Command Downlink (0XFF)

This downlink command is used for watch time calibration.

3 Protocol Data Packet Structure

A basic protocol data packet structure:

Header:0xBD

DEVID: Device ID (unique device identifier)

Message ID: Message ID

Payload: message body, defined differently based on the message ID. Checksum: check sum.

3.1 Data Header

Each data packet starts with 1 byte of Header:

Currently, the Header for the Eufo device upload is fixed to BD.

Header: 0xBD;

3.2 Message Identifier (Message ID)

The content represented by MessageId is detailed in Chapter 4.

3.3 Payload

In the following, the payload refers to the effective body content in the protocol, excluding the header and checksum. The subsequent annotation indicates the length of the body.

The data format used in the payload is shown in the table below:

[U-unsigned; I-signed; X-bitfield; Number - Number of bytes occupied]

All other formats in the following protocol use little-endian format except for ch, u8, i8, and x8.

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Description |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short |
| i8 | Signed Char | 1 | -128-127 | 1 | Short |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Integer |
| x16 | Bitfield | 2 | - | - | Bit (bit) 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit long integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | floating point |

3.5 Checksum

The content included in the checksum includes the payload, as shown in Figure 1. The algorithm is as follows,

The default general version device does not require checksum verification; this part can be ignored, and any byte can be sent for the downlink command.

// The following is the C# code calling method

private string CheckSum(string content)

{

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2");

}

4 Reporting messages

Alarm-related reporting

4.1.1 Alarm Data Upload - 1 (MSGID=0x02)

| Header | DEVID | Message ID | Payload | Checksum |
| --- | --- | --- | --- | --- |
| Token: fixed to 0xBD | DEVID | 0x02 | See below for definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below 见下方定义 |
| 4 | U32 | Timestamp | - | - | Timestamp (a timestamp will be added later when retransmitting) |

Upl_warn contents

| bit (corresponding to the position of 1 in binary) | Name | Description | Original message in little-endian hexadecimal | Converted to big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 14 | Drop alarm | Drop alarm | 0040 | 4000 | 4*4096=16384 |
| 8 | Device wearing | Device wearing | 0001 | 0100 | 1*256=256 |
| 7 | SOS Cancel | SOS Cancel | 8000 | 0080 | 8*16=128 |
| 5 | Sitting Alert (Inactive) | Sitting Alert (Inactive) | 2000 | 0020 | 2*16=32 |
| 4 | Remove (Fallen Off) Device | Remove (Fallen Off) Device | 1000 | 0010 | 1*16=16 |
| 2 | Power Off | Power Off | 0400 | 0004 | 4 |
| 0 | Low Battery | Low Battery | 0100 | 0001 | 1 |

Example:

Wear alarm: BDCACBB80330003795020001E377BD678A

BD:Header

CACBB80330003795: DEVID  Device ID

02:Message ID

0001:Upl_warn alarm content, converted to big-endian: 0100-->Wear alarm

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

8a：checksum

Fall alarm: BDCACBB80330003795021000E377BD678A

BD:Header

CACBB80330003795: DEVID  Device ID

02:Message ID

1000:Upl_warn alarm content, converted to big-endian 0010-->Fall alarm

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

8a：checksum

SOS cancellation: BDCACBB80330003795028000E377BD678A

BD:Header

CACBB80330003795: DEVID  Device ID

02:Message ID

8000: Upl_warn alarm content, converted to big-endian 0080 --> SOS canceled

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

8a：checksum

Drop alarm: BDCACBB80330003795020040E377BD6756

BD:Header

CACBB80330003795: DEVID  Device ID

02:Message ID

0040: Upl_warn alarm content, converted to big-endian 4000 --> Drop alarm

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

E7: checksum

Sitting alert: BDCACBB80330003795022000E377BD678A

BD:Header

CACBB80330003795: DEVID  Device ID

02:Message ID

2000: Upl_warn alarm content, converted to big-endian 0020 --> Sitting alert

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

8A: checksum

Low battery alarm: BDCACBB80330003795020200E377BD678A

BD:Header

CACBB80330003795: DEVID  Device ID

02:Message ID

0100: Upl_warn alarm content, converted to big-endian 0001 --> Low battery alarm

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

8A: checksum

Shutdown alarm: BDCACBB80330003795020400E377BD678A

BD:Header

CACBB80330003795: DEVID  Device ID

02:Message ID

0400:Upl_warn alarm content, converted to big-end 0004 --> shutdown alarm

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

8A: checksum

Note: This alarm is for devices that do not distinguish between shutdown types. For devices that do distinguish shutdown types, see the next section.

4.1.2 Alarm Data Upload-2 (MSGID=0x21) (Supplement to 02) — No general version available

| Header | DEVID | Message ID | Payload | Checksum |
| --- | --- | --- | --- | --- |
| Token: fixed to 0xBD | DEVID | 0x21 | See below for definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | Alarm type |
| N |  | Upl_warn | - | - | Alarm content See below for different Alarm type definitions |
| 4 | U32 | Timestamp |  |  | Timestamp (a timestamp will be added later when retransmitting) |

Alarm type=1

Upl_warn contents

| bit (corresponding to the position of 1 in binary) | Name | Description | Original message in little-endian hexadecimal | Converted to big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging shutdown alarm | Device automatically shuts down while charging (firmware must support this function) | 04000000 | 0004 | 4 |
| 1 | Low Power Shutdown Alarm | Device shuts down due to low battery (firmware support required) | 02000000 | 0002 | 2 |
| 0 | Active Shutdown Alarm | Device is manually shut down (firmware support required) | 01000000 | 0001 | 1 |

Example:

Charging Shutdown Alarm: BDCACBB8033000379521010004000000E377BD6767

BD:Header

CACBB80330003795: DEVID  Device ID

21:Message ID

0100:Alarm type, converted to big-endian 0001 --> Alarm type = 1

04000000:Upl_warn alarm content, converted to big-endian 0004 --> Charging shutdown alarm

E377BD67:Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

checksum

Low Voltage Shutdown Alarm: BDCACBB8033000379521010002000000E377BD6767

BD:Header

CACBB80330003795: DEVID  Device ID

21:Message ID

0100:Alarm type, converted to big-endian 0001 --> Alarm type = 1

02000000: Upl_warn alarm content, converted to big-endian 0002 --> Low Voltage Shutdown Alarm

E377BD67:Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

checksum

Active Shutdown Alarm: BDCACBB8033000379521010001000000E377BD6767

BD:Header

CACBB80330003795: DEVID  Device ID

21:Message ID

0100:Alarm type, converted to big-endian 0001 --> Alarm type = 1

01000000: Upl_warn alarm content, converted to big-endian 0001 --> Active Shutdown Alarm

E377BD67:Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

checksum

Alarm type=5

Upl_warn contents

| Format | Name | Unit | Description |
| --- | --- | --- | --- |
| U16 | Type | 2 | Fixed as 0x05 |
| U32 | Timestamp | 4 | Timestamp |
| U16 | Content Len | 2 | Alarm Content Length |
| U8 | Health Warn Type1 | 1 | Health Threshold Alarm Type: 0x01 Heart Rate, 0x02 Systolic Blood Pressure (SBP), 0x03 Diastolic Blood Pressure (DBP), 0x04 Blood Oxygen, 0x05 Temperature |
| U8 | Type1 | 1 | 01 Less than the set health threshold, 02 Greater than the set health threshold |
| I16 | Health Value1 | 2 | Health Result Value |
| U8 | Health Warn Type2 | 1 | Health Threshold Alarm Type: 0x01 Heart Rate, 0x02 Systolic Blood Pressure (SBP), 0x03 Diastolic Blood Pressure (DBP), 0x04 Blood Oxygen, 0x05 Temperature |
| U8 | Type2 | 1 | 01 Less than the set health threshold, 02 Greater than the set health threshold |
| I16 | Health Value2 | 2 | Health Result Value |
| 。。。。 | 。。。。 | 。。。。 | 。。。。 |

Example:

The device detected that the heart rate (65) is outside the issued health threshold: BDCACBB80330003795210500E377BD6704000102410073

BD: Header

CACBB80330003795:    DEVID Device ID

21: Message ID

0500: Alarm type Convert to big-end 0005 --> Alarm type=5

E377BD67: Timestamp Convert to big-end 67 BD77E3 --> Convert to decimal 1740470243 --> Timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

0400:Content Len alarm content length to big-end0004--> The alarm content length is 4 bytes. Note: If there are multiple health threshold alarms, the length will increase.

01:Health Warn Type health threshold alarm type 01--> Heart rate alarm

02:Type greater than the set health threshold & less than the set health threshold  01--> Less than the set health threshold

4100:Health Value actual reported health value to big-end0041--> Converted to decimal 65 --> The reported heart rate value is 65

73:checksum

4.1.3 SOS upload (MSGID=0XB5)

| Header | DEVID | Message ID | Payload | Checksum |
| --- | --- | --- | --- | --- |
| Token: fixed to 0xBD | DEVID | 0xB5 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Description |
| --- | --- | --- | --- |
| 1 | U8 | Status | Status: 1: SOS |
| 4 | Int32 | timestamp | Timestamp, little endian |

Example:

Hexadecimal raw message:BDCACBB80330003795B501E377BD6755

BD:Header

CACBB80330003795: DEVID Device Number

B5:Message ID

01:Status 01--SOS

E377BD67:Timestamp Convert to big endian 67BD77E3-->Convert to decimal 1740470243-->The timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

55:checksum

4.2 Location-related reporting

4.2.1 GPS/BDS Position Reporting: Position Data Reporting (MSGID=0x03)

| Header | DEVID | Message ID | Payload | Checksum |
| --- | --- | --- | --- | --- |
| Token: fixed to 0xBD | DEVID | 0x03 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude |
| 8 | Double | lat |  | - | latitude |
| 1 | U8 | north_south |  | - | /N or S/ |
| 1 | U8 | east_west |  | - | /E or W/Eastern or Western Longitude |
| 1 | U8 | status |  | - | /A or V/ Valid data or Invalid data |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original16Hexadecimal message: BDCACBB8033000379503963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BD:Header

CACBB80330003795: DEVID Device ID

03:Message ID Message ID

963E870C3E675E40:lon Longitude Convert to Big Endian405E673E0C873E96-->Longitude:121.6131622  Note: Conversion parsing reference below for parsing example

FF2110C2B6343F40:lat Latitude Convert to Big Endian403F34B6C21021FF-->Latitude:31.2059137  Note: Conversion parsing reference below for parsing example

4E:north_south  4E-->Convert to ASCII code N-->North Latitude

45:east_west 45--> Convert to ASCII code E --> East longitude

41:status Data validity  41--> Convert to ASCII code A --> Valid data

E377BD67: Timestamp timestamp Converted to big-endian67BD77E3 --> Convert to decimal10Decimal 1740470243--> Timestamp is1740470243seconds -->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

78:checksum

Note: The parsed satellite（GPS/BDS） latitude and longitude are in WGS-84 coordinate system. If using maps such as Baidu or Amap, the coordinate system needs to be converted.

Parsing satellite positioning message Java code example:

// Message: BD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b // BD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public static void main(String[] args){

// Message 7d9f84ac81815c40 Actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d",16)));  //114.02353966666665

//Message e766926b1d893640 Actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16))) ;//22.535605166666667

HexToStr(data.Substring(“4e”));   //N

HexToStr(data.Substring(“45”));//E

HexToStr(data.Substring(“41”));//A   A indicates valid data, V indicates invalid data, which can be discarded //Message 749d695f actual value 5f699d74

Date date=new Date();

date.setTime(Long.parseLong(“5f699d74",16)*1000);

SimpleDateFormat sdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.2.2 Bluetooth Location Information (LBE Location) (MsgId=0xD6)

| Header | DEVID | Message ID | Payload | Checksum |
| --- | --- | --- | --- | --- |
| Token: fixed to 0xBD | DEVID | 0xD6 | See below for definition | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 0 |
| U8 | Total_groups | 1 | Total number of groups, which may contain multiple sets of information, each set may have multiple ibeacons |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identifier codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identifier codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| ... | ... | ... | ... |

Note: Under the general version, the total number of Bluetooth beacons is fixed at 1, with a maximum of 4 beacons. They are sorted by signal strength from highest to lowest. By default, the watch only recognizes our own beacons. If you need to connect to beacons from other manufacturers, please consult the relevant personnel via WeChat. Example:

Original Hexadecimal Message: BDCACBB80330003795D60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

BD: Header

CACBB80330003795: DEVID Device Number

D6: Message ID

00: Type

01: Only one set of ibeacon data

E377BD67:Timestamp Convert to big endian 67BD77E3-->Convert to decimal 1740470243-->The timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

04: First set has 4 beacon information

4327: Convert to big-endian --> major: 0x2743 --> Convert to decimal 10051

1794: Convert to big-endian --> minor: 0x9417 --> Convert to decimal 37911

AC:  RSSI 0xAC --> Convert to decimal complement -84

4327: Convert to big-endian --> major: 0x2743 --> Convert to decimal 10051

3094: Convert to big endian --> minor:0x9430 --> Convert to decimal 37936

AA:  RSSI 0xAA --> Convert to decimal two's complement -86

4327:Convert to big-endian --> major: 0x2743 --> Convert to decimal 10051

b956:Convert to big-endian --> minor: 0x56b9 --> Convert to decimal 22201

A5:   RSSI 0xA5 --> Convert to decimal two's complement -91

4327: Convert to big-endian --> major: 0x2743 --> Convert to decimal 10051

FE94: Convert to big-endian --> minor: 0x94FE --> Convert to decimal 38142

A5: rssi: 0xA5 --> Convert to decimal two's complement -91

6a -- Checksum

Bluetooth beacon major, minor description: By default, Bluetooth beacons are shipped with a label code,

for example: 00081005345866, then the major of the Bluetooth beacon is 10053, the minor is 45866, and the manufacturer identifier is 0008 (this is of no use and has no practical significance)

4.3 Device information and status reporting

4.3.1 Device Charging Status Upload (MSGID=0xC3)

| Header | DEVID | Message ID | Payload | Checksum |
| --- | --- | --- | --- | --- |
| Token: fixed to 0xBD | DEVID | 0xC3 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Status | / | / | 0: Start, 1: End, 2: Fully Charged |
| 1 | U32 | Timestamp | / | / | Timestamp (a timestamp will be added later when retransmitting) |

Note: This message requires support from the device firmware, defaulting to the latest general version support.  
Sample:

Original Hexadecimal Message: BDCACBB80330003795C301E377BD678A

BD:Header

CACBB80330003795: DEVID Device Number

C3: Message ID

01: Device End Charging

E377BD67:Timestamp Convert to big endian 67BD77E3-->Convert to decimal 1740470243-->The timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

8A: Checksum

Original Hexadecimal Message: BDCACBB80330003795C300DB4D2F668A

BD:Header

CACBB80330003795: DEVID Device Number

C3: Message ID

00: Device Start Charging

E377BD67:Timestamp Convert to big endian 67BD77E3-->Convert to decimal 1740470243-->The timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

8A: Checksum

Original hexadecimal message:BDCACBB80330003795C302DB4D2F668A

BD:Header

CACBB80330003795: DEVID Device Number

C3: Message ID

02:The device is fully charged

E377BD67:Timestamp Convert to big endian 67BD77E3-->Convert to decimal 1740470243-->The timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

8A: Checksum

4.3.2 Battery Signal (MSGID=0xF9)

| Header | DEVID | Message ID | Payload | Checksum |
| --- | --- | --- | --- | --- |
| Token: fixed to 0xBD | DEVID | 0xF9 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery Type 0: 4-level system, 1: 5-level system, 2: Percentage, 3: Voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery Value If Bat_type is 0: The battery value range is 0-3 (0 is 25%, 3 is 100%) If Bat_type is 1: The battery value range is 0-4 (0 is 20%, 4 is 100%) If Bat_type is 2: The battery value range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal Type 0: Percentage, 1: 5-level system 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal strength |
| 1 | U8 | Other_type |  |  | Extended type 0: Full step 1: Incremental step 2: Vibration |
| 4 | U32 | Num |  |  | Extended value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Example:

Original 16 hexadecimal message: BDCACBB80330003795F90104000050000095000000E377BD67AA

BD: Header

CACBB80330003795: DEVID Device ID

F9: Message ID (Message ID) 01: Bat_type Battery type 01 --> 1 --> 5 level battery (0 - 4)

0400:Bat_volt battery level converted to big-endian0004-->4-->Level4corresponds to100% battery

00:Signal_type Signal Type 00-->0--> Percentage

5000:Signal_strength Signal Strength Converted to Big Endian0050--> Convert to Decimal80-->80% Signal Strength

00:Other_type Extended Type 00-->0--> Full Step Count

95000000:Num Extended Value - Step Count Converted to Big Endian00000095--> Convert to Decimal149--> Step Count is149

E377BD67:Timestamp Timestamp Converted to Big Endian67BD77E3--> Convert to Decimal1740470243--> Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

AA: Checksum

4.4 Health-Related Reporting

4.4.1 Health Data (MSGID=0x32)

| Header | DEVID | Message ID | Payload | Checksum |
| --- | --- | --- | --- | --- |
| Token: fixed to 0xBD | DEVID | 0x32 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | bp_high | - | - | Systolic Blood Pressure |
| 1 | U8 | bp_low | - | - | Diastolic Blood Pressure |
| 1 | U8 | Hrs |  |  | Heart Rate |
| 1 | U8 | Spo2 |  |  | Blood Oxygen |
| 2 | U16 | wrist Temp | - | - | Wrist Temperature: 2byte |
| 2 | U16 | Body Temp. |  |  | Body temperature: 2byte |
| 4 | U32 | Step |  |  | Step count: 4byte |
| 1 | U8 | Bat |  |  | Battery level |
| 1 | U8 | sign |  |  | Signal |
| 4 | Int32 | timestamp |  |  | Timestamp, little-endian |

ID type definition (hexadecimal):

01Step count02Heart rate03Body temperature04Wrist temperature06Diastolic pressure07Systolic pressure08Blood oxygen09HRV (special version support)

(05Blood glucose10Number of sit-ups11Running pace and distance12Rope jump speed and count )---Reserved, none

Default general version supports:01 Step Count02 Heart Rate 03 Body Temperature 04 Wrist Temperature  06 Diastolic Pressure 07 Systolic Pressure 08 Blood Oxygen

Example:

Raw 16 hexadecimal message:BDCACBB8033000255D32745153624B016D01040400000464049ED36196

CACBB8033000255D:DEVID Device ID

32 : Message ID Message ID (Message ID)

74:  Systolic Pressure 116

51 : Diastolic Pressure 81

53： Heart Rate 83

62： Blood Oxygen 98

4b01 : Little Endian, Wrist Temperature 33.1

6d01： Little Endian, Body Temperature 36.5

04040000: Step count 1028

04: Level of electric power 4 Value 0 - 4 Mean 20% - 100% (20% 40% 60% 80% 100%)

64: Signal strength 100%

049ed361: Timestamp: Beijing time 2022-01-04 09:08:20 (Timestamp: Beijing time 2022-01-04 09:08:20)

96: Check

4.5 Downlink Feedback Related Reporting

4.5.1 Downlink Feedback (MSGID=0xC0)

| Header | DEVID | Message ID | Payload | Checksum |
| --- | --- | --- | --- | --- |
| Token: fixed to 0xBD | DEVID | 0xC0 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This command is used for feedback on downlink commands, returning the Message IDs of previously received downlink commands (multiple Message IDs can be returned collectively). Some downlink commands do not have C0 feedback.

Raw hexadecimal message: BDC0011720

BD:Header

C0: Message ID

01: length The length of Message ID converted to decimal 01 --> Message ID length is 1 byte

17: Type indicates that the device received the downlink command with Message ID 17

20: checksum

5 Settings

5.1 Downlink

5.1.1 Set Location Reporting Frequency (MSGID=0x17) — Broadcast

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0x17 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether enabled, 1 for enabled, 0 for not enabled | Interval 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether enabled, 1 for enabled, 0 for not enabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether enabled, 1 for enabled, 0 for not enabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether enabled, 1 for enabled, 0 for not enabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |

Note: One day is from 00:00 to 23:59
    Example:

00:00-23:59(All day) Report location data every 10 minutes: BD17010A000000173B00000000000000000000000000000000000000000097

BD:Header

1C:Message ID

01:enable is enabled, converting to decimal01-->1--enabled

0A00:Interval time interval converted to big-endian000A-->converted to decimal10-->time interval is10minutes

00:time_start_h Start time - hour, convert to decimal00--> Start time - hour is00

00:time_start_m Start time - minute, convert to decimal00--> Start time - minute is00

17:time_end_h End time - hour, convert to decimal23--> End time - hour is23

3B:time_end_m End time - minute, convert to decimal59--> End time - minute is59

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in disabled state00

97:checksum

5.1.2 Message Send (MSGID=0X28) — Broadcast

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0x28 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Description |
| --- | --- | --- | --- |
| 1 | U8 | type | Message type. If it is a downlink text message, the Unicode encoding is 05, and the GB2312 encoding is 03. |
| 4 | Uint32 | seqID | The ID of the message, uniqueness. |
| 1 | U8 | CONTENT LEN | Content length. |
| N | N | CONTENT | Content, Unicode encoding (little-endian priority,注意转换， one Chinese character occupies 2 bytes, one letter occupies 2 bytes) GB2312 encoding (little-endian priority, one Chinese character occupies 2 bytes, one letter occupies 1 byte) |

Note: The default firmware version uses GB2312 encoding. 
 Message IDs must not be repeated; the device cannot receive duplicate IDs. 
 Example:

GB2312 encoding: Text message: 你好世界 BD28036895D13F08C4E3BAC3CAC0BDE779

BD:Header

Message ID

03:type Message type Convert to decimal 03-->type=03

6895D13F: seqID Information ID Information ID is 6895D13F

08:CONTENT LEN Content length Convert to decimal 8

C4E3BAC3CAC0BDE7:Convert to string Hello World-->Text message content: Hello World

79:checksum

5.1.3 Settings - Location Priority Settings (0XCE01) - Broadcast

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

Type=01 Location Priority Settings

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 effective this time |
| 2 | U16 | Len | -/- | -/- | Length of the following command |
| n | n |  | -/- | -/- | Body |

Positioning priority settings definition -- see 2.3 Device downlink description -- Device positioning priority issuance (0 xCE01)

Positioning priority settings body: Currently supported general versions: 01--GPS, 03--Bluetooth beacon (additional deployment of Bluetooth beacons is required);

Example:

GPS positioning priority (GPS > Bluetooth beacon): BDCE0100030001030033

BD:Header

CE: Message ID

01: Type 01 Positioning priority settings

00: Validity 00 always valid

0300 : Len converts to big-endian 0003 --> converts to decimal 03 --> body length is 3 bytes

010300:Main text: 01--GPS   03--Bluetooth Beacon positioning priority is GPS > Bluetooth Beacon

33:checksum

Bluetooth positioning priority (Bluetooth Beacon > GPS):BDCE0100030003010033

BD:Header

CE: Message ID

01: Type 01 Positioning priority settings

00: Validity 00 always valid

0300 : Len converts to big-endian 0003 --> converts to decimal 03 --> body length is 3 bytes

030100:Main text: 03--Bluetooth Beacon  01--GPS  positioning priority is Bluetooth Beacon > GPS

33:checksum

Single GPS positioning:BDCE0100030001000033

BD:Header

CE: Message ID

01: Type 01 Positioning priority settings

00: Validity 00 always valid

0300 : Len converts to big-endian 0003 --> converts to decimal 03 --> body length is 3 bytes

030201:Main text: 01--GPS 00--No position 00--No position positioning priority is single GPS positioning

33:checksum

Single Bluetooth Beacon positioning:BDCE0100030003000033

BD:Header

CE: Message ID

01: Type 01 Positioning priority settings

00: Validity 00 always valid

0300 : Len converts to big-endian 0003 --> converts to decimal 03 --> body length is 3 bytes

020000:Main text: 03--Bluetooth Beacon 00--No position 00--No position positioning priority is single Bluetooth Beacon positioning

33:checksum

5.1.4 Settings - Health Sampling Reporting Frequency Settings (0XCE02) — Broadcast

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

Type=02 Health Sampling Reporting Frequency Settings

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=02 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 effective this time |
| 2 | U16 | Len | -/- | -/- | Length of the following command |
| 1 | U8 | Healthtype | -/- | -/- | 00—indicates all |
| 1 | U8 |  | -/- | -/- | Body |
| 1 | u8 | Time Unit | -/- | -/- | Time unit 00—minutes, 01—hours |

Health sampling report frequency setting definition -- see 2.3 Device Downlink Description -- Device health sampling frequency issuance (0xCE02)

Health sampling report frequency setting text: Currently, the general version supports: 00 - all, reserved items -- not yet implemented: 01 Steps 02 Heart Rate 03 Temperature 04 Sleep 05 Blood Pressure 06 Blood Sugar 07 Blood Oxygen

Example:

Health sampling reports once every 5 minutes: BDCE0200030000050033

BD:Header

CE: Message ID

02: Type 02 health sampling report frequency setting

00: Validity 00 always valid

0300: Convert to big-endian 0003 --> convert to decimal 03 --> body length is 3 bytes

Healthtype 00---All

05: Body: Convert decimal 05 --> Health sampling reporting frequency reports once every 5 minutes

00: Time Unit 时间单位 00--minutes

33:checksum

5.1.5 Settings - Switch Settings (0XCE04-24) — Broadcast

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Please refer to the instructions for specific types |
| 1 | u8 | switch | -/- | -/- | Switch 00 On, 02 Off, 01 One-time valid |
| 2 | U16 |  | -/- | -/- | Fixed 0000 |

Device alarm settings - Switch type: Specific function description -- See 2.3 Device Downlink Description

07 Drop Alarm Switch 08 Stay Alarm Switch

18 Sleep Function Switch 24 GPS Always On Switch

Type=07 Drop Alarm Switch
Example:

00---Open:BDCE0700000093

BD:Header

CE: Message ID

07: Type 07 Drop Alarm Switch

02: switch 00 Open

0000: Fixed Value

93: Checksum

02---Close:BDCE0702000093

BD:Header

CE: Message ID

07: Type 07 Drop Alarm Switch

02: switch 02 Close

0000: Fixed Value

93: Checksum

Type=08 Stay Alarm Switch
Example:

00---Open:BDCE0800000093

BD:Header

CE: Message ID

08:Type 08 Stop Alarm Switch

02: switch 00 Open

0000: Fixed Value

93: Checksum

02---Off:BDCE0802000093

BD:Header

CE: Message ID

08:Type 08 Stop Alarm Switch

02: switch 02 Close

0000: Fixed Value

93: Checksum

Type=18 Sleep Function Switch
Example:

00---On, enters sleep mode after a period of inactivity and does not report data:BDCE1800000093

BD:Header

CE: Message ID

18:Type 18 Sleep Function Switch

02: switch 00 Open

0000: Fixed Value

93: Checksum

02---Off,in stationary or charging state, still reports data normally: BDCE1802000093

BD:Header

CE: Message ID

18:Type 18 Sleep Function Switch

02: switch 02 Close

0000: Fixed Value

93: Checksum

Type=24 GPS Always On Switch
Example:

00---On:BDCE2400000093

BD:Header

CE: Message ID

24    :    Type 24 GPS Normally Open Switch

02: switch 00 Open

0000: Fixed Value

93: Checksum

02---Close    :    BDCE2402000093

BD:Header

CE: Message ID

24    :    Type 24 GPS Normally Open Switch

02: switch 02 Close

0000: Fixed Value

93: Checksum

5.1.6 Settings - Drop Sensitivity Settings (0xCE15) — Broadcast

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Description |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Type is 0x15 |
| 1 | U8 | Valid | 1 | Default can be 0, no impact for now |
| 2 | U16 | Len | 2 | Contents length of the following parameters |
| 1 | U8 | Param Type | 1 | Parameter Type: 0x00: Sensitivity, 0x01: Height |
| 1 | U8 | Param Level | 1 | Parameter level: 5 levels (0 - 4) |

Drop sensitivity setting-Parameter description:

Sensitivity: refers to the degree to which the drop algorithm is triggered, providing 5 setting levels (0 - 4): Low - Lower (Medium Low) - Medium - Higher (Medium High) - High.

Height: refers to the height that satisfies the trigger of the fall alarm, providing 5 setting levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m - 2.5m.

If the height is set to "1.5m", the device needs to be at least ≥1.5mto trigger the fall alarm.

Note that sensitivity and height cannot be combined in a message

Example:

Set the sensitivity to "Medium Low": BDCE150002000001FF

BD:Header

CE: Message ID

15:Type Fall Sensitivity Setting

00:Fixed value

0200:Len

00:Param Type  Sensitivity Setting

01:Param Level Sensitivity Level 1-->Low (Medium-Low)

FF:checksum

Set height to “1.5m”: BDCE150002000102FF

BD:Header

CE: Message ID

15:Type Fall Sensitivity Setting

00:Fixed value

0200:Len

01:Param Type  Height Setting

02:Param Level Height Level 2-->1.5m

FF:checksum

5.1.7 Issue Sitting Duration Alarm Trigger Time (MSGID=0XCC)—Broadcast

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xCC | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | minute | -/- | -/- | Duration for the stay alarm trigger (unit: minutes; range: 2 — 60), inactivity within this duration will trigger the stay alarm |

Example:

No activity for 5 consecutive minutes. The watch reports a stay alert (when worn): BDCC05DD

BD: Header

CC: Message ID

05: Duration of stay alarm trigger in decimal 05 --> 5 minutes

DD: checksum

5.1.8 Settings - Device Alarm Settings (0XCE03) -- Broadcast -- Special Version Support

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

Type=03 Device Alarm Settings

| Byte offset | Format | Name | Unit | Description |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Fixed as 0x03 - Device Alarm Settings |
| 1 | U8 | Valid | 1 | Fixed as 0x00 - Health Threshold, always valid |
| 2 | U16 | Cfg Len | 2 | Instruction Length |
| 1*N | U8*N | [Bit7] Enable [Bit6 - 0] Threshold Type | 1*N | [Bit7] Whether to enable this type of threshold detection [Bit6 - 0] Set threshold type: 0x01 Heart Rate, 0x02 Systolic Blood Pressure (SBP), 0x03 Diastolic Blood Pressure (DBP), 0x04 Blood Oxygen, 0x05 Temperature (threshold unit is *10, for example, 37.3 is 373) |
| 2*N | I16*N | Threshold Min | 2*N | Threshold lower limit (this content only exists if threshold detection is enabled) |
| 2*N | I16*N | Threshold Max | 2*N | Upper limit of threshold (this content is available only when threshold detection is enabled) |

Device Alarm Setting Definition - Health Threshold - Refer to 2.3 Device Downhill Description - Health Threshold Setting (0 xCE0300)

Note: The default is off, with thresholds in the normal range. Outside this range, the device will report an alarm.

Heart Rate Threshold Issuance Range (20 - 200), Blood Pressure Systolic Pressure Threshold Issuance Range (20 - 200), Blood Pressure Diastolic Pressure Threshold Issuance Range (20 - 200),

Blood Oxygen Threshold Issuance Range (80 - 100), Temperature Threshold Issuance Range (160 - 600)

Example:

Set Multiple Health Thresholds, 1. Heart Rate [60, 120], 2. Disable Blood Oxygen Threshold Detection, 3. Temperature [36.0, 37.3]: BDCE03000B00813C007800048568017501FF

BD: Header

CE: Message ID

03: Type 03 -- Device Alarm Setting

00: Valid 00 -- Health Threshold, always valid

0 B00: Convert to Big Endian --> 000 B --> Convert to Decimal 11 --> Subsequent Command Report Length (excluding checksum) 11 Bytes

81:Convert to binary-->10000001   bit7:1---Enabled     Enabled state   bit6-0:000001-->0x01  Heart rate

3C00:Convert to big-endian-->003C-->Convert to decimal60, heart rate threshold lower limit is60

7800:Convert to big-endian-->0078-->Convert to decimal120, heart rate threshold upper limit is120

04:Convert to binary-->00000100   bit7:0---Disabled , after closing, no threshold follows   bit6-0:0000100-->0x04  Blood oxygen

85:Convert to binary-->10000101  bit7:1---Enabled     Enabled state   bit6-0:0000101-->0x05  Temperature

6801:Convert to big-endian-->0168-->Convert to decimal360(Actual temperature *10)-->Temperature threshold lower limit is36.0

7501:Convert to big-endian-->0175-->Convert to decimal373(Actual temperature *10)-->Temperature threshold upper limit is37.3

FF:Checksum

5.2 Server time synchronization information

5.2.1 Time calibration—Broadcast

| Byte offset | Format | Name | Description |
| --- | --- | --- | --- |
| 1 | U8 | HEADER | Padding: 0xFF |
| 1 | U8 | SeqId | Padding: 0x10 |
| 2 | U16 | years | Year data |
| 1 | U8 | month | Month data |
| 1 | U8 | Day | Day data |
| 1 | U8 | time | Clock, Data |
| 1 | U8 | Minute | Minutes, Data |
| 1 | U8 | Seconds | Seconds, Data |
| 1 | U8 | End | Padding: 0xFF |

Example:

After the server receives FF00FF, it replies with a downlink command to calibrate the time. Note: The example is for the year 2020 and is only for structural analysis. The current year must be sent; otherwise, it will not parse FF1007E409020B1B28FF

FF : Header

10 : Seqid

07E4: Year (2020)

09 : Month, (09)

02 : Day, (02)

0B : Hour, (11)

1B : Minitus, (27)

28 : Second, (40)

FF : End