W200PL-Lorawan-Communication Protocol

Table of Contents

1 Overview	3

2 Device User Instructions	3

2.1 Device Functions and Usage Instructions	3

2.2 Device Default Reporting Logic	6

2.3 Device Downlink Description	8

3 Protocol Packet Structure	11

3.1 Data Header	12

3.2 Message Identifier (Message ID)	12

3.3 Payload	12

3.5 Checksum	13

4 Report messages	15

4.1 Alarm-related reporting	15

4.1.1 Alarm data upload-1 (MSGID=0x02)	15

4.1.2 Alarm Data Upload - 2 (MSGID=0x21) (Supplement to 02)	19

4.1.3 SOS upload (MSGID=0XB5)	23

4.2 Location-Related Reporting	24

4.2.1 GPS/BDS Location Reporting: Location Data Reporting (MSGID=0x03)	24

4.2.2 Bluetooth Location Information (LBE Location) (MsgId=0xD6)	26

4.3 Device Information and Status Reporting	29

4.3.1 Device Charging Status Upload (MSGID=0xC3)	29

4.3.2 Device Status (MSGID=0xE9)	31

4.3.3 Battery Signal (MSGID=0xF9)	32

4.3.4 Firmware Version Number (MSGID=0xBB)	34

4.4 Health-related Reporting	35

4.4.1 Health Data (MSGID=0x32)	35

4.5 Downlink feedback related reporting.	38

4.5.1 Downlink feedback (MSGID=0xC0).	38

4.5.2 Message Status Report (MSGID=0x28)	39

5 Settings	41

5.1 Downlink	41

5.1.1 Set the location reporting frequency (MSGID=0x17)	41

5.1.2 Message Sending (MSGID=0X28)	43

5.1.3 Settings - Positioning Priority Settings (0XCE01)	45

5.1.4 Settings - Health Sampling Reporting Frequency Settings (0XCE02&0X1C)	47

5.1.5 Setting - Device alarm settings (0XCE03)	52

5.1.6 Settings - Switch Settings (0XCE04-24)	54

5.1.7 Settings - Drop Sensitivity Settings (0xCE15)	59

5.1.8 Settings - Vibration Settings (0XCE23)	60

5.1.9 Issue sedentary stay alarm trigger time (MSGID=0XCC)	62

5.1.10 Power Off and Restart (0x77)	62

5.1.11 Restore Factory Settings (MSGID=0X7B)	63

5.1.12 LORA Parameter Settings (0xCF)	64

5.1.13 Weather warning (0XCB) - high-frequency version has, low-frequency version does not have this function	70

5.2 Server Time Synchronization Information	74

5.2.1 Device Reports Request Time Calibration Data	74

5.2.2 Time Calibration Request Data Reply	74

1 Overview

This protocol is suitable for standard Lorawan protocol devices and currently supports the W200PL product. It uses a 32-bit header for synchronization and terminal identification; employs a low-overhead checksum algorithm for validation protection; and utilizes message identifiers to distinguish between different messages.  
    Parsing needs to be based on actual reports; the document is continually being updated. If any errors or omissions are found, please provide timely feedback. Thank you very much.

2 Device User Instructions

2.1 Device Functions and Usage Instructions

General version: The default high-frequency band is AS923 (923.2-924.6) | low-frequency band is CN470 (470.3-471.7), OTAA-CLASSA. If other frequency bands are needed, requirements can be submitted.

Network access related keys:

OTAA:

Application key: 2B7E151628AED2A6ABF7158809CF4F3C

ABP:

Device address: Last eight digits of the watch DEVEUI

Application session key: EF6D6E2503F57AE2FA151CDA87455F18

Network session key: 2E8C8650B4041672BBB9A399F2DEB427

(1) Power on

Please fully charge the device before using it for the first time. The device will automatically power on when the charging reaches the power required for booting. The charging status displays the charging icon.

A full charge is indicated by a green icon. Note: Do not check the device signal while charging.

Manual power on: Press and hold the up button for 10 seconds and release. The interface will display "Welcome".

Note: The default power-on state is the wearing state, and a fall-off alarm will be reported if no heart rate is detected.

(2) Power off

Low battery shutdown: The screen goes off after the interface displays Byebye.

Manual shutdown: Shutdown: Press and hold the button for more than 10 seconds, and the interface displays Byebye before the screen goes off.

Note: Charging does not shut down when the device is on.

(3) SOS function

Trigger mode: After triggering, the device does not enter sleep mode. Press and hold the down button for 3s, the interface displays SOS SEND OK/SOS sending successful, and then the SOS icon appears.

Cancellation method: In SOS mode, press and hold the down button for 3s, the interface displays SEND CENCEL/SOS canceled, and the SOS icon disappears.

(4) Signal Status

No Signal: The interface signal is a horizontal line, indicating that the device has failed to report or is not connected to the network (out of LoRa gateway range). Signal: The interface signal displays a stepped bar graph.

(5) Device Sleep

Trigger Condition: The device enters sleep mode after being stationary for 40 minutes and does not report positioning health data.

(6) Device Parameters - Settings - Parameter Interface: There are multiple pages that can be switched by scrolling down the screen.

sos (Yes): Indicates that the device can trigger sos by pressing a button.

No: Indicates that sos cannot be triggered by pressing a button.

Health (10 min): Indicates that the device has activated health reporting function,

10 min: Indicates that the current health data reporting frequency is 10 minutes,

Health (No): Indicates that the health reporting function of the device has been turned off.

Positioning (BG, 10 min): Indicates that the device has activated positioning reporting function.

B=G: Indicates that the positioning priority is Bluetooth > GPS,

10min: Indicates that the current positioning reporting frequency is10minutes,

Positioning (No): Indicates that the device's positioning reporting function is turned off

Fall (No): Indicates that the device has turned off the fall alarm trigger function,

Fall (Medium,1.5m): Indicates that the device has enabled the fall alarm trigger function

Medium: Indicates that the device's fall alarm trigger sensitivity is medium,

1.5m: Indicates that the device's fall alarm trigger height is1.5m,

Sedentary (No): Indicates that the device has turned off the sedentary stay alarm trigger function

Sedentary (2min): Indicates that the device has enabled the sedentary stay alarm trigger function,

2min: Indicates that the time to trigger the sedentary stay alarm is2minutes of inactivity,

Health Broadcast:

Yes: Indicates that the device has enabled Bluetooth broadcasting of health data,

No: Indicates that the device has disabled Bluetooth broadcasting of health data.

Vibration duration (default): The default indicates that the current device vibration duration is 1.5 seconds, and a vibration duration of (3.000 s) indicates that the current device vibration duration is 3 seconds.

Health alarm threshold: For example, heart rate threshold (90, 100) BPM indicates that the normal heart rate range is 90-100,

Any values outside the normal range will trigger a health anomaly alert. Heart rate/systolic pressure/diastolic pressure/blood oxygen/body temperature threshold (no): Indicates that the heart rate threshold is turned off.

Allow work sleep (yes):

Yes: Indicates that the device has enabled the sleep function, No: Indicates that the device has disabled the sleep function.

Note: The sleep function refers to the device being in a state of detachment and not reporting data, and the device does not report data when it remains still for 40 minutes.

GPS always-on switch (no):

Yes: The internal interface continuously collects GPS, which can generally speed up GPS positioning time under normal conditions but will increase power consumption.

Defaults to No, no need to enable under normal circumstances

Network access method (otaa)(3)(5M): otaa indicates that the network access method is OTAA,

3--Indicates that re-entry into the network after 3 consecutive failed reports,

5M--Indicates that the re-entry network interval is 5 minutes

Device type (ClassA): The reported downlink method is ClassA

Frequency band ((AS923-925)(AS2)): Indicates that the communication frequency band is AS923-2(923.2-924.6)

Message type (confirm): Indicates that the reported data message is of confirm type, and the watch needs to receive an ack reply to determine that the report was successful

(7) Vibration

Receive the downlink text message, health threshold alarm (the interface has a text reminder and vibrates)

The vibration duration can be controlled by downlink (see section 2.3)

2.2 Device Default Reporting Logic

General Version: Reporting needs to be within the Lora gateway range, otherwise the report will fail. The default reporting failure interval is 5 minutes before rejoining the network.

(1) Location-related Reporting

GPS / Bluetooth Beacon: Default reporting frequency is 10 minutes, default location priority: Bluetooth Beacon > GPS. Bluetooth location is prioritized; if unable to locate, switch to GPS location.

(3) Alarm-related Reporting

SOS Alarm (0 xB5): Triggered actively by the user; the triggering method is described in the previous section.

SOS Cancel (0 x02): Triggered actively by the user to cancel; the triggering method is described in the previous section.

Power Off Alarm (0 x21): Triggered by the device actively shutting down or shutting down due to low battery; the triggering method is described in the previous section.

Wear Removal Alarm (0 x02): The device determines wearing based on health sampling reporting frequency. If a heart rate is detected, it reports wearing; if no heart rate is detected, it reports removal.

Prolonged Sitting Alarm (0 x02): Default trigger for reporting is after 15 minutes of inactivity.

Fall Alarm (0 x02): The device free falls from a certain height, triggering based on fall algorithms.

Low battery alarm (0x02): Triggered when the current battery level of the device is less than or equal to0

Health threshold alarm (0x2105): No alarm by default. After the health threshold is reached, an alarm is reported when the health data is not within the threshold range.

Note: When a health threshold alarm is reported, the device vibrates and the interface displays abnormal values.

(4) Health-related reporting

Step count, heart rate, body temperature&wrist temperature, blood pressure, blood oxygen (0x32): Default reporting frequency is10minutes

(5) Device information and status reporting

Device status (0xE9): Report once when the device is powered on, and report again when the reporting frequency changes.

Charging status (0xC3): Report when charging starts, charging ends, and when fully charged.

Battery signal (0xF9): Report once when the device is powered on, and also report after location and health reports.

Software version number (0xBB): Report once when the device is powered on.

(6) Downlink Feedback

Downlink Feedback (0xC0): The device reports after receiving the server's downlink command.

Message Status Feedback (0x28): After receiving a downlink text message, the device clicks to accept or reject.

(7) Time Synchronization Request Reporting

Time Synchronization Request (FF00FF): The device reports once each time it powers on, requesting the server's downlink command to calibrate the time.

Note: The device may report multiple complete messages in a single packet, meaning that one data packet contains several complete messages. Be careful not to miss anything; the messages are complete and will not be interrupted into the next data packet.

Eg: bdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9bdf9010000006400002800000019a9cf61ca

This data packet contains (0xD6) Bluetooth positioning and (0xF9) battery signal messages.

(0xD6) Bluetooth Positioning: bdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9

(0xF9) Battery Signal: bdf9010000006400002800000019a9cf61ca

2.3 Device Downlink Description

General version:

(1) Device location reporting frequency issued (0x17)

The default reporting frequency is 10 minutes, with a maximum of 1 minute and a minimum of 1440 minutes. After the device receives the downlink command, the device will report according to the time period and frequency of the issued command.

Report data, report according to the default reporting frequency outside the time period, such as: 00:00-18:00 2 minutes location report,

Then report according to the default 10-minute reporting frequency outside the time period

(2) Text message issued (0x28)

Unicode encoding, up to 20 Chinese characters, one Chinese character occupies 2 bytes, one English letter occupies 2 bytesThe previous low-frequency firmware version was: GB2312 encoding, up to 20 Chinese characters, one Chinese character occupies 2 bytes, one English letter occupies 1 byte

Message list - historical messages can save up to 50 messages (Note: Firmware without a message list can save up to 20 messages)

(3) Device location priority issued (0xCE01)

Default location priority Bluetooth>gps,

The priority for positioning is as follows: gps > Bluetooth beacon, gps positioning is prioritized, switch if gps positioning fails.

Bluetooth beacons, when positioning is successful, will not switch to the next positioning priority to generate a location.

(4) Device health sampling frequency issued (0xCE02 & 0X1C)

Device health sampling frequency assignment (0xCE02)

Health sampling frequency is set to a default of 10 minutes, with a maximum reporting interval of 2 minutes.

Device health sampling frequency distribution (0 x1C): — Note that it is different from W200PG

The default frequency for health sampling is 10 minutes, with a maximum of 2 minutes. After the device receives the report-down command, the device will follow the issued command.

Data reporting is limited to specific time periods and frequencies; data outside these time periods will not be reported. For example: if the reporting period is from 00:00 to 18:00 with a 10-minute positioning report, then no reports will be made outside of this time period.

(5) Bluetooth Broadcast Switch (0xCE05)

Bluetooth broadcasting is enabled by default and can be turned off remotely; once turned off, Bluetooth broadcasting will cease.

(6) Sitting duration alarm trigger time distribution (0xCC)

Value range: 2 minutes - 60 minutes. Note that this will only be effective if the sedentary alert switch is turned on.

For example: if the downward trigger time is set to 10 minutes, the device will trigger a sedentary alert if it remains stationary for 10 minutes.

(7) Device shutdown and restart command (0x77)

Note that this command can only take effect when the device is powered on; it cannot receive commands when shut down.

Shutdown command: Under powered-on status, send this command to shut down the device. It cannot receive commands after shutdown.

Restart command: Under powered-on status, send this command to reboot the device.

(8) Fall sensitivity and height command (0xCE15)

The latest universal firmware defaults to a lower fall sensitivity: low (medium-low). Fall height: 1.5m.

Sensitivity: refers to the degree of adherence to the fall algorithm, providing 5 settings levels (0 - 4): Low - Lower (Medium-Low) - Medium - Higher (Medium-High) - High.

Height: refers to the height at which a fall alert is triggered, providing 5 setting levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m - 2.5m.

(9) Sedentary Alarm Switch (0xCE08)

Disabled by default, the device does not report prolonged sitting alarms; when enabled, the default trigger time is 15 minutes, and if there is no movement for 15 minutes, it triggers a prolonged sitting alarm.

(10) Control device to trigger sleep switch issued (0xCE18)

Default is enabled; the device enters sleep mode after 40 minutes of inactivity. Once the command to turn off is received, the device will not enter sleep mode.

(11) Fall alarm switch (0xCE07)

Default state for fall alarm is off; when turned on, the device reports a fall alarm after detecting a drop.

(12) Key-triggered SOS activation switch (0xCE19)

Default is enabled; a long press on the charging line button can trigger SOS, and when off, a long press on the charging line button will not report SOS.

(13) Health threshold setting (0xCE0300)

Health threshold issued as a downlink command; upon receipt, the device receives the normal range for health thresholds. Any health data outside this range will trigger an alarm.

(14) Device vibration settings (0xCE23)

After the downlink command, after the device receives it, the device vibration duration becomes the downlink duration, ranging from 0-60 seconds.

(15) GPS always-on switch (0xCE24)

When enabled, the internal interface always collects GPS. In general environments, it can speed up GPS positioning time, but the power consumption will increase. The default is off.

(16) LORA parameter delivery (0XCF)

Downlink commands can be used to set the network access method, message type, how many times to report failure before re-accessing the network, etc. -- see the downlink command section for details.

(17) Calibration time command downlink (0XFF)

When the device reports a time synchronization request message, this command is immediately downlinked for watch time calibration.

(18) Restore factory settings

After the downlink restores the factory settings, the device restores the default configuration (see the downlink section for details).

(19) Weather warning delivery (0xCB)

Multiple sets of weather alerts can be issued, and each set of weather alerts must be different; alerts of the same type cannot appear.

Note: This function is only available for high-frequency firmware; low-frequency versions do not have it.

3 Protocol Packet Structure

A basic protocol packet structure is shown in Figure 1:

3.1 Data Header

Each packet starts with a 1-byte Header (except for the request calibration time message: ff00ff):

Currently, the Header for Ufov devices is fixed at BD (except for the request calibration time message: ff00ff)

Header: 0xBD;

3.2 Message Identifier (Message ID)

The content represented by MessageId is detailed in Chapter 4.

3.3 Payload

The payload we refer to below refers to the valid body content in the protocol, excluding the head and checksum. The length of the body is noted in parentheses.

The data format used in the payload is shown in the following table:

【U-unsigned；I-signed；X-bitfield；Number-bytes occupied】

The following protocols use little-endian except for ch, u8, i8, and x8

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Explanation |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short |
| i8 | Signed Char | 1 | -128-127 | 1 | Short |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Integer |
| x16 | Bitfield | 2 | - | - | bit 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | unsigned long |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | long |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit long integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating point type |

3.5 Checksum

The content added for the checksum includes the payload, as shown in Figure 1. The algorithm is as follows, and for the default general version devices, checksum is not mandatory and can be ignored. During downlink commands, any arbitrary byte will suffice.

// Assuming content="BDE9000600010A00000A00", the result of sum.ToString("X2") is 07 // The following is a C# code calling method

private string CheckSum(string content)

{

int     sum =     0;

var     bytes = Utility.strToHexByte(content);

foreach     (    var     b     in     bytes)

{

sum += b;

sum %=     0x100    ;

}

sum =     0xff     - sum;

return     sum.ToString(    "X2"    );

}

4 Report messages

4.1 Alarm-related reporting

4.1.1 Alarm data upload-1 (MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0x02 | See below 见下方定义 | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below 见下方定义 |
| 4 | U32 | Timestamp | - | - | Timestamp (a timestamp will be added at the end when resending) |

Upl_warn contents

| bit (the position of 1 in the binary) | Name | Description | Raw message in little-endian 16 hexadecimal | Converted to big-endian 16 hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 14 | Drop alarm | Drop alarm | 0040 | 4000 | 4*4096=16384 |
| 8 | Equipment Wearing | Equipment Wearing | 0001 | 0100 | 1*256=256 |
| 7 | SOS Cancel | SOS Cancel | 8000 | 0080 | 8*16=128 |
| 5 | Long Sitting Alarm (Inactive) | Long Sitting Alarm (Inactive) | 2000 | 0020 | 2*16=32 |
| 4 | Remove (Fall Off) Device | Remove (Fall Off) Device | 1000 | 0010 | 1*16=16 |
| 2 | Power Off | Power Off | 0400 | 0004 | 4 |
| 0 | Low Battery | Low Battery | 0100 | 0001 | 1 |

Example:

Wear Alarm: BD020001E377BD678A

BD:Header

02:Message ID

0001:Upl_warn alarm content, converted to big-endian:0100--> Wear alarm

E377BD67:Timestamp converted to big-endian 67BD77E3--> converted to decimal 1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> converted to Beijing time:2025-02-25 15:57:23

8a:checksum

Fall Alarm: BD021000E377BD678A

BD:Header

02:Message ID

1000:Upl_warn alarm content, converted to big-endian0010--> Fall alarm

E377BD67:Timestamp converted to big-endian 67BD77E3--> converted to decimal 1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> converted to Beijing time:2025-02-25 15:57:23

8a:checksum

SOS Cancel: BD028000E377BD678A

BD:Header

02:Message ID

8000 : Upl_warn alarm content, converted to big-end 0080 --> SOS cancellation

E377BD67:Timestamp converted to big-endian 67BD77E3--> converted to decimal 1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> converted to Beijing time:2025-02-25 15:57:23

8a:checksum

Drop Alarm: BD020040E377BD678A

BD:Header

02:Message ID

0040: Upl_warn alarm content, converted to big-endian4000--> Drop Alarm

E377BD67:Timestamp converted to big-endian 67BD77E3--> converted to decimal 1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> converted to Beijing time:2025-02-25 15:57:23

E7: checksum

Sit Too Long Alarm: BD022000E377BD678A

BD:Header

02:Message ID

2000: Upl_warn alarm content, converted to big-endian0020--> Sit Too Long Alarm

E377BD67:Timestamp converted to big-endian 67BD77E3--> converted to decimal 1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> converted to Beijing time:2025-02-25 15:57:23

8A: checksum

Low Battery Alarm: BD020200E377BD678A

BD:Header

02:Message ID

0100: Upl_warn alarm content, converted to big-endian0001--> Low Battery Alarm

E377BD67:Timestamp converted to big-endian 67BD77E3--> converted to decimal 1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> converted to Beijing time:2025-02-25 15:57:23

8A: checksum

Shutdown Alarm: BD020400E377BD678A

BD:Header

02:Message ID

0400: Upl_warn alarm content, converted to big-endian0004--> Shutdown Alarm

E377BD67:Timestamp converted to big-endian 67BD77E3--> converted to decimal 1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> converted to Beijing time:2025-02-25 15:57:23

8A: checksum

Note: This alarm is for devices that do not distinguish between shutdown types. See the next section for devices that distinguish between shutdown types.

4.1.2 Alarm Data Upload - 2 (MSGID=0x21) (Supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0x21 | See below 见下方定义 | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | Alarm type |
| N |  | Upl_warn | - | - | Alarm content See below for different Alarm type type definitions |
| 4 | U32 | Timestamp |  |  | Timestamp (a timestamp will be added at the end when resending) |

Alarm type=1

Upl_warn contents

| bit (the position of 1 in the binary) | Name | Description | Raw message in little-endian 16 hexadecimal | Converted to big-endian 16 hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging shutdown alarm | Device charging automatic shutdown (firmware requires function support) | 04000000 | 0004 | 4 |
| 1 | Low battery shutdown alarm | Device power too low to shut down (firmware functionality required) | 02000000 | 0002 | 2 |
| 0 | Active shutdown alarm | Device manually shut down (firmware functionality required) | 01000000 | 0001 | 1 |

Example:

Charging shutdown alarm: BD21010004000000E377BD6767

BD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001 --> Alarm type=1

04000000:Upl_warn alarm content, converted to big-endian 0004 --> Charging shutdown alarm

E377BD67:Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> converted to Beijing time: 2025-02-25 15:57:23

67:checksum

Low battery shutdown alarm: BD21010002000000E377BD6767

BD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001 --> Alarm type=1

02000000:Upl_warn alarm content, converted to big-endian 0002-->low battery shutdown alarm

E377BD67:Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> converted to Beijing time: 2025-02-25 15:57:23

67:checksum

Active shutdown alarm: BD21010001000000E377BD6767

BD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001 --> Alarm type=1

01000000:Upl_warn alarm content, converted to big-endian 0001-->active shutdown alarm

E377BD67:Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> converted to Beijing time: 2025-02-25 15:57:23

67:checksum

Alarm type=5

Upl_warn contents

| Format | Name | Unit | Decription |
| --- | --- | --- | --- |
| U16 | Type | 2 | Fixed to 0x05 |
| U32 | Timestamp | 4 | Timestamp |
| U16 | Content Len | 2 | Alarm content length |
| U8 | Health Warn Type1 | 1 | Health threshold alarm types: 0x01 Heart rate, 0x02 Systolic blood pressure (SBP), 0x03 Diastolic blood pressure (DBP), 0x04 Blood oxygen , 0x05 Temperature |
| U8 | Type1 | 1 | 01 less than the set health threshold, 02 greater than the set health threshold |
| I16 | Health Value1 | 2 | Health result value |
| U8 | Type2 | 1 | 01 less than the set health threshold, 02 greater than the set health threshold |
| I16 | Health Value2 | 2 | Health result value |
| .... | .... | .... | .... |

Example:

The device detected that the heart rate (65) is outside the range of the issued health threshold: BD210500E377BD6704000102410073

BD: Header

21:Message ID message id

0500:Alarm type Alarm type Convert to big-endian 0005-->Alarm type=5

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

0400:Content Len Alarm content length Convert to big-endian0004-->Alarm content length is 4 bytes, note: the length will increase if there are multiple health threshold alarms

01:Health Warn Type Health threshold alarm type 01-->Heart rate alarm

02:Type is greater than the set health threshold & less than the set health threshold 01-->less than the set health threshold

4100:Health Value  The actual reported health value converted to big-endian0041-->converted to decimal 65-->reported heart rate value is 65

73:checksum

4.1.3 SOS upload (MSGID=0XB5)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xB5 | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | U8 | Status | Status: 1:SOS |
| 4 | Int32 | timestamp | Timestamp, little-endian (timestamp little Endian) |

Example:

Hexadecimal Raw Message:BDB501E377BD6755

BD: Header

B5: Message ID

01: Status 01--SOS

E377BD67: Timestampconverted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> converted to Beijing time: 2025-02-25 15:57:23

55: checksum

4.2 Location-Related Reporting

4.2.1 GPS/BDS Location Reporting: Location Data Reporting (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0x03 | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude |
| 8 | Double | lat |  | - | latitude |
| 1 | U8 | north_south |  | - | /N or S/ |
| 1 | U8 | east_west |  | - | /E or W/ |
| 1 | U8 | status |  | - | /A or V/ |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original hexadecimal message: BD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BD: Header

03: Message ID message id

963E870C3E675E40: lon longitude Convert to big endian 405E673 E0C873E96-->Longitude: 121.6131622 Note: See the parsing example below for conversion and parsing

FF2110C2B6343F40: lat latitude Convert to big endian `403 F34B6C21021FF-->Latitude: 31.2059137 Note: See the parsing example below for conversion and parsing

4E: north_south 4E-->Convert to ASCII code N-->North latitude

45: east_west 45-->Convert to ASCII code E-->East longitude

41: status data validity 41-->Convert to ASCII code A-->Valid data

E377BD67: Timestamp timestamp Convert to big endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> converted to Beijing time:2025-02-25 15:57:23

78: checksum

Note: The parsed GPS latitude and longitude are in the WGS-84 coordinate system. If the map uses Baidu Gaode, etc., the coordinate system needs to be converted.

Parse satellite positioning message Java code Example:

//Message: BD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public static void main (String[] args){

//Message 7d9f84ac81815c40 actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d", 16)));   //114.02353966666665//Message e766926b1d893640 actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16)));  //22.535605166666667

HexToStr(data.Substring("4e"));    //N

HexToStr(data.Substring("45"));    //E

HexToStr(data.Substring("41"));    //A   A represents valid data, V represents invalid data, can be discarded//Message 749d695f actual value 5f699d74

Date date = new Date();

date.setTime(Long.parseLong("5f699d74",16)*1000);

SimpleDateFormatsdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.2.2 Bluetooth Location Information (LBE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xD6 | See below 见下方定义 | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 0 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple groups of information, each group may have multiple ibeacons |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identifiers) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identifiers) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identifiers) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identifiers) |
| S8 | RssiN | 1 | RSSI (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identifiers) |
| S8 | Rssi0 | 1 | RSSI (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identifiers) |
| S8 | RssiN | 1 | Rssi (Bluetooth beacon signal strength) |
| ... | ... | ... | ... |

Note: In the general version, the total number of Bluetooth beacon groups is fixed to 1, and the maximum number of Bluetooth beacons is 4. The sorting is based on the signal strength from largest to smallest. By default, the watch only recognizes our own beacons. If you need to connect to other beacons, please consult relevant personnel via WeChat.
Example:

Original hexadecimal message: BDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

BD: Header

D6: Message ID

00: type

01: Only one set of ibeacon data

E377BD67: Timestampconverted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> converted to Beijing time: 2025-02-25 15:57:23

04: The first group has 4 beacon information

4327: Convert to big endian --> major: 0x2743 --> Convert to decimal 10051

1794: Convert to big-endian --> minor: 0x9417 --> Convert to decimal 37911

AC: RSSI 0xAC --> Convert to decimal two's complement -84

4327: Convert to big-endian --> major: 0x2743 --> Convert to decimal 10051

3094: Convert to big-endian --> minor: 0x9430 --> Convert to decimal 37936

AA: RSSI 0xAA --> Convert to decimal two's complement -86

4327: Convert to big-endian --> major: 0x2743 --> Convert to decimal 10051

b956: Convert to big-endian --> minor: 0x56b9 --> Convert to decimal 22201

A5: RSSI 0xA5 --> Convert to decimal two's complement -91

4327: Convert to big-endian --> major: 0x2743 --> Convert to decimal 10051

FE94: Convert to big-endian --> minor: 0x94FE --> Convert to decimal 38142

A5: rssi: 0xA5 --> Convert to decimal two's complement -91

6a -- Checksum

Bluetooth beacon major, minor description: The default shipping Bluetooth beacon has a beacon code attached.

For example: 00081005345866, then the major of the Bluetooth beacon is 10053, the minor is 45866, and the manufacturer ID is 0008 (this is useless and has no practical significance).

4.3 Device Information and Status Reporting

4.3.1 Device Charging Status Upload (MSGID=0xC3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xC3 | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Status | / | / | 0 start, 1 end, 2 fully charged |
| 1 | U32 | Timestamp | / | / | Timestamp (a timestamp will be added at the end when resending) |

Note: This message requires device firmware support, the default latest general version supports
Example:

Original hexadecimal message:BDC301E377BD678A

BD: Header

C3:Message ID

01: Device charging completed

E377BD67: Timestampconverted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> converted to Beijing time: 2025-02-25 15:57:23

8A:checksum

Raw hexadecimal message:BDC300DB4D2F668A

BD: Header

C3:Message ID

00:The device starts charging

E377BD67: Timestampconverted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> converted to Beijing time: 2025-02-25 15:57:23

8A:checksum

Raw hexadecimal message:BDC302DB4D2F668A

BD: Header

C3:Message ID

02:The device is fully charged

E377BD67: Timestampconverted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> converted to Beijing time: 2025-02-25 15:57:23

8A:checksum

4.3.2 Device Status (MSGID=0xE9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xE9 | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | type |  | / | Default—00 |
| 2 | u16 | length |  | / | Message Length |
| 1 | u8 | location |  | / | Reporting frequency: 00—- defaults unchanged, 01—- modified by downlink (if there are multiple time periods in the downlink, only report the current time period's reporting frequency) |
| 2 | u16 | frequency |  | / | Frequency, unit - minutes |
| 1 | u8 | health |  | / | Health reporting frequency: 00—- defaults unchanged, 01—- modified by downlink |
| 2 | u16 | frequency |  | / | Frequency: unit - minutes |

Note: This message reports once on startup, and will report once after the frequency is modified by downlink

Example:

Original 16-bit hexadecimal message: BDE9000600010A00010A0033

BD: Header

E9: Message ID

00:type=00

0600:length The length of the following message converted to big-endian -->0006--> converted to decimal 6, the message length is 6 bytes

01:location  01 indicates that the downlink has modified the location reporting frequency

0A00:frequency location reporting frequency converted to big-endian -->000A--> converted to decimal 10, the reporting frequency is 10 minutes

01:health 01 indicates that the downlink has modified the health reporting frequency

0A00:frequency Health reporting frequency converted to big-endian -->000A--> converted to decimal 10, the reporting frequency is 10 minutes

33:checksum

If the device has no health reporting function or no location reporting function, then the values for health reporting frequency and location reporting frequency will remain consistent, indicating the device's reporting frequency

4.3.3 Battery Signal (MSGID=0xF9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xF9 | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery Type 0: 4-level system, 1: 5-level system, 2: Percentage, 3: Voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery Value If Bat_type is 0: The battery value range is 0-3 (0 is 25%, 3 is 100%) If Bat_type is 1: The battery value range is 0-4 (0 is 20%, 4 is 100%) If Bat_type is 2: The battery value range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: Percentage, 1: 5-level system 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal Strength |
| 1 | U8 | Other_type |  |  | Extended Type 0: Full step counting 1: Incremental step counting 2: Vibration |
| 4 | U32 | Num |  |  | Expansion Value |
| 4 | U32 | Timestamp | -/- | -/- | UTC Timestamp (in seconds) |

Example:

Raw 16 Hex Message: BDF90104000050000095000000E377BD67AA

BD: Header

F9: Message ID (Message ID) 01: Bat_type - Battery Type 01 --> 1 --> Level 5 Battery (0 - 4)

0400: Bat_volt - Battery Voltage Value converted to Big Endian 0004 --> 4 --> Level 4 corresponds to 100% battery

00: Signal_type - Signal Type 00 --> 0 --> Percentage

5000: Signal_strength - Signal Strength converted to Big Endian 0050 --> converted to decimal 80 --> 80% Signal Strength

00: Other_type - Extended Type 00 --> 0 --> Full Pedometer

95000000: Num - Extended Value - Pedometer converted to Big Endian 00000095 --> converted to decimal 149 --> Steps count is 149

E377BD67: Timestamp converts to big-endian 67 BD77E3 --> converts to base 10 1740470243 --> timestamp is 1740470243 seconds -->

Converted to standard time format UTC time:2025-02-25 07:57:23--> converted to Beijing time:2025-02-25 15:57:23

AA: checksum

4.3.4 Firmware Version Number (MSGID=0xBB)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xBB | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit Decription |
| --- | --- | --- | --- |
| 1 | UINT8 | Version_len | Software version number length |
| N | ASCII[n] | Version | Software Version Number |

Example:

BDBB1043323331304C2E3437302E41412E3031

BD:Header

BB:Message ID

10:Version_len converted to 10 decimal 16 --> The software version number length is 16 bytes

43323331304C2E3437302E41412E30:converted to string --> C2310L.470.AA.01

31:checksum

4.4 Health-related Reporting

4.4.1 Health Data (MSGID=0x32)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0x32 | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | 00 |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 2 | U16 | contentLength |  |  | Total length of the following content |
| 1 | U8 | ID |  |  | Id (Type + Reported Value Length), see below for ID type definition |
| N | N*U8 | Val1 |  |  | Reported value of Id (byte length varies depending on the reported value length) |
| ... | ... | ... | ... | ... | ... |
| 1 | U8 | ID |  |  | Id (Type + Reported Value Length), see below for ID type definition |
| N | N*U8 | Val1 |  |  | Reported value of Id (byte length varies depending on the reported value length) |

ID type definition (hexadecimal):

01 Steps02 Heart rate 03 Body temperature 04 Wrist temperature 06 Diastolic pressure 07 Systolic pressure 08 Blood oxygen 09 HRV(Supported by special versions)

(05 Blood glucose 10 Number of sit-ups11 Running pace and distance 12 Rope skipping speed and number )---Reserved, not available

Default universal version support:01 Steps02 Heart rate 03 Body temperature 04 Wrist temperature 06 Diastolic pressure 07 Systolic pressure 08 Blood oxygen

Example:

Original16hexadecimal message: BD3200E377BD6711000A1E00114B314A39711A4A0122bc00416212

BD:Header

32    :Message ID

00    :    type    =    00

E377BD67:Timestamp converted to big-endian    67    BD77E3-->convert    to    decimal    1740470243    -->timestamp is    1740470243    seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> converted to Beijing time:2025-02-25 15:57:23

1100    :length of subsequent message converted to big-endian-->    0011    -->convert to decimal    17, length of subsequent message is    17    bytes

0    A:ID    0x0A    -->convert to binary    00001010    the first five bits are    00001    -->convert to hexadecimal    01, representing data ID for step counting (    0x01    ),

the last three bits are    010    -->convert to decimal    02, representing that the length of step counting data is    2    bytes

1E00    :Step report value converted to big-endian-->    001    E-->convert to decimal    30, total step data is    30    steps

11    :ID    0x11    -->convert to binary    00010001    the first five bits are    00010    -->convert to hexadecimal    02, representing data ID for heart rate (    0x02    ),

the last three bits are    001    -->convert to decimal    1, representing that the length of heart rate data is    1    byte

4B: Heart rate reported value converted to decimal -->75, heart rate data is75

31:ID 0x31--> convert to binary00110001 The first five digits are 00110--> convert to16hexadecimal06, representing data ID diastolic blood pressure (0x06),

The last three digits are 001--> convert to decimal1, representing the length of the diastolic blood pressure data as1byte

4A: Blood pressure diastolic reported value, converted to decimal -->74, diastolic blood pressure data is74

39:ID 0x3--> convert to binary00111001 The first five digits are 00111--> convert to16hexadecimal07, representing data ID systolic blood pressure (0x07),

The last three digits are 001--> convert to decimal1, representing the length of the systolic blood pressure data as1 byte

71: Blood pressure systolic reported value, converted to decimal -->113, systolic blood pressure data is113

1A:ID 0x1A--> convert to binary00011010 The first five digits are 00011--> convert to16hexadecimal03, representing data ID body temperature (0x03),

The last three digits are 010--> convert to decimal2, representing the length of the body temperature data as2bytes

4A01: Body temperature reported value, converted to big-endian -->014A--> converted to decimal330-->330*0.1--> body temperature data is33degrees Celsius

22 :ID 0x22 --> Convert to binary 00100010 The first five bits are 00100 --> Convert to hexadecimal 04, representing data ID wrist temperature (0x04),

The last three digits are 010 --> Convert to decimal 2, indicating that the wrist temperature data length is 2 bytes.

BC00: Wrist temperature report value, converted to big-endian --> 00 BC --> Convert to decimal 188 --> 188 * 0.1 --> The wrist temperature data is 18.8 degrees Celsius.

41 :ID  0x41 --> Convert to binary 01000001    The first five digits are 01000 --> Convert to hexadecimal 08, code data ID for blood oxygen (0x08),

The last three digits are 001 --> Convert to decimal 1, indicating that the blood oxygen data length is 1 byte.

62 : Blood oxygen report value, converted to decimal --> 98, blood oxygen data is 98.

12 : checksum.

4.5 Downlink feedback related reporting.

4.5.1 Downlink feedback (MSGID=0xC0).

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xC0. | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID length. |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This command is used for feedback of downlink commands, returning the Message ID of the previously received downlink command (multiple Message IDs can be returned collectively). Some downlink commands do not have C0 feedback.

Original hexadecimal message:BDC0011720

BD: Header

C0:Message ID

01:length Message ID length Convert decimal 01--> Message ID length is 1 byte

17  Type indicates that the device received the downlink command Message ID is 17

20:checksum

4.5.2 Message Status Report (MSGID=0x28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0x28 | See below 见下方定义 | CK_sum |

Payload contents

| Bytes | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | timestamp | -/- | -/- | Timestamp |
| 1 | U8 | Type | -/- | -/- | Character encoding type (corresponding to the line 0X28), GB2312 encoding: 0x03; Unicode encoding: 0x05 |
| 1 | U8 | status | -/- | -/- | Message status - 0x01: Confirmed, 0x02: Rejected |
| 4 | U32 | seqID | -/- | -/- | (corresponding to the message id seqID of line 0X28) |

Example:

Note: The current generic version does not differentiate character encoding types, fixed at 03; future versions will differentiate.

Original hexadecimal message: BD28FD3BA068030268A03BE477

BD: Header

28:Message ID

FD3BA068:timestamp converted to big endian 68A03BFD --> convert to decimal --> timestamp is 1755331581 seconds -->

Converted to standard time format UTC time: 2025-08-16 08:06:21 --> converted to Beijing time: 2025-08-16 16:06:21

03:Type 03 -- fixed value

02:status Message status 02--Interface click reject button--Rejected

68A03BE4:seqID The id of the downlink message is 68A03BE4

77:checksum

5 Settings

5.1 Downlink

5.1.1 Set the location reporting frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0x17 | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |

Note: One day is 00:00-23:59
Example:

00:00-23:59(All day) Report location data every10minutes:BD17010A000000173B00000000000000000000000000000000000000000097

BD:Header

1C:Message ID

01:enable Whether to enable Convert to decimal01-->1--Enabled

0A00:Interval time interval converted to big-endian000A-->converted to decimal10-->time interval is10minutes

00:time_start_h Start time-hour, convert to decimal00-->Start time-hour is00

00:time_start_m Start time-minute, convert to decimal00-->Start time-minute is00

17:time_end_h End time-hour, convert to decimal23-->End time-hour is23

3B:time_end_m End time-minute, convert to decimal59-->End time-minute is59

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000: All filled in disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000: All filled in disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000: All filled in disabled state00

97:checksum

5.1.2 Message Sending (MSGID=0X28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0x28 | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | U8 | type | Message type, if it is a downlink text message, Unicode encoding is 05, GB2312 encoding is 03 |
| 4 | Uint32 | seqID | Unique ID of the message |
| 1 | U8 | CONTENT LEN | Length of content |
| N | N | CONTENT | Content, Unicode encoding (little-endian priority, note conversion, one Chinese character occupies 2 bytes, one letter occupies 2 bytes) GB2312 encoding (little-endian priority, one Chinese character occupies 2 bytes, one letter occupies 1 byte) |

Note: The default latest general version for Chinese character encoding is Unicode, earlier low-frequency firmware versions use GB2312 encoding
Each message ID must be unique; devices cannot receive duplicate IDs
Example:

Unicode encodingText message: hello,world  BD28050300000016680065006C006C006F002C0077006F0072006C006400DD

BD: Header

28: Message ID

05: type message type Convert decimal 05-->type=05

03000000: seqID information id Information id is 03000000

16: CONTENT LEN content length Convert decimal 22, content length is 22 bytes

680065006C006C006F002C0077006F0072006C006400:

Convert every 2 bytes to big-endian --> Convert to Unicode character --> Combine final - text message content:hello,world

DD:checksum

Previously, low-frequency version firmware used GB2312 encoding:Text message: Hello World  BD28036895D13F08C4E3BAC3CAC0BDE779

BD: Header

28:Message ID

03:type message type Convert decimal 03-->type=03

6895D13F: seqID information id Information id is 6895D13F

08: CONTENT LEN content length converted to decimal 8

C4E3BAC3CAC0BDE7: converted to string Hello World --> text message content: Hello World

79: checksum

5.1.3 Settings - Positioning Priority Settings (0XCE01)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xCE | See below 见下方定义 | CK_sum |

Payload contents

Type=01 Positioning Priority Settings

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 effective this time |
| 2 | U16 | Len | -/- | -/- | The length of the following command |
| n | n |  | -/- | -/- | Main text |

Definition of positioning priority settings -- See 2.3 Device Downlink Instructions -- Device positioning priority issuance (0 xCE01)

The main text of positioning priority settings: Currently, the general version supports: 01--GPS, 03--Bluetooth beacon (requires additional deployment of Bluetooth beacons);

Example:

GPS positioning priority (GPS > Bluetooth beacon): BDCE0100030001030033

BD: Header

CE: Message ID

01: Type 01 positioning priority settings

00: Validity 00 always valid

0300: Len converted to big-endian 0003 --> converted to decimal 03 --> main text length is 3 bytes

010300: Main text: 01--GPS 03--Bluetooth beacon positioning priority is GPS > Bluetooth beacon

33:checksum

Bluetooth positioning priority (Bluetooth beacon > GPS):BDCE0100030003010033

BD: Header

CE: Message ID

01: Type 01 positioning priority settings

00: Validity 00 always valid

0300: Len converted to big-endian 0003 --> converted to decimal 03 --> main text length is 3 bytes

030100:Main Text: 03--Bluetooth Beacon  01--GPS  Positioning priority is Bluetooth Beacon > GPS

33:checksum

Single GPS Positioning:BDCE0100030001000033

BD: Header

CE: Message ID

01: Type 01 positioning priority settings

00: Validity 00 always valid

0300: Len converted to big-endian 0003 --> converted to decimal 03 --> main text length is 3 bytes

030201:Main Text: 01--GPS 00--No Positioning 00--No Positioning  Positioning priority is Single GPS Positioning

33:checksum

Single Bluetooth Beacon Positioning:BDCE0100030003000033

BD: Header

CE: Message ID

01: Type 01 positioning priority settings

00: Validity 00 always valid

0300: Len converted to big-endian 0003 --> converted to decimal 03 --> main text length is 3 bytes

020000:Main Text: 03--Bluetooth Beacon 00--No Positioning 00--No Positioning  Positioning priority is Single Bluetooth Beacon Positioning

33:checksum

5.1.4 Settings - Health Sampling Reporting Frequency Settings (0XCE02&0X1C)

Previous firmware used 0xCE02, latest firmware uses 0x1C

Health Sampling Reporting Frequency Settings (0XCE02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xCE | See below 见下方定义 | CK_sum |

Payload contents

Type=02 Health Sampling Reporting Frequency Settings

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=02 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 effective this time |
| 2 | U16 | Len | -/- | -/- | The length of the following command |
| 1 | U8 | Healthtype | -/- | -/- | 00 - Indicates all |
| 1 | U8 |  | -/- | -/- | Main text |
| 1 | u8 | Time Unit | -/- | -/- | Time unit 00 - minutes, 01 - hours |

Definition of health sampling reporting frequency setting--See 2.3 Device downlink description--Device health sampling frequency distribution (0xCE02)

Health sampling reporting frequency setting text: Currently, the general version supports:00- All, reserved item--Not yet implemented: 01 Step count 02 Heart rate 03 Temperature 04 Sleep 05 Blood pressure 06 Blood sugar 07 Blood oxygen

Example:

Health sampling reported once every 5 minutes:BDCE0200030000050033

BD: Header

CE: Message ID

02:Type 02 Health sampling reporting frequency setting

00: Validity 00 always valid

0300:Converted to big-endian 0003-->Converted to decimal 03-->The text length is 3 bytes

00:Healthtype 00---All

05:Body: Convert to decimal 05-->Health sampling report frequency reports once every 5 minutes

00:Time Unit 时间单位 00--minutes

33:checksum

Health sampling reporting frequency setting (0X1C)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed as 0xBD | 0x1C | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |

Definition of health sampling reporting frequency setting -- see 2.3 Device downstream description -- Device health sampling frequency issuance (0x1D)

Note: After setting, report based on time intervals; no reporting outside the time intervals.

Example:

00:00-23:59(All day) Report health data every 10 minutes: BD1C010A000000173B00000000000000000000000000000000000000000097

BD:Header

1C:Message ID

01:enable Whether to enable Convert to decimal01-->1--Enabled

0A00:Interval time interval converted to big-endian000A-->converted to decimal10-->time interval is10minutes

00:time_start_h Start time-hour, convert to decimal00-->Start time-hour is00

00:time_start_m Start time-minute, convert to decimal00-->Start time-minute is00

17:time_end_h End time-hour, convert to decimal23-->End time-hour is23

3B:time_end_m End time-minute, convert to decimal59-->End time-minute is59

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000: All filled in disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000: All filled in disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000: All filled in disabled state00

97:checksum

5.1.5 Setting - Device alarm settings (0XCE03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xCE | See below 见下方定义 | CK_sum |

Payload contents

Type=03 Device alarm settings

| Byte offset | Format | Name | Unit | Decription |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Fixed as 0x03—Device alarm settings |
| 1 | U8 | Valid | 1 | Fixed to 0x00—health threshold, always valid |
| 2 | U16 | Cfg Len | 2 | Instruction Length |
| 1*N | U8*N | [Bit7] Enable [Bit6 - 0] Threshold Type | 1*N | [Bit7] Whether to enable this type of threshold detection [Bit6 - 0] Set threshold type: 0x01 heart rate, 0x02 systolic blood pressure (SBP) 0x03 diastolic blood pressure (DBP), 0x04 blood oxygen, 0x05 temperature (threshold unit is *10, for example, 37.3 is 373) |
| 2*N | I16*N | Threshold Min | 2*N | Lower threshold (this content is available only when threshold detection is enabled) |
| 2*N | I16*N | Threshold Max | 2*N | Upper threshold (this content is available only when threshold detection is enabled) |

Device alarm settings definition - Health threshold -- See 2.3 Device downlink description -- Health threshold setting (0 xCE0300)

Note: The default state is off, and the threshold is in the normal range. The device will report an alarm outside this range.

Heart rate threshold delivery range (20 - 200), systolic blood pressure threshold delivery range (20 - 200), diastolic blood pressure threshold delivery range (20 - 200), blood oxygen threshold delivery range (80 - 100), temperature threshold delivery range (160 - 600)

Example:

Set multiple health thresholds, 1. Heart rate [60, 120], 2. Turn off blood oxygen threshold detection, 3. Temperature [36.0, 37.3]:BDCE03000B00813C007800048568017501FF

BD:Header

CE: Message ID

03: Type 03 -- Device alarm settings

00: Valid 00 -- Health threshold, always valid

0 B00: Convert to big-end--> 000 B--> Convert to decimal 11 --> Subsequent command message byte length (excluding checksum) 11 bytes

81: Convert to binary -->10000001   bit7:1---On     On status   bit6-0:000001-->0x01  Heart rate

3C00: Convert to big-endian -->003C--> Convert to decimal60, the lower limit of the heart rate threshold is60

7800: Convert to big-endian -->0078--> Convert to decimal120, the upper limit of the heart rate threshold is120

04: Convert to binary -->00000100   bit7:0---Off, no threshold after closing   bit6-0:0000100-->0x04  Blood oxygen

85: Convert to binary -->10000101  bit7:1---On     On status   bit6-0:0000101-->0x05  Temperature

6801: Convert to big-endian -->0168--> Convert to decimal360(actual temperature *10)--> The lower limit of the temperature threshold is36.0

7501: Convert to big-endian -->0175--> Convert to decimal373(actual temperature *10)--> The upper limit of the temperature threshold is37.3

FF: checksum

5.1.6 Settings - Switch Settings (0XCE04-24)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xCE | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Please see the instructions for the specific type |
| 1 | u8 | switch | -/- | -/- | Switch 00 ON, 02 OFF, 01 valid for one time |
| 2 | U16 |  | -/- | -/- | Fixed 0000 |

Device Alarm Settings - Switch Type: Specific Function Description -- See 2.3 Device Downlink Description

05 Bluetooth Broadcast Switch        07 Drop Alarm Switch

08 Stay Alarm Switch        18 Sleep Function Switch

19 Key Trigger SOS Alarm Switch 24 GPS Always On Switch

Type=05 Bluetooth Broadcast Switch
Example:

00---ON:BDCE0500000093

BD: Header

CE: Message ID

05:Type 05 Bluetooth Broadcast Switch

00:Switch 00 ON

0000:Fixed value

93:checksum

02---Close:BDCE0502000093

BD: Header

CE: Message ID

05:Type 05 Bluetooth Broadcast Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=07 Fall Alarm Switch
Example:

00---Open:BDCE0700000093

BD: Header

CE: Message ID

07:Type 07 Fall Alarm Switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDCE0702000093

BD: Header

CE: Message ID

07:Type 07 Fall Alarm Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=08 Stay Alarm Switch
Example:

00---On  :  BDCE0800000093

BD: Header

CE: Message ID

08:Type 08 Stop Alarm Switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Off  :  BDCE0802000093

BD: Header

CE: Message ID

08:Type 08 Stop Alarm Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=18 Sleep Function Switch
Example:

00---On, enters sleep mode after being stationary for a period, does not report data  :  BDCE1800000093

BD: Header

CE: Message ID

18:Type 18 Sleep Function Switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Off, in a stationary state or charging state, also reports data normally : BDCE1802000093

BD: Header

CE: Message ID

18:Type 18 Sleep Function Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=19 Key Trigger SOS Alarm Switch
Example:

00---Open:BDCE1900000093

BD: Header

CE: Message ID

19: Type 19 key triggers SOS alarm switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDCE1902000093

BD: Header

CE: Message ID

19: Type 19 key triggers SOS alarm switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=24 GPS normally open switch
    Example:

00---Open:BDCE2400000093

BD: Header

CE: Message ID

24: Type 24 GPS normally open switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDCE2402000093

BD: Header

CE: Message ID

24: Type 24 GPS normally open switch

02:switch switch 02 off

0000:Fixed value

93:checksum

5.1.7 Settings - Drop Sensitivity Settings (0xCE15)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed as 0xBD | 0xCE | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Decription |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Type is 0x15 |
| 1 | U8 | Valid | 1 | Default to 0, no impact at present. |
| 2 | U16 | Len | 2 | Length of parameter content |
| 1 | U8 | Param Type | 1 | Parameter Type: 0x00: Sensitivity, 0x01: Height |
| 1 | U8 | Param Level | 1 | Parameter Level: 5 levels (0 - 4) |

Drop Sensitivity Setting - Parameter Description:

Sensitivity: Refers to the degree satisfying the drop algorithm trigger, providing 5 setting levels (0 - 4): Low - Lower (Medium-Low) - Medium - Higher (Medium-High) - High.

Height: Refers to the height required to satisfy the drop alarm trigger, providing 5 setting levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m - 2.5m.

If the height is set to "1.5m", the device must be at least ≥ 1.5m to potentially trigger the drop alarm.

Note that sensitivity and height cannot be combined in one message for issuance.

Example:

Set the sensitivity to "Medium-Low": BDCE150002000001FF

BD: Header

CE: Message ID

15:Type Drop Sensitivity Setting

00:Fixed Value

0200:Len

00:Param Type Sensitivity Setting

01:Param Level Sensitivity Level 1-->Lower (Medium-Low)

FF:checksum

Set the height to "1.5m": BDCE150002000102FF

BD: Header

CE: Message ID

15:Type Drop Sensitivity Setting

00:Fixed Value

0200:Len

01:Param Type Height Setting

02:Param Level Height Level 2-->1.5m

FF:checksum

5.1.8 Settings - Vibration Settings (0XCE23)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xCE | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Decription |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Fixed to 0x23 |
| 1 | U8 | Valid | 1 | 0x00 |
| 2 | U16 | Cfg Len | 2 | Length of parameter content |
| 1 | U8 | Motor Type | 1 | Setting type: 1. Vibration duration - 0x00 |
| 4 | U32 | Value | 4 | Setting content: 1. Vibration duration (in milliseconds, range 0 - 60 seconds) |

Setting - Vibration settings definition: This message can be used to modify the global vibration duration. Any generated vibration duration will change to the duration set by this command.

Example:

Set the device default global vibration duration to 1.5 seconds (1500 milliseconds): BDCE2300050000dc050000FF

BD: Header

CE: Message ID

23: Type 23 setting - Vibration settings

00: Fixed value 00 - always valid

0500:The length of the following parameter content is converted to big-endian 0005-->converted to decimal 05-->the length of the following parameter content is 5 bytes

00:Motor Type vibration duration setting

dc050000:Value vibration duration converted to big-endian 000005dc-->converted to decimal 1500-->vibration duration is 1500 milliseconds (1.5 seconds)

FF:checksum

5.1.9 Issue sedentary stay alarm trigger time (MSGID=0XCC)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed as 0xBD | 0xCC | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | minute | -/- | -/- | Stay alarm trigger duration (unit: minutes; value range 2 — 60), no activity within this time length will trigger a stay alarm |

Example:

No activity for 5 consecutive minutes The watch reports a stay alarm (when the watch is worn): BDCC05DD

BD:Header

CC:Message ID

05: minute Stay alarm trigger duration Convert decimal 05-->5 minutes

DD: checksum

5.1.10 Power Off and Restart (0x77)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0x77 | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- |  | 01—Power Off, 00—Restart |

Note: Both power off and restart are received while the device is on. Power Off: The command is received while the device is on, causing the watch to turn off. After power off, it cannot receive any commands; Restart: The command is received while the device is on, causing the watch to restart.

Example:

Power Off : BD770193

BD: Header

77 : Message ID

01 : Type 01--Power Off

93:checksum

Restart : BD770093

BD: Header

77 : Message ID

01 : Type 00--Restart

93:checksum

5.1.11 Restore Factory Settings (MSGID=0X7B)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0x7B | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | reset | -/- | -/- | 1 Restore Factory Settings |

Example:

Default factory settings: Default universal version firmware: 10-minute positioning report (all day), 10-minute sampling data report (all day), disable fall alarm, disable sedentary stay alarm,

Positioning priority is Bluetooth positioning >> GPS positioning; Text messages are set to null, personal information is set to null; Bluetooth broadcasting is enabled, OTAA-CLASSA; Re-enter the network after 3 consecutive failed reports, re-entry interval is 5 minutes, each transaction requires ack reply by default

Restore factory settings, device restarts after receiving the command:BD7B018F

BD: Header

7B:Message ID

01:reset 01--Restore factory settings

8F:checksum

5.1.12 LORA Parameter Settings (0xCF)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xCF | See below 见下方定义 | CK_sum |

Payload contents

| Byte size | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | count | -/- | -/- | Number of parameter groups sent |
| 1 | U8 | type | -/- | -/- | Parameter type |
| 1 | U8 | content len | -/- |  | Length of content |
| N | U8 | content | -/- | -/- | Parameter content |
| 1 | U8 | type | -/- | -/- | Parameter type |
| 1 | U8 | content len | -/- |  | Length of content |
| N | U8 | content | -/- | -/- | Parameter content |

type: Parameter type:

type=01 Working mode (Setting this parameter will cause the device to automatically restart)

| Byte size | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | Work mode | -/- | -/- | 00 – Class A, 01 – Class C |

type = 02 Join mode (Setting this parameter will cause the device to restart automatically)

| Byte size | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | Join mode | -/- | -/- | 00 – OTAA, 01 – ABP |

type = 03 Reporting mode (Setting this parameter will cause the device to restart automatically)

| Byte size | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | Report mode | -/- | -/- | 00 – confirm (requires ack reply), 01 – unconfirm (no ack reply required) |

type = 04 Frequency switch (only for high-frequency module) (Setting this parameter will cause the device to restart automatically)

| Byte size | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | C-band | -/- | -/- | 00 — Default Frequency Band 01 — EU868 02 — US915 03 — AU915 04 — AS923_1 05 — AS923_2 06 — KR920 07 — IN865 |

In the reporting mode of type = 05 confirm, not every message receives an ACK; you can set the interval time (in seconds).

| Byte size | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 4 |
| 1 | U32 | Min_interval | -/- | -/- | Interval time (seconds/minutes) 00— Each message needs to receive an ACK, 1~18000 – Interval time between messages receiving ACK during routine reporting. |

type = 06 Continuous reporting failure count in OTAA network mode.

| Byte size | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | failure count | -/- | -/- | In OTAA network mode, the continuous reporting failure must reach a specified count to restart the networking process (range 0-20). |

type = 07 Fixed rate; after this command is set, ADR is disabled by default (the device will automatically restart after this parameter is set).

| Byte size | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | cdata_rate | -/- | -/- | 00 — SF12, 01 — SF11, 02 — SF10, 03 — SF9, 04 — SF8, 05 — SF7 |

type = 08 ADR enable (the device will automatically restart after this parameter is set)

| Byte size | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | adr switch |  |  | 01 — ADR Enable, 00 — ADR Disable |

type = 09 Set frequency group (channel) (the device will automatically restart after this parameter is set)

| Byte size | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | channel |  |  | 00: 0 ~ 7 channels, 8: 8 ~ 15 channels, 16: 16 ~ 23 channels,...Note: 0xFF: Restore default frequency group |

Type = 10 In OTAA mode, the restart module waits for an interval (in minutes) - re-access time

| Byte size | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Len |  |  | Length fixed at 1 |
| 1 | U8 | Minutes |  |  | Restart the module's interval in minutes: 0 ~ 120 |

Example:

A total of 3 sets of parameters were issued: classA mode, ABP network access, unconfirm report, ADR not enabled

Notice: After modifying the downlink, promptly adjust the lora ns configuration and match the parameters to ensure normal communication BDCF04010100020101030101080100dd

BD: Header

CF:Message ID

04: 04--indicates that there are 4 types of parameters being issued

01: type type=01 working mode

01: Len parameter length 01 converted to decimal 01, length is 1

00:  00--class A mode

02: type type=02 network joining mode

01: Len parameter length 01 converted to decimal 01, length is 1

01  01--ABP network joining mode

03: type type=03 reporting mode

01: Len parameter length 01 converted to decimal 01, length is 1

01: 01--unconfirm reporting

08: type type=08 ADR enabled

01: Len parameter length 01 converted to decimal 01, length is 1

00: 00--ADR not enabled

DD:checksum

Issue 1 group Every 20 minutes, a message requiring ACK reply: BDCF010504B0040000DD

BD: Header

CF:Message ID

01: 01--Indicates that there is 1 type of parameter issued

05: type type=05 In the confirm reporting mode, ACK is not obtained for every message, set the interval time

04: Len Parameter length 04 converted to decimal 04, the length is 4 bytes

B0040000:Min_interval converted to big endian 000004B0-->1200 seconds

DD:checksum

5.1.13 Weather warning (0XCB) - high-frequency version has, low-frequency version does not have this function

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD | 0xCB | See below 见下方定义 | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Count | -/- | -/- | How many groups of weather warnings are there? |
| 1 | U8 | Type | -/- | -/- | Weather Warning Type, for specific types see below |
| 1 | U8 | Status | -/- | -/- | Status, for specific status see below |
| … | … | …… |  |  | … |

Weather Type - Status Table:

| Type | Decription | Status | notes - Click on the image to enlarge |
| --- | --- | --- | --- |
| 1 | Storm Warning | 0 | Cancel Alert |
|  |  | 1 - 8 |  |
| 2 | Heavy Rain Warning | 0 | Cancel Alert |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |
| 3 | Thunderstorm Warning | 0 | Cancel Alert |
|  |  | 1 |  |
| 4 | Flood Warning | 0 | Cancel Alert |
|  |  | 1 |  |
| 5 | Debris Flow Warning | 0 | Cancel Alert |
|  |  | 1 |  |
| 6 | Strong Monsoon Warning | 0 | Cancel Alert |
|  |  | 1 |  |
| 7 | Frost Warning | 0 | Cancel Alert |
|  |  | 1 |  |
| 8 | Fire Danger Warning | 0 | Cancel Alert |
|  |  | 1 |  |
|  |  | 2 |  |
| 9 | Temperature Warning | 0 | Cancel Alert |
|  |  | 1 |  |
|  |  | 2 |  |
| 10 | Tsunaami Warning | 0 | Cancel Alert |
|  |  | 1 |  |
| 11 | Heat Index | 0 | Cancel Alert |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |

Note: After issuing, the device will vibrate and display weather warning images. Multiple sets of weather warnings can be issued simultaneously, and each set must be distinct. The same type of warning cannot appear in a single instruction; otherwise, it will be invalid. Example:

10Weather warning issued by group: BD CB 0A 0101 0201 0301 0401 0501 0601 0701 0801 0901 0A01 F5

BD:Header

CB:Message ID

0A:Count weather group count in decimal10--> Distributing10weather groups

0101:Type&Status weather type-status 01--Storm Warning 01--Signal No. 1

0201:Type&Status weather type-status 02--Rainstorm Warning 01--Yellow Rainstorm Warning Signal

0301:Type&Status weather type-status 03--Thunderstorm Warning 01--Thunderstorm Warning

0401:Type&Status weather type-status 04--Flood Warning 01--Special Report on Flooding in Northern New Territories

0501:Type&Status weather type-status 05--Landslide Warning 01--Landslide Warning

0601:Type&Status weather type-status 06--Strong Monsoon Warning 01--Strong Monsoon Signal

0701:Type&Status weather type-status 07--Frost Warning 01--Frost Signal

0801:Type&Status weather type-status 08--Fire Danger Warning 01--Yellow Fire Danger Warning

0901:Type&Status Weather Type-Status 09--Temperature Warning 01--Cold Weather Warning

0A01:Type&Status Weather Type-Status 10--Tsunami Warning 01--Tsunami Warning

F5:checksum

5.2 Server Time Synchronization Information

5.2.1 Device Reports Request Time Calibration Data

Explanation: This message is reported by the watch

After the device is powered on, the watch will automatically send a request time synchronization command. After receiving the synchronization command, it is necessary to reply to the synchronization time data frame for device time synchronization
If the device has been calibrated downstream before shutting down, then after the device restarts, the device interface displays the time before shutting down, and the device will report FF00FF to re-request time calibration

| Byte offset | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | U8 | HEADER | Padding: 0xFF |
| 1 | U8 | SeqId | Padding: 0x00 |
| 1 | U8 | End | Padding: 0xFF |

Example:

FF00FF    // The server must reply immediately upon receipt, without delay

5.2.2 Time Calibration Request Data Reply

| Byte offset | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | U8 | HEADER | Padding: 0xFF |
| 1 | U8 | SeqId | Padding: 0x10 |
| 2 | U16 | years | Year data |
| 1 | U8 | month | Month data |
| 1 | U8 | Day | Date, data |
| 1 | U8 | time | Clock, data |
| 1 | U8 | Minute | Minutes, data |
| 1 | U8 | Seconds | Seconds, data |
| 1 | U8 | End | Padding: 0xFF |

Example:

The server replies with a downlink command after receiving FF00FF, calibrating the time. Note: The example is for the year 2020, only for structural analysis. The current year must be issued; otherwise, FF1007E409020B1B28FF will not be parsed.

FF : Header

10 : Seqid

07E4: Year (2020)

09 : Month，（09）

02 : Day，（02）

0B : Hour，（11）

1B : Minutes，（27）

28 : Second，（40）

FF : End