W300G-4GCat.1-tcp communication protocol

Directory

1 Overview	4

2 Device Instructions	4

2.1 Device Function and Instructions	4

2.2 Device default reporting logic	7

2.3 Device downlink description	9

3 Protocol Data Packet Structure	14

3.1 Data Header	14

3.2 Message Identifier (Message ID)	14

3.3 Token generation mechanism	15

3.4 Payload	15

3.5 Checksum	16

4 Report messages	18

4.1 Connection related reports	18

4.1.1 LNK-LIN (MSGID=0xF0) Request connection (TCP only)	18

4.1.2 Connection Reply (MSGID=0xF1) (TCP Only) - Important	19

4.1.3 New Heartbeat Packet Protocol (MSGID=0xF9)-Important	20

4.2 Alarm-related reporting	22

4.2.1 Alarm data upload - 1 (MSGID=0x02)	22

4.2.2 Alarm data upload-2 (MSGID=0x21) (Supplement to 02)	26

4.2.3 Abnormal Data Alarm (MSGID=0x16)	31

4.3 Location-related reporting	33

4.3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)	33

4.3.2 Upload of WiFi and base station information (MSGID=0xA4)	35

4.3.3 Bluetooth Location Information (BLE Location) (MsgId=0xD6)	40

4.3.4 Base station latitude and longitude reporting (MSGID=0x15) — wifi positioning supplement	43

4.4 Device Information and Status Reporting	45

4.4.1 SIM card ICCID upload (MSGID=0xF3)	45

4.4.2 Device charging status upload (MSGID=0xC3)	46

4.4.3 Status parameter reporting (MSGID=0xA9)	48

4.4.4 Device Status (MSGID=0xE9)	49

4.  5 Health-related reporting	51

4.  5.1 Health data (MSGID=0x32)	51

4.5.2 Device Sleep Analysis Data Upload (MSGID=0xC5)	54

4.6 Downlink Feedback Related Reporting	55

4.6.1 Downlink Feedback (MSGID=0xC0)	55

4.6.2 Message Status Report (MSGID=0x28)	56

4.7 NFC data related report - hardware support is required, this function is not available in the default general version	57

4.7.1 Check-in NFC Data Reporting (MSGID=0xDC)	57

5 Settings	58

5.1 Downlink	58

5.1.1 Set location reporting frequency (MSGID=0x17)	58

5.1.2 Message Send (MSGID=0X28)	61

5.1.3 Settings - Location Priority Setting (0XCE01)	62

5.1.4 Settings - Health sampling report frequency setting (MSGID=0X1C)	65

5.1.5 Settings - Device Alarm Settings (0XCE03)	68

5.1.6 Settings - Switch Settings (0XCE04-24)	70

5.1.7 Setting - Drop Sensitivity Setting (0xCE15)	80

5.1.8 Setting - Vibration Setting (0XCE23)	82

5.1.9 Settings - Interface Display and Hide Settings (0xCE27)	83

5.1.10 IP & Domain Name Settings (MSGID=0xC3) (TCP Only)	85

5.1.11 Issue sedentary stay alarm trigger time (MSGID=0XCC)	87

5.1.12 Shutdown/Restart (MSGID=0x77)	87

5.1.13 Personal Information Issuance (MSGID=0xCA)	88

5.1.14 Sleep Statistics Time Period Setting (MSGID=0X1D)	90

5.1.15 Remote OTA upgrade distribution (MSGID=0xA9)	93

5.1.16 Weather warning (MSGID=0XCB)	95

5.1.17 Restore Factory Settings (MSGID=0X7B)	99

5.1.18 Special Firmware Support - Sending SOS Prompt Message (MSGID=0X80)	99

5.1.19 Special Firmware Support - Issue Text Message Cleanup Time (MSGID=0X2A)	101

1 Overview

This protocol is suitable for 4G CAT1 devices and supports W300G products. It uses a 32-bit data header for synchronization and terminal identification; a low-overhead checksum algorithm for checksum protection; and a message identifier to identify different messages.
It needs to be parsed according to the actual report. The document is currently being continuously maintained. If you find any errors or omissions, please give feedback in time. Thank you very much.
The protocol content is 4g reporting content. If you need the Bluetooth broadcast data protocol, please consult relevant personnel.

2 Device Instructions

2.1 Device Function and Instructions

General version:

(1) Power on:

Please fully charge the device before using it for the first time. The device will automatically power on when the charging reaches the power required for power on. The charging status displays the charging icon.

A full charge is indicated by a green icon. Note: Do not check the device signal while charging.

Manual power on: Press and hold the up key for 10 seconds and release. The interface will display "Welcome".

Note: The default power-on state is the wearing state, and a detachment alarm is reported if no heart rate is detected.

(2) Shutdown:

Low battery shutdown: The screen turns off after the interface displays Byebye.

Manual shutdown: Shutdown: Press and hold the button for more than 10 seconds, the interface displays Byebye and then the screen turns off.

Downlink command shutdown: The downlink shutdown command is issued in the power-on state, the interface displays Byebye and then the screen turns off.

Note: Charging in the power-on state does not shut down.

(3) SOS function:

Trigger mode: After triggering, the device does not enter sleep mode. Long press the down key for 3s, the interface displays SOS SEND OK / SOS sending successfully, and then the SOS icon appears.

Cancel mode: In SOS mode, long press the down key for 3s, the interface displays SEND CENCEL / SOS canceled, and the SOS icon disappears.

(4) Signal status:

No signal: The interface signal is a horizontal line. Signal available: The interface signal has a stepped bar graph.

(5) Device sleep:

Trigger condition: The device does not move for 40 minutes and enters sleep mode, does not report location health data, and only reports F9 heartbeat packets to maintain a long connection.

(6) Ultra-long standby mode:

Manual change: Switch the setting - ultra-long standby mode, click Enable, the device will switch to ultra-long standby mode. Remote command issued: Issue a switch to ultra-long standby mode, the device will switch to ultra-long standby mode. Effect: There is a yellow bar below the battery icon on the time interface, short connection, health and location are checked every 10 minutes,

When the health data is normal and the location has not changed, the data is reported every 1 hour; otherwise, the data is reported every 10 minutes.

(7) Device parameters: Settings - Parameters interface: There are multiple pages, which can be switched by sliding the screen down

sos(Yes): Yes: Indicates that the device can be triggered by pressing the buttonsos,

sos(No): Indicates that it cannot be triggered by pressing the buttonsos

Health(10min): Indicates that the device has enabled the health report function,

10min: Indicates that the current health data reporting frequency is10minutes,

Health(No): Indicates that the device health report function has been turned off

Location(BWG,10min): Indicates that the device has enabled the location report function,

BWG: Indicates the location priority Bluetooth\>wifi>Satellite (GPS/BDS),

10min: Indicates that the current location reporting frequency is10minutes,

Location(No): Indicates that the device location report function has been turned off

Fall (Medium,1.5m): Indicates that the device has enabled the fall alarm trigger function,

Medium: Indicates that the device's fall alarm trigger sensitivity is medium.

1.5m: Indicates that the device's fall alarm trigger height is 1.5m.

Fall (No): Indicates that the device has disabled the fall alarm trigger function.

Sedentary (2min): Indicates that the device has enabled the sedentary stay alarm trigger function.

2min: Indicates that the time to trigger the sedentary stay alarm is 2 minutes of inactivity.

Sedentary (No): Indicates that the device has disabled the sedentary stay alarm trigger function.

Health Broadcast (Yes):

Yes: Indicates that the device has enabled Bluetooth broadcasting of health data.

No: Indicates that the device has disabled Bluetooth broadcasting of health data.

Sleep statistics reporting time: (21:00-08:00), indicating that the sleep statistics reporting time is from 9 PM to 8 AM the next day.

Global vibration duration (1.500s): The current device vibration duration is1.5seconds

Health alarm threshold: such as heart rate threshold (90,100)BPM, indicating that the normal heart rate range is90-100,

Normal range will report health abnormal alarm, heart rate threshold (No): indicates that the heart rate threshold is closed

Allow work hibernation (Yes):

Yes: Indicates that the device has turned on the sleep function, No: Indicates that the device has turned off the sleep function

Note: Sleep function refers to the device falling off state does not report data, the device 40minutes stationary does not report data

Network long connection (Yes):

Yes: Indicates that the current device tcp connection mode: long connection (always maintain the connection state, can receive downlink instructions in real time),

No: Indicates short connection (disconnect after reporting, the device accepts downlink instructions when reporting data) GPSAlways on switch (No):

Yes: The internal interface is always open to collect Satellite (GPS/BDS), which can generally speed up Satellite (GPS/BDS) positioning time in a normal environment, and the power consumption will increase,

The default is No, and it does not need to be turned on in a normal environment

(8) Vibration:

Receive the downlink text message, health threshold alarm (the interface has text reminders and vibration). The vibration duration can be controlled by the downlink (see section 2.3)

2.2 Device default reporting logic

General version:

(1) Connection related reporting

F0 request: The device is a long connection. Under normal server connection and network conditions, it will be reported once at startup -- a reply is required to connect to the server

F9: Heartbeat package reporting, default 4 minutes reporting once, (previous firmware) location and health reporting will also follow a report -- a reply is required to maintain a long connection

(2) Location related reporting

Satellite (GPS/BDS)/wifi/ Bluetooth beacon: Default reporting frequency 10 minutes, default positioning priority: wifi>Satellite (GPS/BDS), wifi positioning is preferred, and Satellite (GPS/BDS) positioning is switched if positioning is not possible

Base station(0x15): The communication base station latitude and longitude obtained by automatically requesting the module after wifi positioning fails. The accuracy is not high and is used for auxiliary reference. This report is generally not available in the environment and cannot be issued to bind the positioning priority and wifi positioning

(3) Alarm related reporting

SOS Alarm (0x02): User actively triggers, see the previous section for triggering methods

SOS Cancel (0x02): The user actively triggers the cancellation. See the previous section for the triggering method.

Shutdown Alarm (0x21): The device actively shuts down or shuts down due to low battery. See the previous section for the triggering method.

Wearing Drop Alarm (0x02): The device determines wearing based on the health sampling and reporting frequency. It reports a wearing alarm when a heart rate is detected, and a drop alarm when no heart rate is detected.

Sedentary Stay Alarm (0x02): Default15minutes of inactivity triggers reporting.

Fall Alarm (0x02): The device falls freely from a certain height and meets the fall algorithm trigger.

Low Battery Alarm (0x02): Triggered when the current battery level of the device is less than or equal to0.

Abnormal Temperature Alarm (0x16): Reported when the device detects a temperature exceeding40degrees Celsius.

Health Threshold Alarm (0x2105): No alarm by default. After the health threshold is set downstream, an alarm is reported when the health data is not within the threshold range. Note: When the health threshold alarm is reported, the device vibrates and the interface displays abnormal values.

(4) Health-related reporting

Step count, heart rate, body temperature&wrist temperature, blood pressure, blood oxygen (0x32): Default reporting frequency10minutes

Sleep ( 0xC5 ): Statistical time period 21 : 00-08 : 00 , during which sleep data will be reported based on the status

(5) Device information and status reporting

Status parameter ( 0xA9 ): A record will be reported when the device is turned on

SIM card ICCID ( 0xF3 ): A record will be reported when the device is turned on

Device status ( 0xE9 ): A record will be reported when the device is turned on, and a record will be reported when the reporting frequency changes

Charging status ( 0xC3 ): Report when charging starts, charging ends, and when fully charged

(6) Downlink feedback

Downlink feedback (0xC0): The device reports after receiving the downlink command from the server

Message status feedback (0x28): After the downlink text message, the device clicks accept or reject

(7) NFC-related data reporting—Note that this item requires watch hardware support, and there is no such reporting in the default general version

Check-in NFC Data Reporting (0xDC): Check-in NFC tag reporting, used to read other NFC tags on the check-in interface and obtain UID for reporting

Note: The device reports in packet reporting, which means that one data packet contains multiple complete messages. Please pay attention not to miss any. The messages are complete and will not be interrupted in the middle and appear in the next data packet.

Eg:BDBDBDBDD6000119A9CF610445270387BF452708A1BC44279D18B74427E518B7F9BDBDBDBDF9010000006400002800000019A9CF61CA

This data packet contains the (0xD6) Bluetooth positioning and (0xF9) power signal messages.

(0xD6) Bluetooth positioning: BDBDBDBDD6000119A9CF610445270387BF452708A1BC44279D18B74427E518B7F9

(0xF9) Power signal: BDBDBDBDF9010000006400002800000019A9CF61CA

2.3 Device downlink description

General version: The downlink command needs to be continuously sent twice within 20 milliseconds to ensure success.

(1) Device location reporting frequency distribution (0x17):

The default reporting frequency is 10 minutes, the highest is 1 minute, and the lowest is 1440 minutes. After the device receives the downlink command, the device presses the time period and frequency of the distribution command.

Report data, no report outside the time period, such as: 00:00-18:00 2 minutes positioning report, then no report outside the time period

(2) Text message delivery (0x28):

Unicode encoding, up to 40 Chinese characters, one Chinese character occupies 2 bytes, and one English letter occupies 2 bytes

Message list - The history message can save up to 50 messages (Note: The firmware without message list can save up to 20 messages)

(3) Device positioning priority delivery (0xCE01):

The default positioning priority is wifi>gps, positioning priority

If the delivery positioning priority is: wifi>gps>Bluetooth beacon, then if wifi cannot be located, switch to gps, and if gps cannot be located, switch to

Bluetooth beacon, when the positioning is successful, the next positioning priority will not be switched to generate positioning

(4) Device health sampling frequency delivery (0x1C):—Note that it is different from W200P

The default health sampling frequency is 10 minutes, and the maximum is 2 minutes. After the device receives the reported downlink command, the device presses the issued command.

Report data according to the time period and frequency, and do not report outside the time period, such as: 00:00-18:00 10 minutes positioning report, then no report outside the time period

(5) Modification of IP and port command issuance (0xC3):

The default universal version points to the Smart Cloud Platform: 118.178.184.219:8825. If you need to change it, please consult relevant personnel or visit the official website.

(6) Sedentary Stay Alarm Trigger Time Issuance (0xCC):

Value range: 2 minutes - 60 minutes, for example: if the downlink is a 10-minute trigger time, the device will trigger and report a sedentary stay alarm if it does not move for 10 minutes.

(7) Device Shutdown and Restart Issuance (0x77):

Note that this command must be in effect when the device is turned on, and cannot be received after shutdown.

Shutdown Issuance: When the device is turned on, the device is shut down, and the downlink cannot be received after shutdown.

Restart Issuance: When the device is turned on, the device restarts.

(8) Fall Sensitivity and Height Issuance (0xCE15):

The latest universal firmware defaults to fall sensitivity: Low (Medium Low). Fall height: 1.5m

Sensitivity: Refers to the degree to which the fall algorithm is satisfied, providing 5 setting levels (0 - 4): Low - Lower (Medium Low) - Medium - Higher (Medium High) - High.

Height: Refers to the height that triggers the fall alarm, providing 5 setting levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m- 2.5m.

(9) Remote OTA Delivery (0xA9):

Remote updates consume traffic and are affected by the network. The device's SIM card has 30M per month, so it cannot be updated too many times.

Otherwise, it will cause insufficient traffic. This function requires device hardware to support remote updates. Previous devices do not support it.

(10) Key Shutdown Switch Command Delivery (0xCE16):

By default, the device can be shut down via a key. After the downlink is closed, the device cannot be shut down via a key.

(11) Control Device to Trigger Sleep Switch Delivery (0xCE18):

The default is the on state. The device enters sleep mode after 40 minutes of inactivity. After the downlink is closed, the device does not enter sleep mode.

(12) Fall Alarm Switch (0xCE07):

The default fall alarm is in the on state. After it is turned off, the device does not report fall alarms.

(13) Key trigger SOS start switch (0xCE19)

The default state is on. A long press of the charging cable button can trigger SOS. When turned off, a long press of the charging cable button will not trigger SOS reporting.

(14) Device Health and Location Reporting Switch (0xCE20)

The default health reporting is on, and the default location reporting is on.

You can set the location / health on and off states separately. When turned off, the device will not report health / location.

(15) Long Connection and Short Connection Mode Switching (0xCE22)

The default is long connection. You can send a downlink command to switch between long and short connections. The device will restart after receiving the command and switch to long/short connection.

Note that the long connection mode consumes slightly more power than the short connection mode. By default, a heartbeat packet (0xF9) is reported every 4 minutes.

(16) Health Threshold Setting (0xCE0300)

Health threshold is issued, downlink command. After the device receives it, the normal range of the health threshold is issued. Health data outside this range will report an alarm.

(17) Device Vibration Setting (0xCE23)

After the downlink command, the device receives it, and the device vibration duration becomes the downlink duration, ranging from 0-60 seconds.

(18) Report location immediately (0xCE0A)

After the downlink command is sent, the device will immediately start locating and reporting after receiving it.

(19) Report health immediately (0xCE0B)

After the downlink command is sent, the device will immediately start health sampling and reporting after receiving it.

(20) Sleep statistics time period distribution (0x1D)

Different time periods can be distributed, such as: 13:00-14:00, then the sleep statistics data will be counted and reported within this time period.

(21) GPS always-on switch (0xCE24):

After it is turned on, the internal interface is always on to collect GPS. In general environments, it can speed up GPS positioning time, but the power consumption will increase. The default is off.

(22) Switch to ultra-long standby mode (0xCE17):

After it is turned on, the device switches to ultra-long standby mode, and there is a yellow bar below the power icon on the time interface, short connection,

Health and location are checked every 10 minutes. If the health data is normal and the location has not changed, the data is reported every 1 hour; otherwise, the data is reported every 10 minutes.

(23) Personal Information Issuance (0xCA)

There is no personal information by default. Chinese name, English name, blood type, contact number, GB2312 encoding can be issued.

Maximum number of bytes: Name 40 bytes, English name 40 bytes, phone number 40 bytes, blood type 10 bytes

(24) Sedentary Stay Alarm Switch (0xCE08)

It is enabled by default, and the trigger time is 15 minutes. After the downlink is closed, the device does not report the sedentary stay alarm.

(25) Weather Warning Issuance (0xCB)

Multiple sets of weather warnings can be issued. Note that each set of weather warnings must be different, and the same type of warning cannot appear.

(26) Data Reporting Switch (0xCE09):

Enable and disable data reporting. The default is enabled. After it is disabled, the device only reports 0xF9 packets to maintain a long connection.

(27) Restore Factory Settings (0x7B):

After the command is issued, the device will revert to the default reporting status. See section 5.1.12 for the default status.

3 Protocol Data Packet Structure

A basic protocol data packet structure is shown in Figure 1:

3.1 Data Header

Each data packet starts with a 4-byte Header or token (in some response messages, timestamp is used instead):

Currently, the token uploaded by Oufu devices is fixed as BDBDBDBD

Header: 0xBD0xBD0xBD0xBD；

Timestamp: 32bits, generated by the server

3.2 Message Identifier (Message ID)

See Chapter 4 for the content represented by MessgeId.

Each time a TCP connection is made, the device will first report a 0xF0 message, which contains the device's unique identifier IMEI.

The server needs to record the IMEI as an identifier and reply with a 0xf1 message. Only then will the device consider the connection successful; otherwise, the connection will be disconnected.

During normal device communication, the long connection reports a 0xF9 heartbeat packet every 4 minutes by default. The 0xF9 heartbeat packet is reported once after the health location is reported.

For short connections, the 0xF9 heartbeat packet is reported once at startup, and the 0xF9 heartbeat packet is reported once after the health location is reported. The reporting of the two is not affected by each other.

Note that after the server receives the F9 message, the server needs to issue a command to reply; otherwise, the connection will be disconnected.

3.3 Token generation mechanism

Currently fixed as BDBDBDBD

3.4 Payload

The payload below refers to the valid body content in the protocol, excluding the head token and checksum. The length of the body is noted after.

The data format used in the payload is shown in the table below:

【U-unsigned；I-signed；X-bitfield；Number-bytes occupied】

The following protocols use little-endian except for ch, u8, i8, and x8

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Description |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short Integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short Integer |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Integer |
| x16 | Bitfield | 2 | - | - | Bit (bit) 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long Integer |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long Integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit Long Integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating point type |

3.5 Checksum

The content added to the checksum includes the payload, as shown in Figure 1. The algorithm is as follows: the default general version of the device does not require checksum, and this part can be ignored. Any byte can be used for the downlink instruction.

//Assuming content="BDBDBDBDE9000600010A00000A00", then the result of sum.ToString("X2") is 07

//The following is the C# code calling method

private string CheckSum(string content){

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2"); }

4 Report messages

4.1 Connection related reports

4.1.1 LNK-LIN (MSGID=0xF0) Request connection (TCP only)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xF0 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | u64 | IMEI | 1 | - | IMEI number (little endian) |
| 2 | x16 | version |  | - | Bitfield see below |

The device's reported F0 request message must have a server downlink 0XF1 message (see the next section for the specific format) reply, otherwise the login will fail

Example:

Original16Hexadecimal message: BDBDBDBDF09B51731BC61603000000C2

BDBDBDBD:4Bytes Header

F0:Message ID

9B51731BC6160300：IMEI number (little endian), converted to big endian: 000316 C61B73519B,

The corresponding decimal is IMEI: 869465050010011 , 0000 : version

C2: checksum

Connection related Q&A:

Each time a new connection is created via TCP, an F0 request is reported first. The F0 contains the IMEI, and then the server records this IMEI.

After that, all the data in this connection belongs to this IMEI.

Note: If F0 is not received or F0 communication is abnormal, you can use a third-party network testing tool to verify whether the server communication is normal.

4.1.2 Connection Reply (MSGID=0xF1) (TCP Only) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Timestamp(unix) | 0xF1 | See below for definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | u32 | Token | 1 | - | Token: Fixed as BDBDBDBD |

Note that the message sequence is different from the normal reporting message. This message is issued by the server, not the content reported by the device. The server reply needs to be replied in the current channel. > Note that the message sequence is different from the normal reporting message. This message is issued by the server, not the content reported by the device. The server reply needs to be replied in the current channel.

Example:

Original16Hex message:07FD8860F1BDBDBDBD2E

07FD8860(Little endian),Timestamp(unix) timestamp (unit: seconds), converted to big endian6088FD07 Converted to decimal:1619590407

Convert to standard time formatUTCTime:2021-04-28 06:13:27

F1:Message ID(Message ID)

BDBDBDBD:Token

2E:checksumchecksum

Timestamp10decimal to standard time format is the result of adding1619590407seconds to 1970-01-0100:00:00. The device uses the timestamp (unit: seconds) in this message to synchronize time.

Note: Injava, the unit is generally milliseconds. Pay attention to converting to the unit: seconds.

Note: The total number of bytes replied by the server is10bytes, such as:0xBD--1byte; if the device receives20bytes, the connection will not be established.

4.1.3 New Heartbeat Packet Protocol (MSGID=0xF9)-Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xF9 | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery type 0: 4-level system, 1: 5-level system, 2: percentage, 3: voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery level If Bat_type is 0: The battery level range is 0-3, (0 is 25%, 3 is 100%) If Bat_type is 1: The battery level range is 0-4 (0 is 20%, 4 is 100%) If Bat_type is 2: The battery level range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: Percentage, 1: 5-level system 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal strength |
| 1 | U8 | Other_type |  |  | Extended type 0: Full amount of steps 1: Incremental steps 2: Vibration |
| 4 | U32 | Num |  |  | Extended Value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Note: The F9 heartbeat packet must have a server downlink reply. The device side considers the server connection to be unbroken only after receiving the server downlink reply; otherwise, it will re-report the F0 request connection.

Example:

Original hexadecimal message: BDBDBDBDF90104000050000095000000E377BD67AA

BDBDBDBD: Header

F9: Message ID

01: Bat_type battery type 01 --> 1 --> 5 level battery (0-4)

0400: Bat_volt battery value converted to big-endian 0004 --> 4 --> level 4 corresponds to 100% battery

00:Signal_type signal type 00-->0-->percentage

5000:Signal_strength Signal strength converted to big-endian0050--> Convert to decimal80-->80% Signal strength

00:Other_type Extended type00-->0--> Full step counting

95000000:Num Extended value - step counting converted to big-endian00000095--> Convert to decimal149--> Number of steps is149

E377BD67:Timestamp Converted to big-endian67BD77E3--> Convert to10decimal1740470243--> Timestamp is1740470243seconds --> Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

AA:checksum

Note: The server can reply with a fixed message upon receiving the F9 heartbeat packet: Fixed reply example BDBDBDBDF301 Number of bytes:6

4.2 Alarm-related reporting

4.2.1 Alarm data upload - 1 (MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x02 | See below for definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below for definition |
| 4 | U32 | Timestamp | - | - | Timestamp (a timestamp will be added after it when re-uploading) |

Upl_warn contents

| bit (corresponds to the position of 1 in binary) | Name | Description | Original message little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 14 | Fall alarm | Fall alarm | 0040 | 4000 | 4*4096=16384 |
| 8 | Device Wearing | Device Wearing | 0001 | 0100 | 1*256=256 |
| 7 | SOS Cancel | SOS Cancel | 8000 | 0080 | 8*16=128 |
| 5 | Sedentary Alarm (No Movement) | Sedentary Alarm (No Movement) | 2000 | 0020 | 2*16=32 |
| 4 | Remove (Fall Off) Device | Remove (Fall Off) Device | 1000 | 0010 | 1*16=16 |
| 2 | Shutdown | Shutdown | 0400 | 0004 | 4 |
| 1 | SOS Alarm | SOS Alarm | 0200 | 0002 | 2 |
| 0 | Low Battery | Low Battery | 0100 | 0001 | 1 |

Example:

Wearing alarm: BDBDBDBD020001E377BD678A

BDBDBDBD: Header

02: Message ID

0001: Upl_warn alarm content, converted to big-endian: 0100 --> Wearing alarm

E377BD67: Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

8a: checksum

Fall off alarm: BDBDBDBD021000E377BD678A

BDBDBDBD: Header

02: Message ID

1000: Upl_warn alarm content, converted to big-endian 0010 --> Fall off alarm

E377BD67: Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

8a: checksum

SOS alarm: BDBDBDBD020200E377BD678A

BDBDBDBD: Header

02: Message ID

0200:Upl_warn alarm content, converted to big-endian0002-->SOS alarm

E377BD67: Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

8a: checksum

SOS Cancellation: BDBDBDBD028000E377BD678A

BDBDBDBD: Header

02: Message ID

8000:Upl_warn alarm content, converted to big-endian0080-->SOS Cancellation

E377BD67: Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

8a: checksum

Fall Alarm: BDBDBDBD020040E377BD678A

BDBDBDBD: Header

02: Message ID

0040:Upl_warn alarm content, converted to big-endian4000-->Fall Alarm

E377BD67: Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

E7: checksum

Sedentary Alarm: BDBDBDBD022000E377BD678A

BDBDBDBD: Header

02: Message ID

2000:Upl_warn alarm content, converted to big-endian0020-->Sedentary Alarm

E377BD67: Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

8A: checksum

Low Battery Alarm: BDBDBDBD020200E377BD678A

BDBDBDBD: Header

02: Message ID

0100:Upl_warn alarm content, converted to big-endian0001-->Low Battery Alarm

E377BD67: Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

8A: checksum

Shutdown alarm: BDBDBDBD020400E377BD678A

BDBDBDBD: Header

02: Message ID

0400:Upl_warn alarm content, converted to big-endian0004-->Shutdown alarm

E377BD67: Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

8A: checksum

Note: This alarm is used for devices that do not distinguish between shutdown types. See the next section for devices that distinguish between shutdown types.

4.2.2 Alarm data upload-2 (MSGID=0x21) (Supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x21 | See below for definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | Alarm type |
| N |  | Upl_warn | - | - | Alarm content See below for different Alarm type type definitions |
| 4 | U32 | Timestamp |  |  | Timestamp (a timestamp will be added after it when re-uploading) |

Alarm type=1

Upl_warn contents

| bit (corresponds to the position of 1 in binary) | Name | Description | Original message little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging shutdown alarm | Device charging automatic shutdown (firmware requires function support) | 04000000 | 0004 | 4 |
| 1 | Low battery shutdown alarm | Device shuts down due to low battery (firmware requires function support) | 02000000 | 0002 | 2 |
| 0 | Active shutdown alarm | Device is manually shut down by someone (firmware requires function support) | 01000000 | 0001 | 1 |

Example:

Charging shutdown alarm: BDBDBDBD21010004000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

04000000:Upl_warn alarm content, converted to big-endian 0004-->charging shutdown alarm

E377BD67:Timestamp converts the timestamp to big-endian 67BD77E3-->converts to decimal 1740470243-->the timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Low battery shutdown alarm: BDBDBDBD21010002000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

02000000:Upl_warn alarm content, converted to big-endian 0002-->Low battery shutdown alarm

E377BD67:Timestamp converts the timestamp to big-endian 67BD77E3-->converts to decimal 1740470243-->the timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Active shutdown alarm: BDBDBDBD21010001000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type Alarm type converted to big-endian 0001-->Alarm type=1

01000000:Upl_warn alarm content, converted to big-endian 0001-->Active shutdown alarm

E377BD67:Timestamp converts the timestamp to big-endian 67BD77E3-->converts to decimal 1740470243-->the timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Alarm type=5

Upl_warn contents

| Format | Name | Unit | Decription |
| --- | --- | --- | --- |
| U16 | Type | 2 | Fixed to 0x05 |
| U32 | Timestamp | 4 | Timestamp |
| U16 | Content Len | 2 | Alarm content length |
| U8 | Health Warn Type1 | 1 | Health threshold alarm type: 0x01 heart rate, 0x02 systolic blood pressure (SBP), 0x03 diastolic blood pressure (DBP), 0x04 blood oxygen , 0x05 temperature |
| U8 | Type1 | 1 | 01 is less than the set health threshold, 02 is greater than the set health threshold |
| I16 | Health Value1 | 2 | Health result value |
| U8 | Health Warn Type2 | 1 | Health threshold alarm type: 0x01 heart rate, 0x02 systolic blood pressure (SBP), 0x03 diastolic blood pressure (DBP), 0x04 blood oxygen , 0x05 temperature |
| U8 | Type2 | 1 | 01 is less than the set health threshold, 02 is greater than the set health threshold |
| I16 | Health Value2 | 2 | Health result value |
| .... | .... | .... | .... |

Example:

The device detected that the heart rate (65) is outside the range of the issued health threshold: BDBDBDBD210500E377BD6704000102410073

BDBDBDBD: Header

21:Message ID message id

0500:Alarm typeAlarm type Convert to big-endian0005-->Alarm type=5

E377BD67:Timestamp The timestamp is converted to big-endian67BD77E3-->converted to decimal 1740470243-->The timestamp is 1740470243 seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

0400:Content Len Alarm content length Convert to big-endian0004-->The alarm content length is 4 bytes, note: the alarm length will increase if there are multiple health thresholds

01:Health Warn Type Health threshold alarm type 01-->Heart rate alarm

02:Type Greater than the set health threshold & less than the set health threshold 01-->Less than the set health threshold

4100: Health Value The actually reported health value is converted to big-endian0041--> Convert to decimal 65 --> The reported heart rate value is 65

73: checksum

4.2.3 Abnormal Data Alarm (MSGID=0x16)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x16 | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type | -/- | -/- | Alarm type (0: abnormal heart rate value alarm, 1: abnormal temperature value alarm) |
| 2 | U16 | Heart | -/- | -/- | Heart rate value |
| 2 | U16 | temperature | -/- | -/- | Temperature |
| 2 | U16 | Pa | -/- | -/- | Air pressure |
| 4 | 4*U8 or U32 | expand | -/- | -/- | 4-byte expansion for other sensors |

Note: The reported outlier has no practical reference value, but only indicates that the current detected data is abnormal (such as the value is out of range or the sensor detection is abnormal, etc.). The outlier will be removed in other normal messages and reflected in this message, which can be ignored.

Example:

Temperature anomaly value alarm: BDBDBDBD16010000FF0F000000000000E6

BDBDBDBD:Header

16:Message ID message id

01:Type alarm type 01-->temperature anomaly value alarm

0000:Heart heart rate value. If there is no such value, fill in 0000 by default.

FF0F:temperature temperature. Convert to big endian 0FFF-->temperature anomaly value FFF

0000:Pa air pressure. If there is no value, fill in 0000 by default.

00000000 expand expansion, if this value is not present, the default is to fill with 0000

E6:checksum

4.3 Location-related reporting

4.3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x03 | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude longitude |
| 8 | Double | lat |  | - | latitude latitude |
| 1 | U8 | north_south |  | - | /N or S/ North or South latitude |
| 1 | U8 | east_west |  | - | /E or W/ East or West longitude |
| 1 | U8 | status |  | - | /A or V/ Valid data or invalid data |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original hexadecimal message: BDBDBDBD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BDBDBDBD:Header

03:Message ID message id

963E870C3E675E40:lon longitude Convert to big endian405E673E0C873E96-->Longitude:121.6131622 Note: Refer to the parsing example below for conversion and parsing

FF2110C2B6343F40: lat dimension Convert to big-endian `403F34B6C21021FF-->Latitude:31.2059137 Note: Refer to the parsing example below for conversion and parsing

4E:north_south4E-->Convert to ASCII code N-->North latitude

45:east_west 45--> Convert to ASCII code E --> East longitude

41:status data validity 41--> convert to ASCII code A --> valid data

E377BD67: Timestamp converted to big-endian 67 BD77E3--> converted to 10 hexadecimal 1740470243 --> timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

78: Checksum

Note: The parsed Satellite (GPS/BDS) latitude and longitude are in the WGS-84 coordinate system. If the map uses Baidu AMAP, etc., the coordinate system needs to be converted.

Parsing satellite positioning message Java code example:

//Message: BDBDBDBD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BDBDBDBD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public static void main(String[] args){

//Message 7d9f84ac81815c40 Actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d",16)));  //114.02353966666665//Message e766926b1d893640 Actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16))) ;//22.535605166666667

HexToStr(data.Substring(“4e”));   //N

HexToStr(data.Substring(“45”));//E

HexToStr(data.Substring(“41”));//A A means valid data, V means invalid data, can be discarded//Message 749d695f actual value 5f699d74

Date date=new Date();

date.setTime(Long.parseLong(“5f699d74",16)*1000);

SimpleDateFormatsdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.3.2 Upload of WiFi and base station information (MSGID=0xA4)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xA4 | See below for definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 1 | u8 | Cell_cnt | 1 | - | Number of base stations (0-7) Number of cell info payload.Valid value:0~7 |
| 2 | u16 | Cell[0].MCC | - | - | MMC of base station (0) mobile country code of cell[0] |
| 2 | u16 | Cell[0].MNC | - | - | MNC of base station (0) mobile network code of cell[0] |
| 2 | u16 | Cell[0].LAC | - | - | LAC of base station (0) Location area code of cell[0] |
| 4 | U32 | Cell[0].CELL_ID | - | - | ID of base station (0) Cell id of cell[0] |
| 2 | i16 | Cell[0].RSSI | - | dbm | Signal strength RSSI of base station (0) RSSI in dbm of cell[0] |
| ... | ... | ... | ... | ... | ... |
| 2 | u16 | Cell[cell_cnt-1.MCC | - | - | MMC of base station (N) mobile country code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.MNC | - | - | MNC of base station (N) mobile network code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.LAC | - | - | LAC of base station (N) Location area code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1].LAC | - | - | LAC of base station (N) Location area code of cell[[cell_cnt-1] |
| 4 | U32 | Cell[cell_cnt-1].CELL_ID | - | - | ID of base station (N) Cell id of cell[[cell_cnt-1] |
| 2 | I16 | Cell[cell_cnt-1].RSSI | - | dbm | Signal strength RSSI of base station (N) RSSI in dbm of cell[[cell_cnt-1] |
| 1 | U8 | Wifi_cnt |  |  | Number of wifi Number 0f wifi |
| 6 | 6*U8 | Wifi[0].bssid[0]-Wifi[0].bssid[5] |  |  | MAC address of wifi(0) |
| 4 | I32 | Wifi[0].RSSI |  |  | RSSI of wifi(0) signal strength |
| ... | ... | ... | ... | ... | ... |
| 6 | 6*U8 | Wifi[Wifi_cnt-1].bssid[0]-Wifi[Wifi_cnt-1].bssid[5] |  |  | MAC address of wifi(N) |
| 4 | I32 | Wifi[0].RSSI |  |  | RSSI of wifi(0) signal strength |

Note: The device may not report base stations every time. The absence of base stations does not affect positioning. If the number of base stations in the message is 0, then the following is directly the number of Wi-Fi networks.

Example:

Original16Hexadecimal message: BDBDBDBDA4E377BD6701CC010000C218D1AD160BF9FF078CBEBE1A8162C6FFFFFFC061180AF42AC1FFFFFF200BC726E000B6FFFFFFA8154DF6517EB2FFFFFFE005C5B1F824CCFFFFFFE8FCAFA02663AFFFFFFF6409805B2B9CAEFFFFFF94

BDBDBDBD: Header

A4: Message ID message id

E377BD67: Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

01: Cell_cnt number of base stations 01 --> number of base stations is 1

CC01: Cell[0].MCC Base station MCC converted to big-endian 01CC --> converted to decimal 460 --> Base station MCC is 460

0000:Cell[0].MNC The base station MNC is converted to big-endian0000-->Convert to decimal0-->The base station MNC is0

C218:Cell[0].LAC Base station LAC converted to big-endian18C2-->Convert to decimal6338-->Base station LAC is6338

D1AD160B:Cell[0].CELL_ID Base station ID converted to big-endian0B16ADD1-->Convert to decimal186035665-->Base station ID is186035665

F9FF:Cell[0].RSSI Base station signal strength converted to big-endian FFF9-->Convert to decimal complement-7-->Base station signal strength is-7

07: Wifi_cnt wifi number 07-->7wifis

8CBEBE1A8162:wifi mac address 8C:BE:BE:1A:81:62-->wifi[0]'s mac address is8C:BE:BE:1A:81:62

C6FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFC6-->Convert to decimal complement-58--->wifi[0]'s RSSI signal strength is-58

C061180AF42A:wifi mac address C0:61:18:0A:F4:2A-->wifi[1]'s mac address is C0:61:18:0A:F4:2A

C1FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFC1-->Convert to decimal complement-63--->wifi[0]'s RSSI signal strength is-63

200BC726E000:wifi mac address 20:0B:C7:26:E0:00-->wifi[2]'s mac address is20:0B:C7:26:E0:00

B6FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFB6-->Convert to decimal complement-74--->wifi[0]'s RSSI signal strength is-74

A8154DF6517E:wifi mac address A8:15:4D:F6:51:7E-->The mac address of wifi[3] is A8:15:4D:F6:51:7E

B2FFFFFF:wifi signal strength RSSI converted to big endian FFFFFFB2-->converted to decimal complement -78--->The RSSI signal strength of wifi[0] is -78

E005C5B1F824:wifi mac address E0:05:C5:B1:F8:24-->The mac address of wifi[4] is E0:05:C5:B1:F8:24

CCFFFFFF:wifi signal strength RSSI converted to big endian FFFFFFCC-->converted to decimal complement -52--->The RSSI signal strength of wifi[0] is -52

E8FCAFA02663:wifi mac address E8:FC:AF:A0:26:63-->The mac address of wifi[5] is E8:FC:AF:A0:26:63

AFFFFFFF:wifi signal strength RSSI converted to big endian FFFFFFAF-->converted to decimal complement -81---->The RSSI signal strength of wifi[0] is -81

6409805B2B9C:wifi mac address 64:09:80:5B:2B:9C-->The mac address of wifi[6] is64:09:80:5B:2B:9C

AEFFFFFF:wifi signal strength RSSI converted to big endian FFFFFFAE-->converted to decimal complement -82--->The RSSI signal strength of wifi[0] is -82

94:checksum

The parameters parsed from the message need to be connected to the map wifi database, and the latitude and longitude data is finally obtained through the map party. Take AMAP connecting to the wiif database as an example:

According to the access network method, the AMAPWiFi positioning protocol document divides it into three positioning scenarios. The following provides corresponding examples for the three positioning scenarios.

1. The network connection method is WiFi access, that is, the accesstype parameter is 1.

http://apilocate.amap.com/position?accesstype=1&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&mmac=50:a7:2b:1e:56:58,-60,alibaba-inc&macs=4c:48:da:25:0b:11,-59,alibaba-

inc|4c:48:da:25:1a:11,-77,alibaba-inc&serverip=10.2.166.4&output=xml&key=<用户 Key>

2. The network connection method is mobile access and the mobile phone card is a non-CDMA card, that is, when the accesstype parameter is 0 and cdma=0:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=0&bts=460,01,40977,2205409,-65&nearbts=460,01,40977,2205409,-65|460,0

1,40977,2205409,-65|460,01,40977,2205409,-65&serverip=10.2.166.4&output=xml&key=<用户 Key>

3. The network connection method is mobile access and the mobile phone card is a CDMA card, that is, when the accesstype parameter is 0 and cdma=1:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=1&bts=13824,1,1838,1674723,575739,-52&serverip=10.2.166.4&output=xml&key=<用户 Key>

W200P Watch: AMAP WiFi positioning protocol calls AMAPinterface description: When selecting parameters, generally select the first type

You can also select the second type, but if the device does not report the base station, you need to remove the two parameters bts and neabts before you can

The first type, example of incoming parameters: mmac is the first WIFI mac address in the message:http://apilocate.amap.com/position?accesstype=1&imei=869907057780989&mmac=04:8C:16:A9:FB:A8,-44,TP-Link&macs=04:8C:16:A9:FB:A8,-44,TP-Link|C8:3A:35:F4:CE:18,-59,TP-Link|1C:78:4E:D5:07:FE,-79,TP-Link|C8:3A:35:9D:0B:E0,-87,TP-Link|4A:D8:90:E8:0A:82,-88,TP-Link|AC:AD:4B:C9:C3:87,-89,TP-Link|A4:16:E7:55:84:AC,-91,TP-Link&output=json&key=xxxx

For the specific AMAP WiFi positioning protocol PDF, you can click to view and download:(not a screenshot, you can click to view)
This document is provided by AMAP, and you can directly ask AMAP for specific docking

4.3.3 Bluetooth Location Information (BLE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xD6 | See below for definition below | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed to 0 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple groups of information, each group may have multiple ibeacons |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (One of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (One of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| ... | ... | ... | ... |

Note: In the general version, the total number of Bluetooth beacon groups is fixed at 1, and the maximum number of Bluetooth beacons is 4. The sorting is based on the signal strength from largest to smallest. By default, the watch only recognizes our own beacons. If you need to connect to other beacons, please consult relevant personnel via WeChat. Example:

Original hexadecimal message: BDBDBDBDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

bdbdbdbd: Header

D6: Message ID

00: type

01: There is only one set of ibeacon data

E377BD67:Timestamp timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

04: The first group has 4 beacon information

4327: Convert to big endian --> major : 0x2743 --> Convert to decimal 10051

1794: Convert to big endian --> minor: 0x9417 --> Convert to decimal 37911

AC:  RSSI 0xAC --> Convert to decimal complement -84

4327: Convert to big endian --> major: 0x2743 --> Convert to decimal 10051

3094: Convert to big endian --> minor:0x9430 --> Convert to decimal 37936

AA:  RSSI 0xAA --> Convert to decimal complement -86

4327:Convert to big endian --> major: 0x2743 --> Convert to decimal 10051

b956:Convert to big endian --> minor:0x56b9 --> Convert to decimal 22201

A5:   RSSI 0xA5 --> Convert to decimal complement -91

4327: Convert to big endian --> major: 0x2743 --> Convert to decimal 10051

FE94: Convert to big-endian --> minor:0x94FE --> Convert to decimal 38142

A5: rssi:0xA5 --> Convert to decimal complement -91

6a -- Checksum

Bluetooth beacon major, minor description: The default shipping Bluetooth beacon has a beacon code attached,

For example: 00081005345866, then the major of the Bluetooth beacon is 10053, the minor is 45866, and the manufacturer ID is 0008 (this is useless and has no practical significance)

4.3.4 Base station latitude and longitude reporting (MSGID=0x15) — wifi positioning supplement

Note: This message is generally when wifi positioning fails, and the module automatically requests the latitude and longitude of the communication base station for positioning (the positioning priority cannot be changed). The location is the location of the communication base station and can only be used as a reference for auxiliary positioning. The accuracy of the communication base station is not high.

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x15 | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | flag |  | -/- | Extended segment identifier |
| 4 | U32 | lon |  | -/- | Longitude (*10 to the 6th power or 7th power) longitude |
| 4 | U32 | lat |  | -/- | latitude (*10 to the power of 6 or 7) latitude |
| 1 | U8 | north_south |  | -/- | /N or S/ South or North latitude |
| 1 | U8 | east_west |  | -/- | /E or W/ East or West longitude |
| 1 | U8 | status |  | -/- | /A or B or V/ A, B are valid data, B corresponds to 7-bit precision, V is invalid data |
| 4 | U32 | Timestamp |  | -/- | Timestamp |
| N |  | Num |  | -/- | The general version has no extensions, the table only retains the definition Definition: The extended segment value 1 can be the nth digit value defined by the extended segment), when multiple extended segments are used simultaneously, the extended segment 1 corresponds to the lowest bit content, and the number of bits n is extended in sequence |
| ... | ... | ... | ... | ... | ... |

Example:

Original16Hexadecimal message: BDBDBDBD1502C8C9E53F855AD30F4E4542E377BD67F4

BDBDBDBD: Header

15: Message ID

02:type  type=02indicates LBS base station positioning

C8C9E53F: Longitude ---> converted to big endian4857963E---> converted to longitude:26.5509509, Note: Refer to4.3.1GPS/ BDS location reporting for conversion and analysis: Positioning data reporting (0x03) section analysis example

855AD30F: Dimension ---> Convert to Big Endian129B82E0--->Convert to Dimension:107.2024008, Note: Conversion analysis reference4.3.1GPS/ BDS Location Reporting: Location data reporting (0x03) Section analysis example

4E:north_south 4E-->Convert to ASCII encoding N-->North latitude

45:east_west 45-->Convert to ASCII encoding E-->East longitude

42:status 42-->Convert to ASCII encoding B, valid data, the precision is to keep 7 decimal places

E377BD67:Timestamp timestamp Convert to big endian67BD77E3-->Convert to10system1740470243-->The timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

08:checksum

4.4 Device Information and Status Reporting

4.4.1 SIM card ICCID upload (MSGID=0xF3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xF3 | See below for definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 10 | 10*U8 | ICCID | 1 | - | ICCID number - Device SIM card number ICCID |

Note: This message is reported once every time the device is turned on.

Example:

Original hexadecimal message:BDBDBDBDF389861118236001639994CC

BDBDBDBD:Header

F3:Message ID

89861118236001639994:ICCID Device SIM card number ICCID is: 89861118236001639994

CC:checksum

4.4.2 Device charging status upload (MSGID=0xC3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Status | / | / | 0 start, 1 end, 2 fully charged |
| 1 | U32 | Timestamp | / | / | Timestamp (a timestamp will be added after it when re-uploading) |

Note: This message requires device firmware support, and the latest general version is supported by default. Example:

Original hexadecimal message::BDBDBDBDC301E377BD678A

BDBDBDBD:Header

C3:Message ID

01:Device finished charging

E377BD67:Timestamp timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

Original hexadecimal message::BDBDBDBDC300DB4D2F668A

BDBDBDBD:Header

C3:Message ID

00:Device started charging

E377BD67:Timestamp timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

Original hexadecimal message::BDBDBDBDC302DB4D2F668A

BDBDBDBD:Header

C3:Message ID

02:Device is fully charged

E377BD67:Timestamp timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

4.4.3 Status parameter reporting (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Number of parameter types |
| 1 |  |  |  |  | （Reserved 00） |
| 1 | u8 | Type | -/- | -/- | Parameter type 1, Type definition: 00—MCU (firmware), 01—Module, 02 Sensor (no such type in the general version) |
| 1 | U8 | NameLen | -/- | -/- | Parameter name 1 length |
| n | N*u8 | Name | -/- | -/- | Parameter name |
| 1 | u8 | Type | -/- | -/- | Parameter type 2, Type definition: 00—MCU (firmware), 01—Module, 02 Sensor (no such type in the general version) |
| 1 | U8 | NameLen | -/- | -/- | Parameter type 2 length |
| n | N*u8 | Name | -/- | -/- | Parameter name |
| ... | ... | ... | ... | ... | ... |

Note: This message is reported once by default at startup, uploading the device firmware version number and module version number, which can be left unparsed. Previously shipped devices had a Message ID of 0xBB, which can be obtained by contacting relevant personnel.

Example:

BDBDBDBDA902000009423732352E4F56303601174E5432364B434E4230304E4E412D4C3032303330393530B5

BDBDBDBD:Header

A9:Message ID

02:TypeCnt indicates that there are 2 types of parameters

00:Reserved field

00:Type=00 00 represents MCU (firmware)

09:The following MCU name length is 9 bytes

423732352E4F563036--->Hexadecimal to text (device firmware version number)--->B725.OV06

01:Type=01 01 represents the module

17:The length of the communication module name following is 23 bytes

4E5432364B434E4230304E4E412D4C3032303330393530:Hexadecimal to text (4G module version number)

--->NT26KCNB00NNA-L02030950

B5---checksum

4.4.4 Device Status (MSGID=0xE9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xE9 | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | type |  | / | Default—00 |
| 2 | u16 | length |  | / | Length of the following message |
| 1 | u8 | location |  | / | Location reporting frequency: 00 — Default not modified, 01 — Modified by downlink (if there are multiple time periods in the downlink, only report the reporting frequency of the current time period) |
| 2 | u16 | frequency |  | / | Frequency, unit - minutes |
| 1 | u8 | health |  | / | Health reporting frequency: 00 — Default not modified, 01 — Modified by downlink |
| 2 | u16 | frequency |  | / | Frequency: unit - minutes |

Note: This message is reported once at startup, and once after the downlink modifies the frequency

Example:

Original hexadecimal message:BDBDBDBDE9000600010A00010A0033

BDBDBDBD:Header

E9:Message ID

00:type=00

0600:The message length after length is converted to big-endian -->0006--> converted to decimal 6, and the message length is 6 bytes

01:location 01 indicates that the downlink has modified the location reporting frequency

0A00:frequency location reporting frequency converted to big-endian --> 000A --> converted to decimal 10, the reporting frequency is 10 minutes

01:health 01 indicates that the downlink has modified the health reporting frequency

0A00:frequency Health reporting frequency converted to big-endian --> 000A --> converted to decimal 10, the reporting frequency is 10 minutes

33:checksum

If the device does not have health reporting function or location reporting function, then the health reporting frequency and

The two values of location reporting frequency will remain the same, indicating the device reporting frequency

4.  5 Health-related reporting

4.  5.1 Health data (MSGID=0x32)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x32 | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | 00 |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 2 | U16 | contentLength |  |  | Total length of the following content |
| 1 | U8 | ID |  |  | Id (Type + Reported Value Length), see below for ID type definition |
| N | N*U8 | Val1 |  |  | Reported value of Id (byte length varies depending on the reported value length) |
| ... | ... | ... | ... | ... | ... |
| 1 | U8 | ID |  |  | Id (Type + Reported Value Length), see below for ID type definition |
| N | N*U8 | Val1 |  |  | Reported value of Id (byte length varies depending on the reported value length) |

ID type definition (hexadecimal):

01 Steps02 Heart Rate 03 Body Temperature 04 Wrist Temperature 06 Diastolic Blood Pressure 07 Systolic Blood Pressure 08 Blood Oxygen 09 HRV(Special version support)

(05 Blood Glucose 10 Sit-up Count11 Running Pace and Distance 12 Jump Rope Speed and Count )---Reserved, currently unavailable

Default universal version support:01 Steps02 Heart Rate 03 Body Temperature 04 Wrist Temperature 06 Diastolic Blood Pressure 07 Systolic Blood Pressure 08 Blood Oxygen

Example:

Original16hexadecimal message: BDBDBDBD3200E377BD6711000A1E00114B314A39711A4A0122bc00416212

BDBDBDBD: Header

32:Message ID

00:type  type=00

E377BD67:Timestamp timestamp Convert to big endian67BD77E3-->Convert to10system1740470243-->The timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

1100: The length of the following message is converted to big-endian -->0011--> converted to decimal17, and the length of the following message is17bytes

0A:ID  0x0A--> converted to binary00001010   The first five digits are 00001--> converted to16hexadecimal01, which represents the data ID step counting (0x01),

The last three digits are 010--> converted to decimal02, which represents the length of the step counting data is2bytes

1E00: The step counting report value is converted to big-endian -->001E--> converted to decimal30, and the total step counting data is30steps

11:ID  0x11--> converted to binary00010001   The first five digits are 00010--> converted to16hexadecimal02, which represents the data ID heart rate (0x02),

The last three digits are 001--> converted to decimal1, which represents the length of the heart rate data is1byte

4B: Heart rate report value converted to decimal -->75, heart rate data is75

31:ID  0x31--> converted to binary00110001   The first five digits are 00110--> converted to16hexadecimal06, which represents the data ID diastolic blood pressure (0x06),

The last three digits are 001 --> converted to decimal 1, representing the length of the diastolic blood pressure data as 1 byte

4 A: Blood pressure diastolic value, converted to decimal --> 74, diastolic blood pressure data is 74

39: ID 0x3 --> converted to binary 00111001 The first five digits are 00111 --> converted to hexadecimal 16 07, representing the data ID systolic blood pressure (0x07),

The last three digits are 001 --> converted to decimal 1, representing the length of the systolic blood pressure data as 1 byte

71: Blood pressure systolic value, converted to decimal --> 113, systolic blood pressure data is 113

1 A:ID 0x1A --> converted to binary 00011010 The first five digits are 00011 --> converted to hexadecimal 16 03, representing the data ID body temperature (0x03),

The last three digits are 010 --> converted to decimal 2, representing the length of the body temperature data as 2 bytes

4 A01: Body temperature reported value, converted to big-endian --> 014 A--> converted to decimal 330 --> 330 * 0.1 --> body temperature data is 33 degrees Celsius

22:ID 0x22 --> converted to binary 00100010 The first five digits are 00100 --> converted to hexadecimal 16 04, representing the data ID wrist temperature (0x04),

The last three digits are 010 --> converted to decimal 2, representing the length of the wrist temperature data as 2 bytes

BC00: Wrist temperature reporting value, converted to big-endian -->00BC --> Convert to decimal188-->188*0.1--> Wrist temperature data is18.8degrees Celsius

41: ID 0x41 --> Convert to binary 01000001 The first five digits are 01000 --> Convert to hexadecimal 08, code data ID blood oxygen (0x08),

The last three digits are 001 --> Convert to decimal 1, representing the blood oxygen data length of 1 byte

62: Blood pressure reported value, converted to decimal --> 98, blood oxygen data is 98

12: checksum

4.5.2 Device Sleep Analysis Data Upload (MSGID=0xC5)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC5 | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 4 | Int32 | DateTime |  | -/- | Upload UTC start time |
| 4 | Int32 | DateTime |  |  | Upload Utc end time |
| 2 | Int16 | Sleepminute |  |  | Upload sleep time in minutes |
| 4 | Int32 | Type |  |  | Sleep state: Upload type, 1 is deep sleep, 2 is light sleep, 3 is wake-up duration |

Example:

Original16Hexadecimal message: BDBDBDBDC5AC338860693B8860210001000000D1

BDBDBDBD: Header

C5:Message ID

AC338860: Start time, converted to big-endian 608833AC--> converted to decimal --> timestamp is1619538860seconds -->

Convert to standard time format UTC time: 2021-04-27 15:54:20-->Convert to Beijing time:2021-04-27 23:54:20

693B8860: End time, converted to big-endian60883BAC--> converted to decimal --> timestamp is1619540908seconds -->

Convert to standard time format UTC time: 2021-04-27 16:28:28-->Convert to Beijing time:2021-04-28 00:28:28

2100:Sleepminute sleep time, converted to big-endian0021--> converted to decimal33--> The total sleep time is33minutes

01000000:Type sleep state, converted to big-endian00000001--> converted to decimal1-->1is deep sleep, deep sleep time is33minutes

D1: checksum

4.6 Downlink Feedback Related Reporting

4.6.1 Downlink Feedback (MSGID=0xC0)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC0 | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID Length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This command is used for feedback on downlink commands , returning the Message ID of the downlink command received earlier (can be a collective return of multiple Message IDs)

Original hexadecimal message:BDBDBDBDC0011720

BDBDBDBD:Header

C0:Message ID

01:length Message ID length Convert decimal 01--> Message ID length is 1 byte

17Type indicates that the device receives the downlink command Message ID is 17

20:checksum

4.6.2 Message Status Report (MSGID=0x28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x28 | See below for definition below | CK_sum |

Payload contents

| Bytes | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | timestamp | -/- | -/- | Timestamp |
| 1 | U8 | Type | -/- | -/- | Character encoding type (corresponding to downlink 0X28), GB2312 encoding: 0x03; Unicode encoding: 0x05 |
| 1 | U8 | status | -/- | -/- | Message status - 0x01: confirmed, 0x02: rejected |
| 4 | U32 | seqID | -/- | -/- | (Corresponds to message ID seqID with downlink 0X28) |

Example:

Note:Currently, the general version does not distinguish character encoding types and is fixed at 03. Subsequent versions will distinguish.

Original hexadecimal message:BDBDBDBD28FD3BA068030268A03BE477

BDBDBDBD:Header

28:Message ID

FD3BA068:timestamp The timestamp is converted to big-endian 68A03BFD-->converted to decimal-->the timestamp is 1755331581 seconds-->

Convert to standard time format UTC time: 2025-08-16 08:06:21-->Convert to Beijing time: 2025-08-16 16:06:21

03:Type 03--fixed value

02:status Message status 02--The interface clicks the reject button--rejected

68A03BE4:seqID The id of the downlink message is 68A03BE4

77:checksum

4.7 NFC data related report - hardware support is required, this function is not available in the default general version

4.7.1 Check-in NFC Data Reporting (MSGID=0xDC)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xDC | See below for definition below | CK_sum |

Payload contents

| Byte | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | 1 – Check-in |
| 1 | U8 | len |  |  | UID Length |
| N | N*U8 | UID |  |  | UID |
| 4 | U32 | UTC |  |  | Timestamp |

Example:

Original Hexadecimal Message:BDBDBDBD DC 01 07 0483E3FA220289 FD3BA068 78

BDBDBDBD:Header

DC:Message ID

01:Type 01--Check-in Type

07:len The length of the NFC tag UID read. Convert the decimal 07 --> UID length is 7 bytes

0483E3FA220289:UID NFC tag UID is: 0483E3FA220289

FD3BA068:timestamp The timestamp is converted to big-endian 68A03BFD-->converted to decimal-->the timestamp is 1755331581 seconds-->

Convert to standard time format UTC time: 2025-08-16 08:06:21-->Convert to Beijing time: 2025-08-16 16:06:21

78:checksum

5 Settings

5.1 Downlink

5.1.1 Set location reporting frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x17 | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |

Note: One day is from 00:00-23:59
Example:

00:00-23:59 Report location every10minutes: BDBDBDBD17010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

17:Message ID

01:enable Whether to enable Convert to decimal01-->1--Enabled

0A00:Interval time interval Convert to big-endian000A-->Convert to decimal10-->Time interval is10minutes

00:time_start_h Start time-hour, convert to decimal00-->Start time-hour is00

00:time_start_m Start time-minute, convert to decimal00-->Start time-minute is00

17:time_end_h End time-hour, convert to decimal23-->End time-hour is 23

3B:time_end_m End time-minute, convert to decimal59-->End time-minute is 59

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

97:checksum

5.1.2 Message Send (MSGID=0X28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x28 | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | U8 | type | Message type, if it is downlink text information, Unicode encoding is 05 |
| 4 | Uint32 | seqID | The id of the information, unique |
| 1 | U8 | CONTENT LEN | Content length |
| N | N | CONTENT | Content, Chinese is Unicode encoded (little-endian first, pay attention to conversion, one Chinese character occupies 2 bytes, one letter occupies 2 bytes) |

Note: The default Chinese character encoding for the Hong Kong general version is Unicode. The information ID must not be repeated for each transaction. The device cannot receive repeated IDs. Example:

Text message: hello,world  BDBDBDBD28050300000016680065006C006C006F002C0077006F0072006C006400DD

BDBDBDBD:Header

28: Message ID

05: type Message type Convert to decimal 05-->type=05

03000000: seqID Information ID Information ID is 03000000

16: CONTENT LEN Content length Convert to decimal 22, content length is 22 bytes

680065006C006C006F002C0077006F0072006C006400:

Convert every 2 bytes to big-endian --> Convert to Unicode character --> Combine final - text message content:hello,world

DD:checksum

5.1.3 Settings - Location Priority Setting (0XCE01)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition below | CK_sum |

Payload contents

Type=01 Location Priority Setting

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 effective this time |
| 2 | U16 | Len | -/- | -/- | Length of the following instruction |
| n | n |  | -/- | -/- | Body |

Location priority setting definition--See2.3 Device downlink description--Device location priority distribution(0xCE01)

Body of location priority settings: Currently, the general version supports:01--Satellite (GPS/BDS),02--wifi(wifi+base station) 03--Bluetooth beacon (requires additional deployment of Bluetooth beacons); Special equipment supports:04--LBSbase station 05--125k

Example:

WiFi positioning priority (WiFi > Bluetooth beacon > Satellite (GPS/BDS)):BDBDBDBDCE0100030002030133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003 --> Converted to decimal 03 --> The length of the text is 3 bytes

020301:Text: 02--WiFi 03--Bluetooth beacon 01--Satellite (GPS/BDS) Positioning priority is WiFi > Bluetooth beacon > Satellite (GPS/BDS)

33:checksum

Satellite (GPS/BDS) positioning priority (Satellite (GPS/BDS)> WiFi > Bluetooth beacon):BDBDBDBDCE0100030001020333

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003 --> Converted to decimal 03 --> The length of the text is 3 bytes

010203:Text: 01--Satellite (GPS/BDS) 02--WiFi 03--Bluetooth beacon Positioning priority is Satellite (GPS/BDS)> WiFi > Bluetooth beacon

33:checksum

Bluetooth positioning priority (Bluetooth beacon > WiFi > Satellite (GPS/BDS)):BDBDBDBDCE0100030003020133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003 --> Converted to decimal 03 --> The length of the text is 3 bytes

030201:Text: 03--Bluetooth beacon 02--WiFi 01--Satellite (GPS/BDS) Positioning priority is Bluetooth beacon > WiFi > Satellite (GPS/BDS)

33:checksum

Single Satellite (GPS/BDS) positioning:BDBDBDBDCE0100030001000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003 --> Converted to decimal 03 --> The length of the text is 3 bytes

030201:Text: 01--GPS 00--No positioning 00--No positioning Positioning priority is single Satellite (GPS/BDS) positioning

33:checksum

Single wifi positioning:BDBDBDBDCE0100030002000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003 --> Converted to decimal 03 --> The length of the text is 3 bytes

020000:Text: 02--wifi 00--No positioning 00--No positioning Positioning priority is single wifi positioning

33:checksum

Single Bluetooth beacon positioning:BDBDBDBDCE0100030003000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003 --> Converted to decimal 03 --> The length of the text is 3 bytes

020000:Text: 03--Bluetooth beacon 00--No positioning 00--No positioning Positioning priority is single Bluetooth beacon positioning

33:checksum

5.1.4 Settings - Health sampling report frequency setting (MSGID=0X1C)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x1D | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |

Definition of health sampling report frequency setting--See2.3 Device downlink description--Device health sampling frequency distribution(0x1D)

Note: After setting, report by time period, and do not report outside the time period

Example:

00:00-23:59(All day) Report health data every10minutes: BDBDBDBD1C010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

1C:Message ID

01:enable Whether to enable Convert to decimal01-->1--Enabled

0A00:Interval time interval Convert to big-endian000A-->Convert to decimal10-->Time interval is10minutes

00:time_start_h Start time-hour, convert to decimal00-->Start time-hour is00

00:time_start_m Start time-minute, convert to decimal00-->Start time-minute is00

17:time_end_h End time - hour, convert to decimal23--> Start time - hour is00

3B:time_end_m End time - minute, convert to decimal59--> Start time - hour is00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

97:checksum

5.1.5 Settings - Device Alarm Settings (0XCE03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition below | CK_sum |

Payload contents

Type=03 Device alarm settings

| Byte offset | Format | Name | Unit | Decription |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Fixed to 0x03 - Device alarm settings |
| 1 | U8 | Valid | 1 | Fixed to 0x00 - Health threshold, always valid |
| 2 | U16 | Cfg Len | 2 | Instruction length |
| 1*N | U8*N | [Bit7] Enable [Bit6 - 0] Threshold Type | 1*N | [Bit7] Whether to enable threshold detection for this type [Bit6 - 0] Set the threshold type: 0x01 heart rate, 0x02 systolic blood pressure (SBP) 0x03 diastolic blood pressure (DBP), 0x04 blood oxygen, 0x05 temperature (threshold unit is *10, for example, 37.3 is 373) |
| 2*N | I16*N | Threshold Min | 2*N | Lower threshold (this content is only available when threshold detection is enabled) |
| 2*N | I16*N | Threshold Max | 2*N | Upper threshold (this content is only available when threshold detection is enabled) |

Device alarm setting definition-Health threshold--See2.3 Device downlink description--Health threshold setting(0xCE0300)

Note: The default is the closed state, and the threshold is within the normal range. The device will report an alarm outside this range.

Heart rate threshold delivery range (20-200), blood pressure systolic threshold delivery range (20-200), blood pressure diastolic threshold delivery range (20-200), blood oxygen threshold delivery range (80-100), temperature threshold delivery range (160-600)

Example:

Set multiple health thresholds,1. Heart rate [60,120],2. Turn off blood oxygen threshold detection,3. Temperature [36.0,37.3]:BDBDBDBDCE03000B00813C007800048568017501FF

BDBDBDBD: Header

CE:Message ID

03:Type 03--Device alarm settings

00:Valid 00--Health threshold, always valid

0B00: Convert to big-endian-->000B--> Convert to decimal11--> The number of bytes of the subsequent instruction message (excluding checksum) 11 bytes

81: Convert to binary-->10000001   bit7:1---Enable     Enabled state   bit6-0:000001-->0x01  Heart rate

3C00: Convert to big-endian-->003C--> Convert to decimal60, the lower limit of the heart rate threshold is60

7800: Convert to big-endian-->0078--> Convert to decimal120, the upper limit of the heart rate threshold is120

04: Convert to binary -->00000100   bit7:0---Off, no threshold after closing bit6-0:0000100-->0x04  Blood oxygen

85: Convert to binary -->10000101  bit7:1---On On state bit6-0:00000101-->0x05  Temperature

6801: Convert to big endian -->0168--> Convert to decimal360(actual temperature *10)--> Lower temperature threshold is36.0

7501: Convert to big endian -->0175--> Convert to decimal373(actual temperature *10)--> Upper temperature threshold is37.3

FF:checksum

5.1.6 Settings - Switch Settings (0XCE04-24)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Please refer to the instructions for the specific type |
| 1 | u8 | switch | -/- | -/- | Switch 00 on, 02 off, 01 single effective |
| 2 | U16 |  | -/- | -/- | Fixed 0000 |

Device Alarm Setting - Switch Type: Detailed Function Description-- See 2.3 Device Downlink Description

05 Bluetooth broadcast switch 06 Location and health data reporting switch ---- Previously used firmware, the latest firmware is no longer used

07 Fall alarm switch 08 Stay alarm switch

0A Immediately report location information 0B Immediately report health data

0D Immediately report version number data 16 Whether the downlink can use the key to shut down

17 Downlink supports long standby mode 18 Sleep function switch

19 Key trigger sos alarm switch 20 Report data switch--Separate settings (Note that the structure of this switch is special, there are differences in the message and table definitions, please see the specific example)

22 Long and short connection mode switching 24 GPS always on switch

Type=05 Bluetooth broadcast switch Example:

00---On :BDBDBDBDCE0500000093

BDBDBDBD:Header

CE:Message ID

05 : Type 05 Bluetooth broadcast switch

00:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE0502000093

BDBDBDBD:Header

CE:Message ID

05:Type 05 Bluetooth broadcast switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=06 Location and health data reporting switch - used by previous firmware, not used by the latest firmware, reserved here
Example:

00---On (will report once immediately after being issued when turned off):BDBDBDBDCE0600000093

BDBDBDBD:Header

CE:Message ID

06:Type 06 Location and Health Data Reporting Switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Off:BDBDBDBDCE0602000093

BDBDBDBD:Header

CE:Message ID

06:Type 06 Positioning and Health Data Reporting Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=07 Fall Alarm Switch
Example:

00---On:BDBDBDBDCE0700000093

BDBDBDBD:Header

CE:Message ID

07:Type 07 Fall Alarm Switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Off:BDBDBDBDCE0702000093

BDBDBDBD:Header

CE:Message ID

07:Type 07 Fall Alarm Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=08 Stay Alarm Switch
Example:

00---Open:BDBDBDBDCE0800000093

BDBDBDBD:Header

CE:Message ID

08:Type 08 Stay Alarm Switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE0802000093

BDBDBDBD:Header

CE:Message ID

08:Type 08 Stay Alarm Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=0A Report location information immediately
Example:

01---Single report:BDBDBDBDCE0A01000093

BDBDBDBD:Header

CE:Message ID

0A:Type 0A Report location information immediately

01:switch switch 01--Single issue single report

0000:Fixed value

93:checksum

Type=0B Report health data immediately
Example:

01---Single report:BDBDBDBDCE0B01000093

BDBDBDBD:Header

CE:Message ID

0B:Type 0B Immediately report health data

01:switch switch 01--Single issue single report

0000:Fixed value

93:checksum

Type=0D Immediately report version number data
Example:

01---Single report:BDBDBDBDCE0D01000093

BDBDBDBD:Header

CE:Message ID

0D:Type 0D Immediately report version number data

01:switch switch 01--Single issue single report

0000:Fixed value

93:checksum

Type=16 Whether the downlink can use the key to shut down
Example:

00---Open:BDBDBDBDCE1600000093

BDBDBDBD:Header

CE:Message ID

16:Type 16 Whether the downlink can use the key to shut down

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE1602000093

BDBDBDBD:Header

CE:Message ID

16:Type 16 Whether the downlink can use the key to shut down

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=17 Downlink supports long standby mode
Example:

00---Open:BDBDBDBDCE1700000093

BDBDBDBD:Header

CE:Message ID

17:Type 17 downlink supports long standby mode

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close::BDBDBDBDCE1702000093

BDBDBDBD:Header

CE:Message ID

17:Type 17 downlink supports long standby mode

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=18 Sleep function switch
Example:

00---Open, enter sleep mode after being stationary for a period of time, and do not report data:BDBDBDBDCE1800000093

BDBDBDBD:Header

CE:Message ID

18:Type 18 Sleep function switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close, report data normally in stationary or charging state:BDBDBDBDCE1802000093

BDBDBDBD:Header

CE:Message ID

18:Type 18 Sleep function switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=19 Key trigger sos alarm switch
Example:

00---Open:BDBDBDBDCE1900000093

BDBDBDBD:Header

CE:Message ID

19 : Type 19 button triggers SOS alarm switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close: BDBDBDBDCE1902000093

BDBDBDBD:Header

CE:Message ID

19 : Type 19 button triggers SOS alarm switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=20 Report data switch - separate settings Example:

Add field: Text length: 2 bytes Text: 0x00 (location data report) + switch (0x00: on, 0x02: off)

0x01 (health data report) + switch (0x00: on, 0x02: off)

Location data report on : BDBDBDBDCE20000200000093

BDBDBDBD:Header

CE:Message ID

20 : Type 20 report data switch

00 : Fixed value

0200 : Text length converted to big-endian 0002-->converted to decimal 02-->text length is 2 bytes

00 : Text: 00--Location data report switch

00: Main text: 00--Enable reporting

93:checksum

Location data reporting disabled: BDBDBDBDCE2000020000293

BDBDBDBD:Header

CE:Message ID

20 : Type 20 report data switch

00 : Fixed value

0200 : Text length converted to big-endian 0002-->converted to decimal 02-->text length is 2 bytes

00 : Text: 00--Location data report switch

02: Main text: 00--Disable reporting

93:checksum

Health data reporting enabled: BDBDBDBDCE20000200010093

BDBDBDBD:Header

CE:Message ID

20 : Type 20 report data switch

00 : Fixed value

0200 : Text length converted to big-endian 0002-->converted to decimal 02-->text length is 2 bytes

01: Main text: 00--Health data reporting switch

00: Main text: 00--Enable reporting

93:checksum

Health data reporting disabled: BDBDBDBDCE20000200010293

BDBDBDBD:Header

CE:Message ID

20 : Type 20 report data switch

00 : Fixed value

0200 : Text length converted to big-endian 0002-->converted to decimal 02-->text length is 2 bytes

01: Main text: 00--Health data reporting switch

02: Main text: 02--Disable reporting

93:checksum

Type=22 Long and short connection mode switching Example:

00---Long connection mode, default is long connection : BDBDBDBDCE2000000093

BDBDBDBD:Header

CE:Message ID

22: Type 22 Long and short connection mode switching

02:switch on 00 to enable long connection mode

0000:Fixed value

93:checksum

02---Short connection mode:BDBDBDBDCE2002000093

BDBDBDBD:Header

CE:Message ID

22: Type 22 Long and short connection mode switching

02:switch Switch 02 Closes Long Connection Mode - Short Connection Mode

0000:Fixed value

93:checksum

Type=24 GPS always-on switch
Example:

00---Open:BDBDBDBDCE2400000093

BDBDBDBD:Header

CE:Message ID

24:Type 24 GPS Always-On Switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE2402000093

BDBDBDBD:Header

CE:Message ID

24:Type24 GPS Always-On Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

5.1.7 Setting - Drop Sensitivity Setting (0xCE15)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Decription |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Type is 0x15 |
| 1 | U8 | Valid | 1 | The default is 0, which has no impact for now |
| 2 | U16 | Len | 2 | Length of content after parameter |
| 1 | U8 | Param Type | 1 | Parameter type: 0x00: Sensitivity, 0x01: Height |
| 1 | U8 | Param Level | 1 | Parameter level: 5 levels (0 - 4) |

Drop sensitivity setting-Parameter description:

Sensitivity: Refers to the degree to which the drop algorithm is triggered, providing5setting levels (0 - 4): Low - Lower (Medium Low) - Medium - Higher (Medium High) - High.

Height: Refers to the height that satisfies the trigger of the drop alarm, providing5setting levels (0 - 4):0.5m - 1.0m - 1.5m - 2.0m - 2.5m.

If the height is set to “1.5m”, the device needs to be at least ≥1.5mto trigger the drop alarm.

Note that sensitivity and height cannot be combined in a message

Example:

Set the sensitivity to "Medium-Low": BDBDBDBDCE150002000001FF

BDBDBDBD:Header

CE:Message ID

15:Type Drop Sensitivity Setting

00 : Fixed value

0200:Len

00:Param Type Sensitivity Setting

01:Param Level Sensitivity Level 1-->Lower (Medium Low)

FF:checksum

Set height to “1.5m”: BDBDBDBDCE150002000102FF

BDBDBDBD:Header

CE:Message ID

15:Type Drop Sensitivity Setting

00 : Fixed value

0200:Len

00:Param Type Height Setting

02:Param Level Height Level 2-->1.5m

FF:checksum

5.1.8 Setting - Vibration Setting (0XCE23)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Decription |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Fixed to 0x23 |
| 1 | U8 | Valid | 1 | 0x00 |
| 2 | U16 | Cfg Len | 2 | Length of content after parameter |
| 1 | U8 | Motor Type | 1 | Setting type: 1. Vibration duration - 0x00 |
| 4 | U32 | Value | 4 | Setting content: 1. Vibration duration (unit: milliseconds, range: 0 - 60 seconds) |

Setting - Vibration setting definition: This message sent downstream can modify the global vibration duration, and any vibration duration will be changed to the duration set by this command.

Example:

Set the device's default global vibration duration to 1.5 seconds (1500 milliseconds):BDBDBDBDCE2300050000dc050000FF

BDBDBDBD:Header

CE:Message ID

23: Type 23 Setting - Vibration Setting

00: Fixed value 00 - always valid

0500:The length of the following parameter content is converted to big-endian 0005-->converted to decimal 05-->the length of the following parameter content is 5 bytes

00: Motor Type Vibration Duration Setting

dc050000: Value Vibration duration converted to big-endian 000005dc-->converted to decimal 1500-->vibration duration is 1500 milliseconds (1.5 seconds)

FF:checksum

5.1.9 Settings - Interface Display and Hide Settings (0xCE27)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Decription |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Type is 0x27 |
| 1 | U8 | Valid | 1 | Fill with 0x00, no impact for now |
| 2 | U16 | Len | 2 | Parameter content length |
| 1*N | U8*N | Param Type | 1*N | [Bit 7] Whether to enable: 0x00: Hide, 0x01: Show [Bit 6:0] Interface type: 0x01: Heart rate, 0x02: Blood pressure, 0x03: Blood oxygen, 0x04: HRV (This interface is not available in the general version, but in the special version), 0x05: Body temperature, 0x06: Step counting, 0x07: Exercise |

Example:

Hide heart rate, blood pressure, and step counting interfaces, and display the blood oxygen interface: BDBDBDBDCE2700040001020683FF

BDBDBDBD: Header

CE:Message ID

27:Type 27 Interface display and hide settings

00: Fixed value

0400: Parameter content length converted to big-endian0004--> converted to decimal04--> parameter content length is4bytes

01: Convert to binary --->00000001 bit7:0-->00 Hidden bit6-0:0000001-->01Heart rate

02: Convert to binary --->00000010 bit7:0-->00 Hidden bit6-0:0000010-->02Blood pressure

06: Convert to binary --->00000110 bit7:0-->00 Hidden bit6-0:0000110-->06Step counting

83: Convert to binary --->10000011 bit7:1-->01 Show bit6-0:0000011-->03Blood oxygen

FF:checksum

5.1.10 IP & Domain Name Settings (MSGID=0xC3) (TCP Only)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | Issue type: Type =1 IPv4, Type =2 IPv6 (reserved - not supported yet), Type =3 Domain name (only supports ASCII encoded domain names for now) |
| 2 | U16 | Port |  |  | Port number (2 bytes) |
| 1 | U8 | Len |  |  | Length of content after parameter |
| N | U8 | Domainname |  |  | Specific IP or domain content |

Note: After distribution, the device will no longer communicate on the original server, but will point to the modified server.

Example:

Type=1 IPv4:Downlink modifies IP to:118.178.184.219, port: 8825 BDBDBDBDC30179220476B2B8DB33

BDBDBDBD:Header

C3:Message ID

01:Type 1--IPv4

7922:Port Convert to big-endian 2279-->Convert to decimal 8825-->Port: 8825

04:Len The length of the content of the following parameters Convert to decimal 04-->The length is 4 bytes

76B2B8DB:Domainname IP 76-->Convert to decimal 118, B2-->Convert to decimal 178, B8-->Convert to decimal 184, DB-->Convert to decimal 219 IP is: 118.178.184.219

33:checksum

Type=3 Domain nameDownlink modifies domain name to:aiday.com.cn, port: 8825

BDBDBDBD:Header

C3:Message ID

03:type=3, domain name

7922:Port Convert to big-endian--2279 -->Convert to decimal Port: 8825

12:Len The length of the content of the following parameters Convert to decimal 18-->The message length is 18 bytes

61696461792e636f6d2e636e:Domainname domain name converted to string --> aiday.com.cn --> domain name is: aiday.com.cn

1D:checksum

5.1.11 Issue sedentary stay alarm trigger time (MSGID=0XCC)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCC | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | minute | -/- | -/- | Stay alarm trigger duration (unit: minutes; value range 2 — 60), inactivity within this time length will trigger a stay alarm |

Example:

Report stay alarm on the watch after 5 consecutive minutes of inactivity (when the watch is worn): BDBDBDBDCC05DD

BDBDBDBD: Header

CC: Message ID

05: minute Stay alarm trigger duration Convert decimal 05-->5 minutes

DD: checksum

5.1.12 Shutdown/Restart (MSGID=0x77)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x77 | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- |  | 01 - Shutdown, 00 - Restart |

Note: Shutdown and restart are both received in the power-on state. Shutdown: The watch shuts down after receiving the command in the power-on state, and no commands can be received after shutdown; Restart: The watch restarts after receiving the command in the power-on state.

Example:

Shutdown:BDBDBDBD770193

BDBDBDBD:Header

77: Message ID

01:Type 01--Shutdown

93:checksum

Restart:BDBDBDBD770093

BDBDBDBD:Header

77: Message ID

01:Type 00--Restart

93:checksum

5.1.13 Personal Information Issuance (MSGID=0xCA)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCA | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | count | -/- |  | Number of information groups issued |
| 1 | U8 | type | -/- |  | Type: 1 – Name, 2 – English Name, 3 – Phone, 4 – Blood Type |
| 1 | U8 | len | -/- |  | Length |
| N | U8 | content | -/- |  | Content (Unicode encoding) |
| ... | ... | ... | ... | ... | ... |

Example:

Name: Cheng Da Wen, English Name: Chan Tai Man, Phone:99886633, Blood Type: A

BDBDBDBDCA0401060B7A2759876502184300680061006E00200054006100690020004D0061006E0003103900390038003800360036003300330004024100DD

BDBDBDBD: Header

CA: Message ID

04:count to decimal04-->4 groups of information

01:type 01--Name

06:len to decimal06-->Length 6 bytes

0B7A27598765:content

0B7A converted to big-endian-->7A0B-->Convert Unicode encoded characters-->Cheng,

2759converted to big-endian-->5927-->Convert Unicode encoded characters-->Da,

8765converted to big-endian-->6587-->Convert Unicode encoded characters-->Wen Chinese name is Cheng Da Wen

02:type 02--English name

18 len to decimal24-->Length 24 bytes

4300680061006E00200054006100690020004D0061006E00:content

Similarly, each 2 bytes are converted to big-endian --> convert to Unicode character --> final: Chan Tai Man

03:type 03--Phone number

10:len Convert to decimal 16-->Length 16 bytes

39003900380038003600360033003300:content

Similarly, each 2 bytes are converted to big-endian --> convert to Unicode string --> final: 99886633

04:type 04--Blood type

02:len Convert to decimal 02-->Length 2 bytes

41: Convert to Unicode string -->A--> Blood type: A

DD: checksum

5.1.14 Sleep Statistics Time Period Setting (MSGID=0X1D)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x1D | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 1 |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 2 |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 3 |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 4 |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |

Example:

The sleep data statistics and reporting time period is 13:00 - 15:00 and 22:30 - 08:00BDBDBDBD1D010D000F0001161E080000000000000000000000FF

BDBDBDBD:Header

1D:Message ID

01:enable Whether to enable Convert decimal 01-->1--Enable

0D:time_start_h Start time - hour Convert decimal 13

00:time_start_m Start time - minute Convert decimal 00 Start statistics and report time is 13:00

0F:time_end_h End time - hour Convert decimal 15

00:time_end_m End time - minute Convert decimal 00 End statistics and report time is 15:00

01:enable Whether to enable Convert decimal 01-->1--Enable

16:time_start_h Start time - hour Convert decimal 22

1E:time_start_m Start time - minute Convert decimal 30 Start statistics and report time is 22:30

08:time_end_h End time - hour Convert decimal 08

00:time_end_m End time - minute Convert decimal 00 End statistics and report time is 08:00

00: enable whether to enable Convert to decimal 00-->0--Not enabled

00000000: All fill 00 in disabled state

00: enable whether to enable Convert to decimal 00-->0--Not enabled

00000000: All fill 00 in disabled state

FF:checksum

5.1.15 Remote OTA upgrade distribution (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Number of types |
| 1 | u8 |  | -/- | -/- | Fixed value (reserved byte 00) |
| 1 | u8 | Type | -/- | -/- | Type 1 00—mcu firmware |
| 1 | U8 | PathLen | -/- | -/- | Path 1 length |
| n | N*u8 | Path | -/- | -/- | Path |
| 2 | U16 | PackageSize |  |  | File package size is fixed at 0000 |
| ... | ... | ... | ... | ... | ... |
| 1 | u8 | Type | -/- | -/- | Type n 00—mcu firmware |
| 1 | U8 | PathLen | -/- | -/- | Path n length |
| n | N*u8 | Path | -/- | -/- | Path |
| 2 | U16 | PackageSize |  |  | File package size is fixed at 0000 |
| ... | ... | ... | ... | ... | ... |

Note: Remote updates consume traffic and are affected by the network. The device's SIM card has 30M per month, so it cannot be updated too many times, otherwise it will cause insufficient traffic.

Example:

The firmware download address is (requires server support for http download):http://tools.aiday.com.cn/File/MCU/W200PG/W200PG_E42.BWGHOL25.bin

After the update is successful, the device will restart and reconnect to the server, reporting a version number change

BDBDBDBDA901000041687474703A2F2F746F6F6C732E61696461792E636F6D2E636E2F46696C652F4D43552F5732303050472F5732303050475F4534322E425747484F4C32352E62696E00000A

BDBDBDBD:Header

A9:Message ID

01:TypeCnt Number of types 01--Indicates1type

00: Reserved byte, fixed value

00:Type 00--Indicates MCU firmware upgrade

41:Path Download path length Decimal conversion 65-->Download path length is 65

687474703A2F2F746F6F6C732E61696461792E636F6D2E636E2F46696C652F4D43552F5732303050472F5732303050475F4534322E425747484F4C32352E62696E

File download path Convert to string--> http://tools.aiday.com.cn/File/MCU/W200PG/W200PG_E42.BWGHOL25.bin

0000:PackageSize Currently fixed at 0000

0 A: checksum

5.1.16 Weather warning (MSGID=0XCB)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCB | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Count | -/- | -/- | How many groups of weather warnings are there? |
| 1 | U8 | Type | -/- | -/- | Weather alert type, see below for specific types |
| 1 | U8 | Status | -/- | -/- | Status, see below for specific statuses |
| … | … | …… |  |  | … |

Weather type - status table:

| Type | Decription | Status | notes - Click image to enlarge |
| --- | --- | --- | --- |
| 1 | Storm warning | 0 | Cancel alarm |
|  |  | 1 - 8 |  |
| 2 | Heavy rain warning | 0 | Cancel alarm |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |
| 3 | Thunderstorm Warning | 0 | Cancel alarm |
|  |  | 1 |  |
| 4 | Flooding Warning | 0 | Cancel alarm |
|  |  | 1 |  |
| 5 | Landslide Warning | 0 | Cancel alarm |
|  |  | 1 |  |
| 6 | Strong Monsoon Warning | 0 | Cancel alarm |
|  |  | 1 |  |
| 7 | Frost Warning | 0 | Cancel alarm |
|  |  | 1 |  |
| 8 | Fire Danger Warning | 0 | Cancel alarm |
|  |  | 1 |  |
|  |  | 2 |  |
| 9 | Temperature Warning | 0 | Cancel alarm |
|  |  | 1 |  |
|  |  | 2 |  |
| 10 | Tsunami Warning | 0 | Cancel alarm |
|  |  | 1 |  |
| 11 | Heat Index | 0 | Cancel alarm |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |

Note: After issuance, the device vibrates to display weather warning images. Multiple sets of weather warnings can be issued simultaneously, but each set must be different. The same type of warning cannot appear in one command, otherwise it will be invalid.
Example:

10Groups of weather warnings issued: BDBDBDBD CB 0A 0101 0201 0301 0401 0501 0601 0701 0801 0901 0A01 F5

BDBDBDBD:Header

CB:Message ID

0A:Count Number of weather groups Convert to decimal10-->Issued10groups of weather

0101:Type&Status Weather type-status01--Typhoon warning01--1Alert signal

0201:Type&Status Weather type-status02--Rainstorm warning01--Yellow rainstorm warning signal

0301:Type&Status Weather type-status03--Thunderstorm warning01--Thunderstorm warning

0401:Type&Status Weather type-status04--Flooding warning01--Special flood report for the northern New Territories

0501:Type&Status Weather type-status05--Landslip warning01--Landslip warning

0601:Type&Status Weather type-status06--Strong monsoon warning01--Strong monsoon signal

0701:Type&Status Weather type-status07--Frost warning01--Frost signal

0801:Type&Status Weather type-status08--Fire danger warning01--Yellow fire danger warning

0901:Type&Status Weather Type-Status 09--Temperature Warning 01--Cold Weather Warning

0A01:Type&Status Weather Type-Status 10--Tsunami Warning 01--Tsunami Warning

F5:checksum

5.1.17 Restore Factory Settings (MSGID=0X7B)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x7B | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | reset | -/- | -/- | 1 Restore Factory Settings |

Example:

Restore factory settings, the device restarts after receiving the command:BDBDBDBD7B018F

BDBDBDBD:Header

7B:Message ID

01:reset 01--Restore Factory Settings

8F:checksum

5.1.18 Special Firmware Support - Sending SOS Prompt Message (MSGID=0X80)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x80 | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Content length |
| N | N*U8 | Content | -/- |  | Content - Unicode encoding, up to 50 characters |

Note: This issuing command is supported by special firmware, and there is no general version for the time being. After the device triggers SOS, it will enter the SOS interface. This command can control the display text content.
Applicable scenarios: When the user encounters an emergency and triggers SOS, the content displayed on the SOS interface can take corresponding actions based on the SOS interface prompt information.

Example:

SOSInterface prompt text: SOS alarm has been sent BDBDBDBD801053004F005300A562668BF25DD153FA51DD

BDBDBDBD:Header

80:Message ID

10:length Content length Convert to decimal16-->Content length is16bytes

53004F005300A562668BF25DD153FA51:Every2bytes are converted to big-endian --> Convert to Unicode characters --> Final: SOS alarm has been sent

DD:checksum

5.1.19 Special Firmware Support - Issue Text Message Cleanup Time (MSGID=0X2A)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x2A | See below for definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | 01 Message Scheduled Cleanup |
| 1 | U8 | Len |  |  | Length of the following message |
| 1 | U8 | hours | -/- |  | Message cleanup time (unit: hours) |

Note: This issued command is for special firmware support, the general version does not have it for now
Example:

Note:If the message cleanup time is 00, the received text message will be cleaned up immediately after receiving the commandBDBDBDBD2A010102DD

BDBDBDBD:Header

2A:Message ID

01:Type 01--Text message scheduled cleanup

01:Len The length of the subsequent message is converted to decimal 01-->The length of the subsequent message is 1 byte (excluding checksum)

02:hours Convert to decimal 02-->Text message cleanup time is 2 hours

DD: checksum