W300GU-4GCat.1-MQTT Communication Protocol

Table of Contents

1 Overview	4

1.1 Transmission Rules	4

1.2 Topic Definition	4

2 Device Instructions	4

2.1 Device Function and Instructions	4

2.2 Device default reporting logic	7

2.3 Device downlink instructions	8

3 Protocol Data Packet Structure	13

3.1 Data Header	13

3.2 Message Identifier (Message ID)	13

3.3 Token Generation Mechanism	13

3.4 Payload	13

3.5 Checksum	15

4 Report messages message	16

4.1 Battery signalRelated reporting	16

4.1.1 Battery signal upload(MSGID=0xF9)	16

4.2 Alarm-related reporting	18

4.2.1 Alarm data upload-1 (MSGID=0x02)	18

4.2.2 Alarm data upload-2(MSGID=0x21)(Supplement to 02)	22

4.2.3 Abnormal Data Alarm (MSGID=0x16)	27

4.3 Location-related Reporting	28

4.3.1 GPS/BDS Location Reporting: Location Data Reporting (MSGID=0x03)	28

4.3.2 WiFi and base station information upload (MSGID=0xA4)	30

4.3.3 Bluetooth Location Information(BLE Location) (MsgId=0xD6)	35

4.3.4 Base station latitude and longitude reporting (MSGID=0x15) — wifi positioning supplement	37

4.3.5UWB Location Information (MsgId=0xD7)	39

4.4 Device Information and Status Reporting	40

4.4.1 ICCID Upload of SIM Card (MSGID=0xF3)	40

4.4.2 Device Charging Status Upload (MSGID=0xC3)	41

4.4.3 Status parameter report (MSGID=0xA9)	43

4.4.4 Device Status (MSGID=0xE9)	45

4.  5 Health-Related Reporting	46

4.  5.1 Health Data (MSGID=0x32)	46

4.5.2 Device sleep analysis data upload (MSGID=0xC5)	48

4.6 Downlink Feedback Related Reporting	49

4.6.1 Downlink Feedback (MSGID=0xC0)	49

4.6.2 Message Status Report (MSGID=0x28)	50

5 Settings	52

5.1 Downlink	52

5.1.1 Set location reporting frequency (MSGID=0x17)	52

5.1.2 Message Send (MSGID=0X28)	54

5.1.2 Settings - Location Priority Settings (0XCE01 & 0XCE02)	55

5.1.4 Settings - Health Sampling Reporting Frequency Settings (MSGID=0X1C)	59

5.1.5 Settings - Device Alarm Settings (0XCE03)	61

5.1.6 Setting - Switch Setting (0XCE04-24)	63

5.1.7 Settings - Drop Sensitivity Setting (0xCE15)	72

5.1.8 Settings - Vibration Settings (0XCE23)	73

5.1.9 IP & Domain Name Settings (MSGID=0xC3)	75

5.1.10 Issue sedentary stay alarm trigger time (MSGID=0XCC)	76

5.1.11 Shutdown and Restart (MSGID=0x77)	77

5.1.12Sleep Statistics Time Period Setting (MSGID=0X1D)	78

5.1.13Weather Warning (MSGID=0XCB)	80

5.1.14UWB Alarm Distance Issuance (0x7900)	84

5.1.15UWB Ranging Alarm Configuration (0x7901)	85

1 Overview

This protocol is suitable for 4G CAT1 devices, currently supporting the W300GU product. It uses a 32-bit data header for synchronization and terminal identification; employs a low-overhead checksum algorithm for checksum protection; and uses message identifiers to identify different messages.
It needs to be parsed according to the actual report. The document is currently being continuously maintained. If you find any errors or omissions, please give feedback in time. Thank you very much.
The protocol content is 4g reporting content. If you need the Bluetooth broadcast data protocol, please consult relevant docking personnel.

1.1 Transmission Rules

Data is transmitted in hexadecimal, and integer data is transmitted in little-endian byte order.

1.2 Topic Definition

Device reporting data Topic: oc_device_report/devices/{Device ID}/properties

Device issuing data Topic: oc_device_report/devices/{Device ID}/down

Downlink format: Downlink raw message16Hexadecimal

Among them, the device ID is the device IMEI number

2 Device Instructions

2.1 Device Function and Instructions

General version:

(1) Power on:

Please fully charge the device before using it for the first time. The device will automatically power on when the charging reaches the power required for power on. The charging status displays the charging icon.

Fully charged is a green icon. Note: Do not check the device signal while charging

Manual power on: Press and hold the up button for 10s and release, the interface will display "Welcome"

Note: The default wearing state is on at startup, and the drop-off alarm is reported if no heart rate is detected.

(2) Shutdown:

Low battery shutdown: The screen displays Byebye and then turns off

Manual shutdown: Shutdown: Press and hold button 1 for more than 10 seconds, the interface displays Byebye and then the screen goes off

Downlink command shutdown: Downlink shutdown command in power-on state, the interface displays Byebye and then the screen goes off Note: Charging does not shut down in power-on state

(3) SOS function:

Trigger mode: After triggering, the device does not enter sleep mode, long press the down key for 3s, the interface displays SOS SEND OK/SOS sending successfully, and then the SOS icon appears

Cancel mode: In SOS mode, long press the down key for 3s, the interface displays SEND CENCEL/SOS canceled, and the SOS icon disappears

(4) Signal status:

No signal: The interface signal is a horizontal line There is a signal: The interface signal has a stepped bar graph

(5) Device sleep:

Trigger condition: The device does not move for 40 minutes and enters sleep mode, does not report location health data, only reports F9 heartbeat packet to maintain long connection

(6) Device parameters: Settings - Parameters interface: A total of multiple pages, which can be switched by sliding the screen

sos(Yes): Yes: Indicates that the device can be triggered by pressing the buttonsos,

sos(No): Indicates that thesos cannot be triggered by pressing the button

Health(10min): Indicates that the device has enabled the health report function,

10min: Indicates that the current health data reporting frequency is10minutes,

Health(No): Indicates that the device health report function has been turned off

Location(BWG,10min): Indicates that the device has enabled the location report function,

BWG: Indicates the location priority Bluetooth\>wifi>Satellite (GPS/BDS),

10min: Indicates that the current location reporting frequency is10minutes,

Location(No): Indicates that the device location report function has been turned off

Fall(Medium,1.5m): Indicates that the device has enabled the fall alarm trigger function,

Middle: Indicates that the device's drop alarm trigger sensitivity is medium,

1.5m: Indicates that the device drop alarm trigger height is1.5m,

Drop (No): Indicates that the device has turned off the drop alarm trigger function

Sedentary (2min): Indicates that the device has turned on the sedentary stay alarm trigger function,

2min: Indicates that the time to trigger the sedentary stay alarm is2minutes without moving,

Sedentary (No): Indicates that the device has turned off the sedentary stay alarm trigger function

Health Broadcast (Yes):

Yes: Indicates that the device has turned on Bluetooth to broadcast health data,

No: Indicates that the device has turned off Bluetooth to broadcast health data

Sleep statistics reporting time: (21:00-08:00), indicating that the sleep statistics reporting time is from21:00 to8:00 the next day

Global vibration duration (1.500s): The current device vibration duration is1.5seconds

Health alarm threshold: such as heart rate threshold (90,100)BPM, indicating that the normal heart rate range is90-100,

Normal range will report health abnormal alarm, heart rate threshold (No): indicates that the heart rate threshold is closed

Allow work hibernation (Yes):

Yes: Indicates that the device has turned on the sleep function, No: Indicates that the device has turned off the sleep function

Note: Sleep function refers to the device falling off state does not report data, the device 40minutes stationary does not report data

Network long connection (Yes):

Yes: Indicates that the current device tcpconnection mode: long connection (always maintain the connection state, can receive downlink instructions in real time),

No: Indicates short connection (disconnect after reporting, the device accepts downlink instructions when reporting data)

GPSAlways on switch (No):

Yes: The internal interface is always open to collect satellite (GPS/BDS), which can generally speed upsatellite (GPS/BDS)positioning time, and the power consumption will increase,

Defaults to No, and does not need to be enabled under normal circumstances.

(8) Vibration:

Receive the downlink text message, health threshold alarm (the interface has text reminders and vibration)

The vibration duration can be controlled by downlink (see section 2.3)

2.2 Device default reporting logic

General version:

(1) Location-related reporting

UWB/Satellite (GPS/BDS)/wifi/Bluetooth beacon: Default reporting frequency10minutes, default positioning priority: UWB>wifi>Satellite (GPS/BDS), UWB positioning is preferred, switch to WIFI positioning if positioning fails, no switching if positioning is successful

Base station (0x15): After wifipositioning fails, automatically request the communication base station's latitude and longitude from the module.---This is only supported in mainland China, not in Hong Kong and overseas. The accuracy is not high and is used for auxiliary reference. Generally, there is no such reporting in the environment. The positioning priority cannot be issued and is bound to wifipositioning.

If there is a base station in the wifipositioning message (A4), there will be no base station (0x15) message

(2) Alarm-related reporting

SOSAlarm (0x02): Triggered by the user, see the previous section for the triggering method

SOS Cancel (0x02): User actively triggers cancellation, see the previous section for triggering methods

Shutdown Alarm (0x21): Device actively shuts down or shuts down due to low battery, see the previous section for triggering methods

Wearing Drop Alarm (0x02): The device determines wearing based on the health sampling reporting frequency. It reports a wearing alarm when a heart rate is measured, and a drop alarm when no heart rate is measured.

Sedentary Stay Alarm (0x02): Default 5 minutes of immobility triggers reporting

Fall Alarm (0x02): The device free falls from a certain height and meets the fall algorithm trigger

Low Battery Alarm (0x02): Triggered when the device's current battery level is less than or equal to 0

Abnormal Temperature Alarm (0x16): Reported when the device detects a temperature exceeding 40 degrees Celsius

Health Threshold Alarm (0x2105): No alarm by default. After the downlink health threshold, if the health data is not within the threshold range, an alarm is reported.

Note: When the health threshold alarm is reported, the device vibrates and the interface displays abnormal values.

UWB Ranging Alarm (0x21): Default is on. All beacons will trigger the alarm. After setting the downlink UWB ranging alarm beacon, it is triggered when the device is less than the issued UWB alarm distance.

Vibration real-time trigger, wearing status reported to the server1only once within minutes; non-wearing status is not reported to the server, and no vibration is generated

(3) Health-related reporting

Step counting, heart rate, body temperature&wrist temperature, blood pressure, blood oxygen (0x32): Default reporting frequency10minutes

Sleep (0xC5): Statistics time period21:00-08:00, the sleep data will be reported according to the status during this time period

(4) Device information and status reporting

Status parameter (0xA9): A report will be made when the device is turned on

SIM card ICCID (0xF3): A report will be made when the device is turned on

Device status (0xE9): A report will be made when the device is turned on, and a report will be made when the reporting frequency is changed

Charging status (0xC3): Report when charging starts, charging ends, and when fully charged

Battery signal (0xF9), reported every 4 minutes by default, (previous firmware) a report will also be made after positioning and health reporting

(6) Downlink feedback

Downlink feedback (0xC0): The server's downlink command is reported after the device receives it.

Message status feedback (0x28): After the downlink text message, the device clicks accept or reject.

Note: The device reports in a combined packet, that is, one data packet contains multiple complete messages. Pay attention not to miss any. The message is a complete message, and there will be no interruption in the middle in the next data packet.

Eg:BDBDBDBDD6000119A9CF610445270387BF452708A1BC44279D18B74427E518B7F9BDBDBDBDF9010000006400002800000019A9CF61CA

This data packet contains messages for (0xD6) Bluetooth positioning and (0xF9) power signal.

(0xD6) Bluetooth positioning: BDBDBDBDD6000119A9CF610445270387BF452708A1BC44279D18B74427E518B7F9

(0xF9) Power signal: BDBDBDBDF9010000006400002800000019A9CF61CA

2.3 Device downlink instructions

General version: The downlink command needs to be continuously sent twice within 20 milliseconds to ensure success.

(1) Device location reporting frequency distribution (0x17):

The default reporting frequency is 10 minutes, with a maximum of 1 minute and a minimum of 1440 minutes. After the device receives the downlink command, the device will follow the time period and frequency of the issued command.

Report data, do not report outside the time period, such as: 00:00-18:00. Report location every 2 minutes, then do not report outside the time period.

(2) Text message delivery (0x28):

Unicode encoding, up to 40 Chinese characters, one Chinese character occupies 2 bytes, and one English letter occupies 2 bytes

Message list - historical messages can save up to 50 items (Note: Firmware without a message list can save up to 20 items)

(3) Device positioning priority delivery (0xCE01):

The default positioning priority is uwb>wifi>satellite (GPS/BDS), positioning priority

If the delivered positioning priority is: wifi>satellite (GPS/BDS)>Bluetooth beacon, then if wifi cannot be located, switch to satellite (GPS/BDS), and if satellite (GPS/BDS) cannot be located, switch to

Bluetooth beacon. When positioning is successful, the next positioning priority will not be switched to generate positioning.

(4) Device health sampling frequency delivery (0x1C): —Note that it is different from W200P

The default health sampling frequency is 10 minutes, up to 2 minutes. After the device receives the downlink command, the device presses the issued command.

Time period and frequency reporting data, no reporting outside the time period, such as: 00:00-18:00 10-minute positioning reporting, then no reporting outside the time period

(5) Modify IP and port command issuance (0xC3):

The server IP (or domain name) and port pointed to can be modified downstream

(6) Sedentary stay alarm trigger time issuance (0xCC):

Value range: 2 minutes - 60 minutes, such as: downstream is 10 minutes trigger time, then the device will trigger and report sedentary stay alarm if it does not move for 10 minutes

(7) Device shutdown and restart issuance (0x77):

Note that this command must be in the device's power-on state to take effect, and cannot be received after shutdown

Shutdown issuance: The device shuts down when this command is sent downstream in the power-on state, and cannot receive downstream after shutdown

Restart issuance: The device restarts when this command is sent downstream in the power-on state

(8) Drop sensitivity and height issuance (0xCE15):

The latest general firmware default drop sensitivity: High (Medium-High). Drop height: 0.5m

Sensitivity: Refers to the degree to which the drop algorithm is satisfied, providing 5 setting levels (0 - 4): Low - Lower (Medium Low) - Medium - Higher (Medium High) - High.

Height: Refers to the height that satisfies the triggering of the fall alarm, providing 5 setting levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m- 2.5m.

(9) Key shutdown switch command issued (0xCE16):

By default, the device can be shut down via a button. After the downlink is closed, the device cannot be shut down via a button.

(10) Control device to trigger sleep switch issued (0xCE18):

The default is the on state. The device enters sleep mode if it is not moved for 40 minutes. After the downlink is closed, the device does not enter sleep mode.

(11) Fall alarm switch (0xCE07):

The default fall alarm is in the on state. After it is turned off, the device does not report fall alarms.

(12) Key trigger sos start switch (0xCE19)

The default is the on state. Long pressing the charging cable button can trigger sos. After it is turned off, long pressing the charging cable button will not trigger the reporting of sos.

(13) Device Health and Location Reporting Switch (0xCE20)

The default health reporting is enabled, and the default location reporting is enabled.

You can set the location / health on and off states separately. After closing, the device will not report health / location.

(14) Health Threshold Setting (0xCE0300)

Health threshold is issued, downlink instruction. After the device receives it, the normal range of health threshold is issued. Health data outside this range will report an alarm.

(15) Device Vibration Setting (0xCE23)

After the downlink instruction, after the device receives it, the device vibration duration becomes the downlink duration, ranging from 0-60 seconds.

(16) Report Location Immediately (0xCE0A)

After the downlink instruction, after the device receives it, the device immediately starts positioning and reports.

(17) Report Health Immediately (0xCE0B)

After receiving the downlink command, the device immediately starts health sampling and reports it.

(18) Sleep Statistics Time Period Distribution (0x1D)

Different time periods can be distributed, such as 13:00-14:00, then sleep statistics data will be collected and reported within this time period.

(19) GPS Always-On Switch (0xCE24):

When enabled, the internal interface will always collectSatellite (GPS/BDS). In general environments, this can speed upSatellite (GPS/BDS)positioning time, but power consumption will increase. The default is off.

(20) Sedentary Stay Alarm Switch (0xCE08)

Enabled by default, the trigger time is 15 minutes. After the downlink is closed, the device will not report sedentary stay alarms.

(21) Weather Warning Distribution (0xCB)

Multiple sets of weather warnings can be distributed. Note that each set of weather warnings must be different, and the same type of warning cannot appear.

(22) Data Reporting Switch (0xCE09):

Enable and disable data reporting. The default is enabled. After it is turned off, the device only reports 0xF9 messages to maintain a long connection.

(23) UWB alarm distance (0x7900):

Controls the distance for UWB ranging alarms, where 0 meters means no alarm

(24) UWB ranging alarm configuration (0x7901):

The default is to enable the ranging alarm state, and the UWB beaconid that triggers the ranging alarm of the watch can be configured downstream

3 Protocol Data Packet Structure

A basic protocol data packet structure is shown in Figure 1:

3.1 Data Header

Each data packet starts with a 4-byte Header or token (in some response messages, timestamp is used instead):

Currently, the token uploaded by Aufu devices is fixed at BDBDBDBD

Header: 0xBD 0xBD0xBD0xBD;

Timestamp: 32bits, generated by the server

3.2 Message Identifier (Message ID)

See Chapter 4 for the content represented by MessageId.

3.3 Token Generation Mechanism

Currently fixed as BDBDBDBD

3.4 Payload

The payload below refers to the valid body content in the protocol, excluding the head token and checksum. The length of the body is noted in parentheses.

The data format used in the payload is shown in the table below:

[U-unsigned; I-signed; X-bitfield; Number - Number of bytes]

The following protocols use little-endian priority except for ch, u8, i8, and x8

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Explanation |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short Integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short Integer |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Integer |
| x16 | Bitfield | 2 | - | - | Bit 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long Integer |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit long integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating point type |

3.5 Checksum

The checksum includes the payload, as shown in Figure 1. The algorithm is as follows. By default, the general-purpose version of the device does not require checksum verification, and this part can be ignored. Any byte can be used for the downlink instruction.

//Suppose content="BDBDBDBDE9000600010A00000A00", then sum.ToString("X2") result is 07//The following is the C# code calling methodprivate string CheckSum(string content)

{

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2");

}

4 Report messages message

4.1 Power signal related report

4.1.1 Power signal upload (MSGID=0xF9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xF9 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery type 0: 4-level system, 1: 5-level system, 2: Percentage, 3: Voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery value If Bat_type is 0: The battery value range is 0-3 (0 is 25%, 3 is 100%) If Bat_type is 1: The battery value range is 0-4 (0 is 20%, 4 is 100%) If Bat_type is 2: The battery value range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: Percentage, 1: 5-level system, 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal strength |
| 1 | U8 | Other_type |  |  | Extended type 0: Full step counting 1: Incremental step counting 2: Vibration |
| 4 | U32 | Num |  |  | Extended value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Example:

Original16hexadecimal message: BDBDBDBDF90104000050000095000000E377BD67AA

BDBDBDBD: Header

F9:Message ID (Message ID)

01:Bat_type Battery Type 01-->1-->5-level battery (0-4)

0400:Bat_volt Battery Value Converted to Big End 0004-->4-->Level4corresponds to100% battery

00:Signal_type Signal Type 00-->0-->Percentage

5000:Signal_strength Signal Strength Converted to Big End0050-->Converted to Decimal80-->80% Signal Strength

00:Other_type Extension Type 00-->0-->Full Step Counting

95000000:Num Extension Value - Step Counting Converted to Big End00000095-->Converted to Decimal149-->Number of Steps is149

E377BD67:Timestamp Timestamp Converted to Big End67BD77E3-->Converted to10Hexadecimal1740470243-->Timestamp is1740470243seconds--> Converted to Standard Time Format UTC Time:2025-02-25 07:57:23-->Converted to Beijing Time:2025-02-25 15:57:23

AA:checksum

4.2 Alarm-related reporting

4.2.1 Alarm data upload-1 (MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x02 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below for definition |
| 4 | U32 | Timestamp | - | - | Timestamp (a timestamp will be added at the end during retransmission) |

Upl_warn contents

| bit (corresponding to the position of 1 in binary) | Name | Description | Original message little-endian hexadecimal | Hexadecimal converted to big-endian | Decimal |
| --- | --- | --- | --- | --- | --- |
| 14 | Fall alarm | Fall alarm | 0040 | 4000 | 4*4096=16384 |
| 8 | Device wearing | Device wearing | 0001 | 0100 | 1*256=256 |
| 7 | SOS canceled | SOS canceled | 8000 | 0080 | 8*16=128 |
| 5 | Sedentary alarm (immobile) | Sedentary alarm (immobile) | 2000 | 0020 | 2*16=32 |
| 4 | Remove (detach) device | Remove (detach) device | 1000 | 0010 | 1*16=16 |
| 2 | Shutdown | Shutdown | 0400 | 0004 | 4 |
| 1 | SOS alarm | SOS alarm | 0200 | 0002 | 2 |
| 0 | Low battery | Low battery | 0100 | 0001 | 1 |

Example:

Wearing alarm: BDBDBDBD020001E377BD678A

BDBDBDBD:Header

02:Message ID

0001:Upl_warn alarm content, converted to big-endian:0100-->Wearing alarm

E377BD67: Timestamp converts the timestamp to big-endian 67BD77E3-->Convert to10Hexadecimal1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

8 a: checksum

Detachment alarm: BDBDBDBD021000E377BD678A

BDBDBDBD:Header

02:Message ID

1000 : Upl_warn alarm content, converted to big-endian 0010 --> detachment alarm

E377BD67: Timestamp converts the timestamp to big-endian 67BD77E3-->Convert to10Hexadecimal1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

8 a: checksum

SOS alarm: BDBDBDBD020200E377BD678A

BDBDBDBD:Header

02:Message ID

0200 : Upl_warn alarm content, converted to big-endian 0002 --> SOS alarm

E377BD67: Timestamp converts the timestamp to big-endian 67BD77E3-->Convert to10Hexadecimal1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

8 a: checksum

SOS cancellation: BDBDBDBD028000E377BD678A

BDBDBDBD:Header

02:Message ID

8000 : Upl_warn alarm content, converted to big-endian 0080 --> SOS cancellation

E377BD67: Timestamp converts the timestamp to big-endian 67BD77E3-->Convert to10Hexadecimal1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

8 a: checksum

Fall alarm: BDBDBDBD020040E377BD678A

BDBDBDBD:Header

02:Message ID

0040 : Upl_warn alarm content, converted to big-endian 4000 --> fall alarm

E377BD67: Timestamp converts the timestamp to big-endian 67BD77E3-->Convert to10Hexadecimal1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

E7: Checksum

Sedentary alarm: BDBDBDBD022000E377BD678A

BDBDBDBD:Header

02:Message ID

2000: Upl_warn alarm content, converted to big-endian 0020 --> Sedentary alarm

E377BD67: Timestamp converts the timestamp to big-endian 67BD77E3-->Convert to10Hexadecimal1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

8A: Checksum

Low battery alarm: BDBDBDBD020200E377BD678A

BDBDBDBD:Header

02:Message ID

0100: Upl_warn alarm content, converted to big-endian 0001 --> Low battery alarm

E377BD67: Timestamp converts the timestamp to big-endian 67BD77E3-->Convert to10Hexadecimal1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

8A: Checksum

Shutdown alarm: BDBDBDBD020400E377BD678A

BDBDBDBD:Header

02:Message ID

0400: Upl_warn alarm content, converted to big-endian 0004 --> Shutdown alarm

E377BD67: Timestamp converts the timestamp to big-endian 67BD77E3-->Convert to10Hexadecimal1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

8A: Checksum

Note: This alarm is used for devices that do not distinguish between shutdown types. See the next section for devices that distinguish between shutdown types.

4.2.2 Alarm data upload-2(MSGID=0x21)(Supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x21 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | Alarm type |
| N |  | Upl_warn | - | - | Alarm content See below for different Alarm type definitions |
| 4 | U32 | Timestamp |  |  | Timestamp (a timestamp will be added at the end during retransmission) |

Alarm type=1

Upl_warn contents

| bit (corresponding to the position of 1 in binary) | Name | Description | Original message little-endian hexadecimal | Hexadecimal converted to big-endian | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging shutdown alarm | Device automatically shuts down when charging (firmware support required) | 04000000 | 0004 | 4 |
| 1 | Low battery shutdown alarm | Device shuts down due to low battery (firmware support required) | 02000000 | 0002 | 2 |
| 0 | Active shutdown alarm | The device was manually shut down (firmware requires function support) | 01000000 | 0001 | 1 |

Example:

Charging shutdown alarm: BDBDBDBD21010004000000E377BD6767

BDBDBDBD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001-->Alarm type=1

04000000:Upl_warn alarm content, converted to big-endian 0004-->Charging shutdown alarm

E377BD67:Timestamp, converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Low battery shutdown alarm: BDBDBDBD21010002000000E377BD6767

BDBDBDBD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001-->Alarm type=1

02000000: Upl_warn alarm content, converted to big-endian 0002 --> low battery shutdown alarm

E377BD67:Timestamp, converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Active shutdown alarm: BDBDBDBD21010001000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type Alarm type converted to big-endian 0001-->Alarm type=1

01000000:Upl_warn alarm content, converted to big-endian 0001-->Active shutdown alarm

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Alarm type=4

Upl_warn contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | 02—UWB ranging alarm |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple sets of information, and each group may contain multiple Ubeacons |
| U8 | Total_PackCount | 1 | Total number of alarm Ubeacons at the current time |
| U32 | UBeacon0 mac | 4 | UBeacon mac |
| U16 | Distance0 | 2 | Distance (in centimeters) |
| U32 | UBeacon1 mac | 4 | UBeacon mac |
| U16 | Distance1 | 2 | Distance (in centimeters) |
| U32 | UBeacon2 mac | 4 | UBeacon mac |
| U16 | Distance2 | 2 | Distance (in centimeters) |
| 。。。。。 | 。。。。。 | 。。。。。 | 。。。。。 |

Example:

Original hexadecimal message:BDBDBDBD210400020103320600009100E70500006800380500005C00E377BD6755

BDBDBDBD:Header

21:Message ID message id

0400:Alarm type is converted to big-endian 0004-->Alarm type=4

02:Type Convert to decimal 02--UWB ranging alarm

01:Total_groups Total number of groups converted to decimal 01, 1 group

03:Total_PackCount Total number of alarm Ubeacons at the current time Convert to decimal 03, 3 UWB beacons

32060000:UBeacon0 mac Converted to big-endian 00000632, UWB beacon id is 00000632

9100:Distance0 Distance Converted to big-endian 0091-->Converted to decimal 145-->The distance from the UWB beacon is 145 cm

E7050000:UBeacon1 mac

6800:Distance1 Distance converted to big-endian 0068-->converted to decimal 105-->The distance from the UWB beacon is 105 cm

38050000:UBeacon2 mac

5C00:Distance2 Distance converted to big-endian 005C-->converted to decimal 92-->The distance from the UWB beacon is 92 cm

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

55:checksum

Alarm type=5

Upl_warn contents

| Format | Name | Unit | Decription |
| --- | --- | --- | --- |
| U16 | Type | 2 | Fixed at 0x05 |
| U32 | Timestamp | 4 | Timestamp |
| U16 | Content Len | 2 | Alarm content length |
| U8 | Health Warn Type1 | 1 | Health threshold alarm type: 0x01 heart rate, 0x02 systolic blood pressure (SBP), 0x03 diastolic blood pressure (DBP), 0x04 blood oxygen , 0x05 temperature |
| U8 | Type1 | 1 | 01 is less than the set health threshold, 02 is greater than the set health threshold |
| I16 | Health Value1 | 2 | Health result value |
| U8 | Health Warn Type2 | 1 | Health threshold alarm type: 0x01 heart rate, 0x02 systolic blood pressure (SBP), 0x03 diastolic blood pressure (DBP), 0x04 blood oxygen , 0x05 temperature |
| U8 | Type2 | 1 | 01 is less than the set health threshold, 02 is greater than the set health threshold |
| I16 | Health Value2 | 2 | Health result value |
| .... | .... | .... | .... |

Example:

The device detects that the heart rate (65) is outside the range of the issued health threshold: BDBDBDBD210500E377BD6704000102410073

BDBDBDBD: Header

21:Message ID message id

0500:Alarm typeAlarm type Convert to big-endian0005-->Alarm type=5

E377BD67:Timestamp Convert to big-endian67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

0400:Content Len Alarm content length Convert to big-endian0004-->Alarm content length is 4 bytes, Note: The length will increase if there are multiple health threshold alarms

01:Health Warn Type Health threshold alarm type 01-->Heart rate alarm

02:Type Greater than the set health threshold & less than the set health threshold 01-->Less than the set health threshold

4100:Health Value Actually reported health value Convert to big-endian0041-->Convert to decimal 65-->Reported heart rate value is 65

73:checksum

4.2.3 Abnormal Data Alarm (MSGID=0x16)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x16 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type | -/- | -/- | Alarm type (0: abnormal heart rate value alarm, 1: abnormal temperature value alarm) |
| 2 | U16 | Heart | -/- | -/- | Heart rate value |
| 2 | U16 | temperature | -/- | -/- | Temperature |
| 2 | U16 | Pa | -/- | -/- | Air pressure |
| 4 | 4*U8 or U32 | expand | -/- | -/- | 4 bytes extension for other sensors |

Note: The reported outlier does not have any practical reference value, it only indicates that the current detected data is abnormal (such as the value exceeding the range or the sensor detecting abnormally, etc.). The outlier will be removed in other normal messages and is reflected in this message, so it can be ignored.

Example:

Temperature anomaly value alarm: BDBDBDBD16010000FF0F000000000000E6

BDBDBDBD:Header

16:Message ID

01:Type alarm type 01-->Temperature anomaly value alarm

0000:Heart rate value, default fill 0000 if no such value

FF0F:temperature temperature Convert to big endian 0FFF-->Temperature anomaly value FFF

0000:Pa air pressure, default fill 0000 if no such value

00000000 expand extension, default fill 0000 if no such value

E6:checksum

4.3 Location-related Reporting

4.3.1 GPS/BDS Location Reporting: Location Data Reporting (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x03 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude longitude |
| 8 | Double | lat |  | - | latitude latitude |
| 1 | U8 | north_south |  | - | /N or S/ North or South latitude |
| 1 | U8 | east_west |  | - | /E or W/ East or West longitude |
| 1 | U8 | status |  | - | /A or V/ Valid data or invalid data |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original16Hexadecimal message: BDBDBDBD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BDBDBDBD:Header

03:Message ID

963E870C3E675E40:lon Longitude, convert to big endian405E673E0C873E96-->Longitude:121.6131622Note: Refer to the parsing example below for conversion and parsing

FF2110C2B6343F40:lat Latitude, convert to big endian`403F34B6C21021FF-->Latitude:31.2059137Note: Refer to the parsing example below for conversion and parsing

4E:north_south  4E-->Convert to ASCII code N-->North latitude

45:east_west 45--> Convert to ASCII code E--> East longitude

41:status data validity 41-->convert to ASCII code A-->valid data

E377BD67:Timestamp timestamp converted to big endian67BD77E3-->convert to10base1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

78:checksum

Note: The parsedsatellite (GPS/BDS)latitude and longitude are in the WGS-84coordinate system. If the map uses Baidu Gaode, etc., the coordinate system needs to be converted

Parsing satellite positioning message java code example:

//Message: BDBDBDBD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BDBDBDBD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public static void main(String[] args){

//Message 7d9f84ac81815c40 actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d",16)));  //114.02353966666665//Message e766926b1d893640 actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16))) ;//22.535605166666667

HexToStr(data.Substring("4e")); //N

HexToStr(data.Substring("45"));//E

HexToStr(data.Substring("41"));//A A indicates valid data, and V indicates invalid data, which can be discarded//Message 749d695f actual value 5f699d74

Date date=new Date();

date.setTime(Long.parseLong("5f699d74",16)*1000);

SimpleDateFormatsdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.3.2 WiFi and base station information upload (MSGID=0xA4)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xA4 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 1 | u8 | Cell_cnt | 1 | - | Number of base stations (0-7) Number of cell info payload.Valid value:0~7 |
| 2 | u16 | Cell[0].MCC | - | - | MMC of base station (0) mobile country code of cell[0] |
| 2 | u16 | Cell[0].MNC | - | - | MNC of base station (0) mobile network code of cell[0] |
| 2 | u16 | Cell[0].LAC | - | - | LAC of base station (0) Location area code of cell[0] |
| 4 | U32 | Cell[0].CELL_ID | - | - | ID of base station (0) Cell id of cell[0] |
| 2 | i16 | Cell[0].RSSI | - | dbm | Signal strength RSSI of base station (0) RSSI in dbm of cell[0] |
| ... | ... | ... | ... | ... | ... |
| 2 | u16 | Cell[cell_cnt-1.MCC | - | - | MMC of base station (N) mobile country code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.MNC | - | - | MNC of base station (N) mobile network code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.LAC | - | - | LAC of base station (N) Location area code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1].LAC | - | - | LAC of base station (N) Location area code of cell[[cell_cnt-1] |
| 4 | U32 | Cell[cell_cnt-1].CELL_ID | - | - | ID of base station (N) Cell id of cell[[cell_cnt-1] |
| 2 | I16 | Cell[cell_cnt-1].RSSI | - | dbm | Signal strength RSSI of base station (N) RSSI in dbm of cell[[cell_cnt-1] |
| 1 | U8 | Wifi_cnt |  |  | Number of wifi Number 0f wifi |
| 6 | 6*U8 | Wifi[0].bssid[0]-Wifi[0].bssid[5] |  |  | MAC address of wifi(0) |
| 4 | I32 | Wifi[0].RSSI |  |  | RSSI of wifi(0) signal strength |
| ... | ... | ... | ... | ... | ... |
| 6 | 6*U8 | Wifi[Wifi_cnt-1].bssid[0]-Wifi[Wifi_cnt-1].bssid[5] |  |  | MAC address of wifi(N) |
| 4 | I32 | Wifi[0].RSSI |  |  | RSSI of wifi(0) signal strength |

Note: The device may not report base stations every time. The absence of base stations does not affect positioning. If the number of base stations in the message is 0, then the following is directly the number of Wi-Fi networks.

Example:

Original16hexadecimal message: BDBDBDBDA4E377BD6701CC010000C218D1AD160BF9FF078CBEBE1A8162C6FFFFFFC061180AF42AC1FFFFFF200BC726E000B6FFFFFFA8154DF6517EB2FFFFFFE005C5B1F824CCFFFFFFE8FCAFA02663AFFFFFFF6409805B2B9CAEFFFFFF94

BDBDBDBD:Header

A4:Message ID

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to10decimal1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

01:Cell_cnt Number of base stations 01-->Number of base stations is1

CC01:Cell[0].MCC Base station MCC Convert to big-endian01CC-->Convert to decimal460-->Base station MCC is460

0000:Cell[0].MNC Base station MNC converted to big-endian0000-->Convert to decimal0-->Base station MNC is0

C218:Cell[0].LAC Base station LAC converted to big-endian18C2-->Convert to decimal6338-->Base station LAC is6338

D1AD160B:Cell[0].CELL_ID Base station ID converted to big-endian0B16ADD1-->Convert to decimal186035665-->Base station ID is186035665

F9FF:Cell[0].RSSI Base station signal strength converted to big-endian FFF9-->Convert to decimal complement-7-->Base station signal strength is-7

07: Wifi_cnt number of wifi 07-->7wifis

8CBEBE1A8162:wifi mac address 8C:BE:BE:1A:81:62-->wifi[0]'s mac address is8C:BE:BE:1A:81:62

C6FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFC6-->Convert to decimal complement-58--->wifi[0]'s RSSI signal strength is-58

C061180AF42A:wifi mac address C0:61:18:0A:F4:2A-->wifi[1]'s mac address is C0:61:18:0A:F4:2A

C1FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFC1-->Convert to decimal complement-63--->wifi[0]'s RSSI signal strength is-63

200BC726E000:wifi mac address 20:0B:C7:26:E0:00-->wifi[2]'s mac address is20:0B:C7:26:E0:00

B6FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFB6-->Convert to decimal complement-74--->wifi[0]'s RSSI signal strength is-74

A8154DF6517E: WiFi MAC address A8:15:4D:F6:51:7E-->The MAC address of wifi[3] is A8:15:4D:F6:51:7E

B2FFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFB2-->converted to decimal complement -78--->The RSSI signal strength of wifi[0] is -78

E005C5B1F824: WiFi MAC address E0:05:C5:B1:F8:24-->The MAC address of wifi[4] is E0:05:C5:B1:F8:24

CCFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFCC-->converted to decimal complement -52--->The RSSI signal strength of wifi[0] is -52

E8FCAFA02663: WiFi MAC address E8:FC:AF:A0:26:63-->The MAC address of wifi[5] is E8:FC:AF:A0:26:63

AFFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFFFAF-->converted to decimal complement -81---->The RSSI signal strength of wifi[0] is -81

6409805B2B9C: WiFi MAC address 64:09:80:5B:2B:9C-->The MAC address of wifi[6] is64:09:80:5B:2B:9C

AEFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFFFAE-->converted to decimal complement -82--->The RSSI signal strength of wifi[0] is -82

94: checksum verification and

The parameters parsed from the message need to be connected to the map wifi database, and the latitude and longitude data is finally obtained through the map party.
Taking AMAP  connecting to the wiif database as an example:

According to the access network method, the AMAP  WiFi positioning protocol document divides it into three positioning scenarios. The following provides corresponding examples for the three positioning scenarios.

1. The network connection method is Wi-Fi access, that is, the accesstype parameter is 1.

http://apilocate.amap.com/position?accesstype=1&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&mmac=50:a7:2b:1e:56:58,-60,alibaba-inc&macs=4c:48:da:25:0b:11,-59,alibaba-

inc|4c:48:da:25:1a:11,-77,alibaba-inc&serverip=10.2.166.4&output=xml&key=<用户 Key>

2. The network connection method is mobile access and the mobile phone card is a non-CDMA card, that is, when the accesstype parameter is 0 and cdma=0:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=0&bts=460,01,40977,2205409,-65&nearbts=460,01,40977,2205409,-65|460,0

1,40977,2205409,-65|460,01,40977,2205409,-65&serverip=10.2.166.4&output=xml&key=<用户 Key>

3. The network connection method is mobile access and the mobile phone card is a CDMA card, that is, when the accesstype parameter is 0 and cdma=1:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=1&bts=13824,1,1838,1674723,575739,-52&serverip=10.2.166.4&output=xml&key=<用户 Key>

W200P watch: Gaode wifi positioning protocol calls Gaode interface description: When selecting parameters, generally choose the first type

You can also choose the second type, but if the device does not report the base station, you need to remove the two parameters bts and neabts before you can

The first type, an example of incoming parameters: mmac is the first WIFI mac address in the messagehttp://apilocate.amap.com/position?accesstype=1&imei=869907057780989&mmac=04:8C:16:A9:FB:A8,-44,TP-Link&macs=04:8C:16:A9:FB:A8,-44,TP-Link|C8:3A:35:F4:CE:18,-59,TP-Link|1C:78:4E:D5:07:FE,-79,TP-Link|C8:3A:35:9D:0B:E0,-87,TP-Link|4A:D8:90:E8:0A:82,-88,TP-Link|AC:AD:4B:C9:C3:87,-89,TP-Link|A4:16:E7:55:84:AC,-91,TP-Link&output=json&key=xxxx

For the specific  AMAP wifi positioning protocol PDF, you can click to view and download:(not a screenshot, you can click to view)
This document is provided by AMAP , you can directly ask AMAP  for specific docking
Note that the name of the downloaded document will change, you can rename it yourself

4.3.3 Bluetooth Location (BLE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xD6 | See below See definition below | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 0 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple groups of information, each group may have multiple ibeacons |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (One of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (One of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (One of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (One of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | RSSI (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (One of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | RSSI (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (One of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| ... | ... | ... | ... |

Note: In the general version, the total number of Bluetooth beacon groups is fixed at 1, and the maximum number of Bluetooth beacons is 4. The sorting is based on signal strength from largest to smallest. By default, the watch only recognizes our own beacons. If you need to connect to other manufacturers' beacons, please consult relevant personnel via WeChat.
Example:

Original hexadecimal message: BDBDBDBDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

bdbdbdbd: Header

D6: Message ID

00: type

01: Only one set of ibeacon data

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

04: The first group has 4 beacon information

4327: Turn big endian-->major : 0x2743-->Convert to decimal 10051

1794: Convert to big-endian --> minor: 0x9417 --> Convert to decimal 37911

AC:  RSSI 0xAC-->Convert to decimal complement -84

4327: Convert to big endian-->major: 0x2743-->Convert to decimal 10051

3094: Convert to big endian-->minor:0x9430-->Convert to decimal 37936

AA:  RSSI 0xAA-->Convert to decimal complement -86

4327:Convert to big endian-->major: 0x2743-->Convert to decimal 10051

b956:Convert to big endian-->minor:0x56b9-->Convert to decimal 22201

A5:   RSSI 0xA5-->Convert to decimal complement -91

4327: Convert to big endian-->major: 0x2743-->Convert to decimal 10051

FE94: Convert to big endian-->minor:0x94FE-->Convert to decimal 38142

A5: rssi:0xA5-->Convert to decimal complement -91

6a --checksum

Bluetooth beacon major, minor description: The default shipping Bluetooth beacon has a beacon code attached.

For example: 00081005345866, then the major of the Bluetooth beacon is 10053, the minor is 45866, and the manufacturer ID is 0008 (this is useless and has no practical significance).

4.3.4 Base station latitude and longitude reporting (MSGID=0x15) — wifi positioning supplement

Note: This message is generally when wifi positioning fails, and the module automatically requests the latitude and longitude of the communication base station for positioning (the positioning priority cannot be changed).
The location is the location of the communication base station, which can only be used as a reference for auxiliary positioning. The accuracy of the communication base station is not high---this is only supported in mainland China, not in Hong Kong or overseas.

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x15 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | flag |  | -/- | Extended segment identifier |
| 4 | U32 | lon |  | -/- | Longitude (*10 to the power of 6 or 7) longitude |
| 4 | U32 | lat |  | -/- | latitude(*10 to the power of 6 or 7) latitude |
| 1 | U8 | north_south |  | -/- | /N or S/ South or North latitude |
| 1 | U8 | east_west |  | -/- | /E or W/ East or West longitude |
| 1 | U8 | status |  | -/- | /A or B or V/A, B are valid data, B corresponds to 7-digit precision, V is invalid data |
| 4 | U32 | Timestamp |  | -/- | Timestamp |
| N |  | Num |  | -/- | The general version has no extensions, and the table only retains the definition. Definition: The value of extension segment 1 can be the nth digit value defined by the extension segment). When multiple extension segments are used simultaneously, extension segment 1 corresponds to the lowest bit content, and the content is extended to bit n in sequence. |
| ... | ... | ... | ... | ... | ... |

Example:

Original16hexadecimal message: BDBDBDBD1502C8C9E53F855AD30F4E4542E377BD67F4

BDBDBDBD:Header

15:Message ID

02:type  type=02indicates LBS base station positioning

C8C9E53F: Longitude--->converted to big endian4857963E--->converted to longitude:26.5509509, Note: Refer to4.3.1GPS/ BDS position reporting for conversion and analysis: Positioning data reporting (0x03) section analysis example

855AD30F: Latitude--->converted to big endian129B82E0--->converted to latitude:107.2024008, Note: Refer to4.3.1GPS/ BDS position reporting for conversion and analysis: Positioning data reporting (0x03) section analysis example

4E:north_south 4E-->convert to ASCII code N-->North latitude

45:east_west 45-->convert to ASCII code E-->East longitude

42:status 42-->convert to ASCII code B, valid data, the precision is to keep7digits after the decimal point

E377BD67: Timestamp converts the timestamp to big-endian 67BD77E3-->Convert to10Hexadecimal1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

08: checksum

4.3.5 UWB Location (MsgId=0xD7)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xD7 | See below See definition below | CK_sum |

Payload contents

| Format() | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 01 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple sets of information, and each group may contain multiple Ubeacons |
| Int32 | Timestamp | 4 | UTC timestamp |
| U8 | Total_PackCount | 1 | Total number of UBeacons at the current time |
| U32 | UBeacon0 mac | 4 | UBeacon devid (little endian first) |
| U16 | Distance0 | 2 | Distance (centimeters) |
| U32 | UBeacon1 mac | 4 | UBeacon devid (little endian first) |
| U16 | Distance1 | 2 | Distance (centimeters) |
| U32 | UBeacon2 mac | 4 | UBeacon devid (little endian first) |
| U16 | Distance2 | 2 | Distance (centimeters) |
| ... | ... | ... | ... |

Note: The W300PGU general version reports a maximum of 1 group and a maximum of 4 UBeacons

Example:

Original16hexadecimal message: BDBDBDBDD70101E377BD6703320600009100E70500006800380500005C00A1

BDBDBDBD:Header

D7: Message ID

01: Type 01

01: Total_groups Total number of groups, converted to decimal01--> The total number of groups is1

E377BD67: Timestamp Convert to big-endian 67BD77E3--> Convert to decimal1740470243--> Timestamp is1740470243seconds -->

Convert to standard time format UTC time: 2025 - 02 - 25 07 : 57 : 23 --> Convert to Beijing time: 2025 - 02 - 25 15 : 57 : 23

03: Total_PackCount Number of UWB beacons Convert to10base03--> Number of UWB beacons is3

32060000: UBeacon0 mac UWB beacon id Convert to big-endian00000632--> UWB beacon id is00000632

6800: Distance0 Ranging distance Convert to big-endian0091--> Convert to decimal145--> Ranging distance is145cm

E7050000: UBeacon1 mac UWB beacon id Convert to big-endian 000005E7--> UWB beacon id is000005E7

9100: Distance1 Ranging distance Convert to big-endian0068--> Convert to decimal104--> Ranging distance is104cm

38050000: UBeacon2 mac UWB beacon id Convert to big-endian00000538--> UWB beacon id is00000538

5 C00: Distance2 Ranging distance converted to big-endian 005 C-->Convert to decimal 92 -->Ranging distance is 92 cm

A1: checksum

4.4 Device Information and Status Reporting

4.4.1 ICCID Upload of SIM Card (MSGID=0xF3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xF3 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 10 | 10*U8 | ICCID | 1 | - | ICCID number - Device SIM card number ICCID |

Note: This message will be reported once every time the device is powered on.

Example:

Original hexadecimal message : BDBDBDBDF389861118236001639994CC

BDBDBDBD:Header

F3: Message ID

89861118236001639994:ICCID Device SIM card number ICCID is: 89861118236001639994

CC:checksum

4.4.2 Device Charging Status Upload (MSGID=0xC3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Status | / | / | 0 start, 1 end, 2 fully charged |
| 1 | U32 | Timestamp | / | / | Timestamp (a timestamp will be added at the end during retransmission) |

Note: This message requires device firmware support, the latest general version supports it by default
Example:

Original hexadecimal message:BDBDBDBDC301E377BD678A

BDBDBDBD:Header

C3:Message ID

01:Device finished charging

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

Original hexadecimal message:BDBDBDBDC300DB4D2F668A

BDBDBDBD:Header

C3:Message ID

00:Device starts charging

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

Original hexadecimal message:BDBDBDBDC302DB4D2F668A

BDBDBDBD:Header

C3:Message ID

02:Device is fully charged

E377BD67:Timestamp Convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

4.4.3 Status parameter report (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Number of parameter types |
| 1 |  |  |  |  | (Reserved 00) |
| 1 | u8 | Type | -/- | -/- | Parameter type 1, Type definition: 00—MCU (Firmware), 01—Module, 02 Sensor (This type is not available in the general version) |
| 1 | U8 | NameLen | -/- | -/- | Length of Parameter Name 1 |
| n | N*u8 | Name | -/- | -/- | Parameter Name |
| 1 | u8 | Type | -/- | -/- | Parameter Type 2, Type Definition: 00—MCU (Firmware), 01—Module, 02 Sensor (This type is not available in the general version) |
| 1 | U8 | NameLen | -/- | -/- | Length of Parameter Type 2 |
| n | N*u8 | Name | -/- | -/- | Parameter Name |
| ... | ... | ... | ... | ... | ... |

Note: This message is reported once by default at startup, uploading the device firmware version number, module version number, which can be left unparsed. The Message ID of previously shipped devices was 0xBB, you can contact relevant personnel to send it.

Example:

BDBDBDBDA902000009423732352E4F56303601174E5432364B434E4230304E4E412D4C3032303330393530B5

BDBDBDBD:Header

A9:Message ID

02:TypeCnt indicates there are 2 types of parameters

00:Reserved field

00:Type=00 00 represents MCU (firmware)

09:The length of the MCU name is 9 bytes

423732352E4F563036--->Hexadecimal to text (device firmware version number)--->B725.OV06

01:Type=01 01 represents the module

17:The length of the communication module name is 23 bytes

4E5432364B434E4230304E4E412D4C3032303330393530:Hexadecimal to text (4G module version number)

--->NT26KCNB00NNA-L02030950

B5---checksum

4.4.4 Device Status (MSGID=0xE9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xE9 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | type |  | / | Default—00 |
| 2 | u16 | length |  | / | Length of the following message |
| 1 | u8 | location |  | / | Location reporting frequency: 00—-default not modified, 01—-modified by downlink (if there are multiple time periods in the downlink, only report the reporting frequency of the current time period) |
| 2 | u16 | frequency |  | / | Frequency, unit - minutes |
| 1 | u8 | health |  | / | Health reporting frequency: 00—-default not modified, 01—-modified by downlink |
| 2 | u16 | frequency |  | / | Frequency: Unit - minutes |

Note: This message is reported once at startup, and once after the downlink modifies the frequency.

Example:

Original Hexadecimal Message:BDBDBDBDE9000600010A00010A0033

BDBDBDBD:Header

E9:Message ID

00:type=00

0600:length The length of the message behind is converted to big-endian -->0006--> converted to decimal 6, and the message length is 6 bytes

01:location 01 indicates that the downlink has modified the location reporting frequency

0A00:frequency The health reporting frequency is converted to big-endian -->000A--> converted to decimal 10, and the reporting frequency is 10 minutes

01:health 01 indicates that the downlink has modified the health reporting frequency

0A00:frequency The health reporting frequency is converted to big-endian -->000A--> converted to decimal 10, and the reporting frequency is 10 minutes

33:checksum

If the device does not have a health reporting function or a location reporting function, the health reporting frequency and

The two values of the location reporting frequency will remain consistent, indicating the device's reporting frequency.

4.  5 Health-Related Reporting

4.  5.1 Health Data (MSGID=0x32)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x32 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | 00 |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 2 | U16 | contentLength |  |  | Total length of the following content |
| 1 | U8 | ID |  |  | Id (Type + Reported Value Length), see below for ID type definitions |
| N | N*U8 | Val1 |  |  | The reported value of Id (the byte length varies depending on the length of the reported value) |
| ... | ... | ... | ... | ... | ... |
| 1 | U8 | ID |  |  | Id (Type + Reported Value Length), see below for ID type definitions |
| N | N*U8 | Val1 |  |  | The reported value of Id (the byte length varies depending on the length of the reported value) |

ID type definition (hexadecimal):

01 Step count02 Heart rate 03 Body temperature 04 Wrist temperature 06 Diastolic pressure 07 Systolic pressure 08 Blood oxygen 09 HRV(Special version support)

(05 Blood glucose 10 Sit-up count11 Running pace and distance 12 Rope skipping speed and count )---Reserved, not available

Default universal version support:01 Step count02 Heart rate 03 Body temperature 04 Wrist temperature 06 Diastolic pressure 07 Systolic pressure 08 Blood oxygen

Example:

Original16Hexadecimal message: BDBDBDBD3200E377BD6711000A1E00114B314A39711A4A0122bc00416212

BDBDBDBD: Header

32:Message ID

00:type  type=00

E377BD67: Timestamp converted to big-endian67BD77E3-->convert to10decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

1100: The length of the following message is converted to big-endian -->0011--> converted to decimal17, and the length of the following message is17bytes

0A:ID 0x0A--> converted to binary00001010The first five digits are00001--> converted to16hexadecimal01, which represents the data ID step counting (0x01),

The last three digits are010--> converted to decimal02, which means the length of the step counting data is2bytes

1E00: The step counting reporting value is converted to big-endian -->001E--> converted to decimal30, and the total step counting data is30steps

11:ID 0x11--> converted to binary00010001The first five digits are00010--> converted to16hexadecimal02, which represents the data ID heart rate (0x02),

The last three digits are001--> converted to decimal1, which means the length of the heart rate data is1byte

4B: Heart rate reporting value converted to decimal -->75, heart rate data is75

31:ID 0x31--> converted to binary00110001The first five digits are00110--> converted to16hexadecimal06, which represents the data ID diastolic blood pressure (0x06),

The last three digits are001--> converted to decimal1, which means the length of the diastolic blood pressure data is1byte

4A: Blood pressure diastolic reporting value, converted to decimal -->74, diastolic blood pressure data is74

39:ID  0x3-->Convert to binary00111001    The first five digits are00111-->Convert to16hexadecimal07, representing data ID systolic blood pressure (0x07),

The last three digits are001-->Convert to decimal1, representing the length of the systolic blood pressure data as1byte

71: Blood pressure systolic reporting value, convert to decimal-->113, systolic blood pressure data is113

1A:ID  0x1A-->Convert to binary00011010   The first five digits are00011-->Convert to16hexadecimal03, representing data ID body temperature (0x03),

The last three digits are010-->Convert to decimal2, representing the length of the body temperature data as2bytes

4A01: Body temperature reporting value, converted to big-endian-->014A-->Convert to decimal330-->330*0.1-->Body temperature data is33degrees Celsius

22:ID  0x22-->Convert to binary00100010   The first five digits are00100-->Convert to16hexadecimal04, representing data ID wrist temperature (0x04),

The last three digits are010-->Convert to decimal2, representing the length of the wrist temperature data as2bytes

BC00: Wrist temperature reporting value, converted to big-endian-->00BC-->Convert to decimal188-->188*0.1-->Wrist temperature data is18.8degrees Celsius

41:ID  0x41-->Convert to binary01000001    The first five digits are01000-->Convert to16hexadecimal08, code data ID blood oxygen (0x08),

Last three digits001--> Convert to decimal1, representing the blood oxygen data length as1byte

62: Blood oxygen reporting value, converted to decimal -->98, blood oxygen data is98

12: checksum

4.5.2 Device sleep analysis data upload (MSGID=0xC5)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC5 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 4 | Int32 | DateTime |  | -/- | Upload UTC start time |
| 4 | Int32 | DateTime |  |  | Upload UTC end time |
| 2 | Int16 | Sleepminute |  |  | Number of uploaded sleep minutes |
| 4 | Int32 | Type |  |  | Sleep state: Upload type, 1 is deep sleep, 2 is light sleep, 3 is wake time |

Example:

Original16Hexadecimal message: BDBDBDBDC5AC338860693B8860210001000000D1

BDBDBDBD:Header

C5: Message ID

AC338860: Start time, converted to big endian608833AC--> converted to decimal --> timestamp is1619538860seconds -->

Convert to standard time format UTC time: 2021-04-27 15:54:20-->Convert to Beijing time:2021-04-27 23:54:20

693B8860: End time, converted to big endian60883BAC--> converted to decimal --> timestamp is1619540908seconds -->

Convert to standard time format UTC time: 2021-04-27 16:28:28-->Convert to Beijing time:2021-04-28 00:28:28

2100: Sleepminute sleep time, converted to big endian0021--> converted to decimal33--> The total sleep time is33minutes

01000000: Type sleep state, converted to big endian00000001--> converted to decimal1-->1is deep sleep, deep sleep time is33minutes

D1: checksum

4.6 Downlink Feedback Related Reporting

4.6.1 Downlink Feedback (MSGID=0xC0)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC0 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID Length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This command is used for feedback of downlink commands , returning the Message ID of the downlink command received earlier (can be multiple Message IDs returned collectively)

Original hexadecimal message:BDBDBDBDC0011720

BDBDBDBD:Header

C0:Message ID

01:length Message ID length Convert decimal 01--> Message ID length is 1 byte

17  Type indicates that the device received the downlink command Message ID is 17

20:checksum

4.6.2 Message Status Report (MSGID=0x28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x28 | See below See definition below | CK_sum |

Payload contents

| Bytes | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | timestamp | -/- | -/- | Timestamp |
| 1 | U8 | Type | -/- | -/- | Character encoding type (corresponding to downlink 0X28), GB2312 encoding: 0x03; Unicode encoding: 0x05 |
| 1 | U8 | status | -/- | -/- | Message status - 0x01: Confirmed, 0x02: Rejected |
| 4 | U32 | seqID | -/- | -/- | (Corresponds to downlink 0X28 message id seqID) |

Example:

Note:The current general version does not distinguish between character encoding types and is fixed at 03. Subsequent versions will distinguish.

Original hexadecimal message:BDBDBDBD28FD3BA068030268A03BE477

BDBDBDBD:Header

28:Message ID

FD3BA068:timestamp Timestamp converted to big-endian 68A03BFD-->converted to decimal-->timestamp is 1755331581 seconds-->

Convert to standard time format UTC time: 2025-08-16 08:06:21-->Convert to Beijing time: 2025-08-16 16:06:21

03:Type 03--fixed value

02:status Message status 02--Interface click reject button--Rejected

68A03BE4:seqID The id of the downlink message is 68A03BE4

77:checksum

5 Settings

5.1 Downlink

5.1.1 Set location reporting frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x17 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |

Note: One day is from 00:00-23:59
Example:

00:00-23:59 Report location every 10minutes: BDBDBDBD17010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

17:Message ID

01:enable is enabled Convert to decimal01-->1--Enabled

0A00:Interval time interval Convert to big-endian000A-->Convert to decimal10-->Time interval is10minutes

00:time_start_h Start time-hour, convert to decimal00-->Start time-hour is00

00:time_start_m Start time-minute, convert to decimal00-->Start time-minute is00

17:time_end_h End time-hour, convert to decimal23-->End time-hour is 23

3B:time_end_m End time-minute, convert to decimal59-->End time-minute is 59

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

97:checksum

5.1.2 Message Send (MSGID=0X28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x28 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Decription |
| --- | --- | --- | --- |
| 1 | U8 | type | Message type, if it is downlink text information, Unicode encoding is 05 |
| 4 | Uint32 | seqID | ID of the information, uniqueness |
| 1 | U8 | CONTENT LEN | Content Length |
| N | N | CONTENT | Content, Chinese is Unicode encoded (little endian first, pay attention to conversion, one Chinese character occupies 2 bytes, one letter occupies 2 bytes) |

Note: The default Chinese character encoding for the Hong Kong general version is Unicode
Information ID cannot be repeated for each transaction, and devices cannot receive repeated IDs
Example:

Text message: hello,world  BDBDBDBD28050300000016680065006C006C006F002C0077006F0072006C006400DD

BDBDBDBD:Header

28: Message ID

05: type message type Convert decimal 05-->type=05

03000000: seqID information ID Information ID is 03000000

16: CONTENT LEN content length Convert decimal 22, content length is 22 bytes

680065006C006C006F002C0077006F0072006C006400:

Every 2 bytes are converted to big-endian --> converted to Unicode characters --> combined final - text message content:hello,world

DD:checksum

5.1.2 Settings - Location Priority Settings (0XCE01 & 0XCE02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See definition below | CK_sum |

Payload contents

Type=01 Location Priority Setting

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 is always valid, 01 is valid this time |
| 2 | U16 | Len | -/- | -/- | Length of subsequent instruction |
| n | n |  | -/- | -/- | Main body |

Definition of positioning priority setting -- See 2.3 Device downlink description -- Device positioning priority delivery (0xCE01)

Positioning priority setting body:

Currently, the general version supports: 01 -- Satellite (GPS/BDS), 02 -- wifi (wifi + base station) 03 -- Bluetooth beacon (requires additional deployment of Bluetooth beacons), 06 --- UWB beacon (requires additional deployment of UWB beacons);

Special equipment support: 04 -- LBS base station 05--125k

Example:

Wifi positioning priority (wifi>Bluetooth beacon>Satellite (GPS/BDS)):BDBDBDBDCE0100030002030133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The body length is 3 bytes

020301:Text: 02--wifi 03--Bluetooth beacon 01--Satellite (GPS/BDS) The positioning priority is wifi>Bluetooth beacon>Satellite (GPS/BDS)

33:checksum

Satellite (GPS/BDS) positioning priority (Satellite (GPS/BDS)>wifi>Bluetooth beacon):BDBDBDBDCE0100030001020333

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The body length is 3 bytes

010203:Text: 01--Satellite (GPS/BDS) 02--wifi 03--Bluetooth beacon The positioning priority is satellite (GPS/BDS)>wifi>Bluetooth beacon

33:checksum

Bluetooth positioning priority (Bluetooth beacon>wifi>Satellite (GPS/BDS)):BDBDBDBDCE0100030003020133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The body length is 3 bytes

030201:Text: 03--Bluetooth beacon 02--wifi 01--Satellite (GPS/BDS) The positioning priority is Bluetooth beacon>wifi>Satellite (GPS/BDS)

33:checksum

UWB positioning priority (UWB beacon>wifi>Satellite (GPS/BDS)):BDBDBDBDCE0100030006020133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The body length is 3 bytes

060201:Text: 06--UWB beacon 02--wifi 01--Satellite (GPS/BDS) The positioning priority is Bluetooth beacon>wifi>Satellite (GPS/BDS)

33:checksum

Single satellite (GPS/BDS) positioning:BDBDBDBDCE0100030001000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The body length is 3 bytes

030201:Text: 01--Satellite (GPS/BDS) 00--No positioning 00--No positioning The positioning priority is single satellite (GPS/BDS) positioning

33:checksum

Single WiFi positioning:BDBDBDBDCE0100030002000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The body length is 3 bytes

020000:Text: 02--wifi 00--No positioning 00--No positioning Positioning priority is single wifi positioning

33:checksum

Single Bluetooth beacon positioning:BDBDBDBDCE0100030003000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The body length is 3 bytes

020000:Text: 03--Bluetooth beacon 00--No positioning 00--No positioning Positioning priority is single Bluetooth beacon positioning

33:checksum

Single UWB beacon positioning:BDBDBDBDCE0100030006000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning priority setting

00:Valid Validity 00 Always valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The body length is 3 bytes

060000:Text: 06--UWB beacon 00--No positioning 00--No positioning Positioning priority is single UWB beacon positioning

33:checksum

5.1.4 Settings - Health Sampling Reporting Frequency Settings (MSGID=0X1C)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x1C | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |

Health sampling reporting frequency setting--See2.3 Device downlink description--Device health sampling frequency distribution(0x1D)

Note: After setting, report according to time period, and do not report outside the time period

Example:

00:00-23:59(All day) Report health data every10minutes: BDBDBDBD1C010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

1C: Message ID

01:enable is enabled Convert to decimal01-->1--Enabled

0A00:Interval time interval Convert to big-endian000A-->Convert to decimal10-->Time interval is10minutes

00:time_start_h Start time-hour, convert to decimal00-->Start time-hour is00

00:time_start_m Start time-minute, convert to decimal00-->Start time-minute is00

17: time_end_h End time - hour, convert to decimal 23 --> Start time - hour is 00

3B: time_end_m End time - minute, convert to decimal 59 --> Start time - hour is 00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

97:checksum

5.1.5 Settings - Device Alarm Settings (0XCE03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See definition below | CK_sum |

Payload contents

Type=03 Device alarm settings

| Byte offset | Format | Name | Unit | Decription |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Fixed to 0x03 - Device alarm settings |
| 1 | U8 | Valid | 1 | Fixed to 0x00 - Health threshold, always valid |
| 2 | U16 | Cfg Len | 2 | Instruction length |
| 1*N | U8*N | [Bit7] Enable [Bit6 - 0] Threshold Type | 1*N | [Bit7] Whether to enable threshold detection of this type [Bit6 - 0] Set the threshold type: 0x01 heart rate, 0x02 Systolic Blood Pressure (SBP) 0x03 Diastolic Blood Pressure (DBP), 0x04 Blood Oxygen, 0x05 Temperature (the threshold unit is *10, for example, 37.3 is 373) |
| 2*N | I16*N | Threshold Min | 2*N | Lower threshold (this content is only available when threshold detection is enabled) |
| 2*N | I16*N | Threshold Max | 2*N | Upper threshold (this content is only available when threshold detection is enabled) |

Device alarm setting definition-Health threshold--See2.3 Device downlink description--Health threshold setting(0xCE0300)

Note: The default is the off state, and the threshold is the normal range. The device will report an alarm outside this range.

Heart rate threshold distribution range (20-200), systolic blood pressure threshold distribution range (20-200), diastolic blood pressure threshold distribution range (20-200), blood oxygen threshold distribution range (80-100), temperature threshold distribution range (160-600)

Example:

Set multiple health thresholds,1. Heart rate [60,120],2. Close blood oxygen threshold detection,3. Temperature [36.0,37.3]:BDBDBDBDCE03000B00813C007800048568017501FF

BDBDBDBD:Header

CE:Message ID

03:Type 03--Device alarm setting

00:Valid 00--Health threshold, always valid

0B00: Convert to big-endian -->000B--> Convert to decimal11--> The number of message bytes after the instruction (excluding checksum) 11 bytes

81: Convert to binary-->10000001 bit7:1---On Open status bit6-0:000001-->0x01 Heart rate

3C00: Convert to big-endian -->003C--> Convert to decimal60, the lower limit of the heart rate threshold is60

7800: Convert to big-endian -->0078--> Convert to decimal120, the upper limit of the heart rate threshold is120

04: Convert to binary-->00000100 bit7:0---Off, no threshold after closing bit6-0:0000100-->0x04 Blood oxygen

85: Convert to binary -->10000101 bit7:1---On Open status bit6-0:0000101-->0x05 Temperature

6801: Convert to big-endian -->0168--> Convert to decimal360(actual temperature *10)--> The lower limit of the temperature threshold is36.0

7501: Convert to big-endian -->0175--> Convert to decimal373(actual temperature *10)--> The upper limit of the temperature threshold is37.3

FF:checksum校验和

5.1.6 Setting - Switch Setting (0XCE04-24)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Please refer to the instructions for the specific type |
| 1 | u8 | switch | -/- | -/- | Switch 00 on, 02 off, 01 single effective |
| 2 | U16 |  | -/- | -/- | Fixed 0000 |

Device alarm setting - switch type: specific function description-- see 2.3 Device downlink description

05 Bluetooth broadcast switch

07 Fall Alarm Switch 08 Stay Alarm Switch

0A Immediately Report Location Information 0B Immediately Report Health Data

0D Immediately Report Version Number Data 16 Whether the downlink can use the key to shut down

17 Downlink supports long standby mode 18 Sleep function switch

19 Key trigger sos alarm switch 20 Report data switch--Separate settings (Note: This switch has a special structure, and there are differences in the message and table definitions. Please see the specific example)

24 GPS Always On Switch

Type=05 Bluetooth broadcast switch
Example:

00---Open:BDBDBDBDCE0500000093

BDBDBDBD:Header

CE:Message ID

05:Type 05 Bluetooth broadcast switch

00:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE0502000093

BDBDBDBD:Header

CE:Message ID

05:Type 05 Bluetooth broadcast switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=07 Fall Alarm Switch
Example:

00---Open:BDBDBDBDCE0700000093

BDBDBDBD:Header

CE:Message ID

07:Type 07 Fall Alarm Switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE0702000093

BDBDBDBD:Header

CE:Message ID

07:Type 07 Fall Alarm Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=08 Stay Alarm Switch
Example:

00---Open:BDBDBDBDCE0800000093

BDBDBDBD:Header

CE:Message ID

08:Type 08 Stay Alarm Switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE0802000093

BDBDBDBD:Header

CE:Message ID

08:Type 08 Stay Alarm Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=0A Report Location Information Immediately
Example:

01---Single Report:BDBDBDBDCE0A01000093

BDBDBDBD:Header

CE:Message ID

0A:Type 0A Report Location Information Immediately

02:switch switch 01--Single issue single report

0000:Fixed value

93:checksum

Type=0B Report Health Data Immediately
Example:

01---Single report:BDBDBDBDCE0B01000093

BDBDBDBD:Header

CE:Message ID

0B:Type 0B Immediately report health data

02:switch switch 01--Single issue single report

0000:Fixed value

93:checksum

Type=0D Immediately report version number data
Example:

01---Single report:BDBDBDBDCE0D01000093

BDBDBDBD:Header

CE:Message ID

0D:Type 0D Immediately report version number data

02:switch switch 01--Single issue single report

0000:Fixed value

93:checksum

Type=16 Whether the downlink can use the key to shut down
Example:

00---Open:BDBDBDBDCE1600000093

BDBDBDBD:Header

CE:Message ID

16:Type 16 Whether the downlink can use the key to shut down

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE1602000093

BDBDBDBD:Header

CE:Message ID

16:Type 16 Whether the downlink can use the key to shut down

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=17 Downlink supports long standby mode
Example:

00---Open:BDBDBDBDCE1700000093

BDBDBDBD:Header

CE:Message ID

17:Type 17 downlink supports long standby mode

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE1702000093

BDBDBDBD:Header

CE:Message ID

17:Type 17 downlink supports long standby mode

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=18 Sleep function switch
Example:

00---Open, enter sleep mode after being still for a period of time, and do not report data:BDBDBDBDCE1800000093

BDBDBDBD:Header

CE:Message ID

18:Type 18 Sleep function switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close, report data normally in static or charging state:BDBDBDBDCE1802000093

BDBDBDBD:Header

CE:Message ID

18:Type 18 Sleep function switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=19 Key trigger sos alarm switch
Example:

00---Open:BDBDBDBDCE1900000093

BDBDBDBD:Header

CE:Message ID

19:Type 19 key triggers SOS alarm switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE1902000093

BDBDBDBD:Header

CE:Message ID

19:Type 19 key triggers SOS alarm switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=20 Report data switch - separate settings
Example:

Add field: Body length: 2 bytesBody: 0x00 (location data report) + switch (0x00: on, 0x02: off)

0x01 (Health data report) + switch (0x00: on, 0x02: off)

Location data report on:BDBDBDBDCE20000200000093

BDBDBDBD:Header

CE:Message ID

20:Type 20 report data switch

00:Fixed value

0200:Body length converted to big-endian 0002--> converted to decimal 02--> body length is 2 bytes

00:Body: 00--Location data report switch

00:Text:00--Enable reporting

93:checksum

Location data reporting off:BDBDBDBDCE2000020000293

BDBDBDBD:Header

CE:Message ID

20:Type 20 report data switch

00:Fixed value

0200:Body length converted to big-endian 0002--> converted to decimal 02--> body length is 2 bytes

00:Body: 00--Location data report switch

02:Text:00--Disable reporting

93:checksum

Health data reporting on:BDBDBDBDCE20000200010093

BDBDBDBD:Header

CE:Message ID

20:Type 20 report data switch

00:Fixed value

0200:Body length converted to big-endian 0002--> converted to decimal 02--> body length is 2 bytes

01:Text:00--Health data reporting switch

00:Text:00--Enable reporting

93:checksum

Health data reporting off:BDBDBDBDCE20000200010293

BDBDBDBD:Header

CE:Message ID

20:Type 20 report data switch

00:Fixed value

0200:Body length converted to big-endian 0002--> converted to decimal 02--> body length is 2 bytes

01:Text:00--Health data reporting switch

02:Text:02--Disable reporting

93:checksum

Type=24 GPS always on switch
Example:

00---Enable:BDBDBDBDCE2400000093

BDBDBDBD:Header

CE:Message ID

24:Type 24 GPS always on switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE2402000093

BDBDBDBD:Header

CE:Message ID

24:Type24 GPS always-on switch

02:switch switch 02 off

0000:Fixed value

93:checksum

5.1.7 Settings - Drop Sensitivity Setting (0xCE15)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Decription |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Type is 0x15 |
| 1 | U8 | Valid | 1 | The default is 0, which has no impact for now. |
| 2 | U16 | Len | 2 | Length of subsequent parameter content |
| 1 | U8 | Param Type | 1 | Parameter type: 0x00: Sensitivity, 0x01: Height |
| 1 | U8 | Param Level | 1 | Parameter level: 5 levels (0 - 4) |

Drop Sensitivity Setting-Parameter Description:

Sensitivity: Refers to the degree to which the drop trigger algorithm is satisfied, providing 5 setting levels (0 - 4): Low - Lower (Medium Low) - Medium - Higher (Medium High) - High.

Height: Refers to the height that satisfies the drop alarm trigger, providing 5 setting levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m - 2.5m.

For example, if the height is set to "1.5m", the device needs to be ≥ 1.5m to trigger the drop alarm.

Note that sensitivity and height cannot be combined in one message.

Example:

Set the sensitivity to "Medium Low": BDBDBDBDCE150002000001FF

BDBDBDBD:Header

CE:Message ID

15: Type Fall Sensitivity Setting

00:Fixed value

0200: Len

00: Param Type Sensitivity Setting

01: Param Level Sensitivity Level 1-->Lower (Medium Low)

FF: checksum

Set height to “1.5m”: BDBDBDBDCE150002000102FF

BDBDBDBD:Header

CE:Message ID

15: Type Fall Sensitivity Setting

00:Fixed value

0200: Len

01:Param Type Height Setting

02:Param Level Height Level 2-->1.5m

FF: checksum

5.1.8 Settings - Vibration Settings (0XCE23)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Decription |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Fixed at 0x23 |
| 1 | U8 | Valid | 1 | 0x00 |
| 2 | U16 | Cfg Len | 2 | Length of subsequent parameter content |
| 1 | U8 | Motor Type | 1 | Setting type: 1. Vibration duration - 0x00 |
| 4 | U32 | Value | 4 | Setting content: 1. Vibration duration (unit: milliseconds, range: 0 - 60 seconds) |

Settings - Vibration Settings Definition: This message can modify the global vibration duration, and any vibration duration becomes the duration set by this command.

Example:

Set the default global vibration duration of the device to 1.5 seconds (1500 milliseconds):BDBDBDBDCE2300050000dc050000FF

BDBDBDBD:Header

CE:Message ID

23: Type 23 Setting - Vibration Setting

00: Fixed value 00 - always valid

0500:The length of the following parameter content is converted to big-endian 0005-->converted to decimal 05-->the length of the following parameter content is 5 bytes

00:Motor Type Vibration duration setting

dc050000:Value Vibration duration converted to big-endian 000005dc-->converted to decimal 1500-->vibration duration is 1500 milliseconds (1.5 seconds)

FF: checksum

5.1.9 IP & Domain Name Settings (MSGID=0xC3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | Issue type: Type =1 IPv4, Type =2 IPv6 (Reserved item - not currently supported), Type =3 Domain name (Currently only supports ASCII encoded domain names) |
| 2 | U16 | Port |  |  | Port number (2 bytes) |
| 1 | U8 | Len |  |  | Length of subsequent parameter content |
| N | U8 | Domainname |  |  | Specific IP or domain name content |

Note: After distribution, the device will not communicate on the original server, but will point to the modified server.

Example:

Type=1 IPv4:Downlink modifies the IP to:118.178.184.219, and the port is: 8825 BDBDBDBDC30179220476B2B8DB33

BDBDBDBD:Header

C3:Message ID

01:Type 1--IPv4

7922:Port Convert the port to big-endian 2279-->Convert to decimal 8825-->Port: 8825

04:Len The length of the following parameter content is converted to decimal 04-->The length is 4 bytes

76B2B8DB:Domainname IP 76-->Convert to decimal 118, B2-->Convert to decimal 178, B8-->Convert to decimal 184, DB-->Convert to decimal 219 IP is: 118.178.184.219

33:checksum

Type=3 Domain nameDownlink modifies the domain name to:aiday.com.cn, and the port is: 8825

BDBDBDBD:Header

C3:Message ID

03:type=3, domain name

7922:Port Convert the port to big-endian--2279 -->Convert to decimal Port: 8825

12:Len parameter content length converts to decimal 18 --> message length is 18 bytes

61696461792e636f6d2e636e:Domainname converts to string --> aiday.com.cn --> domain name is: aiday.com.cn

1D:checksum

5.1.10 Issue sedentary stay alarm trigger time (MSGID=0XCC)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCC | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | minute | -/- | -/- | Stay alarm trigger duration (unit: minutes; value range 2 — 60), no activity within this time length will trigger a stay alarm |

Example:

No activity for 5 consecutive minutes The watch reports a stay alarm (when the watch is worn): BDBDBDBDCC05DD

BDBDBDBD: Header

CC:Message ID

05: minute Stay alarm trigger duration converts to decimal 05-->5 minutes

DD: checksum

5.1.11 Shutdown and Restart (MSGID=0x77)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x77 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- |  | 01—Shutdown, 00—Restart |

Note: Shutdown and restart are both received in the power-on state. Shutdown: The watch shuts down after receiving the command in the power-on state, and no commands can be received after the shutdown; Restart: The watch restarts after receiving the command in the power-on state.

Example:

Shutdown:BDBDBDBD770193

BDBDBDBD:Header

77: Message ID

01:Type 01--Shutdown

93:checksum

Restart:BDBDBDBD770093

BDBDBDBD:Header

77: Message ID

01:Type 00--Restart

93:checksum

5.1.12 Sleep Statistics Time Period Setting (MSGID=0X1D)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x1D | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |

Example:

The sleep data statistics and reporting time period is 13:00 - 15:00 and 22:30 - 08:00BDBDBDBD1D010D000F0001161E080000000000000000000000FF

BDBDBDBD:Header

1D:Message ID

01:enable Whether to enable Convert decimal 01-->1--Enable

0D:time_start_h Start time - hour Convert decimal 13

00:time_start_m Start time - minute Convert decimal 00 Start statistics and report time is 13:00

0F:time_end_h End time - hour Convert decimal 15

00:time_end_m End time - minute Convert decimal 00 End statistics and report time is 15:00

01:enable Whether to enable Convert decimal 01-->1--Enable

16:time_start_h Start time - hour Convert decimal 22

1E:time_start_m Start time - minute Convert decimal 30 Start statistics and report time is 22:30

08:time_end_h End Time - Hour Convert to Decimal 08

00:time_end_m End Time - Minutes Convert to Decimal 00 End statistics and report time is 08:00

00:enable Whether to enable Convert to decimal 00-->0--Not enabled

00000000:All fill 00 in disabled state

00:enable Whether to enable Convert to decimal 00-->0--Not enabled

00000000:All fill 00 in disabled state

FF: checksum

5.1.13 Weather Warning (MSGID=0XCB)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCB | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Count | -/- | -/- | How many groups of weather warnings in total |
| 1 | U8 | Type | -/- | -/- | Weather warning type, see below for specific types |
| 1 | U8 | Status | -/- | -/- | Status, see below for specific statuses |
| … | … | …… |  |  | … |

Weather type - status table:

| Type | Decription | Status | notes - Click on the image to enlarge and view |
| --- | --- | --- | --- |
| 1 | Typhoon Warning | 0 | Cancel Alarm |
|  |  | 1 - 8 |  |
| 2 | Heavy Rain Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |
| 3 | Thunderstorm Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 4 | Flooding Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 5 | Landslide Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 6 | Strong Monsoon Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 7 | Frost Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 8 | Fire Danger Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
| 9 | Temperature Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
| 10 | Tsunami Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 11 | Heat Index | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |

Note: After the issuance, the device vibrates to display weather warning pictures. Multiple sets of weather warnings can be issued simultaneously. Each set of weather warnings must be different. The same type of warning cannot appear in one instruction, otherwise it will be invalid.
Example:

10sets of weather warnings issued: BDBDBDBD CB 0A 0101 0201 0301 0401 0501 0601 0701 0801 0901 0A01 F5

BDBDBDBD:Header

CB:Message ID

0A:Count Number of weather groups Convert to decimal10-->Issue10groups of weather

0101:Type&Status Weather type-status 01--Storm warning 01--1Alert signal

0201:Type&Status Weather Type-Status 02--Heavy Rain Warning 01--Yellow Rainstorm Warning Signal

0301:Type&Status Weather Type-Status 03--Thunderstorm Warning 01--Thunderstorm Warning

0401:Type&Status Weather Type-Status 04--Flooding Warning 01--Special Report on Flooding in the Northern New Territories

0501:Type&Status Weather Type-Status 05--Landslip Warning 01--Landslip Warning

0601:Type&Status Weather Type-Status 06--Strong Monsoon Warning 01--Strong Monsoon Signal

0701:Type&Status Weather Type-Status 07--Frost Warning 01--Frost Signal

0801:Type&Status Weather Type-Status 08--Fire Danger Warning 01--Yellow Fire Danger Warning

0901:Type&Status Weather Type-Status 09--Temperature Warning 01--Cold Weather Warning

0A01:Type&Status Weather Type-Status 10--Tsunami Warning 01--Tsunami Warning

F5:checksum校验和

5.1.14 UWB Alarm Distance Issuance (0x7900)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x79 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type 0—UWB ranging alarm distance |
| 1 | U8 | distance | -/- | -/- | The distance of UWB ranging alarm (unit: meter; value range 0-20). An alarm will be triggered if it is less than or equal to this distance. |

Example:

The alarm distance is 1 meter. If the ranging distance between the watch and the UWB beacon is less than 1 meter, the watch will vibrate and alarm BDBDBDBD790001C3

BDBDBDBD:Header

79:Message ID

00:Type 00--UWB ranging alarm distance

01:distance The distance of UWB ranging alarm is converted to decimal 01-->The distance of UWB ranging alarm is 1 meter

C3:checksum

5.1.15 UWB Ranging Alarm Configuration (0x7901)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x79 | See below See definition below | CK_sum |

Payload contents

| Format | Name | Unit | Decription |
| --- | --- | --- | --- |
| U8 | Type | 1 | Type is 0x01 — UWB beacon ranging alarm configuration |
| U8 | UWB beacon status | 1 | 00 — No alarm for all. After being issued, all UWB beacons will not trigger the UWB ranging alarm of the watch. Under this status, type 01 and 02 commands are invalid. 01 — Add UWB alarm beacon. 02 — Delete UWB alarm beacon. 03 — Set UWB alarm beacon (all covered). FF — All alarms. After being issued, all beacons will trigger the UWB ranging alarm. Under this status, type 01 and 02 commands are invalid. |
| U8 | UWB beacon number | 1 | Number of UWB beacons |
| U16 | UWB beacon 0 mac | 4 | ID of alarm UWB beacon 0 |
| U16 | UWB beacon 1 mac | 4 | ID of alarm UWB beacon 1 |

Example:

Issue all UWB beacons to not trigger the watch ranging alarm: BDBDBDBD790100DD

BDBDBDBD: Header

79 Message ID

01 Type =01 UWB ranging alarm beacon configuration

00 UWB beacon type = 00, all UWB beacons do not trigger the watch UWB ranging alarm

DD checksum

Send all UWB beacons to trigger the watch ranging alarm: BDBDBDBD7901FFDD

BDBDBDBD: Header

79 Message ID

01 Type =01 UWB beacon ranging alarm configuration

FF UWB beacon type =FF, all UWB beacons trigger the watch UWB ranging alarm

DD checksum

Set UWB alarm beacon: The device will clear the previous alarm beacon, and the UWB beacon in this message shall prevail to trigger the UWB ranging alarm

BDBDBDBD7901030390000002E70500001F060000DD

BDBDBDBD ：Header

79 Message ID

01 Type =01 UWB ranging alarm beacon configuration

03 UWB beacon type =03 Set UWB alarm beacon

03 UWB beacon count is3

90000002 UWB beacon0'sid Big-endian conversion 02000090 UWB idis 02000090

E7050000 UWB beacon1'sid Big-endian conversion000005E7 UWB idis000005E7

1F060000 UWB beacon2'sid Big-endian conversion 0000061F UWB idis 0000061F

DD checksum

Add UWB alarm beacon: It does not affect the original alarm beacon. After downlink, the3UWB beacon watches in the message can trigger UWB ranging alarm.

BDBDBDBD7901010390000002E70500001F060000DD

BDBDBDBD ：Header

79 Message ID

01 Type =01 UWB ranging alarm beacon configuration

01 UWB beacon type=01 Add UWB alarm beacon

03 UWB beacon count is3

90000002 UWB beacon0'sid Big-endian conversion 02000090 UWB idis 02000090

E7050000 UWB beacon1'sid Big-endian conversion000005E7 UWB idis000005E7

1F060000 UWB beacon2'sid Big-endian conversion 0000061F UWB idis 0000061F

DD checksum

Delete UWB alarm beacon: The3UWB beacon watches in the message do not trigger UWB ranging alarm.

BDBDBDBD7901020390000002E70500001F060000DD

BDBDBDBD ：Header

79 Message ID

01 Type =01 UWB ranging alarm beacon configuration

02 UWB beacon type=02 Delete UWB alarm beacon

03 UWB beacon count is3

90000002 UWB beacon0'sid Big-endian conversion 02000090 UWB idis 02000090

E7050000 UWB beacon1'sid Big-endian conversion000005E7 UWB idis000005E7

1F060000 UWB beacon2'sid Big-endian conversion 0000061F UWB idis 0000061F

DD checksum