W300GU-4GCat.1-tcp communication protocol

Directory

1 Overview	4

2 Device Usage Instructions	4

2.1 Device Function and Usage Instructions	4

2.2 Device Default Reporting Logic	7

2.3 Device Downlink Instructions	9

3 Protocol Data Packet Structure	15

3.1 Data Header	15

3.2 Message Identifier (Message ID)	15

3.3 Token Generation Mechanism	16

3.4 Payload	16

3.5 Checksum	17

4 Report messages	19

4.1 Connection related reports	19

4.1.1 LNK-LIN (MSGID=0xF0) Request connection (TCP only)	19

4.1.2 Connection Reply (MSGID=0xF1) (TCP Dedicated) - Important	20

4.1.3 New Heartbeat Packet Protocol (MSGID=0xF9) - Important	21

4.2 Alarm-related reporting	23

4.2.1 Alarm data upload - 1 (MSGID=0x02)	23

4.2.2 Alarm Data Upload - 2 (MSGID=0x21) (Supplement to 02)	27

4.2.3 Abnormal Data Alarm (MSGID=0x16)	34

4.  3 Location-related reporting	36

4.  3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)	36

4.3.2 Wi-Fi and base station information upload (MSGID=0xA4)	38

4.3.3 Bluetooth Location Information (BLE Location) (MsgId=0xD6)	43

4.3.4 Base station latitude and longitude reporting (MSGID=0x15)—wifi positioning supplement	46

4.3.4 UWB Location Information (UWB Location) (MsgId=0xD7)	48

4.4 Device information and status report	50

4.4.1 SIM card's ICCID upload (MSGID=0xF3)	50

4.4.2 Device charging status upload (MSGID=0xC3)	51

4.4.3 Status parameter reporting (MSGID=0xA9)	53

4.4.4 Device Status (MSGID=0xE9)	54

4.5 Health-related reporting	56

4.5.1 Health data (MSGID=0x32)	56

4.5.2 Device sleep analysis data upload (MSGID=0xC5)	59

4.6 Downlink feedback related reports	60

4.6.1 Downlink feedback (MSGID=0xC0)	60

4.6.2 Message Status Report (MSGID=0x28)	61

5 Settings	63

5.1 Downlink	63

5.1.1 Set Location Report Frequency (MSGID=0x17)	63

5.1.2 Message Send (MSGID=0X28)	65

5.1.2 Settings - Location priority settings (0XCE01&0XCE02)	67

5.1.4 Settings - Health Sampling Report Frequency Setting (MSGID=0X1C)	70

5.1.5 Setting - Device Alarm Setting (0XCE03)	73

5.1.6 Settings - Switch Settings (0XCE04-24)	75

5.1.7 Settings - Fall Sensitivity Settings (0xCE15)	85

5.1.8 Setting - Vibration Setting (0XCE23)	87

5.1.9 IP & Domain Name Settings (MSGID=0xC3) (TCP Only)	89

5.1.10 Issue sedentary stay alarm trigger time (MSGID=0XCC)	90

5.1.11 Shutdown and restart (MSGID=0x77)	91

5.1.12 Personal Information Distribution (MSGID=0xCA)	92

5.1.13 Sleep Statistics Time Period Setting (MSGID=0X1D)	94

5.1.14 Remote OTA Upgrade Distribution (MSGID=0xA9)	96

5.1.15 Weather Alert (MSGID=0XCB)	98

5.1.16 Restore Factory Settings (MSGID=0X7B)	102

5.1.17 Special firmware support - sending SOS prompt message (MSGID=0X80)	102

5.1.18 Special firmware support - Clear time for sending text messages (MSGID=0X2A)	103

5.1.19 UWB alarm distance issued (0x7900)	104

5.1.20 UWB Ranging Alarm Configuration (0x7901)	105

1 Overview

This protocol is suitable for 4G CAT1 devices and supports W300GU products. It uses a 32-bit data header for synchronization and terminal identification; a low-overhead checksum algorithm for checksum protection; and a message identifier to identify different messages.
It needs to be parsed according to the actual report. The document is currently being continuously maintained. If you find any errors or omissions, please give feedback in time. Thank you very much.
The protocol content is 4g reporting content. If you need Bluetooth broadcast data protocol, please consult relevant docking personnel.

2 Device Usage Instructions

2.1 Device Function and Usage Instructions

General version:

(1) Power on:

Please fully charge the device before using it for the first time. The device will automatically power on when the charging reaches the power required for power on. The charging status displays the charging icon.

A full charge is a green icon. Note: Do not check the device signal while charging.

Manual power on: Press and hold the up button for 10s and release. The interface will display "Welcome".

Note: The default wearing state is on power on, and the detachment alarm is reported if no heart rate is detected.

(2) Power off:

Low battery shutdown: The screen turns off after the interface displays Byebye.

Manual shutdown: Shutdown: Press and hold the button for more than 10 seconds, the interface displays Byebye and then the screen turns off.

Downlink command shutdown: Downlink shutdown command in the power-on state, the interface displays Byebye and then the screen turns off.

Note: Charging in the power-on state does not shut down

(3) SOS function:

Trigger mode: After triggering, the device does not enter sleep mode. Long press the down key for 3s, the interface displays SOS SEND OK/SOS sending successfully, and then the SOS icon appears.

Cancellation mode: In SOS mode, long press the down key for 3s, the interface displays SEND CENCEL/SOS canceled, and the SOS icon disappears.

(4) Signal status:

No signal: The interface signal is a horizontal line. Signal available: The interface signal has a stepped bar graph.

(5) Device sleep:

Trigger condition: The device does not move for 40 minutes and enters sleep mode, does not report location health data, and only reports F9 heartbeat packets to maintain a long connection.

(6) Ultra-long standby mode:

Manual change: Switch settings - ultra-long standby mode, click enable, the device will switch to ultra-long standby mode Remote command issued: Issue a switch to ultra-long standby mode, the device will switch to ultra-long standby mode Effect: There is a yellow bar below the power icon on the time interface, short connection, detect health and location every 10 minutes,

When the health data is normal and the location has not changed, the data is reported every 1 hour; otherwise, the data is reported every 10 minutes.

(7) Device parameters: Settings - Parameters interface: There are multiple pages, which can be switched by sliding the screen down

sos(Yes): Yes: Indicates that the device can be triggered by pressing the buttonsos,

sos(No): Indicates that thesos cannot be triggered by a button

Health(10min): Indicates that the device has enabled the health reporting function,

10min: Indicates that the current health data reporting frequency is10minutes,

Health (No): Indicates that the device health reporting function has been turned off

Location(BWG,10min): Indicates that the device has enabled the location reporting function,

BWG: Indicates the location priority Bluetooth\>wifi>Satellite (GPS/BDS),

10min: Indicates that the current location reporting frequency is10minutes,

Location (No): Indicates that the device location reporting function has been turned off

Fall (Medium,1.5m): Indicates that the device has enabled the fall alarm trigger function,

Medium: Indicates that the device's fall alarm trigger sensitivity is medium.

1.5m: Indicates that the device's fall alarm trigger height is 1.5m.

Fall (No): Indicates that the device has disabled the fall alarm trigger function.

Sedentary (2min): Indicates that the device has enabled the sedentary stay alarm trigger function.

2min: Indicates that the time to trigger the sedentary stay alarm is 2 minutes of inactivity.

Sedentary (No): Indicates that the device has disabled the sedentary stay alarm trigger function.

Health Broadcast (Yes):

Yes: Indicates that the device has enabled Bluetooth broadcasting of health data.

No: Indicates that the device has disabled Bluetooth broadcasting of health data.

Sleep statistics reporting time: (21:00-08:00), indicating that the sleep statistics reporting time is from 9 PM to 8 AM the next day.

Global vibration duration (1.500s): The current device vibration duration is1.5seconds

Health alarm threshold: such as heart rate threshold (90,100)BPM, indicating that the normal heart rate range is90-100,

Health abnormality alarms will be reported outside the normal range. Heart rate threshold (No): indicates that the heart rate threshold is turned off.

Allow work hibernation (Yes):

Yes: Indicates that the device has turned on the hibernation function. No: Indicates that the device has turned off the hibernation function.

Note: The hibernation function means that the device does not report data when it is in the detached state, and the device does not report data when it is stationary for 40minutes.

Network long connection (Yes):

Yes: Indicates that the current device tcp connection mode: long connection (always maintains the connection state, and can receive downlink instructions in real time),

No: Indicates short connection (disconnect after reporting, the device accepts downlink instructions when reporting data) GPSAlways on switch (No):

Yes: The internal interface is always open to collect Satellite (GPS/BDS). In general environments, it can speed up Satellite (GPS/BDS) positioning time, and the power consumption will increase.

The default is No, and it does not need to be turned on in a normal environment.

(8) Vibration:

Receive the downlink text message, health threshold alarm (the interface has text reminders and vibration)

The vibration duration can be controlled by downlink (see section 2.3)

2.2 Device Default Reporting Logic

General version:

(1) Connection-related reporting

F0Request: The device is a long connection. Under normal server connection and network conditions, it will report once at startup--a reply is required to connect to the server.

F9: Heartbeat package report, default4minutes report once, (previous firmware) location and health report will also follow a report, need to reply to maintain a long connection

(2) Location-related reporting

UWB/Satellite (GPS/BDS)/wifi/Bluetooth beacon: Default reporting frequency10minutes, default positioning priority: UWB>wifi>Satellite (GPS/BDS), UWB positioning is preferred, WIFI positioning is switched if positioning fails, and positioning is not switched if positioning is successful.

Base station (0x15): After wifi positioning fails, the communication base station latitude and longitude obtained from the module request automatically.

Low accuracy is used for auxiliary reference. Generally, this is not reported in the environment, and the positioning priority and WiFi positioning cannot be bound.

(3) Alarm-related reporting

SOSAlarm (0x02): User actively triggers, see the previous section for triggering methods

SOSCancel (0x02): User actively triggers cancellation, see the previous section for triggering methods

Shutdown alarm (0x21): Device actively shuts down or low battery shutdown, see the previous section for triggering methods

Wearing and falling off alarm (0x02): The device determines wearing according to the health sampling report frequency. If the heart rate is measured, the wearing alarm is reported. If the heart rate is not measured, the falling off alarm is reported.

Sedentary stay alarm (0x02): Default15minutes of immobility triggers reporting

Fall alarm (0x02): The device falls freely from a certain height and meets the fall algorithm trigger

Low battery alarm (0x02): The current battery level of the device is less than or equal to0when triggered

Abnormal temperature alarm (0x16): Reported when the device detects a temperature exceeding40degrees Celsius

Health threshold alarm (0x2105): No alarm by default. After the downlink health threshold, if the health data is not within the threshold range, an alarm is reported.

Note: When a health threshold alarm is reported, the device vibrates and the interface displays abnormal values.

UWBRange Alarm (0x21): The default is on, and all beacons will trigger the alarm. After setting the downlinkUWBRange Alarm Beacon, it will be triggered when the device is less than the issuedUWBAlarm distance.

Vibration is triggered in real time, and the wearing status is reported to the server only once within1minute; the non-wearing status is not reported to the server and does not generate vibration.

(4) Health-related reporting

Step counting, heart rate, body temperature&wrist temperature, blood pressure, blood oxygen (0x32): Default reporting frequency10minutes

Sleep (0xC5): Statistical time period21:00-08:00, sleep data will be reported according to the status during this time period

(5) Device information and status reporting

Status parameters (0xA9): A record will be reported when the device is turned on

SIM card ICCID (0xF3): A record will be reported when the device is turned on

Device status (0xE9): A record will be reported when the device is turned on, and a record will be reported when the reporting frequency is changed

Charging status (0xC3): Report when charging starts, charging ends, and when fully charged

(6) Downlink Feedback

Downlink Feedback (0xC0): The server's downlink command is reported after the device receives it.

Message Status Feedback (0x28): After the downlink text message, the device clicks accept or reject.

Note: The device reports in a packet reporting manner, that is, one data packet contains multiple complete messages. Pay attention not to miss any. The message is a complete message, and there will be no phenomenon of being interrupted in the middle and in the next data packet.

Eg:BDBDBDBDD6000119A9CF610445270387BF452708A1BC44279D18B74427E518B7F9BDBDBDBDF9010000006400002800000019A9CF61CA

This data packet contains messages for (0xD6) Bluetooth positioning and (0xF9) power signal.

(0xD6) Bluetooth positioning: BDBDBDBDD6000119A9CF610445270387BF452708A1BC44279D18B74427E518B7F9

(0xF9) Power signal: BDBDBDBDF9010000006400002800000019A9CF61CA

2.3 Device Downlink Instructions

General version: Downlink commands need to be continuously sent twice within 20 milliseconds to ensure success.

(1) Device location reporting frequency delivery (0x17):

The default reporting frequency is 10 minutes, with a maximum of 1 minute and a minimum of 1440 minutes. After the device receives the downlink command, the device reports according to the time period and frequency of the issued command.

Report data, and do not report outside the time period, such as: 00:00-18:00 2 minutes location report, then do not report outside the time period

(2) Text message delivery (0x28):

Unicodeencoding, up to40Chinese characters, one Chinese character occupies2bytes, and one English letter occupies2bytes

Message list - The historical messages can save up to50messages (Note: The firmware without a message list can save up to20messages)

(3) Device location priority delivery (0xCE01):

Default location priority uwb>wifi>Satellite (GPS/BDS), location priority

If the issued location priority is: wifi>Satellite (GPS/BDS)>Bluetooth beacon, then if wifi cannot be located, switch to Satellite (GPS/BDS), and if Satellite (GPS/BDS) cannot be located, switch to Bluetooth beacon, when the location is successful, it will not switch to the next location priority to generate location

(4) Device health sampling frequency delivery (0x1C): —Note the difference from W200P

The default health sampling frequency is 10 minutes, with a maximum of 2 minutes. After the device receives the downlink command, the device presses the issued command.

Report data on time period and frequency, and do not report outside the time period, such as: 00:00-18:00 Report location every 10 minutes, then do not report outside the time period

(5) Modify IP and port command issuance (0xC3):

The default universal version points to the Smart Cloud Platform: 118.178.184.219:8825. If you need to change it, please consult relevant personnel or visit the official website.

(6) Sedentary Stay Alarm Trigger Time Issuance (0xCC):

Value range: 2 minutes - 60 minutes, e.g., if the following behavior is a 10-minute trigger time, then the device will trigger a sedentary stay alarm if it remains motionless for 10 minutes.

(7) Device shutdown and restart issuance (0x77):

Note that this command is only effective when the device is powered on and cannot be received after shutdown.

Shutdown command: When this command is sent in the power-on state, the device will shut down and will not be able to receive downlink after shutdown.

Restart issuing: When this command is executed in the power-on state, the device restarts.

(8) Fall Sensitivity and Height Configuration (0xCE15):

The default fall sensitivity for the latest general firmware is: Low (Medium-Low). Fall height: 1.5m

Sensitivity: Refers to the degree to which the fall algorithm is satisfied, providing 5 setting levels (0 - 4): Low - Lower (Medium-Low) - Medium - Higher (Medium-High) - High.

Height: Refers to the height that satisfies the triggering of the fall alarm, providing 5 setting levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m- 2.5m.

(9) Remote OTA Configuration (0xA9):

Remote updates consume data and are affected by the network. The device's SIM card has 30M per month, so it cannot be updated too many times.

Otherwise, there will be insufficient data. This function requires device hardware to support remote updates. Previous devices do not support it.

(10) Key Shutdown Switch Command Configuration (0xCE16):

By default, the device can be shut down via the key. After the downlink is closed, the device cannot be shut down via the key.

(11) Control Device to Trigger Sleep Switch Configuration (0xCE18):

The default is the on state. The device enters sleep mode if it is not moved for 40 minutes. After the downlink is turned off, the device does not enter sleep mode.

(12) Fall Alarm Switch (0xCE07):

The fall alarm is enabled by default. After it is disabled, the device will not report fall alarms.

(13) Button Triggered SOS Start Switch (0xCE19)

It is enabled by default. Long pressing the charging cable button can trigger SOS. After it is disabled, long pressing the charging cable button will not trigger reporting SOS.

(14) Device Health and Location Reporting Switch (0xCE20)

By default, health reporting is enabled, and location reporting is enabled by default.

You can set the location / health to be on or off separately. After it is disabled, the device will not report health / location.

(15) Long Connection and Short Connection Mode Switching (0xCE22)

The default is long connection. You can downlink the long and short connection command to switch. The device will restart after receiving it and become long/short connection. Note that the long connection mode consumes slightly more power than the short connection mode. By default, a heartbeat packet (0xF9) is reported every 4 minutes.

(16) Health Threshold Setting (0xCE0300)

The health threshold is issued, the downlink command. After the device receives it, the normal range of the health threshold is issued. Health data outside this range will report an alarm.

(17) Device Vibration Setting (0xCE23)

After the downlink command, after the device receives it, the device vibration duration becomes the downlink duration, ranging from 0-60 seconds.

(18) Report Location Immediately (0xCE0A)

After the downlink command is issued and the device receives it, the device immediately starts positioning and reporting.

(19) Report Health Immediately (0xCE0B)

After the downlink command is issued and the device receives it, the device immediately starts health sampling and reporting.

(20) Sleep Statistics Time Period Issuance (0x1D)

Different time periods can be issued, such as: 13:00-14:00, then sleep statistics data will be collected and reported within this time period.

(21) GPS Always On Switch (0xCE24):

After it is turned on, the internal interface is always open to collect GPS. Under normal circumstances, it can speed up the GPS positioning time, but the power consumption will increase. The default is off.

(22) Switch to ultra-long standby mode (0xCE17):

After it is turned on, the device switches to ultra-long standby mode, and there is a yellow bar under the battery icon on the time interface, short connection,

Health and location are checked every 10 minutes. If the health data is normal and the location has not changed, the data is reported every 1 hour; otherwise, it is reported every 10 minutes.

(23) Personal information distribution (0xCA)

There is no personal information by default. Chinese name, English name, blood type, contact number, GB2312 encoding can be distributed,

Maximum number of bytes: name 40 bytes, English name 40 bytes, phone number 40 bytes, blood type 10 bytes

(24) Sedentary stay alarm switch (0xCE08)

It is enabled by default, and the trigger time is 15 minutes. After it is turned off, the device does not report sedentary stay alarms.

(25) Weather warning distribution (0xCB)

Multiple sets of weather warnings can be distributed. Note that each set of weather warnings must be different, and the same type of warning cannot appear.

(26) Data Reporting Switch (0xCE09):

Enable and disable data reporting. The default is enabled. After it is disabled, the device only reports 0xF9 messages to maintain a long connection.

(27) Restore Factory Settings (0x7B):

After being issued, the device returns to the default reporting status. See section 5.1.12 for the default status.

(28) UWB Alarm Distance (0x7900):

Controls the distance at which the UWB ranging alarm is triggered, where 0 meters means no alarm.

(29) UWB Ranging Alarm Configuration (0x7901):

The default is to enable the ranging alarm status. The UWB beaconid that triggers the ranging alarm of the watch can be configured downstream.

3 Protocol Data Packet Structure

A basic protocol data packet structure is shown in Figure 1:

3.1 Data Header

Each data packet starts with a 4-byte Header or token (timestamp replaces it in some response messages):

Currently, the token uploaded by OuFu devices is fixed as BDBDBDBD

Header: 0xBD 0xBD0xBD0xBD；

Timestamp: 32bits, generated by the server

3.2 Message Identifier (Message ID)

See Chapter 4 for the content represented by MessgeId.

Each time TCP is connected, the device side will first report the 0xF0 message, which contains the device's unique identifier IMEI.

The server needs to record this IMEI as an identifier and reply with the 0xf1 message. The device will consider the connection successful; otherwise, the connection will be disconnected.

For normal device communication, the long connection reports a 0xF9 heartbeat packet every 4 minutes by default. The 0xF9 heartbeat packet is reported once after the health location is reported.

The short connection reports a 0xF9 heartbeat packet once at startup, and the 0xF9 heartbeat packet is reported once after the health location is reported. The two reports are not affected by each other.

Note that after the server receives the F9 message, the server needs to issue a command to reply, otherwise the connection will be disconnected.

3.3 Token Generation Mechanism

Currently fixed as BDBDBDBD

3.4 Payload

The payload below refers to the valid body content in the protocol except for the head token and checksum. The following note is the length of the body.

The data format used in the payload is shown in the following table:

【U-unsigned；I-signed；X-bitfield；Number - Number of bytes】

The following protocols use little-endian priority except for ch, u8, i8, and x8

| Short | peTypeType | Size (Bytes) | Min/max | Resolution | Explanation |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short Integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short Integer |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Plastic surgery |
| x16 | Bitfield | 2 | - | - | Bit 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long Integer |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit long integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating point type |

3.5 Checksum

The content added by the checksum includes the payload, as shown in Figure 1. The algorithm is as follows. The default general version of the device does not require checksum, and this part can be ignored. Any byte can be used for the downlink instruction.

//Assuming content="BDBDBDBDE9000600010A00000A00", then sum.ToString("X2") result is 07//The following is the C# code calling methodprivate string CheckSum(string content)

{

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2");

}

4 Report messages

4.1 Connection related reports

4.1.1 LNK-LIN (MSGID=0xF0) Request connection (TCP only)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xF0 | See below Definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | u64 | IMEI | 1 | - | IMEI number (little-endian) |
| 2 | x16 | version |  | - | Bitfield see below |

The device's reported F0 request message must have a server downlink 0XF1 message in response (see the next section for the specific format), otherwise the login will fail

Example:

Original16Hexadecimal message: BDBDBDBDF09B51731BC61603000000C2

BDBDBDBD:4Bytes message headerHeader

F0:Message ID(Message id)9B51731BC6160300:IMEI number(little-endian), converted to big-endian:000316C61B73519B,

The corresponding decimal is IMEI:869465050010011,0000:version

C2: checksum

Connection related Q&A:

Each time a new TCP connection is created, an F0 request is first reported, which contains the IMEI, and then the server records this IMEI.

After that, all the data in this connection belongs to this IMEI.

Note: If F0 is not received or F0 communication is abnormal, you can use a third-party network testing tool to verify whether the server communication is normal.

4.1.2 Connection Reply (MSGID=0xF1) (TCP Dedicated) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Timestamp(unix) Timestamp | 0xF1 | See below See below definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | u32 | Token | 1 | - | Token: Fixed as BDBDBDBD |

Note that the message sequence is different from the normal reported message. This message is issued by the server and is not the content reported by the device. The server reply needs to be in the current channel. > Note that the message sequence is different from the normal reported message. This message is issued by the server and is not the content reported by the device. The server reply needs to be in the current channel.

Example:

Original16hexadecimal message:07FD8860F1BDBDBDBD2E

07FD8860:(Little endian),Timestamp(unix) timestamp (unit: seconds), converted to big endian6088FD07 Converted to decimal:1619590407

Converted to standard time formatUTCTime:2021-04-28 06:13:27

F1:Message ID(Message ID):

BDBDBDBD:Token

2E：checksum

The timestamp 10 decimal to standard time format is the result of adding 1619590407 seconds to 1970-01-01 00:00:00

The device synchronizes the time with the timestamp (unit: seconds) in this message.

Note:javaThe general unit is milliseconds, pay attention to the conversion to unit: seconds

Note: The total number of bytes replied by the server downlink is10bytes, such as:0xBD--1byte; if the device receives20bytes, the connection will not be established

4.1.3 New Heartbeat Packet Protocol (MSGID=0xF9) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xF9 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery Type 0: 4-level system, 1: 5-level system, 2: Percentage, 3: Voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery Value If Bat_type is 0: then the battery value range is 0-3 (0 is 25%, 3 is 100%) If Bat_type is 1: then the battery value range is 0-4 (0 is 20%, 4 is 100%) If Bat_type is 2: then the battery value range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: percentage, 1: 5-level system, 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal strength |
| 1 | U8 | Other_type |  |  | Extended type 0: Full step counting 1: Incremental step counting 2: Vibration |
| 4 | U32 | Num |  |  | Extended value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Note: The F9 heartbeat packet must have a server downlink reply. The device considers the server connection to be unbroken only after receiving the server downlink reply; otherwise, it will re-report the F0 request connection.

Example:

Original16hexadecimal message: BDBDBDBDF90104000050000095000000E377BD67AA

BDBDBDBD: Header

F9: Message ID

01: Bat_type battery type 01-->1-->5-level battery (0-4)

0400: Bat_volt battery value converted to big-endian 0004-->4-->level4corresponds to100% battery

00: Signal_type signal type 00-->0-->percentage

5000: Signal_strength signal strength converted to big-endian0050-->converted to decimal80-->80% signal strength

00: Other_type extended type 00-->0-->full step counting

95000000:Num extension value - step count converted to big-endian00000095--> converted to decimal149--> step count is149

E377BD67: Timestamp converted to big-endian67BD77E3--> converted to10decimal1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> Converted to Beijing time:2025-02-25 15:57:23

AA: checksum

Note: The server can reply with a fixed message upon receiving the F9 heartbeat packet: Fixed reply example BDBDBDBDF301 Byte count:6

4.2 Alarm-related reporting

4.2.1 Alarm data upload - 1 (MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x02 | See below See below definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below |
| 4 | U32 | Timestamp | - | - | Timestamp (a timestamp will be added at the end when re-uploading) |

Upl_warn contents

| bit (corresponds to the position of 1 in binary) | Name | Description | Original message little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 14 | Fall alarm | Fall alarm | 0040 | 4000 | 4*4096=16384 |
| 8 | Device wearing | Device wearing | 0001 | 0100 | 1*256=256 |
| 7 | SOS Cancel | SOS Cancel | 8000 | 0080 | 8*16=128 |
| 5 | Sedentary Alarm (No Movement) | Sedentary Alarm (No Movement) | 2000 | 0020 | 2*16=32 |
| 4 | Remove (Fall Off) Device | Remove (Fall Off) Device | 1000 | 0010 | 1*16=16 |
| 2 | Shutdown | Shutdown | 0400 | 0004 | 4 |
| 1 | SOS Alarm | SOS Alarm | 0200 | 0002 | 2 |
| 0 | Low Battery | Low Battery | 0100 | 0001 | 1 |

Example:

Wearing Alarm: BDBDBDBD020001E377BD678A

BDBDBDBD: Header

02:Message ID

0001:Upl_warn alarm content, converted to big-endian:0100--> Wearing alarm

E377BD67: Timestamp converted to big-endian67BD77E3--> converted to10decimal1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> Converted to Beijing time:2025-02-25 15:57:23

8a: checksum

Detachment alarm: BDBDBDBD021000E377BD678A

BDBDBDBD: Header

02:Message ID

1000:Upl_warn alarm content, converted to big-endian0010--> Detachment alarm

E377BD67: Timestamp converted to big-endian67BD77E3--> converted to10decimal1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> Converted to Beijing time:2025-02-25 15:57:23

8a: checksum

SOS alarm: BDBDBDBD020200E377BD678A

BDBDBDBD: Header

02:Message ID

0200:Upl_warn alarm content, converted to big-endian0002--> SOS alarm

E377BD67: Timestamp converted to big-endian67BD77E3--> converted to10decimal1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> Converted to Beijing time:2025-02-25 15:57:23

8a: checksum

SOS cancellation: BDBDBDBD028000E377BD678A

BDBDBDBD: Header

02:Message ID

8000:Upl_warn alarm content, converted to big-endian0080--> SOS cancellation

E377BD67: Timestamp converted to big-endian67BD77E3--> converted to10decimal1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> Converted to Beijing time:2025-02-25 15:57:23

8a: checksum

Fall alarm: BDBDBDBD020040E377BD678A

BDBDBDBD: Header

02:Message ID

0040:Upl_warn alarm content, converted to big-endian4000--> Drop alarm

E377BD67: Timestamp converted to big-endian67BD77E3--> converted to10decimal1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> Converted to Beijing time:2025-02-25 15:57:23

E7: checksum

Sedentary alarm: BDBDBDBD022000E377BD678A

BDBDBDBD: Header

02:Message ID

2000:Upl_warn alarm content, converted to big-endian0020--> Sedentary alarm

E377BD67: Timestamp converted to big-endian67BD77E3--> converted to10decimal1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> Converted to Beijing time:2025-02-25 15:57:23

8A: checksum

Low battery alarm: BDBDBDBD020200E377BD678A

BDBDBDBD: Header

02:Message ID

0100:Upl_warn alarm content, converted to big-endian0001--> Low battery alarm

E377BD67: Timestamp converted to big-endian67BD77E3--> converted to10decimal1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> Converted to Beijing time:2025-02-25 15:57:23

8A: checksum

Shutdown alarm: BDBDBDBD020400E377BD678A

BDBDBDBD: Header

02:Message ID

0400:Upl_warn alarm content, converted to big-endian0004--> Shutdown alarm

E377BD67: Timestamp converted to big-endian67BD77E3--> converted to10decimal1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> Converted to Beijing time:2025-02-25 15:57:23

8A: checksum

Note: This alarm is used for devices that do not distinguish between shutdown types. See the next section for devices that distinguish between shutdown types.

4.2.2 Alarm Data Upload - 2 (MSGID=0x21) (Supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x21 | See below See below definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | Alarm type |
| N |  | Upl_warn | - | - | Alarm content See below for different Alarm type type definitions |
| 4 | U32 | Timestamp |  |  | Timestamp (a timestamp will be added at the end when re-uploading) |

Alarm type=1

Upl_warn contents

| bit (corresponds to the position of 1 in binary) | Name | Description | Original message little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging shutdown alarm | Device charging automatic shutdown (firmware requires function support) | 04000000 | 0004 | 4 |
| 1 | Low battery shutdown alarm | Device power is too low to shut down (firmware requires function support) | 02000000 | 0002 | 2 |
| 0 | Active shutdown alarm | The device is manually shut down by someone (firmware needs function support) | 01000000 | 0001 | 1 |

Example:

Charging shutdown alarm: BDBDBDBD21010004000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

04000000:Upl_warn alarm content, converted to big-endian 0004-->charging shutdown alarm

E377BD67:Timestamp converts the timestamp to big-endian 67BD77E3-->converts to decimal 1740470243-->the timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Low battery shutdown alarm: BDBDBDBD21010002000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

02000000:Upl_warn alarm content, converted to big-endian 0002-->Low battery shutdown alarm

E377BD67:Timestamp converts the timestamp to big-endian 67BD77E3-->converts to decimal 1740470243-->the timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Active shutdown alarm: BDBDBDBD21010001000000E377BD6767

BDBDBDBD:Header

21:Message ID

0100:Alarm type converted to big-endian 0001-->Alarm type=1

01000000:Upl_warn alarm content, converted to big-endian 0001-->Active shutdown alarm

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Alarm type=4

Upl_warn contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | 02—UWB ranging alarm |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple groups of information, and there may be multiple Ubeacons in each group |
| U8 | Total_PackCount | 1 | Total number of alarm Ubeacons at the current time |
| U32 | UBeacon0 mac | 4 | UBeacon mac |
| U16 | Distance0 | 2 | Distance (unit: centimeters) |
| U32 | UBeacon1 mac | 4 | UBeacon mac |
| U16 | Distance1 | 2 | Distance (unit: centimeters) |
| U32 | UBeacon2 mac | 4 | UBeacon mac |
| U16 | Distance2 | 2 | Distance (unit: centimeters) |
| ..... | ..... | ..... | ..... |

Example:

Original hexadecimal message:BDBDBDBD210400020103320600009100E70500006800380500005C00E377BD6755

BDBDBDBD:Header

21:Message ID

0400:Alarm type alarm type converted to big endian 0004-->Alarm type=4

02:Type converts to decimal 02--UWB ranging alarm

01:Total_groups Total number of groups converted to decimal 01, 1 group

03:Total_PackCount Total number of alarm Ubeacons at the current time converted to decimal 03, 3 UWB beacons

32060000:UBeacon0 mac converted to big endian 00000632, UWB beacon id is 00000632

9100:Distance0 distance converted to big endian 0091-->converted to decimal 145-->distance from UWB beacon is 145 cm

E7050000:UBeacon1 mac

6800:Distance1 distance converted to big endian 0068-->converted to decimal 105-->distance from UWB beacon is 105 cm

38050000:UBeacon2 mac

5C00:Distance2 distance converted to big endian 005C-->converted to decimal 92-->distance from UWB beacon is 92 cm

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

55:checksum

Alarm type=5

Upl_warn contents

| Format | Name | Unit | Description |
| --- | --- | --- | --- |
| U16 | Type | 2 | Fixed to 0x05 |
| U32 | Timestamp | 4 | Timestamp |
| U16 | Content Len | 2 | Alarm content length |
| U8 | Health Warn Type1 | 1 | Health threshold alarm type: 0x01 heart rate, 0x02 systolic blood pressure (SBP), 0x03 diastolic blood pressure (DBP), 0x04 blood oxygen, 0x05 temperature |
| U8 | Type1 | 1 | 01 is less than the set health threshold, 02 is greater than the set health threshold |
| I16 | Health Value1 | 2 | Health Outcome Value |
| U8 | Health Warn Type2 | 1 | Health threshold alarm type: 0x01 heart rate, 0x02 systolic blood pressure (SBP), 0x03 diastolic blood pressure (DBP), 0x04 blood oxygen, 0x05 temperature |
| U8 | Type2 | 1 | 01 is less than the set health threshold, 02 is greater than the set health threshold |
| I16 | Health Value2 | 2 | Health Outcome Value |
| .... | .... | .... | .... |

Example:

The device detected that the heart rate (65) is outside the range of the issued health threshold: BDBDBDBD210500E377BD6704000102410073

BDBDBDBD: Header

21:Message ID message id

0500:Alarm typeAlarm type Convert to big-endian0005-->Alarm type=5

E377BD67:Timestamp Convert to big-endian67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

0400:Content Len Alarm content length Convert to big-endian0004-->The alarm content length is 4 bytes. Note: The length will increase if there are multiple health threshold alarms.

01:Health Warn Type Health threshold alarm type 01-->Heart rate alarm

02:Type Greater than the set health threshold & less than the set health threshold 01-->Less than the set health threshold

4100:Health Value Actually reported health value Convert to big-endian0041-->Convert to decimal 65-->The reported heart rate value is 65

73:checksum

4.2.3 Abnormal Data Alarm (MSGID=0x16)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x16 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type | -/- | -/- | Alarm type (0: Heart rate abnormal value alarm, 1: Temperature abnormal value alarm) |
| 2 | U16 | Heart | -/- | -/- | Heart rate value |
| 2 | U16 | temperature | -/- | -/- | Temperature |
| 2 | U16 | Pa | -/- | -/- | Air pressure |
| 4 | 4*U8 or U32 | expand | -/- | -/- | 4 bytes expansion for other sensors |

Note: The reported outlier has no practical reference value, it only indicates that the current detected data is abnormal (such as the value out of range or the sensor detection is abnormal, etc.). The outlier will be removed in other normal messages, and it is reflected in this message, which can be ignored.

Example:

Temperature anomaly value alarm: BDBDBDBD16010000FF0F000000000000E6

BDBDBDBD:Header

16:Message ID

01:Type alarm type 01-->Temperature anomaly value alarm

0000:Heart Heart rate value. The default value is 0000 if there is no such value.

FF0F:temperature Temperature. Convert to big-endian 0FFF-->Temperature anomaly value FFF

0000:Pa Air pressure. The default value is 0000 if there is no such value.

00000000 expand Expand. The default value is 0000 if there is no such value.

E6:checksum

4. 3 Location-related reporting

4. 3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x03 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude Longitude |
| 8 | Double | lat |  | - | latitude latitude |
| 1 | U8 | north_south |  | - | /N or S/ North or South latitude |
| 1 | U8 | east_west |  | - | /E or W/ East or West longitude |
| 1 | U8 | status |  | - | /A or V/ Valid data or invalid data |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original16hexadecimal message: BDBDBDBD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BDBDBDBD:Header

03:Message ID Message ID

963E870C3E675E40:lon longitude Convert to big endian405E673E0C873E96-->Longitude:121.6131622 Note: See the parsing example below for conversion and parsing

FF2110C2B6343F40:lat latitude Convert to big endian`403F34B6C21021FF-->Latitude:31.2059137 Note: See the parsing example below for conversion and parsing

4E:north_south 4E-->Convert to ASCII code N-->North latitude

45:east_west 45-->Convert to ASCII code E-->East longitude

41:status Data validity 41-->Convert to ASCII code A-->Valid data

E377BD67:Timestamp Convert to big endian67BD77E3-->Convert to decimal101740470243-->Timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> Converted to Beijing time:2025-02-25 15:57:23

78:checksum

Note: The parsed Satellite (GPS/BDS) latitude and longitude are in the WGS-84coordinate system. If the map uses Baidu AMAP, etc., the coordinate system needs to be converted.

Parsing satellite positioning message Java code example:

//Message: BDBDBDBD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BDBDBDBD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public static void main(String[] args){

//Message 7d9f84ac81815c40 Actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d",16)));  //114.02353966666665//Message e766926b1d893640 Actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16))) ;//22.535605166666667

HexToStr(data.Substring(“4e”));   //N

HexToStr(data.Substring(“45”));//E

HexToStr(data.Substring(“41”));//A   A indicates valid data, V indicates invalid data, which can be discarded//Message 749d695f Actual value 5f699d74

Date date=new Date();

date.setTime(Long.parseLong(“5f699d74",16)*1000);

SimpleDateFormatsdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.3.2 Wi-Fi and base station information upload (MSGID=0xA4)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xA4 | See below See below definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 1 | u8 | Cell_cnt | 1 | - | Number of base stations (0-7) Number of cell info payload.Valid value:0~7 |
| 2 | u16 | Cell[0].MCC | - | - | MMC of base station (0) mobile country code of cell[0] |
| 2 | u16 | Cell[0].MNC | - | - | MNC of base station (0) mobile network code of cell[0] |
| 2 | u16 | Cell[0].LAC | - | - | LAC of base station (0) Location area code of cell[0] |
| 4 | U32 | Cell[0].CELL_ID | - | - | ID of base station (0) Cell id of cell[0] |
| 2 | i16 | Cell[0].RSSI | - | dbm | Signal strength RSSI of base station (0) RSSI in dbm of cell[0] |
| ... | ... | ... | ... | ... | ... |
| 2 | u16 | Cell[cell_cnt-1.MCC | - | - | MMC of base station (N) mobile country code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.MNC | - | - | MNC of base station (N) mobile network code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.LAC | - | - | LAC of base station (N) Location area code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1].LAC | - | - | LAC of base station (N) Location area code of cell[[cell_cnt-1] |
| 4 | U32 | Cell[cell_cnt-1].CELL_ID | - | - | ID of base station (N) Cell id of cell[[cell_cnt-1] |
| 2 | I16 | Cell[cell_cnt-1].RSSI | - | dbm | Signal strength RSSI of base station (N) RSSI in dbm of cell[[cell_cnt-1] |
| 1 | U8 | Wifi_cnt |  |  | Number of wifi Number 0f wifi |
| 6 | 6*U8 | Wifi[0].bssid[0]-Wifi[0].bssid[5] |  |  | MAC address of wifi(0) |
| 4 | I32 | Wifi[0].RSSI |  |  | Signal strength RSSI of wifi(0) |
| ... | ... | ... | ... | ... | ... |
| 6 | 6*U8 | Wifi[Wifi_cnt-1].bssid[0]-Wifi[Wifi_cnt-1].bssid[5] |  |  | MAC address of wifi(N) |
| 4 | I32 | Wifi[0].RSSI |  |  | Signal strength RSSI of wifi(0) |

Note: The device may not report base stations every time. The absence of base stations does not affect positioning. If the number of base stations in the report is 0, then the following is directly the number of Wi-Fi networks.

Example:

Original16Hexadecimal message: BDBDBDBDA4E377BD6701CC010000C218D1AD160BF9FF078CBEBE1A8162C6FFFFFFC061180AF42AC1FFFFFF200BC726E000B6FFFFFFA8154DF6517EB2FFFFFFE005C5B1F824CCFFFFFFE8FCAFA02663AFFFFFFF6409805B2B9CAEFFFFFF94

BDBDBDBD:Header

A4: Message ID message id

E377BD67: Timestamp converted to big-endian 67 BD77E3--> converted to decimal 1740470243 --> timestamp is 1740470243 seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> Converted to Beijing time:2025-02-25 15:57:23

01: Cell_cnt base station count 01 --> base station count is 1

CC01: Cell[0].MCC Base station MCC converted to big-endian 01 CC--> converted to decimal 460 --> base station MCC is 460

0000: Cell[0].MNC Base station MNC converted to big-endian 0000 --> converted to decimal 0 --> base station MNC is 0

C218: Cell[0].LAC Base station LAC converted to big-endian 18 C2--> converted to decimal 6338 --> base station LAC is 6338

D1AD160B: Cell[0].CELL_ID Base station ID converted to big-endian 0 B16ADD1--> converted to decimal 186035665 --> base station id is 186035665

F9FF: Cell[0].RSSI Base station signal strength converted to big-endian FFF9--> converted to decimal complement -7 --> base station signal strength is -7

07: Wifi_cnt wifi count 07 --> 7 wifi

8CBEBE1A8162: wifi mac address 8C:BE:BE:1A:81:62-->wifi[0]'s mac address is 8C:BE:BE:1A:81:62

C6FFFFFF: Convert WiFi signal strength RSSI to big-endian FFFFFFC6 --> Convert to decimal complement - 58 ---> WiFi [0] RSSI signal strength is - 58

C061180AF42A:wifi mac address C0:61:18:0A:F4:2A-->The MAC address of wifi[1] is C0:61:18:0A:F4:2A

C1FFFFFF:wifi signal strength RSSI converted to big endian FFFFFFC1-->converted to decimal complement -63--->The RSSI signal strength of wifi[0] is -63

200BC726E000:wifi mac address 20:0B:C7:26:E0:00-->The MAC address of wifi[2] is20:0B:C7:26:E0:00

B6FFFFFF:wifi signal strength RSSI converted to big endian FFFFFFB6-->converted to decimal complement -74--->The RSSI signal strength of wifi[0] is -74

A8154DF6517E:wifi mac address A8:15:4D:F6:51:7E-->The MAC address of wifi[3] is A8:15:4D:F6:51:7E

B2FFFFFF:wifi signal strength RSSI converted to big endian FFFFFFB2-->converted to decimal complement -78--->The RSSI signal strength of wifi[0] is -78

E005C5B1F824:wifi mac address E0:05:C5:B1:F8:24-->The MAC address of wifi[4] is E0:05:C5:B1:F8:24

CCFFFFFF:wifi signal strength RSSI converted to big endian FFFFFFCC-->converted to decimal complement -52--->The RSSI signal strength of wifi[0] is -52

E8FCAFA02663:wifi mac address E8:FC:AF:A0:26:63-->The MAC address of wifi[5] is E8:FC:AF:A0:26:63

AFFFFFFF:wifi signal strength RSSI converted to big endian FFFFFFFAF-->converted to decimal complement -81---->The RSSI signal strength of wifi[0] is -81

6409805 B2B9C: wifi mac address 64: 09: 80: 5B: 2B: 9C-->wifi[6]的mac地址为64: 09: 80: 5B: 2B: 9C

AEFFFFFF: Wifi signal strength RSSI converted to big-endian FFFFFFFFAE-->converted to decimal complement -82--->Wifi[0] RSSI signal strength is -82

94: checksum

The parameters parsed from the message need to be connected to the map wifi database, and the latitude and longitude data can be finally obtained through the map side. Take AMAP connecting to the wiif database as an example:

According to the access network method, AMAP wifi positioning protocol document is divided into three positioning scenarios. The following gives corresponding examples for the three positioning scenarios.

1. The network connection method is wifi access, that is, the accesstype parameter is 1

http://apilocate.amap.com/position?accesstype=1&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&mmac=50:a7:2b:1e:56:58,-60,alibaba-inc&macs=4c:48:da:25:0b:11,-59,alibaba-

inc|4c:48:da:25:1a:11,-77,alibaba-inc&serverip=10.2.166.4&output=xml&key=<用户 Key>

2. The network connection method is mobile access and the mobile phone card is a non-cdma card, that is, when the accesstype parameter is 0 and cdma=0:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=0&bts=460,01,40977,2205409,-65&nearbts=460,01,40977,2205409,-65|460,0

1,40977,2205409,-65|460,01,40977,2205409,-65&serverip=10.2.166.4&output=xml&key=<User Key>

3. When the network type is mobile access and the mobile card is a CDMA card, i.e., the accesstype parameter is 0 and cdma=1:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=1&bts=13824,1,1838,1674723,575739,-52&serverip=10.2.166.4&output=xml&key=<User Key>

W200P watch: AMAP wifi positioning protocol calls AMAP interface description: When selecting parameters, generally select the first type

You can also select the second type, but if the device does not report the base station, the bts and neabts parameters need to be removed before it can

The first type, example of incoming parameters: mmac is the first WIFI mac address in the messagehttp://apilocate.amap.com/position?accesstype=1&imei=869907057780989&mmac=04:8C:16:A9:FB:A8,-44,TP-Link&macs=04:8C:16:A9:FB:A8,-44,TP-Link|C8:3A:35:F4:CE:18,-59,TP-Link|1C:78:4E:D5:07:FE,-79,TP-Link|C8:3A:35:9D:0B:E0,-87,TP-Link|4A:D8:90:E8:0A:82,-88,TP-Link|AC:AD:4B:C9:C3:87,-89,TP-Link|A4:16:E7:55:84:AC,-91,TP-Link&output=json&key=xxxx

For the specific AMAP wifi positioning protocol PDF, you can click to view and download:(not a screenshot, you can click to view)
This document is provided by AMAP, you can directly ask AMAP for specific docking
Note that the name of the downloaded document will change, you can rename it yourself

4.3.3 Bluetooth Location Information (BLE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xD6 | See below See below definition | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 0 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple sets of information, and each group may contain multiple iBeacons |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | RSSI (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | RSSI (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | RSSI (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | RSSI (Bluetooth beacon signal strength) |
| ... | ... | ... | ... |

Note: In the general version, the total number of Bluetooth beacon groups is fixed at 1, and the maximum number of Bluetooth beacons is 4. The sorting is based on signal strength from largest to smallest. By default, the watch only recognizes our own beacons. If you need to connect to other manufacturers' beacons, please consult relevant personnel via WeChat. Example:

Original hexadecimal message: BDBDBDBDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

bdbdbdbd: Header

D6: Message ID

00: type

01: Only one set of iBeacon data

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

04: The first group has 4 beacon information

4327: Convert to big endian --> major : 0x2743 --> Convert to decimal 10051

1794: Convert to big endian --> minor: 0x9417 --> Convert to decimal 37911

AC:  RSSI 0xAC --> Convert to decimal complement -84

4327: Convert to big endian --> major: 0x2743 --> Convert to decimal 10051

3094: Convert to big endian --> minor:0x9430 --> Convert to decimal 37936

AA:  RSSI 0xAA --> Convert to decimal complement -86

4327:Convert to big endian --> major: 0x2743 --> Convert to decimal 10051

b956:Convert to big-endian --> minor:0x56b9 --> Convert to decimal 22201

A5: RSSI 0xA5-->Convert to decimal complement -91

4327: Convert to big endian --> major: 0x2743 --> Convert to decimal 10051

FE94: Convert to big-endian -->minor:0x94FE-->Convert to decimal 38142

A5: rssi:0xA5-->Convert to decimal complement -91

6a  --Checksum

Bluetooth beacon major, minor description: The default shipping Bluetooth beacon has a beacon code attached.

For example: 00081005345866, then the major of the Bluetooth beacon is 10053, the minor is 45866, and the manufacturer ID is 0008 (this is useless and has no practical significance).

4.3.4 Base station latitude and longitude reporting (MSGID=0x15)—wifi positioning supplement

Note: This message is generally when wifi positioning fails, and the module automatically requests the latitude and longitude of the communication base station for positioning (the positioning priority cannot be changed). The location is the location of the communication base station, which can only be used as a reference for auxiliary positioning, and the accuracy of the communication base station is not high.

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x15 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | flag |  | -/- | Extended segment identifier |
| 4 | U32 | lon |  | -/- | Longitude (*10 to the power of 6 or 7) longitude |
| 4 | U32 | lat |  | -/- | latitude (*10 to the power of 6 or 7) latitude |
| 1 | U8 | north_south |  | -/- | /N or S/ South or North latitude |
| 1 | U8 | east_west |  | -/- | /E or W/ East or West longitude |
| 1 | U8 | status |  | -/- | /A or B or V/ A, B are valid data, B corresponds to 7-digit precision, V is invalid data |
| 4 | U32 | Timestamp |  | -/- | Timestamp |
| N |  | Num |  | -/- | The universal version has no extensions for the time being. The table only retains the definition. Definition: Extended segment value 1 can be the nth digit value defined by the extended segment). When multiple extended segments are used simultaneously, extended segment 1 corresponds to the lowest digit content, and the content is extended to digit n in sequence. |
| ... | ... | ... | ... | ... | ... |

Example:

Original16Hexadecimal message: BDBDBDBD1502C8C9E53F855AD30F4E4542E377BD67F4

BDBDBDBD: Header

15:Message ID

02:typetype=02indicates LBS base station positioning

C8C9E53F: Longitude ---> Convert to big-endian4857963E---> Convert to longitude:26.5509509, Note: Conversion analysis reference4.3.1GPS/ BDS Location Reporting: Location data reporting (0x03) Section analysis example

855AD30F: Dimension ---> Convert to Big Endian129B82E0--->Convert to Dimension:107.2024008, Note: Refer to section4.3.1GPS/BDS Location Reporting: Location Data Reporting (0x03) for parsing examples

4E:north_south 4E-->Convert to ASCII encoding N-->North latitude

45:east_west 45-->Convert to ASCII encoding E-->East longitude

42:status 42-->Convert to ASCII encoding B, valid data, with a precision of7decimal places

E377BD67: Timestamp converted to big-endian67BD77E3--> converted to10decimal1740470243--> timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> Converted to Beijing time:2025-02-25 15:57:23

08:checksum

4.3.4 UWB Location Information (UWB Location) (MsgId=0xD7)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xD7 | See below See below definition | CK_sum |

Payload contents

| Format() | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 01 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple groups of information, and there may be multiple Ubeacons in each group |
| Int32 | Timestamp | 4 | UTC timestamp |
| U8 | Total_PackCount | 1 | Total number of UBeacons at the current time |
| U32 | UBeacon0 mac | 4 | UBeacon devid (little-endian) |
| U16 | Distance0 | 2 | Distance (centimeters) |
| U32 | UBeacon1 mac | 4 | UBeacon devid (little-endian) |
| U16 | Distance1 | 2 | Distance (centimeters) |
| U32 | UBeacon2 mac | 4 | UBeacon devid (little-endian) |
| U16 | Distance2 | 2 | Distance (centimeters) |
| ... | ... | ... | ... |

Note: The W300PGU general version reports a maximum of 1 group and a maximum of 4 UBeacons.

Example:

Original16Hexadecimal message: BDBDBDBDD70101E377BD6703320600009100E70500006800380500005C00A1

BDBDBDBD: Header

D7: Message ID

01: Type 01

01: Total_groups Total number of groups, converted to decimal01-->Total number of groups is1

E377BD67: Timestamp Convert to big-endian67BD77E3-->Convert to decimal1740470243-->Timestamp is1740470243seconds-->

Converted to standard time format UTC time:2025-02-25 07:57:23--> Converted to Beijing time:2025-02-25 15:57:23

03: Total_PackCount Number of UWB beacons Convert to10decimal03-->Number of UWB beacons is3

32060000: UBeacon0 mac UWB beacon's id is converted to big-endian00000632-->UWB beacon id is00000632

6800: Distance0 Ranging distance converted to big-endian0091-->Convert to decimal145-->Ranging distance is145cm

E7050000: UBeacon1 mac UWB beacon's id converted to big-endian000005E7-->UWB beacon id is000005E7

9100: Distance1 Ranging distance converted to big-endian0068-->Convert to decimal104-->Ranging distance is104cm

38050000: UBeacon2 mac UWB beacon's id converted to big-endian00000538-->UWB beacon id is00000538

5C00: Distance2 Ranging distance converted to big-endian005C-->Convert to decimal92-->Ranging distance is92cm

A1:checksum

4.4 Device information and status report

4.4.1 SIM card's ICCID upload (MSGID=0xF3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xF3 | See below See below definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 10 | 10*U8 | ICCID | 1 | - | ICCID number - Device SIM card number ICCID |

Note: This message is reported once every time the device is turned on.

Example:

Original hexadecimal message:BDBDBDBDF389861118236001639994CC

BDBDBDBD:Header

F3:Message ID

89861118236001639994:ICCID Device SIM card number ICCID is: 89861118236001639994

CC:checksum

4.4.2 Device charging status upload (MSGID=0xC3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Status | / | / | 0 starts, 1 ends, 2 fully charged |
| 1 | U32 | Timestamp | / | / | Timestamp (a timestamp will be added at the end when re-uploading) |

Note: This message requires device firmware support, and the latest general version is supported by default Example:

Original hexadecimal message :BDBDBDBDC301E377BD678A

BDBDBDBD:Header

C3:Message ID

01:Device stopped charging

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

Original hexadecimal message :BDBDBDBDC300DB4D2F668A

BDBDBDBD:Header

C3:Message ID

00:Device started charging

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

Original hexadecimal message :BDBDBDBDC302DB4D2F668A

BDBDBDBD:Header

C3:Message ID

02:Device is fully charged

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

4.4.3 Status parameter reporting (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Number of parameter types |
| 1 |  |  |  |  | (Reserved 00) |
| 1 | u8 | Type | -/- | -/- | Parameter type 1, Type definition: 00—MCU (firmware), 01—Module, 02 Sensor (no such type in the general version) |
| 1 | U8 | NameLen | -/- | -/- | Parameter name 1 length |
| n | N*u8 | Name | -/- | -/- | Parameter name |
| 1 | u8 | Type | -/- | -/- | Parameter type 2, Type definition: 00—MCU (firmware), 01—Module, 02 Sensor (no such type in the general version) |
| 1 | U8 | NameLen | -/- | -/- | Parameter Type 2 Length |
| n | N*u8 | Name | -/- | -/- | Parameter name |
| ... | ... | ... | ... | ... | ... |

Note: This message is reported once by default at startup, uploading the device firmware version number and module version number. It can be left unparsed. The Message ID of previously shipped devices was 0xBB. Contact relevant personnel to issue it.

Example:

BDBDBDBDA902000009423732352E4F56303601174E5432364B434E4230304E4E412D4C3032303330393530B5

BDBDBDBD:Header

A9:Message ID

02:TypeCnt indicates that there are 2 types of parameters

00:Reserved field

00:Type=00 00 represents MCU (firmware)

09:The length of the MCU name after is 9 bytes

423732352E4F563036--->Hexadecimal to text (device firmware version number)--->B725.OV06

01:Type=01 01 represents the module

17:The length of the communication module name after is 23 bytes

4E5432364B434E4230304E4E412D4C3032303330393530:Hexadecimal to text (4G module version number)

--->NT26KCNB00NNA-L02030950

B5---checksum

4.4.4 Device Status (MSGID=0xE9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xE9 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | type |  | / | Default—00 |
| 2 | u16 | length |  | / | Length of the following message |
| 1 | u8 | location |  | / | Location reporting frequency: 00 - Default, not modified; 01 - Modified by downlink (if there are multiple time periods in the downlink, only the reporting frequency of the current time period is reported) |
| 2 | u16 | frequency |  | / | Frequency, unit - minutes |
| 1 | u8 | health |  | / | Health reporting frequency: 00—-default not modified, 01—-modified by downlink |
| 2 | u16 | frequency |  | / | Frequency: unit - minutes |

Note: This message is reported once at startup, and once after the downlink modifies the frequency.

Example:

Original hexadecimal message:BDBDBDBDE9000600010A00010A0033

BDBDBDBD:Header

E9:Message ID

00:type=00

0600:length The length of the message behind is converted to big-endian-->0006-->converted to decimal 6, the message length is 6 bytes

01:location 01 indicates that the downlink has modified the location reporting frequency

0A00:frequency The location reporting frequency is converted to big-endian -->000A--> converted to decimal 10, and the reporting frequency is 10 minutes

01:health 01 indicates that the downlink has modified the health reporting frequency

0A00:frequency The health reporting frequency is converted to big-endian -->000A--> converted to decimal 10, and the reporting frequency is 10 minutes

33:checksum

If the device has no health reporting function or no location reporting function, then the health reporting frequency and

The two values of location reporting frequency will remain the same, indicating the device reporting frequency

4.5 Health-related reporting

4.5.1 Health data (MSGID=0x32)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x32 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | 00 |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 2 | U16 | contentLength |  |  | Total length of content below |
| 1 | U8 | ID |  |  | Id (Type + Reported Value Length), see below for ID type definition |
| N | N*U8 | Val1 |  |  | Reported value of Id (byte length varies depending on the reported value length) |
| ... | ... | ... | ... | ... | ... |
| 1 | U8 | ID |  |  | Id (Type + Reported Value Length), see below for ID type definition |
| N | N*U8 | Val1 |  |  | Reported value of Id (byte length varies depending on the reported value length) |

ID type definition (hexadecimal):

01 Step count02 Heart rate 03 Body temperature 04 Wrist temperature 06 Diastolic pressure 07 Systolic pressure 08 Blood oxygen 09 HRV(Special version support)

(05 Blood glucose 10 Sit-up count11 Running pace and distance 12 Rope skipping speed and count )---Reserved, none for now

Default universal version support:01 Step count02 Heart rate 03 Body temperature 04 Wrist temperature 06 Diastolic pressure 07 Systolic pressure 08 Blood oxygen

Example:

Original16hexadecimal message: BDBDBDBD3200E377BD6711000A1E00114B314A39711A4A0122bc00416212

BDBDBDBD:Header

32:Message ID

00:type  type=00

E377BD67:Timestamp converted to big-endian 67BD77E3-->convert to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

1100: The length of the following message is converted to big-endian-->0011-->converted to decimal17, the length of the following message is17bytes

0A:ID 0x0A-->convert to binary 00001010 The first five digits are 00001-->convert to hexadecimal 01, which represents the data ID step counting (0x01),

The last three digits are010-->convert to decimal02, which means the length of the step counting data is2bytes

1E00: Step counting reported value converted to big-endian-->001E-->converted to decimal30, a total of30steps in the step counting data

11:ID 0x11-->convert to binary 00010001 The first five digits are 00010-->convert to hexadecimal 02, which represents the data ID heart rate (0x02),

The last three digits are001--> convert to decimal1, representing the length of the heart rate data as1byte

4B: Heart rate reporting value converted to decimal -->75, heart rate data is75

31:ID 0x31--> convert to binary00110001 The first five digits are 00110--> convert to16hexadecimal06, representing data ID diastolic blood pressure (0x06),

The last three digits are001--> convert to decimal1, representing the length of the diastolic blood pressure data as1byte

4A: Blood pressure diastolic reporting value, converted to decimal -->74, diastolic blood pressure data is74

39:ID 0x3--> convert to binary00111001 The first five digits are 00111--> convert to16hexadecimal07, representing data ID systolic blood pressure (0x07),

The last three digits are001--> convert to decimal1, representing the length of the systolic blood pressure data as1byte

71: Blood pressure systolic reporting value, converted to decimal -->113, systolic blood pressure data is113

1A:ID 0x1A--> convert to binary00011010 The first five digits are 00011--> convert to16hexadecimal03, representing data ID body temperature (0x03),

The last three digits are010--> convert to decimal2, representing the length of the body temperature data as2bytes

4A01: Body temperature reporting value, converted to big-endian -->014A--> converted to decimal330-->330*0.1--> body temperature data is33degrees Celsius

22:ID  0x22-->Convert to binary00100010   The first five digits are00100-->Convert to16Hexadecimal04, representing data ID wrist temperature (0x04),

The last three digits are010-->Convert to decimal2, representing the length of the wrist temperature data as2bytes

BC00: Wrist temperature reported value, converted to big-endian-->00BC-->Convert to decimal188-->188*0.1-->Wrist temperature data is18.8degrees Celsius

41:ID  0x41-->Convert to binary01000001    The first five digits are01000-->Convert to16Hexadecimal08, code data ID blood oxygen (0x08),

The last three digits are001-->Convert to decimal1, representing the blood oxygen data length as1byte

62: Blood pressure reported value, converted to decimal-->98, blood oxygen data is98

12: checksum

4.5.2 Device sleep analysis data upload (MSGID=0xC5)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC5 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | Int32 | DateTime |  | -/- | Upload UTC start time |
| 4 | Int32 | DateTime |  |  | Upload Utc end time |
| 2 | Int16 | Sleepminute |  |  | Upload sleep time in minutes |
| 4 | Int32 | Type |  |  | Sleep state: Upload type, 1 is deep sleep, 2 is light sleep, 3 is wake time |

Example:

Original16hexadecimal message: BDBDBDBDC5AC338860693B8860210001000000D1

BDBDBDBD: Header

C5:Message ID

AC338860: Start time, converted to big endian608833AC--> converted to decimal --> timestamp is1619538860seconds -->

Convert to standard time format UTC time:2021-04-27 15:54:20-->Convert to Beijing time:2021-04-27 23:54:20

693B8860: End time, converted to big endian60883BAC--> converted to decimal --> timestamp is1619540908seconds -->

Convert to standard time format UTC time: 2021 - 04 - 27 16:28:28 --> Convert to Beijing time: 2021 - 04 - 28 00:28:28

2100: Sleepminute sleep time, converted to big-endian 0021 --> converted to decimal 33 --> The total sleep time is 33 minutes

01000000: Type sleep state, converted to big-endian 00000001 --> converted to decimal 1 --> 1 is deep sleep, deep sleep time is 33 minutes

D1: checksum

4.6 Downlink feedback related reports

4.6.1 Downlink feedback (MSGID=0xC0)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC0 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This instruction is used for feedback of downlink instructions, returning the Message ID of the previously received downlink instruction (multiple Message IDs can be returned collectively)

Original hexadecimal message:BDBDBDBDC0011720

BDBDBDBD:Header

C0:Message ID

01:length Message ID length Convert decimal 01--> Message ID length is 1 byte

17Type indicates that the device received the downlink instruction Message ID is 17

20:checksum

4.6.2 Message Status Report (MSGID=0x28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x28 | See below See below definition | CK_sum |

Payload contents

| Bytes | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | timestamp | -/- | -/- | Timestamp |
| 1 | U8 | Type | -/- | -/- | Character encoding type (corresponding to downlink 0X28), GB2312 encoding: 0x03; Unicode encoding: 0x05 |
| 1 | U8 | status | -/- | -/- | Message Status - 0x01: Confirmed, 0x02: Rejected |
| 4 | U32 | seqID | -/- | -/- | (Corresponds to the downlink 0X28 message ID seqID) |

Example:

Note:Currently, the general version does not distinguish the character encoding type and is fixed to 03. Subsequent versions will distinguish it.

Original hexadecimal message:BDBDBDBD28FD3BA068030268A03BE477

BDBDBDBD:Header

28:Message ID

FD3BA068:timestamp Timestamp converted to big-endian 68A03BFD-->converted to decimal-->timestamp is 1755331581 seconds-->

Convert to standard time format UTC time: 2025-08-16 08:06:21-->Convert to Beijing time: 2025-08-16 16:06:21

03:Type 03--fixed value

02:status Message status 02--The interface clicks the reject button--Rejected

68A03BE4:The seqID downlink message ID is 68A03BE4

77:checksum

5 Settings

5.1 Downlink

5.1.1 Set Location Report Frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x17 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |

Note: A day is from 00:00-23:59
Example:

00:00-23:59 Report location every 10minutes:BDBDBDBD17010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

17:Message ID

01:enable Whether to enable Convert to decimal01-->1--Enabled

0A00:Interval time interval Converted to big-endian000A-->Convert to decimal10-->Time interval is10minutes

00:time_start_h Start time-hour, convert to decimal00-->Start time-hour is00

00:time_start_m Start time-minute, convert to decimal00-->Start time-minute is00

17:time_end_h End time-hour, convert to decimal23-->End time-hour is 23

3B:time_end_m End time-minute, convert to decimal59-->End time-minute is 59

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

97: checksum

5.1.2 Message Send (MSGID=0X28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x28 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Description |
| --- | --- | --- | --- |
| 1 | U8 | type | Message type, if it is downlink text information, Unicode encoding is 05 |
| 4 | Uint32 | seqID | The ID of the information, unique |
| 1 | U8 | CONTENT LEN | Content length |
| N | N | CONTENT | Content, Chinese is Unicode encoded (little-endian first, pay attention to the conversion, one Chinese character occupies 2 bytes, one letter occupies 2 bytes) |

Note: The default Chinese character encoding in the Hong Kong general version is Unicode. The information ID cannot be repeated for each transaction. The device cannot receive repeated IDs. Example:

Text message: hello,world  BDBDBDBD28050300000016680065006C006C006F002C0077006F0072006C006400DD

BDBDBDBD:Header

28: Message ID

05: type message type Convert to decimal 05-->type=05

03000000: seqID information id Information id is 03000000

16: CONTENT LEN content length Convert to decimal 22, content length is 22 bytes

680065006C006C006F002C0077006F0072006C006400:

Convert every 2 bytes to big-endian --> convert to Unicode characters --> combine final - text message content:hello,world

DD:checksum

5.1.2 Settings - Location priority settings (0XCE01&0XCE02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See below definition | CK_sum |

Payload contents

Type=01 Location priority setting

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 is always valid, 01 is valid this time |
| 2 | U16 | Len | -/- | -/- | The length of the following instruction |
| n | n |  | -/- | -/- | Text |

Positioning priority setting definition--See 2.3 Device Downlink Description--Device Positioning Priority Distribution (0xCE01)

Positioning priority setting text:

Currently, the general version supports:01--Satellite (GPS/BDS),02--wifi (wifi+base station)03--Bluetooth beacon (requires additional deployment of Bluetooth beacons),06---UWB beacon (requires additional deployment of UWB beacons);

Special equipment support:04--LBS base station 05--125k

Example:

WiFi positioning priority (WiFi > Bluetooth beacon > Satellite (GPS/BDS)):BDBDBDBDCE0100030002030133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Location Priority Setting

00:Valid Validity 00 Always Valid

0300:Len Converted to big-endian 0003 --> Converted to decimal 03 --> The length of the text is 3 bytes

020301:Text: 02--wifi 03--Bluetooth beacon 01--GPS The positioning priority is wifi>Bluetooth beacon>GPS

33:checksum

Satellite (GPS/BDS) positioning priority (Satellite (GPS/BDS)>wifi>Bluetooth beacon):BDBDBDBDCE0100030001020333

BDBDBDBD:Header

CE:Message ID

01:Type 01 Location Priority Setting

00:Valid Validity 00 Always Valid

0300:Len Converted to big-endian 0003 --> Converted to decimal 03 --> The length of the text is 3 bytes

010203:Text: 01--GPS 02--wifi 03--Bluetooth beacon The positioning priority is GPS>wifi>Bluetooth beacon

33:checksum

Bluetooth positioning priority (Bluetooth beacon>wifi>Satellite (GPS/BDS)):BDBDBDBDCE0100030003020133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Location Priority Setting

00:Valid Validity 00 Always Valid

0300:Len Converted to big-endian 0003 --> Converted to decimal 03 --> The length of the text is 3 bytes

030201:Text: 03--Bluetooth beacon 02--wifi 01--GPS The positioning priority is Bluetooth beacon>wifi>GPS

33:checksum

UWB positioning priority (UWB beacon > wifi > Satellite (GPS/BDS)):BDBDBDBDCE0100030006020133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Location Priority Setting

00:Valid Validity 00 Always Valid

0300:Len Converted to big-endian 0003 --> Converted to decimal 03 --> The length of the text is 3 bytes

060201:Text: 06--UWB beacon 02--wifi 01--Satellite (GPS/BDS) The positioning priority is Bluetooth beacon > wifi > Satellite (GPS/BDS)

33:checksum

Single Satellite (GPS/BDS) positioning:BDBDBDBDCE0100030001000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Location Priority Setting

00:Valid Validity 00 Always Valid

0300:Len Converted to big-endian 0003 --> Converted to decimal 03 --> The length of the text is 3 bytes

030201:Text: 01--Satellite (GPS/BDS) 00--No positioning 00--No positioning The positioning priority is single Satellite (GPS/BDS) positioning

33:checksum

Single wifi positioning:BDBDBDBDCE0100030002000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Location Priority Setting

00:Valid Validity 00 Always Valid

0300:Len Converted to big-endian 0003 --> Converted to decimal 03 --> The length of the text is 3 bytes

020000:Text: 02--wifi 00--No positioning 00--No positioning The positioning priority is single wifi positioning

33:checksum

Single Bluetooth beacon positioning:BDBDBDBDCE0100030003000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Location Priority Setting

00:Valid Validity 00 Always Valid

0300:Len Converted to big-endian 0003 --> Converted to decimal 03 --> The length of the text is 3 bytes

020000:Text: 03--Bluetooth beacon 00--No positioning 00--No positioning The positioning priority is single Bluetooth beacon positioning

33:checksum

Single UWB beacon positioning:BDBDBDBDCE0100030006000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Location Priority Setting

00:Valid Validity 00 Always Valid

0300:Len Converted to big-endian 0003 --> Converted to decimal 03 --> The length of the text is 3 bytes

060000:Text: 06--UWB beacon 00--No positioning 00--No positioning The positioning priority is single UWB beacon positioning

33:checksum

5.1.4 Settings - Health Sampling Report Frequency Setting (MSGID=0X1C)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x1D | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |

Health sampling reporting frequency setting definition -- See 2.3 Device downlink description -- Device health sampling frequency distribution (0x1D)

Note: After setting, report according to time period, and do not report outside the time period

Example:

00:00-23:59(All day) Report health data every 10 minutes: BDBDBDBD1C010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

1C:Message ID

01:enable Whether to enable Convert to decimal01-->1--Enabled

0A00:Interval time interval Converted to big-endian000A-->Convert to decimal10-->Time interval is10minutes

00:time_start_h Start time-hour, convert to decimal00-->Start time-hour is00

00:time_start_m Start time-minute, convert to decimal00-->Start time-minute is00

17:time_end_h End time - hour, convert to decimal 23 --> Start time - hour is 00

3B:time_end_m End time - minute, convert to decimal 59 --> Start time - hour is 00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

97: checksum

5.1.5 Setting - Device Alarm Setting (0XCE03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See below definition | CK_sum |

Payload contents

Type=03 Device alarm setting

| Byte offset | Format | Name | Unit | Description |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Fixed to 0x03 - Device alarm setting |
| 1 | U8 | Valid | 1 | Fixed to 0x00 - Health threshold, always valid |
| 2 | U16 | Cfg Len | 2 | Instruction length |
| 1*N | U8*N | [Bit7] Enable [Bit6 - 0] Threshold Type | 1*N | [Bit7] Whether to enable the threshold detection of this type [Bit6 - 0] Set the threshold type: 0x01 heart rate, 0x02 systolic blood pressure (SBP) 0x03 diastolic blood pressure (DBP), 0x04 blood oxygen, 0x05 temperature (threshold unit is *10, for example, 37.3 is 373) |
| 2*N | I16*N | Threshold Min | 2*N | Lower Threshold (This content is only available when threshold detection is enabled) |
| 2*N | I16*N | Threshold Max | 2*N | Threshold Upper Limit (This content is only available when threshold detection is enabled) |

Device Alarm Settings Definition-Health Threshold--See2.3 Device Downlink Instructions--Health Threshold Settings(0xCE0300)

Note: The default is the closed state, and the threshold is the normal range. The device will report an alarm outside this range.

Heart rate threshold distribution range (20-200), systolic blood pressure threshold distribution range (20-200), diastolic blood pressure threshold distribution range (20-200), blood oxygen threshold distribution range (80-100), temperature threshold distribution range (160-600)

Example:

Set multiple health thresholds,1. Heart rate [60,120],2. Turn off blood oxygen threshold detection,3. Temperature [36.0,37.3]:BDBDBDBDCE03000B00813C007800048568017501FF

BDBDBDBD: Header

CE:Message ID

03:Type 03--Device alarm settings

00:Valid 00--Health threshold, always valid

0B00: Convert to big-endian-->000B--> Convert to decimal11--> The length of the subsequent instruction message (excluding checksum) is 11 bytes

81: Convert to binary -->10000001   bit7:1---On     On status   bit6-0:000001-->0x01  Heart rate

3C00: Convert to big-endian -->003C--> Convert to decimal60, the lower limit of the heart rate threshold is60

7800: Convert to big-endian -->0078--> Convert to decimal120, the upper limit of the heart rate threshold is120

04: Convert to binary -->00000100   bit7:0---Off, no threshold after closing   bit6-0:0000100-->0x04  Blood oxygen

85: Convert to binary -->10000101  bit7:1---On     On status   bit6-0:0000101-->0x05  Temperature

6801: Convert to big-endian -->0168--> Convert to decimal360(actual temperature *10)--> The lower limit of the temperature threshold is36.0

7501: Convert to big-endian -->0175--> Convert to decimal373(actual temperature *10)--> The upper limit of the temperature threshold is37.3

FF: checksum

5.1.6 Settings - Switch Settings (0XCE04-24)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Please see the instructions for the specific type |
| 1 | u8 | switch | -/- | -/- | Switch 00 on, 02 off, 01 single valid |
| 2 | U16 |  | -/- | -/- | Fixed 0000 |

Device alarm setting - switch type: specific function description--see 2.3 Device downlink description

05Bluetooth broadcast switch06Location and health data reporting switch ----Previously used in firmware, no longer used in the latest firmware

07Fall alarm switch08Stay alarm switch

0A Report location information immediately0B Report health data immediately

0D Report version number data immediately16 Whether the downlink can use the button to shut down

17 Downlink supports long standby mode18 Sleep function switch

19 Button triggers sos alarm switch20 Report data switch--Separate settings (Note: This switch structure is special, and there are differences in the message and table definitions. Please see the specific example)

22 Long and short connection mode switching24 GPS always on switch

Type=05 Bluetooth broadcast switch Example:

00---On : BDBDBDBDCE0500000093

BDBDBDBD:Header

CE:Message ID

05:Type 05 Bluetooth broadcast switch

00:switch switch 00 on

0000:Fixed value

93:checksum

02---Off : BDBDBDBDCE0502000093

BDBDBDBD:Header

CE:Message ID

05:Type 05 Bluetooth broadcast switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=06 Location and health data reporting switch - used by previous firmware, not used by the latest firmware, reserved here Example:

00---Open (a report will be sent immediately after being issued in the closed state):BDBDBDBDCE0600000093

BDBDBDBD:Header

CE:Message ID

06:Type 06 Location and health data reporting switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE0602000093

BDBDBDBD:Header

CE:Message ID

06:Type 06 Location and health data reporting switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=07 Fall alarm switch
Example:

00---On:BDBDBDBDCE0700000093

BDBDBDBD:Header

CE:Message ID

07:Type 07 Fall alarm switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE0702000093

BDBDBDBD:Header

CE:Message ID

07:Type 07 Fall alarm switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=08 Stay alarm switch
Example:

00---On:BDBDBDBDCE0800000093

BDBDBDBD:Header

CE:Message ID

08:Type 08 Stay Alarm Switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE0802000093

BDBDBDBD:Header

CE:Message ID

08:Type 08 Stay Alarm Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=0A Immediately Report Location Information
Example:

01---Single Report:BDBDBDBDCE0A01000093

BDBDBDBD:Header

CE:Message ID

0A:Type 0A Immediately Report Location Information

01:switch switch 01--Single issue single report

0000:Fixed value

93:checksum

Type=0B Immediately Report Health Data
Example:

01---Single Report:BDBDBDBDCE0B01000093

BDBDBDBD:Header

CE:Message ID

0B:Type 0B Immediately Report Health Data

01:switch switch 01--Single issue single report

0000:Fixed value

93:checksum

Type=0D Immediately report version number data
Example:

01---Single report:BDBDBDBDCE0D01000093

BDBDBDBD:Header

CE:Message ID

0D:Type 0D Immediately report version number data

01:switch switch 01--Single issue single report

0000:Fixed value

93:checksum

Type=16 Whether the downlink can use the key to shut down Example:

00---Open:BDBDBDBDCE1600000093

BDBDBDBD:Header

CE:Message ID

16:Type 16 Whether the downlink can use the key to shut down

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE1602000093

BDBDBDBD:Header

CE:Message ID

16:Type 16 Whether the downlink can use the key to shut down

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=17 Downlink supports long standby mode Example:

00---Open:BDBDBDBDCE1700000093

BDBDBDBD:Header

CE:Message ID

17:Type 17 Downlink supports long standby mode

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE1702000093

BDBDBDBD:Header

CE:Message ID

17:Type 17 downlink supports long standby mode

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=18 Sleep function switch
Example:

00---On, enters sleep mode after being stationary for a period of time, and does not report data:BDBDBDBDCE1800000093

BDBDBDBD:Header

CE:Message ID

18:Type 18 Sleep function switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Off, normal data reporting in stationary or charging state:BDBDBDBDCE1802000093

BDBDBDBD:Header

CE:Message ID

18:Type 18 Sleep function switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=19 Key trigger sos alarm switch
Example:

00---On:BDBDBDBDCE1900000093

BDBDBDBD:Header

CE:Message ID

19:Type 19 Key trigger sos alarm switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE1902000093

BDBDBDBD:Header

CE:Message ID

19:Type 19 Key trigger sos alarm switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=20 Report data switch - separate setting Example:

Add field: Body length: 2 bytes Body: 0x00 (Location data report)  + Switch (0x00: On, 0x02: Off)

0x01 (Health data report) + Switch (0x00: On, 0x02: Off)

Location data reporting enabled:BDBDBDBDCE20000200000093

BDBDBDBD:Header

CE:Message ID

20:Type 20 Report data switch

00:Fixed value

0200:Body length converted to big-endian 0002--> converted to decimal 02--> body length is 2 bytes

00:Body: 00--Location data report switch

00:Body: 00--Enable reporting

93:checksum

Location data reporting disabled:BDBDBDBDCE2000020000293

BDBDBDBD:Header

CE:Message ID

20:Type 20 Report data switch

00:Fixed value

0200:Body length converted to big-endian 0002--> converted to decimal 02--> body length is 2 bytes

00:Body: 00--Location data report switch

02:Body:00--Close reporting

93:checksum

Health data reporting enabled:BDBDBDBDCE20000200010093

BDBDBDBD:Header

CE:Message ID

20:Type 20 Report data switch

00:Fixed value

0200:Body length converted to big-endian 0002--> converted to decimal 02--> body length is 2 bytes

01:Body:00--Health data reporting switch

00:Body: 00--Enable reporting

93:checksum

Health data reporting disabled:BDBDBDBDCE20000200010293

BDBDBDBD:Header

CE:Message ID

20:Type 20 Report data switch

00:Fixed value

0200:Body length converted to big-endian 0002--> converted to decimal 02--> body length is 2 bytes

01:Body:00--Health data reporting switch

02:Body:02--Close reporting

93:checksum

Type=22 Long and short connection mode switching
Example:

00---Long connection mode, default is long connection:BDBDBDBDCE2000000093

BDBDBDBD:Header

CE:Message ID

22:Type 22 Long and short connection mode switching

02:switch switch 00 Enable long connection mode

0000:Fixed value

93:checksum

02---Short connection mode:BDBDBDBDCE2002000093

BDBDBDBD:Header

CE:Message ID

22:Type 22 Long and short connection mode switching

02:switch switch 02 Close long connection mode -- short connection mode

0000:Fixed value

93:checksum

Type=24 GPS always-on switch Example:

00---Open : BDBDBDBDCE2400000093

BDBDBDBD:Header

CE:Message ID

24 : Type 24 GPS always-on switch

02:switch switch 00 on

0000:Fixed value

93:checksum

02---Close :BDBDBDBDCE2402000093

BDBDBDBD:Header

CE:Message ID

24 : Type24 GPS always-on switch

02:switch switch 02 off

0000:Fixed value

93:checksum

5.1.7 Settings - Fall Sensitivity Settings (0xCE15)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Description |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Type is 0x15 |
| 1 | U8 | Valid | 1 | The default is 0, which has no impact for now. |
| 2 | U16 | Len | 2 | Length of subsequent parameter content |
| 1 | U8 | Param Type | 1 | Parameter type: 0x00: Sensitivity, 0x01: Height |
| 1 | U8 | Param Level | 1 | Parameter level: 5 levels (0 - 4) |

Drop sensitivity setting-Parameter description:

Sensitivity: Refers to the degree to which the drop algorithm is triggered, providing 5 setting levels (0 - 4): Low - Lower (Medium Low) - Medium - Higher (Medium High) - High.

Height: Refers to the height that satisfies the trigger of the fall alarm, providing 5 setting levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m - 2.5m.

If the set height is "1.5m", the device needs to be ≥1.5mto trigger the fall alarm.

Note that sensitivity and height cannot be combined in a message

Example:

Set the sensitivity to "Medium Low": BDBDBDBDCE150002000001FF

BDBDBDBD:Header

CE:Message ID

15:Type Fall Sensitivity Setting

00:Fixed value

0200:Len

00:Param Type Sensitivity Setting

01:Param Level Sensitivity Level 1-->Lower (Medium Low)

FF:checksum

Set the height to “1.5m”: BDBDBDBDCE150002000102FF

BDBDBDBD:Header

CE:Message ID

15:Type Fall Sensitivity Setting

00:Fixed value

0200:Len

01:Param Type Height Setting

02:Param Level Height Level 2-->1.5m

FF:checksum

5.1.8 Setting - Vibration Setting (0XCE23)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Description |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Fixed at 0x23 |
| 1 | U8 | Valid | 1 | 0x00 |
| 2 | U16 | Cfg Len | 2 | Length of subsequent parameter content |
| 1 | U8 | Motor Type | 1 | Setting type: 1. Vibration duration - 0x00 |
| 4 | U32 | Value | 4 | Setting content: 1. Vibration duration (in milliseconds, range 0 - 60 seconds) |

Setting - Vibration setting definition: This message can be used to modify the global vibration duration, and the duration of any vibration will be changed to the duration set by this command.

Example:

The default global vibration duration of the device is 1.5 seconds (1500 milliseconds):BDBDBDBDCE2300050000dc050000FF

BDBDBDBD:Header

CE:Message ID

23: Type 23 Setting - Vibration setting

00: Fixed value 00--always valid

0500:The length of the following parameter content is converted to big-endian 0005-->converted to decimal 05-->the length of the following parameter content is 5 bytes

00:Motor Type Vibration duration setting

dc050000:Value Vibration duration Converted to big-endian 000005dc-->converted to decimal 1500-->vibration duration is 1500 milliseconds (1.5 seconds)

FF:checksum

5.1.9 IP & Domain Name Settings (MSGID=0xC3) (TCP Only)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | Delivery type: Type =1 IPv4, Type =2 IPv6 (reserved - not currently supported), Type =3 Domain name (currently only supports ASCII encoded domain names) |
| 2 | U16 | Port |  |  | Port number (2 bytes) |
| 1 | U8 | Len |  |  | Length of subsequent parameter content |
| N | U8 | Domainname |  |  | Specific IP or domain name content |

Note: After delivery, the device no longer communicates on the original server, but points to the modified server.

Example:

Type=1 IPv4:Downlink modification ip is:118.178.184.219, port is: 8825 BDBDBDBDC30179220476B2B8DB33

BDBDBDBD:Header

C3:Message ID

01:Type 1--IPv4

7922:Port port converted to big-endian 2279-->converted to decimal 8825-->port: 8825

04:Len The length of the following parameter content is converted to decimal 04-->the length is 4 bytes

76B2B8DB: Domainname IP 76-->Convert to decimal 118, B2-->Convert to decimal 178, B8-->Convert to decimal 184, DB-->Convert to decimal 219 IP is: 118.178.184.219

33:checksum

Type=3 Domain nameThe downlink modifies the domain name to:aiday.com.cn, the port is: 8825

BDBDBDBD:Header

C3:Message ID

03:type=3, domain name

7922:Port port converts to big-endian --2279 --> converts to decimal port: 8825

12:Len The length of the following parameter content is converted to decimal 18-->The message length is 18 bytes

61696461792e636f6d2e636e:Domainname domain name converted to string-->aiday.com.cn-->domain name is: aiday.com.cn

1D:checksum

5.1.10 Issue sedentary stay alarm trigger time (MSGID=0XCC)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCC | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | minute | -/- | -/- | Stay alarm trigger duration (unit: minutes; value range 2 — 60). Inactivity within this time period will trigger a stay alarm. |

Example:

No activity for 5 consecutive minutes. The watch reports a stay alarm (when the watch is worn): BDBDBDBDCC05DD

BDBDBDBD:Header

CC: Message ID

05: minute Stay alarm trigger duration Convert decimal 05-->5 minutes

DD: checksum

5.1.11 Shutdown and restart (MSGID=0x77)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x77 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- |  | 01—Shutdown, 00—Restart |

Note: Shutdown and restart are both received in the power-on state. Shutdown: The watch shuts down after receiving the command in the power-on state, and no commands can be received after shutdown; Restart: The watch restarts after receiving the command in the power-on state.

Example:

Shutdown:BDBDBDBD770193

BDBDBDBD:Header

77: Message ID

01:Type 01--Shutdown

93:checksum

Restart:BDBDBDBD770093

BDBDBDBD:Header

77: Message ID

01:Type 00--Restart

93:checksum

5.1.12 Personal Information Distribution (MSGID=0xCA)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCA | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | count | -/- |  | Distribute several sets of information |
| 1 | U8 | type | -/- |  | Type: 1 – Name, 2 – English Name, 3 – Phone, 4 – Blood Type |
| 1 | U8 | len | -/- |  | Length |
| N | U8 | "content" | -/- |  | Content (Unicode encoding) |
| ... | ... | ... | ... | ... | ... |

Example:

Name: Cheng Dawen, English name: Chan Tai Man, phone:99886633, blood type: A

BDBDBDBDCA0401060B7A2759876502184300680061006E00200054006100690020004D0061006E0003103900390038003800360036003300330004024100DD

BDBDBDBD: Header

CA: Message ID

04: count converted to decimal04-->4 groups of information

01:type 01--Name

06:len converted to decimal06-->length 6 bytes

0B7A27598765: content

0B7A converted to big endian-->7A0B-->converted to Unicode encoded character-->Cheng,

2759converted to big endian-->5927-->converted to Unicode encoded character-->Da,

8765Convert to big-endian-->6587-->Convert to Unicode encoded characters-->Text Chinese name is Cheng Da Wen

02:type 02--English name

18 len Convert to decimal24-->Length 24 bytes

4300680061006E00200054006100690020004D0061006E00:content

Similarly, every2bytes are converted to big-endian-->Convert to Unicode characters-->Final: Chan Tai Man

03:type 03--Phone number

10:len Convert to decimal16-->Length 16 bytes

39003900380038003600360033003300:content

Similarly, every2bytes are converted to big-endian-->Convert to Unicode string-->Final: 99886633

04:type 04--Blood type

02:len Convert to decimal02-->Length 2 bytes

41: Convert to Unicode string-->A-->Blood type: A

DD: checksum

5.1.13 Sleep Statistics Time Period Setting (MSGID=0X1D)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x1D | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 1 |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 2 |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 3 |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 4 |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |

Example:

Sleep data statistics and report time period is 13:00 - 15:00 and 22:30 - 08:00BDBDBDBD1D010D000F0001161E080000000000000000000000FF

BDBDBDBD:Header

1D:Message ID

01:enable Whether to enable Convert decimal 01-->1--Enable

0D:time_start_h Start time - hour Convert decimal 13

00:time_start_m Start time - minute Convert decimal 00 Start statistics and report time is 13:00

0F:time_end_h End time - hour Convert decimal 15

00:time_end_m End time - minute Convert decimal 00 End statistics and report time is 15:00

01:enable Whether to enable Convert decimal 01-->1--Enable

16:time_start_h Start Time - Hour Convert to Decimal 22

1E:time_start_m Start Time - Minute Convert to Decimal 30 Start statistics and report time is 22:30

08:time_end_h End Time - Hour Convert to Decimal 08

00:time_end_m End Time - Minute Convert to Decimal 00 End statistics and report time is 08:00

00:enable Whether to enable Convert to decimal 00-->0--Not enabled

00000000:All fill 00 in disabled state

00:enable Whether to enable Convert to decimal 00-->0--Not enabled

00000000:All fill 00 in disabled state

FF:checksum

5.1.14 Remote OTA Upgrade Distribution (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Number of types |
| 1 | u8 |  | -/- | -/- | Fixed value (reserved byte 00) |
| 1 | u8 | Type | -/- | -/- | Type 1 00—mcu firmware |
| 1 | U8 | PathLen | -/- | -/- | Path 1 Length |
| n | N*u8 | Path | -/- | -/- | Path |
| 2 | U16 | PackageSize |  |  | Package size is fixed at 0000 |
| ... | ... | ... | ... | ... | ... |
| 1 | u8 | Type | -/- | -/- | Type n 00—mcu firmware |
| 1 | U8 | PathLen | -/- | -/- | Path n Length |
| n | N*u8 | Path | -/- | -/- | Path |
| 2 | U16 | PackageSize |  |  | Package size is fixed at 0000 |
| ... | ... | ... | ... | ... | ... |

Note: Remote updates consume data and are affected by the network. The device's SIM card has 30M per month, so it cannot be updated too many times, otherwise it will cause insufficient data.

Example:

The firmware download address is (requires server support for HTTP download):http://tools.aiday.com.cn/File/MCU/W200PG/W200PG_E42.BWGHOL25.bin

After the update is successful, the device will restart and reconnect to the server, and report the version number change

BDBDBDBDA901000041687474703A2F2F746F6F6C732E61696461792E636F6D2E636E2F46696C652F4D43552F5732303050472F5732303050475F4534322E425747484F4C32352E62696E00000A

BDBDBDBD:Header

A9:Message ID

01:TypeCnt Number of types 01--Indicates1type

00:Reserved bytes, fixed value

00:Type 00--Indicates MCU firmware upgrade

41:Path Download path length Convert to decimal65-->Download path length is65

687474703A2F2F746F6F6C732E61696461792E636F6D2E636E2F46696C652F4D43552F5732303050472F5732303050475F4534322E425747484F4C32352E62696E

File download path Convert to string-->http://tools.aiday.com.cn/File/MCU/W200PG/W200PG_E42.BWGHOL25.bin

0000:PackageSize Currently fixed at0000

0A:checksum

5.1.15 Weather Alert (MSGID=0XCB)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCB | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Count | -/- | -/- | Total number of weather alert groups |
| 1 | U8 | Type | -/- | -/- | Weather alert type, see below for specific types |
| 1 | U8 | Status | -/- | -/- | Status, see below for specific statuses |
| … | … | …… |  |  | … |

Weather Type - Status Table:

| Type | Description | Status | notes - Click the image to enlarge and view |
| --- | --- | --- | --- |
| 1 | Typhoon Warning | 0 | Cancel Alarm |
|  |  | 1 - 8 |  |
| 2 | Heavy Rain Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |
| 3 | Thunderstorm Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 4 | Flooding Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 5 | Landslip Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 6 | Strong Monsoon Signal | 0 | Cancel Alarm |
|  |  | 1 |  |
| 7 | Frost Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 8 | Fire Danger Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
| 9 | Temperature Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
| 10 | Tsunami warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 11 | Heat index | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |

Note: After the issuance, the device vibrates to display weather warning images. Multiple sets of weather warnings can be issued simultaneously. Each set of weather warnings must be different. The same type of warning cannot appear in one instruction, otherwise it will be invalid.
Example:

10Groups of weather warnings issued: BDBDBDBD CB 0A 0101 0201 0301 0401 0501 0601 0701 0801 0901 0A01 F5

BDBDBDBD:Header

CB:Message ID

0A:Count Number of weather groups Convert to decimal10-->Issue10groups of weather

0101:Type&Status Weather type-status 01--Gale warning 01--1Alert signal

0201:Type&Status Weather type-status 02--Rainstorm warning 01--Yellow rainstorm warning signal

0301:Type&Status Weather type-status 03--Thunderstorm warning 01--Thunderstorm warning

0401:Type&Status Weather type-status 04--Flooding warning 01--Special report on flooding in the northern New Territories

0501:Type&Status Weather Type-Status 05--Landslip Warning 01--Landslip Warning

0601:Type&Status Weather Type-Status 06--Strong Monsoon Warning 01--Strong Monsoon Signal

0701:Type&Status Weather Type-Status 07--Frost Warning 01--Frost Signal

0801:Type&Status Weather Type-Status 08--Fire Danger Warning 01--Yellow Fire Danger Warning

0901:Type&Status Weather Type-Status 09--Temperature Warning 01--Cold Weather Warning

0A01:Type&Status Weather Type-Status 10--Tsunami Warning 01--Tsunami Warning

F5:checksum校验和

5.1.16 Restore Factory Settings (MSGID=0X7B)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x7B | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | reset | -/- | -/- | 1 Restore factory settings |

Example:

Restore factory settings, the device restarts after receiving the command:BDBDBDBD7B018F

BDBDBDBD:Header

7B:Message ID

01:reset 01--Restore factory settings

8F:checksum

5.1.17 Special firmware support - sending SOS prompt message (MSGID=0X80)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x80 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Content length |
| N | N*U8 | Content | -/- |  | Content - Unicode encoding, up to 50 characters |

Note: This command is supported by special firmware, not available in the general version. When the device triggers SOS, it will enter the SOS interface. This command can control the displayed text content.
Applicable scenario: When the user encounters an emergency and triggers SOS, the content displayed on the SOS interface can be used to take corresponding actions based on the SOS interface prompt information.

Example:

SOSInterface prompt text: SOS alarm has been sent BDBDBDBD801053004F005300A562668BF25DD153FA51DD

BDBDBDBD:Header

80:Message ID

10:length Content length Convert to decimal16-->Content length is16bytes

53004F005300A562668BF25DD153FA51:Every2bytes are converted to big-endian --> Convert to Unicode characters --> Final: SOS alarm has been issued

DD:checksum

5.1.18 Special firmware support - Clear time for sending text messages (MSGID=0X2A)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x2A | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | 01 Message scheduled cleanup |
| 1 | U8 | Len |  |  | Length of the following message |
| 1 | U8 | hours | -/- |  | Message cleanup time (unit: hour) |

Note: This command is supported by special firmware, and the general version does not have it
Example:

Note::If the message cleanup time is 00, the received text message will be cleaned up immediately after receiving the instruction BDBDBDBD2A010102DD

BDBDBDBD:Header

2A:Message ID

01:Type 01--Timed cleanup of text messages

01:The message length after Len is converted to decimal 01-->The message length after is 1 byte (excluding checksum)

02:hours converted to decimal 02-->The text message cleanup time is 2 hours

DD:checksum

5.1.19 UWB alarm distance issued (0x7900)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x79 | See below See below definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type 0—UWB ranging alarm distance |
| 1 | U8 | distance | -/- | -/- | UWB ranging alarm distance (unit: meter; value range 0-20), an alarm will be triggered if it is less than or equal to this distance |

Example:

The alarm distance is 1 meter. If the ranging distance between the watch and the UWB beacon is less than 1 meter, the watch will vibrate and alarm BDBDBDBD790001C3

BDBDBDBD:Header

79:Message ID

00:Type 00--UWB ranging alarm distance

01:distance The distance of UWB ranging alarm is converted to decimal 01-->The distance of UWB ranging alarm is 1 meter

C3:checksum

5.1.20 UWB Ranging Alarm Configuration (0x7901)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x79 | See below See below definition | CK_sum |

Payload contents

| Format | Name | Unit | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Type is 0x01—-UWB beacon ranging alarm configuration |
| U8 | UWB beacon status | 1 | 00—-No alarm at all. After the command is issued, all UWB beacons will not trigger the watch UWB ranging alarm. In this state, type 01 and 02 commands are invalid. 01—Add UWB alarm beacon. 02—Delete UWB alarm beacon. 03—Set UWB alarm beacon (all covered). FF—-All alarms. After the command is issued, all beacons will trigger UWB ranging alarm. In this state, type 01 and 02 commands are invalid. |
| U8 | UWB beacon number | 1 | Number of UWB beacons |
| U16 | UWB beacon 0 mac | 4 | ID of alarm UWB beacon 0 |
| U16 | UWB beacon 1 mac | 4 | ID of alarm UWB beacon 1 |

Example:

Send all UWB beacons without triggering watch ranging alarm: BDBDBDBD790100DD

BDBDBDBD: Header

79 Message ID

01 Type=01 UWB ranging alarm beacon configuration

00 UWB beacon type=00, all UWB beacons do not trigger watch UWB ranging alarm

DD checksum

Send all UWB beacons to trigger watch ranging alarm: BDBDBDBD7901FFDD

BDBDBDBD: Header

79 Message ID

01 Type=01 UWB beacon ranging alarm configuration

FF UWB beacon type=FF, all UWB beacons trigger watch UWB ranging alarm

DD checksum

Set UWB alarm beacon: The device will clear the previous alarm beacon, and the UWB beacon in this message shall prevail to trigger the UWB ranging alarm.

BDBDBDBD7901030390000002E70500001F060000DD

BDBDBDBD : Header

79 Message ID

01 Type=01 UWB ranging alarm beacon configuration

03 UWB beacon type=03 Set UWB alarm beacon

03 The number of UWB beacons is3

90000002 UWB beacon0'sid Convert to big endian 02000090 UWB idis 02000090

E7050000 UWB beacon1'sid Convert to big endian000005E7 UWB idis000005E7

1F060000 UWB beacon2'sid Convert to big endian 0000061F UWB idis 0000061F

DD checksum

Add UWB alarm beacon: It does not affect the original alarm beacon. After the downlink, the3UWB beacon watches in the message can trigger the UWB ranging alarm.

BDBDBDBD7901010390000002E70500001F060000DD

BDBDBDBD : Header

79 Message ID

01 Type=01 UWB ranging alarm beacon configuration

01 UWB beacon type=01 Add UWB alarm beacon

03 The number of UWB beacons is3

90000002 UWB beacon0'sid Convert to big endian 02000090 UWB idis 02000090

E7050000 UWB beacon1'sid convert to big endian 000005E7 UWB idis000005E7

1F060000 UWB beacon2'sid Convert to big endian 0000061F UWB idis 0000061F

DD checksum

Delete UWB alarm beacon: In the message,3UWB beacon watches do not trigger UWB ranging alarm

BDBDBDBD7901020390000002E70500001F060000DD

BDBDBDBD : Header

79 Message ID

01 Type=01 UWB ranging alarm beacon configuration

02 UWB beacon type=02 Delete UWB alarm beacon

03 The number of UWB beacons is3

90000002 UWB beacon0'sid Convert to big endian 02000090 UWB idis 02000090

E7050000 UWB beacon1'sid convert to big endian 000005E7 UWB idis000005E7

1F060000 UWB beacon2'sid Convert to big endian 0000061F UWB idis 0000061F

DD checksum