Universal Platform API Interface Push Protocol

Directory

1. Q&A on Push Docking	4

1.1 Docking Instructions - Important	4

1.2 Frequently Asked Questions	5

2. Push data parsing and docking	7

2.1 Health data	7

2.1.1 Health data push collection (type=100)	7

2.1.2 Step counting - If only a single data report is available, use this (type=4)	8

2.1.3 Heart Rate - Use this if only a single data point is reported (type=6)	9

2.1.4 Temperature (body temperature) - if only a single data is reported, use this (type=12)	10

2.1.5 Dual temperature (wrist temperature/body temperature) - if only a single data is reported, use this (type=14)	11

2.1.6 Blood sugar - Special equipment usage (type=10)	11

2.1.7 Blood pressure - If only single data is reported, use this (type=8)	12

2.1.8 Blood Oxygen - Use this if only a single data is reported (type=31)	13

2.1.9 Sleep (type=58)	13

2.1.10 UV ultraviolet (specific device only) (type=185)	14

2.2 Alarm Data	15

2.2.1 Low Battery Alarm (type=18)	15

2.2.2 SOS Alarm (type=19)	15

2.2.3 SOS Cancellation (type=56)	16

2.2.4 Sedentary Stay Alarm (type=36)	16

2.2.5 Fall Alarm (type=110)	17

2.2.6 Shutdown Alarm (General) (type=20)	17

2.2.7 Charging Shutdown Alarm (Firmware version function is differentiated) (type=154)	18

2.2.8 Low Battery Shutdown Alarm (Firmware version function is differentiated) (type=155)	19

2.2.9 Active Shutdown Alarm (Firmware version function is differentiated) (type=156)	19

2.2.10 Bracelet watch wearing and removal alarm (type=57&type=21)	20

2.2.11 UWB ranging alarm (type=159)	21

2.2.12 Device-side health threshold alarm (type=200)	22

2.3 Device status data	23

2.3.1 Battery level (type=30)	23

2.3.2 Charging status (type=66)	24

2.4 Location data	25

2.4.1 GPS positioning (type=16)	25

2.4.2 Wifi positioning (type=5)	26

2.4.3 Bluetooth BLE (Indoor Positioning Data) (type=59)	26

2.4.4 Base station (LBS) positioning data (type=3)	28

2.4.5 UWB positioning (type=160)	29

2.5 Special Push Data - Special Device Support	30

2.5.1 Tamper-proof wristband (wearing/removing, unlocking, strap damage, etc. alarm - unique to tamper-proof wristband)	30

2.5.2 Sleep Entry Sleep Exit Park Entry and Exit No Signal (Specific Device Only) (type=51)	31

2.5.3 Check-in (specific to certain devices) (type=24)	32

2.5.4 Check-out (specific to certain devices) (type=25)	32

2.6 Platform Function Push	33

2.6.1 Fence Alarm Push (type=1)	33

2.7 Downlink feedback data	34

2.7.1 Text message feedback (type=180)	34

3. Call platform interface to downlink instructions	36

3.1 NB/4G Device Command Downlink Description	36

3.1.1 Instruction List - 4G Device Support	37

3.1.2 Instruction List - NBIOT Device Support	45

3.2 Security Check Code Acquisition	52

3.3 Attachment: Weather warning image corresponding table	55

The universal platform supports the synchronous push of device-reported data within the platform to third-party servers through a data push mechanism.
This document will introduce the relevant data parsing exception handling schemes for platform data push data, helping third-party service developers quickly complete docking and deployment.

Please read this agreement carefully. In general, you can complete the docking without technical support. If you need it, we also provide paid technical support.

1. Q&A on Push Docking

1.1 Docking Instructions - Important

(1) Fill in the push form for relevant personnel. The push form document can be downloaded by clicking the above document or contact relevant personnel to obtain it.
(2) After adding the push, the universal platform will push the data to the corresponding push address in POST mode.

Refer to the example of simulating push using Postman

(3) The time format of the [BTUtcTime] field in the push data is divided into two types: 2019/8/2 0:15:35 or 2019-08-02 00:01:05. Please pay attention when parsing; the parsing time is 24-hour format.

(4) The push address server immediately replies after receiving the push data (the reply format and content are arbitrary). Do not reply after processing. The timeout time is 3 seconds if there is no reply. If it exceeds, the connection will be disconnected, and failure to reply will cause delay in subsequent data.

Reply example - such as: {"code":0,"msg":"success","data":null}

(5) HTTPS address, TSL protocol supports TLS1.1. If the push fails due to the TSL protocol being too high, it is recommended that the device directly connect to its own server instead of using push, or be compatible with the TSL protocol.

1.2 Frequently Asked Questions

1. What protocol and method are used to obtain the pushed data?

Using http or https protocol, with the post method

Push the data into formdata

The data is pushed in string format without special processing. You can split the string content yourself according to the specified protocol.

2. How to troubleshoot if no data is pushed?

1. The customer should first check whether the server address provided to Oufu is incorrect.

2. Confirm whether the device has data on the general platform.

3. Check the interface documentation protocol to see if there are any errors in data parsing.

The recipient can use Postman to simulate the data sample submitted in our documentation for testing, and then receive the data we send.

3. Why is the time transmitted in the data 8 hours less?

The transmitted data is UTC time, and some are the current time. Refer to the relevant definitions in 2. Push Data Parsing and Docking for parsing.

4. Why is there an offset in the latitude and longitude position of the transmitted latitude and longitude data?

type=16, the satellite positioning data transmits the original coordinate system (WGS84 coordinate system)type=5, the wifi positioning data transmits the Gaode coordinate system (GCJ-02 coordinate system)

The customer can correct the latitude and longitude according to the map of his own platform.

5. Can the push push platform functions?

The API push only pushes the data reported by the device, not the platform function.

Currently, platform functions cannot be pushed, including: platform health threshold alarm, latitude and longitude corresponding to Bluetooth beacon

6. How to parse Bluetooth location data, such as this instance Bluetooth data: 4327@A358@B3@1573682453?

Major value:4327Hexadecimal code-->Little endian mode:2743-->Decimal code:10051

Minor value: A358 hexadecimal code --> Little-endian mode:58A3 --> Decimal code:22691

RSSI value: B3 hexadecimal code --> int type to decimal: -77

Timestamp:1573682453--> You can directly copy and convert the specific time by Baidu search timestamp tool;

2. Push data parsing and docking

Push data is not used by every product. Please parse it according to the specific product configuration. This document only provides parsing instructions for the push types currently supported by the platform.
The actual pushed data shall prevail. Refer to the device communication protocol and use the general platform report for comparison.

2.1 Health data

2.1.1 Health data push collection (type=100)

| Name | Type | Description |
| --- | --- | --- |
| IMEI | String | The unique identifier IMEI number of the device |
| BTUtcTime | String | The data push time is in UTC time, Time format: yyyy-MM-dd HH:mm:ss |
| bodyTemperature | String | Body temperature |
| wristTemperature | String | Wrist temperature |
| heartbeat | String | Heart rate |
| diastolic | String | Diastolic blood pressure |
| shrink | String | Systolic blood pressure |
| steps | String | Pedometer |
| BloodOxygen | String | Blood oxygen (available if the device supports reporting) |
| type | String | 100 indicates a collection of health data |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot(for example, not a real website)

Push data example:

Description: Health data collection: body temperature (wrist temperature, body temperature) heart rate, blood pressure (diastolic pressure, systolic pressure), blood oxygen, step counting,

Applicable to the general version 0x32 type message report push,

Note: The actual reported data shall prevail. If any of the pushed data is missing, such as blood oxygen,

Indicates that the device has not reported this type of health data, or the collection is not complete

(1) All data is reported---example, subject to the actual push data

IMEI=868488079852388&BTUtcTime=2024-08-12 5:54:20&steps=451&

type=100&heartbeat=92&bodyTemperature=36.7&wristTemperature=33.9

&diastolic=81&shrink=113&BloodOxygen=97

(2) No blood oxygen report --- Example, subject to actual push data

IMEI=868488079852388&BTUtcTime=2024-08-12 5:54:20&steps=451&

type=100&heartbeat=92&bodyTemperature=36.7&wristTemperature=33.9

&diastolic=81&shrink=113

2.1.2 Step counting - If only a single data report is available, use this (type=4)

| Name | Type | Description |
| --- | --- | --- |
| steps | String | Number of steps |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss or yyyy/MM/dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 4 represents step counting data |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

Step counting push data, the bracelet will clear at 0 o'clock or restart every day, pushing the data of the device's power-on on the same day;

If you want to calculate the data of the day, please compare it with the last data. If it is smaller than the last data, it is determined that the device is restarted and this data should be accumulated.

steps=0&BTUtcTime=2018/05/17 10:02:40&IMEI=863137002297055&type=4

2.1.3 Heart Rate - Use this if only a single data point is reported (type=6)

| Name | Type | Description |
| --- | --- | --- |
| roll | String | Flip number [can be ignored] - for special equipment, this field is not available for general equipment |
| heartbeat | String |  |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss or yyyy/MM/dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 6 is heart rate data, 11 is flip data |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

heartbeat=98&BTUtcTime=2018/05/17 10:02:40&IMEI=863137002297055&type=6

2.1.4 Temperature (body temperature) - if only a single data is reported, use this (type=12)

| Name | Type | Description |
| --- | --- | --- |
| bodyTemperature | String | Temperature |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss or yyyy/MM/dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 12 represents temperature (body temperature) |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

bodyTemperature=36.3&BTUtcTime=2018/05/17 10:02:40&IMEI=863137002297055&type=12

2.1.5 Dual temperature (wrist temperature/body temperature) - if only a single data is reported, use this (type=14)

| Name | Type | Description |
| --- | --- | --- |
| bodyTemperature | String | Body temperature |
| wristTemperature | String | Wrist temperature |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss or yyyy/MM/dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 14 represents temperature (wrist temperature/body temperature) |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

bodyTemperature=36.3&wristTemperature=32.3&BTUtcTime=2020/3/31 9:16:49&IMEI=869006030054362&type=14

2.1.6 Blood sugar - Special equipment usage (type=10)

| Name | Type | Description |
| --- | --- | --- |
| bloodSugar | String | Blood sugar level |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss or yyyy/MM/dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 10 represents blood sugar |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

bloodSugar=6&BTUtcTime=2018/05/17 10:02:40&IMEI=863137002297055&type=10

2.1.7 Blood pressure - If only single data is reported, use this (type=8)

| Name | Type | Description |
| --- | --- | --- |
| diastolic | String | Diastolic blood pressure data |
| shrink | String | Systolic blood pressure data |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss or yyyy/MM/dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 8 represents blood pressure |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

diastolic=73&shrink=115&BTUtcTime=2021/11/16 16:13:46&IMEI=863659045668364&type=8

2.1.8 Blood Oxygen - Use this if only a single data is reported (type=31)

| Name | Type | Description |
| --- | --- | --- |
| BloodOxygen | String | Blood oxygen data |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss or yyyy/MM/dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 31 represents blood oxygen |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

BloodOxygen=95&BTUtcTime=2021-11-16 16:13:46&IMEI=863659045668364&type=31

2.1.9 Sleep (type=58)

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss or yyyy/MM/dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| startTime | String | Start time: yyyy/MM/dd HH:mm:ss |
| endTime | String | End time: yyyy/MM/dd HH:mm:ss |
| sleeptype | String | sleeptype=1; Deep sleep, sleeptype=2; Light sleep, sleeptype=3; Wake-up time |
| minute | String | Sleep duration |
| type | String | type=58 represents sleep data |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

BTUtcTime=2020/4/30 10:59:38&IMEI=863084040001521&startTime=2020/4/30 18:57:30&endTime=2020/4/30 18:59:30&sleeptype=2&minute=2&type=58

2.1.10 UV ultraviolet (specific device only) (type=185)

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 185 represents UV ultraviolet |
| UV | String | UV ultraviolet data |

Example:

Push address example:http://bone-developer-edition.ap5.force.com/services/apexrest/iot(used for example, not a real website)

Push data example:

To be updated

2.2 Alarm Data

2.2.1 Low Battery Alarm (type=18)

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 18 represents a low battery alarm |
| AlertInfo | String | Device alarm information |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

BTUtcTime =2018-12-17 10:02:40&IMEI=863137002297055&type=18&AlertInfo=hello

2.2.2 SOS Alarm (type=19)

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 19 represents an SOS alarm |
| AlertInfo | String | Device alarm information |

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

BTUtcTime =2018-12-17 10:02:40&IMEI=863137002297055&type=19&AlertInfo=hello

Note: When the received type=56  AlertInfo=01, it is the SOS cancellation alarm, see the next section

2.2.3 SOS Cancellation (type=56)

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 56 represents SOS cancellation |
| AlertInfo | String | Device alarm information: 01 |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

BTUtcTime =2018-12-17 10:02:40&IMEI=863137002297055&type=56&AlertInfo=01

2.2.4 Sedentary Stay Alarm (type=36)

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 36 represents a sedentary stay alarm |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

BTUtcTime =2018-12-17 10:02:40&IMEI=863137002297055&type=36

2.2.5 Fall Alarm (type=110)

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 110 represents a fall alarm |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

BTUtcTime =2018-12-17 10:02:40&IMEI=863137002297055&type=110

2.2.6 Shutdown Alarm (General) (type=20)

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 20 represents a shutdown alarm |
| AlertInfo | String | Device alarm information |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

BTUtcTime =2018-12-17 10:02:40&IMEI=863137002297055&type=20&AlertInfo=hello

2.2.7 Charging Shutdown Alarm (Firmware version function is differentiated) (type=154)

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 154 represents a charging shutdown alarm |
| AlertInfo | String | Device alarm information |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

BTUtcTime =2018-12-17 10:02:40&IMEI=863137002297055&type=154&AlertInfo=hello

2.2.8 Low Battery Shutdown Alarm (Firmware version function is differentiated) (type=155)

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 155 represents a low battery shutdown alarm |
| AlertInfo | String | Device alarm information |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

BTUtcTime =2018-12-17 10:02:40&IMEI=863137002297055&type=155&AlertInfo=hello

2.2.9 Active Shutdown Alarm (Firmware version function is differentiated) (type=156)

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 156 represents a proactive shutdown alarm |
| AlertInfo | String | Device alarm information |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

BTUtcTime =2018-12-17 10:02:40&IMEI=863137002297055&type=156&AlertInfo=hello

2.2.10 Bracelet watch wearing and removal alarm (type=57&type=21)

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 57 represents a wearing alarm |
| AlertInfo | String | Device alarm information |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

BTUtcTime =2018-05-17 10:02:40&IMEI=863137002297055&type=57&AlertInfo=hello

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 21 represents a removal alarm |
| AlertInfo | String | Device alarm information |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example

BTUtcTime =2018-05-17 10:02:40&IMEI=863137002297055&type=21&AlertInfo=hello

2.2.11 UWB ranging alarm (type=159)

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 159 represents a UWB ranging alarm |
| AlertInfo | String | Device alarm information |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

IMEI=864814079995634&BTUtcTime=2024-10-15 09:16:53&AlertInfo=159&type=159

2.2.12 Device-side health threshold alarm (type=200)

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 200 represents a health threshold alarm |
| AlertInfo | String | Device alarm information: 200 |
| hmsg | String | Health data type - 0/1 - specific value Health data type: HR: Heart rate, T: Temperature, Diastolic: Diastolic blood pressure, Shrink: Systolic blood pressure, BloodOxygen: Blood oxygen; 0: Less than the health threshold set by the device 1: Greater than the health threshold set by the device Specific value: Health value reported by the device |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

Note: The device firmware must support the function of setting health thresholds. Only when the device-side downlink command has set the health threshold,

The health sampling value is not within the set range, then this push will occur.

IMEI=869020064827429&BTUtcTime=2024-12-27 06:19:39&AlertInfo=200&hmsg=Shrink-1-123&type=200

2.3 Device status data

2.3.1 Battery level (type=30)

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss or yyyy/MM/dd HH:mm:ss |
| Format: 2018-12-19 04:21:46 |  |  |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 30 represents battery level data |
| Signal (the actual push field is singal) | String | Signal value |
| battery | String | Percentage of battery level |
| steps | String | Pedometer |
| adddatetime | String | Timestamp (seconds) - can be left unparsed, choose BTUtcTime for parsing |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

IMEI=863137002297055&BTUtcTime =2018-05-17 17:59:13& signal=123& battery=20&type=30

Note: Example of push data in special circumstances

If the data pushes a signal of0and adddatetime (this push is platform-generated, not device-reported),

If none of the others are present, it indicates that the device push is disconnected, the device is offline, and you can check whether the device is turned off or in sleep mode.

Example: singal=0&adddatetime=1723544014&IMEI=860761074623873&BTUtcTime=2024-08-1310:13:34&type=30

2.3.2 Charging status (type=66)

| Name | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| Status | String | Charging status: 0 - Indicates charging started, 1 - Indicates charging ended, 2 - Indicates fully charged |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 66 indicates charging status |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

IMEI=866380079121777&BTUtcTime=2025/1/14 22:48:36&Status=1&type=66

2.4 Location data

2.4.1 GPS positioning (type=16)

| - Name | - Type | - Description |
| --- | --- | --- |
| - BTUtcTime | - String | - When course=0, the data push is in China Beijing time. Time format: yyyy-MM-dd HH:mm:ss |
| - IMEI | - String | - The unique identifier IMEI number of the device |
| - type | - String | - 16 represents GPS data |
| - latStr | - String | - GPS latitude (WGS84 coordinate system) |
| - lngStr | - String | - GPS longitude (WGS84 coordinate system) |
| - speedStr | - String | - The device transmits 0, so it is fixed at 0 |
| - dataContext | - String | - The device has not transmitted it yet, so it is empty |
| - distance | - String | - The device transmits 0, so it is fixed at 0 |
| - course | - String | - The default positioning type is 0—GPS reported latitude and longitude |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

Note: course=0the Beijing time for data push,

The latitude and longitude coordinates are in the WGS84 coordinate system. Pay attention to map coordinate system conversion.

IMEI=863137002297055&BTUtcTime=2018-05-17 17:59:13&latStr=31.210463&lngStr=121.606855&speedStr=0&course=0&dataContext=&distance=0&type=16

2.4.2 Wifi positioning (type=5)

| Name | Type | Description |
| --- | --- | --- |
| IMEI | String | The unique identifier IMEI number of the device |
| timeStr | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| Latitude | String | Gaode coordinate system latitude (GCJ-02 coordinate system) |
| Longitude | String | Gaode coordinate system longitude (GCJ-02 coordinate system) |
| type | String | 5 represents the wifi data type value |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

Note: The time field is timeStr, which is different from other push type fields. Pay attention when parsing.

The latitude and longitude coordinates are GCJ-02 coordinate system. Pay attention to the map coordinate system conversion.

IMEI=869029030185937&timeStr=2019-08-12 03:28:24&type=5&Latitude=28.7110078&Longitude=115.8204161

2.4.3 Bluetooth BLE (Indoor Positioning Data) (type=59)

| Field | Type | Description |
| --- | --- | --- |
| IMEI | String | The unique identifier IMEI number of the device |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| BTInfo | String | There are two types: with beacon power reporting and without beacon power reporting Without beacon power reporting: major@minor@rssi@timestamp (utc time); With beacon power reporting: major@minor@rssi@beacon power (percentage)@timestamp (utc time) |
| type | String | 59 represents the Bluetooth data type value |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot(for example, not a real website)

Push data example:

Without beacon power: IMEI=869029030185937&BTUtcTime=2018-12-17 10:02:40&type=59&BTInfo=4327@9757@C9@1565682098|4327@7554@C0@1565682098|4327@7354@BF@1565682098|4327@8857@BC@1565682098|

Taking Bluetooth beacon encoding 00081005122691 as an example,

Where 10051 is the major manufacturer code, 22691 is the minor device identification code, and the rssi signal value is negative. The specific value is its complement.

The device defaults to scanning only Aufu Bluetooth beacons: UUID (AB8190D5-D11E-4941-ACC4-42F30510B408)

If you need to scan beacons with other specified UUID or MAC address patterns, please contact us.

Example of parsing BTInfo: 4327@9757@C9@1565682098:4327->2743->10051(major),9757->5797->22423(minor),C9->-55,1573682453->2019-08-13 07:41:38

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot(for example, not a real website)

Push data example:

With beacon power: IMEI=869029030185937&BTUtcTime=2018-12-17 10:02:40&type=59&BTInfo=4327@9757@C9@5A@1565682098|4327@7554@C0@5A@1565682098|4327@7354@BF@5A@1565682098|4327@8857@BC@5A@1565682098|

Taking Bluetooth beacon encoding 00081005122691 as an example,

Where 10051 is the major manufacturer code, 22691 is the minor device identification code, and the rssi signal value is negative. The specific value is its complement.

The device defaults to scanning only Aufu Bluetooth beacons: UUID (AB8190D5-D11E-4941-ACC4-42F30510B408)

If you need to scan beacons with other specified UUID or MAC address patterns, please contact us.

Example of parsing BTInfo: 4327@9757@C9@5A@1565682098:4327->2743->10051(major),9757->5797->22423(minor),C9->-55,5A-->90-->90%,1573682453->2019-08-13 07:41:38

2.4.4 Base station (LBS) positioning data (type=3)

| Field | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 3 represents the base station location data |
| latStr | String | GPS latitude (WGS84 coordinate system) |
| lngStr | String | GPS longitude (WGS84 coordinate system) |
| speedStr | String | The device transmits 0, so it is fixed at 0 |
| dataContext | String | The device has not transmitted it yet, so it is empty |
| distance | String | The device transmits 0, so it is fixed at 0 |
| course | String | The default positioning type is 99—representing the latitude and longitude parsed by base station positioning (WGS84 coordinate system) |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

The latitude and longitude coordinates are in the WGS84 coordinate system. Pay attention to the map coordinate system conversion.

IMEI=864814079433941&BTUtcTime=2024-11-05 03:44:32&latStr=44.4013042&lngStr=84.7589754&speedStr=0&course=99&dataContext=&distance=0&type=3

NBIOT device base station LBS positioning mode is not supported temporarily

2.4.5 UWB positioning (type=160)

| Field | Type | Description |
| --- | --- | --- |
| IMEI | String | The unique identifier IMEI number of the device |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| UWBDes | String | Report UWB positioning information, the timestamp is utc time; uwb beacon id@distance (cm)@time (utc time) |
| type | String | 160 represents the UWB positioning data type value, currently the push only supports minute level |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

IMEI=864536079578959&BTUtcTime=2024-12-21 03:45:41&UWBDes=00000538@229@2024-12-21 03:45:41

|03000039@241@2024-12-21 03:45:41|00000632@432@2024-12-21 03:45:41|0000052B@486@2024-12-21 03:45:41|&type=160

UWBDes parsing example:

UWB beacon ID:00000538; Distance:229cm; Reporting time:2024-12-21 03:45:41

UWB beacon ID:03000039; Distance:241cm; Reporting time:2024-12-21 03:45:41

UWB beacon ID:00000632; Distance:432cm; Reporting time:2024-12-21 03:45:41

UWB beacon ID:0000052B; Distance:486cm; Reporting time:2024-12-21 03:45:41

2.5 Special Push Data - Special Device Support

2.5.1 Tamper-proof wristband (wearing/removing, unlocking, strap damage, etc. alarm - unique to tamper-proof wristband)

| Field | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 57 represents wearing, 21 represents removing, 38 represents lock opening, 39 represents strap damage |
| AlertInfo | String | 19 represents wearing, 65 represents removing, 140 represents lock opening, 141 represents strap damage |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

BTUtcTime=2018-12-17 10:02:40&IMEI=863137002297055&AlertInfo=19&type=57

2.5.2 Sleep Entry Sleep Exit Park Entry and Exit No Signal (Specific Device Only) (type=51)

| Field | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 51 represents sleep entry, 52 sleep exit, 57 park entry, 58 park exit, 91 no signal |
| AlertInfo | String | No Signal (only available for type=91) |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

BTUtcTime=2023-08-14 02:53:02&IMEI=862451050468703&type=51

2.5.3 Check-in (specific to certain devices) (type=24)

| Field | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss or yyyy/MM/dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 24 represents check-in |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

BTUtcTime=2018/05/17 10:02:40&IMEI=863137002297055&type=24

2.5.4 Check-out (specific to certain devices) (type=25)

| Field | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss or yyyy/MM/dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 25 represents check-out |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot (for example, not a real website)

Push data example:

BTUtcTime=2018/05/17 10:02:40&IMEI=863137002297055&type=25

2.6 Platform Function Push

2.6.1 Fence Alarm Push (type=1)

| Field | Type | Description |
| --- | --- | --- |
| BTUtcTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| type | String | 1 represents entering the fence, 2 represents exiting the fence |
| AlertInfo | String | Device alarm information |
| Fenceid | String | Fence ID |
| Fencename | String | Fence name |
| Status | String | Fence status entry in (inside the fence), out out (outside the fence) |

Example:

Push address example:http://bone-developer-edition.ap5.force.com/services/apexrest/iot(used for example, not a real website)

Push data example:

Note: The device needs to be bound to an account on the general platform, and this push will only be available if the device fence or account fence is set up.

IMEI=862451050468703&BTUtcTime=2023-08-14 02:53:02&AlertInfo=1&type=1&Fenceid=11@Fencename=Fence1@Status=entry

2.7 Downlink feedback data

2.7.1 Text message feedback (type=180)

| Field | Type | Description |
| --- | --- | --- |
| RecTime | String | The data push time is in UTC time, format yyyy-MM-dd HH:mm:ss |
| IMEI | String | The unique identifier IMEI number of the device |
| Status | String | Yes: Click Accept after receiving the text message, No: Click Reject after receiving the text message |
| SeriNo | String | Text message ID - Hexadecimal |
| type | String | 180: Text message status feedback |

Example:

Push address example: http://bone-developer-edition.ap5.force.com/services/apexrest/iot(for example, not a real website)

Push data example:

Note: This push from the device requires the device firmware to support accepting & rejecting text messages by clicking.

Text message ID issued directly from the general platform web or mobile terminal or by calling the NT11/NT13 interface. The ID is generated based on the timestamp (little-endian first, needs to be converted to big-endian). The IDs of consecutively issued text messages are all incrementing.

For text messages issued by calling the NT12 interface, the text message ID of this push feedback is the text message ID at the time of issuance (16 hexadecimal, no need to convert to big-endian).

IMEI=864814074715128&Status=Yes&SeriNo=73C94767&RecTime=2024-11-28 01:38:12&type=180

IMEI=864814074715128&Status=No&SeriNo=89C94767&RecTime=2024-11-28 01:38:34&type=180

3. Call platform interface to downlink instructions

3.1 NB/4G Device Command Downlink Description

The general platform supports pushing device calls to the platform API interface for downlink commands.
Call [Command/ SendNbCommand] to issue commands to modify the NB/4G device reporting frequency. You can directly call the interface on the platform to issue commands without logging in or other operations.

Call Notes - Important:

A large number of calls to the interface need to be explained in advance, and the device issuing the command must be online.

Interface limit1Up to20calls per minute,

The downlink command is2valid within hours (assuming the device has not communicated with the server but has issued a command, the command is retained for2hours, and will be cleared after expiration)

Interface address:http://www.aiday.com.cn:10502/api/Command/SendNbCommand
Call method: POST

Parameter structure description:

| Parameter | Required/Optional | Type | Position | Description |
| --- | --- | --- | --- | --- |
| CmdCode | Required | String | Body | Command code (see table below for explanation) |
| Params | Required | String | Body | Command parameters (see table below for explanation) |
| IMEI | Required | String | Body | Device IMEI number |
| CheckCode | Required | String | Body | Security check code |

3.1.1 Instruction List - 4G Device Support

| Instruction function | Instruction Code (CmdCode) | Instruction parameter example (Params) | Instruction parameter example description |
| --- | --- | --- | --- |
| Set timed positioning frequency (4g device) | NT01 | “1,0000,2302, 1234567,1,3” | 1: Fixed value, indicating on, 0000: Indicates the start time is 00:00, 2302: Indicates the end time is 23:02, 1234567: Fixed value, 1: Fixed value, 3: Indicates a positioning and reporting frequency of 3 minutes, Unit: minutes (Note: Hours must be converted to minutes when uploading) |
| Set positioning priority (4g device) | NT0D | “1,0,3-2-1” | 1: Fixed value, indicating enabled, 0: Fixed value, 3-2-1: Positioning priority Represents Bluetooth priority, then WiFi, then GPS positioning. Supports the following positioning types: 1: gps 2: wifi 3: Bluetooth beacon 6: UWB beacon (UWB requires device support for 4g+UWB, otherwise downlink is invalid) |
| Watch issued (4g Simplified Chinese version) | NT11 | “3¦test009” | 4G watch message issued; ”3¦”” is fixed, followed by the message content; test009: Message content |
| Watch issued (4g Simplified Chinese version) —Text message with message ID issued | NT12 | “3¦消息id¦test009” | 4G watch message issued; ”3¦“ is fixed, Followed by the message ID, followed by the message content after the message ID; Message ID: Decimal, range (0-4294967295), The ID cannot be repeated when multiple text messages are issued to the same device; test009: Message content |
| Watch issued (4g Traditional Chinese version) Issue text messages with message ID | NT60 | “5¦Message ID¦test009” | 4G watch message delivery; ”5¦“ is fixed, followed by the message ID, the message ID is followed by the message content; Message ID: Decimal, range (0-4294967295), The ID cannot be repeated when multiple text messages are sent from the same device; test009: message content |
| Watch delivery 2 (for the Traditional Chinese version) Switch the language interface to see if Traditional Chinese is supported | NT13 | “Message content” | UNICODE encoded text message delivery -W200PG, Hong Kong general version, -W300G, general version |
| Health sampling frequency delivery (device firmware must support) | NT18 | “2,0,0-2-0” | Instruction parameter [2,0,0-3-0”] 2, health 0,—long-term 0-3-0 First digit, 0: health Second digit 3: interval time (maximum 2 minutes for devices with blood oxygen) Third digit 0-minutes (integer)-hours (integer) |
| Device sedentary stay alarm trigger time | NT16 | “2” | 2: The trigger time is 2 minutes; Time range: 2-60 minutes, the rest are invalid |
| Device fall sensitivity issuance (referring to the degree to which the fall algorithm is satisfied) | NT39 | “0,1” | Instruction parameters: 0,1 0 indicates fall sensitivity 1 indicates sensitivity level low to medium (range 0-4: low - medium low - medium - medium high - high) |
| Device triggers fall alarm height issuance (referring to the height that satisfies triggering the fall alarm) | NT40 | "1,1" | Instruction parameters: 1,1 1 indicates the height at which the fall alarm is triggered 1 indicates that the specific level corresponds to a height of 1.0 meter (range 0 - 4: 0.5m - 1.0m - 1.5m - 2.0m - 2.5m ) |
| GPS always-on switch (The GPS interface is always on after it is turned on, It can speed up GPS positioning in general environments, but it will increase power consumption after it is turned on.) This function requires device firmware support | NT51 | "24,0" | Instruction parameters: 24,0 24 indicates the GPS internal always-on interface 0: Indicates that the GPS is always on, and the power consumption will increase 2: Indicates that the GPS is always off—the default is off, which does not affect positioning in normal environments |
| Weather warning issued (requires device firmware support) | NT24 | “3,1-0,3-1,2-2“ | See Section 3.3 for the image types corresponding to weather warnings 3: Indicates that the number of weather warnings issued is 3 1-0: 1 indicates the weather warning group issued: gale warning, 0 indicates the warning is canceled 3-1: 3 indicates the weather warning group issued: thunderstorm warning, 1 indicates the warning type: thunderstorm warning 2-2: 2 indicates the weather warning group issued: rainstorm warning, 2 indicates the warning type: red rainstorm warning signal Note: When issuing multiple weather warnings, there cannot be duplicate weather warning groups, such as: “3,2-1,3-1,2-2“, there are two rainstorm warnings in it, It is incorrect. A maximum of 11 different weather warning groups can be issued |

Example:

Method: POST

request: http://www.aiday.com.cn:10502/api/Command/SendNbCommand

Content-type:application/json

{

"CmdCode":"NT01",

"Params": "1,0000,2302,1234567,1,3",

"IMEI":"867726035705658",

"CheckCode":"xxxxxx"

//Note: xxxx needs to be obtained according to the push address request verification code, please refer to section 3.2 for details

}

Feedback on calling the interface:
(1) Example of successful call feedback:

{

"Content": "OK",

"Message": “指令已下发至平台【注:20241001起指令下发新增必填校验参数CheckCode,

该参数请通过接口http://www.aiday.com.cn:10502/api/Command/searchCheckCodeByUrl

方法体{'PushUrl':'您的推送地址'}获取或查看欧孚最新推送文档】”

"State": 0

}

(2) Example of failed call feedback

1.2024 - 10 - 01 After that, CheckCode is empty

{

"Content": "校验码验证失败，请检查后再试！", "State": 300

}

2. Call exceeds the number of times: on the premise that the issue exceeds 20 times per minute

{

"Content": "下发失败，超过每分钟20次最大请求次数请稍等一分钟再试！", "State": 500

}

3. Verification code error, fill in the verification code but verification failed

{

"Content": "校验码验证失败，请检查后再试！", "State": 300

}

4. The device has not reported data for 12 hours, the platform status is offline, there is no communication, the platform cannot issue downlink instructions, and the device cannot receive downlink instructions

{

“ Content ”: “下发失败，设备已离线请检查后再试!”

“State”: 0

}

Platform offline conditions:

- The device has not reported any data for 12 hours,

2. The device reports a shutdown alarm,

3. The device signal is 0 (indicating the device reported no signal data, which is transmitted and reported offline)

3.1.2 Instruction List - NBIOT Device Support

| Instruction function | Instruction Code (CmdCode) | Instruction Parameter Example (Params) | Instruction parameter example description |
| --- | --- | --- | --- |
| Set Timed Positioning Frequency (NB Device) | NB01 | “1,5,00:30,23:59” | 1: On 5: Time interval, ,00:30: Start time in hours and minutes (24-hour format) 23:59: End time in hours and minutes (24-hour format) , Hours must be converted to minutes when uploading) |
| Delete scheduled location frequency (supported by special firmware versions) | NB01 | "1" | Scheduled positioning frequency time |
| Set temperature alarm threshold (special firmware version support) | NB02 | "1,-2,37" | 1: On; -2: Minimum temperature; 37: Maximum temperature |
| Heart rate alarm threshold setting (special firmware version support) | NB03 | "1,26,30" | 1: On; 26: Minimum heart rate 30: Maximum heart rate |
| Heart rate detection cycle setting (supported by special firmware versions) | NB04 | “1,62” | 1: On; 62: Time interval (minutes) |
| Temperature detection cycle setting (supported by special firmware versions) | NB05 | “1, 65” | 1: On; 65: Time interval (minutes) |
| NB heartbeat (supported by special firmware versions) | NB06 | "1, 60"" | 1: On; 60: Time interval (minutes) |
| NB Blood Oxygen Blood Glucose Threshold Setting (Special Firmware Version Supported) | NB09 (corresponding to type 1), NB0A (corresponding to type 2) | “1,90,95,5” | 1: Type 1: Blood oxygen 2 blood glucose; 90: Minimum blood oxygen 95: Maximum blood oxygen |
| NB Family Number Setting (Special Firmware Version Supported) | NB0B | “小明,12345678900,小雪,12345678900” | 小明: Contact person, 12345678900: Number |
| NB Device Operation Mode (Special Firmware Version Supported) | NB0C | "1,1,1023,2345" | Type: 1 Indoor 2 Outdoor 3 Inpatient 4 Home; Enable time period 1 Enable 0 Disable |
| NB device function positioning | NB0D | “1,0,3-2-1” | 1: Fixed value, indicating on, 0: Fixed value, 3-2-1: Positioning priority Represents Bluetooth priority then wifi then GPS positioning, supports the following positioning types: 1: gps 2: wifi 3: Bluetooth beacon |
| Name setting (special firmware version support) | NB0E | "IMEI, Zhang San" | The former is IMEI followed by the name content |
| Message Settings (Special Firmware Version Supported) | NB0F | “timing,position,event.” | Adjustable order, not controlled |
| Watch issued (NB device general) | NB11 | “3¦test009” | NB watch message issued; ”3¦“ is fixed, followed by the message content |

Example:

Method: POST

request: http://www.aiday.com.cn:10502/api/Command/SendNbCommand

Content-type:application/json

{

"CmdCode":"NB01",

"Params": "1,5,00:30,23:59",

"IMEI":"867726035705658",

"CheckCode":"xxxxxx"

//Note: xxxx needs to be obtained according to the push address request verification code, please refer to section 3.2 for details

}

Feedback on calling the interface:
(1) Example of successful call feedback:

{

"Content": "OK",

"Message": “指令已下发至平台【注:20241001起指令下发新增必填校验参数CheckCode,

该参数请通过接口http://www.aiday.com.cn:10502/api/Command/searchCheckCodeByUrl

方法体{'PushUrl':'您的推送地址'}获取或查看欧孚最新推送文档】”

"State": 0

}

(2) Example of failed call feedback

1.2024 - 10 - 01 After that, CheckCode is empty

{

"Content": "校验码验证失败，请检查后再试！", "State": 300

}

2. Call exceeds the number of times: on the premise that the issue exceeds 20 times per minute

{

"Content": "下发失败，超过每分钟20次最大请求次数请稍等一分钟再试！", "State": 500

}

3. Verification code error, fill in the verification code but verification failed

{

"Content": "校验码验证失败，请检查后再试！", "State": 300

}

4. The device has not reported data for 12 hours, the platform status is offline, there is no communication, the platform cannot issue downlink instructions, and the device cannot receive downlink instructions

{

“ Content ”: “下发失败，设备已离线请检查后再试!”

“State”: 0

}

Platform offline conditions:

- The device has not reported any data for 12 hours,

- 2. The device reports a shutdown alarm,

- 3. The device signal is 0 (indicating the device reported no signal data, which is transmitted and reported offline)

3.2 Security Check Code Acquisition

Interface address:http://www.aiday.com.cn:10502/api/command/searchCheckCodeByUrl
Calling method: POST

Parameter structure description:

| Parameter | Required/Optional | Type | Position | Description |
| --- | --- | --- | --- | --- |
| PushUrl | Required | String | Body | Push address provided by the customer |
| CheckCode | None | String | Within the returned Info | Security check code |

Example:

Method: POST

request：http://www.aiday.com.cn:10502/api/command/searchCheckCodeByUrl

Content-type:application/json

{

"PushUrl":"http://106.54.41.177:8101/badge/data/api/push"

}

(1) Successful call feedback example:

{

"Info": {

"PushUrl": "http://106.54.41.177:8101/badge/data/api/push",

"CheckCode": "320020931c14f05fedddff967067092a"//Security check code

},

"Message": "查询成功！",

"State": 200

}

(2) Feedback on failed calls Example:

1. Query failed, no push address was entered

{

"Message": "查询失败，推送地址未录入!",

"State": 300

}

2. Query successful

{

"Message": "查询成功！",

"State": 200

}

3. It is not the http or https protocol, but an arbitrarily filled string

{

"Message": "查询失败，请录入有效的推送路径!",

"State": 600

}

4. Called more than the number of times

{

"Message": "查询失败，超过每分钟20次最大请求次数请稍等一分钟再试！",

"State": 500

}

3.3 Attachment: Weather warning image corresponding table

Weather type - status table:

| Type | Decription | Status | notes - Click the image to enlarge and view |
| --- | --- | --- | --- |
| 1 | Typhoon Warning | 0 | Cancel Alarm |
|  |  | 1 - 8 |  |
| 2 | Heavy Rain Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |
| 3 | Thunderstorm Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 4 | Flooding Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 5 | Landslip Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 6 | Strong Monsoon Signal | 0 | Cancel Alarm |
|  |  | 1 |  |
| 7 | Frost Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 8 | Fire Danger Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
| 9 | Temperature Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
| 10 | Tsunami Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 11 | Heat Index | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |