G618GU-4GCat.1-tcp communication protocol

Directory

1 Overview	3

2 Device Usage Instructions	3

2.1 Device Function and Usage Instructions	3

2.2 Device default reporting logic	4

2.3 Device downlink description	6

3 Protocol Data Packet Structure	8

3.1 Data Header	8

3.2 Message ID	8

3.3 Token Generation Mechanism	9

3.4 Payload	9

3.5 Checksum	10

4 Report messages message	12

4.1 Connection related report	12

4.1.1 LNK-LIN (MSGID=0xF0) Request connection (TCP dedicated)	12

4.1.2 Connection Reply (MSGID=0xF1) (TCP Only) - Important	13

4.1.3 New Heartbeat Packet Protocol (MSGID=0xF9)-Important	14

4.2 Alarm-related reporting	16

4.2.1 Alarm data upload-1 (MSGID=0x02)	16

4.2.2 Alarm Data Upload-2 (MSGID=0x21) (Supplement to 02)	19

4.3 Location-related reporting	24

4.3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)	24

4.3.2 Wi-Fi and base station information upload (MSGID=0xA4)	26

4.3.3 Bluetooth Positioning Information (BLE Location) (MsgId=0xD6)	31

4.3.3 UWB定位信息(UWB Location)（MsgId=0xD7）	34

4.4 Device information and status reporting	36

4.4.1 SIM card's ICCID upload (MSGID=0xF3)	36

4.4.2 Device charging status upload (MSGID=0xC3)	36

4.4.3 Status Parameter Reporting (MSGID=0xA9)	38

4.4.4 Device Status (MSGID=0xE9)	40

4.5 Downlink feedback related reporting	42

4.5.1 Downlink feedback (MSGID=0xC0)	42

5 Settings	43

5.1 Downlink	43

5.1.1 Set location reporting frequency (MSGID=0x17)	43

5.1.2 Setting - Positioning priority setting (0XCE01&0XCE02)	45

5.1.3 Settings - Switch settings (0XCE04-24)	49

5.1.4 IP&Domain Name Settings (0xC3) (TCP Dedicated)	54

5.1.5 Shutdown and restart (MSGID=0x77)	56

5.1.6 UWB Alarm Distance Issuance (0x7900)	57

1 Overview

This protocol is suitable for 4G CAT1 devices, currently supporting the G618GU product, mainly used with UB01 (UWB beacon) or G908GU-T. It uses a 32-bit data header for synchronization and terminal identification; uses a low-overhead verification algorithm for verification protection; and uses message identifiers to identify different messages.
It needs to be parsed according to the actual report. The document is currently being continuously maintained. If you find any errors or omissions, please give feedback in time. Thank you very much.
The protocol content is the 4g report content. If you need the Bluetooth broadcast data protocol, please consult the relevant docking personnel.

2 Device Usage Instructions

2.1 Device Function and Usage Instructions

General version:

(1) Power on:

Please fully charge before using for the first time. Charging automatically turns on the device. Charging light effect: red light. Fully charged light effect: green light is always on.

Manual power on in shutdown state: Connect the charging cable (there is a button on the charging cable), long press the charging cable button for 2 seconds, power on light effect: red and green lights flash alternately.

(2) Shutdown:

Charging: Automatic shutdown Manual shutdown: Shutdown: Long press the button on the charging cable for 10 seconds, keep pressing until the red light flashes 3 times, then goes off, and release immediately Low battery shutdown: Flashing red light and then goes off Issue shutdown command: The device flashes a red light and then goes off after receiving the platform's downlink command

(3) SOS function:

Trigger: Connect the charging cable, long press the button on the charging cable for 3 seconds, release after the red light is on for a long time, sos light effect: red light is on for a long time Cancel: Under the red light is on for a long time, long press the button on the charging cable for 3 seconds and the red light goes off to cancel the SOS alarm

(4) Device status:

Normal operation: Green light flashes once at intervals Low device power: Red light flashes at intervals

(5) Device sleep:

Trigger condition: The device does not move for 20 minutes and enters sleep mode, and does not report data

2.2 Device default reporting logic

General version:

(1) Connection related reporting

F0 request: The device is a short link, under normal server connection and network conditions,

It will be reported once at startup, and then each report will request a connection--a reply is required to connect to the server

F9: Power and signal reporting, a message will be reported along with location reporting -- requires server reply

(2) Location-related reporting

UWB /Satellite (GPS/BDS)/wifi/Bluetooth beacon: Default reporting frequency 10 minutes, default positioning priority: UWB>wifi>Satellite (GPS/BDS), UWB positioning first, switch to wifi positioning if UWB positioning is not available, switch to Satellite (GPS/BDS) if wifi positioning is not available

(3) Alarm-related reporting

SOS alarm (0x02): User actively triggers, see the previous section for triggering method

SOS cancellation (0x02): User actively triggers cancellation, see the previous section for triggering method

Shutdown alarm (0x21): Device actively shuts down or shuts down due to low battery, see the previous section for triggering method

Low battery alarm (0x02): Triggered when the device's current battery level is less than or equal to 0

Fall alarm (0x02): The device free falls from a certain height and meets the fall algorithm trigger

UWB ranging alarm (0x21): The device's default dangerous distance is 5m, and the alarm is reported once every 1 minute by default

(4) Device information and status reporting

Status parameter (0xA9): A record will be reported when the device is turned on

SIM card ICCID (0xF3): A record will be reported when the device is turned on

Device status (0xE9): A record will be reported when the device is turned on, and a record will be reported when the reporting frequency is changed

Charging status (0xC3): Report when charging starts, charging ends, and when fully charged---report when charging and turning on

The default is charging shutdown, and the charging status is not reported when charging is shutdown

(5) Downlink feedback

Downlink feedback (0xC0): Reported after the device receives the downlink command from the server,

Note: There is a case of packet reporting for device reporting, that is, a data packet contains multiple complete messages. Be careful not to miss it. The message is a complete message, and there will be no phenomenon of being interrupted in the middle and in the next data packet.

Eg:bdbdbdbdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9bdbdbdbdf9010000006400002800000019a9cf61ca

This data packet contains messages for (0xD6) Bluetooth positioning and (0xF9) battery signal.

(0xD6) Bluetooth positioning: bdbdbdbdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9

(0xF9) Battery signal: bdbdbdbdf9010000006400002800000019a9cf61ca

2.3 Device downlink description

General version: The downlink command needs to be continuously downlinked twice within 20 milliseconds to ensure success
Note: For short-link devices, the downlink command is received by the device when the device reports it, and it takes effect in the next reporting cycle after receiving it.

(1) Device positioning reporting frequency issuance (0x17):

The default reporting frequency is 10 minutes, and the maximum is 1 minute. After the device receives the downlink command, the device reports according to the time period and frequency of the issued command.

Report data, and report according to the default reporting frequency outside the time period, such as: 00:00-18:00 2-minute positioning report,

Then report according to the default 10-minute reporting frequency outside the time period

(2) Device positioning priority issuance (0xCE01):

Default positioning priority UWB>wifi>Satellite (GPS/BDS), positioning priority

The positioning priority is as follows: wifi>Satellite (GPS/BDS)>Bluetooth beacon. If wifi cannot be located, switch to Satellite (GPS/BDS). If gps cannot be located, switch to

Bluetooth beacon. When the positioning is successful, the next positioning priority will not be switched to generate positioning.

(3) Modify IP and port instruction delivery (0xC3):

The default general version points to the Smart Cloud Platform: 118.178.184.219:8825. If you need to change it, please consult relevant personnel or visit the official website.

(4) Bluetooth broadcast switch delivery (0xCE05):

The default is the on state. After it is turned off, the device does not perform Bluetooth broadcasting.

(5) Fall alarm switch (0xCE07):

The default fall alarm is on. After it is turned off, the device does not report fall alarms.

(6) Key shutdown switch instruction delivery (0xCE16):

By default, the device can be shut down via the key. After the downlink is turned off, the device cannot be shut down via the key.

(7) Control device triggers sleep switch issuance (0xCE18):

The default is on. The device enters sleep mode if it is not moved for 40 minutes. After the downlink is turned off, the device does not enter sleep mode.

(8) Button triggers SOS start switch (0xCE19)

The default is on. Long press the charging cable button to trigger SOS. After it is turned off, long press the charging cable button will not trigger the report of SOS.

(9) Control device charging on/off state (0xCE21):

The default charging is off. After the downlink is turned on, the device will not shut down when charging, and the device will report the charging status.

(10) Long connection and short connection mode switching (0xCE22)

The default is short connection. The downlink switches the long connection command. After the device receives it, it restarts and becomes a long connection. Note that the power consumption of the long connection mode will increase. The default state is to report a heartbeat package every 4 minutes (0xF9).

(11) UWB alarm distance (0x79):

Control the distance of UWB ranging alarm, where 0 meters means no alarm

(12) Send shutdown and restart when the device is turned on (0x77)

This command can only be sent when the device is powered on, and it is invalid when the device is powered off. After the downlink command, the device will be restarted/shut down.

3 Protocol Data Packet Structure

A basic protocol data packet structure is shown in Figure 1:

3.1 Data Header

Each data packet starts with a 4-byte Header or token (timestamp replaces it in some response messages):

Currently, the token uploaded by OuFu devices is fixed as BDBDBDBD

Header: 0xBD0xBD0xBD0xBD；

Timestamp: 32bits, generated by the server

3.2 Message ID

See Chapter 4 for the content represented by MessgeId.

Each time a TCP connection is made, the device will first report a 0xF0 message, which contains the device's unique identifier IMEI.

The server needs to record the IMEI as an identifier and reply with a 0xf1 message. Only then will the device consider the connection successful; otherwise, it will disconnect.

During normal device communication, the long connection reports a 0xF9 heartbeat packet every 4 minutes by default. The 0xF9 heartbeat packet is reported once after a health location report.

For short connections, the 0xF9 heartbeat packet is reported once at startup, and once after a health location report. The reporting of the two is not affected by each other.

Note that after the server receives the F9 message, it needs to issue a command to reply; otherwise, the connection will be disconnected.

3.3 Token Generation Mechanism

Currently fixed as BDBDBDBD

3.4 Payload

The payload we refer to below is the valid content in the protocol, excluding the head token and checksum. The length of the content is noted after it.

The data format used in the payload is shown in the table below:

【U-unsigned；I-signed；X-bitfield；Number-bytes occupied】

In the following protocol, little-endian is used except for ch, u8, i8, and x8.

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Description |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short Integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short Integer |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Integer |
| x16 | Bitfield | 2 | - | - | Bit 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long Integer |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long Integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit Long Integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating point type |

3.5 Checksum

The content added to the checksum includes the payload, as shown in Figure 1. The algorithm is as follows. By default, the general-purpose version of the device does not require checksum verification, and this part can be ignored. Any byte can be used for the downlink instruction.

//Assuming content="BDBDBDBDE9000600010A00000A00", then the result of sum.ToString("X2") is 07//The following is the C# code calling method

private string CheckSum(string content)

{

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2");

}

4 Report messages message

4.1 Connection related report

4.1.1 LNK-LIN (MSGID=0xF0) Request connection (TCP dedicated)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xF0 | See below for definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | u64 | IMEI | 1 | - | IMEI number (little endian) |
| 2 | x16 | version |  | - | Bitfield see below |

The device's reported F0 request message must have a server downlink 0XF1 message in response (see the next section for the specific format), otherwise the login will fail.

Example:

Original16hexadecimal message: BDBDBDBDF09B51731BC61603000000C2

BDBDBDBD:4byte message headerHeader

F0: Message ID(Message ID)9B51731BC6160300: IMEI number(little endian), converted to big endian:000316C61B73519B,

The corresponding decimal is IMEI:869465050010011,0000: version

C2: checksum

Connection related Q&A:

Each time a new connection is created via TCP, an F0 request is reported first. The F0 contains the IMEI, and then the server records this IMEI.

After that, all the data in this link belongs to this IMEI.

Note: If F0 is not received or F0 communication is abnormal, you can use a third-party network testing tool to verify whether the server communication is normal.

4.1.2 Connection Reply (MSGID=0xF1) (TCP Only) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Timestamp (unix) timestamp | 0xF1 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | u32 | Token | 1 | - | Token: Fixed as BDBDBDBD |

Note that the message sequence is different from the normal reported message. This message is issued by the server, not the content reported by the device. The server reply needs to be replied in the current channel. > Note that the message sequence is different from the normal reported message. This message is issued by the server, not the content reported by the device. The server reply needs to be replied in the current channel.

Example:

Original16hexadecimal message:07FD8860F1BDBDBDBD2E07FD8860(little endian first),Timestamp(unix) timestamp (unit: seconds), converted to big endian6088FD07 converted to decimal: 1619590407

Convert to standard time formatUTCtime:2021-04-28 06:13:27F1:Message ID(message id):BDBDBDBD:Token2E:checksumchecksum

The timestamp10decimal to standard time format is the result of adding1619590407 seconds to 1970-01-0100:00:00.

The device synchronizes time with the timestamp (unit: seconds) in this message.

Note: Injava, the unit is generally milliseconds. Pay attention to converting to the unit: seconds.

Note: The total number of bytes replied by the server downstream is 10 bytes, such as: 0xBD--1 byte; if the device receives 20 bytes, the connection will not be established

4.1.3 New Heartbeat Packet Protocol (MSGID=0xF9)-Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xF9 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery type 0: 4-level system, 1: 5-level system, 2: percentage, 3: voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery level If Bat_type is 0: The battery level range is 0-3, (0 is 25%, 3 is 100%) If Bat_type is 1: The battery level range is 0-4 (0 is 20%, 4 is 100%) If Bat_type is 2: The battery level range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: Percentage, 1: 5-level system, 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal strength |
| 1 | U8 | Other_type |  |  | Extended type 0: Full step counting 1: Incremental step counting 2: Vibration |
| 4 | U32 | Num |  |  | Extended value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Note: The F9 heartbeat packet must have a server downlink reply. The device considers the server connection to be unbroken only when it receives the server downlink reply; otherwise, it will re-report the F0 request connection.

Example:

Original hexadecimal message: BDBDBDBDF90104000050000095000000E377BD67AA

BDBDBDBD: Header

F9: Message ID

01: Bat_type battery type 01 --> 1 --> 5-level battery (0-4)

0400: Bat_volt battery value converted to big-endian 0004 --> 4 --> Level 4 corresponds to 100% battery

00: Signal_type signal type 00 --> 0 --> percentage

5000: Signal_strength signal strength converted to big-endian 0050 --> converted to decimal 80 --> 80% signal strength

00:Other_type extended type 00-->0--> Full amount of step counting

95000000:Num extended value - step count converted to big-endian00000095--> converted to decimal149--> step count is149

E377BD67: Timestamp converted to big-endian67BD77E3--> converted to10base1740470243--> timestamp is1740470243seconds --> converted to standard time format UTC time:2025-02-25 07:57:23--> converted to Beijing time:2025-02-25 15:57:23

AA: checksum

Note: The server receives the F9 heartbeat packet and can reply with a fixed message: Fixed reply example BDBDBDBDF301 Number of bytes:6

4.2 Alarm-related reporting

4.2.1 Alarm data upload-1 (MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x02 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below |
| 4 | U32 | Timestamp | - | - | Timestamp (a timestamp will be added later when retransmitting) |

Upl_warn contents

| bit (corresponding to the position of 1 in binary) | Name | Description | Original message in little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Shutdown | Shutdown | 0400 | 0004 | 4 |
| 0 | Low battery | Low battery | 0100 | 0001 | 1 |
| 14 | Fall alarm | Fall alarm | 0040 | 4000 | 16384 |
| 1 | SOS alarm | SOS alarm | 0200 | 0002 | 2 |
| 7 | SOS Cancel | SOS Cancel | 8000 | 0080 | 128 |

Example:

Low battery alarm: BDBDBDBD020200E377BD678A

BDBDBDBD:Header

02:Message ID

0100:Upl_warn alarm content, converted to big-endian0001-->Low battery alarm

E377BD67:Timestamp, converted to big-endian67BD77E3-->converted to10decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8A：checksum

Shutdown alarm: BDBDBDBD020400E377BD678A

BDBDBDBD:Header

02:Message ID

0400:Upl_warn alarm content, converted to big-endian0004-->Shutdown alarm

E377BD67:Timestamp, converted to big-endian67BD77E3-->converted to10decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8A：checksum

Note: This alarm is used for devices that do not distinguish between shutdown types. See the next section for devices that distinguish between shutdown types.

SOS Alarm: BDBDBDBD020200E377BD678A

BDBDBDBD:Header

02:Message ID

0200:Upl_warn alarm content, converted to big-endian0002-->SOS alarm

E377BD67:Timestamp, converted to big-endian67BD77E3-->converted to decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8a：checksum

SOS Cancellation: BDBDBDBD028000E377BD678A

BDBDBDBD:Header

02:Message ID

8000:Upl_warn alarm content, converted to big-endian0080-->SOS cancellation

E377BD67:Timestamp, converted to big-endian67BD77E3-->converted to decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

8a：checksum

Fall Alarm: BDBDBDBD020040E377BD678A

BDBDBDBD:Header

02:Message ID

0040:Upl_warn alarm content, converted to big-endian4000-->Fall alarm

E377BD67:Timestamp, converted to big-endian67BD77E3-->converted to decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

E7: Checksum

4.2.2 Alarm Data Upload-2 (MSGID=0x21) (Supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x21 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | Alarm type |
| N |  | Upl_warn | - | - | Alarm content See below for different Alarm type type definitions |
| 4 | U32 | Timestamp |  |  | Timestamp (a timestamp will be added later when retransmitting) |

Alarm type=1

Upl_warn contents

| bit (corresponding to the position of 1 in binary) | Name | Description | Original message in little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging shutdown alarm | Device charging automatic shutdown (firmware requires function support) | 04000000 | 0004 | 4 |
| 1 | Low battery shutdown alarm | Device power is too low to shut down (firmware requires function support) | 02000000 | 0002 | 2 |
| 0 | Active shutdown alarm | The device is manually shut down by someone (firmware requires function support) | 01000000 | 0001 | 1 |

Example:

Charging shutdown alarm: BDBDBDBD21010004000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

04000000:Upl_warn alarm content, converted to big-endian 0004-->charging shutdown alarm

E377BD67:Timestamp converts the timestamp to big-endian 67BD77E3-->converts to decimal 1740470243-->the timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Low battery shutdown alarm: BDBDBDBD21010002000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

02000000:Upl_warn alarm content, converted to big-endian 0002-->Low battery shutdown alarm

E377BD67:Timestamp converts the timestamp to big-endian 67BD77E3-->converts to decimal 1740470243-->the timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Active shutdown alarm: BDBDBDBD21010001000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type alarm type, converted to big-endian 0001-->Alarm type=1

01000000:Upl_warn alarm content, converted to big-endian 0001-->Active shutdown alarm

E377BD67:Timestamp timestamp, converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Alarm type=4

Upl_warn contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | 02—UWB ranging alarm |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple groups of information, each group may contain multiple Ubeacons |
| U8 | Total_PackCount | 1 | Total number of alarm UBeacons at the current time |
| U32 | UBeacon0 mac | 4 | UBeacon mac |
| U16 | Distance0 | 2 | Distance (in centimeters) |
| U32 | UBeacon1 mac | 4 | UBeacon mac |
| U16 | Distance1 | 2 | Distance (in centimeters) |
| U32 | UBeacon2 mac | 4 | UBeacon mac |
| U16 | Distance2 | 2 | Distance (in centimeters) |
| ..... | ..... | ..... | ..... |

Example:

Original hexadecimal message:BDBDBDBD210400020103320600009100E70500006800380500005C00E377BD6755

BDBDBDBD:Header

21:Message ID message id

0400:Alarm type Alarm type converted to big-endian 0004-->Alarm type=4

02:Type Convert to decimal 02--UWB ranging alarm

01:Total_groups Total number of groups converted to decimal 01, 1 group

03:Total_PackCount Total number of alarm Ubeacons at the current time Convert to decimal 03, 3 UWB beacons

32060000:UBeacon0 mac Converted to big-endian 00000632, UWB beacon id is 00000632

9100:Distance0 Distance Converted to big-endian 0091-->Converted to decimal 145-->The distance from the UWB beacon is 145 cm

E7050000:UBeacon1 mac

6800:Distance1 distance converted to big-endian 0068-->converted to decimal 105-->the distance to the UWB beacon is 105 cm

38050000:UBeacon2 mac

5C00:Distance2 distance converted to big-endian 005C-->converted to decimal 92-->the distance to the UWB beacon is 92 cm

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

55:checksum

4.3 Location-related reporting

4.3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x03 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude longitude |
| 8 | Double | lat |  | - | latitude |
| 1 | U8 | north_south |  | - | /N or S/ North or South latitude |
| 1 | U8 | east_west |  | - | /E or W/ East or West longitude |
| 1 | U8 | status |  | - | /A or V/ Valid data or invalid data |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original hexadecimal message: BDBDBDBD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BDBDBDBD:Header

03:Message ID

963E870C3E675E40:lon longitude, convert to big-endian405E673E0C873E96-->Longitude:121.6131622 Note: See the parsing example below for conversion and parsing

FF2110C2B6343F40:lat latitude, convert to big-endian `403F34B6C21021FF-->Latitude:31.2059137 Note: See the parsing example below for conversion and parsing

4E:north_south 4E-->Convert to ASCII code N-->North latitude

45:east_west 45-->Convert to ASCII code E-->East longitude

41:status data validity 41-->Convert to ASCII code A-->Valid data

E377BD67:Timestamp, convert to big-endian 67BD77E3-->Convert to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

78:checksum

Note: The parsed Satellite (GPS/BDS) latitude and longitude are in the WGS-84 coordinate system. If the map uses Baidu AMAP, etc., the coordinate system needs to be converted.

Parsing satellite positioning message Java code example:

//Message: BDBDBDBD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BDBDBDBD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public static void main(String[] args){

//Message 7d9f84ac81815c40 actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d",16)));  //114.02353966666665//Message e766926b1d893640 actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16))) ;//22.535605166666667

HexToStr(data.Substring(“4e”));   //N

HexToStr(data.Substring(“45”));//E

HexToStr(data.Substring(“41”));//A   A indicates valid data, V indicates invalid data, can be discarded//Message 749d695f actual value 5f699d74

Date date=new Date();

date.setTime(Long.parseLong("5f699d74",16)*1000);

SimpleDateFormatsdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.3.2 Wi-Fi and base station information upload (MSGID=0xA4)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xA4 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 1 | u8 | Cell_cnt | 1 | - | Number of cell info payload. Valid value: 0~7 |
| 2 | u16 | Cell[0].MCC | - | - | Mobile country code of cell[0] |
| 2 | u16 | Cell[0].MNC | - | - | MNC of base station (0) mobile network code of cell[0] |
| 2 | u16 | Cell[0].LAC | - | - | LAC of base station (0) Location area code of cell[0] |
| 4 | U32 | Cell[0].CELL_ID | - | - | ID of base station (0) Cell id of cell[0] |
| 2 | i16 | Cell[0].RSSI | - | dbm | Signal strength RSSI of base station (0) RSSI in dbm of cell[0] |
| ... | ... | ... | ... | ... | ... |
| 2 | u16 | Cell[cell_cnt-1.MCC | - | - | MMC of base station (N) mobile country code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.MNC | - | - | MNC of base station (N) mobile network code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.LAC | - | - | LAC of base station (N) Location area code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1].LAC | - | - | LAC of base station (N) Location area code of cell[[cell_cnt-1] |
| 4 | U32 | Cell[cell_cnt-1].CELL_ID | - | - | ID of base station (N) Cell id of cell[[cell_cnt-1] |
| 2 | I16 | Cell[cell_cnt-1].RSSI | - | dbm | Signal strength RSSI of base station (N) RSSI in dbm of cell[[cell_cnt-1] |
| 1 | U8 | Wifi_cnt |  |  | Number of Wi-Fi networks Number 0f wifi |
| 6 | 6*U8 | Wifi[0].bssid[0]-Wifi[0].bssid[5] |  |  | MAC address of Wi-Fi (0) |
| 4 | I32 | Wifi[0].RSSI |  |  | RSSI signal strength of Wi-Fi (0) |
| ... | ... | ... | ... | ... | ... |
| 6 | 6*U8 | Wifi[Wifi_cnt-1].bssid[0]-Wifi[Wifi_cnt-1].bssid[5] |  |  | MAC address of Wi-Fi (N) |
| 4 | I32 | Wifi[0].RSSI |  |  | RSSI signal strength of Wi-Fi (0) |

Note: The device may not report base stations every time. The absence of base stations does not affect positioning. If the number of base stations in the report is 0, then the following is directly the number of Wi-Fi networks.

Example:

Original hexadecimal message: BDBDBDBDA4E377BD6701CC010000C218D1AD160BF9FF078CBEBE1A8162C6FFFFFFC061180AF42AC1FFFFFF200BC726E000B6FFFFFFA8154DF6517EB2FFFFFFE005C5B1F824CCFFFFFFE8FCAFA02663AFFFFFFF6409805B2B9CAEFFFFFF94

BDBDBDBD:Header

A4: Message ID

E377BD67:Timestamp, converted to big-endian67BD77E3-->converted to10decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

01: Cell_cnt number of base stations 01 --> The number of base stations is 1

CC01: Cell[0].MCC Base station MCC converted to big-endian 01CC--> converted to decimal 460 --> Base station MCC is 460

0000: Cell[0].MNC Base station MNC converted to big-endian 0000 --> converted to decimal 0 --> Base station MNC is 0

C218: Cell[0].LAC Base station LAC converted to big-endian 18C2--> converted to decimal 6338 --> Base station LAC is 6338

D1AD160B: Cell[0].CELL_ID Base station ID converted to big-endian 0B16ADD1--> converted to decimal 186035665 --> Base station ID is 186035665

F9FF: Cell[0].RSSI Base station signal strength converted to big-endian FFF9--> converted to decimal complement -7 --> Base station signal strength is -7

07: Wifi_cnt number of wifi 07 --> 7 wifi

8CBEBE1A8162:wifi mac address 8C:BE:BE:1A:81:62 -->wifi[0]'s mac address is 8C:BE:BE:1A:81:62

C6FFFFFF: Convert WiFi signal strength RSSI to big-endian FFFFFFC6 --> Convert to decimal complement - 58 ---> WiFi [ 0 ] RSSI signal strength is - 58

C061180AF42A:wifi mac address C0:61:18:0A:F4:2A-->The MAC address of wifi[1] is C0:61:18:0A:F4:2A

C1FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFC1-->converted to decimal complement -63--->The RSSI signal strength of wifi[0] is -63

200BC726E000:wifi mac address 20:0B:C7:26:E0:00-->The MAC address of wifi[2] is20:0B:C7:26:E0:00

B6FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFB6-->converted to decimal complement -74--->The RSSI signal strength of wifi[0] is -74

A8154DF6517E:wifi mac address A8:15:4D:F6:51:7E-->The MAC address of wifi[3] is A8:15:4D:F6:51:7E

B2FFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFB2-->converted to decimal complement -78--->The RSSI signal strength of wifi[0] is -78

E005C5B1F824:wifi mac address E0:05:C5:B1:F8:24-->The MAC address of wifi[4] is E0:05:C5:B1:F8:24

CCFFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFCC-->converted to decimal complement -52--->The RSSI signal strength of wifi[0] is -52

E8FCAFA02663:wifi mac address E8:FC:AF:A0:26:63-->The MAC address of wifi[5] is E8:FC:AF:A0:26:63

AFFFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFFFAF-->converted to decimal complement -81---->The RSSI signal strength of wifi[0] is -81

6409805B2B9C:wifi mac address 64:09:80:5B:2B:9C-->wifi[6]'s mac address is 64:09:80:5B:2B:9C

AEFFFFFF:wifi signal strength RSSI converted to big-endian FFFFFFFFAE-->converted to decimal complement-82--->wifi[0]'s RSSI signal strength is -82

94:checksum

The parameters parsed from the message need to be connected to the map wifi database, and the latitude and longitude data can be finally obtained through the map party. Taking AMAP connecting to the wiif database as an example:

According to the access network method, AMAP wifi positioning protocol document is divided into three positioning scenarios. The following gives corresponding examples for the three positioning scenarios.

1. The network connection method is wifi access, that is, the accesstype parameter is 1

http://apilocate.amap.com/position?accesstype=1&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&mmac=50:a7:2b:1e:56:58,-60,alibaba-inc&macs=4c:48:da:25:0b:11,-59,alibaba-

inc|4c:48:da:25:1a:11,-77,alibaba-inc&serverip=10.2.166.4&output=xml&key=<用户 Key>

2. The network connection method is mobile access and the mobile phone card is a non-cdma card, that is, when the accesstype parameter is 0 and cdma=0:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=0&bts=460,01,40977,2205409,-65&nearbts=460,01,40977,2205409,-65|460,0

1,40977,2205409,-65|460,01,40977,2205409,-65&serverip=10.2.166.4&output=xml&key=<User Key>

3. When the network type is mobile access and the phone card is a CDMA card, that is, when the accesstype parameter is 0 and cdma=1:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=1&bts=13824,1,1838,1674723,575739,-52&serverip=10.2.166.4&output=xml&key=<User Key>

W200P watch: AMAP wifi positioning protocol calls AMAP interface description: When selecting parameters, generally select the first type

You can also choose the second type, but if the device does not report the base station, the bts and neabts parameters need to be removed before it can

The first type, example of incoming parameters: mmac is the first WIFI mac address in the messagehttp://apilocate.amap.com/position?accesstype=1&imei=869907057780989&mmac=04:8C:16:A9:FB:A8,-44,TP-Link&macs=04:8C:16:A9:FB:A8,-44,TP-Link|C8:3A:35:F4:CE:18,-59,TP-Link|1C:78:4E:D5:07:FE,-79,TP-Link|C8:3A:35:9D:0B:E0,-87,TP-Link|4A:D8:90:E8:0A:82,-88,TP-Link|AC:AD:4B:C9:C3:87,-89,TP-Link|A4:16:E7:55:84:AC,-91,TP-Link&output=json&key=xxxx

For the specific AMAP wifi positioning protocol PDF, you can click to view and download:(not a screenshot, you can click to view)
This document is provided by AMAP, you can directly ask AMAP for specific docking
Note that the name of the downloaded document will change, you can rename it yourself

4.3.3 Bluetooth Positioning Information (BLE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xD6 | See below See definition below | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 0 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple sets of information, and each group may contain multiple iBeacons |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | RSSI (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | RSSI (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | RSSI (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | RSSI (Bluetooth beacon signal strength) |
| ... | ... | ... | ... |

Note: In the general version, the total number of Bluetooth beacon groups is fixed at 1, and the maximum number of Bluetooth beacons is 4. The sorting is based on signal strength from largest to smallest. By default, the watch only recognizes our own beacons. If you need to connect to other manufacturers' beacons, please consult relevant personnel via WeChat. Example:

Original hexadecimal message: BDBDBDBDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

bdbdbdbd: Header

D6: Message ID

00: type

01: Only one set of iBeacon data

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

04: The first group has 4 beacon information

4327: Convert to big endian --> major : 0x2743 --> Convert to decimal 10051

1794: Convert to big endian --> minor: 0x9417 --> Convert to decimal 37911

AC:  RSSI 0xAC --> Convert to decimal complement -84

4327: Convert to big endian --> major: 0x2743 --> Convert to decimal 10051

3094: Convert to big endian --> minor:0x9430 --> Convert to decimal 37936

AA:  RSSI 0xAA --> Convert to decimal complement -86

4327:Convert to big endian --> major: 0x2743 --> Convert to decimal 10051

b956:Convert to big-endian --> minor:0x56b9 --> Convert to decimal 22201

A5: RSSI 0xA5-->Convert to decimal complement -91

4327: Convert to big endian --> major: 0x2743 --> Convert to decimal 10051

FE94: Convert to big-endian-->minor:0x94FE-->Convert to decimal 38142

A5: rssi:0xA5-->Convert to decimal complement -91

6a --Checksum

Bluetooth beacon major, minor description: The default shipping Bluetooth beacon has a beacon code attached.

For example: 00081005345866, then the major of the Bluetooth beacon is 10053, the minor is 45866, and the manufacturer ID is 0008 (this is useless and has no practical significance)

4.3.3 UWB Location（MsgId=0xD7）

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xD7 | See below See definition below | CK_sum |

Payload contents

| Format() | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 01 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple groups of information, each group may contain multiple Ubeacons |
| Int32 | Timestamp | 4 | UTC timestamp |
| U8 | Total_PackCount | 1 | Total number of UBeacons at the current time |
| U32 | UBeacon0 mac | 4 | UBeacon devid (little-endian) |
| U16 | Distance0 | 2 | Distance (cm) |
| U32 | UBeacon1 mac | 4 | UBeacon devid (little-endian) |
| U16 | Distance1 | 2 | Distance (cm) |
| U32 | UBeacon2 mac | 4 | UBeacon devid (little-endian) |
| U16 | Distance2 | 2 | Distance (cm) |
| ... | ... | ... | ... |

Note: The general version reports a maximum of 1 group and a maximum of 4 UBeacons.

Example:

Original hexadecimal message: BDBDBDBDD70101E377BD6703320600009100E70500006800380500005C00A1

BDBDBDBD:Header

D7: Message ID

01: Type 01

01: Total_groups Total number of groups, converted to decimal01--> Total number of groups is1

E377BD67: Timestamp Convert to big-endian 67BD77E3--> Convert to decimal1740470243--> Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23-->Convert to Beijing time:2025-02-25 15:57:23

03: Total_PackCount Number of UWB beacons Convert10to decimal03--> The number of UWB beacons is3

32060000: UBeacon0 mac UWB beacon's id converted to big endian00000632-->UWB beacon id is00000632

6800: Distance0 Ranging distance converted to big endian0091-->Convert to decimal145-->Ranging distance is145cm

E7050000: UBeacon1 mac UWB beacon's id converted to big endian000005E7-->UWB beacon id is000005E7

9100: Distance1 Ranging distance converted to big endian0068-->Convert to decimal104-->Ranging distance is104cm

38050000: UBeacon2 mac UWB beacon's id converted to big endian00000538-->UWB beacon id is00000538

5C00: Distance2 Ranging distance converted to big endian005C-->Convert to decimal92-->Ranging distance is92cm

A1:checksum

4.4 Device information and status reporting

4.4.1 SIM card's ICCID upload (MSGID=0xF3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xF3 | See below See definition below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 10 | 10*U8 | ICCID | 1 | - | ICCID number - Device SIM card number ICCID |

Note: This message is reported once every time the device is powered on.

Example:

Original hexadecimal message:BDBDBDBDF389861118236001639994CC

BDBDBDBD:Header

F3:Message ID

89861118236001639994:ICCID Device SIM card number ICCID is: 89861118236001639994

CC:checksum

4.4.2 Device charging status upload (MSGID=0xC3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Status | / | / | 0 starts, 1 ends, 2 is fully charged |
| 1 | U32 | Timestamp | / | / | Timestamp (a timestamp will be added later when retransmitting) |

Note: This message requires device firmware support. The latest general version is supported by default. This message will only be reported when the downlink command is enabled: charging startup. Example:

Original hexadecimal message:BDBDBDBDC301E377BD678A

BDBDBDBD:Header

C3:Message ID

01:Device finished charging

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

Original hexadecimal message:BDBDBDBDC300DB4D2F668A

BDBDBDBD:Header

C3:Message ID

00:Device started charging

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

Original hexadecimal message:BDBDBDBDC302DB4D2F668A

BDBDBDBD:Header

C3:Message ID

02:Device is fully charged

E377BD67:Timestamp converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

4.4.3 Status Parameter Reporting (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Number of parameter types |
| 1 |  |  |  |  | (Reserved 00) |
| 1 | u8 | Type | -/- | -/- | Parameter type 1, Type definition: 00—MCU (firmware), 01—module, 02 sensor (generic version does not have this type) |
| 1 | U8 | NameLen | -/- | -/- | Parameter name 1 length |
| n | N*u8 | Name | -/- | -/- | Parameter name |
| 1 | u8 | Type | -/- | -/- | Parameter Type 2, Type Definition: 00—MCU (Firmware), 01—Module, 02 Sensor (This type is not currently available in the general version) |
| 1 | U8 | NameLen | -/- | -/- | Parameter type 2 length |
| n | N*u8 | Name | -/- | -/- | Parameter name |
| ... | ... | ... | ... | ... | ... |

Note: This message is reported once by default when the device is powered on, uploading the device firmware version number, module version number, which can be left unparsed. Previously shipped devices had a Message ID of 0xBB, which can be sent by contacting relevant personnel.

Example:

BDBDBDBDA902000009423732352E4F56303601174E5432364B434E4230304E4E412D4C3032303330393530B5

BDBDBDBD:Header

A9:Message ID

02:TypeCnt indicates there are 2 types of parameters

00:Reserved field

00:Type=00  00 represents MCU (firmware)

09:MCU name length 9 bytes

423732352E4F563036--->Hex to text (device firmware version number)--->B725.OV06

01:Type=01  01 represents module

17:Module name length after 23 bytes

4E5432364B434E4230304E4E412D4C3032303330393530:Hex to text (4G module version number)

--->NT26KCNB00NNA-L02030950

B5---checksum

4.4.4 Device Status (MSGID=0xE9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xE9 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | type |  | / | Default—00 |
| 2 | u16 | length |  | / | Message length behind |
| 1 | u8 | location |  | / | Location reporting frequency: 00 -- Default unmodified, 01 -- Modified by downlink (if there are multiple time periods in the downlink, only report the reporting frequency of the current time period) |
| 2 | u16 | frequency |  | / | Frequency, unit - minutes |
| 1 | u8 | health |  | / | Health reporting frequency: 00 -- Default unmodified, 01 -- Modified by downlink |
| 2 | u16 | frequency |  | / | Frequency: Unit - minutes |

Note: This message is reported once at startup, and once after the downlink modifies the frequency. The latest general version has it, but previous versions do not.

Example:

Original hexadecimal message:BDBDBDBDE9000600010A00010A0033

BDBDBDBD:Header

E9:Message ID

00:type=00

0600:length The message length after length is converted to big-endian --> 0006 --> converted to decimal 6, the message length is 6 bytes

01:location 01 indicates that the downlink has modified the location reporting frequency

0A00:frequency location reporting frequency is converted to big-endian --> 000A --> converted to decimal 10, the reporting frequency is 10 minutes

01:health 01 indicates that the downlink has modified the health reporting frequency

0A00:frequency Health reporting frequency is converted to big-endian --> 000A --> converted to decimal 10, the reporting frequency is 10 minutes

33:checksum

If the device does not have health reporting function or location reporting function, the two values of health reporting frequency and location reporting frequency will remain the same, indicating the device reporting frequency

4.5 Downlink feedback related reporting

4.5.1 Downlink feedback (MSGID=0xC0)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC0 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This command is used for feedback of downlink commands, returning the Message ID of the previously received downlink command (can be a collective return of multiple Message IDs).

Original hexadecimal message:BDBDBDBDC0011720

BDBDBDBD:Header

C0:Message ID

01:length Message ID length Convert to decimal 01--> Message ID length is 1 byte

17Type indicates that the device received the downlink command Message ID is 17

20:checksum

5 Settings

5.1 Downlink

5.1.1 Set location reporting frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x17 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - hour |  |
| 1 | u8 | time_start_m |  |  | - minute |  |
| 1 | u8 | time_end_h |  |  | - hour |  |
| 1 | u8 | time_end_m |  |  | - minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - hour |  |
| 1 | u8 | time_start_m |  |  | - minute |  |
| 1 | u8 | time_end_h |  |  | - hour |  |
| 1 | u8 | time_end_m |  |  | - minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - hour |  |
| 1 | u8 | time_start_m |  |  | - minute |  |
| 1 | u8 | time_end_h |  |  | - hour |  |
| 1 | u8 | time_end_m |  |  | - minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for disabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - hour |  |
| 1 | u8 | time_start_m |  |  | - minute |  |
| 1 | u8 | time_end_h |  |  | - hour |  |
| 1 | u8 | time_end_m |  |  | - minute |  |

Note: A day is from 00:00-23:59
Example:

00:00-23:59 Report location every10minutes: BDBDBDBD17010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

17:Message ID

01:enable is enabled to decimal01-->1--enabled

0A00:Interval time interval converted to big-endian000A-->converted to decimal10-->time interval is10minutes

00:time_start_h Start time-hour, convert to decimal00-->Start time-hour is00

00:time_start_m Start time-minute, convert to decimal00-->Start time-minute is00

17:time_end_h End time-hour, convert to decimal23-->End time-hour is 23

3B:time_end_m End time-minute, convert to decimal59-->End time-minute is 59

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

00:enable Whether to enable Convert to decimal00-->0--Not enabled

000000000000:All filled in the disabled state00

97:checksum

5.1.2 Setting - Positioning priority setting (0XCE01&0XCE02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See definition below | CK_sum |

Payload contents

Type=01 Positioning priority setting

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 is always valid, 01 is effective this time |
| 2 | U16 | Len | -/- | -/- | The length of the following instruction |
| n | n |  | -/- | -/- | Text |

Location priority setting definition--See 2.3 Device downlink description--Device location priority distribution (0xCE01)

Location priority setting text:

Currently, the general version supports:01--Satellite (GPS/BDS) ,02--wifi (wifi+base station)03--Bluetooth beacon (requires additional deployment of Bluetooth beacons),06---UWB beacon (requires additional deployment of UWB beacons);

Special equipment support:04--LBS base station 05--125k

Example:

Wifi positioning priority (wifi>Bluetooth beacon>Satellite (GPS/BDS)):BDBDBDBDCE0100030002030133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning Priority Setting

00:Valid Validity 00 Always Valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The length of the text is 3 bytes

020301:Text: 02--wifi 03--Bluetooth beacon 01--Satellite (GPS/BDS) Positioning priority is wifi>Bluetooth beacon>Satellite (GPS/BDS)

33:checksum

Satellite (GPS/BDS) positioning priority (GPS>wifi>Bluetooth beacon):BDBDBDBDCE0100030001020333

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning Priority Setting

00:Valid Validity 00 Always Valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The length of the text is 3 bytes

010203:Text: 01--Satellite (GPS/BDS)  02--wifi 03--Bluetooth beacon Positioning priority is Satellite (GPS/BDS)>wifi>Bluetooth beacon

33:checksum

Bluetooth positioning priority (Bluetooth beacon>wifi>Satellite (GPS/BDS)):BDBDBDBDCE0100030003020133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning Priority Setting

00:Valid Validity 00 Always Valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The length of the text is 3 bytes

030201:Text: 03--Bluetooth beacon 02--wifi 01--Satellite (GPS/BDS)  Positioning priority is Bluetooth beacon>wifi>Satellite (GPS/BDS)

33:checksum

UWB positioning priority (UWB beacon>wifi>Satellite (GPS/BDS)):BDBDBDBDCE0100030006020133

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning Priority Setting

00:Valid Validity 00 Always Valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The length of the text is 3 bytes

060201:Body: 06--UWB beacon 02--wifi 01--Satellite (GPS/BDS)  Positioning priority is Bluetooth beacon>wifi>Satellite (GPS/BDS)

33:checksum

Single Satellite (GPS/BDS) positioning:BDBDBDBDCE0100030001000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning Priority Setting

00:Valid Validity 00 Always Valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The length of the text is 3 bytes

030201:Text: 01--Satellite (GPS/BDS)  00--No positioning 00--No positioning Positioning priority is single Satellite (GPS/BDS)  positioning

33:checksum

Single wifi positioning:BDBDBDBDCE0100030002000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning Priority Setting

00:Valid Validity 00 Always Valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The length of the text is 3 bytes

020000:Text: 02--wifi 00--No positioning 00--No positioning Positioning priority is single wifi positioning

33:checksum

Single Bluetooth beacon positioning:BDBDBDBDCE0100030003000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning Priority Setting

00:Valid Validity 00 Always Valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The length of the text is 3 bytes

020000:Text: 03--Bluetooth beacon 00--No positioning 00--No positioning Positioning priority is single Bluetooth beacon positioning

33:checksum

Single UWB beacon positioning:BDBDBDBDCE0100030006000033

BDBDBDBD:Header

CE:Message ID

01:Type 01 Positioning Priority Setting

00:Valid Validity 00 Always Valid

0300:Len Converted to big-endian 0003-->Converted to decimal 03-->The length of the text is 3 bytes

060000:Text: 06--UWB beacon 00--No positioning 00--No positioning Positioning priority is single UWB beacon positioning

33:checksum

5.1.3 Settings - Switch settings (0XCE04-24)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xCE | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | See the instructions for specific types |
| 1 | u8 | switch | -/- | -/- | Switch 00 on, 02 off, 01 single effective |
| 2 | U16 |  | -/- | -/- | Fixed 0000 |

Device alarm settings - switch type: detailed function description--see 2.3 Device downlink description

05 Bluetooth broadcast switch

16 Whether the downlink can use the key to shut down

18 Sleep function switch

19 Key trigger SOS alarm switch

21 Charging is power on or power off state

22 Long and short connection mode switching

Type=05 Bluetooth broadcast switch
Example:

00---Open:BDBDBDBDCE0500000093

BDBDBDBD:Header

CE:Message ID

05:Type 05 Bluetooth broadcast switch

00:switch switch 00 on

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE0502000093

BDBDBDBD:Header

CE:Message ID

05:Type 05 Bluetooth broadcast switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=07 Fall alarm switch
Example:

00---Open:BDBDBDBDCE0700000093

BDBDBDBD:Header

CE:Message ID

07:Type 07 Fall alarm switch

02:switch on/off 00 on

0000:Fixed value

93:checksum

02---Off:BDBDBDBDCE0702000093

BDBDBDBD:Header

CE:Message ID

07:Type 07 Fall Alarm Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=16 Whether the downlink can use the key to shut down
Example:

00---On:BDBDBDBDCE1600000093

BDBDBDBD:Header

CE:Message ID

16:Type 16 Whether the downlink can use the key to shut down

02:switch on/off 00 on

0000:Fixed value

93:checksum

02---Off:BDBDBDBDCE1602000093

BDBDBDBD:Header

CE:Message ID

16:Type 16 Whether the downlink can use the key to shut down

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=18 Sleep function switch
Example:

00---On, enter sleep after being still for a period of time, and do not report data:BDBDBDBDCE1800000093

BDBDBDBD:Header

CE:Message ID

18:Type 18 hibernation function switch

02:switch on/off 00 on

0000:Fixed value

93:checksum

02---Close, Static or Charging Status, also normally reports data: BDBDBDBDCE1802000093

BDBDBDBD:Header

CE:Message ID

18: Type 18 Sleep Function Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=19 Key Triggered SOS Alarm Switch Example:

00---Open: BDBDBDBDCE1900000093

BDBDBDBD:Header

CE:Message ID

19: Type 19 Key Triggered SOS Alarm Switch

02:switch on/off 00 on

0000:Fixed value

93:checksum

02---Close: BDBDBDBDCE1902000093

BDBDBDBD:Header

CE:Message ID

19: Type 19 Key Triggered SOS Alarm Switch

02:switch switch 02 off

0000:Fixed value

93:checksum

Type=21 Charging is Power On or Power Off Status Example:

00---Charging is Power On Status: BDBDBDBDCE2100000093

BDBDBDBD:Header

CE:Message ID

21: Type 21 Charging Power On or Charging Power Off

02: switch Switch 00 Charging is Power On Status

0000:Fixed value

93:checksum

02---Charging is off:BDBDBDBDCE2102000093

BDBDBDBD:Header

CE:Message ID

21: Type 21 Charging Power On or Charging Power Off

02:switch switch 02 Charging is off

0000:Fixed value

93:checksum

Type=22 Long and short connection mode switching
Example:

00---Long connection mode, default is long connection:BDBDBDBDCE2000000093

BDBDBDBD:Header

CE:Message ID

22:Type 22 Long and short connection mode switching

02:switch switch 00 Enable long connection mode

0000:Fixed value

93:checksum

02---Short connection mode:BDBDBDBDCE2002000093

BDBDBDBD:Header

CE:Message ID

22:Type 22 Long and short connection mode switching

02:switch switch 02 Turn off long connection mode--Short connection mode

0000:Fixed value

93:checksum

5.1.4 IP&Domain Name Settings (0xC3) (TCP Dedicated)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | Issue type: Type =1 IPv4, Type =2 IPv6 (reserved item - not supported yet), Type =3 domain name (currently only supports ascii encoded domain names) |
| 2 | U16 | Port |  |  | Port number (2 bytes) |
| 1 | U8 | Len |  |  | Length of subsequent parameter content |
| N | U8 | Domainname |  |  | Specific IP or domain name content |

Note: After the issuance, the device will not communicate on the original server, but will point to the modified server issued

Example:

Type=1 IPv4:Downlink modifies ip to:118.178.184.219, port is: 8825 BDBDBDBDC30179220476B2B8DB33

BDBDBDBD:Header

C3:Message ID

01:Type 1--IPv4

7922:Port port converted to big-endian 2279-->converted to decimal 8825-->port: 8825

04:Len The length of the following parameter content is converted to decimal 04-->The length is 4 bytes

76B2B8DB:Domainname IP 76-->converted to decimal 118, B2-->converted to decimal 178, B8-->converted to decimal 184, DB-->converted to decimal 219 IP is: 118.178.184.219

33:checksum

Type=3 Domain nameDownstream modification domain name is:aiday.com.cn, port is: 8825

BDBDBDBD:Header

C3:Message ID

03:type=3, domain name

7922:Port port converts to big-endian -- 2279 --> converts to decimal port: 8825

12:Len The length of the content after the parameter is converted to decimal 18 --> The message length is 18 bytes

61696461792e636f6d2e636e:Domainname domain name converted to string --> aiday.com.cn --> domain name is: aiday.com.cn

1D:checksum

5.1.5 Shutdown and restart (MSGID=0x77)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x77 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- |  | 01—Shutdown, 00—Restart |

Note: Shutdown and restart are received in the power-on state. Shutdown: The watch shuts down after receiving the command in the power-on state, and no commands can be received after the shutdown; Restart: The watch restarts after receiving the command in the power-on state.

Example:

Shutdown:BDBDBDBD770193

BDBDBDBD:Header

77: Message ID

01:Type 01--Shutdown

93:checksum

Restart:BDBDBDBD770093

BDBDBDBD:Header

77: Message ID

01:Type 00--Restart

93:checksum

5.1.6 UWB Alarm Distance Issuance (0x7900)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed as 0xBD 0xBD 0xBD 0xBD | 0x79 | See below See definition below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Decription |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type 0—UWB ranging alarm distance |
| 1 | U8 | distance | -/- | -/- | UWB ranging alarm distance (unit: meters; value range 0-20), an alarm will be triggered if it is less than or equal to this distance |

Example:

The alarm distance is 1 meter. If the ranging distance between the watch and the UWB beacon is less than 1 meter, the watch will vibrate and alarm BDBDBDBD790001C3

BDBDBDBD:Header

79: Message ID

00: Type 00--UWB ranging alarm distance

01: distance UWB ranging alarm distance converted to decimal 01-->UWB ranging alarm distance is 1 meter

C3: checksum