G805G-4GCat.1-tcp communication protocol

Directory

1 Overview	3

2 Device Instructions	3

2.1 Device Function and Instructions	3

2.2 Device default reporting logic	4

2.3 Device Downlink Description	5

3 Protocol Data Packet Structure	8

3.1 Data Header	8

3.2 Message ID	8

3.3 Token Generation Mechanism	9

3.4 Payload	9

3.5 Checksum	10

4 Report messages message	12

4.1 Connection-related reporting	12

4.1.1 LNK-LIN (MSGID=0xF0) Request connection (TCP only)	12

4.1.2 Connection Reply (MSGID=0xF1) (TCP Only) - Important	13

4.1.3 New Heartbeat Package Protocol (MSGID=0xF9) - Important	14

4.2 Alarm-related reporting	16

4.2.1 Alarm Data Upload-1 (MSGID=0x02)	16

4.2.2 Alarm Data Upload-2 (MSGID=0x21) (Supplement to 02)	18

4.3 Location-related reporting	22

4.3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)	22

4.3.2 Upload of WiFi and base station information (MSGID=0xA4)	24

4.3.3 Bluetooth Location Information (BLE Location) (MsgId=0xD6)	29

4.4 Device Information and Status Reporting	32

4.4.1 ICCID Upload of SIM Card (MSGID=0xF3)	32

4.4.2 Device Charging Status Upload (MSGID=0xC3)	33

4.4.3 Status parameter reporting (MSGID=0xA9)	34

4.4.4 Device Status (MSGID=0xE9)	36

4.5 Downlink Feedback Related Reporting	38

4.5.1 Downlink Feedback (MSGID=0xC0)	38

5 Settings	40

5.1 Downlink	40

5.1.1 Set the location reporting frequency (MSGID=0x17)	40

5.1.2 Settings - Positioning Priority Settings (0XCE01 & 0XCE02)	42

5.1.3 Settings - Switch Settings (0XCE04-24)	45

5.1.4 IP&Domain Name Settings (0xC3) (TCP Dedicated)	50

1 Overview

This protocol is suitable for 4G CAT1 devices, currently supporting G805G products. It uses a 32-bit data header for synchronization and terminal identification; uses a low-overhead verification algorithm for verification protection; and uses message identifiers to identify different messages. It needs to be parsed according to the actual report. The document is currently being continuously maintained. If any errors or omissions are found, please provide feedback in a timely manner. Thank you very much. The protocol content is the 4g reporting content. If you need the Bluetooth broadcast data protocol, please consult the relevant contact person.

2 Device Instructions

2.1 Device Function and Instructions

General version:

(1) Power on:

Please fully charge the device before using it for the first time. It will automatically power on when charging. Charging light effect: red light flashes, fully charged light effect: green light is always on.

Manual power on: Press and hold the SOS button for more than 5 seconds.

Power on light effect: red and green lights flash alternately, voice prompt "Welcome to use the intelligent IoT terminal"

(2) Shutdown:

Charging: Automatic shutdown

Manual shutdown: Shutdown: Press and hold the SOS button for more than 15 seconds, the light flashes and then goes off, and the voice prompts "Please pay attention to the device actively shutting down"

Low battery shutdown: The green and red lights flash and then go off

(3) SOS function:

Trigger: After triggering, the device does not enter sleep mode. Press and hold the button for 5s, the red light flashes and then release, the red light is always on, and the voice prompts "Enter the alarm mode" Cancel: When the red light is always on, press and hold the button for 5 seconds, the red light flashes and then release, cancel the SOS alarm, and the voice prompts "Close the alarm mode"

(4) Device status:

Not networked: Short press the SOS button, the light effect is red light Networked: Short press the SOS button, the light effect is green light

Device battery is low: Red light flashes, voice prompts "Device battery is low, please charge in time"

(5) Device sleep:

Trigger condition: The device does not move for 20 minutes and enters sleep mode without reporting data

2.2 Device default reporting logic

General version:

(1) Connection related reporting

F0 request: The device is a short link, under normal server connection and network conditions,

The startup will report once, and then each subsequent report will request a connection--A response is required to connect to the server

F9: Power and signal reporting, a report will follow when location is reported -- requires server reply

(2) Location-related reporting

Satellite (GPS/BDS)/ wifi / Bluetooth beacon: Default reporting frequency 10 minutes, default positioning priority: Bluetooth beacon > wifi > Satellite (GPS/BDS), Bluetooth positioning first, switch to wifi positioning if no positioning, switch to Satellite (GPS/BDS) if no wifi positioning

(3) Alarm-related reporting

SOS Alarm (0x02): User actively triggers, see previous section for triggering method

SOS Cancel (0x02): User actively triggers cancellation, see previous section for triggering method

Shutdown alarm (0x21): Device actively shuts down or low battery shuts down, see previous section for triggering method

Low battery alarm (0x02): Device current battery level is less than or equal to 0 to trigger

(4) Device information and status reporting

Status parameters (0xA9): A record will be reported when the device is turned on

SIM card ICCID (0xF3): Report once when booting up

Device status (0xE9): Report once when booting up, and report once when the reporting frequency changes

Charging status (0xC3): Report when charging starts, charging ends, and when fully charged---report when booting up while charging

The default is charging shutdown, and the charging status is not reported when charging shutdown

(5) Downlink feedback

Downlink feedback (0xC0): The device reports after receiving the downlink command from the server

Note: The device reports in a combined packet, that is, a data packet contains multiple complete messages. Pay attention not to miss any. The message is a complete message, and there will be no interruption in the middle in the next data packet.

Eg:bdbdbdbdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9bdbdbdbdf9010000006400002800000019a9cf61ca

This data packet contains messages for (0xD6) Bluetooth positioning and (0xF9) power signal

(0xD6) Bluetooth positioning: bdbdbdbdd6000119a9cf610445270387bf452708a1bc44279d18b74427e518b7f9

(0xF9) Battery signal: bdbdbdbdf9010000006400002800000019a9cf61ca

2.3 Device Downlink Description

General version: The downlink command needs to be continuously downlinked twice within 20 milliseconds to ensure success. Note: For short-link devices, the downlink command is only received by the device when the device reports. After receiving it, it takes effect in the next reporting cycle.

(1) Device location reporting frequency distribution (0x17):

The default reporting frequency is 10 minutes, and the maximum is 1 minute. After the device receives the downlink command, the device reports according to the time period and frequency of the distribution command.

Report data, report according to the default reporting frequency outside the time period, such as: 00:00-18:00 2 minutes location report,

Then report according to the default 10-minute reporting frequency outside the time period

(2) Device location priority distribution (0xCE01):

Default location priority wifi>Satellite (GPS/BDS), location priority

If the distribution location priority is: wifi>Satellite (GPS/BDS)>Bluetooth beacon, then wifi cannot locate and switch to Satellite (GPS/BDS), Satellite (GPS/BDS)cannot locate and switch

Bluetooth beacon, when the positioning is successful, will not switch to the next positioning priority to generate positioning

(3) Modification of IP and port command issuance (0xC3):

The default general version points to the Smart Cloud Platform: 118.178.184.219:8825. If you need to change it, please consult relevant personnel or visit the official website.

(4) Bluetooth broadcast switch issuance (0xCE05):

The default is the on state, and the device does not perform Bluetooth broadcasting after it is turned off.

(5) Key shutdown switch command issuance (0xCE16):

By default, the device can be shut down via the button. After the downlink is closed, the device cannot be shut down via the button.

(6) Control device to trigger sleep switch issuance (0xCE18):

The default is the on state, and the device enters sleep mode if it does not move for 40 minutes. After the downlink is closed, the device does not enter sleep mode.

(7) Key trigger sos start switch (0xCE19)

The default is the on state, and long pressing the e button can trigger sos. After it is turned off, long pressing the button will not trigger the reported sos.

(8) Control device charging on/off status (0xCE21):

Default charging is off. After the downlink is enabled, the device will not shut down when charging, and the device will report the charging status.

(9) Long connection and short connection mode switching (0xCE22)

The default is short connection. The downlink switches the long connection command. After the device receives it, it restarts and becomes a long connection. Note that the power consumption of the long connection mode will increase. The default state is to report a heartbeat package every 4 minutes (0xF9).

3 Protocol Data Packet Structure

A basic protocol data packet structure is shown in Figure 1:

3.1 Data Header

Each data packet starts with a 4-byte Header or token (in some response messages, timestamp is used instead):

Currently, the token uploaded by Oufu devices is fixed to BDBDBDBD

Header: 0xBD0xBD0xBD0xBD;

Timestamp: 32bits, generated by the server

3.2 Message ID

See Chapter 4 for the content represented by MessageId.

Each time TCP is connected, the device will first report the 0xF0 message, which contains the device's unique identifier IMEI.

The server needs to record the IMEI as an identifier and reply with the 0xf1 message. Only then will the device consider the connection successful, otherwise it will disconnect.

For normal device communication, the long connection reports a 0xF9 heartbeat packet every 4 minutes by default. The 0xF9 heartbeat packet is reported once after the health location is reported.

The short connection reports a 0xF9 heartbeat packet once at startup, and the 0xF9 heartbeat packet is reported once after the health location is reported. The two reports are not affected by each other.

Note that after the server receives the F9 message, the server needs to issue a command to reply, otherwise the connection will be disconnected.

3.3 Token Generation Mechanism

Currently fixed as BDBDBDBD

3.4 Payload

The payload we refer to below refers to the valid body content in the protocol, excluding the head token and verification code. The length of the body is noted in parentheses.

The data format used in the payload is shown in the table below:

[U-unsigned; I-signed; X-bitfield; Number - number of bytes]

The following protocols use little-endian priority except for ch, u8, i8, and x8

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Explanation |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short Integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short Integer |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Integer |
| x16 | Bitfield | 2 | - | - | Bit 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long Integer |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long Integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit long integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating point type |

3.5 Checksum

The checksum includes the payload, as shown in Figure 1. The algorithm is as follows. By default, the general-purpose version of the device does not require checksum verification, and this part can be ignored. Any byte can be used for the downlink instruction.

//Assume content="BDBDBDBDE9000600010A00000A00", then the result of sum.ToString("X2") is 07//The following is the C# code calling method

private string CheckSum(string content)

{

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2");

}

4 Report messages message

4.1 Connection-related reporting

4.1.1 LNK-LIN (MSGID=0xF0) Request connection (TCP only)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xF0 | See below见下方定义 | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | u64 | IMEI | 1 | - | IMEI number (little endian) |
| 2 | x16 | version |  | - | 位域，见下文 |

The device's reported F0 request message must have a server downlink 0XF1 message in response (see the next section for the specific format); otherwise, login will fail.

Example:

Original16hexadecimal message: BDBDBDBDF09B51731BC61603000000C2

BDBDBDBD:4byte message headerHeader

F0:Message ID(Message ID)9B51731BC6160300: IMEI number(little-endian mode), converted to big-endian:000316C61B73519B,

The corresponding decimal is IMEI:869465050010011,0000: version

C2: checksum

Connection-related Q&A:

Each time a new connection is created via TCP, an F0 request is reported first. The F0 contains the IMEI, and then the server records this IMEI.

After that, all the data in this connection belongs to this IMEI.

Note: If you do not receive F0 or experience F0 communication errors, you can use a third-party network testing tool to verify whether server communication is normal.

4.1.2 Connection Reply (MSGID=0xF1) (TCP Only) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Timestamp (unix) | 0xF1 | See below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | u32 | Token | 1 | - | Token: Fixed as BDBDBDBD |

Note that the message order is different from the normal reported message. This message is sent by the server, not the content reported by the device. The server reply needs to be replied in the current channel > Note that the message order is different from the normal reported message. This message is sent by the server, not the content reported by the device. The server reply needs to be replied in the current channel

Example:

Original16hexadecimal message:07FD8860F1BDBDBDBD2E

07FD8860(little endian),Timestamp(unix) timestamp (unit: seconds), converted to big endian6088FD07 converted to decimal:1619590407

Convert to standard time formatUTCTime:2021-04-28 06:13:27

F1:Message ID(Message ID)

BDBDBDBD:Token

2E:checksum

Timestamp: Convert the base 10 to standard time format. It is the result of adding 1619590407 seconds to 1970-01-01 00:00:00. The device uses the timestamp (in seconds) in this message to synchronize the time.

Note: The unit in Java is generally milliseconds. Pay attention to convert to the unit: seconds.

Note: The total number of bytes of the server downlink reply is 10 bytes, such as: 0xBD--1 byte; if the device receives 20 bytes, the connection will not be established.

4.1.3 New Heartbeat Package Protocol (MSGID=0xF9) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xF9 | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery type 0: 4-level system, 1: 5-level system, 2: Percentage, 3: Voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery value If Bat_type is 0: the battery value range is 0-3 (0 is 25%, 3 is 100%) If Bat_type is 1: the battery value range is 0-4 (0 is 20%, 4 is 100%) If Bat_type is 2: the battery value range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: Percentage, 1: 5-level system 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal Strength |
| 1 | U8 | Other_type |  |  | Extended type 0: Full step counting 1: Incremental step counting 2: Vibration |
| 4 | U32 | Num |  |  | Extended value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Note: The F9 heartbeat packet must have a server downlink reply. The device considers the server connection to be unbroken only after receiving the server downlink reply; otherwise, it will re-report the F0 request to connect.

Example:

Original16Hexadecimal message: BDBDBDBDF90104000050000095000000E377BD67AA

BDBDBDBD: Header

F9:Message ID

01:Bat_type battery type 01-->1-->5-level battery (0-4)

0400:Bat_volt battery value converted to big-endian 0004-->4-->Level4corresponds to100% battery

00:Signal_type signal type 00-->0-->percentage

5000:Signal_strength signal strength converted to big-endian0050-->converted to decimal80-->80% signal strength

00:Other_type extension type 00-->0-->full step counting

95000000:Num extension value - step counting converted to big-endian00000095-->converted to decimal149-->number of steps is149

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10base1740470243-->timestamp is1740470243seconds--> converted to standard time format UTC time:2025-02-25 07:57:23-->converted to Beijing time:2025-02-25 15:57:23

AA:checksum

Note: The server receives the F9 heartbeat packet and can reply with a fixed message: Fixed reply example BDBDBDBDF301 Number of bytes:6

4.2 Alarm-related reporting

4.2.1 Alarm Data Upload-1 (MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x02 | See below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below for definition |
| 4 | U32 | Timestamp | - | - | Timestamp (timestamp will be added at the end during retransmission) |

Upl_warn contents

| bit (corresponding to the position of 1 in binary) | Name | Description | Original message in little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Shutdown | Shutdown | 0400 | 0004 | 4 |
| 0 | Low battery | Low battery | 0100 | 0001 | 1 |
| 1 | SOS alarm | SOS alarm | 0200 | 0002 | 2 |
| 7 | SOS canceled | SOS canceled | 8000 | 0080 | 128 |

Example:

Low battery alarm: BDBDBDBD020200E377BD678A

BDBDBDBD:Header

02:Message ID

0100:Upl_warn alarm content, converted to big-endian0001-->Low battery alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8 A: checksum

Shutdown alarm: BDBDBDBD020400E377BD678A

BDBDBDBD:Header

02:Message ID

0400: Upl_warn alarm content, converted to big-endian 0004 --> shutdown alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8 A: checksum

Note: This alarm is used for devices that do not distinguish between shutdown types. See the next section for devices that distinguish between shutdown types.

SOS alarm: BDBDBDBD020200E377BD678A

BDBDBDBD: Header

02: Message ID

0200: Upl_warn alarm content, converted to big-endian 0002 --> SOS alarm

E377BD67: Timestamp Convert to big-endian 67 BD77E3--> Convert to decimal 1740470243 --> The timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8 a: checksum

SOS Cancellation: BDBDBDBD028000E377BD678A

BDBDBDBD: Header

02: Message ID

8000:Upl_warn alarm content, converted to big-endian0080-->SOS cancellation

E377BD67: Timestamp Convert to big-endian 67 BD77E3--> Convert to decimal 1740470243 --> The timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

8 a: checksum

4.2.2 Alarm Data Upload-2 (MSGID=0x21) (Supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x21 | See below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | Alarm type |
| N |  | Upl_warn | - | - | Alarm content See below for different Alarm type type definitions |
| 4 | U32 | Timestamp |  |  | Timestamp (timestamp will be added at the end during retransmission) |

Alarm type=1

Upl_warn contents

| bit (corresponding to the position of 1 in binary) | Name | Description | Original message in little-endian hexadecimal | Big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging shutdown alarm | Automatic shutdown when the device is charging (firmware function support required) | 04000000 | 0004 | 4 |
| 1 | Low battery shutdown alarm | Device shuts down due to low battery (firmware support required) | 02000000 | 0002 | 2 |
| 0 | Proactive shutdown alarm | Device is manually shut down by someone (firmware support required) | 01000000 | 0001 | 1 |

Example:

Charging shutdown alarm: BDBDBDBD21010004000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

04000000:Upl_warn alarm content, converted to big-endian 0004-->charging shutdown alarm

E377BD67:Timestamp converts the timestamp to big-endian 67BD77E3-->converts to decimal 1740470243-->the timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Low battery shutdown alarm: BDBDBDBD21010002000000E377BD6767

BDBDBDBD:Header

21:Message ID message id

0100:Alarm type alarm type Converted to big-endian 0001-->Alarm type=1

02000000:Upl_warn alarm content, converted to big-endian 0002-->Low battery shutdown alarm

E377BD67:Timestamp converts the timestamp to big-endian 67BD77E3-->converts to decimal 1740470243-->the timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

Active shutdown alarm: BDBDBDBD21010001000000E377BD6767

BDBDBDBD:Header

21:Message ID

0100:Alarm type Converted to big-endian 0001-->Alarm type=1

01000000:Upl_warn alarm content, converted to big-endian 0001 --> Active shutdown alarm

E377BD67:Timestamp Converted to big-endian 67BD77E3-->Converted to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

67:checksum

4.3 Location-related reporting

4.3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x03 | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude longitude |
| 8 | Double | lat |  | - | latitude latitude |
| 1 | U8 | north_south |  | - | /N or S/ North or South latitude |
| 1 | U8 | east_west |  | - | /E or W/ East or West longitude |
| 1 | U8 | status |  | - | /A or V/ Valid data or invalid data |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original hexadecimal message: BDBDBDBD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BDBDBDBD:Header

03:Message ID

963E870C3E675E40:lon longitude, convert to big-endian405E673E0C873E96-->longitude:121.6131622 Note: Refer to the parsing example below for conversion and parsing

FF2110C2B6343F40:lat latitude, convert to big-endian `403F34B6C21021FF-->latitude:31.2059137 Note: Refer to the parsing example below for conversion and parsing

4E:north_south 4E-->convert to ASCII code N-->North latitude

45:east_west 45-->convert to ASCII code E-->East longitude

41:status data validity 41-->convert to ASCII code A-->Valid data

E377BD67:Timestamp, convert to big-endian 67BD77E3-->convert to decimal 1740470243-->timestamp is 1740470243 seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

78: Checksum

Note: The parsed GPS latitude and longitude are in the WGS-84 coordinate system. If the map uses Baidu AMAP, etc., the coordinate system needs to be converted.

Parsing satellite positioning message Java code example:

//Message: BDBDBDBD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BDBDBDBD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public static void main(String[] args){

//Message 7d9f84ac81815c40 Actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d",16)));  //114.02353966666665//Message e766926b1d893640 Actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16))) ;//22.535605166666667

HexToStr(data.Substring(“4e”));   //N

HexToStr(data.Substring(“45”));//E

HexToStr(data.Substring(“41”));//A A indicates valid data, V indicates invalid data, which can be discarded//Message 749d695f actual value 5f699d74

Date date = new Date();

date.setTime(Long.parseLong("5f699d74",16)*1000);

SimpleDateFormatsdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.3.2 Upload of WiFi and base station information (MSGID=0xA4)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xA4 | See below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 1 | u8 | Cell_cnt | 1 | - | Number of base stations (0-7) Number of cell info payload.Valid value:0~7 |
| 2 | u16 | Cell[0].MCC | - | - | MMC of base station (0) mobile country code of cell[0] |
| 2 | u16 | Cell[0].MNC | - | - | MNC of base station (0) mobile network code of cell[0] |
| 2 | u16 | Cell[0].LAC | - | - | LAC of base station (0) Location area code of cell[0] |
| 4 | U32 | Cell[0].CELL_ID | - | - | ID of base station (0) Cell id of cell[0] |
| 2 | i16 | Cell[0].RSSI | - | dbm | Signal strength RSSI of base station (0) RSSI in dbm of cell[0] |
| ... | ... | ... | ... | ... | ... |
| 2 | u16 | Cell[cell_cnt-1.MCC | - | - | MMC of base station (N) mobile country code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.MNC | - | - | MNC of base station (N) mobile network code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.LAC | - | - | LAC of base station (N) Location area code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1].LAC | - | - | LAC of base station (N) Location area code of cell[[cell_cnt-1] |
| 4 | U32 | Cell[cell_cnt-1].CELL_ID | - | - | ID of base station (N) Cell id of cell[[cell_cnt-1] |
| 2 | I16 | Cell[cell_cnt-1].RSSI | - | dbm | RSSI signal strength of base station (N) RSSI in dbm of cell[[cell_cnt-1] |
| 1 | U8 | Wifi_cnt |  |  | wifi count Number 0f wifi |
| 6 | 6*U8 | Wifi[0].bssid[0]-Wifi[0].bssid[5] |  |  | MAC address of wifi(0) |
| 4 | I32 | Wifi[0].RSSI |  |  | RSSI of wifi(0) |
| ... | ... | ... | ... | ... | ... |
| 6 | 6*U8 | Wifi[Wifi_cnt-1].bssid[0]-Wifi[Wifi_cnt-1].bssid[5] |  |  | MAC address of wifi(N) |
| 4 | I32 | Wifi[0].RSSI |  |  | RSSI of wifi(0) |

Note: The device report does not necessarily have a base station every time. The absence of a base station does not affect positioning. If the number of base stations in the report is 0, then the number of Wi-Fi connections will follow directly.

Example:

Original16hexadecimal message: BDBDBDBDA4E377BD6701CC010000C218D1AD160BF9FF078CBEBE1A8162C6FFFFFFC061180AF42AC1FFFFFFE005C5B1F824CCFFFFFFE8FCAFA02663AFFFFFFF6409805B2B9CAEFFFFFF94200BC726E000B6FFFFFFA8154DF6517EB2FFFFFFE005C5B1F824CCFFFFFFE8FCAFA02663AFFFFFFF6409805B2B9CAEFFFFFF94

BDBDBDBD:Header

A4: Message ID message id

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to decimal1740470243-->timestamp is1740470243seconds-->

Convert to standard time format UTC time: 2025-02-25 07:57:23-->Convert to Beijing time: 2025-02-25 15:57:23

01: Cell_cnt number of base stations 01 --> number of base stations is 1

CC01: Cell[0].MCC Base station MCC converted to big-endian 01CC--> converted to decimal 460 --> Base station MCC is 460

0000: Cell[0].MNC Base station MNC converted to big-endian 0000 --> converted to decimal 0 --> Base station MNC is 0

C218: Cell[0].LAC Base station LAC converted to big-endian 18C2--> converted to decimal 6338 --> Base station LAC is 6338

D1AD160B: Cell[0].CELL_ID Base station ID converted to big-endian 0B16ADD1--> converted to decimal 186035665 --> Base station id is 186035665

F9FF: Cell[0].RSSI Base station signal strength converted to big-endian FFF9--> converted to decimal complement -7 --> Base station signal strength is -7

07: Wifi_cnt number of wifi connections 07 --> 7 wifi connections

8 CBEBE1A8162: wifi mac address 8 C:BE:BE: 1 A: 81 : 62 -->The mac address of wifi[ 0 ] is 8 C:BE:BE: 1 A: 81 : 62

C6FFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFC6 --> converted to decimal complement -58---> WiFi[0]'s RSSI signal strength is -58

C061180AF42A: WiFi MAC address C0:61:18:0A:F4:2A--> WiFi[1]'s MAC address is C0:61:18:0A:F4:2A

C1FFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFC1 --> converted to decimal complement -63---> WiFi[0]'s RSSI signal strength is -63

200BC726E000: WiFi MAC address 20:0B:C7:26:E0:00--> WiFi[2]'s MAC address is 20:0B:C7:26:E0:00

B6FFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFB6 --> converted to decimal complement -74---> WiFi[0]'s RSSI signal strength is -74

A8154DF6517E: WiFi MAC address A8:15:4D:F6:51:7E--> WiFi[3]'s MAC address is A8:15:4D:F6:51:7E

B2FFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFB2 --> converted to decimal complement -78---> WiFi[0]'s RSSI signal strength is -78

E005C5B1F824: WiFi MAC address E0:05:C5:B1:F8:24--> WiFi[4]'s MAC address is E0:05:C5:B1:F8:24

CCFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFCC --> converted to decimal complement -52---> WiFi[0]'s RSSI signal strength is -52

E8FCAFA02663: WiFi MAC address E8:FC:AF:A0:26:63--> WiFi[5]'s MAC address is E8:FC:AF:A0:26:63

AFFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFFFAF --> converted to decimal complement -81 ----> WiFi [0] RSSI signal strength is -81

6409805 B2B9C: WiFi MAC address 64: 09: 80: 5B: 2B: 9C--> WiFi [6] MAC address is 64: 09: 80: 5B: 2B: 9C

AEFFFFFF: WiFi signal strength RSSI converted to big-endian FFFFFFFFAE--> converted to decimal complement -82 ---> WiFi [0] RSSI signal strength is -82

94: checksum

The parameters parsed from the message need to be connected to the map WiFi database, and the latitude and longitude data is finally obtained through the map side. Take the AMAP connecting to the WiFi database as an example:

According to the access network method, AMAP WiFi positioning protocol document is divided into three positioning scenarios. The following gives corresponding examples for the three positioning scenarios.

1. The network connection method is WiFi access, that is, the accesstype parameter is 1

http://apilocate.amap.com/position?accesstype=1&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&mmac=50:a7:2b:1e:56:58,-60,alibaba-inc&macs=4c:48:da:25:0b:11,-59,alibaba-

inc|4c:48:da:25:1a:11,-77,alibaba-inc&serverip=10.2.166.4&output=xml&key=<用户 Key>

2. When the network connection method is mobile access and the mobile phone card is a non-CDMA card, that is, when the accesstype parameter is 0 and cdma=0:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=0&bts=460,01,40977,2205409,-65&nearbts=460,01,40977,2205409,-65|460,0

1,40977,2205409,-65|460,01,40977,2205409,-65&serverip=10.2.166.4&output=xml&key=<User Key>

3. When the network connection method is mobile access and the mobile phone card is a CDMA card, that is, when the accesstype parameter is 0 and cdma=1:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=1&bts=13824,1,1838,1674723,575739,-52&serverip=10.2.166.4&output=xml&key=<User Key>

W200P watch: AMAP wifi positioning protocol called AMAP interface description: When selecting parameters, generally choose the first type

You can also choose the second type, but if the device does not report the base station, the bts and neabts parameters need to be removed before it can

The first type, example of incoming parameters: mmac is the first WIFI mac address in the messagehttp://apilocate.amap.com/position?accesstype=1&imei=869907057780989&mmac=04:8C:16:A9:FB:A8,-44,TP-Link&macs=04:8C:16:A9:FB:A8,-44,TP-Link|C8:3A:35:F4:CE:18,-59,TP-Link|1C:78:4E:D5:07:FE,-79,TP-Link|C8:3A:35:9D:0B:E0,-87,TP-Link|4A:D8:90:E8:0A:82,-88,TP-Link|AC:AD:4B:C9:C3:87,-89,TP-Link|A4:16:E7:55:84:AC,-91,TP-Link&output=json&key=xxxx

For the specific AMAP wifi positioning protocol PDF, you can click to view and download:(not a screenshot, you can click to view)
This document is provided by AMAP, you can directly ask AMAP for specific docking
Note that the name of the downloaded document will change, you can rename it yourself

4.3.3 Bluetooth Location Information (BLE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xD6 | See below | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 0 |
| U8 | Total_groups | 1 | Total number of groups, there may be multiple sets of information, and each group may contain multiple ibeacons |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| ... | ... | ... | ... |

Note: In the general version, the total number of Bluetooth beacon groups is fixed to 1, and the maximum number of Bluetooth beacons is 4. The sorting is based on the signal strength from strongest to weakest. By default, the watch only recognizes our own beacons. If you need to connect to other manufacturers' beacons, please consult relevant personnel via WeChat. Example:

Original hexadecimal message: BDBDBDBDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

bdbdbdbd: Header

D6: Message ID

00: type

01: Only one set of iBeacon data

E377BD67:Timestamp. Convert to big-endian 67BD77E3 --> Convert to decimal 1740470243 --> The timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

04: The first group has 4 beacon information

4327: Convert to big-endian --> major : 0x2743 --> Convert to decimal 10051

1794: Convert to big-endian --> minor: 0x9417 --> Convert to decimal 37911

AC: RSSI 0xAC-->Convert to decimal complement -84

4327: Convert to big endian --> major: 0x2743 --> Convert to decimal 10051

3094: Convert to big endian --> minor: 0x9430 --> Convert to decimal 37936

AA:  RSSI 0xAA --> Convert to decimal complement -86

4327:Convert to big endian --> major: 0x2743 --> Convert to decimal 10051

b956:Convert to big endian --> minor: 0x56b9 --> Convert to decimal 22201

A5:   RSSI 0xA5 --> Convert to decimal complement -91

4327: Convert to big endian --> major: 0x2743 --> Convert to decimal 10051

FE94: Convert to big endian --> minor: 0x94FE --> Convert to decimal 38142

A5: rssi:0xA5 --> Convert to decimal complement -91

6a -- Checksum

Bluetooth beacon major, minor description: The default shipping Bluetooth beacon has a beacon code on it,

Like: 00081005345866, then the major of the Bluetooth beacon is 10053, the minor is 45866, and the manufacturer ID is 0008 (this is useless and has no practical significance)

4.4 Device Information and Status Reporting

4.4.1 ICCID Upload of SIM Card (MSGID=0xF3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xF3 | See below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 10 | 10*U8 | ICCID | 1 | - | ICCID number - ICCID number of the device SIM card |

Note: This message will be reported once every time the device is powered on.

Example:

Original hexadecimal message:BDBDBDBDF389861118236001639994CC

BDBDBDBD:Header

F3:Message ID

89861118236001639994:ICCID Device SIM card number ICCID is: 89861118236001639994

CC:checksum

4.4.2 Device Charging Status Upload (MSGID=0xC3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Status | / | / | 0 start, 1 end, 2 fully charged |
| 1 | U32 | Timestamp | / | / | Timestamp (timestamp will be added at the end during retransmission) |

Note: This message requires device firmware support. The latest general version supports it by default. This message will only be reported when the downlink command is enabled: charging and booting.
Example:

Original hexadecimal message:BDBDBDBDC301E377BD678A

BDBDBDBD:Header

C3:Message ID

01:Device finished charging

E377BD67:Timestamp. Convert to big-endian 67BD77E3 --> Convert to decimal 1740470243 --> The timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

Original hexadecimal message:BDBDBDBDC300DB4D2F668A

BDBDBDBD:Header

C3:Message ID

00:Device starts charging

E377BD67:Timestamp. Convert to big-endian 67BD77E3 --> Convert to decimal 1740470243 --> The timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

Original hexadecimal message:BDBDBDBDC302DB4D2F668A

BDBDBDBD:Header

C3:Message ID

02:Device is fully charged

E377BD67:Timestamp. Convert to big-endian 67BD77E3 --> Convert to decimal 1740470243 --> The timestamp is 1740470243 seconds -->

Convert to standard time format UTC time: 2025-02-25 07:57:23 --> Convert to Beijing time: 2025-02-25 15:57:23

8A:checksum

4.4.3 Status parameter reporting (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Number of parameter types |
| 1 |  |  |  |  | (Reserved 00) |
| 1 | u8 | Type | -/- | -/- | Parameter type 1, Type definition: 00—MCU (firmware), 01—module, 02 sensor (general version does not have this type) |
| 1 | U8 | NameLen | -/- | -/- | Parameter Name 1 Length |
| n | N*u8 | Name | -/- | -/- | Parameter Name |
| 1 | u8 | Type | -/- | -/- | Parameter Type 2, Type Definition: 00—MCU (Firmware), 01—Module, 02 Sensor (General Version does not have this type) |
| 1 | U8 | NameLen | -/- | -/- | Parameter Type 2 Length |
| n | N*u8 | Name | -/- | -/- | Parameter Name |
| ... | ... | ... | ... | ... | ... |

Note: This message is reported once by default when the device is powered on, uploading the device firmware version number and module version number, which can be left unparsed. The Message ID of the previously shipped device was 0xBB, and relevant personnel can be contacted to send it.

Example:

BDBDBDBDA902000009423732352E4F56303601174E5432364B434E4230304E4E412D4C3032303330393530B5

BDBDBDBD:Header

A9:Message ID

02:TypeCnt indicates that there are 2 types of parameters

00:Reserved field

00:Type=00 00 represents MCU (firmware)

09:The following MCU name length is 9 bytes

423732352E4F563036--->Hexadecimal to text (device firmware version number)--->B725.OV06

01:Type=01 01 represents the module

17:The following communication module name length is 23 bytes

4E5432364B434E4230304E4E412D4C3032303330393530:Hexadecimal to text (4G module version number)

--->NT26KCNB00NNA-L02030950

B5---checksum

4.4.4 Device Status (MSGID=0xE9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xE9 | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | type |  | / | Default—00 |
| 2 | u16 | length |  | / | Length of the following message |
| 1 | u8 | location |  | / | Location reporting frequency: 00—-default not modified, 01—-modified by downlink (if there are multiple time periods in the downlink, only the reporting frequency of the current time period is reported) |
| 2 | u16 | frequency |  | / | Frequency, unit - minutes |
| 1 | u8 | health |  | / | Health reporting frequency: 00 - Default not modified, 01 - Modified by downlink |
| 2 | u16 | frequency |  | / | Frequency: Unit - minutes |

Note: This message is reported once at startup, and once after the downlink modifies the frequency. The latest general version has it, but the previous version does not.

Example:

Original hexadecimal message:BDBDBDBDE9000600010A00010A0033

BDBDBDBD:Header

E9:Message ID

00:type=00

0600:length The length of the message behind it is converted to big-endian --> 0006 --> converted to decimal 6, and the message length is 6 bytes

01:location 01 indicates that the location reporting frequency has been modified by the downlink

0A00:frequency location reporting frequency converted to big-endian --> 000A --> converted to decimal 10, the reporting frequency is 10 minutes

01:health 01 indicates that the health reporting frequency has been modified by the downlink

0A00:frequency Health reporting frequency converted to big-endian --> 000A --> converted to decimal 10, the reporting frequency is 10 minutes

33:checksum

If the device does not have a health reporting function or a location reporting function, the two values of health reporting frequency and location reporting frequency will remain the same, indicating the device reporting frequency.

4.5 Downlink Feedback Related Reporting

4.5.1 Downlink Feedback (MSGID=0xC0)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xC0 | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID Length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This command is used for feedback of downlink commands, returning the Message ID of the downlink command received earlier (multiple Message IDs can be returned collectively).

Original Hexadecimal Message:BDBDBDBDC0011720

BDBDBDBD:Header

C0:Message ID

01:length Message ID length Convert decimal 01--> Message ID length is 1 byte

17Type Indicates that the device receives the downlink command Message ID is 17

20:checksum

5 Settings

5.1 Downlink

5.1.1 Set the location reporting frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0x17 | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | - Hour |  |
| 1 | u8 | time_start_m |  |  | - Minute |  |
| 1 | u8 | time_end_h |  |  | - Hour |  |
| 1 | u8 | time_end_m |  |  | - Minute |  |

Note: One day is 00:00-23:59
Example:

00:00-23:59 Report location every10minutes: BDBDBDBD17010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

17:Message ID

01:enable is enabled or not Convert to decimal01-->1--Enabled

0A00:Interval time interval Convert to big-endian000A-->Convert to decimal10-->Time interval is10minutes

00:time_start_h Start time-hour, convert to decimal00-->Start time-hour is00

00:time_start_m Start time-minute, convert to decimal00-->Start time-minute is00

17:time_end_h End time-hour, convert to decimal23-->End time-hour is 23

3B:time_end_m End time-minute, convert to decimal59-->End time-minute is 59

00: enable whether to enable Convert to decimal00-->0--Not enabled

000000000000: All filled in disabled state 00

00: enable whether to enable Convert to decimal00-->0--Not enabled

000000000000: All filled in disabled state 00

00: enable whether to enable Convert to decimal00-->0--Not enabled

000000000000: All filled in disabled state 00

97: checksum

5.1.2 Settings - Positioning Priority Settings (0XCE01 & 0XCE02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xCE | See below | CK_sum |

Payload contents

Type=01 Positioning priority setting

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 is always valid, 01 is effective this time |
| 2 | U16 | Len | -/- | -/- | Length of the following instruction |
| n | n |  | -/- | -/- | Body |

Location priority setting definition -- See 2.3 Device Downlink Description -- Device Location Priority Distribution (0xCE01)

Location priority setting body:

Currently, the general version supports: 01 -- Satellite (GPS/BDS), 02 -- wifi (wifi + base station) 03 -- Bluetooth beacon (requires additional deployment of Bluetooth beacons),

Special equipment support: 04 -- LBS base station 05--125k

Example:

Wifi positioning priority (wifi>Bluetooth beacon>Satellite (GPS/BDS)) : BDBDBDBDCE0100030002030133

BDBDBDBD:Header

CE : Message ID

01 : Type 01 Location priority setting

00 : Valid Validity 00 Always valid

0300 : Len Converted to big-endian 0003-->Convert to decimal 03-->The body length is 3 bytes

020301:Text: 02--wifi 03--Bluetooth beacon 01--Satellite (GPS/BDS) The positioning priority is wifi>Bluetooth beacon>Satellite (GPS/BDS)

33:checksum

Satellite (GPS/BDS)positioning priority (Satellite (GPS/BDS)>wifi>Bluetooth beacon):BDBDBDBDCE0100030001020333

BDBDBDBD:Header

CE : Message ID

01 : Type 01 Location priority setting

00 : Valid Validity 00 Always valid

0300 : Len Converted to big-endian 0003-->Convert to decimal 03-->The body length is 3 bytes

010203:Text: 01--Satellite (GPS/BDS) 02--wifi 03--Bluetooth beacon Positioning priority is Satellite (GPS/BDS)>wifi>Bluetooth beacon

33:checksum

Bluetooth positioning priority (Bluetooth beacon>wifi>Satellite (GPS/BDS)):BDBDBDBDCE0100030003020133

BDBDBDBD:Header

CE : Message ID

01 : Type 01 Location priority setting

00 : Valid Validity 00 Always valid

0300 : Len Converted to big-endian 0003-->Convert to decimal 03-->The body length is 3 bytes

030201:Text: 03--Bluetooth beacon 02--wifi 01--Satellite (GPS/BDS) Positioning priority is Bluetooth beacon>wifi>Satellite (GPS/BDS)

33:checksum

Single Satellite (GPS/BDS) positioning:BDBDBDBDCE0100030001000033

BDBDBDBD:Header

CE : Message ID

01 : Type 01 Location priority setting

00 : Valid Validity 00 Always valid

0300 : Len Converted to big-endian 0003-->Convert to decimal 03-->The body length is 3 bytes

030201:Text: 01--Satellite (GPS/BDS)  00--No positioning 00--No positioning Positioning priority is single Satellite (GPS/BDS) positioning

33:checksum

Single wifi positioning:BDBDBDBDCE0100030002000033

BDBDBDBD:Header

CE : Message ID

01 : Type 01 Location priority setting

00 : Valid Validity 00 Always valid

0300 : Len Converted to big-endian 0003-->Convert to decimal 03-->The body length is 3 bytes

020000:Text: 02--wifi 00--No positioning 00--No positioning Positioning priority is single wifi positioning

33:checksum

Single Bluetooth beacon positioning:BDBDBDBDCE0100030003000033

BDBDBDBD:Header

CE : Message ID

01 : Type 01 Location priority setting

00 : Valid Validity 00 Always valid

0300 : Len Converted to big-endian 0003-->Convert to decimal 03-->The body length is 3 bytes

020000:Text: 03--Bluetooth beacon 00--No positioning 00--No positioning Positioning priority is single Bluetooth beacon positioning

33:checksum

5.1.3 Settings - Switch Settings (0XCE04-24)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xCE | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Please refer to the instructions for specific types |
| 1 | u8 | switch | -/- | -/- | Switch 00 on, 02 off, 01 single effective |
| 2 | U16 |  | -/- | -/- | Fixed 0000 |

Device alarm settings - switch type: specific function description--see 2.3 Device downlink description

05 Bluetooth broadcast switch

16 Whether the downlink can use the button to shut down

18 Sleep function switch

19 Key trigger SOS alarm switch

21 Is charging on or off?

22 Short and long connection mode switching

Type=05 Bluetooth broadcast switch
Example:

00---On:BDBDBDBDCE0500000093

BDBDBDBD:Header

CE : Message ID

05:Type 05 Bluetooth broadcast switch

00:switch on 00 on

0000:Fixed value

93:checksum

02---Off:BDBDBDBDCE0502000093

BDBDBDBD:Header

CE : Message ID

05:Type 05 Bluetooth broadcast switch

02:switch off 02 off

0000:Fixed value

93:checksum

Type=16 Can the downlink use the key to shut down
Example:

00---Open:BDBDBDBDCE1600000093

BDBDBDBD:Header

CE : Message ID

16:Type 16 Can the downlink use the button to shut down?

02:switch switch 00 Open

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE1602000093

BDBDBDBD:Header

CE : Message ID

16:Type 16 Can the downlink use the button to shut down?

02:switch off 02 off

0000:Fixed value

93:checksum

Type=18 Sleep function switch
Example:

00---Open, enter sleep mode after being still for a period of time, and do not report data:BDBDBDBDCE1800000093

BDBDBDBD:Header

CE : Message ID

18:Type 18 Sleep function switch

02:switch switch 00 Open

0000:Fixed value

93:checksum

02---Close, Reports data normally in a static or charging state:BDBDBDBDCE1802000093

BDBDBDBD:Header

CE : Message ID

18:Type 18 Sleep function switch

02:switch off 02 off

0000:Fixed value

93:checksum

Type=19 Key triggers SOS alarm switch
Example:

00---Open:BDBDBDBDCE1900000093

BDBDBDBD:Header

CE : Message ID

19:Type 19 key triggers SOS alarm switch

02:switch switch 00 Open

0000:Fixed value

93:checksum

02---Close:BDBDBDBDCE1902000093

BDBDBDBD:Header

CE : Message ID

19:Type 19 key triggers SOS alarm switch

02:switch off 02 off

0000:Fixed value

93:checksum

Type=21 Charging is power on or power off
Example:

00---Charging is power on:BDBDBDBDCE2100000093

BDBDBDBD:Header

CE : Message ID

21:Type 21 Charging power on or charging power off

02:switch switch 00 Charging is power on

0000:Fixed value

93:checksum

02---Charging is power off:BDBDBDBDCE2102000093

BDBDBDBD:Header

CE : Message ID

21:Type 21 Charging power on or charging power off

02:switch switch 02 Charging is power off

0000:Fixed value

93:checksum

Type=22 Long and short connection mode switching
Example:

00---Long connection mode, default is long connection:BDBDBDBDCE2000000093

BDBDBDBD:Header

CE : Message ID

22:Type 22 Long and short connection mode switching

02:switch switch 00 Enable long connection mode

0000:Fixed value

93:checksum

02---Short connection mode:BDBDBDBDCE2002000093

BDBDBDBD:Header

CE : Message ID

22:Type 22 Long and short connection mode switching

02:switch switch 02 Turn off long connection mode--short connection mode

0000:Fixed value

93:checksum

5.1.4 IP&Domain Name Settings (0xC3) (TCP Dedicated)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed at 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | Issue type: Type =1 IPv4, Type =2 IPv6 (Reserved item - not supported yet), Type =3 Domain name (Only supports ascii encoded domain names for now) |
| 2 | U16 | Port |  |  | Port number (2 bytes) |
| 1 | U8 | Len |  |  | Length of content of subsequent parameters |
| N | U8 | Domainname |  |  | Specific IP or domain name content |

Note: After distribution, the device will not communicate on the original server, but will point to the modified server.

Example:

Type=1 IPv4:Downlink modifies the IP to:118.178.184.219, and the port is: 8825 BDBDBDBDC30179220476B2B8DB33

BDBDBDBD:Header

C3:Message ID

01:Type 1--IPv4

7922:Port Convert the port to big-endian 2279-->Convert to decimal 8825-->Port: 8825

04:Len The length of the following parameter content is converted to decimal 04-->The length is 4 bytes

76B2B8DB:Domainname IP 76-->Convert to decimal 118, B2-->Convert to decimal 178, B8-->Convert to decimal 184, DB-->Convert to decimal 219 IP is: 118.178.184.219

33:checksum

Type=3 Domain nameDownlink modifies the domain name to:aiday.com.cn, and the port is: 8825

BDBDBDBD:Header

C3:Message ID

03:type=3, domain name

7922:Port Convert the port to big-endian--2279 -->Convert to decimal Port: 8825

12:Len followed by parameter content length, converted to decimal 18 --> message length is 18 bytes

61696461792e636f6d2e636e:Domainname converted to string --> aiday.com.cn --> domain name is: aiday.com.cn

1D:checksum