W300PGW-4G CAT.1+WIFI Data Transmission - TCP Communication Protocol

Table of Contents

1 Overview	4

2 Device Usage Instructions	4

2.1 Device Functions and Usage Instructions	4

2.2 Default Reporting Logic of the Device	7

2.3 Device Downlink Description	9

3 Protocol Data Packet Structure	12

3.1 Data Header	12

3.2 Message Identifier (Message ID)	13

3.3 Token Generation Mechanism	13

3.4 Payload	13

3.5 Checksum	14

4 Report messages	16

4.1 Connection related report	16

4.1.1 LNK-LIN (MSGID=0xF0) request connection (TCP only)	16

4.1.2 Connection reply (MSGID=0xF1) (TCP specific) - Important	17

4.1.3 New Heartbeat Packet Protocol (MSGID=0xF9) - Important	18

4.2 Alarm-related reporting	20

4.2.1 Alarm data upload -1 (MSGID=0x02)	20

4.2.2 Alarm Data Upload-2 (MSGID=0x21) (Supplement to 02)	23

4.2.3 Exception Data Alarm (MSGID=0x16)	27

4.3 Location-related Report	29

4.3.1 GPS/BDS Location Report: Positioning data report (MSGID=0x03)	29

4.3.2 Wifi and Base Station Information Upload (MSGID=0xA4)	30

4.3.3 Bluetooth positioning information (BLE Location) (MsgId=0xD6)	35

4.4 Device Information and Status Reporting	37

4.4.1 Device Charging Status Upload (MSGID=0xC3)	37

4.4.2 Status Parameter Reporting (MSGID=0xA9)	39

4.4.3 Device Status (MSGID=0xE9)	40

4.5 Health-related Reporting	42

4.5.1 Health Data (MSGID=0x32)	42

4.5.2 Device sleep analysis data upload (MSGID=0xC5)	44

4.6 Downlink feedback related reporting	45

4.6.1 Downlink feedback (MSGID=0xC0)	45

4.6.2 Message status report (MSGID=0x28)	46

5 Settings	48

5.1 Downlink	48

5.1.1 Set Position Reporting Frequency (MSGID=0x17)	48

5.1.2 Message Sending (MSGID=0X28)	50

5.1.3 Settings - Positioning Priority Settings (0XCE01)	51

5.1.4 Settings - Health sampling reporting frequency setting (MSGID=0X1C)	54

5.1.5 Settings - Device Alarm Settings (0XCE03)	56

5.1.6 Settings - Switch Settings (0XCE04-24)	58

5.1.7 Settings - Drop Sensitivity Settings (0xCE15)	65

5.1.8 Settings - Vibration Settings (0XCE23)	67

5.1.9 IP & Domain Name Settings (MSGID=0xC3) (TCP specific)	68

5.1.10 Issuing Sedentary Stay Alarm Trigger Time (MSGID=0XCC)	69

5.1.11 Power Off and Restart (MSGID=0x77)	70

5.1.12 Personal Information Transmission (MSGID=0xCA)	71

5.1.13 Sleep Statistics Time Period Setting (MSGID=0X1D)	73

5.1.14 Remote OTA Upgrade Issuance (MSGID=0xA9)	75

5.1.15 Weather Warning (MSGID=0XCB)	76

5.1.16 Restore Factory Settings (MSGID=0X7B)	80

1 Overview

This protocol is suitable for 4G+ WiFi devices and supports the W300PGW product. It uses a 32-bit data header for synchronization and terminal identification; a low-overhead checksum algorithm is employed for validation protection; message identifiers are used to signify different messages.  
The parsing should be based on the actual reporting. The document is currently under continuous maintenance, and any errors or omissions found should be promptly reported. Thank you very much.  
The protocol content pertains to TCP reporting. For Bluetooth broadcast data protocols, please consult the relevant contact personnel.

2 Device Usage Instructions

2.1 Device Functions and Usage Instructions

General Version:

(1) Power On:

Before the first use, please charge the device fully. It will automatically power on when the battery reaches the required charge. The charging status will display a charging icon,

with a fully charged indicator showing a green icon. Note: Do not check the device signal while it is charging.

Manual Power On: Long press the upper button for 10 seconds and then release it. The interface will display the word "Welcome." Note: The default power-on state is in wear mode, and if no heart rate is detected, it will report a disconnection alarm.

(2) Power Off:

Low Battery Power Off: The screen turns off after displaying "Byebye."

Manual Power Off: Long press the button for more than 10 seconds, the screen displays "Byebye" and then turns off.

Remote Power Off Command: While in the powered-on state, executing a shutdown command will result in the screen displaying "Byebye" before turning off.

Note: Charging while powered on does not turn off the device.

(3) SOS Function:

Trigger Method: Once triggered, the device will not enter sleep mode. Long press the down button for 3 seconds, the screen displays "SOS SEND OK" and then the SOS icon appears.

Cancel Method: In SOS mode, long press the down button for 3 seconds, the screen displays "SEND CANCEL/SOS Cancelled," and the SOS icon disappears.

(4) Signal Status:

Leftmost Signal: No 4G network or not using 4G: the signal displays a horizontal line. There is a signal or it is in use: the signal shows a stepped bar graph.

Second signal:

Not connected to WiFi or not using WiFi: The interface signal shows a horizontal line. When there is a signal or in use: The interface signal displays a WiFi icon.

(5) Device Sleep:

Trigger condition: The device remains stationary for 40 minutes, entering sleep mode, and does not report location health data, only reporting F9 heartbeat packets to maintain a long connection.

In wearing status, the device reports location and health data; in falling status, the device does not report.

(6) Device Parameters: Settings - Parameters interface: There are multiple pages, which can be switched by scrolling down the screen.

sos (Yes): Yes: Indicates that the device can trigger sos by pressing the key.

sos (No): Indicates that sos cannot be triggered by pressing the key.

Data Reporting (Yes): Indicates that data will be reported currently.

Allow Work Sleep (Yes):

Yes: Indicates that the device has activated the sleep function; No: Indicates that the device has deactivated the sleep function.

Note: The sleep function refers to the state where the device does not report data when it is in a dropped state. The device remains stationary for 40 minutes without reporting data.

Health (10 min): Indicates that the device has enabled health reporting function.

10 min: Indicates that the current health data reporting frequency is 10 minutes.

Health (No): Indicates that the device health reporting function has been turned off.

Location (B, 10): Indicates that the device has enabled location reporting function.

B: Indicates that the location priority is single Bluetooth positioning.

10: Indicates that the current location reporting frequency is 10 minutes.

Location (No): Indicates that the device location reporting function has been turned off.

Drop (Low, 1.5 m): Indicates that the device has enabled drop alarm trigger function.

Low: Indicates that the sensitivity for triggering the drop alarm is low.

1.5m: Indicates that the fall alarm of the device is triggered at a height of 1.5m.

Drop (No): Indicates that the device has disabled the drop alarm trigger function.

Sitting for a long time (15 min): Indicates that the device has enabled the long sitting alarm trigger function.

15 min: Indicates that the time for triggering the long sitting alarm is 15 minutes of inactivity.

Sitting for a long time (No): Indicates that the device has disabled the long sitting alarm trigger function.

Health Broadcast (Yes):

Yes: Indicates that the device has enabled Bluetooth broadcasting of health data.

No: Indicates that the device has disabled Bluetooth broadcasting of health data.

Sleep Statistics Reporting Time: (21:00 - 08:00), indicates that the sleep statistics reporting time is from 21:00 to 08:00 the next day.

Global Vibration Duration (1.500s): The current vibration duration of the device is 1.5 seconds.

Health Alarm Threshold: For example, heart rate threshold (90, 100) BPM, indicates that the normal range for heart rate is 90-100.

Health anomalies outside the normal range will trigger an alert, heart rate threshold (No): indicates that the heart rate threshold is turned off.

Network long connection (Yes):

Yes: indicates that the current device is in TCP connection mode: long connection (maintains a continuous connection state, allowing real-time receipt of downstream instructions).

Satellite always-on collection switch (Yes):

Yes: the internal interface continuously collects satellite (GPS/BDS) data, which can generally accelerate satellite (GPS/BDS) positioning time in normal environments, but power consumption will increase.

By default, this is set to No, and in normal environments, it does not need to be turned on.

(8) Vibration:

When receiving a downstream text message, a health threshold alert is activated (the interface shows a text reminder and vibrates).

The duration of the vibration can be controlled through downstream settings (see section 2.3).

(9) Module settings: Settings - Module settings interface.

You can set the WiFi and 4G mode in this interface.

Setting method: Click to select one option and wait for a restart to switch.

wifi and 4G: Prefer 4G communication for data transmission. If there is no 4G network, check for wifi; if available, use wifi until it has no connection, then check for 4G.

In this mode, the signal displayed on the interface is the one being used.

Wifi only: Even if the wifi cannot connect to the router, it will not switch to 4G.

4G only mode: Even if there is no 4G signal, it will not switch to wifi.

(10) Interface Switching

The default startup screen is the time interface. Press the down key to switch to the menu interface.

In screen-off status, press the up key to light up the screen.

When in the health interface (heart rate, blood pressure, body temperature, blood oxygen), press the up key or tap the touchscreen to start health sampling.

2.2 Default Reporting Logic of the Device

General Version:

(1) Connection-related reporting

F0 request: The device is a long connection; under normal server connection and network conditions, it will report once upon startup -- a response is needed to connect with the server

F9: Heartbeat report, defaults to reporting once every 4 minutes. (In previous firmware) when location and health are reported, it will also report once -- a response is needed to maintain the long connection

(2) Location-related reporting

Satellite (GPS/BDS)/wifi/Bluetooth beacon: Default reporting frequency is 10 minutes (in wearing state), default location priority: Bluetooth > wifi location > Satellite (GPS/BDS) location

Bluetooth beacon > wifi > Satellite (GPS/BDS), Bluetooth beacon location has the highest priority; if unable to locate, switch to wifi location, if wifi location is also unavailable, switch to Satellite (GPS/BDS) location

(3) Alarm-related reporting

SOS alarm (0 x02): Triggered by the user actively; the triggering method is detailed in the previous section

SOS cancellation (0 x02): Triggered by the user actively canceling; the triggering method is detailed in the previous section

Shutdown alarm (0 x21): Activated by the device due to power off or low battery shutdown; the triggering method is detailed in the previous section

Wearing loss alarm (0 x02): The device determines wearing status based on the health sampling reporting frequency. When heart rate is detected, it reports wearing alarm; when heart rate is not detected, it reports falling off alarm.

Prolonged Sitting Alert (0 x02): Default 15 minutes of inactivity triggers a report (wearing status)

Fall Alert (0 x02): Device free falls from a certain height, triggering the fall algorithm

Low Battery Alert (0 x02): Triggered when the current battery level is less than or equal to 0

Abnormal Temperature Alert (0 x16): Reports an abnormal value when the device detects a temperature exceeding 40 degrees Celsius (the value has no reference significance)

Health Threshold Alert (0 x2105): Default not to alert; when falling below the health threshold, health data is reported as out of range

Note: When the health threshold alert is reported, the device vibrates, and the interface will display the abnormal value

(4) Health-Related Reporting

Step count, heart rate, body temperature & wrist temperature, blood pressure, blood oxygen (0 x32): Default reporting frequency 10 minutes (wearing status)

Sleep (0 xC5): Statistics time period 21:00-08:00; sleep data will be reported based on status during this time period

(5) Device Information and Status Reporting

SIM card ICCID (0 xF3): report one piece on startup

Status parameters (0 xA9): report one piece on startup

Device status (0 xE9): report one piece on startup, and report one piece when the reporting frequency changes

Charging status (0 xC3): report when charging starts, ends, and when fully charged

(6) Downlink feedback

Downlink feedback (0xC0): report when the device receives the server's downlink command

Message status feedback (0x28): after receiving a downlink text message, the device clicks accept or reject

Note: The device may report a bundled report situation, meaning that one data packet contains multiple complete messages. Be careful not to miss any; the messages are complete and will not be interrupted in the next data packet.

Eg: BDBDBDBDD6000119A9CF610445270387BF452708A1BC44279D18B74427E518B7F9BDBDBDBDF9010000006400002800000019A9CF61CA

This data packet contains (0xD6) Bluetooth positioning and (0xF9) battery signal messages.

(0xD6) Bluetooth positioning: BDBDBDBDD6000119A9CF610445270387BF452708A1BC44279D18B74427E518B7F9

(0xF9) Power Signal: BDBDBDBDF9010000006400002800000019A9CF61CA

2.3 Device Downlink Description

General Version: The downlink command requires two continuous downlinks within 20 milliseconds to ensure success.

(1) Device Location Reporting Frequency Command (0x17):

Default reporting frequency 10 minutes (wearing status), default location priority: Bluetooth > wifi location > satellite (GPS/BDS) location, maximum 1 minute, minimum 1440 minutes. After the downlink command is received, the device will report data according to the commanded time period and frequency.

Data will not be reported outside the time period, for example: 00:00-18:00, 2 minutes location report, then no report outside this time period.

(2) Text Message Command (0x28):

Unicode encoding, up to 40 Chinese characters, one Chinese character occupies 2 bytes, one English letter occupies 2 bytes.

Message List - Up to 50 historical messages saved (Note: Firmware without a message list can store up to 20 messages).

(3) Device Location Priority Command (0xCE01):

Default positioning priority: Bluetooth > Wifi > Satellite (GPS/BDS). Note: Indoor positioning does not reach satellites (GPS/BDS).

(4) Device health sampling frequency issuance (0x1C): — Note that it is different from W200P

Healthy sampling frequency default 10 minutes (wearing status), maximum 2 minutes. After the device receives the reported downlink command, the device acts according to the issued instruction.

Data reporting for time segments and frequency; no reporting outside the time segments. For example: 00:00-18:00, with 10-minute positioning reporting, means no reporting outside this time segment.

(5) Modify IP and port command issued (0xC3):

The default generic version points to the Smart Cloud Platform: 118.178.184.219:8825. If you need to change it, please consult the relevant liaison personnel or visit the official website.

(6) Sitting duration alarm trigger time issued (0xCC):

Value range: 2 minutes - 60 minutes. For example, if the following is a trigger time of 10 minutes, the device will trigger a sedentary alert if it remains inactive for 10 minutes.

(7) Device shutdown and restart command (0x77):

Note that this instruction will only take effect when the device is powered on and cannot be received when it is powered off.

Shutdown command: Under the power-on state, executing this command will turn off the device, and it cannot receive further commands after shutdown.

Reboot command: Sending this command while the device is powered on will restart the device.

(8) Drop sensitivity and height settings (0xCE15):

The latest general firmware default drop sensitivity: low (medium-low). Drop height: 1.5m.

Sensitivity: Refers to the level meeting the drop algorithm, with 5 setting levels (0 - 4): low - medium-low - medium - medium-high - high.

Height: Refers to the height required to trigger the drop alarm, with 5 setting levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m - 2.5m.

(9) Remote OTA settings (0xA9):

Remote updates consume data and are affected by network conditions. The device's SIM card has a limit of 30MB per month, so updates cannot be too frequent,

otherwise, it may cause insufficient data. This feature requires hardware support for remote updates; older devices do not support it.

(10) Power off command via button settings (0xCE16):

By default, the device can be powered off using the button. After sending the command to turn off, the device cannot be powered off using the button.

(11) Control device trigger sleep switch issued (0xCE18):

Default is in the on state; the device enters sleep mode after 40 minutes of inactivity. After being turned off, the device will not enter sleep mode.

(12) Drop alarm switch (0xCE07):

Default drop alarm is in the on state; after being turned off, the device will not report drop alarms.

(13) Key-triggered SOS activation switch (0xCE19)

Default is in the on state; long-pressing the charging cable button can trigger SOS. After being turned off, long-pressing the charging cable button will not trigger SOS reporting.

(14) Device health and positioning report switch (0xCE20)

Default health reporting is in the on state, and default positioning reporting is in the on state.

You can separately set the on and off status for positioning/health reporting; after being turned off, the device will not report health/positioning.

(15) Health threshold setting (0xCE0300)

Health threshold issued, downward instruction. After the device receives it, the issued health threshold is within the normal range. Health data outside this range will trigger an alarm.

(16) Device Vibration Setting (0xCE23)

After receiving the downlink command, the device's vibration duration will become the duration specified in the command, ranging from 0 to 60 seconds.

(17) Immediate Location Reporting (0xCE0A)

After receiving the downlink command, the device will immediately start locating and reporting.

(18) Immediate Health Reporting (0xCE0B)

After receiving the downlink command, the device will immediately start health sampling and reporting.

(19) Sleep Statistics Time Period Dispatch (0x1D)

Different time periods can be set for dispatch, for example: 13:00-14:00, during which sleep statistics will be collected and reported.

(20) Personal Information Dispatch (0xCA)

By default, there is no personal information; downlink can include Chinese name, English name, blood type, contact number, and GB2312 encoding.

The maximum byte size: Name 40 bytes, English name 40 bytes, phone 40 bytes, blood type 10 bytes

(21) Sedentary alert switch (0xCE08)

Enabled by default, trigger time is 15 minutes. After going down, the device will not report the sedentary alert.

(22) Weather warning issuance (0xCB)

Multiple sets of weather warnings can be issued. Note that each set of weather warnings must be different and cannot contain the same type of warning.

(23) Restore factory settings (0x7B):

After issuance, the device restores to the default reporting state

3 Protocol Data Packet Structure

A basic protocol data packet structure is shown in Figure 1:

3.1 Data Header

Each data packet begins with a 4-byte header or token (in some response messages, the timestamp is used instead):

The token for the Eufo device is currently fixed as BDBDBDBD

Header    :    0xBD 0xBD 0xBD 0xBD;

Timestamp    :    32 bits, generated by the server

3.2 Message Identifier (Message ID)

The content represented by MessgeId is detailed in Chapter 4.

Each time a TCP connection is established, the device first reports a 0xF0 message, which contains the unique identifier IMEI of the device.

The server needs to record the IMEI as an identifier and respond with a 0xf1 message; only then will the device consider the connection successful, otherwise it will disconnect.

For normal communication, the long connection defaults to reporting a 0xF9 heartbeat packet every 4 minutes, and a 0xF9 heartbeat packet will be reported after the health positioning report.

The short connection reports a 0xF9 heartbeat packet once when powered on, and a 0xF9 heartbeat packet will be reported after the health positioning report; the two reports do not affect each other.

Note that after the server receives the F9 message, it needs to send back an instruction response; otherwise, the connection will be disconnected.

3.3 Token Generation Mechanism

The fixed value is currently BDBDBDBD

3.4 Payload

In the following context, the payload refers to the valid body content of the protocol, excluding the head token and checksum. The length of the body is noted afterward.

The data formats used in the payload are as shown in the table below:

[U-unsigned; I-signed; X-bitfield; Number - bytes occupied]

In the following protocol, except for ch, u8, i8, x8, all are in little-endian format.

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Description |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short Integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short Integer |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Integer |
| x16 | Bitfield | 2 | - | - | Bit 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long Integer |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long Integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit Long Integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating Point |

3.5 Checksum

The checksum and the added content include the payload, as shown in Figure 1. The algorithm is as follows; by default, general version devices do not require checksums, and this part can be ignored. A random byte can be used for the downlink command.

//Assuming content="BDBDBDBDE9000600010A00000A00", the result of sum.ToString("X2") is 07// Below is the C# code calling method

private string CheckSum(string content)

{

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum = 0xff - sum;

return sum.ToString("X2");

}

4 Report messages

4.1 Connection related report

4.1.1 LNK-LIN (MSGID=0xF0) request connection (TCP only)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xF0 | See below | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 8 | u64 | IMEI | 1 | - | IMEI number (little-endian) |
| 2 | x16 | version |  | - | Bitfield see below |

The F0 request message reported by the device must have a reply from the server’s downlink 0XF1 message (see the next section for the specific format); otherwise, the login will fail.

Example:

Raw 16 hexadecimal message: BDBDBDBDF09B51731BC61603000000C2

BDBDBDBD: 4 bytes message header Header

F0: Message ID (message id) 9 B51731BC6160300: IMEI number (little-endian), converted to big-endian: 000316 C61B73519B,

Corresponding decimal for IMEI: 869465050010011,

0000：version

C2: checksum

Connection-related Q&A:

Each time tcp creates a new connection, it first reports an F0 request, which carries the IMEI. The server then records this IMEI.

Afterward, all data in this connection corresponds to this IMEI.

Note: If F0 is not received or if there is an exception in F0 communication, you can use a third-party network testing tool to verify whether the server communication is normal.

4.1.2 Connection reply (MSGID=0xF1) (TCP specific) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Timestamp (unix) timestamp | 0xF1 | See below for definitions | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 4 | u32 | Token | 1 | - | Token: Fixed to BDBDBDBD |

Note that the message order is different from the normal reported messages; this message is sent by the server and is not content reported by the device. The server response must be in the current channel.

Example:

Raw Hexadecimal Message: 07FD8860F1BDBDBDBD2E

07FD8860 (little-endian), Timestamp (unix timestamp in seconds), converted to big-endian 6088FD07 converted to decimal: 1619590407

Convert to standard time format UTC time: 2021-04-28 06:13:27

F1: Message ID (message id):

BDBDBDBD: Token

2E: checksum

The timestamp in decimal converted to standard time format is derived by adding 1619590407 seconds to 1970-01-01 00:00:00.

The device synchronizes time using the timestamp (in seconds) from this message.

Note: The standard unit in Java is milliseconds, please convert to the unit: seconds

Note: The total number of bytes in the server's downlink response is 10 bytes, for example: 0xBD--1 byte; if the device receives 20 bytes, it will not establish a connection.

4.1.3 New Heartbeat Packet Protocol (MSGID=0xF9) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xF9 | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery Type 0: Level 4, 1: Level 5, 2: Percentage, 3: Voltage Value |
| 2 | u16 | Bat_volt |  | -/- | Battery Level If Bat_type is 0: the battery level range is 0-3 (0 means 25%, 3 means 100%) If Bat_type is 1: the battery level range is 0-4 (0 means 20%, 4 means 100%) If Bat_type is 2: the battery level range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: percentage, 1: 5-level system 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal strength |
| 1 | U8 | Other_type |  |  | Extended type 0: full step counting 1: incremental step counting 2: vibration |
| 4 | U32 | Num |  |  | Extension Value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Note: F9 heartbeat packets must receive a server downlink reply. The device only considers the server connection to be active upon receiving the server's downlink reply; otherwise, it will re-report the F0 request connection.

Example:

Raw 16 hexadecimal message: BDBDBDBDF90104000050000095000000E377BD67AA

BDBDBDBD: Header

F9: Message ID

01: Bat_type battery level type 01 --> 1 --> 5 levels of battery (0 - 4)

0400: Bat_volt battery value converted to big-endian 0004 --> 4 --> level 4 corresponds to 100% battery

00: Signal_type signal type 00 --> 0 --> percentage

5000: Signal_strength signal strength converted to big-endian 0050 --> converted to decimal 80 -> 80% signal strength

00: Other_type extended type 00-->0--> Full-step count

95000000: Num extended value - step count Convert to big-endian 00000095--> Convert to decimal 149--> Step count is 149

E377BD67: Timestamp timestamp Convert to big-endian 67BD77E3--> Convert to 10 decimal 1740470243--> Timestamp is 1740470243 seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

AA: checksum

Note: The server can respond with a fixed message when receiving an F9 heartbeat packet: Fixed response example BDBDBDBDF301 Number of bytes:6

4.2 Alarm-related reporting

4.2.1 Alarm data upload -1 (MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0x02 | See below for definitions | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below defined |
| 4 | U32 | Timestamp | - | - | Timestamp (a timestamp will be added at the end during retransmission) |

Upl_warn contents

| bit (corresponding to the position of 1 in binary) | Name | Description | Raw message in little-endian hexadecimal | Converted to big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 14 | Fall alarm | Fall alarm | 0040 | 4000 | 4*4096=16384 |
| 8 | Device wearing | Device wearing | 0001 | 0100 | 1*256=256 |
| 7 | SOS canceled | SOS canceled | 8000 | 0080 | 8*16=128 |
| 5 | Long sit alarm (inactive) | Long sit alarm (inactive) | 2000 | 0020 | 2*16=32 |
| 4 | Remove (detached) device | Remove (detached) device | 1000 | 0010 | 1*16=16 |
| 2 | Power off | Power off | 0400 | 0004 | 4 |
| 1 | SOS alarm | SOS alarm | 0200 | 0002 | 2 |
| 0 | Low battery | Low battery | 0100 | 0001 | 1 |

Example:

Alarm wear: BDBDBDBD020001E377BD678A

BDBDBDBD:Header

02:Message ID

0001:Upl_warn alarm content, converted to big-endian:0100-->Wear Alert

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10base1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

8a：checksum

Drop Alert: BDBDBDBD021000E377BD678A

BDBDBDBD:Header

02:Message ID

1000:Upl_warn alarm content, converted to big-endian0010-->Drop Alert

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10base1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

8a：checksum

SOS Alert: BDBDBDBD020200E377BD678A

BDBDBDBD:Header

02:Message ID

0200:Upl_warn alarm content, converted to big-endian0002-->SOS Alert

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10base1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

8a：checksum

SOS Cancel: BDBDBDBD028000E377BD678A

BDBDBDBD:Header

02:Message ID

8000: Upl_warn alarm content, converted to big-endian0080--> SOS cancellation

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10base1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

8a：checksum

Drop alarm: BDBDBDBD020040E377BD678A

BDBDBDBD:Header

02:Message ID

0040: Upl_warn alarm content, converted to big-endian4000--> Drop alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10base1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

E7: checksum

Prolonged sitting alarm: BDBDBDBD022000E377BD678A

BDBDBDBD:Header

02:Message ID

2000: Upl_warn alarm content, converted to big-endian0020--> Prolonged sitting alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10base1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

8A: checksum

Low battery alarm: BDBDBDBD020100E377BD678A

BDBDBDBD:Header

02:Message ID

0100: Upl_warn alarm content, converted to big-endian0001--> Low battery alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10base1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

8A: checksum

Shutdown alarm: BDBDBDBD020400E377BD678A

BDBDBDBD:Header

02:Message ID

0400:Upl_warn alarm content, converted to big-end0004--> Shutdown alarm

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10base1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

8A: checksum

Note: This alarm is for devices that do not differentiate shutdown types. For devices that differentiate shutdown types, see the next section.

4.2.2 Alarm Data Upload-2 (MSGID=0x21) (Supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0x21 | See below for definitions | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | Alarm type |
| N |  | Upl_warn | - | - | Alarm content See below for definitions of different Alarm types |
| 4 | U32 | Timestamp |  |  | Timestamp (when supplementing transmission, a timestamp will be added at the end) See definitions of different Alarm types, not necessarily at the end |

Alarm type=1

Upl_warn contents

| bit (corresponding to the position of 1 in binary) | Name | Description | Raw message in little-endian hexadecimal | Converted to big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Shutdown alarm for charging | Automatic shutdown when charging (firmware function support required) | 04000000 | 0004 | 4 |
| 1 | Low battery shutdown alarm | Device shutdown due to low battery (firmware function support required) | 02000000 | 0002 | 2 |
| 0 | Active shutdown alarm | Device manually shut down by user (firmware function support required) | 01000000 | 0001 | 1 |

Example:

Charging shutdown alarm: BDBDBDBD21010004000000E377BD6767

BDBDBDBD   :   Header

21   :   Message ID

0100   :   Alarm type, converted to big-endian 0001 --> Alarm type = 1

04000000   :   Upl_warn alarm content, converted to big-endian 0004 --> Charging shutdown alarm

E377BD67:Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert standard time format UTC time: 2025-02-25 07:57:23 --> Convert Beijing time: 2025-02-25 15:57:23

67 : checksum

Low power shutdown alarm: BDBDBDBD21010002000000E377BD6767

BDBDBDBD   :   Header

21   :   Message ID

0100   :   Alarm type, converted to big-endian 0001 --> Alarm type = 1

02000000 : Upl_warn alarm content, converted to big-endian 0002 --> low power shutdown alarm

E377BD67:Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert standard time format UTC time: 2025-02-25 07:57:23 --> Convert Beijing time: 2025-02-25 15:57:23

67 : checksum

Active shutdown alarm: BDBDBDBD21010001000000E377BD6767

BDBDBDBD   :   Header

21   :   Message ID

0100   :   Alarm type, converted to big-endian 0001 --> Alarm type = 1

01000000 : Upl_warn alarm content, converted to big-endian 0001 --> active shutdown alarm

E377BD67:Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert standard time format UTC time: 2025-02-25 07:57:23 --> Convert Beijing time: 2025-02-25 15:57:23

67 : checksum

Alarm type=5

Upl_warn contents

| Format | Name | Unit | Description |
| --- | --- | --- | --- |
| U16 | Type | 2 | Fixed to 0x05 |
| U32 | Timestamp | 4 | Timestamp |
| U16 | Content Len | 2 | Alarm content length |
| U8 | Health Warn Type1 | 1 | Health threshold alarm type: 0x01 Heart Rate, 0x02 Systolic Blood Pressure (SBP), 0x03 Diastolic Blood Pressure (DBP), 0x04 Blood Oxygen, 0x05 Temperature |
| U8 | Type1 | 1 | 01 less than the set health threshold, 02 greater than the set health threshold |
| I16 | Health Value1 | 2 | Health result value |
| U8 | Health Warn Type2 | 1 | Health threshold alarm type: 0x01 Heart Rate, 0x02 Systolic Blood Pressure (SBP), 0x03 Diastolic Blood Pressure (DBP), 0x04 Blood Oxygen, 0x05 Temperature |
| U8 | Type2 | 1 | 01 less than the set health threshold, 02 greater than the set health threshold |
| I16 | Health Value2 | 2 | Health result value |
| 。。。。 | 。。。。 | 。。。。 | 。。。。 |

Example:

The device detected that the heart rate (65) is outside the issued health threshold range: BDBDBDBD210500E377BD6704000102410073

BDBDBDBD: Header

21: Message ID

0500: Alarm type, converted to big-endian 0005 --> Alarm type=5

E377BD67: Timestamp, converted to big-endian 67 BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25    07:57:23 --> converted to Beijing time: 2025-02-25 15:57:23

0400: Content Length of alarm message, converted to big-endian 0004 --> length of alarm content is 4 bytes, note: multiple health threshold alarms will increase the length

01: Health Warn Type, health threshold alarm type 01 --> heart rate alarm

02: Type greater than the set health threshold & less than the set health threshold 01 --> less than the set health threshold

4100:Health Value The reported health value converted to big-endian0041--> converted to decimal 65 --> reported heart rate value is 65

73:checksum

4.2.3 Exception Data Alarm (MSGID=0x16)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0x16 | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type | -/- | -/- | Alarm Type (0: Heart rate abnormal value alarm, 1: Temperature abnormal value alarm) |
| 2 | U16 | Heart | -/- | -/- | Heart rate value |
| 2 | U16 | temperature | -/- | -/- | Temperature |
| 2 | U16 | Pa | -/- | -/- | Air pressure |
| 4 | 4*U8 or U32 | expand | -/- | -/- | 4 bytes expansion for other sensors |

Note: The reported abnormal values have no actual reference significance; they only indicate that the currently detected data is abnormal (such as values exceeding the range or abnormal sensor detection). Abnormal values will be removed in other normal messages and reflected in this message, which can be ignored for parsing.

Example:

Temperature abnormal value alarm: BDBDBDBD16010000FF0F000000000000E6

BDBDBDBD:Header

16:Message ID

01:Type alarm type 01 --> Temperature abnormal value alarm

0000:Heart rate value defaults to 0000 if not present

FF0F:Temperature in big-endian 0FFF --> Temperature abnormal value FFF

0000:Pa pressure default fill 0000 if this value is not present

00000000 expand, default fill 0000 if this value is not present

E6:checksum

4.3 Location-related Report

4.3.1 GPS/BDS Location Report: Positioning data report (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0x03 | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude |
| 8 | Double | lat |  | - | latitude |
| 1 | U8 | north_south |  | - | /N or S/ |
| 1 | U8 | east_west |  | - | /E or W/ |
| 1 | U8 | status |  | - | /A or V/ |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Raw16Hexadecimal message: BDBDBDBD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BDBDBDBD:Header

03:Message ID

963E870C3E675E40: lon longitude  convert to big-endian405E673E0C873E96--> Longitude:121.6131622  Note: Conversion analysis can be referred to in the example below

FF2110C2B6343F40: lat latitude  convert to big-endian`403F34B6C21021FF--> Latitude:31.2059137  Note: Conversion analysis can be referred to in the example below

4E: north_south  4E--> Convert to ASCII code N--> North latitude

45: east_west 45--> Convert to ASCII code E--> East longitude

41: status data validity  41--> Convert to ASCII code A--> Valid data

E377BD67: Timestamp timestamp  convert to big-endian67BD77E3--> Convert to decimal101740470243--> Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

78: checksum

Note: The parsed satellite (GPS/BDS) longitude and latitude are in WGS-84 coordinate system. If the map uses Baidu, Gaode, etc., the coordinate system needs to be converted.

Example of Java code for parsing satellite positioning messages:

// Message: BDBDBDBD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BDBDBDBD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b

public static void main(String[] args){

//Message 7d9f84ac81815c40 Actual Value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d",16)));  //114.02353966666665//Message e766926b1d893640 Actual Value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16))) ;//22.535605166666667

HexToStr(data.Substring(“4e”));   //N

HexToStr(data.Substring(“45”));//E

HexToStr(data.Substring(“41”));//A   A indicates valid data, while V indicates invalid data, which can be discarded//Message 749d695f Actual Value 5f699d74

Date date=new Date();

date.setTime(Long.parseLong(“5f699d74",16)*1000);

SimpleDateFormat sdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.3.2 Wifi and Base Station Information Upload (MSGID=0xA4)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xA4 | See below for definitions | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 1 | u8 | Cell_cnt | 1 | - | Number of cell info payload. Valid value: 0~7 |
| 2 | u16 | Cell[0].MCC | - | - | Mobile country code of cell[0] |
| 2 | u16 | Cell[0].MNC | - | - | Mobile network code of cell[0] |
| 2 | u16 | Cell[0].LAC | - | - | Location area code of cell[0] |
| 4 | U32 | Cell[0].CELL_ID | - | - | Cell id of cell[0] |
| 2 | i16 | Cell[0].RSSI | - | dbm | RSSI in dbm of cell[0] |
| ... | ... | ... | ... | ... | ... |
| 2 | u16 | Cell[cell_cnt-1.MCC | - | - | mobile country code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.MNC | - | - | mobile network code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.LAC | - | - | Location area code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1].LAC | - | - | Location area code of cell[[cell_cnt-1] |
| 4 | U32 | Cell[cell_cnt-1].CELL_ID | - | - | Cell id of cell[[cell_cnt-1] |
| 2 | I16 | Cell[cell_cnt-1].RSSI | - | dbm | RSSI in dbm of cell[[cell_cnt-1] |
| 1 | U8 | wifi个数 |  |  | Number 0f wifi |
| 6 | 6*U8 | Wifi[0].bssid[0]-Wifi[0].bssid[5] |  |  | wifi(0) MAC address |
| 4 | I32 | Wifi[0].RSSI |  |  | wifi(0) signal strength RSSI |
| ... | ... | ... | ... | ... | ... |
| 6 | 6*U8 | Wifi[Wifi_cnt-1].bssid[0]-Wifi[Wifi_cnt-1].bssid[5] |  |  | wifi(N) MAC address |
| 4 | I32 | Wifi[0].RSSI |  |  | wifi(0) signal strength RSSI |

Note: The device may not always report a base station; the absence of a base station does not affect positioning. If the number of base stations in the message is 0, the following directly represents the number of wifis.

Example:

Raw 16 Hexadecimal Message: BDBDBDBDA4E377BD6701CC010000C218D1AD160BF9FF078CBEBE1A8162C6FFFFFFC061180AF42AC1FFFFFF 200 BC726E000B6FFFFFFA8154DF6517EB2FFFFFFE005C5B1F824CCFFFFFFE8FCAFA02663AFFFFFFF6409805B2B9CAEFFFFFF94

BDBDBDBD:Header

A4: Message ID

E377BD67:Timestamp converted to big-endian67BD77E3-->converted to10base1740470243-->Timestamp is1740470243seconds-->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

01: Cell_cnt (number of base stations) 01 --> Number of base stations is 1

CC01:Cell[ 0 ].MCC Base Station MCC converted to Big Endian 01 CC --> converted to decimal 460 --> Base Station MCC is 460

0000 :Cell[ 0 ].MNC Base Station MNC converted to Big Endian 0000 --> converted to decimal 0 --> Base Station MNC is 0

C218:Cell[ 0 ].LAC Base Station LAC converted to Big Endian 18 C2 --> converted to decimal 6338 --> Base Station LAC is 6338

D1AD160B:Cell[ 0 ].CELL_ID Base Station ID converted to Big Endian 0 B16ADD1 --> converted to decimal 186035665 --> Base Station ID is 186035665

F9FF:Cell[ 0 ].RSSI Base Station Signal Strength converted to Big Endian FFF9 --> converted to decimal signed -7 --> Base Station Signal Strength is -7

07 : Wifi_cnt number of wifis 07 --> 7 wifis

8 CBEBE1A8162:wifi MAC address 8 C:BE:BE: 1 A: 81 : 62 --> wifi[ 0 ]'s MAC address is 8 C:BE:BE: 1 A: 81 : 62

C6FFFFFF:wifi signal strength RSSI converted to Big Endian FFFFFFFC6 --> converted to decimal signed -58 --> wifi[ 0 ]'s RSSI signal strength is -58

C061180AF42A:wifi MAC address C0: 61 : 18 : 0 A:F4: 2 A --> wifi[ 1 ]'s MAC address is C0: 61 : 18 : 0 A:F4: 2 A

C1FFFFFF:wifi signal strength RSSI converted to Big Endian FFFFFFFC1 --> converted to decimal signed -63 --> wifi[ 0 ]'s RSSI signal strength is -63

200BC726E000:wifi MAC address 20:0B:C7:26:E0:00--> The MAC address of wifi[2] is20:0B:C7:26:E0:00

B6FFFFFF: wifi signal strength RSSI converted to big-endian FFFFFFB6 --> converted to decimal two's complement - 74 ---> wifi[ 0 ]'s RSSI signal strength is - 74

A8154DF6517E: wifi MAC address A8: 15 : 4 D: F6: 51 : 7 E --> wifi[ 3 ]'s MAC address is A8: 15 : 4 D: F6: 51 : 7 E

B2FFFFFF: wifi signal strength RSSI converted to big-endian FFFFFFB2 --> converted to decimal two's complement - 78 ---> wifi[ 0 ]'s RSSI signal strength is - 78

E005C5B1F824: wifi MAC address E0: 05 : C5: B1: F8: 24 --> wifi[ 4 ]'s MAC address is E0: 05 : C5: B1: F8: 24

CCFFFFFF: wifi signal strength RSSI converted to big-endian FFFFFFCC --> converted to decimal two's complement - 52 ---> wifi[ 0 ]'s RSSI signal strength is - 52

E8FCAFA02663: wifi MAC address E8: FC: AF: A0: 26 : 63 --> wifi[ 5 ]'s MAC address is E8: FC: AF: A0: 26 : 63

AFFFFFFF: wifi signal strength RSSI converted to big-endian FFFFFFAF --> converted to decimal two's complement - 81 ---> wifi[ 0 ]'s RSSI signal strength is - 81

6409805 B2B9C: wifi MAC address 64 : 09 : 80 : 5 B : 2 B : 9 C --> wifi[ 6 ]'s MAC address is 64 : 09 : 80 : 5 B : 2 B : 9 C

AEFFFFFF: wifi signal strength RSSI converted to big-endian FFFFFFAE --> converted to decimal two's complement - 82 ---> wifi[ 0 ]'s RSSI signal strength is - 82

94 : checksum

The parameters parsed from the message need to interface with the map Wi-Fi database to ultimately obtain latitude and longitude data through the map provider.  
    Taking the integration of Amap with the Wi-Fi database as an example:

Amap classifies the positioning scenes into three types based on the network access method described in the Amap Wi-Fi positioning protocol document. Below are corresponding examples for the three positioning scenes:

1. The access method is via Wi-Fi, that is, the accesstype parameter is 1.

http://apilocate.amap.com/position?accesstype=1&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&mmac=50:a7:2b:1e:56:58,-60,alibaba-inc&macs=4c:48:da:25:0b:11,-59,alibaba-

inc|4c:48:da:25:1a:11,-77,alibaba-inc&serverip=10.2.166.4&output=xml&key=<User Key>

2. The access method is mobile access and the SIM card is not a CDMA card, that is, the accesstype parameter is 0 and cdma=0:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=0&bts=460,01,40977,2205409,-65&nearbts=460,01,40977,2205409,-65|460,0

1,40977,2205409,-65|460,01,40977,2205409,-65&serverip=10.2.166.4&output=xml&key=<User Key>

When the network access method is mobile access and the SIM card is a CDMA card, that is, when the accesstype parameter is 0 and cdma=1:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=1&bts=13824,1,1838,1674723,575739,-52&serverip=10.2.166.4&output=xml&key=<User Key>

W200P watch: Instructions for calling the Amap Wi-Fi positioning protocol interface. Generally select the first option for parameters.

You can also select the second option, but if the device has not reported the base station, then the parameters bts and neabts need to be removed.

First option, input parameters Example: mmac is the first Wi-Fi MAC address in the message <http://apilocate.amap.com/position?accesstype=1&imei=869907057780989&mmac=04:8C:16:A9:FB:A8,-44,TP-Link&macs=04:8C:16:A9:FB:A8,-44,TP-Link|C8:3A:35:F4:CE:18,-59,TP-Link|1C:78:4E:D5:07:FE,-79,TP-Link|C8:3A:35:9D:0B:E0,-87,TP-Link|4A:D8:90:E8:0A:82,-88,TP-Link|AC:AD:4B:C9:C3:87,-89,TP-Link|A4:16:E7:55:84:AC,-91,TP-Link&output=json&key=xxxx

For the specific Amap Wi-Fi positioning protocol PDF, you can click to view and download: This document is provided by Amap, and specific integration inquiries can be made directly to Amap. Note that the document’s name may change upon download, so feel free to rename it.

4.3.3 Bluetooth positioning information (BLE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xD6 | See below for definitions | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 0 |
| U8 | Total_groups | 1 | Total number of groups; there may be multiple sets of information, and each group may contain multiple iBeacons. |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identifier codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identifier codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identifier codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identifier codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | Total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identifier codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identifier codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identifier codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identifier codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| ... | ... | ... | ... |

Note: In the general version, the total number of Bluetooth beacons is fixed at 1, with a maximum of 4 beacons. They are sorted by signal strength from highest to lowest. By default, the watch only recognizes our own beacons. If you need to interface with other brands' beacons, please consult the relevant personnel via WeChat.Example:

Raw Hexadecimal Message: BDBDBDBDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

bdbdbdbd: Header

D6: Message ID

00: type

01 : Only one set of ibeacon data

E377BD67 : Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert standard time format UTC time: 2025-02-25 07:57:23 --> Convert Beijing time: 2025-02-25 15:57:23

04 : The first set has 4 beacon information

4327 : Converted to big-endian --> major: 0x2743 --> converted to decimal 10051

1794 : Converted to big-endian --> minor: 0x9417 --> converted to decimal 37911

AC : RSSI 0xAC --> converted to decimal two's complement -84

4327 : Converted to big-endian --> major: 0x2743 --> converted to decimal 10051

3094 : Converted to big-endian --> minor: 0x9430 --> converted to decimal 37936

AA : RSSI 0xAA --> converted to decimal two's complement -86

4327 : Converted to big-endian --> major: 0x2743 --> converted to decimal 10051

b956:Convert to big-endian --> minor:0x56b9 --> Convert to decimal 22201

A5:   RSSI 0xA5 --> Convert to decimal two's complement -91

4327 : Converted to big-endian --> major: 0x2743 --> converted to decimal 10051

FE94: Convert to big-endian --> minor: 0x94FE --> Convert to decimal 38142

A5: rssi: 0xA5 --> Convert to decimal two's complement -91

6a -- Checksum

Bluetooth beacon major, minor description: By default, Bluetooth beacons are shipped with a label code,

for example: 00081005345866, where the Bluetooth beacon's major is 10053, minor is 45866, and manufacturer identifier is 0008 (this has no use or practical significance)

4.4 Device Information and Status Reporting

4.4.1 Device Charging Status Upload (MSGID=0xC3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Status | / | / | 0 Start, 1 End, 2 Fully Charged |
| 1 | U32 | Timestamp | / | / | Timestamp (a timestamp will be added at the end during retransmission) |

Note: This message requires device firmware support; by default, the latest generic version supports it.  
    Example:

Original Hexadecimal Message: BDBDBDBDC301E377BD678A

BDBDBDBD:Header

C3: Message ID

01: Device Ends Charging

E377BD67 : Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert standard time format UTC time: 2025-02-25 07:57:23 --> Convert Beijing time: 2025-02-25 15:57:23

8A: Checksum

Original Hexadecimal Message: BDBDBDBDC300DB4D2F668A

BDBDBDBD:Header

C3: Message ID

00: Device Starts Charging

E377BD67 : Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert standard time format UTC time: 2025-02-25 07:57:23 --> Convert Beijing time: 2025-02-25 15:57:23

8A: Checksum

Original Hexadecimal Message: BDBDBDBDC302DB4D2F668A

BDBDBDBD:Header

C3: Message ID

02: Device Fully Charged

E377BD67 : Timestamp converted to big-endian 67BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Convert standard time format UTC time: 2025-02-25 07:57:23 --> Convert Beijing time: 2025-02-25 15:57:23

8A: Checksum

4.4.2 Status Parameter Reporting (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Number of parameter types |
| 1 |  |  |  |  | (Reserved 00) |
| 1 | u8 | Type | -/- | -/- | Parameter type 1, Type definition: 00—MCU (firmware), 01—module, 02—sensor (this type is not available in the general version) |
| 1 | U8 | NameLen | -/- | -/- | Length of parameter name 1 |
| n | N*u8 | Name | -/- | -/- | Parameter name |
| 1 | u8 | Type | -/- | -/- | Parameter type 2, Type definition: 00—MCU (firmware), 01—module, 02—sensor (this type is not available in the general version) |
| 1 | U8 | NameLen | -/- | -/- | Parameter Type 2 Length |
| n | N*u8 | Name | -/- | -/- | Parameter name |
| ... | ... | ... | ... | ... | ... |

Note: This message is reported once at startup, uploading the device firmware version number and module version number, which can be ignored. The Message ID for previously shipped devices is 0xBB; please contact the relevant personnel for issuance.

Example:

BDBDBDBDA902000009423732352E4F56303601174E5432364B434E4230304E4E412D4C3032303330393530B5

BDBDBDBD:Header

A9: Message ID

02: TypeCnt indicates there are 2 types of parameters

00: Reserved field

00: Type=00  00 represents MCU (firmware)

09: Length of the following MCU name is 9 bytes

423732352E4F563036--->Hex to text (device firmware version number)--->B725.OV06

01: Type=01  01 represents the module

17: The length of the communication module name is 23 bytes

4E5432364B434E4230304E4E412D4C3032303330393530 : Hexadecimal to text (4G module version number)

--->NT26KCNB00NNA-L02030950

B5---checksum

4.4.3 Device Status (MSGID=0xE9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xE9 | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | type |  | / | Default—00 |
| 2 | u16 | length |  | / | Message length following |
| 1 | u8 | location |  | / | Reporting frequency for positioning: 00—default not modified, 01—modified by downlink (if there are multiple time periods for downlink, only report the current time period's reporting frequency) |
| 2 | u16 | frequency |  | / | Frequency, unit - minutes |
| 1 | u8 | health |  | / | Health reporting frequency: 00—default not modified, 01—modified by downlink |
| 2 | u16 | frequency |  | / | Frequency: unit - minutes |

Note: This message reports once upon powering on, and will report once again after the frequency is modified by downlink

Example:

Original hexadecimal message: BDBDBDBDE9000600010A00010A0033

BDBDBDBD:Header

E9 : Message ID

00 : type=00

0600: length The message length after converting to big-endian --> 0006 --> converting to decimal 6, the message length is 6 bytes

01:location 01 indicates that the downlink has modified the positioning reporting frequency

0A00:frequency The positioning reporting frequency converted to big-endian --> 000A --> converted to decimal 10, with a reporting frequency of 10 minutes

01:health 01 indicates that the downlink has modified the health reporting frequency

0A00:frequency The health reporting frequency converted to big-endian --> 000A --> converted to decimal 10, with a reporting frequency of 10 minutes

33:checksum checksum

If the device has no health reporting function or no positioning reporting function, then the health reporting frequency and

the positioning reporting frequency will be consistent, indicating the device's reporting frequency

4.5 Health-related Reporting

4.5.1 Health Data (MSGID=0x32)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0x32 | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | 00 |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 2 | U16 | contentLength |  |  | Total length of subsequent content |
| 1 | U8 | ID |  |  | Id (type + reported value length), see the definition of ID type below |
| N | N*U8 | Val1 |  |  | Reported value of Id (varies in byte length according to reported value length) |
| ... | ... | ... | ... | ... | ... |
| 1 | U8 | ID |  |  | Id (type + reported value length), see the definition of ID type below |
| N | N*U8 | Val1 |  |  | Reported value of Id (varies in byte length according to reported value length) |

ID type definition (hexadecimal):

01Step count02Heart rate03Body temperature04Wrist temperature06Diastolic pressure07Systolic pressure08Blood oxygen09HRV (supported in special versions)

(05Blood sugar10Number of sit-ups11Pace and distance of running12Speed and count of jump rope)---Reserved, not yet available

Default Universal Version Supports:01 Steps02 Heart Rate03 Body Temperature04 Wrist Temperature06 Diastolic Pressure07 Systolic Pressure08 Blood Oxygen

Example:

Original Hexadecimal Message: BDBDBDBD3200E377BD6711000A1E00114B314A39711A4A0122bc00416212

BDBDBDBD: Header

32: Message ID

00: type type = 00

E377BD67: Timestamp converted to big-endian 67 BD77E3 --> Converted to decimal 1740470243 --> Timestamp is 1740470243 seconds -->

Convert to standard time format UTC time:2025-02-25 07:57:23--> Convert to Beijing time:2025-02-25 15:57:23

1100: Length of the following message converted to big-endian --> 0011 --> Converted to decimal 17, the length of the following message is 17 bytes

0A: ID 0x0A --> Converted to binary 00001010 The first five bits are 00001 --> Converted to hexadecimal 01, representing data ID steps (0x01),

The last three bits are 010 --> Converted to decimal 02, representing that the length of step data is 2 bytes

1E00: Reported step value converted to big-endian --> 001E --> Converted to decimal 30, total steps are 30

11: ID 0x11 --> Converted to binary 00010001 The first five bits are 00010 --> Converted to hexadecimal 02, representing data ID heart rate (0x02),

The last three digits are    001    --> Convert to decimal    1    , indicating that the heart rate data length is    1    byte

4    B: Heart rate reporting value Convert to decimal-->    75    , heart rate data is    75

31    : ID    0x31    --> Convert to binary    00110001       The first five digits are    00110    --> Convert to hexadecimal    06    , indicating data ID diastolic pressure (    0x06    ),

The last three digits are    001    --> Convert to decimal    1    , indicating that the diastolic pressure data length is    1    byte

4    A: Diastolic pressure reporting value, convert to decimal-->    74    , diastolic pressure data is    74

39    : ID    0x3    --> Convert to binary    00111001        The first five digits are    00111    --> Convert to hexadecimal    07    , indicating data ID systolic pressure (    0x07    ),

The last three digits are    001    --> Convert to decimal    1    , indicating that the systolic pressure data length is    1    byte

71    : Systolic pressure reporting value, convert to decimal-->    113    , systolic pressure data is    113

1    A: ID    0x1A    --> Convert to binary    00011010       The first five digits are    00011    --> Convert to hexadecimal    03    , indicating data ID body temperature (    0x03    ),

The last three digits are    010    --> Convert to decimal    2    , indicating that the body temperature data length is    2    bytes

4 A01: Temperature report value, converted to big-endian --> 014 A --> Convert to decimal 330 --> 330 * 0.1 --> Temperature data is 33 degrees Celsius

22:ID  0x22--> Convert to binary00100010   The first five bits are00100--> Convert to hexadecimal04, representing data ID wrist temperature (0x04),

The last three bits are010--> Convert to decimal2, representing the length of the wrist temperature data as2bytes

BC00: Wrist temperature reported value, converted to big-endian-->00BC--> Convert to decimal188-->188*0.1--> Wrist temperature data is18.8degrees Celsius

41:ID  0x41--> Convert to binary01000001    The first five bits are01000--> Convert to hexadecimal08, code data ID blood oxygen (0x08),

The last three bits are001--> Convert to decimal1, representing the length of the blood oxygen data as1byte

62: Blood oxygen reported value, converted to decimal-->98, blood oxygen data is98

12: checksum

4.5.2 Device sleep analysis data upload (MSGID=0xC5)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xC5 | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | Int32 | DateTime |  | -/- | Upload UTC start time |
| 4 | Int32 | DateTime |  |  | Upload UTC end time |
| 2 | Int16 | Sleep minute |  |  | Upload sleep duration in minutes |
| 4 | Int32 | Type |  |  | Sleep status: Upload type, 1 is deep sleep, 2 is light sleep, 3 is awake duration |

Example:

Raw 16 hexadecimal message: BDBDBDBDC5AC338860693B8860210001000000D1

BDBDBDBD: Header

C5: Message ID

AC338860: Start time, converted to big-endian 608833 AC --> converted to decimal --> timestamp is 1619538860 seconds -->

Converted to standard time format UTC: 2021 - 04 - 27  15 : 54 : 20 --> converted to Beijing time: 2021 - 04 - 27  23 : 54 : 20

693 B8860: End time, converted to big-endian 60883 BAC --> Convert to decimal --> Timestamp is 1619540908 seconds -->

Convert to standard time format UTC time:2021-04-27 16:28:28--> Convert to Beijing time:2021-04-28 00:28:28

2100: Sleep minute sleep time, converted to big-endian0021--> Convert to decimal33--> Statistics of sleep time is33minutes

01000000: Type sleep state, converted to big-endian00000001--> Convert to decimal1-->1is deep sleep, deep sleep time is33minutes

D1: checksum

4.6 Downlink feedback related reporting

4.6.1 Downlink feedback (MSGID=0xC0)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xC0 | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This command is used to provide feedback on downlink instructions, returning the Message ID(s) of the previously received downlink instructions (multiple Message IDs can be returned collectively).

Raw hex message: BDBDBDBDC0011720

BDBDBDBD:Header

C0: Message ID

01: length Message ID length in decimal 01 --> Message ID length is 1 byte

17: Type indicates the device received the downlink instruction Message ID is 17

20: checksum

4.6.2 Message status report (MSGID=0x28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0x28 | See below for definitions | CK_sum |

Payload contents

| Bytes | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | timestamp | -/- | -/- | Timestamp |
| 1 | U8 | Type | -/- | -/- | Character encoding type (corresponding to the following 0X28), GB2312 encoding: 0x03; Unicode encoding: 0x05 |
| 1 | U8 | status | -/- | -/- | Message Status - 0x01: Confirmed, 0x02: Rejected |
| 4 | U32 | seqID | -/- | -/- | (Corresponding message id seqID for downlink 0X28) |

Example:

Note: Currently, the general version does not differentiate character encoding types, fixed at 03. Future versions will differentiate.

Original Hexadecimal Message: BDBDBDBD28FD3BA068030268A03BE477

BDBDBDBD:Header

28: Message ID

FD3BA068: timestamp - convert to big-endian 68A03BFD --> convert to decimal --> timestamp is 1755331581 seconds -->

Convert to standard time format UTC time: 2025-08-16 08:06:21 --> convert to Beijing time: 2025-08-16 16:06:21

03: Type 03 -- fixed value

02: status - message status 02 -- interface click to reject button -- rejected

68A03BE4:The seqID of the downlink message is 68A03BE4

77:checksum

5 Settings

5.1 Downlink

5.1.1 Set Position Reporting Frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0x17 | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |

Note: A day's time is 00:00-23:59
Example:

00:00-23:59 Report location every 10 minutes: BDBDBDBD17010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

17:Message ID

01:enable whether enabled Convert to decimal 01-->1--Enabled

0A00:Interval time interval Convert to big-endian 000A-->Convert to decimal 10-->Time interval is10minutes

00:time_start_h Start time - hour, convert to decimal00-->Start time - hour is00

00:time_start_m Start time - minute, convert to decimal00-->Start time - minute is00

17:time_end_h End time - hour, convert to decimal23-->Start time - hour is00

3B:time_end_m End time - minute, convert to decimal59-->Start time - hour is00

00:enable whether enabled Convert to decimal00-->0--Not enabled

000000000000: All fields filled in disabled status00

00:enable whether enabled Convert to decimal00-->0--Not enabled

000000000000: All fields filled in disabled status00

00:enable whether enabled Convert to decimal00-->0--Not enabled

000000000000: All fields filled in disabled status00

97:checksum

5.1.2 Message Sending (MSGID=0X28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0x28 | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Description |
| --- | --- | --- | --- |
| 1 | U8 | type | Message type; if it is a downlink text message, the Unicode encoding is 05 |
| 4 | Uint32 | seqID | Message ID, must be unique |
| 1 | U8 | CONTENT LEN | Content length |
| N | N | CONTENT | Content, in Chinese as Unicode encoding (little-endian, note the conversion, one Chinese character occupies 2 bytes, one letter occupies 2 bytes) |

Note: The default Chinese character encoding for the Hong Kong common version is Unicode. Each message ID must be unique; repeating IDs cannot be received by the device. Example:

Text message: hello,worldBDBDBDBD28050300000016680065006C006C006F002C0077006F0072006C006400DD

BDBDBDBD:Header

28: Message ID

05: type message type, convert to decimal 05 --> type=05

03000000: seqID information ID, information ID is 03000000

16: CONTENT LEN length of content, convert to decimal 22, length of content is 22 bytes

680065006C006C006F002C0077006F0072006C006400:

Every 2 bytes converted to big-endian --> convert to Unicode characters --> combine final text message content: hello,world

DD:checksum

5.1.3 Settings - Positioning Priority Settings (0XCE01)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definitions | CK_sum |

Payload contents

Type=01 Priority Setting for Positioning

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 effective this time |
| 2 | U16 | Len | -/- | -/- | Length of the following command |
| n | n |  | -/- | -/- | Body |

Location priority setting definition -- see 2.3 Device downlink description -- Device location priority issuance (0xCE01)

Location priority setting body: Currently supported common versions: 01-- Satellite (GPS/BDS), 02-- Wi-Fi (Wi-Fi + base station) 03-- Bluetooth beacon (additional deployment of Bluetooth beacon required); Special devices support: 04-- LBS base station 05-- 125k

Example:

Wi-Fi location priority (Wi-Fi > Bluetooth beacon > Satellite (GPS/BDS)): BDBDBDBDCE0100030002030133

BDBDBDBD:Header

CE : Message ID

01:Type 01 Positioning Priority Setting

00:Valid Validity 00 Always valid

0300:Len Convert to Big Endian 0003--> Convert to Decimal 03--> Body length is 3 bytes

020301:Body: 02--wifi 03--Bluetooth Beacons 01--Satellite (GPS/BDS) Positioning priority is wifi > Bluetooth Beacons > Satellite (GPS/BDS)

33:checksum checksum

Satellite (GPS/BDS) has the highest priority (Satellite (GPS/BDS) > wifi > Bluetooth Beacons):BDBDBDBDCE0100030001020333

BDBDBDBD:Header

CE : Message ID

01:Type 01 Positioning Priority Setting

00:Valid Validity 00 Always valid

0300:Len Convert to Big Endian 0003--> Convert to Decimal 03--> Body length is 3 bytes

010203:Body: 01--Satellite (GPS/BDS) 02--wifi 03--Bluetooth Beacons Positioning priority is Satellite (GPS/BDS) > wifi > Bluetooth Beacons

33:checksum checksum

Bluetooth has the highest priority (Bluetooth Beacons > wifi > Satellite (GPS/BDS)):BDBDBDBDCE0100030003020133

BDBDBDBD:Header

CE : Message ID

01:Type 01 Positioning Priority Setting

00:Valid Validity 00 Always valid

0300:Len Convert to Big Endian 0003--> Convert to Decimal 03--> Body length is 3 bytes

030201:Body: 03--Bluetooth Beacons 02--wifi 01--Satellite (GPS/BDS) Positioning priority is Bluetooth Beacons > wifi > Satellite (GPS/BDS)

33:checksum checksum

Single Satellite (GPS/BDS) Positioning:BDBDBDBDCE0100030001000033

BDBDBDBD:Header

CE : Message ID

01:Type 01 Positioning Priority Setting

00:Valid Validity 00 Always valid

0300:Len Convert to Big Endian 0003--> Convert to Decimal 03--> Body length is 3 bytes

030201:Body: 01--Satellite (GPS/BDS) 00--No Positioning 00--No Positioning Positioning priority is Single Satellite (GPS/BDS) Positioning

33:checksum checksum

Single WiFi positioning: BDBDBDBDCE0100030002000033

BDBDBDBD:Header

CE : Message ID

01:Type 01 Positioning Priority Setting

00:Valid Validity 00 Always valid

0300:Len Convert to Big Endian 0003--> Convert to Decimal 03--> Body length is 3 bytes

020000:Main text:02--wifi 00--no positioning 00--no positioning Positioning priority is single wifi positioning

33:checksum checksum

Single Bluetooth beacon positioning:BDBDBDBDCE0100030003000033

BDBDBDBD:Header

CE : Message ID

01:Type 01 Positioning Priority Setting

00:Valid Validity 00 Always valid

0300:Len Convert to Big Endian 0003--> Convert to Decimal 03--> Body length is 3 bytes

020000:Main text:03--Bluetooth beacon 00--no positioning 00--no positioning Positioning priority is single Bluetooth beacon positioning

33:checksum checksum

5.1.4 Settings - Health sampling reporting frequency setting (MSGID=0X1C)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0x1C | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |  |

Health sampling reporting frequency setting definition--See2.3 Device downlink description--Device health sampling frequency issuance (0x1D)

Note: After setting, report based on time period; do not report outside the time period

Example:

00:00-23:59(All day) Report health data every 10 minutes: BDBDBDBD1C010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

1C:Message ID

01:enable whether enabled Convert to decimal 01-->1--Enabled

0A00:Interval time interval Convert to big-endian 000A-->Convert to decimal 10-->Time interval is10minutes

00:time_start_h Start time - hour, convert to decimal00-->Start time - hour is00

00:time_start_m Start time - minute, convert to decimal00-->Start time - minute is00

17:time_end_h End time - hour, converted to decimal23-->End time - hour is23

3B:time_end_m End time - minutes, convert to decimal59--> End time - minutes is59

00:enable whether enabled Convert to decimal00-->0--Not enabled

000000000000: All fields filled in disabled status00

00:enable whether enabled Convert to decimal00-->0--Not enabled

000000000000: All fields filled in disabled status00

00:enable whether enabled Convert to decimal00-->0--Not enabled

000000000000: All fields filled in disabled status00

97:checksum

5.1.5 Settings - Device Alarm Settings (0XCE03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definitions | CK_sum |

Payload contents

Type=03 Device Alarm Settings

| Byte offset | Format | Name | Unit | Description |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Fixed to 0x03 — Device Alarm Settings |
| 1 | U8 | Valid | 1 | Fixed to 0x00 — Health threshold, always valid |
| 2 | U16 | Cfg Len | 2 | Command length |
| 1*N | U8*N | [Bit7] Enable [Bit6 - 0] Threshold Type | 1*N | [Bit7] Whether to enable this type of threshold detection [Bit6 - 0] Set threshold type: 0x01 Heart Rate, 0x02 Systolic Blood Pressure (SBP) 0x03 Diastolic Blood Pressure (DBP), 0x04 Blood Oxygen, 0x05 Temperature (threshold unit is *10, for example, 37.3 is 373) |
| 2*N | I16*N | Threshold Min | 2*N | Threshold Lower Limit (this content is available only when threshold detection is enabled) |
| 2*N | I16*N | Threshold Max | 2*N | Threshold Upper Limit (this content is available only when threshold detection is enabled) |

Device Alarm Settings Definition - Health Threshold - Refer to 2.3 Device Downlink Description - Health Threshold Settings (0 xCE0300)

Note: The default status is off, with the threshold in the normal range. Outside this range, the device will report an alarm.

Heart Rate Threshold Range (20 - 200), Systolic Blood Pressure Threshold Range (20 - 200), Diastolic Blood Pressure Threshold Range (20 - 200), Blood Oxygen Threshold Range (80 - 100), Temperature Threshold Range (160 - 600)

Example:

Set multiple health thresholds: 1. Heart Rate [60, 120], 2. Disable Blood Oxygen Threshold Detection, 3. Temperature [36.0, 37.3]:BDBDBDBDCE03000B00813C007800048568017501FF

BDBDBDBD: Header

CE:Message ID

03:Type 03--Device Alarm Settings

00:Valid 00--Health Threshold, always valid

0B00:Convert to Big Endian-->000B-->Convert to Decimal11-->The following command report byte length (excluding checksum) 11bytes

81:Convert to Binary-->10000001   bit7:1---Enabled     Enabled State   bit6-0:000001-->0x01  Heart Rate

3C00:Convert to Big Endian-->003C-->Convert to Decimal60, the lower limit of heart rate threshold is60

7800:Convert to Big Endian-->0078-->Convert to Decimal120, the upper limit of heart rate threshold is120

04:Convert to Binary-->00000100   bit7:0---Disabled, no thresholds follow after disabled state   bit6-0:0000100-->0x04  Blood Oxygen

85:Convert to Binary-->10000101  bit7:1---Enabled     Enabled State   bit6-0:0000101-->0x05  Temperature

6801:Convert to Big Endian-->0168-->Convert to Decimal360(Actual Temperature *10)-->The lower limit of temperature threshold is36.0

7501: Convert to big-end -->0175--> Convert to decimal373(Actual temperature *10) --> Temperature threshold upper limit is37.3

FF: checksum

5.1.6 Settings - Switch Settings (0XCE04-24)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Please refer to the instructions for specific types |
| 1 | u8 | switch | -/- | -/- | Switch 00 ON, 02 OFF, 01 one-time effective |
| 2 | U16 |  | -/- | -/- | Fixed 0000 |

Device Alarm Settings - Switch Type: Specific Function Description -- See 2.3 Device Downlink Description

05 Bluetooth Broadcast Switch

07 Drop Alarm Switch        08 Stay Alarm Switch

0A Immediately Report Location Information    0B Immediately Report Health Data

0D Immediately report version number data  16 Can the power off feature be used via buttons

18 Sleep function switch        19 Button-triggered SOS alarm switch

20 Data reporting switch--Separate settings (note that this switch structure is special, and there are differences between message and table definitions, please refer to specific examples)

Type=05 Bluetooth broadcast switch
Example:

00---On:BDBDBDBDCE0500000093

BDBDBDBD:Header

CE : Message ID

05:Type 05 Bluetooth broadcast switch

00:switch switch 00 on

0000:fixed value

93:checksum

02---Off:BDBDBDBDCE0502000093

BDBDBDBD:Header

CE : Message ID

05:Type 05 Bluetooth broadcast switch

02: switch 02 is turned off

0000:fixed value

93:checksum

Type=07 Drop Alarm Switch
Example:

00---On: BDBDBDBDCE0700000093

BDBDBDBD:Header

CE : Message ID

07: Type 07 Drop Alarm Switch

02: switch is On 00

0000:fixed value

93:checksum

02---Off: BDBDBDBDCE0702000093

BDBDBDBD:Header

CE : Message ID

07: Type  07 Drop Alarm Switch

02: switch 02 is turned off

0000:fixed value

93:checksum

Type=08 Stay Alarm Switch
Example:

00---On: BDBDBDBDCE0800000093

BDBDBDBD:Header

CE : Message ID

08: Type 08 Stay Alarm Switch

02: switch is On 00

0000:fixed value

93:checksum

02---Off: BDBDBDBDCE0802000093

BDBDBDBD:Header

CE : Message ID

08:Type 08 Stay Alarm Switch

02: switch 02 is turned off

0000:fixed value

93:checksum

Type=0A Immediately Report Location Information
Example:

01---Single Report:BDBDBDBDCE0A01000093

BDBDBDBD:Header

CE : Message ID

0A:Type 0A Immediately Report Location Information

01:switch Switch 01--Single Issuance Single Report

0000:fixed value

93:checksum

Type=0B Immediately Report Health Data
Example:

01---Single Report:BDBDBDBDCE0B01000093

BDBDBDBD:Header

CE : Message ID

01:Type 0B Immediately Report Health Data

02:switch Switch 01--Single Issuance Single Report

0000:fixed value

93:checksum

Type=0D Immediately Report Version Number Data
Example:

01---Single Report: BDBDBDBDCE0D01000093

BDBDBDBD:Header

CE : Message ID

0D: Type 0D immediately reports the version number data

01:switch Switch 01--Single Issuance Single Report

0000:fixed value

93:checksum

Type=16 Can the key be used for shutdown in downlink? Example:

00---On: BDBDBDBDCE1600000093

BDBDBDBD:Header

CE : Message ID

16: Type 16 Can the key be used for shutdown in downlink

02: switch is On 00

0000:fixed value

93:checksum

02---Off: BDBDBDBDCE1602000093

BDBDBDBD:Header

CE : Message ID

16: Type 16 Can the key be used for shutdown in downlink

02: switch 02 is turned off

0000:fixed value

93:checksum

Type=18 Sleep function switch Example:

00---On, enters sleep mode after being still for a while, does not report data: BDBDBDBDCE1800000093

BDBDBDBD:Header

CE : Message ID

18: Type 18 Sleep function switch

02: switch is On 00

0000:fixed value

93:checksum

02---Off, in still or charging state, also reports data normally: BDBDBDBDCE1802000093

BDBDBDBD:Header

CE : Message ID

18:Type 18 Sleep Function Switch

02: switch 02 is turned off

0000:fixed value

93:checksum

Type=19 Key-Triggered SOS Alarm Switch
Example:

00---On:BDBDBDBDCE1900000093

BDBDBDBD:Header

CE : Message ID

19:Type 19 Key-Triggered SOS Alarm Switch

02: switch is On 00

0000:fixed value

93:checksum

02---Off:BDBDBDBDCE1902000093

BDBDBDBD:Header

CE : Message ID

19:Type 19 Key-Triggered SOS Alarm Switch

02: switch 02 is turned off

0000:fixed value

93:checksum

Type=20 Data Reporting Switch—Independently Configured
Example:

Add Field: Body Length: 2 BytesBody: 0x00 (Location Data Reporting) +Switch (0x00: On, 0x02: Off)

0x01 (Health Data Reporting) +Switch (0x00: On, 0x02: Off)

Location Data Reporting Enabled:BDBDBDBDCE20000200000093

BDBDBDBD:Header

CE : Message ID

20:Type 20 Data Reporting Switch

Fixed value

0200:Body length converted to big-endian 0002-->converted to decimal 02-->Body length is 2 bytes

00:Body:00--Position data reporting switch

00:Body:00--Enable reporting

93:checksum

Position data reporting off:BDBDBDBDCE2000020000293

BDBDBDBD:Header

CE : Message ID

20:Type 20 Data Reporting Switch

Fixed value

0200:Body length converted to big-endian 0002-->converted to decimal 02-->Body length is 2 bytes

00:Body:00--Position data reporting switch

02:Body:00--Disable reporting

93:checksum

Health data reporting on:BDBDBDBDCE20000200010093

BDBDBDBD:Header

CE : Message ID

20:Type 20 Data Reporting Switch

Fixed value

0200:Body length converted to big-endian 0002-->converted to decimal 02-->Body length is 2 bytes

01:Body:00--Health data reporting switch

00:Body:00--Enable reporting

93:checksum

Health data reporting off:BDBDBDBDCE20000200010293

BDBDBDBD:Header

CE : Message ID

20:Type 20 Data Reporting Switch

Fixed value

0200:Body length converted to big-endian 0002-->converted to decimal 02-->Body length is 2 bytes

01:Body:00--Health data reporting switch

02:Body:02--Disable reporting

93:checksum

5.1.7 Settings - Drop Sensitivity Settings (0xCE15)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Description |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Type is 0x15 |
| 1 | U8 | Valid | 1 | Default is 0, no impact at present |
| 2 | U16 | Len | 2 | Length of subsequent parameter content |
| 1 | U8 | Param Type | 1 | Parameter type: 0x00: Sensitivity, 0x01: Height |
| 1 | U8 | Param Level | 1 | Parameter level: 5 levels (0 - 4) |

Drop sensitivity settings - Parameter description:

Sensitivity: Refers to the degree of meeting the drop algorithm trigger, providing 5 settings levels (0 - 4): Low - Lower (Medium-Low) - Medium - Higher (Medium-High) - High.

Height: Refers to the height required to trigger the drop alarm, providing 5 settings levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m - 2.5m.

If the height is set to "1.5m", the device needs to be at least ≥ 1.5m to possibly trigger the fall alarm.

Note that sensitivity and height cannot be combined in one message.

Example:

Set sensitivity to "Medium-Low": BDBDBDBDCE150002000001FF

BDBDBDBD:Header

CE : Message ID

15:Type drop sensitivity setting

Fixed value

0200:Len

00:Param Type sensitivity setting

01:Param Level sensitivity level 1-->Lower (Medium-Low)

FF:checksum

Set height to "1.5m": BDBDBDBDCE150002000102FF

BDBDBDBD:Header

CE : Message ID

15:Type drop sensitivity setting

Fixed value

0200:Len

01:Param Type height setting

02:Param Level height level 2-->1.5m

FF:checksum

5.1.8 Settings - Vibration Settings (0XCE23)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Description |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Fixed to 0x23 |
| 1 | U8 | Valid | 1 | 0x00 |
| 2 | U16 | Cfg Len | 2 | Length of subsequent parameter content |
| 1 | U8 | Motor Type | 1 | Setting Type: 1. Vibration Duration - 0x00 |
| 4 | U32 | Value | 4 | Setting Content: 1. Vibration Duration (in milliseconds, range 0 - 60 seconds) |

Setting - Vibration Settings Definition: Downlink this message can modify the global vibration duration, any generated vibration duration will change to the duration set by this command

Example:

Set device default global vibration duration to 1.5 seconds (1500 milliseconds): BDBDBDBDCE2300050000dc050000FF

BDBDBDBD:Header

CE : Message ID

23: Type 23 Settings - Vibration Settings

00: Fixed value 00--always valid

0500:The length of the subsequent parameter content converted to big-endian 0005-->converted to decimal 05-->the length of the subsequent parameter content is 5 bytes

00:Motor Type vibration duration setting

dc050000:Value vibration duration converted to big-endian 000005dc-->converted to decimal 1500-->vibration duration is 1500 milliseconds (1.5 seconds)

FF:checksum

5.1.9 IP & Domain Name Settings (MSGID=0xC3) (TCP specific)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | Dispatch Type: Type =1 IPv4, Type =2 IPv6 (reserved item - not currently supported), Type =3 Domain Name (currently only supports ASCII encoded domain names) |
| 2 | U16 | Port |  |  | Port number (2 bytes) |
| 1 | U8 | Len |  |  | Length of subsequent parameter content |
| N | U8 | Domainname |  |  | Specific IP or domain name content |

Note: After dispatching, the device does not communicate on the original server, but points to the modified server dispatched.

Example:

Type = 1 IPv4: Modify the IP to: 118.178.184.219, port: 8825 BDBDBDBDC30179220476B2B8DB33

BDBDBDBD:Header

C3: Message ID

01:Type 1--IPv4

7922:Port The port converted to big-endian 2279 --> converted to decimal 8825 --> port: 8825

04:Len Length of the following parameter content converted to decimal 04 --> length is 4 bytes

76B2B8DB:Domain name IP 76 --> converted to decimal 118, B2 --> converted to decimal 178, B8 --> converted to decimal 184, DB --> converted to decimal 219 IP is: 118.178.184.219

33:checksum checksum

Type = 3 Domain name Modify the domain name to: aiday.com.cn, port: 8825

BDBDBDBD:Header

C3: Message ID

03:type=3, domain name

7922:Port The port converted to big-endian -- 2279 --> converted to decimal port: 8825

12:Len Length of the following parameter content converted to decimal 18 --> message length is 18 bytes

61696461792e636f6d2e636e:Domain name The domain name converted to string --> aiday.com.cn --> domain name is: aiday.com.cn

1D: checksum

5.1.10 Issuing Sedentary Stay Alarm Trigger Time (MSGID=0XCC)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xCC | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | minute | -/- | -/- | The duration for triggering the stay alarm (unit: minutes; range: 2 — 60). If there is no activity during this time, the stay alarm will be triggered. |

Example:

After 5 minutes of inactivity, the watch reports a stay alarm (while being worn): BDBDBDBDCC05DD

BDBDBDBD: Header

CC: Message ID

05: minute stay alarm trigger duration converted to decimal 05 --> 5 minutes

DD: checksum

5.1.11 Power Off and Restart (MSGID=0x77)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0x77 | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- |  | 01—Shutdown, 00—Restart |

Note: Shutdown and restart are both received in a powered-on state. Shutdown: The watch powers off upon receiving the command while on. It cannot receive any commands after shutdown; Restart: The watch restarts upon receiving the command while on.

Example:

Shutdown: BDBDBDBD770193

BDBDBDBD:Header

77: Message ID

01: Type 01—Shutdown

93:checksum

Restart: BDBDBDBD770093

BDBDBDBD:Header

77: Message ID

01: Type 00—Restart

93:checksum

5.1.12 Personal Information Transmission (MSGID=0xCA)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xCA | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | count | -/- |  | Distribute several sets of information |
| 1 | U8 | type | -/- |  | Type: 1 – Name, 2 – English Name, 3 – Phone, 4 – Blood Type |
| 1 | U8 | len | -/- |  | Length |
| N | U8 | content | -/- |  | Content (Unicode encoding) |
| ... | ... | ... | ... | ... | ... |

Example:

Name: Cheng Dawan, English Name: Chan Tai Man, Phone: 99886633, Blood Type: A

BDBDBDBDCA0401060B7A2759876502184300680061006E00200054006100690020004D0061006E0003103900390038003800360036003300330004024100DD

BDBDBDBD: Header

CA: Message ID

04: count converted to decimal04--> 4 sets of information

01:type 01--Name

06:len Convert to decimal06-->Length 6 bytes

0B7A27598765:content

0B7A converted to big-endian-->7A0B-->convert to Unicode character-->Cheng,

2759converted to big-endian-->5927-->convert to Unicode character-->Da,

8765converted to big-endian-->6587-->convert to Unicode character-->Wen  The Chinese name is Cheng Dawan

02:type 02--English name

18 len Convert to decimal24-->Length 24 bytes

4300680061006E00200054006100690020004D0061006E00:content

Similarly, each2bytes converted to big-endian-->converted to Unicode character-->Final: Chan Tai Man

03 : type 03 -- phone number

10:len Convert to decimal16--> length of 16 bytes

39003900380038003600360033003300:content

Similarly, each2bytes is converted to big-endian--> Convert to Unicode string --> Final: 99886633

04:type 04-- blood type

02:len Convert to decimal02--> length of 2 bytes

41: Convert to Unicode string-->A--> blood type: A

DD: checksum

5.1.13 Sleep Statistics Time Period Setting (MSGID=0X1D)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0x1D | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 1 |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 2 |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 3 |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |
| 1 | u8 | enable | -/- | -/- | Whether to enable, 1 for enabled, 0 for not enabled | Time Period 4 |
| 1 | u8 | time_start_h |  |  | -Hour |  |
| 1 | u8 | time_start_m |  |  | -Minute |  |
| 1 | u8 | time_end_h |  |  | -Hour |  |
| 1 | u8 | time_end_m |  |  | -Minute |  |  |

Example:

Sleep data statistics and reporting time periods are 13:00 - 15:00 and 22:30 - 08:00BDBDBDBD1D010D000F0001161E080000000000000000000000FF

BDBDBDBD:Header

1D:Message ID

01:enable whether to enable Convert to decimal 01-->1--Enabled

0D:time_start_h start time - hours Convert to decimal 13

00:time_start_m start time - minutes Convert to decimal 00 Starts counting and reporting time as 13:00

0F:time_end_h end time - hours Convert to decimal 15

00:time_end_m end time - minutes Convert to decimal 00 Ends counting and reporting time as 15:00

01:enable whether to enable Convert to decimal 01-->1--Enabled

16:time_start_h start time - hours Convert to decimal 22

1E:time_start_m start time - minutes Convert to decimal 30 Starts counting and reporting time as 22:30

08:time_end_h end time - hours Convert to decimal 08

00:time_end_m end time - minutes Convert to decimal 00 Ends counting and reporting time as 08:00

00:enable whether to enable  Convert decimal 00-->0--not enabled

00000000:All unused status filled with 00

00:enable whether to enable  Convert decimal 00-->0--not enabled

00000000:All unused status filled with 00

FF:checksum

5.1.14 Remote OTA Upgrade Issuance (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Type Count |
| 1 | u8 |  | -/- | -/- | Fixed Value (Reserved Byte 00) |
| 1 | u8 | Type | -/- | -/- | Type 1 00—MCU Firmware |
| 1 | U8 | PathLen | -/- | -/- | Length of Path 1 |
| n | N*u8 | Path | -/- | -/- | Path |
| 2 | U16 | PackageSize |  |  | File package size fixed at 0000 |
| ... | ... | ... | ... | ... | ... |
| 1 | u8 | Type | -/- | -/- | Type n 00—MCU firmware |
| 1 | U8 | PathLen | -/- | -/- | Path n length |
| n | N*u8 | Path | -/- | -/- | Path |
| 2 | U16 | PackageSize |  |  | File package size fixed at 0000 |
| ... | ... | ... | ... | ... | ... |

Note: Remote updates consume data and are affected by the network. The device SIM card has a monthly limit of 30MB, and excessive updates may lead to insufficient data.

Example:

The firmware download address (requires server support for HTTP downloads): http://tools.aiday.com.cn/File/MCU/W200PG/W200PG_E42.BWGHOL25.bin

After a successful update, the device will restart and reconnect to the server, reporting version number changes.

BDBDBDBDA901000041687474703A2F2F746F6F6C732E61696461792E636F6D2E636E2F46696C652F4D43552F5732303050472F5732303050475F4534322E425747484F4C32352E62696E00000A

BDBDBDBD:Header

A9:Message ID

01:TypeCnt Type count 01--indicates1type

Reserved byte, fixed value

00 :Type  00 -- Indicates MCU firmware upgrade

41 :Path  Download path length converted to decimal 65 --> Download path length is 65

687474703A2F2F746F6F6C732E61696461792E636F6D2E636E2F46696C652F4D43552F5732303050472F5732303050475F4534322E425747484F4C32352E62696E

File download path converted to string --> http:/tools.aiday.com.cn/File/MCU/W200PG/W200PG_E42.BWGHOL25.bin

0000 :PackageSize  Currently fixed at 0000

0 A: checksum

5.1.15 Weather Warning (MSGID=0XCB)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0xCB | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Count | -/- | -/- | How many weather warnings |
| 1 | U8 | Type | -/- | -/- | Weather warning type, see specific types below |
| 1 | U8 | Status | -/- | -/- | Status, see specific status below |
| … | … | …… |  |  | … |

Weather type-status table:

| Type | Description | Status | notes - click on the image to enlarge |
| --- | --- | --- | --- |
| 1 | Storm warning | 0 | Cancel alarm |
|  |  | 1 - 8 |  |
| 2 | Heavy rain warning | 0 | Cancel alarm |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |
| 3 | Thunderstorm Warning | 0 | Cancel alarm |
|  |  | 1 |  |
| 4 | Flood warning | 0 | Cancel alarm |
|  |  | 1 |  |
| 5 | Debris flow warning | 0 | Cancel alarm |
|  |  | 1 |  |
| 6 | Strong monsoon warning | 0 | Cancel alarm |
|  |  | 1 |  |
| 7 | Frost warning | 0 | Cancel alarm |
|  |  | 1 |  |
| 8 | Fire danger warning | 0 | Cancel alarm |
|  |  | 1 |  |
|  |  | 2 |  |
| 9 | Temperature warning | 0 | Cancel alarm |
|  |  | 1 |  |
|  |  | 2 |  |
| 10 | Tsunami warning | 0 | Cancel alarm |
|  |  | 1 |  |
| 11 | Heat index | 0 | Cancel alarm |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |

Note: After issuance, the device will vibrate and display weather warning images. Multiple sets of weather warnings can be issued simultaneously, and each set must be distinct. The same type of warning cannot appear in one command; otherwise, it will be invalid.  
Example:

10 sets of weather warnings issued: BDBDBDBD CB 0 A 0101 0201 0301 0401 0501 0601 0701 0801 0901 0 A 01 F 5

BDBDBDBD:Header

CB:Message ID

0A:Count weather group count in decimal10--> Dispatch10 groups of weather

0101:Type&Status weather type-status 01--Storm warning 01--Signal number 1 alert

0201:Type&Status weather type-status 02--Heavy rain warning 01--Yellow heavy rain warning signal

0301:Type&Status weather type-status 03--Thunderstorm warning 01--Thunderstorm alert

0401:Type&Status weather type-status 04--Flood warning 01--Special report of flooding in northern New Territories

0501:Type&Status weather type-status 05--Landslide warning 01--Landslide alert

0601:Type&Status weather type-status 06--Strong monsoon warning 01--Strong monsoon signal

0701:Type&Status weather type-status 07--Frost warning 01--Frost signal

0801:Type&Status weather type-status 08--Fire danger warning 01--Yellow fire danger warning

0901:Type&Status Weather Type-Status 09--Temperature Warning 01--Cold Weather Alert

0A01:Type&Status Weather Type-Status 10--Tsunami Warning 01--Tsunami Alert

F5:checksum Checksum

5.1.16 Restore Factory Settings (MSGID=0X7B)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: fixed to 0xBD 0xBD 0xBD 0xBD | 0x7B | See below for definitions | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | reset | -/- | -/- | 1 Restore Factory Settings |

Example:

Restore factory settings, the device reboots after receiving the command:BDBDBDBD7B018F

BDBDBDBD:Header

7B:Message ID

01:reset 01--Restore Factory Settings

8F:checksum Checksum