W300PW-WIFI data transmission - tcp communication protocol

Directory

1 Overview	4

2 Equipment Operating Instructions	4

2.1 Device Functions and User Instructions	4

2.2 Device default reporting logic	6

2.3 Device Downlink Description	8

3 Protocol Data Packet Structure	11

3.1 Data Header	11

3.2 Message Identifier (Message ID)	12

3.3 Token Generation Mechanism	12

3.4 Payload	12

3.5 Checksum	13

4 Reporting messages	15

4.1 Connection-related reports	15

4.1.1 LNK-LIN (MSGID=0xF0) connection request (TCP only)	15

4.1.2 Connection Response (MSGID=0xF1) (TCP only) - Important	16

4.1.3 New Heartbeat Packet Protocol (MSGID=0xF9) - Important	17

4.2 Alarm-related reporting	18

4.2.1 Alarm Data Upload-1 (MSGID=0x02)	19

4.2.2 Alarm data upload-2(MSGID=0x21)(Supplement to 02)	22

4.2.3 Abnormal data alarm (MSGID=0x16)	26

4.3 Location-related reporting	28

4.3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)	28

4.3.2 Wifi and Base Station Information Upload (MSGID=0xA4)	30

4.3.3 Bluetooth positioning information (BLE Location) (MsgId=0xD6)	34

4.4 Device Information and Status Reporting	36

4.4.1 Device Charging Status Upload (MSGID=0xC3)	36

4.4.2 Status Parameter Reporting (MSGID=0xA9)	38

4.4.3 Device Status (MSGID=0xE9)	40

4.5 Health-related Reporting	41

4.5.1 Health Data (MSGID=0x32)	41

4.5.2 Device Sleep Analysis Data Upload (MSGID=0xC5)	43

4.6 Downlink feedback related reports	44

4.6.1 Downlink feedback (MSGID=0xC0)	44

4.6.2 Message Status Report (MSGID=0x28)	45

5 Settings	46

5.1 Downlink	46

5.1.1 Set positioning reporting frequency (MSGID=0x17)	46

5.1.2 Message Sending (MSGID=0X28)	49

5.1.3 Setting - Positioning Priority Setting (0XCE01)	50

5.1.4 Settings - Health Sampling Report Frequency Setting (MSGID=0X1C)	52

5.1.5 Set - Device Alarm Settings (0XCE03)	55

5.1.6 Settings - Switch Settings (0XCE04-24)	57

5.1.7 Settings - Drop Sensitivity Setting (0xCE15)	64

5.1.8 Settings - Vibration Settings (0XCE23)	66

5.1.9 IP & Domain Name Settings (MSGID=0xC3) (TCP Dedicated)	67

5.1.10 Issuing sedentary stay alarm trigger time (MSGID=0XCC)	68

5.1.11 Power off and restart (MSGID=0x77)	69

5.1.12 Personal Information Distribution (MSGID=0xCA)	70

5.1.13 Sleep Statistics Time Period Settings (MSGID=0X1D)	71

5.1.14 Remote OTA upgrade issued (MSGID=0xA9)	74

Weather Alert (MSGID=0XCB)	75

5.1.16 Restore Factory Settings(MSGID=0X7B)	79

1 Overview

This agreement is suitable for WiFi devices, supporting the W300PW product. It uses a 32-bit data header for synchronization and terminal identification; employs a low-overhead checksum algorithm for validation protection; and utilizes message identifiers to distinguish different messages.
The parsing should be based on the actual report, and the document is currently under continuous maintenance. If any errors or omissions are found, please provide feedback in a timely manner. Thank you very much.
The protocol content concerns TCP report data, and for Bluetooth broadcast data protocols, please consult the relevant liaison personnel.

2 Equipment Operating Instructions

2.1 Device Functions and User Instructions

Universal Version:

(1) Power on:

Before using for the first time, please fully charge the device. It will automatically power on once the battery reaches the necessary charge for operation, and the charging status will be indicated by the charging icon.

Fully charged is indicated by a green icon. Note: Do not check device signal while charging.
Manual power on: Long press the top button for 10 seconds and release, the interface will display "Welcome".
Note: The power-on default is wearing status, and no heart rate detected will trigger a fall alert.

(2) Shut down:

Power off: The screen turns off after displaying "Byebye".

Manual shutdown: Press and hold the button for more than 10 seconds. The screen turns off after displaying "Byebye".

Shutdown via command: When powered on, send the shutdown command. The screen turns off after displaying "Byebye". Note: Charging while powered on will not initiate a shutdown.

(3) SOS function:

Trigger method: After triggering, the device will not enter sleep mode. Press and hold the button for 3 seconds. The screen displays "SOS SEND OK" / "SOS sent successfully", and the SOS icon appears.

Cancellation method: In SOS mode, press and hold the button for 3 seconds. The screen displays "SEND CANCEL" / "SOS cancelled", and the SOS icon disappears.

(4) Signal status:

No wifi connection: The signal on the screen is shown as a horizontal line.

Signal available: The signal on the screen is represented as a stepped bar graph.

(5) Device sleep:

Trigger condition: The device is inactive for 40 minutes, enters sleep mode, does not report positioning health data, and only reports F9 heartbeat packets to maintain a long connection. In wearing status, the device reports positioning and health data; in detached status, the device does not report.

(6) Device parameters: Settings - Parameter interface: There are multiple pages, which can be switched by scrolling down the screen

sos (Yes): Yes: Indicates that the device can trigger sos through the button

sos (No): Indicates that sos cannot be triggered through the button

Data reporting (Yes): Indicates that data will be reported currently

Allow work sleep (Yes):

Yes: Indicates that the device has turned on the sleep function, No: Indicates that the device has turned off the sleep function

Note: The sleep function means that the device does not report data in a detached state, and if the device is stationary for 40 minutes, it will not report data

Health (10 min): Indicates that the device has turned on the health reporting function,

10 min: Indicates that the current health data reporting frequency is 10 minutes,

Health (No): Indicates that the device health reporting function has been turned off

Location (B, 10): Indicates that the device has enabled the location reporting function,

B: Indicates that the priority of the location is single Bluetooth location,

10: Indicates that the current location reporting frequency is 10 minutes,

Location (No): Indicates that the device has disabled the location reporting function.

Drop (Low, 1.5 m): Indicates that the device has enabled the drop alarm trigger function,

Low: Indicates that the sensitivity for the drop alarm trigger is low,

1.5m: Indicates that the height for the drop alarm trigger is 1.5m,

Drop (No): Indicates that the device has disabled the drop alarm trigger function.

Prolonged Sitting (15 min): Indicates that the device has enabled the prolonged sitting alert trigger function,

15 min: Indicates that the time to trigger the prolonged sitting alert is 15 minutes of inactivity,

Sedentary (No): Indicates that the device has turned off the sedentary alert trigger function

Health Broadcast (Yes):

Yes: Indicates that the device has turned on Bluetooth broadcasting health data,

No: Indicates that the device has turned off Bluetooth broadcasting health data

Sleep reporting time: (21:00 - 08:00), indicating that the sleep reporting time is from 9 PM to 8 AM the next day

Global vibration duration (1.500s): The current device vibration duration is 1.5 seconds

Health alarm threshold: For example, heart rate threshold (90, 100) BPM, indicating that the normal heart rate range is 90-100,

Any value outside the normal range will report a health abnormal alarm, heart rate threshold (No): Indicates that the heart rate threshold is turned off

Network long connection (Yes):

Yes: Indicates that the current device TCP connection mode is long connection (maintains a connection state continuously, can receive downstream instructions in real time)

(8) Vibration:

Received a downstream text message, health threshold alarm (the interface has text reminders and vibrates)

The vibration duration can be controlled downstream (see section 2.3)

2.2 Device default reporting logic

Universal Version:

(1) Connection related reporting

F0 request: The device is a long link. Under normal connection and network conditions, it will report once upon startup -- a reply is needed to connect to the server

F9: Heartbeat package report, default report once every 4 minutes (previous firmware), also reports a message when location and health are reported -- a reply is needed to maintain a long connection

(2) Location related reporting

Satellite (GPS/BDS)/Wi-Fi/Bluetooth beacon: default reporting frequency 10 minutes (wearing status), default location priority: single Wi-Fi location

(3) Alarm related reporting

SOS alarm (0x02): User-initiated, triggering method see previous section

SOS Cancel (0 x02): The user actively triggers the cancellation; see the previous section for the triggering method.

Shutdown Alarm (0 x21): The device initiates a shutdown or shuts down due to low battery; the triggering method is detailed in the previous section.

Wear Disconnection Alarm (0 x02): The device determines wearing status based on health sampling reporting frequency. If heart rate is detected, it reports a wearing alarm; if no heart rate is detected, it reports a disconnection alarm.

Prolonged Sitting Alarm (0 x02): Default trigger for being stationary for 15 minutes (wearing status).

Fall Alarm (0 x02): The device falls freely from a certain height, triggering the fall algorithm.

Low Battery Alarm (0 x02): Triggered when the current battery level is less than or equal to 0.

Abnormal Temperature Alarm (0 x16): Reports an abnormal value when the device detects a temperature exceeding 40 degrees Celsius (the value has no reference significance).

Health Threshold Alarm (0 x2105): Default is no alarm; when the health data goes outside the threshold range after the downward health threshold, an alert is reported.

Note: When the health threshold alarm is reported, the device vibrates, and the screen displays the abnormal value.

(4) Health-Related Reporting

Step count, heart rate, body temperature & wrist temperature, blood pressure, blood oxygen (0 x32): Default reporting frequency is 10 minutes (wearing status).

Sleep (0 xC5): Statistical time period 21: 00-08: 00, during which sleep data will be reported based on the status.

(5) Device Information and Status Reporting

Status Parameter (0 xA9): One report during the power-on.

Device Status (0 xE9): One report during power-on, with another report when there is a change in reporting frequency.

Charging Status (0 xC3): Report when charging starts, ends, and when fully charged.

(6) Downstream Feedback

Downstream Feedback (0xC0): Report when the device receives a downstream command from the server.

Message Status Feedback (0x28): After receiving a downstream text message, the device clicks to accept or reject.

Note: The device reporting may involve encapsulation reporting, meaning one data packet contains multiple complete messages. Ensure nothing is overlooked; messages are complete and will not be interrupted in the next data packet.

Eg: BDBDBDBDD6000119A9CF610445270387BF452708A1BC44279D18B74427E518B7F9BDBDBDBDF9010000006400002800000019A9CF61CA

This data packet contains messages for Bluetooth positioning (0xD6) and battery signal (0xF9).

(0xD6) Bluetooth positioning: BDBDBDBDD6000119A9CF610445270387BF452708A1BC44279D18B74427E518B7F9

(0xF9) Battery signal: BDBDBDBDF9010000006400002800000019A9CF61CA

2.3 Device Downlink Description

General version: Downlink commands need to be continuously downlinked twice within 20 milliseconds to ensure success.

(1) Device positioning reporting frequency command (0x17):

Default reporting frequency: 10 minutes (wearing status); single Bluetooth positioning, maximum 1 minute, minimum 1440 minutes. After receiving the downlink command, the device will report data according to the time period and frequency specified by the command.

Data will not be reported outside the time period. For example: 00:00-18:00, 2-minute positioning report means no reporting outside this time period.

(2) Text message downlink command (0x28):

Unicode encoding, up to 40 Chinese characters; one Chinese character occupies 2 bytes, and one English letter occupies 2 bytes.

Message list - up to 50 historical messages can be saved (Note: Firmware without a message list can save up to 20 messages).

(3) Device positioning priority distribution (0xCE01):

Default positioning priority: When connected to wifi, indoor positioning cannot access satellites (GPS/BDS)

(4) Device health sampling frequency issuance (0x1C): — Note that it is different from W200P

The default health sampling frequency is 10 minutes (when worn), with a maximum of 2 minutes. After the device receives the downlink command, it follows the issued instructions.

Data reporting for time periods and frequency will not be reported outside of the specified period. For example: 00:00-18:00, if there is a 10-minute positioning report, then there will be no reporting outside of this time period.

(5) Modify the IP and port command issuance (0xC3):

The default general version points to the Smart Cloud Platform: 118.178.184.219:8825. If you need to change it, please consult the relevant liaison personnel or visit the official website.

(6) Sedentary Alert Trigger Time Issuance (0xCC):

Value range: 2 minutes - 60 minutes. For example, if the following is a 10-minute trigger time, the device will trigger a sedentary stay alert if it remains motionless for 10 minutes.

(7) Device shutdown and restart command (0x77):

Note that this instruction will only take effect when the device is powered on; it cannot receive signals when turned off.

Device Shutdown Command: Executing this command while the device is powered on will turn it off, and it will not be able to receive subsequent commands after shutdown.

Reboot Command: Executing this command while the device is powered on will restart the device.

(8) Drop Sensitivity and Height Settings (0xCE15):

Latest universal firmware default drop sensitivity: low (medium-low). Drop height: 1.5m

Sensitivity: Refers to the degree of compliance with the drop algorithm, providing 5 setting levels (0 - 4): Low - Medium-Low - Medium - Medium-High - High.

Height: Refers to the height that triggers the drop alarm, providing 5 setting levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m - 2.5m.

(9) Remote OTA Command (0xA9):

Remote updates consume data and are affected by network conditions. The device's SIM card has a monthly limit of 30MB, so it cannot be updated too many times,

otherwise it may result in insufficient data situation. This functionality requires hardware support for remote updates; older devices do not support this.

(10) Key Shutdown Switch Command Issued (0xCE16):

The default device can be powered off by pressing a key. After the shutdown, the device cannot be powered off by pressing a key.

(11) Control device to trigger sleep switch issue (0xCE18):

By default, it is in the on state; the device enters sleep mode after 40 minutes of inactivity. After shutdown, the device does not enter sleep mode.

(12) Drop alarm switch (0xCE07):

Default drop alarm is in the on state; after being turned off, the device does not report drop alarms.

(13) Key-triggered SOS activation switch (0xCE19)

By default, it is in the on state. Long pressing the charging cable button can trigger SOS. After being turned off, long pressing the charging cable button will not trigger SOS reporting.

(14) Device health and location reporting switch (0xCE20)

Default health reporting is in the on state, default location reporting is in the on state.

You can separately set the status of location/health to on or off; after being turned off, the device does not report health/location.

(15) Health Threshold Setting (0xCE0300)

The health threshold is issued, and the downlink command is sent. After the device receives it, the issued health threshold is within the normal range. Health data outside this range will trigger an alarm.

(16) Device vibration setting (0xCE23)

After the downlink command is received by the device, the duration of device vibration changes to the commanded duration, ranging from 0 to 60 seconds.

(17) Immediate location reporting (0xCE0A)

After the downlink command is received by the device, it immediately starts locating and reports the position.

(18) Immediate health reporting (0xCE0B)

After the downlink command is received by the device, it immediately starts health sampling and reports the data.

(19) Sleep statistics time period issued (0x1D)

Different time periods can be issued, for example: 13:00-14:00, during which the sleep statistics will be compiled and reported.

(20) Personal information issued (0xCA)

Default: no personal information, can upload Chinese name, English name, blood type, contact phone number, GB2312 encoding,

Maximum byte count: Name 40 bytes, English name 40 bytes, Phone 40 bytes, Blood type 10 bytes

(21) Sedentary Alert Switch (0xCE08)

Default is on, trigger time is 15 minutes; after disabling, the device will not report sedentary alerts

(22) Weather Alert Dispatch (0xCB)

Multiple groups of weather alerts can be issued; note that each group must be different and cannot have the same type of alert

(23) Restore Factory Settings (0x7B):

After issuance, the device will revert to the default reporting status

3 Protocol Data Packet Structure

A basic protocol data packet structure is shown in Figure 1:

3.1 Data Header

Each data packet starts with a 4-byte Header or token (in some response messages, timestamp is used instead):

Currently, the token uploaded by Eufo devices is fixed at BDBDBDBD

Header: 0xBD 0xBD 0xBD 0xBD;

Timestamp: 32 bits, generated by the server

3.2 Message Identifier (Message ID)

The content represented by MessageId is detailed in Chapter 4.

Every time a TCP connection is established, the device side first reports the 0xF0 message, which contains the device's unique identifier IMEI.

The server needs to record this IMEI as identification and reply with the 0xF1 message; only then will the device side consider this connection successful, otherwise it will disconnect.

Under normal communication, the long connection defaults to reporting a 0xF9 heartbeat packet every 4 minutes, and the 0xF9 heartbeat packet will be reported once after the health position is reported.

In a short connection, the 0xF9 heartbeat packet is reported once on startup, and once again after the health position is reported; the reporting of the two does not affect each other.

Note that after the server receives the F9 message, it must issue a command in response; otherwise, the connection will be disconnected.

3.3 Token Generation Mechanism

Currently fixed as BDBDBDBD

3.4 Payload

Here, the payload refers to the valid body content in the protocol, excluding the head token and the checksum. The following indicates the length of the body.

The data format used in the payload is shown in the table below:

[U-unsigned; I-signed; X-bitfield; Number-bytes occupied]

In the following protocol, all data formats except for ch, u8, i8, and x8 use little-endian format.

| Short | peTypeType | Size(Bytes) | Min/max | Resolution | Explanation |
| --- | --- | --- | --- | --- | --- |
| CH | ASCII/ISO 8859.1 | 1 | - | - | Character |
| u8 | Unsigned Char | 1 | 0-255 | 1 | Unsigned Short Integer |
| i8 | Signed Char | 1 | -128-127 | 1 | Short |
| x8 | Bitfield | 1 | - | - | Bit |
| u16 | Unsigned Short | 2 | 0-65,535 | 1 | Unsigned Integer |
| i16 | Unsigned Short | 2 | -32,768-32,767 | 1 | Integer |
| x16 | Bitfield | 2 | - | - | Bit (bit) 2 |
| u32 | Unsigned Long | 4 | 0..4,294,967,295 | 1 | Unsigned Long |
| i32 | Signed Long | 4 | -2,147,483,648-2,147,483,647 | 1 | Long integer |
| u64 | Uint64_t | 8 | 0-18,446,744,073,709,551,616 | 1 | Unsigned 64-bit long integer |
| float | float | 4 | -3.4410e38-3.410e38 | - | Floating point |

3.5 Checksum

The checksum and the added content include the payload, as shown in Figure 1. The algorithm is as follows. By default, general version devices do not strictly require a checksum, and this part can be ignored; when sending downward commands, any one byte will suffice.

// Assuming content="BDBDBDBDE9000600010A00000A00", the result of sum.ToString("X2") is 07// Below is the C# method for calling CheckSum(string content)

{

int sum = 0;

var bytes = Utility.strToHexByte(content);

foreach (var b in bytes)

{

sum += b;

sum %= 0x100;

}

sum =     0xff     - sum;

return     sum.ToString(     "X2"     );

}

4 Reporting messages

4.1 Connection-related reports

4.1.1 LNK-LIN (MSGID=0xF0) connection request (TCP only)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xF0 | See below见下方定义 | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 8 | u64 | IMEI | 1 | - | IMEI number (little-endian mode) |
| 2 | x16 | version |  | - | Bitfield see below |

The device's F0 request message must have a server downlink 0XF1 message (specific format see next section) response; otherwise, login will fail.

Example:

Raw 16 hexadecimal message: BDBDBDBDF09B51731BC61603000000C2

BDBDBDBD: 4 bytes message header Header

F0: Message ID (message id) 9 B51731BC6160300: IMEI number (little-endian mode), converted to big-endian: 000316 C61B73519B,

Corresponding decimal for IMEI: 869465050010011, 0000: version

C2: checksum

Connection-related Q&A:

When TCP creates a new connection each time, it will first report the F0 request, which carries the IMEI. The server then records this IMEI,

and all data within this connection will correspond to this IMEI.

Note: If F0 is not received or there is an exception in F0 communication, you can use a third-party network testing tool to verify whether the server communication is normal.

4.1.2 Connection Response (MSGID=0xF1) (TCP only) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Timestamp (unix) timestamp | 0xF1 | See below for definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 4 | u32 | Token | 1 | - | Token: Fixed as BDBDBDBD |

Note that the message sequence is different from the normal reporting message; this message is sent by the server and is not reported content from the device. The server reply needs to be sent back on the current channel.

Example:

Raw hexadecimal message: 07FD8860F1BDBDBDBD2E

07FD8860 (little-endian), Timestamp (unix) timestamp (unit: seconds), converted to big-endian 6088FD07, converted to decimal: 1619590407

Converted to standard time format UTC time: 2021-04-28 06:13:27

F1: Message ID (message id):

BDBDBDBD: Token

2E: checksum

The timestamp in decimal converted to standard time format is obtained by adding 1619590407 seconds to 1970-01-01 00:00:00.

The device uses the timestamp (unit: seconds) in this message to synchronize time.

Note: In Java, the unit is generally milliseconds, be sure to convert to unit: seconds.

Note: The total number of bytes in the server's downlink reply is 10 bytes, for example: 0xBD--1 byte; if the device receives 20 bytes, a connection will not be established.

4.1.3 New Heartbeat Packet Protocol (MSGID=0xF9) - Important

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xF9 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Bat_type |  |  | Battery type 0: 4-level, 1: 5-level, 2: Percentage, 3: Voltage value |
| 2 | u16 | Bat_volt |  | -/- | Battery value If Bat_type is 0: then the battery value range is 0-3 (0 is 25%, 3 is 100%) If Bat_type is 1: then the battery value range is 0-4 (0 is 20%, 4 is 100%) If Bat_type is 2: then the battery value range is 0-100 |
| 1 | U8 | Signal_type |  |  | Signal type 0: Percentage, 1: 5-level system, 2: CSQ value |
| 2 | I16 | Signal_strength |  |  | Signal strength |
| 1 | U8 | Other_type |  |  | Extended type 0: Full step counting 1: Incremental step counting 2: Vibration |
| 4 | U32 | Num |  |  | Extended value |
| 4 | U32 | Timestamp | -/- | -/- | UTC timestamp (in seconds) |

Note: The F9 heartbeat packet must receive a server downlink reply. The device only considers the server link to be active after receiving the server's downlink reply; otherwise, it will resend the F0 request to connect.

Example:

Raw hexadecimal message: BDBDBDBDF90104000050000095000000E377BD67AA

BDBDBDBD: Header

F9: Message ID (message ID) 01: Bat_type battery type 01 --> 1 --> Level 5 (0-4) 0400: Bat_volt battery value converted to big-endian 0004 --> 4 --> Level 4 corresponds to 100% battery 00: Signal_type signal type 00 --> 0 --> Percentage 5000: Signal_strength signal strength converted to big-endian 0050 --> converted to decimal 80 --> 80% signal strength 00: Other_type extended type 00 --> 0 --> Total step count 95000000: Num extended value - step count converted to big-endian 00000095 --> converted to decimal 149 --> step count is 149

E377BD67: Timestamp converted to big-endian 67 BD77E3 --> converted to decimal 1740470243 --> timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

AA: checksum

Note: The server can respond to the F9 heartbeat packet with a fixed reply message: Fixed reply example BDBDBDBDF301  Byte count: 6

4.2 Alarm-related reporting

4.2.1 Alarm Data Upload-1 (MSGID=0x02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x02 | See below for definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 2 | x16 | Upl_warn | - | - | Alarm content see below 定义 |
| 4 | U32 | Timestamp | - | - | Timestamp (will be added at the end when re-uploaded) |

Upl_warn contents

| bit (corresponds to the position of 1 in binary) | Name | Description | Original message in little-endian hexadecimal | Converted to big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 14 | Drop Alarm | Drop Alarm | 0040 | 4000 | 4*4096=16384 |
| 8 | Device Wearing | Device Wearing | 0001 | 0100 | 1*256=256 |
| 7 | SOS Cancel | SOS Cancel | 8000 | 0080 | 8*16=128 |
| 5 | Long Sitting Alarm (Inactive) | Long Sitting Alarm (Inactive) | 2000 | 0020 | 2*16=32 |
| 4 | Remove (Fall Off) Device | Remove (Fall Off) Device | 1000 | 0010 | 1*16=16 |
| 2 | Power off | Power off | 0400 | 0004 | 4 |
| 1 | SOS alarm | SOS alarm | 0200 | 0002 | 2 |
| 0 | Low battery | Low battery | 0100 | 0001 | 1 |

Example:

Wear alarm: BDBDBDBD020001E377BD678A

BDBDBDB: Header

02: Message ID (消息id)

0001: Upl_warn alarm content, converted to big-endian: 0100 --> Wear alarm

E377BD67: Timestamp time stamp converted to big-endian 67 BD77E3 --> converted to decimal 1740470243 --> Timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

8a: checksum

Detachment alarm: BDBDBDBD021000E377BD678A

BDBDBDB: Header

02: Message ID (消息id)

1000 : Upl_warn alarm content, converted to big-end 0010 --> disconnection alarm

E377BD67: Timestamp time stamp converted to big-endian 67 BD77E3 --> converted to decimal 1740470243 --> Timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

8a: checksum

SOS Alarm: BDBDBDBD020200E377BD678A

BDBDBDB: Header

02: Message ID (消息id)

0200:Upl_warn alarm content, converted to big-endian0002-->SOS Alarm

E377BD67: Timestamp time stamp converted to big-endian 67 BD77E3 --> converted to decimal 1740470243 --> Timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

8a: checksum

SOS Cancel: BDBDBDBD028000E377BD678A

BDBDBDB: Header

02: Message ID (消息id)

8000:Upl_warn alarm content, converted to big-endian0080-->SOS Cancel

E377BD67: Timestamp time stamp converted to big-endian 67 BD77E3 --> converted to decimal 1740470243 --> Timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

8a: checksum

Fall Alarm: BDBDBDBD020040E377BD678A

BDBDBDB: Header

02: Message ID (消息id)

0040:Upl_warn alarm content, converted to big-endian4000-->Fall Alarm

E377BD67: Timestamp time stamp converted to big-endian 67 BD77E3 --> converted to decimal 1740470243 --> Timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

E7: checksum verification

Sitting Too Long Alarm: BDBDBDBD022000E377BD678A

BDBDBDB: Header

02: Message ID (消息id)

2000:Upl_warn alarm content, converted to big-endian0020-->Sitting Too Long Alarm

E377BD67: Timestamp time stamp converted to big-endian 67 BD77E3 --> converted to decimal 1740470243 --> Timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

8A: checksum verification

Low battery alarm: BDBDBDBD020100E377BD678A

BDBDBDB: Header

02: Message ID (消息id)

0100:Upl_warn alarm content, converted to big-endian0001-->Low battery alarm

E377BD67: Timestamp time stamp converted to big-endian 67 BD77E3 --> converted to decimal 1740470243 --> Timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

8A: checksum verification

Shutdown alarm: BDBDBDBD020400E377BD678A

BDBDBDB: Header

02: Message ID (消息id)

0400:Upl_warn alarm content, converted to big-endian0004-->Shutdown alarm

E377BD67: Timestamp time stamp converted to big-endian 67 BD77E3 --> converted to decimal 1740470243 --> Timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

8A: checksum verification

Note: This alarm is used for devices that do not differentiate shutdown types; devices that differentiate shutdown types are mentioned in the next section

4.2.2 Alarm data upload-2(MSGID=0x21)(Supplement to 02)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x21 | See below for definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Drscription |
| --- | --- | --- | --- | --- | --- |
| 2 | U16 | Alarm type |  |  | Alarm type |
| N |  | Upl_warn | - | - | Alarm content See below for different Alarm type definitions |
| 4 | U32 | Timestamp |  |  | Timestamp (a timestamp will be added at the end during retransmission) See different definitions for Alarm type; not necessarily at the end |

Alarm type=1

Upl_warn contents

| bit (corresponds to the position of 1 in binary) | Name | Description | Original message in little-endian hexadecimal | Converted to big-endian hexadecimal | Decimal |
| --- | --- | --- | --- | --- | --- |
| 2 | Charging shutdown alarm | Device charging automatically shuts down (firmware needs to support this function) | 04000000 | 0004 | 4 |
| 1 | Low battery shutdown alarm | Device shuts down due to extremely low battery (firmware needs to support this function) | 02000000 | 0002 | 2 |
| 0 | Manual shutdown alarm | Device has been manually shut down (firmware needs to support this function) | 01000000 | 0001 | 1 |

Example:

Charging shutdown alarm: BDBDBDBD21010004000000E377BD6767

BDBDBDBD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001-->Alarm type=1

04000000:Upl_warn content, converted to big-endian 0004-->Charging shutdown alarm

E377BD67:Timestamp, converted to big-endian 67BD77E3-->Converted to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

67:Checksum

Low battery shutdown alarm: BDBDBDBD21010002000000E377BD6767

BDBDBDBD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001-->Alarm type=1

02000000:Upl_warn content, converted to big-endian 0002-->Low battery shutdown alarm

E377BD67:Timestamp, converted to big-endian 67BD77E3-->Converted to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

67:Checksum

Active shutdown alarm: BDBDBDBD21010001000000E377BD6767

BDBDBDBD:Header

21:Message ID

0100:Alarm type, converted to big-endian 0001-->Alarm type=1

01000000:Upl_warn content, converted to big-endian 0001-->Active shutdown alarm

E377BD67:Timestamp, converted to big-endian 67BD77E3-->Converted to decimal 1740470243-->Timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

67:Checksum

Alarm type=5

Upl_warn contents

| Format | Name | Unit | Description |
| --- | --- | --- | --- |
| U16 | Type | 2 | Fixed to 0x05 |
| U32 | Timestamp | 4 | Timestamp |
| U16 | Content Len | 2 | Alarm Content Length |
| U8 | Health Warn Type1 | 1 | Health Threshold Alarm Type: 0x01 Heart Rate, 0x02 Systolic Blood Pressure (SBP), 0x03 Diastolic Blood Pressure (DBP), 0x04 Blood Oxygen, 0x05 Temperature |
| U8 | Type1 | 1 | 01 Less than the set health threshold, 02 Greater than the set health threshold |
| I16 | Health Value1 | 2 | Health Outcome Values |
| U8 | Health Warn Type2 | 1 | Health Threshold Alarm Type: 0x01 Heart Rate, 0x02 Systolic Blood Pressure (SBP), 0x03 Diastolic Blood Pressure (DBP), 0x04 Blood Oxygen, 0x05 Temperature |
| U8 | Type2 | 1 | 01 Less than the set health threshold, 02 Greater than the set health threshold |
| I16 | Health Value2 | 2 | Health Outcome Values |
| ...... | ...... | ...... | ...... |

Example:

The device detected that the heart rate ( 65 ) is outside the issued health threshold: BDBDBDBD210500E377BD6704000102410073

BDBDBDBD: Header

21: Message ID

0500: Alarm type converted to big-endian0005--> Alarm type=5

E377BD67: Timestamp converted to big-endian67BD77E3--> converted to decimal 1740470243 --> Timestamp is 1740470243 seconds -->

Standard Time Format Conversion UTC Time: 2025-02-25 07:57:23 --> Converted to Beijing Time: 2025-02-25 15:57:23

0400:Content Len alarm content length to big-end0004-->Alarm content length is 4 bytes. Note: multiple health threshold alarms will increase the length.

01:Health Warn Type health threshold alarm type01-->Heart rate alarm

02:Type greater than the set health threshold & less than the set health threshold01-->Less than the set health threshold

4100:Health Value actual reported health value to big-end0041-->Converted to decimal 65 -->Reported heart rate value is 65

73:checksum

4.2.3 Abnormal data alarm (MSGID=0x16)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x16 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type | -/- | -/- | Alarm type (0: heart rate abnormal value alarm, 1: temperature abnormal value alarm) |
| 2 | U16 | Heart | -/- | -/- | Heart rate value |
| 2 | U16 | temperature | -/- | -/- | Temperature |
| 2 | U16 | Pa | -/- | -/- | Pressure |
| 4 | 4*U8 or U32 | expand | -/- | -/- | 4-byte expansion for other sensors |

Note: The reported abnormal values have no actual reference significance; they simply indicate that abnormal data has been detected (such as values exceeding the range or sensor malfunction, etc.). Abnormal values will be removed from other normal messages and will be reflected in this message, which does not need to be parsed.

Example:

Temperature abnormal value alarm: BDBDBDBD16010000FF0F000000000000E6

BDBDBDBD : Header

16:Message ID

01: Type alarm type 01 --> Temperature abnormal value alarm

0000: Heart rate value, default to 0000 if this value is not present

FF0F: Temperature, convert to big-endian 0FFF --> Temperature abnormal value FFF

0000: Atmospheric pressure, default to 0000 if this value is not present

00000000 expand, default to 0000 if this value is not present

E6: Checksum

4.3 Location-related reporting

4.3.1 GPS/BDS location reporting: Location data reporting (MSGID=0x03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x03 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 8 | Double | lon |  | - | longitude |
| 8 | Double | lat |  | - | latitude |
| 1 | U8 | north_south |  | - | /N or S/ |
| 1 | U8 | east_west |  | - | /E or W/ |
| 1 | U8 | status |  | - | /A or V/ Valid data or invalid data |
| 4 | U32 | Timestamp |  | - | Timestamp |

Example:

Original16Hexadecimal Message: BDBDBDBD03963E870C3E675E40FF2110C2B6343F404E4541E377BD6799

BDBDBDBD: Header

03: Message ID

963E870C3E675E40: lon Longitude  Convert to Big Endian405E673E0C873E96--> Longitude:121.6131622  Note: Conversion analysis reference example below

FF2110C2B6343F40: lat Latitude  Convert to Big Endian403F34B6C21021FF--> Latitude:31.2059137  Note: Conversion analysis reference example below

4E: north_south  4E--> Convert to ASCII Code N--> North Latitude

45: east_west 45--> Convert to ASCII Code E--> East Longitude

41: status Data Validity  41--> Convert to ASCII Code A--> Valid Data

E377BD67: Timestamp Convert to Big Endian67BD77E3--> Convert to Decimal1740470243--> Timestamp:1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

78: checksum

Note: The extracted latitude and longitude of the satellite (GPS/BDS) are in the WGS-84 coordinate system. If the map uses Baidu or Amap, the coordinate system needs to be converted.

Java code example for parsing satellite positioning messages:

// Message: BDBDBDBD037d9f84ac81815c40e766926b1d8936404e4541749d695f0b//BDBDBDBD03 7d9f84ac81815c40 e766926b1d893640 4e 45 41 749d695f 0b public static void main(String[] args){

// Message 7d9f84ac81815c40 actual value 405c8181ac849f7d

Double.longBitsToDouble(Long.parseLong("405c8181ac849f7d", 16)));   //114.02353966666665 // Message e766926b1d893640 actual value 4036891d6b9266e7

Double.longBitsToDouble(Long.parseLong("4036891d6b9266e7", 16)));   //22.535605166666667

HexToStr(data.Substring("4e"));    //N

HexToStr(data.Substring("45"));    //E

HexToStr(data.Substring("41"));    //A A indicates valid data, while V indicates invalid data and can be disregarded // Message 749d695f actual value 5f699d74

Date date= new Date();

date.setTime(Long.parseLong("5f699d74", 16) * 1000);

SimpleDateFormatsdf = new SimpleDateFormat("yyyyMMddHHmmss");

System.out.println(sdf.format(date));  //2020-09-22 14:45:08

}

4.3.2 Wifi and Base Station Information Upload (MSGID=0xA4)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xA4 | See below for definition | CK_sum |

Payload contents

| Byte Offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 1 | u8 | Cell_cnt | 1 | - | Number of base stations (0-7) Number of cell info payload. Valid value: 0~7 |
| 2 | u16 | Cell[0].MCC | - | - | Mobile Country Code of Base Station (0) mobile country code of cell[0] |
| 2 | u16 | Cell[0].MNC | - | - | Mobile Network Code of Base Station (0) mobile network code of cell[0] |
| 2 | u16 | Cell[0].LAC | - | - | Location area code of cell[0] |
| 4 | U32 | Cell[0].CELL_ID | - | - | Cell id of cell[0] |
| 2 | i16 | Cell[0].RSSI | - | dbm | RSSI in dbm of cell[0] |
| 。。。 | 。。。 | 。。。 | 。。。 | 。。。 | 。。。 |
| 2 | u16 | Cell[cell_cnt-1.MCC | - | - | mobile country code of cell[0] |
| 2 | u16 | Cell[cell_cnt-1.MNC | - | - | Mobile Network Code of Cell [0] of Base Station (N) |
| 2 | u16 | Cell [cell_cnt-1].LAC | - | - | Location Area Code of Cell [0] of Base Station (N) |
| 2 | u16 | Cell [cell_cnt-1].LAC | - | - | Location Area Code of Cell [cell_cnt-1] of Base Station (N) |
| 4 | U32 | Cell [cell_cnt-1].CELL_ID | - | - | Cell ID of Cell [cell_cnt-1] of Base Station (N) |
| 2 | I16 | Cell [cell_cnt-1].RSSI | - | dbm | Signal Strength RSSI in dBm of Cell [cell_cnt-1] of Base Station (N) |
| 1 | U8 | Wifi_cnt |  |  | Number 0f wifi |
| 6 | 6*U8 | Wifi[0].bssid[0]-Wifi[0].bssid[5] |  |  | MAC address of wifi(0) |
| 4 | I32 | Wifi[0].RSSI |  |  | Signal strength RSSI of wifi(0) |
| 。。。 | 。。。 | 。。。 | 。。。 | 。。。 | 。。。 |
| 6 | 6*U8 | Wifi[Wifi_cnt-1].bssid[0]-Wifi[Wifi_cnt-1].bssid[5] |  |  | MAC address of wifi(N) |
| 4 | I32 | Wifi[0].RSSI |  |  | Signal strength RSSI of wifi(0) |

Note: The number of base stations reported by the device may not always be present. The absence of base stations does not affect positioning. If the number of base stations in the message is 0, the following directly indicates the number of wifi.

Example:

Original16Hexadecimal message: BDBDBDBDA4E377BD6701CC010000C218D1AD160BF9FF078CBEBE1A8162C6FFFFFFC061180AF42AC1FFFFFF200BC726E000B6FFFFFFA8154DF6517EB2FFFFFFE005C5B1F824CCFFFFFFE8FCAFA02663AFFFFFFF6409805B2B9CAEFFFFFF94

BDBDBDB: Header

A4: Message ID

E377BD67: Timestamp time stamp converted to big-endian 67 BD77E3 --> converted to decimal 1740470243 --> Timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

01: Cell_cnt Number of base stations 01--> Number of base stations is1

CC01: Cell[0].MCC Base station MCC converted to big-endian01CC --> Converted to decimal460--> Base station MCC is460

0000: Cell[0].MNC Base station MNC converted to big-endian0000--> Converted to decimal0--> Base station MNC is0

C218: Cell[0].LAC Base station LAC converted to big-endian18C2 --> Converted to decimal6338--> Base station LAC is6338

D1AD160B: Cell[0].CELL_ID Base station ID converted to big-endian0B16ADD1 --> Converted to decimal186035665--> Base station ID is186035665

F9FF: Cell[0].RSSI Base station signal strength converted to big-endian FFF9 --> Converted to decimal Two's complement -7--> Base station signal strength is -7

07: Wifi_cnt Number of wifi 07-->7 wifi

8CBEBE1A8162: wifi MAC address 8C:BE:BE:1A:81:62--> wifi[0]’s MAC address is8C:BE:BE:1A:81:62

C6FFFFFF: wifi signal strength RSSI converted to big-endian FFFFFFFC6 --> Converted to decimal Two's complement -58---> wifi[0]’s RSSI signal strength is -58

C061180AF42A: wifi MAC address C0: 61 : 18 : 0 A:F4: 2 A --> The MAC address of wifi[ 1 ] is C0: 61 : 18 : 0 A:F4: 2 A

C1FFFFFF: wifi signal strength RSSI converted to big-endian FFFFFFC1 --> converted to decimal two's complement - 63 ---> The RSSI signal strength of wifi[ 0 ] is - 63

200 BC726E000: wifi MAC address 20 : 0 B:C7: 26 : E0 : 00 --> The MAC address of wifi[ 2 ] is 20 : 0 B:C7: 26 : E0 : 00

B6FFFFFF: wifi signal strength RSSI converted to big-endian FFFFFFB6 --> converted to decimal two's complement - 74 ---> The RSSI signal strength of wifi[ 0 ] is - 74

A8154DF6517E: wifi MAC address A8: 15 : 4 D:F6: 51 : 7 E --> The MAC address of wifi[ 3 ] is A8: 15 : 4 D:F6: 51 : 7 E

B2FFFFFF: wifi signal strength RSSI converted to big-endian FFFFFFB2 --> converted to decimal two's complement - 78 ---> The RSSI signal strength of wifi[ 0 ] is - 78

E005C5B1F824: wifi MAC address E0: 05 : C5:B1:F8: 24 --> The MAC address of wifi[ 4 ] is E0: 05 : C5:B1:F8: 24

CCFFFFFF: wifi signal strength RSSI converted to big-endian FFFFFFCC --> converted to decimal two's complement - 52 ---> The RSSI signal strength of wifi[ 0 ] is - 52

E8FCAFA02663: wifi MAC address E8:FC:AF:A0: 26 : 63 --> The MAC address of wifi[ 5 ] is E8:FC:AF:A0: 26 : 63

AFFFFFFF: wifi signal strength RSSI converted to big-endian FFFFFFAF --> converted to decimal two's complement - 81 ----> The RSSI signal strength of wifi[ 0 ] is - 81

6409805 B2B9C: wifi MAC address 64:09:80:5B:2B:9C --> The MAC address of wifi[6] is 64:09:80:5B:2B:9C

AEFFFFFF: wifi signal strength RSSI converted to big-end FFFFFFAE --> converted to decimal two's complement - 82 ---> wifi[ 0 ]'s RSSI signal strength is - 82

94: checksum

Parameters parsed from the message need to be interfaced with the map's wifi database to ultimately obtain latitude and longitude data. For example, integrating the WiFi database with Amap:

Amap categorizes the positioning scenarios into three types based on the network access method stated in the Amap WiFi positioning protocol document. Below are examples for the three scenarios:

1. The access method is WiFi, i.e., the accesstype parameter is 1

http://apilocate.amap.com/position?accesstype=1&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&mmac=50:a7:2b:1e:56:58,-60,alibaba-inc&macs=4c:48:da:25:0b:11,-59,alibaba-

inc|4c:48:da:25:1a:11,-77,alibaba-inc&serverip=10.2.166.4&output=xml&key=<User Key>

2. The access method is mobile access and the SIM card is non-CDMA, i.e., the accesstype parameter is 0 and cdma=0:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=0&bts=460,01,40977,2205409,-65&nearbts=460,01,40977,2205409,-65|460,0

1,40977,2205409,-65|460,01,40977,2205409,-65&serverip=10.2.166.4&output=xml&key=<User Key>

3. The access method is mobile access and the SIM card is a CDMA card, which means the accesstype parameter is 0 and cdma=1:

http://apilocate.amap.com/position?accesstype=0&imei=352315052834187&smac=E0:DB:55:

E4:C7:49&cdma=1&bts=13824,1,1838,1674723,575739,-52&serverip=10.2.166.4&output=xml&key=<User Key>

W200P watch: Explanation of the Gaode Wi-Fi positioning protocol's API: When selecting parameters, generally choose the first option.

You can also select the second option, but if the device has not reported the base station, both bts and nearbts parameters need to be removed for it to work.

The first option, example of parameters to enter: mmac is the first Wi-Fi MAC address in the message. http://apilocate.amap.com/position?accesstype=1&imei=869907057780989&mmac=04:8C:16:A9:FB:A8,-44,TP-Link&macs=04:8C:16:A9:FB:A8,-44,TP-Link|C8:3A:35:F4:CE:18,-59,TP-Link|1C:78:4E:D5:07:FE,-79,TP-Link|C8:3A:35:9D:0B:E0,-87,TP-Link|4A:D8:90:E8:0A:82,-88,TP-Link|AC:AD:4B:C9:C3:87,-89,TP-Link|A4:16:E7:55:84:AC,-91,TP-Link&output=json&key=xxxx

Specific Gaode Wi-Fi positioning protocol PDF can be downloaded by clicking here: This document is provided by Gaode Map, for specific integration, you can directly ask Gaode Map. Note that the document name may change upon downloading, feel free to rename it.

4.3.3 Bluetooth positioning information (BLE Location) (MsgId=0xD6)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xD6 | See below for definition | CK_sum |

Payload contents

| Format | Name | Scale | Description |
| --- | --- | --- | --- |
| U8 | Type | 1 | Currently fixed at 0 |
| U8 | Total_groups | 1 | The total number of groups, which may contain multiple pieces of information, each group may have multiple iBeacons |
| Int32 | Utc | 4 | Timestamp |
| U8 | Total_PackCount | 1 | The total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identification codes) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| Int32 | Timestamp | 4 | Timestamp |
| U8 | Total_PackCount | 1 | The total number of Bluetooth beacons at the current time |
| U16 | Major0 | 2 | Major (one of the Bluetooth beacon identifiers) |
| U16 | Minor0 | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | Rssi0 | 1 | Rssi (Bluetooth beacon signal strength) |
| U16 | MajorN | 2 | Major (one of the Bluetooth beacon identifiers) |
| U16 | MinorN | 2 | Minor (one of the Bluetooth beacon identification codes) |
| S8 | RssiN] | 1 | Rssi (Bluetooth beacon signal strength) |
| 。。。 | 。。。 | 。。。 | 。。。 |

Note: In the common version, the total number of Bluetooth beacons is fixed at 1, and the maximum number of Bluetooth beacons is 4. They are sorted by signal strength from high to low. The default smartwatch only recognizes our own beacons. If you need to connect with beacons from other brands, please consult the relevant personnel via WeChat. Example:

Raw hexadecimal message: BDBDBDBDD60001E377BD670443271794AC43273094AA4327B956A54327FE94A56A

bdbdbdbd: Header

D6: Message ID

00: type

01: Only one set of ibeacon data

E377BD67:Timestamp  converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

04: First set with 4 beacon information

4327: Converted to big-endian-->major : 0x2743-->converted to decimal 10051

1794: Converted to big-endian-->minor: 0x9417-->converted to decimal 37911

AC:  RSSI 0xAC-->converted to decimal signed -84

4327: Converted to big-endian-->major: 0x2743-->converted to decimal 10051

3094: Converted to big-endian-->minor: 0x9430-->converted to decimal 37936

AA:  RSSI 0xAA-->convert to decimal two's complement -86

4327:convert to big-endian-->major: 0x2743-->convert to decimal 10051

b956:convert to big-endian-->minor:0x56b9-->convert to decimal 22201

A5:   RSSI 0xA5-->convert to decimal two's complement -91

4327: Converted to big-endian-->major: 0x2743-->converted to decimal 10051

FE94: convert to big-endian-->minor:0x94FE-->convert to decimal 38142

A5: rssi:0xA5-->convert to decimal two's complement -91

6a --checksum

Bluetooth beacon major, minor description: The default shipped Bluetooth beacon has a label code,

For example: 00081005345866, then the Bluetooth beacon's major is 10053, minor is 45866, and the manufacturer identifier is 0008 (this is of no use and practical significance)

4.4 Device Information and Status Reporting

4.4.1 Device Charging Status Upload (MSGID=0xC3)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Status | / | / | 0: Start, 1: End, 2: Fully Charged |
| 1 | U32 | Timestamp | / | / | Timestamp (will be added at the end when re-uploaded) |

Note: This message requires support from the device firmware; the latest general version is supported by default. 
    Example:

Raw hexadecimal message: BDBDBDBDC301E377BD678A

BDBDBDBD : Header

C3: Message ID

01: Device has finished charging

E377BD67:Timestamp  converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

8A: Checksum

Raw hexadecimal message: BDBDBDBDC300DB4D2F668A

BDBDBDBD : Header

C3: Message ID

00: Device has started charging

E377BD67:Timestamp  converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

8A: Checksum

Original Hex Message: BDBDBDBDC302DB4D2F668A

BDBDBDBD : Header

C3: Message ID

02:The device is fully charged

E377BD67:Timestamp  converted to big-endian 67BD77E3-->converted to decimal 1740470243-->timestamp is 1740470243 seconds-->

Converted to standard time format UTC: 2025-02-25 07:57:23-->Converted to Beijing time: 2025-02-25 15:57:23

8A: Checksum

4.4.2 Status Parameter Reporting (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Number of Parameter Types |
| 1 |  |  |  |  | (Reserved 00) |
| 1 | u8 | Type | -/- | -/- | Parameter Type 1, Type Definition: 00—MCU (Firmware), 01—Module, 02—Sensor (General version does not have this type) |
| 1 | U8 | NameLen | -/- | -/- | Length of Parameter Name 1 |
| n | N*u8 | Name | -/- | -/- | Parameter Name |
| 1 | u8 | Type | -/- | -/- | Parameter Type 2, Type Definition: 00—MCU (Firmware), 01—Module, 02—Sensor (This type is not available in the general version) |
| 1 | U8 | NameLen | -/- | -/- | Parameter Type 2 Length |
| n | N*u8 | Name | -/- | -/- | Parameter Name |
| 。。。 | 。。。 | 。。。 | 。。。 | 。。。 | 。。。 |

Note: This message is set to report once upon startup, uploading the device firmware version number and module version number, which do not need to be parsed. Previously shipped devices have a Message ID of 0xBB; please contact relevant personnel for issuance.

Example:

BDBDBDBDA902000009423732352E4F56303601174E5432364B434E4230304E4E412D4C3032303330393530B5

BDBDBDBD : Header

A9:Message ID

02:TypeCnt indicates there are 2 types of parameters

00:Reserved Field

00:Type=00  00 represents MCU (Firmware)

09 : Length of the following MCU name is 9 bytes

423732352E4F563036--->Hexadecimal to text (Device firmware version) ---> B725.OV06

01 : Type=01  01 represents the module

17 : Length of the following communication module name is 23 bytes

4E5432364B434E4230304E4E412D4C3032303330393530 : Hexadecimal to text (4G module version)

--->NT26KCNB00NNA-L02030950

B5---Checksum

4.4.3 Device Status (MSGID=0xE9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xE9 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | type |  | / | Default—00 |
| 2 | u16 | length |  | / | Length of the subsequent message |
| 1 | u8 | location |  | / | Positioning report frequency: 00 — Default not modified, 01 — Modified via downlink (if there are multiple time periods in the downlink, only report the frequency of the current time period) |
| 2 | u16 | frequency |  | / | Frequency, unit - minutes |
| 1 | u8 | health |  | / | Health report frequency: 00 — Default not modified, 01 — Modified via downlink |
| 2 | u16 | frequency |  | / | Frequency: unit - minutes |

Note: This message will report once upon startup, and will report again after the frequency is modified via downlink

Example:

Original Hexadecimal Message: BDBDBDBDE9000600010A00010A0033

BDBDBDBD : Header

E9: Message ID

00: type=00

0600: length The length of the following message converted to big-endian --> 0006 --> converted to decimal 6, the message length is 6 bytes

01: location 01 indicates that the downlink has modified the positioning reporting frequency

0A00: frequency Positioning reporting frequency converted to big-endian --> 000A --> converted to decimal 10, the reporting frequency is 10 minutes

01: health 01 indicates that the downlink has modified the health reporting frequency

0A00: frequency Health reporting frequency converted to big-endian --> 000A --> converted to decimal 10, the reporting frequency is 10 minutes

33: checksum

If the device has no health reporting function or no positioning reporting function, then the health reporting frequency and

The positioning reporting frequency will remain consistent between the two values, indicating the device's reporting frequency.

4.5 Health-related Reporting

4.5.1 Health Data (MSGID=0x32)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x32 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | 00 |
| 4 | U32 | Timestamp |  |  | Timestamp |
| 2 | U16 | contentLength |  |  | Total length of the content that follows |
| 1 | U8 | ID |  |  | Id (type + reported value length), ID type definition is provided below |
| N | N*U8 | Val1 |  |  | Reported value of Id (the byte length varies according to the reported value length) |
| 。。。 | 。。。 | 。。。 | 。。。 | 。。。 | 。。。 |
| 1 | U8 | ID |  |  | Id (type + reported value length), ID type definition is provided below |
| N | N*U8 | Val1 |  |  | Reported value of Id (the byte length varies according to the reported value length) |

ID Type Definition (Hexadecimal):

01  Step Count  02  Heart Rate  03  Body Temperature  04  Wrist Temperature  06  Diastolic Pressure  07  Systolic Pressure  08  Blood Oxygen  09  HRV (Special Version Support)

( 05  Blood Glucose  10  Number of Sit-ups  11  Running Pace and Distance  12  Jump Rope Speed and Count ) --- Reserved, None

Default General Version Support:  01  Step Count  02  Heart Rate  03  Body Temperature  04  Wrist Temperature  06  Diastolic Pressure  07  Systolic Pressure  08  Blood Oxygen

Example:

Original 16 Hexadecimal Message: BDBDBDBD3200E377BD6711000A1E00114B314A39711A4A0122bc00416212

BDBDBDBD: Header

32: Message ID

00: type      type = 00

E377BD67: Timestamp Convert to Big-endian  67 BD77E3 --> Convert to Decimal  1740470243 --> Timestamp is 1740470243 seconds -->

Converted to standard time format UTC: 2025-02-25 07:57:23 --> Converted to Beijing time: 2025-02-25 15:57:23

1100: Subsequent Message Length Convert to Big-endian --> 0011 --> Convert to Decimal  17, Subsequent Message Length is 17 bytes

0A:ID  0x0A--> Convert to Binary00001010   First five bits are00001--> Convert to Hexadecimal01, representing data ID step count (0x01),

The last three digits are 010 --> Convert to decimal 02, representing the length of the step data as 2 bytes

1E00: Step reporting value converted to big-endian --> 001 E --> Convert to decimal 30, total step data is 30 steps

11: ID 0x11 --> Convert to binary 00010001  The first five digits are 00010 --> Convert to hexadecimal 02, representing data ID heart rate (0x02),

The last three digits are 001 --> Convert to decimal 1, representing the length of the heart rate data as 1 byte

4 B: Heart rate reporting value converted to decimal --> 75, heart rate data is 75

31: ID 0x31 --> Convert to binary 00110001  The first five digits are 00110 --> Convert to hexadecimal 06, representing data ID diastolic pressure (0x06),

The last three digits are 001 --> Convert to decimal 1, representing the length of the diastolic pressure data as 1 byte

4 A: Blood pressure diastolic pressure reporting value converted to decimal --> 74, diastolic pressure data is 74

39: ID 0x3 --> Convert to binary 00111001  The first five digits are 00111 --> Convert to hexadecimal 07, representing data ID systolic pressure (0x07),

The last three digits are 001 --> Convert to decimal 1, representing the length of the systolic pressure data as 1 byte

71: Blood pressure systolic pressure reporting value, converted to decimal -->113, systolic pressure data is113

1A:ID  0x1A--> converted to binary00011010   The first five bits are00011--> converted to hexadecimal03, representing data ID body temperature (0x03),

The last three bits are010--> converted to decimal2, representing the length of body temperature data as2bytes

4A01: Body temperature reporting value, converted to big-endian -->014A--> converted to decimal330-->330*0.1--> body temperature data is33degrees Celsius

22:ID  0x22--> converted to binary00100010   The first five bits are00100--> converted to hexadecimal04, representing data ID wrist temperature (0x04),

The last three bits are010--> converted to decimal2, representing the length of wrist temperature data as2bytes

BC00: Wrist temperature reporting value, converted to big-endian -->00BC--> converted to decimal188-->188*0.1--> wrist temperature data is18.8degrees Celsius

41:ID  0x41--> converted to binary01000001    The first five bits are01000--> converted to hexadecimal08, code data ID blood oxygen (0x08),

The last three bits are001--> converted to decimal1, representing blood oxygen data length as1byte

62: Blood oxygen reporting value, converted to decimal -->98, blood oxygen data is98

12 : checksum

4.5.2 Device Sleep Analysis Data Upload (MSGID=0xC5)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xC5 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | Int32 | DateTime |  | -/- | Upload UTC Start Time |
| 4 | Int32 | DateTime |  |  | Upload UTC End Time |
| 2 | Int16 | Sleepminute |  |  | Uploaded Sleep Time in Minutes |
| 4 | Int32 | Type |  |  | Sleep Status: Upload Type, 1 for Deep Sleep, 2 for Light Sleep, 3 for Duration Awake |

Example:

Raw 16 Hex Message: BDBDBDBDC5AC338860693B8860210001000000D1

BDBDBDBD: Header

C5: Message ID

AC338860: Start time, converted to big-endian  608833  AC --> convert to decimal --> timestamp is  1619538860  seconds -->

Convert to standard time format UTC time:  2021  -  04  -  27      15  :  54  :  20 --> convert to Beijing time:  2021  -  04  -  27      23  :  54  :  20

693  B8860: End time, converted to big-endian  60883  BAC --> convert to decimal --> timestamp is  1619540908  seconds -->

Convert to standard time format UTC time:  2021  -  04  -  27      16  :  28  :  28 --> convert to Beijing time:  2021  -  04  -  28      00  :  28  :  28

2100: Sleepminute sleep time, converted to big-endian  0021 --> convert to decimal  33 --> total sleep time is  33  minutes

01000000: Type sleep state, converted to big-endian  00000001 --> convert to decimal  1 -->  1 is deep sleep, deep sleep time is  33  minutes

D1: checksum

4.6 Downlink feedback related reports

4.6.1 Downlink feedback (MSGID=0xC0)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xC0 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | length | -/- |  | Message ID Length |
| N | n*U8 | Type | -/- |  | N Message IDs |

Example:

This command is used for feedback on the downlink command, returning the Message IDs of previously received downlink commands (multiple Message IDs can be returned collectively).

Original Hex Message: BDBDBDBDC0011720

BDBDBDBD : Header

C0: Message ID

01: Length Message ID length converted to decimal 01 --> Message ID length is 1 byte

17: Type indicates that the device received the downlink command Message ID as 17

20: checksum

4.6.2 Message Status Report (MSGID=0x28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x28 | See below for definition | CK_sum |

Payload contents

| Bytes | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 4 | U32 | timestamp | -/- | -/- | Timestamp |
| 1 | U8 | Type | -/- | -/- | Character encoding type (corresponding to the following 0X28), GB2312 encoding: 0x03; Unicode encoding: 0x05 |
| 1 | U8 | status | -/- | -/- | Message status - 0x01: Confirmed, 0x02: Rejected |
| 4 | U32 | seqID | -/- | -/- | (corresponding message ID seqID for the following 0X28) |

Example:

Note: The current generic version does not distinguish between character encoding types, fixed to 03, later versions will differentiate.

Original hex message: BDBDBDBD28FD3BA068030268A03BE477

BDBDBDBD : Header

28:Message ID

FD3BA068: timestamp timestamp converted to big-endian 68A03BFD --> converted to decimal --> timestamp is 1755331581 seconds -->

Convert to standard time format UTC time: 2025-08-16 08:06:21-->Convert to Beijing time: 2025-08-16 16:06:21

03:Type 03--Fixed Value

02:status message status 02--Interface click reject button--Rejected

68A03BE4:seqID The ID of the downlink message is 68A03BE4

77:checksum Checksum

5 Settings

5.1 Downlink

5.1.1 Set positioning reporting frequency (MSGID=0x17)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x17 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether enabled, 1 for enabled, 0 for not enabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether enabled, 1 for enabled, 0 for not enabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether enabled, 1 for enabled, 0 for not enabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether enabled, 1 for enabled, 0 for not enabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |

Note: One day's time is from 00:00 to 23:59
Example:

00:00-23:59Report location every 10 minutes: BDBDBDBD17010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

17:Message ID

01:Enable 是否启用  Convert to decimal01-->1--enabled

0A00:Interval 时间间隔 Convert to big-endian000A-->Convert to decimal10-->Interval is10minutes

00:time_start_h Start time - hours, convert to decimal00-->Start time - hours is00

00:time_start_m Start time - minutes, convert to decimal00-->Start time - minutes is00

17:time_end_h End time - hours, convert to decimal23--> Start time - hours is00

3B:time_end_m End time - minutes, convert to decimal59--> Start time - hours is00

00:enable Whether enabled, convert to decimal00-->0-- Not enabled

000000000000: Unused status fully filled00

00:enable Whether enabled, convert to decimal00-->0-- Not enabled

000000000000: Unused status fully filled00

00:enable Whether enabled, convert to decimal00-->0-- Not enabled

000000000000: Unused status fully filled00

97:checksum Checksum

5.1.2 Message Sending (MSGID=0X28)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x28 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Description |
| --- | --- | --- | --- |
| 1 | U8 | type | Message type, if it is a downlink text message, the Unicode code is 05 |
| 4 | Uint32 | seqID | Unique ID of the information |
| 1 | U8 | CONTENT LEN | Content Length |
| N | N | CONTENT | Content, in Chinese as Unicode encoding (little-endian, note the conversion; one Chinese character occupies 2 bytes, one letter occupies 2 bytes) |

Note: The default Chinese character encoding for the Hong Kong universal version is Unicode.  
    Each information ID must be unique; duplicate IDs cannot be received by the device.  
    Example:

Text message: hello,worldBDBDBDBD28050300000016680065006C006C006F002C0077006F0072006C006400DD

BDBDBDBD : Header

28    :  Message ID

05    :  Type of message, decimal conversion 05 --> type=05

03000000    :  seqID information ID, information ID is 03000000

16    :  CONTENT LEN length of content, decimal conversion 22, length of content is 22 bytes

680065006C006C006F002C0077006F0072006C006400    :

Convert every 2 bytes to big-endian --> convert to Unicode character --> combine to form final text message content: hello,world

DD:checksum

5.1.3 Setting - Positioning Priority Setting (0XCE01)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

Type=01 Positioning Priority Setting

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Type=01 |
| 1 | u8 | Valid | -/- | -/- | Validity, 00 always valid, 01 valid for this time |
| 2 | U16 | Len | -/- | -/- | Length of the subsequent instruction |
| n | n |  | -/- | -/- | Main body |

Positioning priority setting definition -- See 2.3 Device Downlink Description -- Device positioning priority issuing (0 xCE01)

Positioning Priority Set Text: Currently, the general version supports: 01--Satellite (GPS/BDS), 02--wifi (wifi + base station) 03--Bluetooth beacon (requires additional deployment of Bluetooth beacons); special device support: 04--LBS base station 05--125k

Example:

wifi positioning priority (wifi > Bluetooth beacon > Satellite (GPS/BDS)) : BDBDBDBDCE0100030002030133

BDBDBDBD : Header

CE : Message ID

01 : Type 01 Positioning Priority Setting

00 : Validity 00 always valid

0300 : Len converted to Big Endian 0003 --> converted to decimal 03 --> text length is 3 bytes

020301 : Text: 02--wifi 03--Bluetooth beacon 01--Satellite (GPS/BDS) positioning priority is wifi > Bluetooth beacon > Satellite (GPS/BDS)

33: checksum

Satellite (GPS/BDS) positioning priority (Satellite (GPS/BDS) > wifi > Bluetooth beacon) : BDBDBDBDCE0100030001020333

BDBDBDBD : Header

CE : Message ID

01 : Type 01 Positioning Priority Setting

00 : Validity 00 always valid

0300 : Len converted to Big Endian 0003 --> converted to decimal 03 --> text length is 3 bytes

010203 : Text: 01--Satellite (GPS/BDS) 02--wifi 03--Bluetooth beacon positioning priority is Satellite (GPS/BDS) > wifi > Bluetooth beacon

33: checksum

Bluetooth positioning priority (Bluetooth beacon > wifi > Satellite (GPS/BDS)) : BDBDBDBDCE0100030003020133

BDBDBDBD : Header

CE : Message ID

01 : Type 01 Positioning Priority Setting

00 : Validity 00 always valid

0300 : Len converted to Big Endian 0003 --> converted to decimal 03 --> text length is 3 bytes

030201:Main text: 03--Bluetooth Beacon 02--WiFi 01--Satellite (GPS/BDS)  The positioning priority is Bluetooth Beacon > WiFi > Satellite (GPS/BDS)

33: checksum

Single Satellite (GPS/BDS) Positioning:BDBDBDBDCE0100030001000033

BDBDBDBD : Header

CE : Message ID

01 : Type 01 Positioning Priority Setting

00 : Validity 00 always valid

0300 : Len converted to Big Endian 0003 --> converted to decimal 03 --> text length is 3 bytes

030201:Main text: 01--Satellite (GPS/BDS) 00--No Positioning 00--No Positioning The positioning priority is Single Satellite (GPS/BDS) positioning

33: checksum

Single WiFi Positioning:BDBDBDBDCE0100030002000033

BDBDBDBD : Header

CE : Message ID

01 : Type 01 Positioning Priority Setting

00 : Validity 00 always valid

0300 : Len converted to Big Endian 0003 --> converted to decimal 03 --> text length is 3 bytes

020000:Main text: 02--WiFi 00--No Positioning 00--No Positioning The positioning priority is Single WiFi positioning

33: checksum

Single Bluetooth Beacon Positioning:BDBDBDBDCE0100030003000033

BDBDBDBD : Header

CE : Message ID

01 : Type 01 Positioning Priority Setting

00 : Validity 00 always valid

0300 : Len converted to Big Endian 0003 --> converted to decimal 03 --> text length is 3 bytes

020000:Main text: 03--Bluetooth Beacon 00--No Positioning 00--No Positioning The positioning priority is Single Bluetooth Beacon positioning

33: checksum

5.1.4 Settings - Health Sampling Report Frequency Setting (MSGID=0X1C)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x1C | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether enabled, 1 for enabled, 0 for not enabled | Time period 1 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether enabled, 1 for enabled, 0 for not enabled | Time Period 2 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether enabled, 1 for enabled, 0 for not enabled | Time Period 3 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether enabled, 1 for enabled, 0 for not enabled | Time Period 4 |
| 2 | U16 | Interval |  |  | Time interval (minutes) |  |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |  |

Health Sampling Report Frequency Setting Definition -- See 2.3 Device Downlink Description -- Device Health Sampling Frequency Issuance (0x1D)

Note: After setting, reports will be submitted based on the time period; outside the time period, no reports will be submitted.

Example:

00:00-23:59(All day) Health data reported every 10 minutes: BDBDBDBD1C010A000000173B00000000000000000000000000000000000000000097

BDBDBDBD:Header

1C:Message ID

01:Enable 是否启用  Convert to decimal01-->1--enabled

0A00:Interval 时间间隔 Convert to big-endian000A-->Convert to decimal10-->Interval is10minutes

00:time_start_h Start time - hours, convert to decimal00-->Start time - hours is00

00:time_start_m Start time - minutes, convert to decimal00-->Start time - minutes is00

17:time_end_h End time - hour, converted to decimal23--> End time - hour is23

3B:time_end_m End time - minute, converted to decimal59--> End time - minute is59

00:enable Whether enabled, convert to decimal00-->0-- Not enabled

000000000000: Unused status fully filled00

00:enable Whether enabled, convert to decimal00-->0-- Not enabled

000000000000: Unused status fully filled00

00:enable Whether enabled, convert to decimal00-->0-- Not enabled

000000000000: Unused status fully filled00

97:checksum Checksum

5.1.5 Set - Device Alarm Settings (0XCE03)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

Type=03 Device Alarm Settings

| Byte offset | Format | Name | Unit | Description |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Fixed to 0x03—Device Alarm Settings |
| 1 | U8 | Valid | 1 | Fixed to 0x00—Health threshold, always valid |
| 2 | U16 | Cfg Len | 2 | Instruction length |
| 1*N | U8*N | [Bit7] Enable [Bit6 - 0] Threshold Type | 1*N | [Bit7] Whether to enable this type of threshold detection [Bit6 - 0] Set threshold type: 0x01 Heart Rate, 0x02 Systolic Blood Pressure (SBP) 0x03 Diastolic Blood Pressure (DBP), 0x04 Blood Oxygen, 0x05 Temperature (threshold unit is *10, for example, 37.3 is 373) |
| 2*N | I16*N | Threshold Min | 2*N | Lower limit of threshold (this content is only available when threshold detection is enabled) |
| 2*N | I16*N | Threshold Max | 2*N | Upper limit of threshold (this content is only available when threshold detection is enabled) |

Device Alarm Settings Definition - Health Threshold -- See 2.3 Device Downstream Description -- Health Threshold Settings (0 xCE0300)

Note: The default is the off state, and the threshold is in the normal range. Devices will report alarms when outside this range.

The heart rate threshold range is (20 - 200), the blood pressure systolic threshold range is (20 - 200), the blood pressure diastolic threshold range is (20 - 200), the blood oxygen threshold range is (80 - 100), and the temperature threshold range is (160 - 600).

Example:

Set multiple health thresholds: 1. Heart rate [60, 120], 2. Turn off blood oxygen threshold detection, 3. Temperature [36.0, 37.3]: BDBDBDBDCE03000B00813C007800048568017501FF

BDBDBDBD: Header

CE: Message ID

03: Type 03 -- Device alarm settings

00: Valid 00 -- Health thresholds, persistently valid

0 B00: Convert to big-endian --> 000 B --> Convert to decimal 11 --> Length of the following command message (not including checksum) 11 bytes

81: Convert to binary --> 10000001 bit7: 1 --- Enabled    On state   bit6- 0: 000001 --> 0x01  Heart rate

3 C00: Convert to big-endian --> 003 C --> Convert to decimal 60, heart rate lower threshold is 60

7800: Convert to big-endian --> 0078 --> Convert to decimal 120, heart rate upper threshold is 120

04   : Convert to binary -->  00000100     bit7:  0  --- Off, no threshold after off state   bit6- 0 :  0000100  -->  0x04    Blood Oxygen

85   : Convert to binary -->  10000101    bit7:  1  --- On     On state   bit6- 0 :  0000101  -->  0x05    Temperature

6801   : Convert to big-endian -->  0168  --> Convert to decimal  360  (actual temperature *  10) --> Minimum temperature threshold is  36.0

7501   : Convert to big-endian -->  0175  --> Convert to decimal  373  (actual temperature *  10) --> Maximum temperature threshold is  37.3

FF: Checksum

5.1.6 Settings - Switch Settings (0XCE04-24)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- | -/- | Please refer to the instructions for specific types |
| 1 | u8 | switch | -/- | -/- | Switch 00 On, 02 Off, 01 Single valid |
| 2 | U16 |  | -/- | -/- | Fixed 0000 |

Device Alarm Settings - Switch Type: Specific Function Description -- See 2.3 Device Downlink Explanation

05 Bluetooth Broadcast Switch

07 Fall Alarm Switch        08 Stay Alarm Switch

0A Immediately Report Location Information    0B Immediately Report Health Data

0D Immediately Report Version Number Data  16 Can Downlink Use Key Shutdown

18 Sleep Function Switch        19 Key Trigger SOS Alarm Switch

20 Data Reporting Switch-- Separate Settings (Note: This switch structure is special, and there are differences in message and table definitions. Please refer to specific examples)

Type=05 Bluetooth Broadcast Switch
Example:

00---On:BDBDBDBDCE0500000093

BDBDBDBD : Header

CE : Message ID

05:Type 05 Bluetooth Broadcast Switch

00:Switch 00 On

0000:Fixed value

93:Checksum

02---关闭:BDBDBDBDCE0502000093

BDBDBDBD : Header

CE : Message ID

05:Type 05 Bluetooth Broadcast Switch

02:Switch 02 closed

0000:Fixed value

93:Checksum

Type=07 Fall alarm switch
Example:

00---开启:BDBDBDBDCE0700000093

BDBDBDBD : Header

CE : Message ID

07:Type 07 fall alarm switch

02:Switch 00 opened

0000:Fixed value

93:Checksum

02---关闭:BDBDBDBDCE0702000093

BDBDBDBD : Header

CE : Message ID

07:Type  07 fall alarm switch

02:Switch 02 closed

0000:Fixed value

93:Checksum

Type=08 Stop Alarm Switch
Example:

00---Open: BDBDBDBDCE0800000093

BDBDBDBD : Header

CE : Message ID

08: Type 08 Stay Alarm Switch

02:Switch 00 opened

0000:Fixed value

93:Checksum

02---Close: BDBDBDBDCE0802000093

BDBDBDBD : Header

CE : Message ID

08: Type 08 Stay Alarm Switch

02:Switch 02 closed

0000:Fixed value

93:Checksum

Type=0A Report Position Information Immediately
Example:

01---Single Report: BDBDBDBDCE0A01000093

BDBDBDBD : Header

CE : Message ID

0A: Type 0A Report Position Information Immediately

01: switch 01--Single Issuance Single Report

0000:Fixed value

93:Checksum

Type=0B Report Health Data Immediately
Example:

01---Single Report: BDBDBDBDCE0B01000093

BDBDBDBD : Header

CE : Message ID

01: Type 0B immediate report health data

02: switch switch 01--single dispatch single report

0000:Fixed value

93:Checksum

Type=0D immediate report version number data  
    Example:

01---single report: BDBDBDBDCE0D01000093

BDBDBDBD : Header

CE : Message ID

0D: Type 0D immediate report version number data

01: switch 01--Single Issuance Single Report

0000:Fixed value

93:Checksum

Type=16 can the downlink use the button to shut down?  
    Example:

00---enable: BDBDBDBDCE1600000093

BDBDBDBD : Header

CE : Message ID

16: Type 16 can the downlink use the button to shut down?

02:Switch 00 opened

0000:Fixed value

93:Checksum

02---disable: BDBDBDBDCE1602000093

BDBDBDBD : Header

CE : Message ID

16: Type 16 can the downlink use the button to shut down?

02:Switch 02 closed

0000:Fixed value

93:Checksum

Type=18 Sleep Function Switch
Example:

00---Enabled, enters sleep mode after a period of inactivity, data not reported: BDBDBDBDCE1800000093

BDBDBDBD : Header

CE : Message ID

18:Type 18 sleep function switch

02:Switch 00 opened

0000:Fixed value

93:Checksum

02---Disabled, in a static or charging state, data is reported normally: BDBDBDBDCE1802000093

BDBDBDBD : Header

CE : Message ID

18:Type 18 sleep function switch

02:Switch 02 closed

0000:Fixed value

93:Checksum

Type=19 Button-triggered SOS alarm switchExample:

00---Enabled:BDBDBDBDCE1900000093

BDBDBDBD : Header

CE : Message ID

19:Type 19 button-triggered SOS alarm switch

02:Switch 00 opened

0000:Fixed value

93:Checksum

02---Disabled:BDBDBDBDCE1902000093

BDBDBDBD : Header

CE : Message ID

19:Type 19 button-triggered SOS alarm switch

02:Switch 02 closed

0000:Fixed value

93:Checksum

Type=20 Data reporting switch—set separatelyExample:

Add field: Body length: 2 bytesBody: 0x00 (positioning data report) + switch (0x00: enabled, 0x02: disabled)

0x01 (Health data reporting) + switch (0x00: On, 0x02: Off)

Positioning data reporting On:BDBDBDBDCE20000200000093

BDBDBDBD : Header

CE : Message ID

20:Type 20 reporting data switch

00:Fixed value

0200:Body length  converted to big-endian 0002 --> converted to decimal 02 --> body length is 2 bytes

00:Body: 00 -- Positioning data reporting switch

00:Body: 00 -- Start reporting

93:Checksum

Positioning data reporting Off:BDBDBDBDCE2000020000293

BDBDBDBD : Header

CE : Message ID

20:Type 20 reporting data switch

00:Fixed value

0200:Body length  converted to big-endian 0002 --> converted to decimal 02 --> body length is 2 bytes

00:Body: 00 -- Positioning data reporting switch

02:Body: 00 -- Stop reporting

93:Checksum

Health data reporting On:BDBDBDBDCE20000200010093

BDBDBDBD : Header

CE : Message ID

20:Type 20 reporting data switch

00:Fixed value

0200:Body length  converted to big-endian 0002 --> converted to decimal 02 --> body length is 2 bytes

01: Main Text: 00--Health Data Reporting Switch

00:Body: 00 -- Start reporting

93:Checksum

Health Data Reporting Closed: BDBDBDBDCE20000200010293

BDBDBDBD : Header

CE : Message ID

20:Type 20 reporting data switch

00:Fixed value

0200:Body length  converted to big-endian 0002 --> converted to decimal 02 --> body length is 2 bytes

01: Main Text: 00--Health Data Reporting Switch

02:Main Text:02--Close Reporting

93:Checksum

5.1.7 Settings - Drop Sensitivity Setting (0xCE15)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Description |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Type is 0x15 |
| 1 | U8 | Valid | 1 | Default is 0, no impact at this time |
| 2 | U16 | Len | 2 | Length of subsequent parameter content |
| 1 | U8 | Param Type | 1 | Parameter type: 0x00: Sensitivity, 0x01: Height |
| 1 | U8 | Param Level | 1 | Parameter level: 5 levels (0 - 4) |

Drop Sensitivity Setting - Parameter Description:

Sensitivity: Refers to the degree that triggers the drop algorithm, offering 5 setting levels (0 - 4): Low - Lower (Medium-Low) - Medium - Higher (Medium-High) - High.

Height: Refers to the height at which the drop alarm is triggered, offering 5 setting levels (0 - 4): 0.5m - 1.0m - 1.5m - 2.0m - 2.5m.

If the height is set to "1.5m," the device must at least be ≥ 1.5m to potentially trigger the drop alarm.

Note that sensitivity and height cannot be combined into a single message.

Example:

Set sensitivity to "Medium-Low": BDBDBDBDCE150002000001FF

BDBDBDBD : Header

CE : Message ID

15:Type Drop Sensitivity Setting

00:Fixed value

0200:Len

00:Param Type Sensitivity Setting

01:Param Level Sensitivity Level 1 --> Lower (Medium-Low)

FF: checksum check sum

Set height to "1.5m": BDBDBDBDCE150002000102FF

BDBDBDBD : Header

CE : Message ID

15:Type Drop Sensitivity Setting

00:Fixed value

0200:Len

01:Param Type Height Setting

02:Param Level Height Level 2-->1.5m

FF: checksum check sum

5.1.8 Settings - Vibration Settings (0XCE23)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xCE | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Unit | Description |
| --- | --- | --- | --- | --- |
| 1 | U8 | Type | 1 | Fixed to 0x23 |
| 1 | U8 | Valid | 1 | 0x00 |
| 2 | U16 | Cfg Len | 2 | Length of subsequent parameter content |
| 1 | U8 | Motor Type | 1 | Setting Type: 1. Vibration Duration - 0x00 |
| 4 | U32 | Value | 4 | Setting Content: 1. Vibration Duration (unit: milliseconds, range 0 - 60 seconds) |

Settings - Vibration Settings Definition: This message can modify the global vibration duration. Any resulting vibration duration will change to the duration set by this command.

Example:

Set the device's default global vibration duration to 1.5 seconds (1500 milliseconds): BDBDBDBDCE2300050000dc050000FF

BDBDBDBD : Header

CE : Message ID

23: Type 23 Settings - Vibration Settings

00: Fixed value 00 -- always valid

0500: Length of the subsequent parameter content converted to big-endian 0005 --> converted to decimal 05 --> subsequent parameter content length is 5 bytes

00: Motor Type Vibration Duration Setting

dc050000: Value Vibration Duration converted to big-endian 000005dc --> converted to decimal 1500 --> vibration duration is 1500 milliseconds (1.5 seconds)

FF: checksum check sum

5.1.9 IP & Domain Name Settings (MSGID=0xC3) (TCP Dedicated)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xC3 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | Type |  |  | Issuance Type: Type = 1 IPv4, Type = 2 IPv6 (Reserved - not currently supported), Type = 3 Domain Name (Currently only supports ASCII encoded domain names) |
| 2 | U16 | Port |  |  | Port number (2 bytes) |
| 1 | U8 | Len |  |  | Length of subsequent parameter content |
| N | U8 | Domainname |  |  | Specific IP or domain content |

Note: After issuance, the device will not communicate on the original server, pointing to the modified server issued.

Example:

Type=1 IPv4:Downlink modification IP to:118.178.184.219, port: 8825  BDBDBDBDC30179220476B2B8DB33

BDBDBDBD : Header

C3: Message ID

01:Type 1--IPv4

7922:Port converted to big-endian 2279-->converted to decimal 8825-->port: 8825

04:Len length of the following parameter content converted to decimal 04-->length is 4 bytes

76B2B8DB:Domainname IP  76-->converted to decimal 118, B2-->converted to decimal 178, B8-->converted to decimal 184, DB-->converted to decimal 219   IP is: 118.178.184.219

33: checksum

Type=3 Domain nameDownlink modification domain name to:aiday.com.cn, port: 8825

BDBDBDBD : Header

C3: Message ID

03:type=3, domain name

7922:Port endianness conversion to big-endian --2279  --> Convert to decimal  Port: 8825

12:Length of the parameters after Len converted to decimal 18 --> Message length is 18 bytes

61696461792e636f6d2e636e:Domain name converted to string --> aiday.com.cn --> Domain name is: aiday.com.cn

1D:checksum

5.1.10 Issuing sedentary stay alarm trigger time (MSGID=0XCC)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xCC | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | minute | -/- | -/- | Stay alarm trigger duration (unit: minutes; range 2 — 60), inactivity within this time will trigger the stay alarm |

Example:

No activity for 5 consecutive minutes triggers stay alarm on the watch (in wearing state): BDBDBDBDCC05DD

BDBDBDBD: Header

CC:Message ID

05: minute The duration of the stay alarm trigger converts to decimal 05 --> 5 minutes

DD: checksum

5.1.11 Power off and restart (MSGID=0x77)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x77 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | Type | -/- |  | 01—Power off, 00—Restart |

Note: Power off and restart are both received while in the powered-on state. Power off: Command received in the powered-on state, the watch powers off and cannot receive any commands afterward; Restart: Command received in the powered-on state, the watch restarts.

Example:

Power off: BDBDBDBD770193

BDBDBDBD : Header

77:  Message ID

01:  Type 01--Power off

93:Checksum

Restart: BDBDBDBD770093

BDBDBDBD : Header

77:  Message ID

01: Type 00--Restart

93:Checksum

5.1.12 Personal Information Distribution (MSGID=0xCA)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xCA | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | U8 | count | -/- |  | Number of information sets distributed |
| 1 | U8 | type | -/- |  | Type: 1 – Name, 2 – English Name, 3 – Phone, 4 – Blood Type |
| 1 | U8 | len | -/- |  | Length |
| N | U8 | content | -/- |  | Content (Unicode encoding) |
| 。。。 | 。。。 | 。。。 | 。。。 | 。。。 | 。。。 |

Example:

Name: Cheng Da Wen, English name: Chan Tai Man, Phone: 99886633, Blood type: A

BDBDBDBDCA0401060B7A2759876502184300680061006E00200054006100690020004D0061006E0003103900390038003800360036003300330004024100DD

BDBDBDBD: Header

CA:Message ID

04:count converted to decimal04-->4 groups of information

01:type 01--Name

06:len converted to decimal06-->Length of 6 bytes

0B7A27598765:content

0B7A converted to Big Endian-->7A0B-->converted to Unicode character-->Cheng,

2759 converted to Big Endian-->5927-->converted to Unicode character-->Da,

8765 converted to Big Endian-->6587-->converted to Unicode character-->Wen  The Chinese name is Cheng Dawen

02:type 02--English name

18 len Convert to Decimal24--> Length 24 bytes

4300680061006E00200054006100690020004D0061006E00:content

Similarly, every2bytes is converted to big-endian--> Convert to Unicode characters --> Final: Chan Tai Man

03:type 03-- Phone number

10:len Convert to Decimal16--> Length 16 bytes

39003900380038003600360033003300:content

Similarly, every2bytes is converted to big-endian--> Convert to Unicode string --> Final: 99886633

04:type 04-- Blood type

02:len Convert to Decimal02--> Length 2 bytes

41: Convert to Unicode string--> A --> Blood type: A

DD: checksum

5.1.13 Sleep Statistics Time Period Settings (MSGID=0X1D)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x1D | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | u8 | enable | -/- | -/- | Whether enabled, 1 for enabled, 0 for not enabled | Time period 1 |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether enabled, 1 for enabled, 0 for not enabled | Time Period 2 |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether enabled, 1 for enabled, 0 for not enabled | Time Period 3 |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |
| 1 | u8 | enable | -/- | -/- | Whether enabled, 1 for enabled, 0 for not enabled | Time Period 4 |
| 1 | u8 | time_start_h |  |  | -hour |  |
| 1 | u8 | time_start_m |  |  | -minute |  |
| 1 | u8 | time_end_h |  |  | -hour |  |
| 1 | u8 | time_end_m |  |  | -minute |  |

Example:

The sleep data statistics and reporting time periods are 13:00 - 15:00 and 22:30 - 08:00 BDBDBDBD1D010D000F0001161E080000000000000000000000FF

BDBDBDBD : Header

1D:Message ID

01:enable whether to enable, converted to decimal 01 --> 1 -- enabled

0D:time_start_h start time - hours, converted to decimal 13

00:time_start_m start time - minutes, converted to decimal 00, the start statistics and reporting time is 13:00

0F:time_end_h end time - hours, converted to decimal 15

00:time_end_m end time - minutes, converted to decimal 00, the end statistics and reporting time is 15:00

01:enable whether to enable, converted to decimal 01 --> 1 -- enabled

16:time_start_h start time - hours, converted to decimal 22

1E:time_start_m Start time - minutes  Convert to decimal 30 Start statistic and report time at 22:30

08:time_end_h End time - hours  Convert to decimal 08

00:time_end_m End time - minutes Convert to decimal 00 End statistic and report time at 08:00

00:enable Whether enabled  Convert to decimal 00-->0--not enabled

00000000:All filled with 00 for not enabled status

00:enable Whether enabled  Convert to decimal 00-->0--not enabled

00000000:All filled with 00 for not enabled status

FF: checksum check sum

5.1.14 Remote OTA upgrade issued (MSGID=0xA9)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xA9 | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | TypeCnt | -/- | -/- | Type count |
| 1 | u8 |  | -/- | -/- | Fixed value (reserved byte 00) |
| 1 | u8 | Type | -/- | -/- | Type 1 00—mcu firmware |
| 1 | U8 | PathLen | -/- | -/- | Path 1 Length |
| n | N*u8 | Path | -/- | -/- | 路径 |
| 2 | U16 | PackageSize |  |  | The file package size is fixed at 0000 |
| 。。。 | 。。。 | 。。。 | 。。。 | 。。。 | 。。。 |
| 1 | u8 | Type | -/- | -/- | Type n 00—mcu firmware |
| 1 | U8 | PathLen | -/- | -/- | Path n length |
| n | N*u8 | Path | -/- | -/- | 路径 |
| 2 | U16 | PackageSize |  |  | The file package size is fixed at 0000 |
| 。。。 | 。。。 | 。。。 | 。。。 | 。。。 | 。。。 |

Note: Remote updates consume data and are affected by network conditions. The device SIM card has a limit of 30MB per month, and too many updates may result in insufficient data.

Example:

The firmware download address is (requires server support for HTTP download): http://tools.aiday.com.cn/File/MCU/W200PG/W200PG_E42.BWGHOL25.bin

Upon successful update, the device will restart and reconnect to the server, reporting the version number change.

BDBDBDBDA901000041687474703A2F2F746F6F6C732E61696461792E636F6D2E636E2F46696C652F4D43552F5732303050472F5732303050475F4534322E425747484F4C32352E62696E00000A

BDBDBDBD:Header

A9:Message ID

01:TypeCnt Number of Types 01--indicates1type

00:Reserved bytes, fixed value

00:Type 00--indicates MCU firmware upgrade

41:Path Download Path Length Converted to Decimal65-->Download Path Length is65

687474703A2F2F746F6F6C732E61696461792E636F6D2E636E2F46696C652F4D43552F5732303050472F5732303050475F4534322E425747484F4C32352E62696E

File Download Path Converted to String-->http://tools.aiday.com.cn/File/MCU/W200PG/W200PG_E42.BWGHOL25.bin

0000:PackageSize Currently fixed at0000

0A:checksum

Weather Alert (MSGID=0XCB)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0xCB | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 0 | U8 | Count | -/- | -/- | Number of weather warning groups |
| 1 | U8 | Type | -/- | -/- | Weather warning types, see details below |
| 1 | U8 | Status | -/- | -/- | Status, see details below |
| … | … | …… |  |  | … |

Weather type - status table:

| Type | Description | Status | notes - click the image to enlarge |
| --- | --- | --- | --- |
| 1 | Storm warning | 0 | Cancel Alarm |
|  |  | 1 - 8 |  |
| 2 | Heavy Rain Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |
| 3 | Thunderstorm Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 4 | Flood Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 5 | Debris Flow Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 6 | Strong Monsoon Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 7 | Frost Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 8 | Fire Danger Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
| 9 | Temperature Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
| 10 | Tsunami Warning | 0 | Cancel Alarm |
|  |  | 1 |  |
| 11 | Heat Index | 0 | Cancel Alarm |
|  |  | 1 |  |
|  |  | 2 |  |
|  |  | 3 |  |

Note: After being issued, the device will vibrate and display weather warning images. Multiple sets of weather warnings can be issued simultaneously, and each set must be distinct. The same type of warning cannot appear in a single command; otherwise, it will be invalid.   
    Example:

10    Sets of weather warnings issued: BDBDBDBD CB     0    A    0101      0201      0301      0401      0501      0601      0701      0801      0901      0    A    01    F    5

BDBDBDBD:Header

CB:Message ID

0A:Count of weather groups in decimal 10 --> issue 10 sets of weather warnings

0101:Type&Status Weather type-status 01 -- Storm warning 01 -- Level 1 alarm signal

0201:Type&Status Weather type-status 02 -- Heavy rain warning 01 -- Yellow heavy rain warning signal

0301:Type&Status Weather type-status 03 -- Thunderstorm warning 01 -- Thunderstorm alert

0401:Type&Status Weather type-status 04 -- Flood warning 01 -- Special flood report for Northern New Territories

0501:Type&Status Weather type-status 05 -- Landslide warning 01 -- Landslide alert

0601:Type&Status Weather Type-Status 06--Strong Monsoon Warning 01--Severe Monsoon Signal

0701:Type&Status Weather Type-Status 07--Frost Warning 01--Frost Signal

0801:Type&Status Weather Type-Status 08--Fire Hazard Warning 01--Yellow Fire Hazard Alert

0901:Type&Status Weather Type-Status 09--Temperature Warning 01--Cold Weather Alert

0A01:Type&Status Weather Type-Status 10--Tsunami Warning 01--Tsunami Alert

F5:checksum

5.1.16 Restore Factory Settings(MSGID=0X7B)

| Header | Message ID | Payload | Checksum |
| --- | --- | --- | --- |
| Token: Fixed to 0xBD 0xBD 0xBD 0xBD | 0x7B | See below for definition | CK_sum |

Payload contents

| Byte offset | Format | Name | Scale | Unit | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | u8 | reset | -/- | -/- | 1 Restore Factory Settings |

Example:

Restore factory settings, the device will restart after receiving the command: BDBDBDBD7B018F

BDBDBDBD : Header

7B : Message ID

01 : reset 01--restore factory settings

8F : checksum